| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-DOC-0070 |  |
|  |  |  | English only |  |
| WG(s): 6 |  | 3nd FG IPTV meeting: Mountain View 22 – 26 January 2007 |  |  |
| OUTPUT DOCUMENT |  |  |  |  |
| Source: | Editor |  |  |  |
| Title: | Working Document: IPTV Middleware, Application and Content Platforms |  |  |  |

CONTENTS

1	Scope	2

2	References	2

3	Definitions	3

4	Abbreviations	5

5	Conventions	7

6	Content Provisioning	7

7	Middleware	9

8	Metadata	9

9	Retransmission and Legacy Related Issues	10

10	Accessibility	10

11	Browser-based System	10

IPTV Middleware, Applications, and Content Platforms

# Scope

This working document identifies and defines middleware platforms, including applications, content formats, and their uses, that facilitate effective and interoperable use of an IPTV system for presenting and interacting with IPTV services.

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this working document. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this working document are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this working document does not give it, as a stand-alone document, the status of a Recommendation.

[H.222AM1]	ITU-T Recommendation H.222.0 Amd 1(2002), Information technology – Generic coding of moving pictures and associated audio information: Systems Amendment 1: Carriage of metadata over ITU-T Rec. H.222.0 | ISO/IEC 13818-1 streams

[Y.101]	ITU-T Recommendation Y.101 (2000), Global Information Infrastructure terminology: Terms and definitions

[J.90]	ITU-T Recommendation J.90 (2000), Electronic programme guides for delivery by digital cable television and similar methods - Reference operating scenario and requirements

[J.200]	ITU-T Recommendation J.200 (2001), Worldwide common core  Application environment for digital interactive television services

[J.190]	ITU-T Recommendation J.190 (2002), Architecture of MediaHomeNet that supports cable-based services

[Q.1702]	ITU-T Recommendation Q.1702 (2002), Long-term vision of network aspects for systems beyond IMT-2000

[T.171]	ITU-T Recommendation T.171 (1996), Protocols for interactive audiovisual services: Coded representation of multimedia and hypermedia objects

[F.700]	ITU-T Recommendation T.700 (2000), Framework Recommendation for multimedia services

[T.174]	ITU-T Recommendation T.174 (1996), Application programming interface (API) for MHEG-1

[J.98]	ITU-T Recommendation J.98 (2003), Metadata requirements for video-on-demand in cable networks

[J.148]	ITU-T Recommendation J.148 (2003), Requirements for an objective perceptual multimedia quality model

# Definitions

This working document uses or defines the following terms:

Note: In this clause [aaa] notation after title of a term indicates source of the definition for that term.

  - Application [Y.101]: A functional implementation realised as software running in one or spread over several interplaying hardware entities.

3.2	Application instance: An occurrence of an application

3.3	Application manager [J.200]: The entity that is responsible for managing the lifecycle of the applications. It manages applications running in both the Presentation engine and Execution engine if both are present.

3.4	Application Program Interface (API) [J.200]:Consists of software libraries that provide uniform access to system services.

3.5	BroadcastTV (BTV): Same as the traditional CATV, and the channel switch and channel selection are implemented through screen menu or the remote controller. [0058]

3.6	Character[J.200]: A specific "letter" or other identifiable symbol, e.g. "A".

3.7	Character encoding [J.200]: Mapping between an integer input value, and the textual character that is represented by this mapping, e.g. in ASCII value 65 (decimal) is character "A", or shift-JIS for Japanese characters.

3.8	Character set:  See character encoding.

3.9	Content [J.200]:A general term that refers to any of the following: application, application resource collection, or application resource.

3.10	Content protection: Act of preventing unauthorized use of any content.

3.11	Electronic Program Guide (EPG) [J.200]: A Program table that is presented electronically.

3.12	EPG provider [J.90]:The entity that collects, collates and assembles the elements of information that constitute the EPG database.

3.13	Electronic Service Guide (ESG): Description for available services. Through the information in the ESG, the user can select the services and items he/she is interested in and find stored items on the terminal. (ETSI, DVB-CBMS)

3.14	Execution Engine (EE) [J.200]: instructions and associated data and media content. An execution engine may be implemented with an operating system, computer language compilers, interpreters, and Application Interfaces (APIs), which a procedural application may use to present audiovisual content, interact with a user, or execute other tasks, which are not evident to the user. A common example of an execution engine is the JavaTV software environment, using the Java programming language and byte code interpreter, JavaTV APIs, and a Java Virtual Machine for program execution.

3.15	Extended Channel: Means to organise different operation content together, including live channel, TVOD, information service, EPG and so on. For each extended channel, there is a related channel number. [0062]

3.16	Globally Executable MHP: A terminal specification based on MHP that enables applications to interoperate across OCAP, MHP and other GEM based platforms. (CableLabs, OC-SP-OCAP1.0-I16-050803, Section 3.2.1)

3.17	Home Network (HN) [J.190]:Short-range communications system designed for the residential environment, in which two or more devices exchange information under some sort of standard control.

3.18	Interactive Television (iTV): A service in which the user can send requests, within a browser environment, to the service provider in order to obtain additional information. This capability requires channels to have what is known as a 'return path'.

3.19	Interactive Program Guide (IPG)

- The IPG is a service facilitated by the middleware, which provides the consumer detailed information about the content available to them. (AITS, ATIS-0800002)

- Ability of the ITF(Terminal Function) to display and allow user selection of program, content, and service descriptions (AITS, ATIS-0800002)

3.20	IPTV Terminal Function (ITF): Functionality within the consumer network that is responsible for terminating the IP signal, and converting the content into a renderable format -- e.g., a STB. (AITS, ATIS-0800002)

3.21	Lifetime/Lifecycle of an application: Characterizes the time from which the application is loaded to the time the application is destroyed. (CableLabs, OC-SP-OCAP1.0-I16-050803, Section 3.2.1)

3.22	Metadata [J.98]:Metadata is descriptive data associated with a content asset package or file. It may vary in depth from merely identifying the content package title or information to populate an EPG to providing a complete index of different scenes in a movie or providing business rules detailing how the content package may be displayed, copied, or sold. Separate uses for metadata have originated from the studios, distribution networks (Cable, Satellite), down to the CPE (STBs, PVRs).

3.23	Metadata format [H.222AMD1]: Identifies the coding format of metadata.

3.24	Metadata service [H.222AMD1]: Coherent set of metadata of the same format delivered to a receiver for a specific purpose.

3.25	Middleware: A layer of software between applications and resources, which consists of a set of service enablers that allow multiple functionalities running on one or more devices in an IPTV system to interact across a network.

3.26	Multimedia [J.148]: The combination of multiple forms of media such as audio, video, text, graphics, fax, and telephony in the communication of information.

3.27	Multimedia application [T.174]: An application which involves the presentation of multimedia information to the user.

3.28	Multimedia service [Q.1702]: A service in which the interchanged information consists of more than one type, such as text, graphics, sound, image and video.

3.29	Multimedia representation [T.171]: The property of handling several types of representation media.

3.30	Navigation: The process of selection and movement among analogue and digital services offered on the cable network. The service information tables defined in this protocol assist in the navigation process by providing physical service locations, channel names and numbers for user reference. Those tables supporting electronic program guides also assist the navigation process. (ITU-T, J.94 Amd. 1 (00), B.3.2.3)

3.31	Presentation: The operation of rendering the content of a document in a form perceptible to a human being. (ITU-T, T.411 (93), 3.158)

3.32	Presentation engine [J.200]: A subsystem in a receiver that evaluates and presents declarative applications consisting of content, such as audio, video, graphics, and text primarily based on presentation rules defined in the presentation engine. A presentation engine also responds to formatting information, or "markup", associated with the content, to user inputs, and to script statements, which control presentation behaviour and initiate other processes in response to user input and other events. A common example of a presentation engine is an HTML browser, capable of displaying text and graphic content formatted in HTML, with interactive behaviour programmed in ECMA script.

3.33	Service [Y.101]: A structure set of capabilities intended to support applications.

3.34	Set-top box (STB): A device that contains demodulator, de-multiplexer, decoder, other functionalities and interfaces related to signal reception and presentation of the distributed programme at the subscriber's site.

3.35	Stream [J.200]: A unidirectional continuous flow of content.

3.36	Telecommunication service [F.700]: Set of telecommunication capabilities that works in a complementary and cooperative way in order to let users perform applications.

3.37	Time-shifting (Digital Recording / Playback): technology allows an audio/video stream to be digitally recorded and replayed at a later time (for example, PVR).. (CableLabs, OC-SP-OCAP1.0-I16-050803, Section 1.2.2.2.2.5, pp. 3)

3.38	Time-shifted TV: Time-shifted TV service breaks the user’s conventional habit that the user can watch TV program only according to fix timetable so that the user can discretionarily control his time to watch TV. The implementation of Time-shifted TV is to store the TV program into the network system while broadcasting TV program, the TV program will be stored according to prearranged time and displayed on the user terminal. PVR service includes the following services: Networked Personal Video recorder(nPVR),Client Personal Video Recorder(cPVR), Time shift TV and TV of Yesterday. [0062]

3.39	User Device:  Also known as Home Network End-Devices (HNED), Home Network Device (HND), Consumer Equipment (CE), terminal, and physical device. A piece of HW equipment running its SW and attached to a Home Network and being identified by a GUID – e.g., a MAC address. A single Device can be used by one or more Users. (AITS, ATIS-0800002)

3.40	User Interface: Also known as UI, a user interface is the sensory and behavioural aspects of a program that are presented to a user. The term is generally used to denote the menuing and navigational constructs of a program. (CableLabs, OC-SP-OCAP1.0-I16-050803, Section 3.2.1)

3.41	Video-on-Demand (VoD): A service in which the subscriber can view video content whenever desired.  The operating assumption is that the content is stored on the provider’s VoD server. Subscriber accesses the movie from a library directory which may include search engine that accesses movie description and rating.  Subscribers typically have the ability to pause, play, rewind, fast forward the content, or even stop viewing it and return to it at a later time when using this service.

# Abbreviations

API	Application Programming Interface

AV	Audio Video

DVB	Digital Video Broadcasting

EPG	Electronic Program Guide

ESG	Electronic Service Guide

ETSI	European Telecommunications Standards Institute

GEM	Globally Executable MHP

GIF	Graphics Interchange Format

GUI	Graphical User Interface

HN	Home Network

HTML	Hyper Text Mark-up Language

HTTP	Hyper Text Transport Protocol

I/O	Input / Output

ID	IDentifier

IETF	Internet Engineering Task Force

IHDN	In Home Digital Network

IP	Internet Protocol

IPR	Intellectual Property Rights

ISDN	Integrated Services Digital Network

ISO	International Organization for Standardization

ITU	International Telecommunication Union

ITV	Interactive Television

JDK	Java Development Kit

JFIF	JPEG File Interchange Format

JPEG	Joint Picture Expert Group

LMDS	Local Multipoint Distribution System

MAC	Media Access Control

MHP	Multimedia Home Platform

MIME	multipurpose internet mail extensions

MMDS	Multipoint Microwave Distribution System

MPEG	Moving Picture Experts Group

NTSC	National Television Systems Committee

NVOD	Near-Video-On-Demand

OCAP	OpenCable Application Platform

OS	Operating System

OSD	On Screen Display

PFR	Portable Font Resource

PNG	Portable Network Graphics

PSI	Program Specific Information

PSIP	Program and System Information Protocol

RAM	Random Access Memory

ROM	Read Only Memory

SI	Service Information

TCP	Transmission Control Protocol

TS	Transport Stream

UDP	User Datagram Protocol

UI	User Interface

URL	Uniform Resource Locator

UTF8	Universal Transformation Format 8

VM	Virtual Machine

WAN	Wide Area Network

XML	eXtensible Markup Language

# Conventions

TBD

[Note: The text of the following sections is provisional, and needs to be modified, filled out, and rearranged.]

<<BEGIN>>

- Content Provisioning

  - Introduction

  - General functional architecture

  - Functional modules

  - Interior interfaces

  - Exterior interfaces

1. Introduction

Content provision in IPTV provides licensed TV programs, movies, music and other contents that would be formatted to transport in IPTV network. For protecting right of content owner, the contents should be encrypted in this domain. There are content-oriented operation and control in this domain which would be very different from that in traditional radio and television. For example, content provision should format the metadata according to requirements from service providers and bargain and balance with service providers by content provision strategy.

For fulfilling above function, a functional architecture for content provision is proposed and shown as following figure. The interfaces between content provision and other IPTV component are also identified in the figure.

Figure1 functional architecture for content provision

As shown in figure1, Content Provision should input raw contents and metadata from content owner. Here “content owner” is the entity holding or being licensed raw contents, which would include audiovisual contents such as TV channels, movies, advertisement and relevant metadata. One physical entity might provide audiovisual contents and metadata together. However it exists also pure metadata providers.

Content Provision Layer consists of 3 functional modules, namely Content Aggregating, Content Right Management and Content Control.

- Content Aggregating:

  - It aggregates contents such as TV channels, movies, metadata etc from content owner.

  - It may function as content and metadata ratifying, which means that it needs to ratify content license, content access prioritization and parental control level. However it is either not necessary if the content owner is dependable and licensed.

- Content Adaption:

  - This module should do content and metadata adapting, that means that before going to the IPTV services network, the content should be processed for meeting the service requirements. For example, it may need to encode the content from raw format into more efficient format, such as mpeg4/avc for saving bandwidth, to extract content index or labels for future content searching and to re-edit the metadata for different application scenarios.

- Content Right Management:

  - This module functions for content right protection according to requirements of content owners. For example, it should produce and manage the content encryption key, produce and delivery content right objects according to content consuming requirement and so on.

- Content Provision Control:

  - It is core module of content provision and responsible for any functions necessary for content-oriented business and control.

  - It should function as content catalog, quality monitoring and control, storage and searching etc.

  - It should function as content business, such as content-oriented billing, content oriented charging and introducing third-party content provider.

  - Another important function is content and metadata verification. To insure the quality of content source, it must do the verification for the content qualify, conjunction between video content and metadata and so on.

  - It shall function to issue live/off-lined contents to IPTV services control and indicate where the services provider would go and find the formatted contents.

It is also found that there should be some interfaces existing between content provision and other sub-systems. They are named according to the general architecture specified in the T05-FG.IPTV-DOC-0056.

The interfaces are:

- Interface A: Content Stream

- Interface B:

  - B1: Content-oriented  billing information, content verification, Metadata information

  - B2: interaction between Digital Rights Management located in Content Provision and DRM proxy located in IPTV Service Control.

- Interface C: Interaction between Digital Rights Management and DRM client

- Interface Mcp: interaction for content provision system management

The interface between content provision and content owner has beyond the IPTV architecture scope.

It is also needed to point out that IPTV system should allow connecting to 3rd applications servers. So there 2 3rd application blocks circled by broken line in figure1 and also a 3rd applications client in IPTV end system. The location and relationship between 3rd application and IPTV system is for information while talking about IPTV architecture.

TBD

- Middleware

7.1 Definition of IPTV Middleware

7.2 Scope of IPTV Middleware

7.3 IPTV Middleware Architecture

    - Generic Architecture

    - Middleware Architecture of IPTV Terminal

7.4 Functionality and Classification of IPTV Components/APIs

7.4.1 Classification and Definition of IPTV Middleware Components

7.4.2 Functionality of IPTV Terminal APIs

TBD

- Metadata [Note: This section needs to be modified, and expanded]

  - Introduction

  - Metadata representation

  - Metadata delivery

  - Metadata security

  - Service and content metadata

  - Content delivery metadata

  - Usage restrictions and licensing metadata

  - Content security metadata

  - User metadata (usage environment, ITF capabilities, user profile, user preference)

TBD

- Retransmission and Legacy Related Issues

IPTV service can be combined with other platform’s services such as terrestrial (or cable, satellite) A/V channel and interactive data service. Sometimes it is necessary to re-transmit other platform’s services for the transparent service without contents processing. The integrated interactive service system may provide customer with easy interface about several platform’s services and allow service provider to make flexible, cost-effective, and efficient system by defining schema which is presented regardless of real time broadcasting and additional valued data service.

- Accessibility

TBD

- Browser-Based System

TBD

<<END>>

ANNEX: Requirements from Mountain View

- EPG may have subscription/notification or polling methods to periodically update contents list in pull mode. (e.g. RSS registration/feed) (User-customized contents list)

- EPG may allow user to store contents list metadata. (User-customized contents list)

- EPG may be able to transform a one metadata format into some standardized format in order to support user-customized contents list metadata. (e.g., XSLT) (User-customized contents list)

- EPG may allow for fast transition of menu/item toggling. (Navigation efficiency)

- EPG may have shortcut mechanism on menu. (Navigation efficiency)

- EPG may provide a way to set the number of items presented on screen at once. (Appearance)

- EPG may allow user to move or rearrange the screen layout for personalized EPG (e.g., AJAX, DOM) (Appearance)

  - Middleware should provide service enabling API.

  - Middleware API should be open, flexible, granular, self-contained and components-based.

  - Middleware components should facilitate service processes interaction among one or more IPTV devices (e.g. a server and its clients, such as STB, security system, or VOD server) in an IPTV system.

  - Middleware should provide API to support a variety of mixed media format to be presented in a single window, e.g an HTML txt page embedding a video stream.

  - Middleware should provide API to support managed picture in picture function.

  - Middleware should provide API to change the style of EPG.

## Requirements for PVRs and trick mode

When using pause and replay, IPTV terminals should also replay the associated accessibility features.

After IPTV_ESI_028 INSERT:

When content is recorded, all accessibility features shall be recordable with the main content, whilst retaining the synchronisation of the original. [ref accessibility features]

The IPTV terminal should allow the user to select which (if any) accessibility features are recorded.

Accessibility features (captions, subtitles, descriptive audio and multiple video streams such as for sign-language) shall be available and synchronised with the main content when viewing in normal playback.

Accessibility features may be rendered with the main content when viewing in high-speed playback.

When paused, captions, subtitles and multiple video streams shall be synchronised.