| The following information has been agreed by WG2 to be under further consideration by WG2 prior to inclusion in formal output documents. INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-DOC-0139 |  |
|  |  |  | English only |  |
| WG(s): 2 |  | 5th FG IPTV meeting: Geneva, 23 – 31 July 2007 |  |  |
| OUTPUT DOCUMENT |  |  |  |  |
| Source: | Editor |  |  |  |
| Title: | Living List: Performance monitoring for IPTV |  |  |  |

### Item 1 (Busan Meeting)

Source: FG IPTV-C-0153, ZTE, “A Proposal for performance monitoring content”

2.2 A method of network status monitoring

The network status can be divided into three levels as UNLOADED, LOADED or (and) CONGESTED. The receiver sends receiver reports to the sender periodically. The receiver report includes such information as loss packet rate, delay and jitter, etc. By processing loss packet rate, we can estimate which level current network status belongs to.

The method is shown in Figure 1, which includes the following steps.

The sender receives receiver report including packet loss rate

Figure 1 Network state estimation flowchart

Streaming server estimates network status

Filter handles packet loss rate

packet loss rate

The sender sends multimedia data

- Step 1: The sender sends multimedia data at normal rate.

- Step 2: The sender (or the reference point) receives receiver report including loss packet rate from the client (or the sampling point) periodically.

- Step 3: The sender (or the reference point) processes loss packet rate to estimate current network status.

The influence of abrupt change of network bandwidth (ACNB) should be considered. If packet loss is extraordinarily high, we thought that ACNB occurs. Two main reasons may lead to ACNB. One is transient network jitter and the other is network deterioration. ACNB from network jitter only lasts a short time, thus the reference point should ignore the impact.  Therefore, the low-pass filter is used as shown in Figure 2

Unloaded

Loaded

Congested

Loss(%)

low-pass filter

packet loss

Figure 2 Classification of Network State

- Step 4: According to the value of  , we can estimate current network status.

3. Proposal

This proposal gives the catalogue of performance monitoring content and presents a method to estimate network status by loss packet rate from the point of application .We hope that they can be included in the standard document or become a part of document as reference information.

4. References

Schulzrinne, H., Casner, S., Frederick, R., and V. Jacobson, "RTP: A Transport Protocol for Real-Time Applications", RFC 1189, GMD Fokus, Precept Software, Inc., Xerox Palo Alto Research Center, Lawrence Berkeley National Laboratory, January 1996.

- _____________

Item 2 (MountainView Meeting)

Source IPTV-C-268, UTStarcom, “Performance statistics for Content Distribution”

Item 3 (MountainView Meeting)

Source IPTV-C-362, ZTE,  “A method of indexing multi-media packet data for multi-media quality monitoring”

Item 4  (MountainView Meeting)

Source IPTV-C-411, Telchemy, “IPTV QoS/QoE Metrics”

Item 5: (MountainView Meeting)

Source IPTV-C-413, Telechemy, “Proposed QoE algorithm based on PSNR Estimation”

Item 6  (MountainView Meeting)

Source IPTV-IL-0040, ATIS IIF,  ATIS 800004

Item 7 (Bled, Slovenia Meeting)

Source FG IPTV-0493, Huawei Technologies, “A proposal on overlay measurement”

Item 8 (Geneva Meeting)

Source FG IPTV-682 (Pixelmetrix) “Suggested additions to FG-IPTV-DOC-0089 “Performance monitoring for IPTV”

Suggested additions to FG-IPTV-DOC-0089 “Performance monitoring for IPTV”

[Editor’s Note: Only Proposal 1 from Contribution 682 to be included in the living list]

Item 9 (Geneva Meeting)

Source FG IPTV-IL-88 (ATIS IIF) – “ATIS IIF Liaison to ITU-T FG IPTV on "QoS Metrics for Linear Broadcast IPTV" (IIF-WT-014R9)”

_______________