| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-DOC-0163 |  |
|  |  |  | English only |  |
| WG(s): 6 |  | 6th FG IPTV meeting: Tokyo, Japan, 15-19 October 2007 |  |  |
| OUTPUT DOCUMENT |  |  |  |  |
| Source: | Editors |  |  |  |
| Title: | Working Document: IPTV Middleware |  |  |  |

Table of Content

1.	Scope	3

2.	References	3

3.	Definitions	3

4.	Abbreviations and acronyms	4

5	IPTV Middleware	4

6	Terminal Middleware Architecture	8

6.1	APIs definitions, descriptions and classification	10

6.2	Definitions and descriptions of IPTV Middleware Components APIs	11

6.2.1	System and Resource Management Component	11

6.2.2	Media Management Component	12

6.2.3	Communication Component	12

6.2.4	Security Component	13

6.2.5	Metadata Access Component	13

6.2.6	User Interaction Component	13

6.3	Classification of IPTV Middleware Components APIs	14

7	Service Platform Middleware	14

IPTV Middleware

# Scope

This document describes IPTV middleware architecture and application programming interfaces (APIs).

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this working document. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this working document are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this working document does not give it, as a stand-alone document, the status of a Recommendation

[ITU-T  J.200]	: ITU-T Recommendation J.200 (2001), Worldwide common core - Application environment for digital interactive television services

# Definitions

3.1	This working document uses the following terms defined elsewhere:

TBD

3.2	This working document defines the following terms:

3.2.1	Resource Abstraction Interface (Layer): an interface layer between resource and middleware service which encapsulates OS operations and resources layer and provide abstract view for resource layer.

3.2.2	Resources (Layer): the native hardware and software resources that provide middleware services component of certain functionality through resource abstraction layer, and middleware service functions are provided to application layer through the API of the middleware services.

3.2.3	Service Components (Layer): the pure native components that offer functionalities to application layers regardless of type of software and hardware in the resource layer.

# Abbreviations and acronyms

This working document uses the following abbreviations and acronyms.

API	Application Programming Interfaces

BOSS	Back Office Support Systems

CDN	Content Delivery Network

DRM	Digital Rights Management

GUI	Graphical User Interface

HTML	Hypertext Markup Language

HTTP	Hypertext Transfer Protocol

P2P	Peer-to-Peer

RAL	Resource Abstraction Layer

RAM	Random Access Memory

RTSP	Real Time Streaming Protocol

VoD	Video on Demand

# 5	IPTV Middleware

The IPTV Middleware supports the variety of functionalities (e.g. EPG, PVR, gaming, etc.) provided by the IPTV Architecture to the Terminal Devices.

Terminal Middleware

Bridge

Network Resource Abstract Layer (RAL)

Application Mgt

Clients of Platform

Middleware

API

IPTV Application

Network Transport

(e.g. FTTH, ATM, 3G)

Home Network

Service Platform Middleware

Application

Management

Functions

Resource

Management

Function

Content Provider

Function

(DRM…)

Content Delivery

Functions

(Stream…)

Service Control

Functions

(EPG…)

Management

Function

(SMS…)

End-user Functions

(Codec…)

Resource Mgt

Terminal RAL

Figure 1: IPTV middleware architecture

The figure above is an overview of the IPTV middleware architecture.

- IPTV Applications Layer

Application layer is the layer where operators and 3rd parties provide services and applications. These services and applications includes EPG application, VOD, Live TV, PVR, games, Internet applications as well as other value-added services, and etc.

- API Layer

A set of interface for service provider or manufacturer to build specific applications and be presented in granular based for variety of utilized purpose.

- Service API – a service enabling typed API such as STB authentication or get subscriber balance

- Function based API – for some specific feature like EPG page insert/delete

- Programming API – especially for terminal device development kit (SDK). For example, to allocate memory buffer.

- Support varies OS (Windows, WindowsCE, or Linux flavor OS), common protocol (IP, SOAP/XML) and programming interface (C/C++, Java, PHP)

- IPTV Middleware:

The IPTV Middleware is divided in a Service Platform Middleware and a Terminal Middleware; the both are linked through a Bridge.

Service Platform Middleware

The Service Platform Middleware invokes the lower layer network interfaces to control service network resources, and provide application functions (APIs) for upper layer.

It is recommended the Service Platform Middleware uses the functional modules available in the IPTV infrastructure as described in [REF WG1 output doc, currently DOC-115] (Content Delivery Functions, Service Control Functions, Management Functions, Content Provider Functions). In additions to these, the Service Platform Middleware also provides:

- Resource Management Function, a functional module to manage system resource in IPTV terminals and servers.

- Application Management Function, a functional module to manage the life cycle of the application and interaction operation between applications.

Terminal Middleware

It is recommended the Terminal Middleware uses the End User Functions as defined in [REF WG1 output doc, currently DOC-115]. In addition to these, the Terminal Middleware also provides:

- Resource Management Function, a functional module to manage system resource in IPTV terminals and servers.

- Application Management Function, a functional module to manage the life cycle of the application and interaction operation between applications.

- Optionally, a client of Service Platform Middleware.

- Optionally, a Multimedia Application Platform [Ref WG6 document IPTV multimedia application platforms, currently DOC-146].

- Optionally a Presentation Engine.

- Resource Abstract Layer

Editor Note: this section should be discussed with WG5

Resource Abstraction Layer (RAL) is to make middleware be able to be executed on selective software and hardware. The RAL includes:

- Software Resource, such as divers, OS from Windows to Linux flavor, and etc.

- Hardware Resource, computing devices (CPU), storage devices, firmware (codec), rendering devices (display, speaker), IO devices.

The resource abstract in an IPTV terminal can be removable storage, decoder, remote controller, and etc.

Network Resource Abstract Layer is mainly to shield all kind of network resources (including network protocol) in the lower layer, and provides the transparent interfaces for upper layer.

- Network Transport Functions Layer

Network Transport Functions is used to support the interaction and communication middleware service component and applications. Network Transport Functions are based on IP protocols.

# 6	Terminal Middleware Architecture

The following diagram is view of IPTV terminal middleware architecture (this architecture is compatible with the architecture defined in ITU-T J.200 figure 1: Application environment system architecture). This architecture is technology agnostic and is suitable to any kind of implementation.

Figure 2: IPTV terminal middleware architecture – overview

Resource Abstraction Layer (RAL)

The IPTV terminal middleware is hardware-agnostic. A specific Resource Abstraction Layer (RAL) exist for a specific hardware/OS – this provides the necessary interface to the lower layers (RAM, network access, hard drive, USB-port, etc.). The RAL interface is designed so that device drivers can be written irrespective of middleware software in the service logic adaptation layer.

Service logic adaptation

The service logic adaptation layer is made of Service Components. The Service Components are pure native components that offer functionalities common to all middleware implementations (e.g service selection & presentation, Service Information management, PVR, security system). They are used and enriched by the Application Services in order to simplify the development of service components and applications above it.

The definition and the scope of services depend on the concrete functionalities deployed in the IPTV system.

Nevertheless, some Service Components can be defined that are generic in nature:

- System and Resource Management Component is used to manage IPTV terminal device resources, and provide Software Initialization API, Upgrade and Download API, System Resources Management API, and Terminal Management API to the upper application layer.

- Media Management Component: Responsible for managing media streaming, media presentation and media storage, and provide Media Service API to the up layer application.

- Communication Component is responsible for IPTV terminal device communications, which including the following functionalities,:

  - Communication management

  - Modem management

  - Protocol management (HTTP, RTSP, etc.)

  - Internet Access

- Security Component is responsible for security mechanism of whole system, including subscriber authentication, media authorization, network security, software upgrade security, and service application security. This Middleware Service Component provides Security and Authentication API and DRM API to the upper application layer.

- Metadata Access Component is responsible for metadata system access, the metadata presentation and service selection management, and provides Metadata Access API to the upper application layer.

- User Interaction Component is responsible for interacting with end-users, and dispatch events originated by end-user to application layer User Interaction API to the upper application layer.

All this components can optionally use the functions available on the network.

Presentation engines

The Presentation Engines comprises of various engines along with a set of high-level services. This layer is built on top of the Service Components layer. In some implementation this layer may not exist.

User experience and applications

The Applications are either downloaded or resident in nature. In particular, an Application is powered by a Presentation Engine (e.g. HTML Browser). An Application can either have full or restricted access to the presentation engine features. Moreover some Applications may directly access the Service Components without using a Presentation Engine.

The Application Manager is responsible for managing the life cycle of the application and interaction operation between applications. The Application Manager is part of the Middleware but should be directly addressed by the Applications except for registration purpose.

## 6.1	APIs definitions, descriptions and classification

This chapter presents a classification of the components making this API.

## 6.2	Definitions and descriptions of IPTV Middleware Components APIs

### 6.2.1	System and Resource Management Component

Software Initialize and Diagnosis API: an IPTV Terminal Middleware API that is responsible to initial IPTV terminal device, prepare running environment and provide diagnostic capabilities to IPTV terminal device.

- Configure IPTV terminal device;

- Task initialization, such as application management, DRM task, subscriber authentication process, etc;

- Network connection initialization, such as, to get network parameter, configure access mode, and etc;

- Resource initialization, such as memory, timer, I/O equipment, A/V decoder and other system resource;

- Coordinate with other IPTV terminal APIs, such as, to work with Security and Authentication API to start user authentication procedure, to work together with Upgrade and Download API to check software version and perform software upgrade procedure;

- Diagnosis for IPTV terminal device.

Upgrade and Download API: an IPTV Terminal Middleware API that is responsible for dynamic downloading or upgrading software for IPTV terminal device.

- Software upgrade and download for application layer

- Software upgrade and download for middleware service

- Software upgrade and download for resource layer

- Coordination with Security and Authentication API to check validity of software and data

Terminal Management API: an IPTV Terminal Middleware API that is responsible to provides IPTV terminal management and configuration function.

- Remote management

- Log management

- Software version management

- Server parameter configuration, such as server address configuration

- Access mode and parameter configuration

- Media device parameter configuration, such decoder

- Subscriber configuration, such access account, service account

System Resource Management API: an IPTV Terminal Middleware API that is responsible to schedule resource used by application and  manage  their status.

- Status registration of resource usage by application

- Allocation and scheduling of resource to application

- Resource release management

Removable Storage API: an IPTV Terminal Middleware API that is responsible to access information available in a removable storage device (e.g. smartcard, USB key)

- Access User Information, such as user authentication, user ID, network access authority

- Access service information, such as service profile authorized by service providers

- Access billing information, such as prepaid account, fees consumed

- Access user preferences such as accessibility features and display settings

### 6.2.2	Media Management Component

Media Service API: an IPTV Terminal Middleware API that is responsible to manage media streaming, media presentation and media storage.

- Media streaming management, such as media streaming session set up and control for VoD, multicast linear TV, unicast linear TV, and time shift.

- Media decoding management

- Media rendering and trick mode management, such as play, stop, pause, and resume

- Caption presentation

- Trigger of DRM process

- Media buffer management

- PVR and storage management

### 6.2.3	Communication Component

Internet Access API: an IPTV Terminal Middleware API that is responsible to control and manage the basic operations of Open Internet resident clients.

- Internet client management, such as Web browser, e-mail and news client

- Internet client control

### 6.2.4	Security Component

Security and Authentication API: an IPTV Terminal Middleware API that is responsible for security mechanism of whole system, including subscriber authentication, network security, software upgrade, and service application security, etc.

- Subscriber authentication and identification information management

- Service application authorization

- Software upgrade and download authentication

- Network security policy management

- Key, token and registration information management

- Parental control related information management

- Content purchasing information management

DRM API: an IPTV Terminal Middleware API that is responsible to process right control messages, process right management messages, and enable upper application layer to access the right DRM system.

- License management

- Right management

- Key management

- Decryption of media stream and data stream

### 6.2.5	Metadata Access Component

Metadata API: an IPTV Terminal Middleware API that is responsible for metadata system access, the metadata presentation and service selection management.

- Metadata system access

- Metadata delivery management

- Metadata presentation, such as program scheduler and event catalogue

- Service selection management

- Metadata compression & decompression

### 6.2.6	User Interaction Component

End User Interaction API: an IPTV Terminal Middleware API that is responsible to interact with end users, and dispatch events originated by end user  to application layer.

- Interaction with end user, via keyboard, or mouse, or remote controller

- Receiving and dispatching event from end user to application layer

- Service provider’s terms and conditions notification to end user

- Display of service provider logo

GUI API: an IPTV Terminal Middleware API that is responsible to provide GUI engine to upper application to implement graph design, display and control function.

- Graph design and display function;

- UI control interface design and display function;

- Set image and video attribute;

## 6.3	Classification of IPTV Middleware Components APIs

The following table provides the list of recommended and optional APIs.

Table 1: IPTV Middleware Components API classification

| API | Recommended | Optional |
| --- | --- | --- |
| Software Initialize and Diagnosis | X |  |
| Upgrade and Download | X |  |
| Terminal management |  | X |
| System resource management |  | X |
| Removable storage |  | X |
| Media Service | X |  |
| Internet Access |  | X |
| Security and Authentication | X |  |
| DRM | X |  |
| Metadata | X |  |
| End User Interaction | X |  |
| GUI |  | X |

# 7	Service Platform Middleware

Editor Note: need contributions

_____________________________