| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 2 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  | TD 3 (WP 2/2) |
|  |  | English only Original: English |
| Question(s): | 6/2 | Geneva, 15-24 February 2005 |
| TEMPORARY DOCUMENT |  |  |
| Source: | TSB (Originally Rapporteur for former Q.7/2) |  |
| Title: | Draft New Recommendation E.772, version 0.0.2, May 2002 – Input to Rapporteur Group |  |

This document was originally published as TD 5 (WP3/2) in the May 2002 meeting

E.772 - Network grade of service parameters and target values for packet-switched public land mobile services

Draft Recommendation E.772 belongs to a set of Recommendations in the E.750 Series to be finalized with priority in ITU-T 2001-2004 Study Period.

This document contains a possible structure for E.772. Where appropriate, this Recommendation re-visits in the packet-switching context, definitions and target values for GOS parameters introduced in E.771, the companion recommendation on GOS parameters for circuit-switched land mobile services.

Draft E.772 considers the continued penetration of mobile services, the technological advances for 2G systems and IP networks, the standardization work on 3G systems carried out in ITU and in other standardization bodies, and the foreseen coexistence of 2G and 3G systems over a significant period of time.

TRAFFIC ENGINEERING FOR PERSONAL COMMUNICATIONS

Draft New Recommendation E.772

Network grade of service parameters and target values for packet-switched public land mobile services

Version 0.0.2

Summary

This Recommendation applies to terrestrial based land mobile systems. Focused on packet-switched services, the goal of the Recommendation is to provide targets for GOS parameters to be used in dimensioning the radio transmission resources, the fixed infrastructure of the radio access network, and the core network hosting the switching and mobility support functionality. Recognizing that second and third generation systems will coexist over a significant period of time, the scope of this Recommendation covers teletraffic aspects related to both system generations.

Source

<text to be supplied by TSB>

Foreword

<text to be supplied by TSB>

Table of Contents

1	Scope	5

2	Related Recommendations	6

3	Terms and definitions	6

4	Abbreviations	7

5	Grade of service parameters	8

5.1	Call set-up	9

5.1.1	Post-selection delay	9

5.1.1.1	Authentication and privacy	10

5.1.1.2	Paging/alerting of a terminal/user	10

5.1.1.3	Routing number transfer	10

5.1.2	Answer signal delay	11

5.1.3	Probability of end-to-end blocking	11

5.2	Information transfer	11

5.2.1	Probability of unsuccessful land cellular handover	11

5.2.1.1	Definition of probability of unsuccessful land cellular handover	12

5.3	Call clear down	13

5.3.1	Call release delay	13

6	Target values for GOS parameters	13

6.1	Call set-up	13

6.1.1	Post-selection delay	13

6.1.1.1	Post-selection delay in the fixed network	13

6.1.1.2	Delay in authentication and cipher mode setting	13

6.1.1.3	Paging/alerting delay	13

6.1.1.4	Delay in obtaining routing number	14

6.1.1.5	Target values for post-selection delays	14

6.1.2	Answer signal delay	16

6.1.3	 Probability of end-to-end blocking	16

6.1.3.1	Probability of blocking on the radio channels	17

6.1.3.2	Target values for blocking on the radio channels	17

6.1.3.3	Probability of blocking on PLMN-to-fixed network circuits	18

6.1.3.4	Target values for blocking on PLMN-to-fixed network circuits	18

6.2	 Information transfer	20

6.2.1	Probability of unsuccessful land cellular handover	20

6.2.1.1	Target values for probability of unsuccessful handover handling	20

6.3	 Call clear down	21

6.3.1	Call release delay	21

6.3.1.1	Target values for call release delay	21

7	History	22

Bibliography	22

ANNEX A	23

A.1	Scope of traffic engineering for 3G systems	23

ANNEX B	24

B.1	Post-selection delay	24

C.1	Network elements and signaling messages	30

TRAFFIC ENGINEERING FOR PERSONAL COMMUNICATIONS

Draft New Recommendation E.772

Network grade of service parameters and target values for packet-switched public land mobile services

(Version 0.0.2, reference Version 0.0.1 TD 3/2-007)

# 1	Scope

This Recommendation proposes network Grade of Service (GOS) parameters for current and evolving land mobile services, i. e. those supported by evolving second generation systems and the implementation of the members in the IMT-2000 family. These parameters are defined, and their target values specified, assuming that the network and the network components are operating in their normal mode (i.e. are fully operational). Further, the parameters and their target values assume normal (as opposed to distress or emergency) traffic.

In general, the target values specified for evolved second generation systems should be considered as minimum requirements to third generation systems (where applicable), i.e. the target GOS for third generation systems should be equal to, if not more stringent than, the targets specified in this Recommendation for evolved second generation systems.

The scope of this Recommendation is delimited with reference to the following dimensions: addressed plane, connection phase, spanned network segment, medium individuality in multimedia connections, and interoperation between evolved second generation systems and IMT-2000. In particular, this Recommendation

- is concerned with both the user and the control plane;

- introduces GOS parameters classified according to whether they relate to the call set-up, the information transfer, or the call clear down phase of a connection;

- proposes, where possible, separate GOS target values associated with the radio resources, the radio access network and the core network of mobile systems;

- leaves for further study the consideration of the individual mediums comprising a multimedia connection;

- leaves for further study the teletraffic aspects related to interoperation between evolved second generation systems and IMT-2000.

# 2	Related Recommendations

The following Recommendations are the ones applicable at the time of publication of this Recommendation.

| ITU-T E.720 | ISDN grade of service concept |
| --- | --- |
| ITU-T E.721 | Network grade of service parameters and target values for circuit-switched services in the evolving ISDN |
| ITU-T E.723 | Grade of service parameters for Signalling System No. 7 networks |
| ITU-T E.751 | Reference connections for traffic engineering of land mobile networks |
| ITU-T E.771 | Network grade of service parameters and target values for circuit-switched public land mobile services |
| ITU-T E.780 | Dimensioning methods for Public Land Mobile Systems |
| ITU-T E.800 | Terms and definitions related to quality of service and network performance including dependability |
| ITU-T I.352 | Network performance objectives for connection processing delays in an ISDN. |
| ITU-T Q.1001 | General aspects of public land mobile networks. |
| ITU-T Q.1002 | Network functions. |
| ITU-T Q.1003 | Location registration procedures. |
| ITU-T Q.1701 | Framework for IMT-2000 Networks |
| ITU-R M.817 | Future Public Land Mobile Telecommunication Systems (FPLMTS). |
| ITU-R M.1079 | Speech and voiceband data performance requirements for Future Public Land Mobile Telecommunication Systems (FPLMTS). |
| ITU-R M.1079-1 | Performance and Quality of Service Requirements for International Mobile Telecommunications-2000 (IMT-2000) |
| ITU-R M.1224 | Vocabulary of Terms for International Mobile Telecommunications-2000 (IMT-2000) |
| ITU-R M.1308 | Evolution of Land Mobile Systems Towards IMT-2000 |

# 3	Terms and definitions

This Recommendation defines the following terms.

| Administrative domain | <text to be provided> |
| --- | --- |
| Call | <text to be provided> |
| Connection | <text to be provided> |
| Interoperation | <text to be provided> |
| Interworking | <text to be provided> |
| Roaming | <text to be provided> |

# 4	Abbreviations

For the purposes of this Recommendation, the following abbreviations are used.

| AC | Authentication Centre |
| --- | --- |
| ACM | Address Complete Message |
| ANS | Answer Message |
| BER | Bit Error Rate |
| BS | Base Station |
| BSS | Base Station System |
| CCS | Common Channel Signalling |
| CDMA | Code Division Multiple Access |
| CSMA-CD | Carrier Sense Multiple Access/Collision Detection |
| DSS 1 | Digital Subscriber Signalling No. 1 |
| FDMA | Frequency Division Multiple Access |
| F-F | Fixed-to-Fixed |
| F-M | Fixed-to-Mobile |
| FPLMTS | Future Public Land Mobile Telecommunication Systems |
| GGSN | Gateway GPRS Support Node |
| GSM | Global System for Mobile communication |
| GOS | Grade of Service |
| HDLC | High-level Data Link Control |
| HLR | Home Location Register |
| IAM | Initial Address Message |
| IMT-2000 | International Mobile Telecommunications-2000 |
| IN | Intelligent Network |
| ISDN | Integrated Services Digital Network |
| ISUP | ISDN User Part |
| LE | Local Exchange |
| MMF | Mobility Management Functions |
| MS | Mobile Station |
| MSC | Mobile Switching Centre |
| M-F | Mobile-to-Fixed |
| M-M | Mobile-to-Mobile |
| PABX | Private Automatic Branch Exchange |
| PDC | Personal Digital Cellular |
| PLMN | Public Land Mobile Network |
| PSTN | Public Switched Telephone Network |
| RNC | Radio Network Controller |
| SGSN | Serving GPRS Support Node |
| SS No. 7 | Signalling System No. 7 |
| TDMA | Time Division Multiple Access |
| TE | Transit Exchange |
| UMTS | Universal Mobile Telecommunication System |
| UPT | Universal Personal Telecommunication |
| VLR | Visitor Location Register |

# 5	Grade of service parameters

In this Recommendation, the traffic GOS parameters are specified for mobile packet-switched services as indicated in Table 1/E.772. The table also indicates that, whenever applicable, separate GOS targets are considered for the following three call types

•	fixed network-to-mobile network calls (F-M);

•	mobile network-to-fixed network calls (M-F);

•	mobile network-to-mobile network calls utilizing the fixed network as transit network (M-M).

TABLE 0/E.772

Scope of E.772

[Editor Note: Table to be completed]

| Call phase | GOS parameter | Call type |  |  |
| --- | --- | --- | --- | --- |
|  |  | F-M | M-F | M-M |
| Call set-up |  |  |  |  |
|  | post-selection delay |  |  |  |
|  | answer signal delay |  |  |  |
|  | probability of end-to-end blocking; this probability includes the following three components: - probability of blocking on radio links; - probability of blocking on PLMN-to-fixed network circuits; - probability of blocking in the fixed (transit) network |  |  |  |
| Information transfer |  |  |  |  |
|  | probability of unsuccessful land cellular handover |  |  |  |
| Call clear down |  |  |  |  |
|  | call release delay |  |  |  |

Editor Note: Begin of text from E.771 for consideration and possible adaptation in New Draft E.772

Pre-selection delay for M-F and M-M calls is not included because mobile stations originate calls by simply keying in the called number and pressing the "send" key.

The definitions of these traffic GOS parameters are given below. The delay GOS parameters are based on the message flows in Recommendation Q.931 (DSS 1) and Signalling System No. 7 (ISUP) protocols as indicated, for example, in Figure A.1/E.713.

## 5.1	Call set-up

### 5.1.1	Post-selection delay

Post-selection delay (enbloc sending) is defined as the time interval from the instant the first bit of the initial SETUP message containing all the selection digits is passed by the calling terminal to the access signalling system until the last bit of the first message indicating call disposition is received by the calling terminal (ALERTING message in case of successful call).

NOTES

1	In the case of mobile-originated (i.e. M-F or M-M) connections, the starting instant is the activation of the "Send" key in the calling terminal.

2	In case of automatic answering terminals, the ALERTING message is replaced by the CONNECT message.

The post-selection delays comprises delays associated with operations such as authentication, paging/alerting and transfer of routing number.

#### 5.1.1.1	Authentication and privacy

Authentication of users and/or terminals in a public land mobile network is a key requirement to provide protection against unauthorized (fraudulent) access and misuse of information about subscribers, and to provide privacy during radio transmissions (protection against eavesdropping). In the second and third generation digital mobile system, the latter is achieved through encryption of voice, data and signalling information and requires exchange of encryption parameters (keys) during the authentication process.

The authentication procedure involves at least one set of challenge-response messages in order to exchange and compare appropriate authentication parameters and set up the encryption mode (privacy) over the radio path. It is assumed that the authentication procedure is invoked at call origination by a mobile station and will impact post-selection delays for M-F and M-M calls. For mobile terminated calls (F-M and M-M calls), the delay for paging/alerting includes the time required for authentication and cipher mode setting for the called mobile station (see  5.1.1.2).

#### 5.1.1.2	Paging/alerting of a terminal/user

Before an incoming call can be established to a mobile user, the exact location (the current serving base station system) for the mobile station needs to be identified. This is achieved through the "paging procedure" whereby the current serving mobile switching centre pages the mobile station with a paging broadcast to all base station systems within its domain. The paged terminal (if in "attached" state) automatically responds to the page and establishes its current location. User alerting at the mobile station (ringing) and alerting message to the calling user (ring back) is applied only after the terminal has answered the page.

Though no database access is associated with the paging procedure itself, a number of operations need to be completed before the "alert" procedure can be invoked for the called and calling parties [e.g. authentication and cipher mode setting which requires interaction with the local database of a visited network (VLR) and possibly with the user's home network database (HLR), sending of call set-up message to the mobile station].

#### 5.1.1.3	Routing number transfer

For every call terminating at a mobile station (from a fixed or mobile network), the home network database (HLR) of the called mobile user needs to be interrogated to obtain a "routing number". If the called mobile user is roaming in another mobile network (or the same network but within the domain of another HLR), the HLR in turn will interrogate the serving VLR for the called mobile station to obtain the routing number. Thus, in the worst case two database consultations will be required to obtain a routing number (Figure 3/E.751). The query/response delays involved in obtaining the routing number for the mobile station will contribute to the post-selection delays for F-M and M-M calls.

### 5.1.2	Answer signal delay

Answer signal delay is defined as the time interval from the instant that the called terminal passes the first bit of the CONNECT message to its access signalling system until the last bit of the CONNECT message is received by the calling terminal.

### 5.1.3	Probability of end-to-end blocking

The probability of end-to-end blocking is the probability that any call attempt will be unsuccessful due to a lack of network resources.

NOTE – The lack of control plane resources during the call set-up phase may also contribute to end-to-end blocking. This aspect is for further study.

Editor Note: End of text from E.771 for consideration and possible adaptation in New Draft E.772

Editor Note: Begin of text from E.771 requiring thorough revisitation for possible adaptation in New Draft E.772

## 5.2	Information transfer

### 5.2.1	Probability of unsuccessful land cellular handover

One of the requirements of cellular systems is the ability to hand over calls in progress as the mobile terminal/user moves between cells (inter-cell handover). When a call is in progress, an analogue of the transmission quality of the radio channel – e.g. radio channel power level, BER (Bit Error Rate), etc. – associated with a mobile is monitored by the active BS controller. If the transmission quality drops below a predetermined level, indicating that the mobile may be leaving the cell, an automatic sequence of operations is initiated to hand over the call to a new cell/channel combination. Cells are usually not regularly shaped in practice due to terrain and radio signal factors and their radio coverage must overlap to some extent. This overlap provides a window during which the handover should be completed without substantially affecting the quality of the connection in a predetermined time interval.

Handover requests may be issued for a variety of reasons, including load balancing, inter-operator arrangements, emergency calls, and requirements on minimum transmission quality. In the latter case, the technical provisions by which a handover is performed depend on the radio access technique, e.g. whether TDMA or CDMA based.

For certain types of multiple access and of channel allocation (e.g. TDMA combined with Dynamic Channel Allocation) handover between channels of the same cell may be required, i.e. intra-cell handover. For certain other types of multiple access schemes (e.g. CDMA) intra-cell handover is not called for; however, the signal strength tolerance and the timeliness of the inter-cell handover may be more demanding.

The course of actions following an unsuccessful handover attempt depend on system operation and user behaviour. Some systems are operated such that unsuccessful handover attempts are queued or reattempted after a specified time. If the conditions which led to issue a handover attempt persist (e.g. as measured through radio channel power level, BER, etc.), users experience continued deteriorated service quality. In some systems, when a handover request is queued (reattempted) for more than a certain delay time (number of times), the call is released. These limits take into consideration both efficient network resource usage and inconvenience to the user through exposure to the low quality of the radio channel. In some other systems, no call release is automatically initiated and eventually the users abandon the calls if inconvenience is excessive or quality perceived as not matching the subscribed/negotiated levels. The relationship between resource usage and user inconvenience is an important design issue which impacts traffic engineering. This relationship is for further study.

Another reason for a handover to fail is if excessive transmission errors are encountered during the handover signalling. However this is a radio system design problem and not a concern for traffic engineering.

Handovers which involve moving from one MSC to another MSC, possibly making use of the fixed network facilities, are for further study.

NOTE – The probability of an unsuccessful handover is a critical parameter in a cellular system, as an unsuccessful handover affects a call already in progress. Future cellular systems will involve cells of much smaller size than those in current systems and hence more frequent handovers are likely to occur. The frequency of handovers will depend on factors such as cell size, average call holding time, average speed of the mobile user (which could vary with the time of the day, the area of the city, etc.), and geographical distribution and mobility behaviour of mobiles within the cells. The GOS specifications for the handover parameters need to be quite stringent, and how well and easily these requirements are met by a cellular system will depend on the efficiency and sophistication of the handover algorithm(s) and the processing capacity.

#### 5.2.1.1	Definition of probability of unsuccessful land cellular handover

This parameter is the probability that a handover attempt fails because of lack of radio resources in the target cell, or because of a lack of free resources for establishing the new network connection. The failure condition is based either on a specified time interval since the handover request was first issued or on a threshold on signal strength.

Editor Note: End of text from E.771 requiring thorough revisitation for possible adaptation in New Draft E.772

Editor Note: Begin of text from E.771 for consideration and possible adaptation in New Draft E.772

## 5.3	Call clear down

### 5.3.1	Call release delay

Call release delay is defined as the time interval from the instant the DISCONNECT message is passed by the user terminal which terminated the call to the access signalling system, until the RELEASE message is received by the same terminal (indicating that the terminals can initiate/receive a new call).

# 6	Target values for GOS parameters

## 6.1	Call set-up

### 6.1.1	Post-selection delay

Depending on the call type (F-M, M-F or M-M), the post-selection delay will be an aggregation of the following components:

•	base post-selection delay in the fixed network;

•	delay for authentication and cipher mode setting;

•	delay for paging/alerting; and

•	delay in obtaining the routing number.

#### 6.1.1.1	Post-selection delay in the fixed network

Recommendation E.721 provides targets for average post-selection delays in the evolving ISDN/PSTN networks for normal traffic loads. The delays are specified for short (local), medium (toll) and long (international) connections. The specified values are 3.0, 5.0 and 8.0 seconds, respectively, for the above three connection lengths. These values can be used as the base contribution of the fixed transit network in a mobile originated and/or mobile terminated call.

#### 6.1.1.2	Delay in authentication and cipher mode setting

Depending on system design, the authentication procedure may involve a database access (for retrieving the authentication and privacy parameters) as well as additional messaging and real-time processing of authentication response and encryption keys. The delay contribution for this procedure should therefore include the target value assigned for average query-response delays (e.g. 1.5 seconds) and additional delay of 1.0 second to reflect the messaging and computation delays, with a total (average) authentication delay of 2.5 seconds.

#### 6.1.1.3	Paging/alerting delay

Besides the database access delay for authentication and cipher mode setting for the called mobile station, paging/alerting involves additional procedures. In some second generation systems (e.g. GSM) the target for maximum time to alert is 4 seconds for successful first page (assumed probability equal to 0.85) and 15 seconds for final attempt (assumed probability equal to 0.15). Further, in some systems, a timer of 3 to 6 seconds is applied for the first page so that subsequent pages cannot be activated before the expiration of this timer. An average delay of 4.0 seconds may be considered suitable for paging/alerting.

#### 6.1.1.4	Delay in obtaining routing number

For routing number transfer, one database lookup (interrogating the HLR) is always required, and a second database access (interrogating the VLR for roamers) is required if the called mobile station is a roamer. Assuming that the probability that the called mobile station is a roamer (e.g. probability equal to 0.33), an average delay of 2.0 seconds (e.g. 1.5 plus 0.5 seconds) for obtaining a routing number can be used as a suitable target.

#### 6.1.1.5	Target values for post-selection delays

Table 1 gives the target values for post-selection delays in second generation mobile systems and their immediate derivatives.

TABLE 1/E.772

Proposed (average) target values for post-selection delays

| Post-selection delay (secs) |  |  |  |
| --- | --- | --- | --- |
| Call type | F-M | M-F | M-M |
| Authentication/ ciphering | 0.0 | 2.5 | 2.5 |
| Paging/alerting | 4.0 | 0.0 | 4.0 |
| Routing number transfer | 2.0 | 0.0 | 2.0 |
| Post-selection delay (fixed network) |  |  |  |
| local connection | 3.0 | 3.0 | 3.0 |
| toll connection | 5.0 | 5.0 | 5.0 |
| international connection | 8.0 | 8.0 | 8.0 |
| Total |  |  |  |
| local connection | 9.0 | 5.5 | 11.5 |
| toll connection | 11.0 | 7.5 | 13.5 |
| international connection | 14.0 | 10.5 | 16.5 |
| NOTES 1 The following assumptions apply: – all values represent mean delays; – an M-M call uses PSTN/ISDN as the transit network; – for F-M and M-M calls the called terminal is already authenticated; – the percentage of M-M calls is generally very low (less than 10%). 2 Values in the table relate to normal traffic conditions. Target values for overload conditions are for further study. 3 The influence of the satellite segment in satellite-based land mobile networks is for further study. 4 Individual component delays for authentication, paging, routing number transfer are for reference only and do not represent GOS limits, only the total post selection delay values are GOS targets. |  |  |  |

The values in Table 1 include the time consumed in acquiring the radio resources. Typical (average) times for acquiring radio resources in second generation systems, as related to the whole post-selection process range from 2 to 3 seconds, depending on the operating environment.

### 6.1.2	Answer signal delay

It is proposed that the target values provided in Recommendation E.721 be used as a basis for this parameter with additional allowance of 0.25 sec for the delay introduced by the radio part (i.e. radio components and the signalling across the radio interface). The answer signal delay target values are indicated in Table 2.

TABLE 2/E.772

Proposed (average) target values for answer signal delays

| Answer signal delay (secs) |  |  |  |
| --- | --- | --- | --- |
| Call type | F-M | M-F | M-M |
| local connection | 1.0 | 1.0 | 1.25 |
| toll connection | 1.75 | 1.75 | 2.0 |
| international connection | 2.75 | 2.75 | 3.0 |
| NOTES 1 The following assumptions apply: – all values represent mean delays; – an M-M call uses PSTN/ISDN as the transit network; – target values apply to first and second generation public land mobile networks; – the percentage of M-M calls is generally very low (less than 10%). 2 Values in the table relate to normal traffic conditions. Target values for overload conditions are for further study. 3 The influence of the satellite segment in satellite-based land mobile networks is for further study. |  |  |  |

### 6.1.3		Probability of end-to-end blocking

The contribution of cellular systems to the probability of end-to-end blocking includes two components particular to the mobile systems. These are:

•	probability of blocking on the radio channels; and

•	probability of blocking on PLMN-to-fixed network circuits.

In existing mobile systems, blocking on radio channels is typically in the order of 5-10% and PLMN to fixed network circuits have been engineered to 1%.

Mobile systems should evolve towards a situation in which the contribution of the mobile network is comparable to the existing standards for fixed network trunk groups. For example:

•	radio channel blocking less than 1%;

•	PLMN to fixed network circuit blocking less than 0.5%.

With these values, the blocking contribution of the mobile system would provide M-F and F-M connections with a medium length fixed network extension with an end-to-end blocking increase similar to the increase that would be experienced by adding a remote switching unit to the fixed network.

#### 6.1.3.1	Probability of blocking on the radio channels

The basic dimensioning problem in cellular systems is to decide how many radio channels are required in each cell to provide a pre-specified blocking GOS. Independently of how channels are assigned to cells, a cell is provided with a pool of radio channels and four streams of arriving traffic, representing F-M, M-F, M-M calls, and handover attempts competing for the channels. In general, most cellular systems give preference to handover attempts either by assigning them higher priority or by serving them on a delay basis. The other calls (i.e. F-M, M-F and M-M) are normally served on a loss basis.

The two main factors that have an impact on radio channel blocking performance are:

•	characteristics (e.g. mean and variance) of traffic offered to each cell;

•	characteristics of handover traffic into and out of each of the cells (e.g. mean and variance).

The numerical limits of radio channel blocking standards will depend upon the type of call. Blocking for M-F (F-M) calls within a cellular system is defined as the probability that no free radio channel is available to provide a path between the calling mobile and the network (the network and the called mobile). Blocking for M-M calls is the probability that no free radio channel is available to establish a path either between the calling mobile and the MSC or between the MSC and the called mobile, or both.

Traffic characteristics in PLMN differ considerably from those of fixed networks because of both time and space dimensions to traffic variations in PLMNs. Measurement issues and methods for mobile traffic characterization are for further study.

#### 6.1.3.2	Target values for blocking on the radio channels

See Table 4.

TABLE 4/E.772

Proposed (average) target values for blocking 
on the radio channels

| Probability of blocking on the radio channels |  |  |
| --- | --- | --- |
| Call type | F-M, M-F | M-M |
| Probability of blocking on the radio links | (10-2) | f.s. |
| NOTES 1 Values in the table relate to mean values; percentile values, where applicable, are for further study. 2 Values in the table relate to normal traffic conditions. Target values for overload conditions are for further study. 3 The influence of satellite-based land mobile domain is for further study. f.s. Further study |  |  |

#### 6.1.3.3	Probability of blocking on PLMN-to-fixed network circuits

A very large proportion of cellular calls must pass through the PSTN or ISDN. There are a number of alternative architectures for the landline segment of the PLMN serving a metropolitan network. Among them are: MSC being treated like a PABX, connected by a dedicated trunk group to a single local exchange; MSC serving the area with the status of a local exchange; MSC being treated like a local exchange and connected to a transit exchange (see Recommendation E.220). Standard teletraffic methods apply to these situations, provided the number and locations of the MSC and the BS controllers are pre-specified. Blocking in the PLMN to PSTN, or ISDN circuits is the probability that no free circuit is available to establish a path between the PLMN and the PSTN, or ISDN. The blocking of the PLMN-PSTN, or PLMN-ISDN interconnection trunks basically depends on the volume of traffic that is offered to the trunk group. This traffic is usually smooth (i.e. a peakedness factor of less than one).

Technically, there is no difference between the PLMN to PSTN, or ISDN interconnecting circuits and the inter-exchange circuits in the PSTN or ISDN.

#### 6.1.3.4	Target values for blocking on PLMN-to-fixed network circuits

Table 5 gives the target values for blocking on PLMN-to-fixed network circuits. These values are suggested to be considered for existing or near-term mobile systems. Future mobile systems under study, such as FPLMTS or UMTS, are targeted to perform generally better than existing or near-term systems. The target values in Table 5 are then to be considered as upper bounds for future mobile systems.

TABLE 5/E.772

Proposed (average) target values for blocking on 
PLMN-to-fixed network circuits

| Probability of blocking on PLMN to fixed network circuits |  |  |
| --- | --- | --- |
| Call type | F-M and M-F | M-M |
| Probability of blocking on PLMN to fixed network circuits | (5)(10-3) | f.s. |
| NOTES 1 Values in the table relate to mean values; percentile values, where applicable, are for further study. 2 Values in the table relate to normal traffic conditions. Target values for overload conditions are for further study. 3 The influence of satellite-based land mobile domain is for further study. f.s. Further study |  |  |

Editor Note: End of text from E.771 for consideration and possible adaptation in New Draft E.772

Editor Note: Begin of text from E.771 requiring thorough revisitation for possible adaptation in New Draft E.772

## 6.2		Information transfer

### 6.2.1	Probability of unsuccessful land cellular handover

#### 6.2.1.1	Target values for probability of unsuccessful handover handling

Table 6 gives the target values for unsuccessful handover handling. These values are suggested to be considered for existing or near-term mobile systems.

The current PLMN provide a probability of unsuccessful handover failure equal to 0.01 which is comparable to the blocking probability on call originations. However, since handover failure affects calls already in progress and since many systems are starting to use dynamic or adaptive channel assignments to improve handover performance, it is proposed that a target value of 0.005 be assigned to this parameter (normal traffic load and radio transmission conditions).

Future mobile systems under study, such as FPLMTS or UMTS, are targeted to perform generally better than existing or near-term systems. The target values in Table 6 are then to be considered as upper bounds for future mobile systems.

TABLE 6/E.772

Proposed (average) target values for unsuccessful 
land cellular handover

| Probability of unsuccessful land cellular handover |  |
| --- | --- |
| Call type | F-M, M-F, M-M |
| Probability of unsuccessful land cellular handover | (5)(10-3) |
| NOTES 1 Values in the table relate to mean values; percentile values, where applicable, are for further study. 2 Values in the table relate to normal traffic conditions. Target values for overload conditions are for further study. 3 The influence of satellite-based land mobile domain is for further study. f.s. Further study |  |

Editor Note: End of text from E.771 requiring thorough revisitation for possible adaptation in New Draft E.772

Editor Note: Begin of text from E.771 for consideration and possible adaptation in New Draft E.772

## 6.3		Call clear down

### 6.3.1	Call release delay

#### 6.3.1.1	Target values for call release delay

This parameter captures the delay encountered within the PLMN radio part (radio interface and radio components) for sending and receiving call release-related messages (e.g. DISCONNECT, RELEASE/CLEAR, ACK) and for actual release of radio resources. For example, if a mobile station is the calling party and initiates call release (Calling Party Release), it will initiate a DISCONNECT message when the "end" button is pressed. On receiving the DISCONNECT message, the MSC initiates the release of the connection towards the called party, and also instructs the mobile station to clear. Once the mobile station has cleared and sent an ACK message, the MSC instructs the BSS to release the dedicated resources and initiates the release of the terrestrial channel. The release procedure is complete when the BSS sends an ACK message indicating that it has cleared the dedicated resources for the call.

It is proposed that a target value of 1.0 sec (average at normal load) be assigned to this parameter. (The corresponding target for ISDN in Recommendation E.721 is 0.4 sec), see Table 3.

TABLE 3/E.772

Proposed (average) target values for call release delay

| Call release delay (secs) |  |  |  |
| --- | --- | --- | --- |
| Call type | F-M, M-F, M-M |  |  |
| Calling, or called, party clears |  | 1.0 |  |
| NOTES 1 Values in the table relate to mean values; percentile values, where applicable, are for further study. 2 Values in the table relate to normal traffic conditions. Target values for overload conditions are for further study. 3 The influence of the satellite segment in satellite based land mobile networks is for further study. |  |  |  |

Editor Note: End of text from E.771 for consideration and possible adaptation in New Draft E.772

# 7	History

Recommendation E.772 was first published in 200x.

# Bibliography

–	AVELLANEDA (O.A.), PANDYA (R.N.): Traffic Grade of Service Standards for Cellular Mobile Radio Systems – Issues and Approaches, 12th Int. Teletraffic Congress, paper No. 5.2B.6, Turin, 1-8 June, 1988.

# ANNEX A

Scope of traffic engineering for mobile systems

## A.1	Scope of traffic engineering for 3G systems

Figure A.1/E.772 shows the scope of E.772 with respect to realizations of third generation systems that will coexist with second generation based systems.

Note 1 -  Direct connection between SGSN and GGSN indicates co-location of supported functionality.

Note 2 - In the circuit switched domain, data traffic is routed through the MSC. The Iu interface is terminated in the MSC where:

- Iucs user plane is terminated;

- radio frames are adapted into 64 kbit/s circuits (these circuits are then switched to the transcoder pool, transcoded into G.711 law on 64 kbit/s circuits and switched again to the destination).

Note 3 - In the packet switched domain

- SGSN ensures mobility management, session management and packet-relaying functions;

- GGSN allows interworking between the GPRS access network and external IP Packet Data Networks (PDNs) such as ISPs, Corporate Networks or the Internet.

FIGURE A.1/E.772

Reference configuration for a 3G system where both circuit switched and packed switched services are supported.

Editor Note: Begin of text from E.771 for consideration and possible adaptation in New Draft E.772

# ANNEX B

Cellular systems

## B.1	Post-selection delay

A mobile subscriber who wants to communicate with another party in the PSTN, or ISDN, i.e. establish a Mobile-to-Fixed (M-F) call, or to another mobile, i.e. a Mobile-to-Mobile (M-M) call, must be assigned one channel (e.g. FDMA or TDMA) at his serving base station in order to complete his call. This is normally accomplished by a signalling sequence which is sent first over one of several dedicated access channels (radio data links) common to all subscribers in the system and, next, over a land data link which connects the subscriber's BS controller to the home MSC. The random access signalling protocol is usually similar to the carrier sense multiple access with collision detection (CSMA-CD) scheme. Due to the nature of the access channel, it is possible to observe congestion and capacity waste when collisions occur. In the case of a mobile-terminated (i.e. F-M or M-M) call, a paging message is simultaneously sent by the MSC over the land data links to all BS controllers in the location area of the MSC registered in the HLR/VLR databases. The controllers relay the paging message over the paging channels assigned to their cells and, once the called mobile has recognized its page, it responds over the access channel.

In any PLMN, post-selection delay is one of the most important system performance criteria. The numerical limits of post-selection delay standards would depend on the type of call (i.e. F-M, M-M or M-F). The main factors which determine the delays experienced by a call being routed through the PLMN are (depending on the architecture of the PLMN):

•	transmission delays for call initiation signals generated by mobile units (including retransmissions to resolve collisions);

•	signalling delays on the land data link joining the MSC and each BS controller;

•	switching and processing delays in the MSCs;

•	signalling delays on the Common Channel System connection between MSC and location/registration databases.

The data link connection is usually based on the HDLC protocol with full duplex. Since contention exists in the data path, heavily loaded systems will experience higher delays. An important consideration is that large call set-up delays on mobile-terminated calls (F-M and M-M) can result in non-productive use of PSTN, or ISDN, resources.

Figure B.1/E.772 shows an example of the call set-up phases for an M-F connection in the case of interconnection of fixed and cellular mobile networks (see Figure l/E.751-.

++++++++++

Editor Note: This text makes use of terminology adopted for the UMTS system in the IMT-2000 family.

The call set-up procedure is performed in four steps, as indicated in Fig. B.1/E.772. In the first step (“RRC establish”), a set of dedicated control channels (DCCH) is set up for conveying the signaling information exchange between the mobile (UE) and the network (UTRAN and CN) in all later stages of the connection establishment. More in detail, the mobile on a special channel (random access channel, RACH), first issues a Request message. The network responds by checking the availability of radio resources required by the DCCHs and notifying the mobile the connection parameters – notably the scrambling code to be used for identifying the cannel. This step concludes (RRC connection set-up complete message) when the mobile configures its control channels according to the indications from the network. The time taken by this step is around 2 seconds.

In the second step (“Non Access Stratum, NAS, signaling connection”), the mobile notifies the type of requested service (voice or packet data) via a Service request message. The network reacts by checking whether the user profile entitles to such a service and, in the affirmative case, notifies the mobile with a message, which allows continuing through the subsequent stages (Call_proceeding message). The time taken by this step is around 3 seconds.

In the third step (“Radio Access Bearer, RAB, assignment”), CN asks UTRAN to provide the radio resources necessary to carry the requested service. As a result, RNC checks the availability of the resources and, in the affirmative case, notifies the CN with a RAB_assignment message. The time taken by this step is around 2 seconds. As indicated in Fig. B.1/E.772, the delays described so far represent the UMTS contribution to the post dialing delays (around 7 seconds).

Following the RAB assignment phase, connection build up continues in the “external network”. The related protocol processing is obviously outside the scope and control of the testbed. When the connection through the external network is established, CN is notified via an alert message. The time taken to establish a connection in the external network is 5 seconds with the testbed arrangement.  This is the last contribution to the post selection delay (around 12 seconds in total).

In the fourth and final step (“Non Access Stratum, NAS, signaling”), the mobile is notified (via an Alert message) that the called party is receiving the ringing tone. The time from issuing of the alert message by the external network to end of the call set-up procedure includes the called party answer delay.

Note  - The names of netwok elements and signaling messages relate to the UMTS system in the IMT-2000 family

FIGURE B.1/E.772

Typical events associated with the call set-up phase, and related GOS parameters.

++++++++++

Editor Note: End of text from E.771 for consideration and possible adaptation in New Draft E.772

ANNEX C

Handover handling

## C.1	Network elements and signaling messages

Editor Note: This text makes use of terminology adopted for the UMTS system in the IMT-2000 family.

The handover procedure starts when the mobile issues the measurement-based triggering message, as indicated in Fig. C.1/E.772. This event is the final stage of a process in which aspects such as power level in the current and in the adjacent cells, as well as operation stability are evaluated. Responding to this message, the RAN checks whether sufficient radio resources are available for activating a new branch (Radio Admission Control). In the affirmative case, resource allocation is enforced via an exchange of messages between RNC and Node B, with the latter controlling the new branch. The check procedure takes 800 milliseconds, whereas the message exchange takes 700 milliseconds. Upon completion of this phase, RNC sends to the mobile a message confirming the allocation and a request for the mobile to update its list of active branches (Active_set_update message). After 300 milliseconds, the mobile notifies about the updating (Active_set_update_complete message). To conclude the procedure, RNC sends three messages to the mobile to locally set the parameters to be used in the subsequent measurement cycles.

In summary, the total time taken by the soft handover procedure is 1.8 seconds.

Note  - The names of netwok elements and signaling messages relate to the UMTS system in the IMT-2000 family

FIGURE C.1/E.772

Typical events associated with the handover handling  procedure, and related GOS parameters.

______________

A definition of handover for teletraffic purposes is given in Recommendation E.600.

Support of personal mobility within FPLMTS environments (so-called "FPLMTS user mobility") may involve significant database interrogation and extended use of IN capabilities.

Also, the access function may use a "marked idle" concept, to gain flexibility of assigning resources; the TDMA concept is especially suited to this type of logical operation.

The MSC function may be only conceptual; in physical form, this may be a digital central office with Intelligent Network capabilities, as in the case of integrated mobile-fixed network operation.