| INTERNATIONAL TELECOMMUNICATION UNION |  | Focus Group On "From/In/To Cars Communication II" |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  | FG CarCom-C-14 |
|  |  | English only Original: English |
|  |  | Geneva, 6 March 2009 |
| CONTRIBUTION |  |  |
| Source: | Chairman of the Focus Group |  |
| Title: | Draft 3 Specification "Wideband Hands-Free Communication in Motor Vehicles" |  |

Wideband Hands Free Communication in Motor Vehicles

Summary

This Specification defines test methodologies for and standard behaviour of wideband hands free communications terminals when used within Motor Vehicles. The purpose is to provide a consistent and high Quality of Service for the users of such devices.

Keywords

Hands-Free, Headset, Motor Vehicles, Quality of Service, QoS

Contents

1	Scope	8

2	References	8

3	Definitions	10

4	Abbreviations	11

5	Conventions	12

6	How to Use the Specification	13

7	Test Arrangement	14

7.1	Test Arrangement in a Car	17

7.1.1	Microphone related Simulation	17

7.1.2	Positioning of the Hands-free Terminals	18

7.1.3	Artificial Mouth	19

7.1.4	Artificial Ear	19

7.1.5	Influence of the transmission system	19

7.1.6	Calibration and Equalization	19

7.1.6.1	Calibration:	19

7.1.6.2	Reference Measurement:	20

7.1.7	System Simulator Settings	20

7.1.8	Environmental Conditions	20

8	Digital Interfaces for Development, Debugging and Test	20

8.1	Interfaces and Access Points	20

8.2	Test Setup and Tests	22

8.2.1	Recording and Insert Background Noise	22

8.2.2	Recording and Insert Near-end Speech Recordings	22

8.2.3	One Way Speech Quality in Sending	23

8.2.4	Speech Distortion in Double Talk	23

9	Test Signals and Test Signal Levels	24

9.1.1	Signals	24

9.1.2	Background Noise Signals	24

10	Measurement Parameters and Requirements for Microphones used in Speakerphone Hands-Free Systems	26

10.1	Microphone Measurements in Anechoic Conditions	26

10.1.1	Microphone Sensitivity	26

10.1.1.1	Requirements	26

10.1.1.2	Test	26

10.1.2	Microphone Frequency Response	27

10.1.2.1	Requirements	27

10.1.2.2	Test	27

10.1.3	Microphone Directional Characteristics	28

10.1.3.1	Requirements	28

10.1.3.2	Test	28

10.1.4	Microphone Distortion	29

10.1.4.1	Requirements	29

10.1.4.2	Test	29

10.1.5	Maximum Sound Pressure Level	29

10.1.5.1	Requirements	29

10.1.5.2	Test	29

10.1.6	Self Noise	30

10.1.6.1	Requirements	30

10.1.6.2	Test	30

10.2	Microphone Measurements in the Car	30

10.2.1	Microphone Output Level in the Car	31

10.2.1.1	Requirements	31

10.2.1.2	Test	32

10.2.2	Overload point	32

10.2.2.1	Requirements	32

10.2.2.2	Test	32

10.2.3	Microphone Frequency Response in the Car	33

10.2.3.1	Requirements	33

10.2.3.2	Test	33

10.2.4	Idle Channel Noise	34

10.2.4.1	Requirements	34

10.2.4.2	Test	34

10.2.5	Ambient Noise Rejection (in the Car)	35

10.2.5.1	Requirements	35

10.2.5.2	Test	35

11	Measurement Parameters and Requirements for Hands-Free Terminals	37

11.1	Preparation Measurements	37

11.2	Delay	37

11.2.1	Delay in Sending Direction	37

11.2.1.1	Requirements	37

11.2.1.2	Test	38

11.2.2	Delay in Receiving Direction	38

11.2.2.1	Requirements	38

11.2.2.2	Test	39

11.3	Loudness Ratings	40

11.3.1	Requirements	40

11.3.2	Test Loudness Rating in Sending Direction	40

11.3.3	Test Loudness Rating in Receiving Direction	41

11.3.4	Variation of Loudness Rating in Sending Direction	41

11.3.4.1	Requirements	41

11.3.4.2	Test	41

11.3.5	Variation of Loudness Rating in Receiving Direction	42

11.3.5.1	Requirements	42

11.3.5.2	Test	42

11.4	Sensitivity Frequency Responses	43

11.4.1	Sending Sensitivity Frequency Response	43

11.4.1.1	Requirements	43

11.4.1.2	Test	44

11.4.2	Receiving Sensitivity Frequency Response	44

11.4.2.1	Requirements	44

11.4.2.2	Test	46

11.5	Speech Quality during Single Talk	46

11.5.1	One Way Speech Quality in Sending	46

11.5.1.1	Requirement	46

11.5.1.2	Test	47

11.5.2	One Way Speech Quality in Receiving	47

11.5.2.1	Requirement	47

11.5.2.2	Test	47

11.6	Idle Channel Noise	47

11.6.1	Idle Channel Noise in Sending Direction	47

11.6.1.1	Requirements	47

11.6.1.2	Test	47

11.6.2	Idle Channel Noise in Receiving Direction	48

11.6.2.1	Requirements	48

11.6.2.2	Test	48

11.7	Out-of-Band Signals	49

11.7.1	Discrimination against Out-of-Band Signals in Sending Direction	49

11.7.1.1	Requirements	49

11.7.1.2	Test	49

11.7.2	Spurious Out-of-Band Signal in Receiving Direction	50

11.7.2.1	Requirements	50

11.7.2.2	Test	50

11.8	Distortion in Sending	50

11.8.1.1	Requirement	51

11.8.1.2	Test	51

11.9	Distortion in Receiving	51

11.9.1	Requirements	51

11.9.2	Test	51

11.10	Echo Performance without background noise	52

11.10.1	Terminal Coupling Loss (TCLw)	52

11.10.1.1	Requirements	52

11.10.1.2	Test	52

11.10.2	Echo Level vs. Time	53

11.10.2.1	Requirements	53

11.10.2.2	Test	53

11.10.3	Spectral Echo Attenuation	54

11.10.3.1	Requirements	54

11.10.3.2	Test	54

11.10.4	Initial Convergence without Background Noise	55

11.10.4.1	Requirements	55

11.10.4.2	Test	55

11.10.5	Initial Convergence with Background Noise	56

11.10.5.1	Requirements	56

11.10.5.2	Test	56

11.10.6	Echo Performance with Time Variant Echo Path	57

11.10.6.1	Requirements	57

11.10.6.2	Test	57

11.10.7	Switching Characteristics	58

11.10.7.1	Activation in Sending Direction	58

11.10.7.2	Activation in Receiving Direction	60

11.10.7.3	Attenuation Range in Sending Direction	61

11.10.7.4	Attenuation Range in Receiving Direction	63

11.11	Doubletalk Performance	64

11.11.1	Attenuation Range in Sending Direction during Double Talk: AH,S,dt	64

11.11.1.1	Requirements	64

11.11.1.2	Test	65

11.11.2	Attenuation Range in Receiving Direction during Double Talk: AH,R,dt	67

11.11.2.1	Requirements	67

11.11.2.2	Test	67

11.11.3	Detection of Echo Components during Double Talk	68

11.11.3.1	Requirements	68

11.11.3.2	Test	69

11.11.4	Sent Speech Attenuation during Double Talk	71

11.11.4.1	Requirements	71

11.11.4.2	Test	71

11.12	Background Noise Transmission	73

11.12.1	Ambient Noise Rejection	73

11.12.1.1	Requirements	73

11.12.1.2	Test	73

11.12.2	Background Noise Transmission after Call Setup	74

11.12.2.1	Requirements	74

11.12.2.2	Test	75

11.12.3	Quality of Background Noise Transmission (with Far End Speech)	76

11.12.3.1	Requirements	76

11.12.3.2	Test	76

11.12.4	Quality of Background Noise Transmission (with near End Speech)	77

11.12.4.1	Requirements	77

11.12.4.2	Test	77

11.12.5	"Comfort Noise" Injection	78

12	Verification of the Transmission Performance of Bluetooth Enabled Phones	79

12.1	Interface definition and calibration	79

12.1.1	BT Delay in Sending Direction	81

12.1.1.1	Requirements	81

12.1.1.2	Test	81

12.1.2	BT Delay in Receiving Direction	81

12.1.2.1	Requirements	81

12.1.2.2	Test	82

12.2	BT Loudness Ratings	82

12.2.1	Requirements	82

12.2.2	Test BT Junction Loudness Rating in Sending Direction	82

12.2.3	Test BT Junction Loudness Rating in Receiving Direction	83

12.2.4	BT Linearity in Sending Direction	83

12.2.4.1	Requirements	83

12.2.4.2	Test	83

12.2.5	BT Linearity in Receiving Direction	84

12.2.5.1	Requirements	84

12.2.5.2	Test	84

12.3	BT Sensitivity Frequency Responses	85

12.3.1	BT Sending Sensitivity Frequency Response	85

12.3.1.1	Requirements	85

12.3.1.2	Test	85

12.3.2	BT Receiving Sensitivity Frequency Response	85

12.3.2.1	Requirements	85

12.3.2.2	Test	86

12.4	BT Noise Cancellation Test in Sending	86

12.4.1	Requirements	86

12.4.2	Test	86

12.5	BT Speech Quality during Single Talk	87

12.5.1	One Way Speech Quality in Sending	87

12.5.1.1	Requirement	87

12.5.1.2	Test	87

12.5.2	BT One Way Speech Quality in Receiving	87

12.5.2.1	Requirement	87

12.5.2.2	Test	88

12.5.3	Verification of Disabled Echo Control	88

12.5.3.1	Requirements	88

12.5.3.2	Test	88

13	Car to Car Communication	90

14	Guidance on subjective testing	90

15	Corded Headsets	96

15.1	Connector Type	96

15.2	Connector Wiring and Electrical Specifications	96

15.3	Headset Receive Characteristics	96

15.4	Headset Transmit Characteristics	96

15.5	Standard Behaviour in the Presence of Corded Headsets	96

16	Wireless Headsets	96

16.1	Wireless Headset Types	97

16.2	Test Methodology for Verification of Standard Behaviour	97

16.3	Standard Behaviour in the Presence of Wireless Headsets	97

16.3.1	Associate Headset with Vehicle	97

16.3.2	Enter the Vehicle	97

16.3.3	Receive a Call	97

16.3.4	Make a Call	97

16.3.5	Terminate a Call	97

16.3.6	Exit the Vehicle	97

16.3.7	Listen to Other Audio Source	97

16.3.8	Switch between Other Audio and Telephony	97

16.3.9	Intra-vehicle Communications	97

A.1	One Way Speech Quality in Sending	99

A.2	One Way Speech Quality in Receiving	99

C	Performance Rating	102

C.1	Overview	102

C.2	Test categories and rating types	102

C.3	Speech and Background Noise Quality in Sending Direction	104

C.4	Speech Quality in Receiving Direction (in the car under test)	111

C.5	Echo Cancellation Performance	114

C.6	Hands-free System Stability Tests (Car to Car)	122

# Scope

The aim of this Specification is the definition of use cases, test methods and requirements for wideband hands-free communication in vehicles. The Specification covers

- build in hands-free systems

- after market hands-free carkits

- corded headsets

- wireless headsets

to be used in vehicles for communication.

Furthermore the compatibility between narrowband and wideband implementations is addressed.

The specification addresses the test of complete systems as well as the subsystems hands-free microphone and the telephone with Bluetooth link used to interconnect the hands-free system to the mobile network.

For testing the test setup and the recommended environmental conditions are described.

The methods, the analysis and the performance parameters described in this Specification are based on test signals and test procedures as defined in ITU-T Recommendations P.50, P.501, P.502 and P.340, P.380 and ETSI ES 202 739 and ETSI ES 202 740.

Although important subjective testing is outside the scope of this Specification. Guidance on how to conduct subjective tests can be found in XXXXX

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation

[1]	Berger, J., Results of objective speech quality assessment including receiving terminals using the advanced TOSQA2001, ITU-T Contribution, Dec. 2000, COM 12-20-E

[2]	ETSI EG 202 396-1: Speech quality performance in the presence of background noise;   Part 1: Background noise simulation technique and background noise database

[3]	Fingscheidt, T., Suhadi, S., Quality Assessment of Speech Enhancement Systems by Separation of Enhanced Speech, Noise, and Echo, INTERSPEECH 2007, Antwerpen, Belgium, Aug. 2007.

[4]	IEC 60120: Electroacoustics - Octave-band and fractional-octave-band filters, 1995

[5]	IEC 60268-4: Sound system equipment - Part 4: Microphones, 2004

[6]	ISO 3: “Preferred Numbers – Series of Preferred Numbers”.

[7]	ITU-T Recommendation G.122, Influence of National Systems on Stability and Talker Echo in International Connections

[8]	ITU-T Recommendation B.12 (1988), Use of  the Decibel and the Neper in Telecommunications

[9]	ITU-T Recommendation P.340, Transmission Characteristics and Speech Quality Parameters of Hands-free Telephones

[10]	ITU-T Recommendation P.380, Electroacoustic Measurements on Headsets

[11]	ITU-T Recommendation P.48, Specification for an Intermediate Reference System

[12]	ITU-T Recommendation P.50 (1993), Artificial Voices

[13]	ITU-T Recommendation P.501, Test Signals for Use in Telephonometry

[14]	ITU-T Recommendation P.502, Objective Analysis Methods for Speech Communication Systems, Using Complex Test Signals

[15]	ITU-T Recommendation P.56, Objective Measurement of Active Speech Level

[16]	ITU-T Recommendation P.57, Artificial Ears

[17]	ITU-T Recommendation P.58, Head and Torso Simulators for Telephonometry

[18]	ITU-T Recommendation P.581 (05/00), Use of Head and Torso Simulators (HATS) for Hands-free Terminal Testing

[19]	ITU-T Recommendation P.64, Telephone Transmission Quality Objective Measuring Apparatus,
Determination of Sensitivity/Frequency Characteristics of Local Telephone Systems

[20]	ITU-T Recommendation P.79, Calculation of Loudness Rating for Telephone Sets

[21]	ITU-T Recommendation P.800, Methods for Subjective Determination of Transmission Quality

[22]	ITU-T Recommendation P.800.1, Mean Opinion Score (MOS) terminology

[23]	ITU-T Recommendation P. 862, Perceptual Evaluation of Speech Quality (PESQ)

[24] 	ITU-T Recommendation P. 862.1, Mapping Function for Transforming P.862 Raw Result Scores to MOS-LQO

[25]	Sottek, R., Genuit, K., Models of Signal Processing in Human Hearing, International Journal of Electronics and Communications, p. 157-165, 2005

ES 202 739/40

# Definitions

artificial ear: device incorporating an acoustic coupler and a calibrated microphone for the measurement of the sound pressure and having an overall acoustic impedance similar to that of the median adult human ear over a given frequency band

codec: combination of an analogue-to-digital encoder and a digital-to-analogue decoder operating in opposite directions of transmission in the same equipment

Composite Source Signal (CSS): Signal composed in time by various signal elements.

ear-Drum Reference Point (DRP): point located at the end of the ear canal, corresponding to the ear-drum position

freefield reference point: point located in the free sound field, at least in 1,5 m distance from a sound source radiating in free air (in case of a head and torso simulator [HATS] in the center of the artificial head with no artificial head present)

freefield equalization: the transfer characteristics of the artificial head is equalized in such a way that for frontal sound incidence in anechoic conditions the frequency response of the artificial head is flat This equalization is specific to the HATS used.

Hands-Free Reference Point (HFRP) :A point located on the axis of the artificial mouth, at 50 cm from the outer plane of the lip ring, where the level calibration is made, under free-field conditions. It corresponds to the measurement point 11, as defined in ITU-T Rec. P.51.

hands-free terminal: telephone set that does not require the use of hands during the communications session; examples are headset, speakerphone and group-audio terminal.

Head And Torso Simulator (HATS) for telephonometry: manikin extending downward from the top of the head to the waist, designed to simulate the sound pick-up characteristics and the acoustic diffraction produced by a median human adult and to reproduce the acoustic field generated by the human mouth

Headset: device which includes telephone receiver and transmitter which is typically secured to the head or the ear of the wearer.

MOS-LQO (Mean Opinion Score – Listening-only Quality Objective): the score is calculated by means of an objective model which aims at predicting the quality for a listening-only test situation. Objective measurements made using the model given in ITU-T Rec. P.862 give results in terms of MOS-LQO. (for further information see Annex A)

MOS-TQO: the score is calculated by means of an objective model which aims at predicting the quality for a talking-only test situation. Methods generating a MOS-TQO are currently under development and not yet standardized.

Mouth Reference Point (MRP): is located on axis and 25 mm in front of the lip plane of a mouth simulator

Nominal setting of the volume control: when a receive volume control is provided, the setting which is closest to the nominal RLR of 2 dB

Receive loudness rating (RLR): the loudness loss between an electric interface in the network and the listening subscriber's ear. (The loudness loss is here defined as the weighted (dB) average of driving e.m.f. to measured sound pressure.)

Send loudness rating (SLR): the loudness loss between the speaking subscriber's mouth and an electric interface in the network. (The loudness loss is here defined as the weighted (dB) average of driving sound pressure to measured voltage.)

Wideband speech: voice service with enhanced quality compared to PCM G.711 and allowing the transmission of a vocal frequency range of at least 150 Hz to 7 kHz

# Abbreviations

<Include all abbreviations used in this Recommendation>

ACR			Absolute category rating

A/D			Analogue/Digital

AGC			Automatic Gain Control

AH,R			Attenuation Range in receiving direction

AH,R,dt		Attenuation Range in receiving direction during Double Talk

AH,S			Attenuation Range in sending direction

AH,S,dt			Attenuation Range in sending direction during Double Talk

BGN			BackGround Noise

BT			Bluetooth

BTR			Bluetooth Reference Point

CSS			Composite Source Signal

D/A			Digital/Analogue

D 			D-Value is computed directly from measurements of the difference Sm between the send sensitivities for diffuse and direct sound, Ssi (diff) and Ssi (direct), respectively.

Sm = Ssi (diff)  Ssi (direct)	(E-2)

D is computed as a weighted average of Sm

DELSM		DELSM is sometimes used for Sm. (see D-Value)

DRP			Drum Reference Point

DTX			Discontinuous Transmission

ERP			Ear Reference Point

FFT			Fast Fourier Transform

HATS			Head And Torso Simulator

HATS-HFRP	Head And Torso Simulator – Hands Free Reference Point

HF System		Hands Free System

HFT			Hands Free Terminal

JLR			Junction Loudness Rating

LS,min			minimum activation level (Sending Direction)

MOS			Mean Opinion Score

MRP			Mouth Reference Point

NR			Noise Reduction

PCM			Pulse Code Modulation

POI			Point Of Interconnection

QoS			Quality of Service

RLR			Receiving Loudness Rating

SLR			Sending Loudness Rating

Ssi(diff)			Diffuse field sensitivity

Ssi(direct)		Direct sound sensitivity

S/N			Signal to Noise Ratio

TCLw			weighted Terminal Coupling Loss

Tr,R			built-up time (Receiving Direction)

Tr,S			built-up time (Sending Direction)

# Conventions

dBm: absolute power level relative to 1 milliwatt, expressed in dB

dBm0: absolute power level in dBm referred to a point of zero relative level (0 dBr point)

dBm0p: weighted dBm0, according to ITU-T Recommendation O.41

dBm0(C): C weighted dBm0, according to ISO 1999

dBPa: sound pressure level relative to 1 Pa, expressed in dB

dBPa(A): A-weighted sound pressure level relative to 1 Pa, expressed in dB

dBSPL: sound pressure level relative to 20µPa, expressed in dB; (94dBSPL=0dBPa)

dBV(P): P-weighted voltage relative to 1 V, expressed in dB, acc. to O.41

dBr: relative power level of a signal in a transmission path referred to the level at a reference point on the path (0 dBr point)

N: Newton

Vrms: Voltage – root mean square

cPa: Compressed Pascal, sound pressure at the output of the hearing model in the “Relative Approach” after nonlinear signal processing by the human ear

# How to Use the Specification

The specification addresses different parts and stages of development of hands-free terminals.

In case of speakerphone and headset hands-free terminals section 11 describes the appropriate tests and requirements. All tests and requirements described here shall be met if a system is considered to be compliant with this specification. If not mentioned specifically the setup as described in chapter 7.1 is applied and the requirements are identical for headset and speakerphone hands-free systems.

In case of speakerphone hands-free testing different sections may apply when focussing on different parts or components of the system:

- The test of hands-free microphones is described in section 10

- A digital interface concept for testing and debugging (not mandatory) is described in section 8

- The test of the performance of the Bluetooth link when using a mobile phone with Bluetooth interface to be connected to the hands-free system is described in section 12

Note: Additional subjective testing should be performed. Information how to conduct such types of test can be found in XXX

The applicability of the different sections of the standard during a typical development process in the car industry is shown in Figure 1.

Figure 1:	Typical development cycle for a car speakerphone hands-free system and the applicability of the different sections of this specification during this process

# Test Arrangement

The acoustical interface for all hands-free terminals (speakerphones and headsets) is realized by using an artificial head (HATS - Head and Torso Simulator) according to ITU-T Recommendation P.58 [17]. The properties of the artificial head shall conform to ITU-T Recommendation P.58 [17] for sending as well as for receiving acoustical signals.

All hands-free terminals are connected to a system simulator conforming to the required transmission standard with implemented, calibrated audio interface. For some requirements in this standard the performance limits depend on the transmission system and the speech codec used in this transmission system. The corresponding tables will be found in each chapter. Table 7.1 provides an overview of the wideband-band speech codecs used for the tests.

Table 7.1: Overview of speech codecs used

| System | Codec |
| --- | --- |
| GSM 850, 900,1800,1900 | AMR-WB (ITU-T G.722.2)@12.65 kbit/s |
| UMTS (WCDMA) | AMR-WB (ITU-T G.722.2) @12.65 kbit/s |
| CDMA2000 (IS-2000) | VMR-WB (TIA-1016)@?? EVRC-WB (TIA-127-C)@?? |
| Voice over Data Network (VoDN) | ITU-T G.722 ITU-T G.722.2 ITU-T G.729.1 ITU-T G.711.1 |

The settings of the system simulator shall be chosen so that the audio signal is not influenced by any signal processing (e.g. DTX).

The test signals are fed electrically to the system simulator or acoustically to the artificial head. The test arrangement is shown in figure 2.

Note 1: Different codecs as well as the variation of the bitrate of codecs with variable bitrates will influence the speech quality. In order to take into account “real life” conditions bitrates used in the real network should be used for testing and optimization.

Note 2: For some mobile phones used in the hands-free setup the signal processing cannot be switched of completely. Therefore care should be taken to use only such phones for the tests which do not introduce additional AGC.

Figure 2:	Test Arrangement for Hands-free Terminal

The test circuit for microphone measurements is shown in figure 3.

Figure. 3:	Test Arrangement for Hands-free Microphones and Microphone Arrangements

Care has to be taken that the ripple of the supply voltage does not exceed 0.5 mVrms. Furthermore the ripple on the microphone output signal shall not exceed 0.5 mVrms measured in narrowband. Rl shall be > 10kΩ.

## Test Arrangement in a Car

### Microphone related Simulation

The transmission performances of car hands-free terminals are measured in a car cabin. In order to simulate a realistic driving situation, background noise is inserted using a 4-loudspeaker arrangement with subwoofer while measurements with background noise are conducted. In figure 4 the simulation arrangement is shown. The test arrangement conforms to ETSI EG 202 396-1, The source signal used is recorded by a measurement microphone positioned close to the hands-free microphone. If possible the output signal of the hands-free microphone can be used directly. The recordings are conducted in a real car. The loudspeaker arrangement is equalized and calibrated so that the power density spectrum measured at the microphone position is equal to the recorded one. For equalization either the measurement microphone or the hands-free microphone used for recording is used.  The maximum deviation of the A-weighted sound pressure level shall be  1dB. The third octave power density spectrum between 100 Hz and 10 kHz shall not deviate more than  3 dB from the original spectrum. A detailed description of the equalization procedure as well a database with background noises can be found in ETSI EG 202 396-1.

Figure 4: Test arrangement with background noise simulation

### Positioning of the Hands-free Terminals

The speakerphone hands-free terminal is installed according to the requirements of the manufacturers. The positioning of the microphone/microphone array and loudspeaker are given by the manufacturer. If no position requirements are given, the test lab will fix the arrangement. Typically, the microphone is positioned close to the in-door mirror, the loudspeaker is typically positioned in the footwell of the driver, respectively of the co-driver. In any case the exact positioning has to be noted. Hands-free terminals installed by the car manufacturer are measured in the original arrangement.

Headset hands-free terminals are positioned according to the requirements of the manufacturer. If no position requirements are given, the test lab will fix the arrangement.

If not stated otherwise, the artificial head (HATS Head and Torso Simulator according to ITU-T P.58 [17]) is positioned at the driver’s seat for the measurement. The position has to be in line with the average user’s position, therefore all positions and sizes of users have to be taken into account. Typically the 95% of the tallest people and 5% of the smallest people have to be considered . The size of these persons can be derived e.g. from the ‘antrometric data set' for the corresponding year (e.g. based on data used by the car manufacturers). The position of the HATS (mouth/ears) within the positioning arrangement is given individually by each car manufacturer. The position used has to be reported in detail in the test report. If no requirements for positioning are given, the distance from the microphone to the MRP  is defined by the test lab.

By using suitable measures (marks in the car, relative position to A-, B-pillar, height from the floor etc) the exact reproduction of the artificial head position must be possible at any later time.

Note: Different positions of the artificial head may highly influence the test results. Depending on the application different positions of the artificial head may be chosen for the tests. It is recommended to check the worst case position e.g. those positions where the SNR and/or the speech quality in sending may be worse.

### Artificial Mouth

The artificial mouth of the artificial head shall conform to ITU-T Recommendation P.58 [17]. The artificial mouth is equalized at the MRP according to ITU-T Recommendation P.340 [9].

In the case of speakerphone hands-free terminals the sound pressure level is calibrated at the HATS-HFRP so that the average level at HATS-HFRP is -28,7 dBPa. The sound pressure level at the MRP has to be corrected correspondingly. The detailed description for equalization at the MRP and level correction at the HATS-HFRP can be found in ITU-T Recommendation P.581 [18].

### Artificial Ear

For speakerphone hands-free terminals the ear signal of the right ear of the artificial head is used (for the cars where the steering wheel is on the right hand side, the left ear is used). The artificial head is free-field equalized, more detailed information can be found in ITU-T Recommendation P.581 [18].

For headset hands-free terminals the type of ear to be used and the positioning is described in ITU-T Recommendation P.380 [10].

### Influence of the transmission system

Measurements may be influenced by signal processing (different speech codecs, DTX, comfort noise insertion …) depending on the transmission system and the system simulator used in the test setup. If requirements cannot be fulfilled due to impairments introduced by the transmission system or the system simulator, reference measurements of the hands-free unit or measurements without acoustical components should be made documenting this behavior.

### Calibration and Equalization

The following preparation has to be completed before running the tests:

#### Calibration:

- Acoustical calibration of the measurement microphones as well as of HATS microphone

- Calibration and equalization of the artificial mouth at the MRP

- HATS-HFRP calibration (for speakerphone hands-free terminals only)

Equalization (for speakerphone hands-free terminals only):

- Free-field equalization of the artificial head, in case of more than one loudspeaker diffusefield equalization is used.

#### Reference Measurement:

- For the compensation of the different power density spectra of the measurement signals it is required to refer the measured power density spectra to the power density spectra of the test signal. This is denoted as a reference measurement.

- In sending direction the reference spectrum is recorded at the MRP.

- In receiving direction the reference spectrum is recorded at the electrical interface.

### System Simulator Settings

All settings of the system simulator have to ensure that the audio signal is not disturbed by any processing and the transmission of the HF signal is error-free. DTX shall be switched-off. For all networks the RF-level shall be set to maximum. The settings shall be reported in the test protocol.

For measurements in GSM or UMTS networks the AMR-WB codec with the a bitrate of 12.65 kb/s is used.

### Environmental Conditions

Unless specified otherwise the background noise level shall be less than -54 dBPa(A) inside the car.

# Digital Interfaces for Development, Debugging and Test

The interface concept and tests described in this section is optional and may be used for the purpose of development, debugging, and testing of hands-free implementations specifically during the development and optimization process. It can be applied if the digital interfaces are available, typically in the case of prototype or development boards, or in the case of factory-fitted HF devices.

## Interfaces and Access Points

Digital interfaces allow to record and investigate signals at the specified access points. Some of the digital interfaces at access points before the HF system processing should also allow for writing/adding a digital signal to the signal path. This is true for the sending as well as for the receiving path.

Depending on the access point, any of the following three access means should be possible:

- READ: Writing the respective signal into a file

- WRITE: Replacing a certain signal in the system by a digital signal from a file

- ADD: Adding a digital signal from a file to a certain signal in the system

Figure 5 gives an overview to the digital interfaces that are useful for development, debugging, and test.

Figure 5:	Digital Interfaces for the HF system

The digital interfaces (DI) are called DI-{R | S}n with R standing for receiving path and S standing for sending path. The number n is used to distinguish between different digital interfaces in sending and receiving path, respectively.

DI-R1 can be used to record transmitted far-end speech (READ) or to test the hands-free device under test using recorded signals without actual involvement of a system tester (WRITE).

DI-R2 in comparison with DI-R1 can be used to evaluate the HF systems core algorithms in the receiving path. Here only READ access should be realized.

In some systems further digital signal processing may be used, connected digitally or analog to the HF algorithmic core system. In this case DI-R3 yields useful signals to evaluate this system component. Such further acoustic signal processing may comprise an artificial bandwidth extension, or it may comprise typical audio processing functions related to a number of loudspeakers used (equalizers, room effects).

In the sending path, DI-S2 is the access point of highest interest. If any of the digital access points is realized, this one shall be realized as well. It allows recording (READ) of any test case signals after the AD converter. Developers and testers may choose this access point to pre-record all near-end noises in their test scenario, stemming from real driving situations or from a background noise playback arrangement. Also they may choose to pre-record all near-end speech or speech-like signals in their test scenario. DI-S2 should also allow WRITE access.

Given unchanged analog processing and AD conversion in the sending path, the recorded noise and near-end signals can then be used to repeat test cases in an efficient way. This becomes possible by digital offline addition of near-end speech and noise, and by adding this signal to the sending input path to the HF system DI-S2inadd (ADD), while the HF system is in real-time operation in sending and receiving path. In such case only the echo needs to be available in the car cabin. Therefore, no exact positioning of the HATS is required, or no HATS at all is necessary. A reduction in test effort is achieved by avoiding background noise simulation or even testing with real driving noise.

Finally, DI-S1 allows to access the HF system output signal in sending direction (READ). This signal gives important information about the core HF system’s functionality: Namely acoustic echo cancellation and noise reduction. In end-to-end test simulations (near-end to far-end) it is sometimes hard to relate problematic HF equipment behavior to the HF system stand-alone, or to its interaction with speech codec or network-sided voice enhancement devices. Investigation of signals recorded via DI-S1 may give an answer to this question.

If digital interfaces are implemented for a HF system at least one of the following formats shall be supported:

- 16 bit linear PCM

- G. 722 @ 64 kb/s

The sampling frequency of the digital interfaces should be 16 kHz, except where processing in the HF system is performed at different sampling rates. When using different sampling rates at the test system appropriate up- and down-sampling should be used.

## Test Setup and Tests

In general, the digital interfaces can be used in virtually all test cases described in section 10. If digital interfaces are available, the following recordings and tests should be done.

### Recording and Insert Background Noise

In many test cases background noises are required. Recording of the background noises can be performed digitally via interface DI-S2, feed-back into the system and addition to the microphone signal can be performed digitally with interface DI-S2inadd.

### Recording and Insert Near-end Speech Recordings

In many test cases near-end speech or artificial voice signals are required. Recording can be performed digitally via interface DI-S2, feed-back into the system and addition to the microphone signal can be performed digitally with interface DI-S2inadd.

### One Way Speech Quality in Sending

In analogy to 10.5.1 the one way speech quality in sending can be measured with stored near-end test signals (see annex) via interface DI-S2. Feed-back during the test shall be done via interface DI-S2inadd. Two measurement points shall be used: At first the electrical reference point (POI) in order to perform the test for requirement 10.5.1 yielding MOS LQO-W(POI). Secondly, the measurement can be done via the DI-S1 interface yielding MOS LQO-W(S1). Here the requirement is:

MOS-LQOW(S1) ≥ MOS- LQOW(POI) ≥ 3,6.

The value of  DELTA =  MOS-LQOW(S1) – MOS-LQOW(POI) can be considered to be the degradation caused by the codecs and the network.

### Speech Distortion in Double Talk

The digital interface allows for a comfortable measurement of the distortion of the speech component in sending in double talk. The test is aimed to help optimizing the signal processing of the HFT algorithmic core system with respect to speech quality during double talk.

The test is based on the same stored near-end speech test signals as used in section 7.2.3 (see annex A.1) recorded via interface DI-S2. These signals are used as reference signals for the determination of the speech distortion during double talk in sending.

The far-end speech test signals are the ones defined in annex A.2.

The processing steps for the test are the following:

- Before starting the double talk tests the test lab should ensure that the echo canceller is fully converged. This can be done by an appropriate training sequence (see also section 10.11).

- The HF system is to be processed in real-time with the speech input signals on both sides (interface DI-R1 in receiving, and DI-S2inadd in sending). It must be ensured that always different talkers are used for receiving and sending direction. In 25% of the test cases two female voices shall be applied, in 25% of the test cases two male voices shall be applied, and in 50% of the test cases different genders in receiving and sending direction shall be used. The echo as captured by the microphone is then added in real-time to the stored near-end speech signal accessed through interface DI-S2inadd.

- During processing, the echo signal is digitally stored via DI-S2. Also the enhanced speech signal at the output of the HF system in sending is stored via DI-S1.

- Using the echo (DI-S2), the near-end speech (DI-S2inadd), the output of the HF system in sending (DI-S1), and the signal at the electrical reference point (POI) in sending, the following speech distortion measurements shall be applied.

Speech distortion shall be evaluated in terms of the quality of the speech component (1) at DI-S1 and (2) at the POI with the stored speech signal at DI-S2inadd as reference.

The speech component of the signal at DI-S1 or at POI can be extracted using the signal separation methodology as described in [3], using a Blackman window of 1024 samples with a frame shift of  ≤ 128 samples. In analogy to 7.2.3 the requirement is stated as:

MOS-LQON(S1) ≥ MOS- LQON(POI) ≥ 2,5.

The MOS-LQON analysis is performed based on ITU-T Recommendation P.862 [23], [24]. The value of  DELTA =  MOS LQO-N(S1) - MOS LQO-N(POI) can be considered to be the speech degradation caused by the codecs and the network.

# Test Signals and Test Signal Levels

### Signals

Speech-like signals are used for the measurements which can be found in ITU-T Recommendations P.50 [12] and P.501 [13]. Detailed information about the test signal used is found in the corresponding chapter of this standard. In case CSS according to P.501 is used shaping of the wideband CSS spectrum is applied. The shaping response characteristics described in Fig.6/P.501 [xx] is applied but with extension of the 5dB/oct. shaping response characteristics from 4 kHz to 8 kHz.

For wide-band hands-free terminals all test signals - which are used in receiving direction - have to be band-limited. The band limitation is achieved by bandpass filtering in the frequency range between 50 Hz and 8 kHz using bandpass filtering providing 24 dB/octave. In sending direction the test signals are used without band limitation.

All test signal levels are referred to the average level of the test signals, averaged over the complete test sequence length if not described otherwise. In receiving direction the band-limited test signal is measured, in sending direction no band-limitation is applied.

The average signal levels for the measurements are as follows:

- -16 dBm0 in receiving direction (typical signal level in networks)

- -4.7 dBPa in sending direction at the MRP (typical average speech levels) [equivalent to 
-28,7 dBPa at the HATS-HFRP]

Note: If different networks signal levels are to be used in a tests this is stated in the individual test. The “Lombard Effect” (increased talker speech level due to high background noise) is considered in the background noise tests.

Some tests require exact synchronization of test signals in the time domain. Therefore, it is required to take into account the delays of the terminals. When analyzing signals any delay introduced by the test system codecs and terminals have to be taken into account accordingly.

### Background Noise Signals

For some measurements typical background noise is inserted. This is described in the corresponding chapter. In general such background noise should be car-specific and should be simulated for the car cabin tested. The test lab (together with the manufacturer) will decide which background noise is used for the test. Car-specific parameters e.g. driving with open roof in a cabriolet have to be taken into account. Specific driving situations e.g. driving with open window may be taken into account as well. In general it is recommended to conduct all tests during constant driving conditions simulating fixed driving speed (e.g. 130 km/h). Under this condition it is most easy to conduct reproducible measurements.

If no requirements are made by the car manufacturers, a minimum sound pressure level of -24dBPa(A), measured at the right ear of the artificial head has to be achieved. In any case the recording of a real driving noise with constant speed shall be used.

- Recording of Driving Noise

Background noise is recorded in the real car. The measurement microphone is positioned close to the hands-free microphone. Alternatively the hands-free microphone can be used for the recording of the background noise if the microphone is easily assessable.

- Playback of the Recorded Background Noise

One possibility is that the test lab employs a 4-loudspeaker arrangement for acoustic background noise reproduction in the car cabin. Typically 2 loudspeakers are mounted in the front and in the rear (left and right side). The loudspeaker should be carefully positioned in order to minimize disturbances of the transmission paths between loudspeakers and hands-free microphone and the artificial head at the driver’s seat. Details can be found in EG 202 396-1.

Alternatively the background noise can be inserted electrically to the microphone signal. Therefore the background noise signal recorded at the electrical output of the hands-free microphone(s) is inserted at the electrical access point which was used for the recording. Appropriate electronics allowing the mix of the previously recorded background noise signal(s) with the microphone signal(s) at this access point has to be provided, see Figure 6. The test lab has to ensure the right calibration of the two signals.

As a third alternative, the background noise can be digitally recorded at the DI-S2 interface in figure 5 and later digitally inserted (added) as described in section 7 via interface DI-S2inadd in figure 5.

Note: Both with analogue as well as digital electrical feedback of the noise signal (alternatives 2 and 3) structure-borne noise can be captured as well.

Figure 6: Setup for analog electrical insertion of the prerecorded background noise signal

Note: structure borne noise is also covered with this arrangement, which is part of the microphone recording.

# Measurement Parameters and Requirements for Microphones used in Speakerphone Hands-Free Systems

## Microphone Measurements in Anechoic Conditions

The scope of these measurements is the verification of microphone parameters in a defined acoustic environment without the influence of integration such as mounting, orientation and in car acoustics.

### Microphone Sensitivity

#### Requirements

The microphone sensitivity has to be measured in the free sound field. The sensitivity is referring to the sound pressure of the undisturbed free sound field (in the absence of the microphone). The sensitivity is measured at the output of the test circuit according to figure 3.

The microphone sensitivity at 1kHz shall be 300 mV/Pa 3dB when measured in the direction of it’s maximum sensitivity.

#### Test

- The test signal is a sine wave of 1kHz at a level of 0dBPa at the microphone position in the undisturbed free sound field.

- The microphone is positioned in a distance of 1m in the acoustic center line of the loudspeaker.

- The microphone is oriented to the loudspeaker with it’s direction of maximum sensitivity.

- The sensitivity is determined in mV/Pa.

Further information can be found in IEC60268-4 [5].

### Microphone Frequency Response

#### Requirements

The microphone frequency response has to be  measured in the free sound field. The frequency response is referring to the sound pressure of the undisturbed free sound field (in the absence of the microphone). The frequency response is measured at the output of the test circuit according to figure 3.

Table 10.1: Tolerance Mask for the Sending Sensitivity Frequency Response

| Frequency (Hz) | Upper Limit | Lower Limit |
| --- | --- | --- |
| 100 | 0 |  |
| 125 | 0 |  |
| 200 | 0 | -14 |
| 315 | 0 | -13 |
| 400 | 0 | -12 |
| 500 | 0 | -11 |
| 630 | 0 | -10 |
| 1 000 | 0 | -8 |
| 1 300 | 2 | -8 |
| 1 600 | 3 | -8 |
| 2 000 | 4 | -8 |
| 3 100 | 4 | -8 |
| 4 000 | 4 | -8 |
| 8 000 | 4 |  |
| Note: All sensitivity values are expressed in dB on an arbitrary scale |  |  |

Note1: Depending on customer demands other tolerance schemes than described in Table 10.1. may be applied and have to be defined in an equivalent format.

Note2: Ideally the response characteristics of the microphone should be flat in the frequency range of wideband transmission (100Hz – 7kHz). However, especially in the presence of background noise a bandwidth limitation may be desirable. No explicit recommendation can be given here since such limitation would depend on level and spectral content of the background noise and ideally should be adaptive. If however a bandwidth limitation is introduced, it should be made at both, at the high and low frequencies.

#### Test

- The test signals are sine waves at a level of 0dBPa at the microphone position in the undisturbed free sound field covering at least the defined frequency range.

- The microphone is positioned in a distance of 1m in the acoustic center line of the loudspeaker.

- The microphone is oriented to the loudspeaker with it’s direction of maximum sensitivity.

- The sensitivity for each frequency is determined in mV/Pa.

Further information can be found in IEC60268-4 [5].

### Microphone Directional Characteristics

The directional characteristic of a microphone is described by different sensitivities at different angels of sound incidence.

#### Requirements

The front to back ratio is the ratio between the sensitivity in direction of highest sensitivity and the sensitivity at the angle of lowest sensitivity expressed in dB at 1kHz. The front to back ratio is measured at the output of the test circuit according to figure 3.

To achieve appropriate noise reduction the front to back ratio shall be at least 10dB.

Note: Depending on mounting and orientation also lower front to back ratios can be an advantage.

#### Test

- The test signal is a sine wave of 1kHz at a level of 0dBPa at the microphone position in the undisturbed free sound field.

- The microphone is positioned in a distance of 1m in the acoustic center line of the loudspeaker.

- The first measurement is done with the microphone oriented to the loudspeaker with it’s direction of maximum sensitivity. The second measurement is done with the microphone oriented to the loudspeaker with it’s direction of minimum sensitivity. If the direction of minimum sensitivity is not known, it has to be determined by rotating the microphone until the minimum is found.

- The front to back ratio is determined in dB.

Further information can be found in IEC60268-4 [5].

### Microphone Distortion

#### Requirements

The microphone distortion is referring to the sound pressure of the undisturbed free field. The distortion is measured at the output of the test circuit according to figure 3.

The total harmonic distortion with a sound pressure level of  0dBPa (94dBSPL) at the position of the microphone shall be less than 1% in the narrowband frequency range.

#### Test

- The test signal is a sine wave with a frequency of 300 Hz, 500 Hz, 1 kHz, 2 kHz  at a level of 0dBPa.

- The microphone is positioned in acoustic center line of the loudspeaker.

- The microphone is oriented to the loudspeaker with it’s direction of maximum sensitivity.

- The Total Harmonic Distortion is expressed in %.

Care has to be taken, that the loudspeaker is able to produce the required sound pressure level with a lower distortion than the microphone under test.

### Maximum Sound Pressure Level

#### Requirements

The maximum sound pressure is defined by the sound pressure level where the total harmonic distortion of the microphone at 1kHz is 3% in the narrowband frequency range. The total harmonic distortion is measured at the output of the test circuit according to figure 3.

The maximum sound pressure level should be higher than 106dBSPL for a microphone with a typical sensitivity of 300mV/Pa.

#### Test

- The test signal is a sine wave with a frequency of 1 kHz and an increasing level to determine the level of 3% total harmonic distortion.

- The microphone is positioned in acoustic center line of the loudspeaker.

- The microphone is oriented to the loudspeaker with it’s direction of maximum sensitivity.

- The maximum sound pressure level is expressed in dBSPL or dBPa.

Care has to be taken, that the loudspeaker is able to produce the required sound pressure level with a lower distortion as the microphone under test.

Note: With a good microphone design the maximum sound pressure level is electrically limited by the supply circuit referring to figure 3. A microphone with higher sensitivity will reach the electrical output limits at a lower sound pressure level compared to a another microphone with lower sensitivity.

### Self Noise

#### Requirements

The maximum self noise measured at the output of the test circuit according to figure 3 in quiet conditions shall be less than -72 dBV(P).

#### Test

- For the measurement no test signal is used.

- The microphone has to be powered with a low noise voltage supply.

- The self noise is measured at the output of the test circuit according to figure 3 in the frequency range between 100 Hz and 8 kHz, A-weighting has to applied.

- The self noise is expressed in dBV(A).

Care has to be taken, that the environmental noise is below the equivalent self noise of the microphone.

## Microphone Measurements in the Car

Positioning of Hands-free Microphones

The speech quality in hands-free communication is significantly affected by the positioning of the hands-free microphone.  As the optimal microphone position can vary strongly depending on vehicle design as well as on specific requirements, there is no universally valid rule for the positioning of the microphone. However, there are some guidelines which should be considered. Nevertheless in practice this often means to find the best compromise, as not all requirements can be equally fulfilled.

- The hands-free microphone should always be placed as close as possible to the speaker, as within the near field of a sound source (in a vehicle this is up to 80-100 cm) the speech level drops by 1/d². In practical applications this typically means an analogous loss in signal-to-noise ratio. For this reason a single microphone placed nearby might give a better performance than a microphone array, which is placed further away.

- There has to be a direct path between the speaker’s mouth and the microphone. If this is not given this might result in a significant decrease in signal-to-noise ratio as well as in speech quality since the speech signal becomes reverberant.

- The direction of the highest sensitivity of the microphone should point in direction of the speaker’s mouth. If different seating positions or several speakers at are to be covered by one microphone a compromise for the microphone position has to be found, as the direction of the highest sensitivity might not cover all. However, this often means a significantly reduced performance in comparison to an optimal alignment of the microphone for a single speaker. In this case the application of additional microphones might be considered to achieve an optimal speech quality.

- A direct airstream to the microphone, e.g. from the air conditioning, has to be avoided as the speech signal might be highly disturbed by wind buffeting.

- Saturation of the microphone by loudspeakers nearby, e.g. by a centre-speaker has to be avoided. If necessary the levels of the affected loudspeakers have to be reduced.

Coupling of structure born sound to the microphone has to be avoided.

Note1: When the microphone is measured in the car , it is recommended to use the power supply provided by the car/car hands-free system.

Note2: If the microphone is integrated digitally in the car it is recommended to measure the microphone at a digital access point if available. Care should be taken in order to correctly calibrate the access point.

### Microphone Output Level in the Car

#### Requirements

The microphone sensitivity is determined from MRP to the output of the test circuit according to figure 3.

For typical applications the microphone output voltage should be in the range of

50  mVrms  +/-3dB with 0dBPa at MRP.

(equivalent to a microphone sensitivity of about 300 mV/Pa and a measurement in anechoic conditions at 50 cm distance microphone – MRP)

However, depending on specific electrical/acoustical designs, arrangements inside the car or others the sensitivity requirement may be different. Therefore this requirement has to be adapted to the individual arrangement in a car.

#### Test

- The test signal is a one third octave noise signal with a mid-frequency of 1 kHz and a level of -10 dBPa  measured at the MRP.

- The microphone sensitivity is determined in a car with a microphone installed. The test arrangement is according to the arrangement described in chapter 7.1.

- The output voltage is determined in mV.

### Overload point

#### Requirements

The overload resistance shall be > 15 dB (referred to a nominal sound pressure level of –4.7 dB Pa at the MRP and a distance of 50 cm).

#### Test

- The test signal is a one third octave noise signal with a mid-frequency of 1 kHz and a level -10 dBPa  and + 5 dBPa measured at the MRP.

- The overload point is determined in anechoic conditions. The distance between the MRP and the microphone is 30 cm. (Note: Since the artificial mouth is unable to produce a sound pressure of 10.3 dBPa the distance between artificial mouth and microphone is reduced to 30 cm).

- The output voltage is determined in mV. The deviation of the measured sensitivities shall be less than 0.1 dB.

### Microphone Frequency Response in the Car

#### Requirements

The microphone frequency response is measured from the MRP to the output of the test circuit according to figure 3.

The tolerance mask for the sensitivity frequency response in sending direction is given in table 10.2, the limit at intermediate frequencies lies on a straight line drawn between the given values on a log (frequency) – linear (dB) scale.

Table 10.2: Tolerance Mask for the Sending Sensitivity Frequency Response

| Frequency (Hz) | Upper Limit | Lower Limit |
| --- | --- | --- |
| 100 | 0 |  |
| 125 | 0 |  |
| 200 | 0 | -14 |
| 315 | 0 | -13 |
| 400 | 0 | -12 |
| 500 | 0 | -11 |
| 630 | 0 | -10 |
| 1 000 | 0 | -8 |
| 1 300 | 2 | -8 |
| 1 600 | 3 | -8 |
| 2 000 | 4 | -8 |
| 3 100 | 4 | -8 |
| 4 000 | 4 | -8 |
| 8 000 | 4 |  |
| Note: All sensitivity values are expressed in dB on an arbitrary scale |  |  |

Note: Ideally the response characteristics of the microphone should be flat in the frequency range of wideband transmission (100Hz – 7kHz). However, especially in the presence of background noise a bandwidth limitation may be desirable. No explicit recommendation can be given here since such limitation would depend on level and spectral content of the background noise and ideally should be adaptive. If however a bandwidth limitation is introduced, it should be made at both, at the high and low frequencies.

#### Test

- The test arrangement is according to chapter 7.1.

- The test signal artificial voice according to ITU-T Recommendation P.50 [12] is used. Alternatively, a periodic noise signal or CS signal according to ITU-T Recommendation P.501 [13] can be used. The correct activation of the measurement object during the measurement has to be ensured by the test lab. The artificial mouth is equalized at the MRP, the test signal level shall be -4.7 dBPa at the MRP. The test signal level is the average level of the complete test signal. Finally the level at the HATS-HFRP is adjusted to -28.7 dBPa.

- The measured power density spectrum at the MRP is used as the reference power density spectrum for determining the frequency response in sending direction.

- The sensitivity frequency response is determined in third octave intervals as given by the R. 40 series of preferred numbers in ISO 3 [6] for frequencies of 100 Hz and 8 kHz inclusive. For calculation the average measured level is referred to the level of the reference signal in each frequency band averaged over the complete test sequence length.

- The sensitivity is determined in dBV/Pa.

### Idle Channel Noise

#### Requirements

The maximum idle channel noise in sending direction measured at the output of the test circuit according to figure 3 in quiet conditions shall be less than -72 dBV(A). Spectral peaks in the frequency domain shall not exceed the averaged spectrum by more than 10 dB:

Note: It is recognized that fan noise which is individually different for each car depending microphone and fan arrangement may contribute significantly to the noise perceived by the far end user. In order to determine the level and spectral content of this noise under different operating conditions a noise test as described below may be used.

#### Test

- For the measurement no test signal is used. In order to ensure a reliable activation of active microphone arrangements an activation signal is inserted before the actual measurement. The activation signal consists of a sequence of 4 Composite Source Signals according to ITU-T Recommendation P.501 [13]. The spectrum of the test at the MRP is calibrated under free-field conditions. The level of the activation signal is 
- 28.7 dBPa measured at the HATS-HFRP.

- The test arrangement is according to chapter 7.1.

The idle channel noise is measured at the output of the test circuit according to figure 3 in the frequency range between 100 Hz and 8 kHz. The measurement requires a time window which starts exactly at the end of the activation signal. Any “ringing” of filters or receivers shall be taken into account, the time window must be shifted accordingly. The length of the time window is 1 second which is the averaging time for the idle channel noise. The test lab has to ensure the correct activation of the microphone/microphone arrangement during the measurement. If the microphone arrangement is deactivated during measurement, the measurement window has to be cut to the duration when the microphone remains activated.

The power density spectrum of the noise signal is determined using FFT (8 k  samples/48 kHz sampling rate or equivalent). A Hanning window is used.

If it is known that the microphone stays activated without any activation signal, no activation signal is required. In this case a simple noise measurement is conducted.

3.	The idle channel noise is determined by A-weighting. Spectral peaks are measured in the frequency domain. The average noise spectrum used for determining the spectral peak should be calculated as the arithmetic mean of the noise spectrum values when stated in dBVp.

### Ambient Noise Rejection (in the Car)

#### Requirements

Microphones typically have to operate with high background noise without causing any impairment. Any background noise should be reduced as much as possible. It is a known fact that low bit rate speech coders which are optimized for speech sometimes react very sensitive on background noise.

The ambient noise rejection is determined as D value from DELSM according to ITU-T Recommendation G.111. D should be ≥ -13 dB.

Note:  Considering a microphone without additional signal processing lower D-values than specified have to be expected. The table below gives measured D-values with an omnidirectional microphone on typical car installation positions. With an optimal integrated and oriented directive microphone improvements up to 3dB compared to the omnidiretional microphone are possible. Vice versa a wrong orientation or poor integration can lead to lower values. The D-value required depends on distance of the microphone to the talker, the directivity of the microphone, placement and integration in the car and others. A D-value of -13dB can be achieved for excellent designs only.

Table 10.3: D-Value depending on distance and positioning in a car

| Distance | D-value | Remark |
| --- | --- | --- |
| mm | dB |  |
| 445 | -20,94 | Mic Position = center of OHC, small driver |
| 455 | -20,62 | Mic Position s = center of OHC, large driver |
| 450 | -20,11 | Mic Position = mirror base, small driver |
| 530 | -22,07 | Mic Position = mirror base, large driver |
| 220 | -18,26 | Mic Position = sun visor, small driver |
| 245 | -16,40 | Mic Position = sun visor, medium driver |
| 270 | -18,56 | Mic Position = sun visor, large driver |

#### Test

- The test arrangement is according to chapter 7.1.

- Depending on the manufacturer/test lab requirements the background noise - which represents a typical driving condition - is inserted. The test shall be conducted under constant background noise conditions. The background noise has to be inserted at minimum 5 seconds before the actual measurement starts in order to allow adaptation of possible background noise reduction algorithms.

- The sending signal is recorded at the output of the test circuit according to figure 3. The measurements are performed in third octave bands according to IEC 60120 [4] in bands 4 - 17 (200 Hz - 4 kHz). In each band the diffuse field sensitivity Ssi(diff) is measured. The result is scaled in dBV/Pa.

- The speech signal is simulated by a Composite Source Signal according to ITU-T Recommendation P.501 [13] with a duration of  2 CS sequences. The level at the MRP is -4.7 dBPa. The direct sensitivity Ssi(direct) is measured again in third octave bands according to IEC 60120 [4] in bands 4 - 17 (200 Hz-4 kHz) and scaled in dBV/Pa.

- The D value according to ITU-T Recommendation P.79 [20], Annex E, formula E2 and E3 is calculated in bands 4-17. The coefficients Ki as described in table E1 are used.

### SNR Improvement (in the Car)

#### Requirements

The SNR improvement provided by the microphone should be ≥ 3 dB.

#### Test

- The test arrangement is according to chapter 7.1. The hands-free microphone is replaced by an omnidirectional microphone with a mostly flat frequency response characteristics.

- Depending on the manufacturer/test lab requirements the background noise - which represents a typical driving condition - is inserted. The test shall be conducted under constant background noise conditions. The background noise has to be inserted at minimum 5 seconds before the actual measurement starts in order to allow adaptation of possible background noise reduction algorithms.

- The sending signal (background noise only) is recorded at the output of the test circuit according to figure 3. The output level LBGNomni is measured between 100 Hz and 8kHz. The result is scaled in dBV/Pa.

- The speech signal is simulated by a Composite Source Signal according to ITU-T Recommendation P.501 [13] with a duration of  2 CS sequences. The output level LSomni is measured between 100 Hz and 8kHz. The result is scaled in dBV/Pa.

- The SNRomni is determined by SNRomni=LSomni-LBGNomni.

- Steps 1 – 4 are repeated using the hands-free microphone. The measured levels are LBGNhftmic and LShftmic.

- The SNRhftmic is determined by SNRhftmic=LShftmic-LBGNhftmic.

- The SNR improvement is calculated by SNRhftmic-SNRomni

# Measurement Parameters and Requirements for Hands-Free Terminals

Note: The measurements described in this section do not cover bandwidth extension technologies. If such techniques are used they should be disabled for the tests.

## Preparation Measurements

## Delay

We should specify a round trip delay, decision to be taken next meeting.

### Delay in Sending Direction

#### Requirements

The delay in sending direction is measured from the MRP (Mouth Reference Point) to POI (reference speech codec of the system simulator, output). The delay measured in sending direction  is

Ts+ tSystem .

The delay Ts shall be less than 50 ms.

Note 1: The delay should be minimized! This can, e.g., be accomplished by designing the speech decoder output, the Bluetooth link, and the hands-free system in a way, that sample-based processing and frame-based processing interoperate by using common buffers at their interfaces. Careful matching of frame shift and DFT size for the signal processing in the hands-free system to the Bluetooth link and to the speech coder allows to (partially) embed the delay of one block into the preceding one.

Note 2: The delay requirement assumes a delay of maximum 8 ms inserted by a potential Bluetooth link. Therefore tests should be made with a Bluetooth mobile phone which introduces a low delay.

Figure 11.1: Different blocks contributing to the delay in sending

The system delay tSystem is depending on the transmission method used and the network simulator. The delay tsystem must be known.

#### Test

- For the measurements a Composite Source Signal (CSS) according to ITU-T Recommendation P.501 [13] is used. The pseudo random noise (pn)-part of the CSS has to be longer than the maximum expected delay. It is recommended to use a pn sequence of 16 k samples (with 48 kHz sampling rate). The test signal level is -4.7 dBPa at the MRP. For speakerphone hands-free terminals the test signal level is adjusted to -28.7 dBPa at the HATS-HFRP (see ITU-T Recommendation P.581 [18]). The equalization of the artificial mouth is made at the MRP.

The reference signal is the original signal (test signal).

The setup of the hands-free terminal is in correspondence to chapter 7.1.

2.	The delay is determined by cross-correlation analysis between the measured signal at the electrical access point and the original signal. The measurement is corrected by delays which are caused by the test equipment.

- The delay is measured in ms and the maximum of the cross-correlation function is used for the determination.

### Delay in Receiving Direction

#### Requirements

The delay in receiving direction is measured from POI (input of the reference speech coder of the system simulators) to the Drum Reference Point (DRP). The delay measured in receiving direction is

Tr+ tSystem .

The delay Tr shall be less than 50 ms.

Note 1: The delay should be minimized! This can, e.g., be accomplished by designing the speech decoder output, the Bluetooth link, and the hands-free system in a way, that sample-based processing and frame-based processing interoperate by using common buffers at their interfaces. Careful matching of frame shift and DFT size for the signal processing in the hands-free system to the Bluetooth link and to the speech coder allows to (partially) embed the delay of one block into the preceding one.

Note 2: The delay requirement assumes a delay of maximum 8 ms inserted by a potential Bluetooth link. Therefore tests should be made with a Bluetooth mobile phone which introduces a low delay.

Figure 11.2: Different blocks contributing to the delay in receiving

The system delay tSystem is depending on the transmission system and on the network simulator used.  The delay tSystem must be known.

#### Test

- For the measurements a Composite Source Signal (CSS) according to ITU-T Recommendation P.501 [13] is used. The pseudo random noise (pn)-part of the CSS has to be longer than the maximum expected delay. It is recommended to use a pn sequence of 16 k samples (with 48 kHz sampling rate). The test signal level is -16 dBm0 at the electrical interface (POI).

The reference signal is the original signal (test signal).

- The test arrangement is according to chapter 7.1. For the measurement of speakerphone hands-free terminals the artificial head is free-field equalized according to ITU-T Recommendation P.581 [18]. The equalized output signal of the right ear is used for the measurement. For headset hands-free terminals the sound pressure measured at the DRP and corrected to the ERP according to ITU-T Recommendation P.57 [16].

- The delay is determined by cross-correlation analysis between the measured signal at the DRP and the original signal. The measurement is corrected by delays which are caused by the test equipment.

- The delay is measured in ms and the maximum of the cross-correlation function is used for the determination.

## Loudness Ratings

### Requirements

The nominal values of SLR/RLR from/to the electrical reference point (POI) shall be

For speakerphone hands-free terminals:

SLR = 13 dB +/- 4dB;

RLR = 2 dB +/- 4dB.

For headset hands-free terminals:

SLR = 8 dB +/- 4dB;

RLR = 2 dB +/- 4dB.

If a user-specific volume control is provided, the requirement for RLR given above shall be measured at least for one setting of the volume control. It is recommended to provide a volume control which allows a loudness increase by at least 15 dB referred to the nominal value of RLR. The volume control range shall allow the setting of S/N 6 dB for all signal and noise conditions. This will allow sufficient loudness of the speech signal in receiving direction in the presence of high background noise.

Note: It is recognized that the car may be a working place. Therefore care has to be taken not to exceed the limits for daily noise exposure defined in the different regional standards and directives for working places.

### Test Loudness Rating in Sending Direction

- The test signal used for the measurements shall be artificial voice according to ITU-T Recommendation P.50 [13]. The test signal is equalized at the MRP, the test signal level is –4.7 dBPa at the MRP. The test signal level is the average level of the complete test signal. For speakerphone hands-free terminals the level at the HATS-HFRP is adjusted to –28.7 dBPa.

The measured power density spectrum at the MRP is used as the reference power-density spectrum for determining the sending sensitivity.

- The test arrangement is according to chapter 7.1. The sending sensitivity is calculated from each band of the 20 frequencies given in Table 1 of ITU-T Recommendation P.79 [20], bands 1 - 20.

For the calculation the average measured level at the electrical reference point for each frequency band is referred to the average test signal level measured in each frequency band at the MRP.

- The sensitivity is expressed in terms of dBV/Pa and the SLR shall be calculated according to ITU-T Recommendation P.79 [20], Annex A

### Test Loudness Rating in Receiving Direction

- The test signal used for the measurements shall be artificial voice according to ITU-T Recommendation P.50 [12]. The test signal is -16 dBm0, measured at the electrical reference point and averaged over the complete test signal sequence.

- The test arrangement is according to chapter 7.1. For the measurement of speakerphone hands-free terminals the artificial head is free-field equalized according to ITU-T Recommendation P.581 [18]. The equalized output signals of both artificial ears are used for the measurement. The equalized output signal of each artificial ear is power-averaged on the total time of analysis; the "right" and "left" signals are voltage-summed for each 1/3 octave band frequency band. For headset hands-free terminals the sound pressure measured at the DRP of the right ear and corrected to the ERP according to ITU-T Recommendation P.57 [16]. The receiving sensitivity is determined by the bands 1 - 20 according to Table 1 of ITU-T Recommendation P.79 [20].

For the calculation the average signal level of each frequency band is referred to the signal level of the reference signal measured in each frequency band.

- The sensitivity is expressed in terms of dBPa/V and the RLR shall be calculated according to ITU-T Recommendation P.79 [20], Annex A without the LE factor.

- For speakerphone hands-free terminals the correction 8 dB according to ITU-T Recommendation P.340 [9] is used for the correction of the measurement results.

- The test is repeated for maximum volume control setting

### Variation of Loudness Rating in Sending Direction

#### Requirements

For acoustical signal level variation in the range of -3 dB /+6 dB from the nominal signal level the measured SLR shall not deviate more than +/- 0.5 dB from the SLR measured with nominal signal level.

Note 1: It is recognized that under certain conditions the use of AGC not fulfilling the requirements stated above is useful. This e.g. may be under certain network conditions. Under such conditions the linearity requirement may not be appropriate.

#### Test

- The test signal used for the measurements shall be artificial voice according to ITU-T Recommendation P.50 [12]. The test signals are  -7.7 dBPa and +1.3 dBPa, measured at the MRP. The test signal level is the average level of the complete test signal. For speakerphone hands-free terminals the level at the HATS-HFRP is adjusted to –31.7 dBPa and –22.7 dBPa

The measured power density spectrum at the MRP is used as the reference power-density spectrum for determining the sending sensitivity.

- The test arrangement is according to chapter 7.1. The sending sensitivity is calculated from each band of the 20 frequencies given in Table 1 of ITU-T Recommendation P.79 [20], bands 1 - 20.

For the calculation the average measured level at the electrical reference point for each frequency band is referred to the average test signal level measured in each frequency band at the MRP.

- The sensitivity is expressed in terms of dBV/Pa and the SLR shall be calculated according to ITU-T Recommendation P.79 [20], Annex A.

- For both signal levels the measured result is compared to the measured with nominal signal level.

### Variation of Loudness Rating in Receiving Direction

#### Requirements

With nominal volume control setting for network signal level variations of +/- 5 dB from the nominal signal level the measured RLR shall not deviate more than +/- 0.5 dB from the RLR measured with nominal signal level and nominal volume control setting.

#### Test

- The test signal used for the measurements shall be artificial voice according to ITU-T Recommendation P.50 [12]. The test signals are  -11 dBm0 and -21 dBm0, measured at the electrical reference point and averaged over the complete test signal sequence.

- The test arrangement is according to chapter 7.1. For the measurement of speakerphone hands-free terminals the artificial head is free-field equalized according to ITU-T Recommendation P.581 [13]. The equalized output signals of both artificial ears are used for the measurement. The equalized output signal of each artificial ear is power-averaged on the total time of analysis; the "right" and "left" signals are voltage-summed for each 1/3 octave band frequency band. For headset hands-free terminals the sound pressure measured at the DRP of the right ear and corrected to the ERP according to ITU-T Recommendation P.57 [16]. The receiving sensitivity is determined by the bands 1 - 20 according to Table 1 of ITU-T Recommendation P.79 [20].

For the calculation the average signal level of each frequency band is referred to the signal level of the reference signal measured in each frequency band.

- The sensitivity is expressed in terms of dBPa/V and the RLR shall be calculated according to ITU-T Recommendation P.79 [20], Annex A without the LE factor.

- For speakerphone hands-free terminals the correction 8 dB according to ITU-T Recommendation P.340 [9] is used for the correction of the measurement results.

- For both signal levels the measured result is compared to the measured with nominal signal level.

## Sensitivity Frequency Responses

### Sending Sensitivity Frequency Response

#### Requirements

The sending sensitivity frequency response is measured from the MRP to the output of the speech codec at the electrical point (output of the system simulators, POI).

The tolerance mask for the sending sensitivity frequency response is shown in Table 11.1, the mask is drawn by straight lines between the breaking points in Table 11.1 on a logarithmic (frequency) - linear (dB sensitivity) scale.

Table 11.1: Tolerance Mask for the Sending Sensitivity Frequency Response

| Frequency | Upper limit | Lower limit |
| --- | --- | --- |
| 100 Hz | 4 dB | - dB |
| 125 Hz | 4 dB | -10 dB |
| 200 Hz | 4 dB | -4 dB |
| 1 000 Hz | 4 dB | -4 dB |
| 5 000 Hz | (see note) | -4 dB |
| 6 300 Hz | 9 dB | -7 dB |
| 8 000 Hz | 9 dB | - dB |
| NOTE: The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) - logarithmic (Hz) scale. |  |  |

Note: Ideally the response characteristics of the microphone should be flat in the frequency range of wideband transmission (100Hz – 7kHz). However, especially in the presence of background noise a bandwidth limitation may be desirable. No explicit recommendation can be given here since such limitation would depend on level and spectral content of the background noise and ideally should be adaptive. If however a bandwidth limitation is introduced, it should be made at both, at the high and low frequencies.

#### Test

- The test signal used for the measurements shall be artificial voice according to ITU-T Recommendation P.50 [12]. The test signal is equalized at the MRP, the test signal level is –4.7 dBPa at the MRP. The test signal level is the average level of the complete test signal. For speakerphone hands-free terminals the level at the HATS-HFRP is adjusted to –28.7 dBPa.

The measured power density spectrum at the MRP is used as the reference power density spectrum for determining the sending sensitivity.

- The test arrangement is according to chapter 7.1. The sending sensitivity frequency response is determined in third octave bands as given by the R. 40 series of preferred numbers in ISO 3 [6] for frequencies from 100 Hz to 8 kHz inclusive. In each third octave band the level of the measured signal is referred to the level of the reference signal averaged over the complete test sequence length.

- The sensitivity is determined in dBV/Pa.

### Receiving Sensitivity Frequency Response

#### Requirements

The receiving sensitivity frequency response is measured from the electrical reference point (input of the system simulators, POI) to the ERP when headset hands-free terminals are measured. For speakerphone hands-free terminals the sound pressure of the free-field equalized HATS is measured.

The tolerance mask for the receiving sensitivity frequency response is shown in Table 11.2, the mask is drawn by straight lines between the breaking points in Table 11.2 on a logarithmic (frequency) - linear (dB sensitivity) scale.

Table 11.2: Tolerance Mask for the Receiving Sensitivity Frequency Response

| Frequency (Hz) |  | Upper Limit |  | Lower Limit |  |
| --- | --- | --- | --- | --- | --- |
| 200 |  | 0 |  |  |  |
| 250 |  | 0 |  |  |  |
| 315 |  | 0 |  | -15 |  |
| 400 |  | 0 |  | -12 |  |
| 500 |  | 0 |  | -12 |  |
| 630 |  | 0 |  | -12 |  |
| 800 |  | 0 |  | -12 |  |
| 1 000 |  | 0 |  | -12 |  |
| 1 300 |  | 0 |  | -12 |  |
| 1 600 |  | 0 |  | -12 |  |
| 2 000 |  | 0 |  | -12 |  |
| 2 500 |  | 0 |  | -12 |  |
| 3 100 |  | 0 |  | -12 |  |
| 4 000 |  | 0 |  |  |  |
| Note: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |  |  |  |
| Frequency | Upper limit |  | Lower limit |  |  |
| 125 Hz | 8 dB |  | - dB |  |  |
| 200 Hz | 8 dB |  | -12 dB |  |  |
| 250 Hz | 8 dB |  | -9 dB |  |  |
| 315 Hz | 7 dB |  | -6 dB |  |  |
| 400 Hz | 6 dB |  | -6 dB |  |  |
| 5 000 Hz | 6 dB |  | -6 dB |  |  |
| 6 300 Hz | 6 dB |  | -9 dB |  |  |
| 8 000 Hz | 6 dB |  | - dB |  |  |

NOTE 1:	Referring to ETS 300 245-6 [Error: Reference source not found], low limit has been modified: no requirement at 160 Hz, -12 dB at 200 Hz and -9 dB at 250 Hz instead of -15 dB, -9 dB and -6 dB. Rationale: better balanced response curve and avoiding necessity in most case to introduce « bass boost » for amplification.

#### Test

- The test signal used for the measurements shall be artificial voice according to ITU-T Recommendation P.50 [12]. The test signal is -16 dBm0, measured at the electrical reference point and averaged over the complete test signal sequence.

- The test arrangement is according to chapter 7.1. For the measurement of speakerphone hands-free terminals the artificial head is free-field equalized according to ITU-T Recommendation P.581 [18]. The equalized output signals of both artificial ears are used for the measurement. The equalized output signal of each artificial ear is power-averaged on the total time of analysis; the "right" and "left" signals are voltage-summed for each 1/3 octave band frequency band. For headset hands-free terminals the sound pressure measured at the DRP and corrected to the ERP according to ITU-T Recommendation P.57 [16]. The receiving sensitivity frequency response is determined in third octaves as given by the R. 40 series of the preferred numbers in ISO 3 [6] for frequencies from 100 Hz to 8 kHz inclusive. In each third octave band the level of the measured signal is referred to the level of the reference signal, averaged over the complete test sequence length.

- The sensitivity is determined in dBPa/V.

Note: Different listener position should be taken into account. Therefore the measurement should be repeated by moving the seat with the artificial head in different, typical positions.

## Speech Quality during Single Talk

### One Way Speech Quality in Sending

#### Requirement

The nominal values for the speech quality measured from/to the electrical reference point (POI) shall be

MOS-LQOW  3,6

#### Test

A test method for measuring the one way speech quality is described in the annex.

### One Way Speech Quality in Receiving

#### Requirement

The nominal values for the speech quality measured from/to the electrical reference point (POI) shall be

MOS-LQOW  3.6

#### Test

A test method for measuring the one way speech quality is described in the annex.

## Idle Channel Noise

All tests are conducted with average RF-signal power settings. It is recommended to check the requirement in addition with different RF-power settings. The requirement should be fulfilled for all RF-power settings.

For the measurements it is desirable to have a background noise level of less than -74 dBPa(A) inside the car, a minimum background noise level of -64dBPa(A) should not be exceeded.

### Idle Channel Noise in Sending Direction

#### Requirements

The maximum idle channel noise in sending direction, measured at the electrical reference point (POI) in quiet conditions shall be less than  -64 dBm0(A).

No peaks in the frequency domain higher than 10 dB above the average noise spectrum should occur.

#### Test

- For the measurement no test signal is used. In order to ensure a reliable activation of the terminal an activation signal is inserted before the actual measurement. The activation signal consists of a sequence of 4 Composite Source Signals according to ITU-T Recommendation P.501 [13]. The spectrum of the test signal at the MRP is equalized under free-field conditions. The level of the activation signal is - 28.7 dBPa measured at the HATS-HFRP.

- The test arrangement is described in chapter 7.1.

The idle channel noise is measured at the electrical reference point in the frequency range between 100 Hz and 8 kHz. The measurement requires a time window which starts exactly at the end of the activation signal. Any “ringing” of filters or receivers or reverberance influence shall be taken into account, the time window must be shifted accordingly. The length for the time window is 1 second which is the averaging time for the idle channel noise. The test lab has to ensure that the terminal is activated during the measurement. If the terminal is deactivated during the measurement, the measurement window has to be cut to the duration while the terminal remains activated.

The power density spectrum of the noise signal is determined using FFT (8 k  samples/48 kHz sampling rate or equivalent). A Hanning window is used.

- The idle channel noise is determined by A- weighting. Spectral peaks are measured in the frequency domain. The average noise spectrum used for determining the spectral peak should be calculated as the arithmetic mean of the noise spectrum values when stated in dBm0(A).

### Idle Channel Noise in Receiving Direction

#### Requirements

The requirements for the maximum noise produced by the hands-free terminal in case no signal is applied to the receiving direction is as follows:

If a user-specific volume control is provided, it is adjusted to the RLR value close to the nominal value. Hands-free terminals without user-specific volume controls are measured in normal operating conditions. The idle channel noise level measured at the DRP shall be less than -53 dBPa (A).

No peaks in the frequency domain higher than 10 dB above the average noise spectrum should occur.

#### Test

- For the measurements no test signal is used. In order to ensure a reliable activation of the terminal an activation signal is inserted before the actual measurement. The activation signal consists of a sequence of 4 Composite Source Signals according to ITU-T Recommendation P.501 [13]. The level of the activation level is adjusted to –16 dBm0, measured at the electrical reference point. The level of the activation signal is averaged over the complete duration of the activation signal.

- The test arrangement is according to chapter 7.1. For the measurement of speakerphone hands-free terminals the artificial head is free-field equalized according to ITU-T Recommendation P.581 [13]. The equalized output signal of the right ear is used for the measurement. For headset hands-free terminals the sound pressure measured at the DRP and corrected to the ERP according to ITU-T Recommendation P.57 [16].

The idle channel noise is measured at the DRP in the frequency range between 50 Hz and 10 kHz. The measurement requires a time window which starts exactly at the end of the activation signal. Any ringing of filters or receivers or reverberance influence shall be taken into account. The time window must be shifted accordingly. The length of the time window is 1 second which is the averaging time for the idle channel noise.

The power density spectrum of the noise signal is determined using FFT (8 k samples/48 kHz sampling rate or equivalent). A Hanning window is used.

3.	The idle channel noise is A-weighted. Spectral peaks are measured in the frequency domain. The average noise spectrum used for determining the spectral peak should be calculated as the arithmetic mean of the noise spectrum values when stated in dBPa(A).

## Out-of-Band Signals

For the measurements it is desirable to have a background noise level of less than -74 dBPa(A) inside the car, a minimum background noise level of -64dBPa(A) should not be exceeded.

### Discrimination against Out-of-Band Signals in Sending Direction

#### Requirements

If out-of-band signals are generated at the MRP, they are transmitted to the terminal and possibly to the input of the speech decoder. For signals which are measured at the output of the speech decoder at the electrical reference point (POI) the following requirements shall apply:

For the measurement a white Gaussian noise band-limited in the frequency range between 8 kHz and 10 kHz with a level of –4.7 dBPa at the MRP is used. The total level - measured in a frequency range from 200 Hz to 7 kHz - is measured at the electrical reference point (POI) and shall be less than 35 dB referred to the reference level. The reference level is determined using artificial voice according to ITU-T Recommendation P.50 [13], band-limited in the frequency range between 100 Hz and 8 kHz with a level of –4.7 dBPa at the MRP. For this signal the in-band level averaged over the complete reference signal length is determined at the electrical reference point.

#### Test

- The test arrangement is according to chapter 7.1.

- In order to ensure a reliable activation of the hands-free terminal an activation signal is generated before the actual measurement starts. The activation signal consists of a sequence of 4 Composite Source Signals (CSS) according to ITU-T Recommendation P.501 [13]. The level of the activation level shall be –4.7 dBPa, measured at the MRP. The level of the activation signal is averaged over the complete activation sequence signal.

- Directly after the activation signal the actual test signal is inserted. A test signal is inserted exactly after the voiced sound of the last CSS burst (instead of the pn sequence). The duration of the test signal amounts to 200 ms.

- The test signal is a white Gaussian noise, band-limited from 8 kHz to 10 kHz with a level of –4.7 dBPa at the MRP. The level of the test signal is averaged over the complete test signal sequence.

- For the analysis a rectangular window is used which is adapted to the test signal duration (200 ms). Any “ringing” of filters or receivers or reverberance influence shall be taken into account, the time window must be shifted accordingly. The signal level is determined in the frequency range from 300 Hz to 3.4 kHz at the electrical reference point (POI). The level of the reference signal (artificial voice according to ITU-T Recommendation P.50 [12], band-limited from 100 Hz to 8 kHz, -4.7 dBPa at the MRP) is determined at the electrical reference point (POI) as well.

Note: With low sensitivity in sending direction the measured noise level may already exceed the required minimum out-of-band level.

### Spurious Out-of-Band Signal in Receiving Direction

#### Requirements

The test signal used is artificial voice according to ITU-T Recommendation P.50 [2], band-limited in a frequency range between 100 Hz and 7 kHz with a level of -12 dBm0 in receiving direction. The level of the out-of-band signal is measured in a frequency range between 8.6 and 16 kHz at the hands-free loudspeaker and shall be at least 45 dB below the level of the reference signal. The level of the reference signal is determined by measuring the acoustical level of the in-band signal at the hands-free loudspeaker.

#### Test

- The test signal shall be artificial voice according to ITU-T Recommendation P.50 [12]. The level of the test signal is averaged over the complete test signal sequence.

- The output signal of a measurement microphone positioned close to the hands-free loudspeaker is used for the measurement. By this the S/N of the tests can be improved as compared to measurements conducted with the artificial head. For headset hands-free terminals currently no test can be recommended due to the limitations of the specification for HATS and ear simulators.

- The level of the out-of-band signal is determined between 8.6 and 16 kHz. The reference level is determined by measuring the in-band signal at the hands-free loudspeaker.

Note 1: With low sensitivity in receiving direction the measured noise level may already exceed the required minimum out-of-band level.

Note 2: This measurement method does not apply to systems including artificial bandwidth extension.

## Distortion in Sending

The distortion in sending up to 8 kHz is measured from the MRP to the electrical reference point (input of the system simulator, POI).

Note: It is recognized that for some systems including AGC or companding techniques the distortion test does not lead to valid results.

For the measurements it is desirable to have a background noise level of less than -74 dBPa(A) inside the car, a minimum background noise level of -64dBPa(A) should not be exceeded.

#### Requirement

The harmonic distortion in sending should be not higher than 3%.

#### Test

- The test signal is a sinusoidal signal with a frequency of 300 Hz, 500 Hz, 1kHz and 2kHz. The test signal level is -4.7 dBPa. In order to guarantee a reliable activation of the hands-free terminal a sequence of 4 Composite Source Signals according to ITU-T Recommendation P.501 [13] is sent to the terminal before the actual test signal. The activation signal level is -4.7 dBPa, measured at the MRP. The activation signal level is averaged over the total length of the activation signal.

- The test signal is inserted immediately after the activation sequence, after the voiced sound of the last CSS-burst (instead of the pn-sequence).The test signal duration is 200 ms.

- For the analysis a Hanning window is used which is adapted to the duration of the test signal (200 ms).

- The harmonic distortion produced by the hands-free terminal is measured at the electrical reference point.

## Distortion in Receiving

The distortion in receiving measured from the POI to the artificial ear up to 15 kHz.

Note: It is recognized that for some systems including AGC, companding techniques or bandwidth extension the distortion test does not lead to valid results.

For the measurements it is desirable to have a background noise level of less than -74 dBPa(A) inside the car, a minimum background noise level of -64dBPa(A) should not be exceeded.

### Requirements

The distortion in receiving is measured from a linear access point (access point at the hands-free system where no nonlinear and time variant signal processing except the speech coder is present) to the DRP.

The harmonic distortion should be less than 3% when producing a sound pressure level needed to achieve a S/N 6 dB (see Ch. 7, preparation measurements) and for the maximum volume control setting.

This test is applicable if a linear access point without any non linear signal processing to the loudspeaker amplifier is available. If this access point is not available the measurement may be conducted with some care since non linear processing may influence the test result.

### Test

- The test signal is a sinusoidal signal with a frequency of 300 Hz, 500 Hz, 1 kHz, 2kHz and 3 kHz. The test signal level is the level measured at the linear access point when inserting a test signal with -16 dBm0 at the POI. In order to guarantee a reliable activation of the hands-free terminal a sequence of 4 Composite Source Signals according to ITU-T Recommendation P.501 [13] is sent to the terminal before the actual test signal. The activation signal level is the level equivalent to the level when inserting a test signal at the POI with -16dBm0, measured at the linear access point. The activation signal level is averaged over the total length of the activation signal.

- The test signal is inserted immediately after the activation sequence, after the voiced sound of the last CSS-burst (instead of the pn-sequence).The test signal duration is 200 ms.

- For the analysis a Hanning window is used which is adapted to the duration of the test signal (200 ms).

- The harmonic distortion is measured for each test signal frequency.

Note: This measurement method does not apply to systems including artificial bandwidth extension.

## Echo Performance without background noise

Due to the expected delay in networks the echo loss presented at the electrical reference point (POI) should be at least 50 dB during single talk. This echo loss (TCLW) should be achieved for a wide range of acoustical environments and delays.

Note: When realizing echo loss by speech-activated attenuation/gain control "Comfort Noise" should be inserted in case the signal is completely suppressed.

For the measurements it is desirable to have a background noise level of less than -74 dBPa(A) inside the car, a minimum background noise level of -64dBPa(A) should not be exceeded.

### Terminal Coupling Loss (TCLw)

#### Requirements

The TCLW in quiet environments should be at least 50 dB for nominal setting of the volume control. For maximum setting of the volume control TCLW should be higher than 50 dB. The implemented echo control mechanism should provide a sufficient echo loss for all typical environments and typical impulse responses.

When conducting the tests it should checked whether the signal measured is an echo signal and not comfort noise inserted in sending direction in order to mask an echo signal or noise emitted by the loudspeakers. This could be checked e.g. by conducting the idle channel noise measurement with maximum volume control setting.

Note: There may be implementations where echo problems may be observed although the TCLw test gives a high number. In such cases it is recommended to verify the echo performance by subjective tests including different situations which are not addressed in this test.

#### Test

- All tests are conducted in the car cabin, the test arrangement is described in chapter 7.1. The noise level measured at the electrical access point (idle channel noise) shall be less than -63 dBm0. The attenuation between the input of the electrical reference point to the output of the electrical reference point is measured using a speech-like test signal.

- Before the actual measurement a training sequence consisting of 10 seconds of artificial voice (male) and 10 seconds of artificial voice (female) according to ITU-T Recommendation P.50 [12] is inserted. The training sequence level shall be -16 dBm0.

- The test signal is a pn sequence according to ITU-T Recommendation P.501 [13] with a length of 4096 points (48 kHz sampling rate) and a crest factor of 6 dB. The duration of the test signal is 250 ms, the test signal level is -3 dBm0. The low crest factor is achieved by random alternation of the phase between -180 ° and +180 °.

- TCLW is calculated according to ITU-T Recommendation G.122 [7], annex B, clause B.4 (trapezoidal pseudo rule). For the calculation the average measured echo level at each frequency band is referred to the average level of the test signal measured in each frequency band. For the measurement a time window has to be applied which is adapted to the duration of the actual test signal (250 ms).

### Echo Level vs. Time

#### Requirements

This test is intended to verify that the system will maintain sufficient echo attenuation during single talk. The measured echo attenuation during single talk should not decrease by more than 6 dB from the maximum measured during the test

Note 1: The echo path is kept constant during this test, and the test should begin 5 seconds after the initial application of a reference signal such that a steady state converged condition is achieved

Note 2: The analysis is conducted only during the active signal part

#### Test

- The test arrangement is according to chapter 7.1.

- The test signal consists of periodically repeated Composite Source Signal according to ITU-T Recommendation P.501 [13] with an average level of -5 dBm0 as well as an average level of -25 dBm0. The echo signal is analyzed during a period of at least 2.8 s which represents 8 periods of the CS signal. The integration time for the level analysis shall be 35 ms, the analysis is referred to the level analysis of the reference signal. In addition the test is repeated with artificial voice according to ITU-T Recommendation P.50. One sequence of male and one sequence of female voice is used. The average test signal level is -16 dBm0. The echo signal is analyzed during the complete test signal. The integration time for the level analysis shall be 35 ms, the analysis is referred to the level analysis of the reference signal.

- The measurement result is displayed as attenuation vs. time. The exact synchronization between input and output signal has to be guaranteed.

Note: When testing using CSS the analysis is conducted only during the active signal part, the pauses between the Composite Source Signals are not analyzed. The analysis time is reduced by the integration time of the level analysis (35 ms).

### Spectral Echo Attenuation

#### Requirements

The echo attenuation vs. frequency shall be below the tolerance mask given in Table 11.3.

Table 11.3: Spectral Echo Attenuation Mask

| Frequency (Hz) | Upper Limit |
| --- | --- |
| 100 Hz | -41 dB |
| 1 300 Hz | -41 dB |
| 3 450 Hz | -46 dB |
| 5 200 Hz | -46 dB |
| 7 500 Hz | -37 dB |
| 8 000 Hz | -37 dB |
| Note 1: All sensitivity values are expressed in dB on an arbitrary scale. Note 2: The limit at intermediate frequencies lies on a straight line drawn between the given values on a log (frequency) – linear (dB) scale. |  |

During the measurement it should be ensured that the measured signal is really the echo signal and not the Comfort Noise which possibly may be inserted in sending direction in order to mask the echo signal.

Note: This requirement should be fulfilled at any point in time. Therefore it should be verified at different time intervals of the test sequence.

#### Test

- The test arrangement is according to chapter 7.1.

- Before the actual measurement a training sequence is fed in consisting of 10 seconds CS-Signal according to ITU-T Recommendation P.501 [13]. The level of the training sequence is -16 dBm0.

- The test signal consists of a periodically repeated Composite Source Signal. The measurement is carried out under steady-state conditions. The average test signal level is -16 dBm0, averaged over the complete test signal. 4 CS-Signals including the pauses are used for the measurement which results in a test sequence length of 1.4 s. The power density spectrum of the measured echo signal is referred to the power density spectrum of the original test signal. The analysis is conducted using FFT with 8 k points (48 kHz sampling rate or equivalent , rectangular window).

- The spectral echo attenuation is analyzed in the frequency domain in dB.

### Initial Convergence without Background Noise

#### Requirements

The initial convergence (echo attenuation vs. time) during single talk immediately after activating the hands-free terminal with maximum volume control setting should conform to the requirement in Figure 11.3.

Figure 11.3: 	Initial convergence, ERL vs. time

#### Test

- The test arrangement is described in Ch. 6.1. The noise level measured at the electrical access point (idle channel noise) shall be less than -63 dBm0.

- The test signal is applied immediately after setting up the call and setting the volume control to its maximum.

- The test signal is a Composite Source Signal according to ITU-T Recommendation P.501 [13] repeated periodically. The average signal level is -16 dBm0. The echo signal is analyzed over a period of at least 5s. The analysis integration time is 35 ms, the analysis is referred to the level analysis of the reference signal. In addition the test is repeated with artificial voice according to ITU-T Recommendation P.50. One sequence of male and a second one consisting of female voice are used. The starting point of each signal is as defined by the start of the sequence provided by the ITU-T in Recommendation P.50. The average test signal level is -16 dBm0. The echo signal is analyzed over a period of at least 5s. The analysis integration time is 35 ms, the analysis is referred to the level analysis of the reference signal.

- The measurement is displayed as echo attenuation vs. time, measured signal and reference signal have to be synchronized in time.

Note 1: The analysis of the CSS is performed only on the active signal parts, the pauses between the bursts of the Composite Source Signal are not analyzed. The analysis time is reduced by the time constant of the level analysis due to the integration time of 35 ms.

Note 2: The required performance for artificial voice signals should be achieved for different starting points of the artificial voice signal.

### Initial Convergence with Background Noise

#### Requirements

The initial convergence (echo attenuation vs. time) during single talk immediately after activating the hands-free terminal with background noise and with maximum volume control setting should conform to the requirement in Figure 11.4.

L/dB

t/ms

Figure 11.4: Initial convergence with background noise, requirement on echo signal level vs. time

#### Test

- The test arrangement is described in Ch. 6.1.

- The background noise defined by the manufacturer/test house is played back at least 5 s before the start of the actual measurement. This allows time for some adaptive algorithms in the hands-free unit which are constantly monitoring the microphone signal to stabilize, (e.g. AGC, NR). The test is conducted under simulated constant driving conditions.

- The test signal is applied immediately after setting up the call and setting the volume control to its maximum.

- The test signal is a Composite Source Signal according to ITU-T Recommendation P.501 [13] repeated periodically. The average signal level is -16 dBm0. The echo signal is analyzed over a period of at least 5s. The analysis integration time is 35 ms . In addition the test is repeated with artificial voice according to ITU-T Recommendation P.50. One sequence of male and a second one consisting of female voice are used. The starting point of each signal is as defined by the start of the sequence provided by the ITU-T in Recommendation P.50. The average test signal level is -16 dBm0. The echo signal is analyzed over a period of at least 5s. The analysis integration time is 35 ms.

- The measurement is displayed as echo attenuation vs. time.

Note 1: The analysis of the CSS is performed only on the active signal parts, the pauses between the bursts of the Composite Source Signal are not analyzed. The analysis time is reduced by the time constant of the level analysis due to the integration time of 35 ms.

Note 2: The required performance for artificial voice signals should be achieved for different starting points of the artificial voice signal.

### Echo Performance with Time Variant Echo Path

#### Requirements

This test is intended to verify that the system will maintain sufficient echo attenuation during time variant echo path. The measured echo attenuation measured with a time varying echo path should not decrease by more than 6 dB from the maximum measured during the test.

The time variant echo path is realized e.g. by a piece of  cardboard, wood or plastics with a size of 30cm x 40 cm which is positioned on the co-drivers seat and which is constantly moving from 0 (cardboard plain on the co-drivers seat) to 90 (mostly parallel to the backrest) and back to 0 with a rate of 90/s or by quietly open/close a door during the measurement.

#### Test

- Before conducting the test the echo canceller should be fully converged.

- The test arrangement is according to chapter 7.1.

- Before starting the measurement the movement of the cardboard is switched on.

- The test signal consists of periodically repeated Composite Source Signal according to ITU-T Recommendation P.501 [13] with an average level of -5 dBm0 as well as an average level of -25 dBm0. The echo signal is analyzed during a period of at least 2.8 s which represents 8 periods of the CS-Signal. The integration time for the level analysis shall be 35 ms, the analysis is referred to the level analysis of the reference signal. In addition the test is repeated with artificial voice according to ITU-T Recommendation P.50. A sequence of male and female voice is used. The average test signal level is -16 dBm0. The echo signal is analyzed during the complete test signal. The integration time for the level analysis shall be 35 ms, the analysis is referred to the level analysis of the reference signal.

- The measurement result is displayed as attenuation vs. time. The exact synchronization between input and output signal has to be guaranteed.

Note: When using the CSS the analysis is conducted only during the active signal part, the pauses between the Composite Source Signals are not analyzed. The analysis time is reduced by the integration time of the level analysis (35 ms).

### Switching Characteristics

#### Activation in Sending Direction

The activation in sending direction is mainly determined by the built-up time Tr,S,min and the minimum activation level (LS,min). The minimum activation level is the level required to remove the inserted attenuation in sending direction during idle mode. The built-up time is determined for the test signal burst which is applied with the minimum activation level.

The activation level described in the following is always referred to the test signal level at the mouth reference point (MRP).

##### Requirements

The minimum activation level Ls,min should be  -20 dBPa.

The built-up time Tr,S,min (measured with minimum activation level) should be  50 ms.

##### Test

The structure of the test signal is shown in figure 11.5.. The test signal consists of CSS components according to ITU-T Recommendation P.501 [13] with increasing level for each CSS burst.

Figure 11.5:	Test signal to determine the minimum activation level and the built-up time

The settings of the test signal are as follows:

Table 11.4: Settings of the CSS in Sending

|  | CSS Duration/ Pause Duration | Level of the first CS Signal (active Signal Part at the MRP) | Level Difference between two Periods of the Test Signal |
| --- | --- | --- | --- |
| CSS to Determine Switching Characteristic in Sending Direction | 248.62 ms / 451.38 ms | -23 dBPa *) | 1 dB |

*) Note1: The level of the active signal part corresponds to an average level of  -24.7 dBPa at the MRP for the CSS according to ITU-T Recommendation P. 501 assuming a pause of 101.38 ms.

Note 2: When testing speakerphone hands-free system the signal level is corrected at the HATS-HFRP.

It is assumed that the pause length of 451.38 ms is longer than the hang-over time so that the test object is back to idle mode after each CSS burst.

- The test arrangement is described in chapter 7.1.

- The level of the transmitted signal is measured at the electrical reference point. The measured signal level is referred to the test signal level and displayed vs. time. The levels are calculated from the time domain using an integration time of 5 ms.

- The minimum activation level is determined from the CSS burst which indicates the first activation of the test object. The time between the beginning of the CSS burst and the complete activation of the test object is measured.

Note: If the measurement using the CS-Signal does not allow to clearly identify the minimum activation level, the measurement may be repeated by using a one syllable word instead of the CS-Signal. The word used should be of similar duration, the average level of the word must be adapted to the CS-Signal level of the according CSS-burst.

#### Activation in Receiving Direction

The activation in receiving direction is mainly determined by the built-up time Tr,R,min and the minimum activation level (LR,min). The minimum activation level is the level required to remove completely any attenuation inserted during the idle mode. The built-up time is determined from the level variation of the transmitted test signal which is applied with a minimum activation level.

The activation level described in the following is always referred to the test signal level at the electrical reference point (POI).

In order to guarantee a higher accuracy when recording the transmitted signal in receiving direction a measurement microphone is used for this test and positioned close to the loudspeaker of the hands-free terminal.

##### Requirements

The minimum activation level LR,min should be  -35.7 dBm0 (measured during the active signal part).

The built-up time Tr,R,min (measured with minimum activation level) should be  50 ms.

##### Test

The signal construction is shown in figure 11.5. The test signal settings are as follows:

Table 11.5: Settings of the CSS in Receiving

|  | CSS Duration/ Pause Duration | Level of the first CS Signal (active Signal Part at the POI) | Level Difference between two Periods of the Test Signals |
| --- | --- | --- | --- |
| CSS to Determine Switching Characteristics in Receiving Direction | 248.62 ms / 451.38 ms | -38.7 dBm0 *) | 1 dB |

*) Note: The level of the active signal part corresponds to an average level of -40 dBm0 at the POI for the CSS according to ITU-T Recommendation P. 501 assuming a pause of 101.38 ms.

- The test arrangement is according to chapter 7.1.

- The transmitted signal is recorded by a microphone positioned close to the loudspeaker. For headset hands-free terminals the sound pressure measured at the DRP and corrected to the ERP according to ITU-T Recommendation P.57 [16]. The measured signal level is referred to the test signal level and displayed vs. time. The integration time of the level analysis used should be 5 ms.

- The minimum activation level is determined from the CSS burst indicating the first activation of the test object. The duration between beginning of this CSS burst and the complete activation of the terminal is measured.

Note: If the measurement using the CS-Signal does not allow to clearly identify the minimum activation level, the measurement may be repeated by using a one syllable word instead of the CS-Signal. The word used should be of similar duration, the average level of the word must be adapted to the CS-Signal level of the according CSS-burst.

#### Attenuation Range in Sending Direction

The attenuation range in sending direction is determined by applying the test signal in sending direction after the terminal was activated in receiving direction. During the measurement the attenuation range in sending direction (AH,s) and the built-up time in sending direction (Tr,S) is determined.

##### Requirements

The attenuation range AH,S should be less than 20 dB.

The built-up time Tr,S should be less than 50 ms. It is recommended to reduce the attenuation within 15 ms to at least 13 dB below the final value.

##### Test

The structure of the test signals is shown in figure 11.6. It consists of periodically repeated Composite Source Signal bursts used for activating the receiving direction and the voiced sound used to measure the sending direction.

Figure11.6:	Structure of the test signal for measuring the attenuation range

The test signals are synchronized in time at the acoustical interface. The delay of the test arrangement should be constant during the measurement.

##### Test

- The test arrangement is according to chapter 7.1.

- The test signal used is according to figure 11.6, the receiving direction is activated first. The measurement parameters are as follows:

Table 11.6: Signal levels for double talk tests in Sending and Receiving

|  | Receiving Direction | Sending Direction (at the MRP ) |
| --- | --- | --- |
| Average Signal Level (including 101.38 ms Pauses) | -16 dBm0 | --- |
| Active Signal Part | -14.7 dBm0 | -3 dBPa |

The level in receiving direction is determined at the electrical reference point.

- The level is determined as level vs. time calculated from the time domain. The integration time of the levels analysis is 5 ms. The attenuation range is determined by calculating the difference between the measured level between the beginning of the test signal in sending direction (t1 in figure 11.6) until complete activation in sending direction.

#### Attenuation Range in Receiving Direction

The attenuation range in receiving direction is determined after the terminal was activated in sending direction before. During the measurement the attenuation range in receiving direction (AH,R) as well as the built-up time in receiving direction (Tr,R) are determined.

##### Requirements

The attenuation AH,R should be less than 15 dB.

The built-up time Tr,R should be less than 50 ms. It is recommended to reduce the attenuation within 15 ms to less than 9 dB.

##### Test

The structure of the test signal is shown in figure 11.6. Again CSS bursts are used for activating the opposite direction (now sending direction) and the voiced sound is used to measure the receiving direction. The test signals are synchronized in time at the acoustical interface. The delay of the test arrangement should be constant during the measurement.

- The test arrangement is according to chapter 7.1.

- The test signal shown in figure 11.6 is used, the sending direction is activated first.

The measurement parameters are as follows:

Table 11.7: Signal levels for double talk tests in Sending and Receiving

|  | Receiving Direction | Sending Direction (at the MRP ) |
| --- | --- | --- |
| Average Level (including 101.38 ms Pauses) | --- | -4.7 dBPa |
| Active Signal Part | -14.7 dBm0 | -3 dBPa |

The level in receiving direction is determined at the electrical reference point.

- The level is determined as level vs. time calculated from the time domain. The integration time of the level analysis is 5 ms. The attenuation is determined by calculating the difference between the beginning of the measured test signal in receiving direction (t1 in figure 11.6) and the complete activation in receiving direction.

## Doubletalk Performance

Note: Before starting the double talk tests the test lab should ensure that the echo canceller is fully converged. This can be done by an appropriate training sequence.

During double talk the speech is mainly determined by 2 parameters: impairment caused by echo during double talk and level variation between single and double talk (attenuation range).

In order to guarantee sufficient quality under double talk conditions the Talker Echo Loudness Rating should be high and the attenuation inserted should be as low as possible. Terminals which do not allow double talk in any case should provide a good echo attenuation which is realized by a high attenuation range in this case.

The most important parameters determining the speech quality during double talk are (see ITU-T Recommendations P.340 [9] and P.502 [14]):

- Attenuation range in sending direction during double talk AH,S,dt

- Attenuation range in receiving direction during double talk AH,R,dt

- Echo attenuation during double talk

### Attenuation Range in Sending Direction during Double Talk: AH,S,dt

#### Requirements

Based on the level variation in sending direction during double talk AH,S,dt the behavior of hands-free terminals can be classified according to table 11.8.

Table 11.8: Categorization of double talk capability according to ITU-T P.340

| Category (according to ITU-T P.340) |  | 1 |  | 2a | 2b | 2c |  | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Full Duplex Capability |  | Partial Duplex Capability |  |  |  | No Duplex Capability |  |
| AH,S,dt [dB] |  | £3 |  | £6 | £9 | £12 |  | >12 |

The requirements apply for nominal and maximum setting of the receive volume control.

The requirements apply for nominal signal levels in sending and receiving direction as well as for the level combinations +6 dB (re. nominal level) in sending/-6 dB (re. nominal level) in receiving and +6 dB (re. nominal level) in receiving/-6 dB (re. nominal level) in sending. Furthermore the test is conducted with nominal levels but with maximum setting of the volume control.

Note: If the maximum setting of the volume control is chosen such that nonlinearities occur in the echo path the double talk performance will decrease.

In general table 11.8 provides a quality classification of terminals regarding double talk performance. However, this does not mean that a terminal which is category 1 based on the double talk performance is of high quality concerning the overall quality as well.

#### Test

The test signal to determine the attenuation range during double talk is shown in figure 11.7. A sequence of uncorrelated CS-Signals is used which is inserted in parallel in sending and receiving direction

s(t) - Signal for one Direction

sdt(t) - Double Talk Signal

Signal timing for attenuation range in sending

.

Figure 11.7:	Double Talk Test Sequence with overlapping CS-Signals in sending and receiving direction

Figure 11.7 indicates that the sequences overlap partially. The beginning of the CSS sequence (voiced sound, black) is overlapped by the end of the pn-sequence (white) of the opposite direction. During the active signal parts of one signal the analysis can be conducted in sending and receiving direction. The analysis times are shown in figure 11.7 as well. The test signals are synchronized in time at the acoustical interface. The delay of the test arrangement should be constant during the measurement.

The settings for the test signals are as follows:

Table 11.9: Timing of the double talk sequences

|  | Receiving Direction | Sending Direction |
| --- | --- | --- |
| Voiced Sound | 69.92 ms** | 72.69 ms* |
| Pseudo Random Noise sequence/Noise Sequence | 200 ms | 200 ms |
| Pause Length between two Signal Bursts | 130.08 ms | 127.31 ms |
| .Average Signal Level (Assuming an Original Pause length of 101.38 ms) | -16 dBm0 | -4.7 dBPa |
| Active Signal Parts | -14.7 dBm0 | -3 dBPa |

* 14 repetitions of the voiced sound for double talk according to ITU-T P.501 [13]

** 23 repetitions of the voiced sound according to ITU-T P.501 [13]

The tests are repeated with maximum volume control setting in Receiving direction.

- The test arrangement is according to chapter 7.1. Before the actual test a training sequence for the echo canceller consisting of 10 s male and 10 s female voice according to ITU-T P.50 [12] with a level of -16 dBm0 is applied to the electrical reference point.

- When determining the attenuation range in sending direction the signal measured at the electrical reference point is referred to the test signal inserted.

- The level is determined as level vs. time from the time domain. The integration time of the level analysis is 5 ms. The attenuation is determined from the level difference measured at the beginning of the double talk always with the beginning of the CS-Signal in sending direction until its complete activation (during the pause in the receiving channel). The analysis is performed over the complete signal starting with the second CS-Signal. The first CS-Signal is not used for the analysis.

- The test is repeated for all level combinations as defined in the requirements.

### Attenuation Range in Receiving Direction during Double Talk: AH,R,dt

To ensure a higher accuracy measuring the transmitted signal in receiving direction a measurement microphone is used which is positioned as close as possible to the loudspeaker of the hands-free terminal.

#### Requirements

Based on the level variation in receiving direction during double talk AH,R,dt the behavior of the hands-free terminal can be classified according to the table 11.10.

Table 11.10: Categorization of double talk capability according to ITU-T P.340

| Category (according to ITU-T P.340) | 1 |  | 2a |  | 2b | 2c | 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Full Duplex Capability |  | Partial Duplex Capability |  |  |  | No Duplex Capability |
| AH,R,dt [dB] | £3 |  | £5 |  | £8 | £10 | >10 |  |

The tests are repeated with maximum volume control setting in Receiving direction.

In general table 11.10 provides a quality classification of terminals regarding double talk performance. However, this does not mean that a terminal which is category 1 based on the double talk performance is of high quality concerning the overall quality as well.

#### Test

The test signal to determine the attenuation range during double talk is shown in figure 11.7. A sequence of uncorrelated CSSignals is used which is inserted in parallel in sending and receiving direction. The test signals are synchronized in time at the acoustical interface. The delay of the test arrangement should be constant during the measurement.

The settings for the test signals are as follows:

Table 11.11: Timing of the double talk sequences

|  | Receiving Direction | Sending Direction |
| --- | --- | --- |
| Voiced Sound | 69.92 ms | 72.69 ms |
| Pseudo Random Noise sequence | 200 ms | 200 ms |
| Pause Length between two Signal Bursts | 130.08 ms | 127.31 ms |
| Average Signal Level (Assuming an Original pause Length of 101.38 ms) | -16 dBm0 | -4.7 dBPa |
| Active Signal Parts | -14.7 dBm0 | -3 dBPa |

The tests are repeated with maximum volume control setting in Receiving direction.

- The test arrangement is according to chapter 7.1.

- When determining the attenuation range in receiving direction the signal measured at the loudspeaker of the hands-free terminal is referred to the test signal inserted. For headset hands-free terminals the sound pressure is measured at the DRP and corrected to the ERP according to ITU-T Recommendation P.57 [16].

- The level is determined as level vs. time from the time domain. The integration time of the level analysis is 5 ms. The attenuation is determined from the level difference measured at the beginning of the double talk always with the beginning of the CS-Signal in receiving direction until its complete activation (during the pause in the sending channel). The analysis is performed over the complete signal starting with the second CS-Signal. The first CS-Signal is not used for the analysis.

- The test is repeated for all level combinations as defined in the requirements.

### Detection of Echo Components during Double Talk

#### Requirements

The echo attenuation during double talk is based on the parameter Talker Echo Loudness Rating (TELRdt). It is assumed that the terminal at the opposite end of the connection provides nominal Loudness Rating (SLR + RLR = 10 dB). “Echo Loss” is the echo suppression provided by the hands-free terminal measured at the electrical reference point. Under these conditions the requirements given in table 11.12 below are applicable (more information can be found in annex A of the ITU-T Recommendation P.340 [9]).

Table 11.12: Categorization of double talk capability according to ITU-T P.340

| Category (according to ITU-T P.340) | 1 | 2a | 2b | 2c | 3 |
| --- | --- | --- | --- | --- | --- |
|  | Full Duplex Capability | Partial Duplex Capability |  |  | No Duplex Capability |
| Echo Loss [dB] |  27 |  23 |  17 |  11 | < 11 |

#### Test

- The test arrangement is according to chapter 7.1.

- The double talk signal consists of a sequence of orthogonal signals which are realized by voice-like modulated sine waves spectrally shaped similar to speech. The measurement signals used are shown in the figure 11.8 below. A detailed description can be found in ITU-T Recommendation P.501 [13].

The signals are fed simultaneously in sending and receiving direction. The level in sending direction is –4.7 dBPa at the MRP (nominal level), the level in receiving direction is  -16 dBm0 at the electrical reference point (nominal level).

sFM1,2(t) =AFM1,2∗cos(2ptn∗F01,2) ;		n= 1,2,....

sAM1,2(t) = AAM1,2∗cos(2πtFAM1,2) ;

The settings for the signals are as follows:

Receiving Direction				Sending Direction

__________________________________________________________________________

fm[Hz]			fmod(fm)[Hz]	Fam[Hz]		fm[Hz]		fmod(fm)[Hz]	Fam[Hz]

250			 5		3		270		 5		3

500			 10		3		540		 10		3

750			 15		3		810		 15		3

1000			 20		3		1080		 20		3

1250			 25		3		1350		 25		3

1500			 30		3		1620		 30		3

1750			 35		3		1890		 35		3

2000			 40		3		2160		 35		3

2250			 40		3		2400		 35		3

2500			 40		3		2900		 35		3

2750			 40		3		3150		 35		3

3000			 40		3		3400		 35		3

3250			 40		3		3650		 35		3

3500			 40		3		3900		 35		3

3750			 40		3

Parameters of the Shaping Filter: Low Pass Filter, 5 dB/oct.

Figure 11.8:	 Parameters of the two test signals for double talk measurement based on AM-FM modulated sine waves

- The test signal is measured at the electrical reference point (sending direction). The measured signal consists of the double talk signal which was fed in by the artificial mouth and the echo signal. The echo signal is filtered by comb filter using mid-frequencies and bandwidth according to the signal components of the signal in receiving direction (see ITU-T Recommendation P.501 [13]). The filter will suppress frequency components of the double talk signal.

- In each frequency band which is used in receiving direction the echo attenuation can be measured separately. The requirement for category 1 is fulfilled if in any frequency band the echo signal is either below the signal noise or below the required limit. If echo components are detectable, the classification is based on table 11.12 above. The echo attenuation is to be achieved for each individual frequency band from 200 Hz to 3450 Hz according to the different categories.

### Sent Speech Attenuation during Double Talk

#### Requirements

The sent speech attenuation during double talk is based on the parameter AH,S,dt.

Based on the level variation in sending direction during double talk AH,S,dt the behavior of hands-free terminals can be classified according to the following table.

Table 11.13: Categorization of double talk capability according to ITU-T P.340

| Category (according to ITU-T P.340) |  | 1 |  | 2a | 2b | 2c |  | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Full Duplex Capability |  | Partial Duplex Capability |  |  |  | No Duplex Capability |  |
| AH,S,dt [dB] |  | £3 |  | £6 | £9 | £12 |  | >12 |

The requirements apply for nominal and maximum setting of the receive volume control.

The requirements apply for nominal signal levels in sending and receiving direction as well as for the level combinations +6 dB (re. nominal level) in sending/-6 dB (re. nominal level) in receiving and +6 dB (re. nominal level) in receiving/-6 dB (re. nominal level) in sending. Furthermore the test is conducted with nominal levels but with maximum setting of the volume control.

In general table 11.13 provides a quality classification of terminals regarding double talk performance. However, this does not mean that a terminal which is category 1 based on the double talk performance is of high quality concerning the overall quality as well.

The test is conducted in addition to the test described in 11.11.1 in order to guaranteed that no switching device with short reaction time is classified as a duplex or partially duplex system.

#### Test

- The test arrangement is according to chapter 7.1.

- The double talk signal consists of a sequence of orthogonal signals which are realized by voice-like modulated sine waves spectrally shaped similar to speech. The measurement signals used are shown in the figure 11.9 below. A detailed description can be found in ITU-T Recommendation P.501 [13].

The signals are fed simultaneously in sending and receiving direction. The level in sending direction is –4.7 dBPa at the MRP (nominal level), the level in receiving direction is  -16 dBm0 at the electrical reference point (nominal level).

sFM1,2(t) =AFM1,2∗cos(2ptn∗F01,2) ;		n= 1,2,....

sAM1,2(t) = AAM1,2∗cos(2πtFAM1,2) ;

The settings for the signals are given as follows:

Receiving Direction				Sending Direction

__________________________________________________________________________

fm[Hz]			fmod(fm)[Hz]	Fam[Hz]		fm[Hz]		fmod(fm)[Hz]	Fam[Hz]

250			 5		3		270		 5		3

500			 10		3		540		 10		3

750			 15		3		810		 15		3

1000			 20		3		1080		 20		3

1250			 25		3		1350		 25		3

1500			 30		3		1620		 30		3

1750			 35		3		1890		 35		3

2000			 40		3		2160		 35		3

2250			 40		3		2400		 35		3

2500			 40		3		2900		 35		3

2750			 40		3		3150		 35		3

3000			 40		3		3400		 35		3

3250			 40		3		3650		 35		3

3500			 40		3		3900		 35		3

3750			 40		3

Parameters of the Shaping Filter: Low Pass Filter, 5 dB/oct.

Figure 11.9:	 Parameters of the two test signals for double talk measurement based on AM-FM modulated sine waves

- The test signal is measured at the electrical reference point (sending direction). The measured signal consists of the double talk signal which was fed in by the artificial mouth and the echo signal. The double talk signal (send signal) is filtered by comb filter using mid-frequencies and bandwidth according to the signal components of the signal in sending direction (see ITU-T Recommendation P.501 [13]). The filter will suppress frequency components of the echo signal.

- In each frequency band which is used in sending direction the sent speech attenuation AH,S,dt can be measured separately. The requirement for category 1 is fulfilled if in each frequency band the attenuation of the signal in sending direction is below the required limit. If attenuation is detectable, the classification is based on table 11.13 above. The sent speech attenuation AH,S,dt is to be achieved for each individual frequency band from 200 Hz to 3550 Hz according to the different categories.

- The test is repeated for all level combinations as defined in the requirements.

## Background Noise Transmission

### Ambient Noise Rejection

#### Requirements

The efficiency of noise reduction is calculated as D value from DELSM according to ITU-T Recommendation G.111. The result shall be higher than –13 dB. Note that a D value higher than 0 dB is recommended.

#### Test

- The test arrangement is according to chapter 7.1.

- According to specifications of manufacturer/test lab a realistic background noise is played back. The noise scenario should represent a constant driving situation. Before starting the actual measurement the background noise should be applied for at least 5 seconds in order to allow adaptation of the noise reduction algorithms.

- The signal in sending direction is recorded at the electrical reference point (POI). The measurements are carried out in third octave according to IEC 60120 [4] in the frequency bands 4 - 17 (200 Hz - 4 kHz). In each band the diffuse field sensitivity Ssi(diff) is measured. The result is expressed in dBV/Pa.

- The near end speech is simulated by applying a Composite Source Signal according to ITU-T Recommendation P.501 [13] with a duration of  2 CSS periods. The level is adjusted to +1.3 dBPa at the MRP in order to take into account the Lombard Effect. The direct sound sensitivity Ssi(direct) is also calculated in third octave bands according to IEC 60120 [4] in the frequency bands 4 - 17 (200 Hz - 4 kHz). The result is expressed in dBV/Pa.

- The D value is calculated according to ITU-T Recommendation P.79 [20], annex E, formula E2 and E3 in the frequency bands 4 - 17. The coefficients Ki are used according to table E1.

### SNR Improvement provided by the HFT algorithm

#### Requirements

The SNR improvement provided by the HFT algorithm should be ≥ 6 dB.

This test is applicable only for systems where the noise canceling function of the hands-free system can be deactivated.

#### Test

- The test arrangement is according to chapter 7.1. The noise canceling function of the hands-free system is deactivated.

- Depending on the manufacturer/test lab requirements the background noise - which represents a typical driving condition - is inserted. The test shall be conducted under constant background noise conditions. The background noise has to be inserted at minimum 5 seconds before the actual measurement starts in order to allow adaptation of possible background noise reduction algorithms.

- The sending signal (background noise only) is recorded at the electrical reference point (POI). The output level LBGNncoff is measured between 100 Hz and 8kHz. The result is scaled in dBV/Pa.

- The speech signal is simulated by a Composite Source Signal according to ITU-T Recommendation P.501 [13] with a duration of  2 CS sequences. The output level LSncoff is measured between 100 Hz and 8kHz. The result is scaled in dBV/Pa.

- The SNRncoff is determined by SNRncoff=LSncoff-LBGNncoff.

- Steps 1 – 4 are repeated with the hands-free noise canceling function activated. The measured levels are LBGNncon and LSncon.

- The SNRncon is determined by SNRncon=LSncon-LBGNncon.

- The SNR improvement is calculated by SNRncon-SNRncoff

### Background Noise Transmission after Call Setup

#### Requirements

The analysis based on the Relative Approach [25] (see Annex B) should not indicate remarkable characteristics exceeding 6 cp/cPa. The first transmitted signal peak in sending direction should not cause higher excitation than 15 cp/cPa between 300 Hz and 3.4 kHz.

#### Test

- The test arrangement is given in chapter 7.1.

- According to the specification of the manufacturer/test lab the background noise is played back. The test should be carried out during a constant driving situation.

- The terminal is switched off and on again (to provide a reset) and a call established by the system simulation. The incoming call is answered at the terminal. Special care should be taken not to produce any disturbances or unwanted noise by touching the terminals housing while answering the incoming call.

- The transmitted signal in sending direction is recorded at the POI starting at least 1 s before the call is answered and for at least 7 s after the call is established. The analysis range is chosen to 8 s including an initial pause of 1 s before the call was established.

- The recorded signal is analyzed using the Relative Approach [25].

### Speech Quality in the Presence of Background Noise

#### Requirements

Speech Quality for wideband systems can be tested based on ETSI EG 202 396-3 [xx]. The test method is applicable for wideband transmission systems. The test method described leads to three MOS-LQO quality numbers:

N-MOS-LQOw: 	Transmission quality of the background noise

S-MOS-LQOw: 	Transmission quality of the speech

G-MOS-LQOw:		Overall transmission quality

According to specifications of manufacturer/test lab a realistic background noise is played back  For the background noises chosen  the following requirements apply:

Background noises for driving speed ≤ 80 km/h:

N-MOS-LQOw ≥ 3.0

S-MOS-LQOw ≥ 3.0

G-MOS-LQOw ≥ 3.0

Background noises for driving speed ≤ 130 km/h:

N-MOS-LQOw ≥ 2.5

S-MOS-LQOw ≥ 2.5

G-MOS-LQOw ≥ 2.5

NOTE: 	It is recommended to test the terminal performance with different types of background eg. open window, different types of road surfaces and other relevant conditions. Esp time variant conditions should be taken into account.

#### Test

- The test arrangement is given in chapter 7.1.

- According to the specification of the manufacturer/test lab the background noise is played back.  The background noise should be applied for at least 5 s in order to adapt noise reduction algorithms in advance the test.

- The near end speech signal consists of  8 sentences of speech (2 male and 2 female talkers, 2 sentences each). Appropriate speech samples can be found in ITU T Recommendation P.501 [XX]. The preferred language is French since the objective method was validated with French language. The test signal level is  4,7 dBPa at the MRP. Three signals are required for the tests:

- The clean speech signal is used as the undisturbed reference (see [xx]).

- The speech plus undisturbed background noise signal is recorded at the terminal’s microphone position using an omni directional measurement microphone with a linear frequency response between 50Hz and 12 kHz.

- The sending signal is recorded at the electrical reference point.

N-MOS-LQOw, S-MOS LQOw and G-MOS LQOw are calculated as described in EG 202 396-3 [xx].

### Quality of Background Noise Transmission (with Far End Speech)

#### Requirements

The test is carried out applying the Composite Source Signal in receiving direction. During and after the end of Composite Source Signal bursts (representing the end of far end speech simulation) the signal level in sending direction should not vary more than 10 dB (during transition to transmission of background noise without far end speech).

#### Test

- The test arrangement is according to chapter 7.1.

- According to the specification of the manufacturer/test lab the background noise is played back. The test should be carried out during a constant driving situation.

- First the measurement is conducted without inserting the signal at the far end. At least 10 of noise are recorded. The background signal level versus time is calculated using a time constant of 35 ms. This is the reference signal.

- In a second step the same measurement is conducted but with inserting the CS-Signal at the far end. The exactly identical background noise signal is applied. The background noise signal must start at the same point in time which was used for the measurement without far end signal. The background noise should be applied for at least 5 seconds in order to allow adaptation of the noise reduction algorithms. After at least 5 seconds a Composite Source Signal according to ITU-T Recommendation P.501 [13] is applied in receiving direction with a duration of  2 CSS periods. The test signal level is –16 dBm0 at the electrical reference point.

- The sending signal is recorded at the electrical reference point. The test signal level versus time is calculated using a time constant of 35 ms.

- The level variation in sending direction is determined during the time interval when the CS-Signal is applied and after it stops. The level difference is determined from the difference of the recorded signal levels vs. time between reference signal and the signal measured with far end signal.

### Quality of Background Noise Transmission (with near End Speech)

#### Requirements

The test is carried out applying a simulated speech signal in sending direction. During and after the end of the simulated speech signal (Composite Source Signal bursts) the signal level in sending direction should not vary more than 10 dB.

#### Test

- The test arrangement is according to chapter 7.1.

- According to the specification of the manufacturer/test lab the background noise is played back. The test should be carried out during a constant driving situation. The background noise should be applied for at least 5 s in order to allow adaptation of the noise reduction algorithms.

- The near end speech is simulated using the Composite Source Signal according to ITU-T Recommendation P.501 [13] with a duration of  2 CSS periods. The test signal level is 
–4.7 dBPa at the MRP. For speakerphone hands-free systems the HATS-HFRP correction has to be applied.

- The sending signal is recorded at the electrical reference point. The test signal level versus time is calculated using a time constant of 35 ms.

- First the measurement is conducted without inserting the signal at the near end. The signal level is analyzed vs. time. In a second step the same measurement is conducted but with inserting the CS-Signal at the near end. The level variation is determined by the difference between the background noise signal level without inserting the CS-Signal and the maximum level of the noise signal during and after the CSS-bursts in sending direction.

### "Comfort Noise" Injection

This section is applicable only if comfort noise is inserted by the hands-free unit.

Requirements

- The level of comfort noise is adjusted in a range of +2 and –5 dB to the original (transmitted) background noise. The noise level is calculated with psophometric weighting.

- The spectral difference between comfort noise and original (transmitted) background noise shall be in between the mask given through straight lines between the breaking points on a logarithmic (frequency) - linear (dB sensitivity) scale as given in table 11.14.

Table 11.14: Requirements for Spectral Adjustment of Comfort Noise (Mask)

| Frequency (Hz) | Upper Limit | Lower Limit |
| --- | --- | --- |
| 200 | 12 | -12 |
| 800 | 12 | -12 |
| 800 | 10 | -10 |
| 2000 | 10 | -10 |
| 2000 | 6 | -6 |
| 4000 8000 | 6 6 | -6 -6 |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Test

- The test arrangement is according to chapter 7.1. Background noise is played back.

- The test signal is applied in receiving direction consisting of an initial pause of 10 s and a periodical repetition of the Composite Source Signal in receiving direction (duration  10 s) with nominal level to enable comfort noise injection.

- The transmitted signal is recorded in sending direction at the electrical reference point.

- The power density spectra measured in sending direction during the initial pause of the test signal (8k FFT/48kHz sampling rate or equivalent, averaged over  5 s) is referred to the power density spectrum determined during the period with the periodical repetition of the Composite Source Signals in receiving direction (8k FFT/48kHz sampling rate or equivalent, averaged over  5 s). Spectral differences between both power density spectra are analyzed and compared to the requirements given in table 11.14.

- The level of the transmitted signal in sending direction is determined during the initial pause of the test signal in receiving direction and referred to the level of the transmitted signal in sending direction determined during the application of the test signal in receiving direction. Both levels are calculated using psophometric weighting.

# Verification of the Transmission Performance of Bluetooth Enabled Phones

## Interface definition and calibration

The principle of the Bluetooth testing is shown in Figure 12.1:

Figure 12.1:	Bluetooth test setup

The mobile phone under test is connected to the Bluetooth reference interface at the Bluetooth reference point BTR and to the Network (System) Simulator. The Bluetooth reference interface as well as the network simulator reference interface are calibrated to the electrical inputs and outputs of the test system. While the 0 dBr point is clearly defined for the network reference interface, the calibration of the Bluetooth reference interface in principle is free. However, in order to get a calibrated setup, the following calibration procedure is recommended:

Figure 12.2:	Calibration of the Bluetooth reference interface

The Bluetooth reference interface is calibrated using a mobile phone with known characteristics which does not introduce any amplification or attenuation in the Bluetooth link. No additional signal processing except the Bluetooth coding shall be active. Typically a “reference phone” is selected by evaluating a variety of different phones of different brands and selecting the one which closest matches the requirements with respect to sensitivity and deactivated signal processing.

With the reference mobile phone linked to the system simulator and at the same time connected to the hands free terminal via the BT link, a SLR test is performed according to the specifications outlined in chapter 11.3. The signal level measured at the electrical reference point during this test is the reference signal level, SBTref ,which is to be used when calibrating an arbitrary mobile phone connected to the Bluetooth reference interface..

With an arbitrary mobile phone linked to the system simulator and at the same time connected to the Bluetooth reference interface via the BT link, the level at the Bluetooth reference point is adjusted until the level measured at the electrical reference point reaches the reference signal level. The corresponding signal level at the Bluetooth reference point is SBTsnd. The test method is as outlined in chapter 11.3. However the acoustically calibrated excitation signal – properly corrected for attenuation due to distance between MRP-HFRP – is applied at the Bluetooth reference point rather than the HFRP.

Note: The influence of the frequency response characteristic of the hands-free phone is not considered however the method ensures a similar overall signal level inserted in the Bluetooth link and the phone.

Note: Evaluation boards from Bluetooth chipset vendors may be used for implementation of the Bluetooth reference interface.

### BT Delay in Sending Direction

#### Requirements

The delay in sending direction is measured from the BTR to POI (reference speech codec of the system simulator, output). The delay measured in sending direction  is

TBTs+ tSystem .

TBTs shall be less than 10 ms.

Note 1: The delay TBTs should be minimized!

The system delay tSystem is depending on the transmission method used and the network simulator. The delay tsystem must be known.

#### Test

- For the measurements a Composite Source Signal (CSS) according to ITU-T Recommendation P.501 [13] is used. The pseudo random noise (pn)-part of the CSS has to be longer than the maximum expected delay. It is recommended to use a pn sequence of 16 k samples (with 48 kHz sampling rate). The test signal level is SBTsnd.

The reference signal is the original signal (test signal).

The test setup in correspondence to chapter 12.1.

2.	The delay is determined by cross-correlation analysis between the measured signal at the electrical access point and the original signal. The measurement is corrected by delays which are caused by the test equipment.

- The delay is measured in ms and the maximum of the cross-correlation function is used for the determination.

### BT Delay in Receiving Direction

#### Requirements

The delay in receiving direction is measured from POI (input of the reference speech coder of the system simulators) to the BT reference interface. The delay measured in receiving direction is

TBTr+ tSystem .

TBTr shall be less than 10 ms.

Note 1: The Delay TBTr should be minimized!

The system delay tSystem is depending on the transmission system and on the network simulator used.  The delay tSystem must be known.

#### Test

- For the measurements a Composite Source Signal (CSS) according to ITU-T Recommendation P.501 [13] is used. The pseudo random noise (pn)-part of the CSS has to be longer than the maximum expected delay. It is recommended to use a pn sequence of 16 k samples (with 48 kHz sampling rate). The test signal level is -16 dBm0 at the electrical interface (POI).

The reference signal is the original signal (test signal).

- The test arrangement is according to chapter 12.1.

- The delay is determined by cross-correlation analysis between the measured signal at the BTR and the original signal. The measurement is corrected by delays which are caused by the test equipment.

- The delay is measured in ms and the maximum of the cross-correlation function is used for the determination.

## BT Loudness Ratings

### Requirements

The nominal values of JLR from and to the electrical reference point (POI) should be

JLRBTsnd = 0 ±0.5 dB;

JLRBTrcv = 0 ±0.5 dB

However, it is recognized that different Bluetooth implementations deviate significantly (± 6dB) from this level. In order to maintain a good system performance appropriate corrections have to be integrated in the hands-free implementation to ensure a good system performance. Based on these tests appropriate level corrections can be inserted.

### Test BT Junction Loudness Rating in Sending Direction

- The test signal used for the measurements shall be CSS according to ITU-T Recommendation P.501 [13]. The test signal level is SBTsnd, the level is averaged over the complete test signal.

The measured power density spectrum BT reference interface is used as the reference power-density spectrum for determining the BT sending sensitivity.

- The test arrangement is according to chapter 12.1.. The BT sending sensitivity is calculated from each band of the 14 frequencies given in Table A.2 of ITU-T Recommendation P.79 [20], bands 4 - 17.

For the calculation the average measured level at the electrical reference point for each frequency band is referred to the average test signal level measured in each frequency band at the BT reference interface.

- The sensitivity is expressed in dBV/V, the BT Junction Loudness Rating JLR BTsnd shall be calculated according to ITU-T Recommendation P.79 [20], formula A - 23d, band 4 - 17, M = 0.175 and the weighting factors Wj for JLR according to ITU-T Recommendation P.79 [20], Table A.2.

### Test BT Junction Loudness Rating in Receiving Direction

- The test signal used for the measurements shall be CSS according to ITU-T Recommendation P.501 [13]. The test signal is -16 dBm0, measured at the electrical reference point and averaged over the complete test signal sequence.

- The test arrangement is according to chapter 12.1. For the calculation the averaged level at the BT reference interface is used. The BT receiving sensitivity is determined by the bands 4 - 17 according to Table A.2 of ITU-T Recommendation P.79 [20].

For the calculation the average signal level of each frequency band is referred to the signal level of the reference signal measured in each frequency band.

- The sensitivity is expressed in dBV/V, the BT Junction Loudness Rating in receiving JLRBTrcv shall be calculated according to ITU-T Recommendation P.79 [20], formula A -23d, band 4-17, M = 0.175 and the weighting factors Wj for JLR according to Table A.2 of ITU-T Recommendation P.79 [20].

### BT Linearity in Sending Direction

#### Requirements

The test is aimed to detect any amplitude non-linearities including AGC or companding. For acoustical signal level variation in the range of -40 dB /+5 dB from the nominal signal level SBTsnd the measured JLRBTsnd shall not deviate more than +/- 0.5 dB from the JLRBTsnd measured with the nominal signal level SBTsnd.

#### Test

- The test signal used for the measurements shall be CSS according to ITU-T Recommendation P.501 [13]. The test signals are in the range of -40 dBV to 5 dBV in steps of 5dB relative to the nominal signal level SBTsnd, measured at the BTR. The test signal level is the average level of the complete test signal.

The measured power density spectrum at the BTR is used as the reference power-density spectrum for determining the sending sensitivity.

- The test arrangement is according to chapter 12.1. The BT sending sensitivity is calculated from each band of the 14 frequencies given in Table A.2 of ITU-T Recommendation P.79 [20], bands 4 - 17.

For the calculation the average measured level at the electrical reference point for each frequency band is referred to the average test signal level measured in each frequency band at the BT reference interface.

- The sensitivity is expressed in dBV/V, the BT Junction Loudness Rating JLR BTsnd shall be calculated according to ITU-T Recommendation P.79 [20], formula A - 23d, band 4 - 17, M = 0.175 and the weighting factors in sending direction according to ITU-T Recommendation P.79 [20], Table A.2.

### BT Linearity in Receiving Direction

#### Requirements

The test is aimed to detect any amplitude non-linearities including AGC or companding. For network signal level variations in the range of -40dB to +5 dB relative to the nominal signal level the measured JLRBTrcv shall not deviate more than +/- 0.5 dB from the JLRBTrcv measured with nominal signal level.

#### Test

- The test signal used for the measurements shall be CSS according to ITU-T Recommendation P.501 [13]. The test signals are in the range of -40 dBV to 5 dBV in steps of 5dB relative to the nominal signal level measured at the electrical reference point. The test signal level is the average level of the complete test signal.

The measured power density spectrum at the electrical reference point is used as the reference power-density spectrum for determining the sending sensitivity.

- The test arrangement is according to chapter 12.1. The BT receiving sensitivity is calculated from each band of the 14 frequencies given in Table A.2 of ITU-T Recommendation P.79 [20], bands 4 - 17.

For the calculation the average measured level at the BTR for each frequency band is referred to the average test signal level measured in each frequency band at the electrical reference interface.

- The sensitivity is expressed in dBV/V, the BT Junction Loudness Rating JLR BTsnd shall be calculated according to ITU-T Recommendation P.79 [20], formula A - 23d, band 4 - 17, M = 0.175 and the weighting factors in sending direction according to ITU-T Recommendation P.79 [20], Table A.2.

## BT Sensitivity Frequency Responses

### BT Sending Sensitivity Frequency Response

#### Requirements

The sending sensitivity frequency response is measured from BTR to POI (reference speech codec of the system simulator, output).

The tolerance mask for the sending sensitivity frequency response is shown in Table 12.1, the mask is drawn by straight lines between the breaking points in Table 12.1 on a logarithmic (frequency) - linear (dB sensitivity) scale.

Table 12.1: Tolerance Mask for the BT Sending Sensitivity Frequency Response

| Frequency (Hz) | Upper Limit | Lower Limit |
| --- | --- | --- |
| 200 | 0 | -2 |
| 3 100 | 0 | -2 |
| 3 400 | 0 | -3 |
| Note: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

#### Test

- The test signal used for the measurements shall be artificial voice according to ITU-T Recommendation P.50 [12]. The test signal is SBTsnd, the level is averaged over the complete test signal.

The measured power density spectrum at the BT reference interface is used as the reference power density spectrum for determining the BT sending sensitivity.

- The test arrangement is according to chapter 12.1. The BT sending sensitivity frequency response is determined in third octave bands as given by the R. 40 series of preferred numbers in ISO 3 [6] for frequencies from 100 Hz to 4 kHz inclusive. In each third octave band the level of the measured signal is referred to the level of the reference signal averaged over the complete test sequence length.

- The sensitivity is determined in dBV/V.

### BT Receiving Sensitivity Frequency Response

#### Requirements

The receiving sensitivity frequency response is measured from the electrical reference point (input of the system simulators, POI) to the BT reference interface.

The tolerance mask for the receiving sensitivity frequency response is shown in Table 12.2, the mask is drawn by straight lines between the breaking points in Table 12.2 on a logarithmic (frequency) - linear (dB sensitivity) scale.

Table 12.2: Tolerance Mask for the Receiving Sensitivity Frequency Response

| Frequency (Hz) | Upper Limit | Lower Limit |
| --- | --- | --- |
| 200 | 0 | -2 |
| 3 100 | 0 | -2 |
| 3 400 | 0 | -3 |
| Note: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

#### Test

- The test signal used for the measurements shall be artificial voice according to ITU-T Recommendation P.50 [12]. The test signal is -16 dBm0, measured at the electrical reference point and averaged over the complete test signal sequence.

- The test arrangement is according to chapter 12.1. The BT receiving sensitivity frequency response is determined in third octaves as given by the R. 40 series of the preferred numbers in ISO 3 [6] for frequencies from 100 Hz to 4 kHz inclusive. In each third octave band the level of the measured signal is referred to the level of the reference signal, averaged over the complete test sequence length.

- The sensitivity is determined in dBV/V.

## BT Noise Cancellation Test in Sending

### Requirements

The objective of this test is to check whether no noise cancellation is active in the mobile phone. No acoustic echo control shall be active in the mobile phone The mobile phone noise cancellation is measured from BTR to POI (reference speech codec of the system simulator, output).

The attenuation of the simulated background noise test signal shall not deviate more than 1 dB for all periods of the test signal.

### Test

- The test arrangement is according to chapter 12.1.

- The test signal used for the measurements shall be pink noise with a duration of 5s followed by a pause of 3s, both repeated 3 times. The test signal during the active parts of the signal is SBTsnd, the level is averaged over the active parts of the test signal.

- The level of the transmitted signal is measured at the electrical reference point. The measured signal level is referred to the test signal level and displayed vs. time. The levels are calculated from the time domain using an integration time of 5 ms

- The attenuation vs. time is determined for each pink noise section.

Note: If a non linear or time variant behaviour of the phone is observed the tests as described in section 11.12.6 can be applied to determine the behaviour of the phone more in detail. Instead of inserting the tests signals acoustically they have to be inserted electrically.

## BT Speech Quality during Single Talk

### One Way Speech Quality in Sending

#### Requirement

The nominal values for the speech quality measured from/to the electrical reference point (POI) shall be

MOS-LQON  4,0

#### Test

The test is using P.862 [23].

- The test signals used are the English test sequences as specified in ITU-T Recommendation P.501 [13] (2 male speakers, 2 female speakers, two sentences each). The test signal level is SBTsnd, measured at the BTR, the test signal level is measured as "active speech level" according to ITU-T Recommendation P.56 [15]. The speech activity should be between 30% and 70%.

The original speech signal is used as the reference signal for the determination of the speech quality.

- The test arrangement is according to chapter 12.1. MOS-LQON is determined.

The calculation is made using the signal recorded at the electrical interface.

- The one way speech quality is determined as MOS-LQON.

### BT One Way Speech Quality in Receiving

#### Requirement

The nominal values for the speech quality measured from/to the electrical reference point (POI) shall be

MOS LQO-N  4.0

#### Test

The test is using P.862 [23].

- The test signals used are the English test sequences as specified in ITU-T Recommendation P.501 [13] (2 male speakers, 2 female speakers, two sentences each). The test signal is -16 dBm0, the test signal level is measured as „active speech level“ according to ITU-T Recommendation P.56 [15]. The speech activity should be between 30% and 70%.

The original speech signal is used as the reference signal for the determination of the speech quality.

- The test arrangement is according to chapter 12.1. MOS-LQON is determined. The signal measured at the BTR is used for the calculation

- The one way speech quality is determined as MOS-LQON.

### Verification of Disabled Echo Control

#### Requirements

No acoustic echo control shall be active in the mobile phone. An artificial echo path consisting of an attenuation of 20 dB/40 dB and a delay of 20 ms is inserted at the Bluetooth reference interface. The difference between the echo loss measured with 20 dB echo loss and 40 dB echo loss measured echo loss shall be 20 dB +/-0.2 dB.

#### Test

- For the test an artificial echo path is inserted at the BTR. The test setup is shown in Figure 12.3:

Figure 12.3:	Test setup with artificial echo loss

- The attenuation between the input of the electrical reference point to the output of the electrical reference point is measured using a speech-like test signal.

- Before the actual measurement a training sequence consisting of 10 seconds of artificial voice (male) and 10 seconds of artificial voice (female) according to ITU-T Recommendation P.50 [12] is inserted. The training sequence level shall be -16 dBm0.

- The test signal is a pn sequence according to ITU-T Recommendation P.501 [13] with a length of 4096 points (48 kHz sampling rate) and a crest factor of 6 dB. The duration of the test signal is 250 ms, the test signal level is -3 dBm0. The low crest factor is achieved by random alternation of the phase between -180 ° and +180 °.

- TCLW is calculated according to ITU-T Recommendation G.122 [7], annex B, clause B.4 (trapezoidal pseudo rule). For the calculation the average measured echo level at each frequency band is referred to the average level of the test signal measured in each frequency band. For the measurement a time window has to be applied which is adapted to the duration of the actual test signal (250 ms).

- The difference between the echo loss measured with 20 dB echo loss and 40 dB echo loss is determined.

# Car to Car Communication

# Guidance on subjective testing

Beside objective testing of hands-free telephones also a subjective performance evaluation is necessary. The tests described here – in addition to the tests as described in the ITU-T Recommendations of the P.800 series – are targeted mainly to “in situ” hands-free tests for optimizing hands-free systems in the target car and under conditions which are not covered by the objective test specification.

For conducting the tests the hands-free system under test has to be installed in the target car, which will be referenced here in the document as near-end. As far-end either serves a landline phone (car to land test) or an observing car equipped also with the hands-free system under test (car to car test). It is recommended to not only test the hands-free system in a landline connection but also in a connection car to car because the latter case can be regarded as a worst case scenario resulting in worse hands-free quality compared to landline connections.

The evaluation of the hands-free performance should be done for different background noise scenarios, such as different driving speeds, fan/defrost settings, etc).

For the main part of the subjective tests native language should be used. For the recordings additional languages can be selected.

Since conversational tests are rather time consuming most of the hands-free tests are conducted as single-talk and double-talk tests following a clear given structure.

The evaluation is done at the far-end and/or the near-end, depending on the type of the test category.

To conduct the tests as effective as possible it is advantageous to use a tool providing the test persons on both sides of the telephone connection with the detailed test procedure and the possibility to easily doing the rating.

The performance evaluation of the hands-free system covers categories like

- echo cancellation (echo intensity, speed of convergence, etc.)

- double talk performance (echo during double talk, speech level variation, etc.)

- speech and background noise quality in sending direction (level, level variation, speech distortion, etc)

- speech quality in receiving direction (level, level variation, speech distortion)

- stability of the echo canceller for “closed loop” connection when doing car to car hands-free communication.

For the evaluation some ITU-T recommendations can serve as guideline, such as [21], [22] and others from the P.800 series. The judgment is done according to rating scales given for each test case. The offered rating scales are of MOS type having grades 1 to 5 to be chosen from were 5 denotes “best” and 1 denotes “worst” quality. Some of the rating scales are designed for a more diagnostic purpose (e.g. “echo duration”).

The evaluation has to be done by experts who are experienced with subjective testing of hands-free systems. However, some of the tests described here could be conducted with naive subjects when following the procedures described in the relevant ITU-T Recommendations in the P.800 series.

During the tests the signals on near-end and far-end may be recorded to be used later on for third-party listening evaluation.

Table 14.1 and Table 14.2 show possible test scenarios and rating categories.

Table 14.1: Overview of Car to Landline Tests

| Echo Canceller |  | Rating |
| --- | --- | --- |
| Single Talk, | 3 typical driving scenarios (Germany: e.g. 0 km/h, 100 km/h, 130 km/h, 160 km/h) Fan/defrost (off or „worst case“) Receiving volume: nominal, maximum, varying Enclosure dislocation | Disturbance caused by echo Echo characteristics: - Intensity - Duration - Frequency of occurrence Background noise variation |
| Double Talk | 3 typical driving scenarios (Germany: e.g. 0 km/h, 100 km/h, 130 km/h, 160 km/h) Receiving volume: nominal, maximum, varying Enclosure dislocation | Disturbance caused by echo Echo characteristics: - Intensity - Duration - Frequency of occurrence Speech level variation Speech intelligibility / listening effort |
| Conversation | 0 km/h | Disturbance caused by echo Echo characteristics: - Intensity - Duration - Frequency of occurrence Speech level variation Speech intelligibility / listening effort |
| Speech and Background Noise Quality (Sending Direction) |  | Rating |
| Stationary Noise | 3 typical driving scenarios (Germany: e.g. 0 km/h, 100 km/h, 130 km/h, 160 km/h) Driver’s window closed/opened Several fan settings | Speech level Speech level fluctuation Speech sound quality Intelligibility / listening effort Speech naturalness Background noise quality Signal to noise ratio |
| Transient Noise | Fan: switching on/off/change setting Indicator noises, wiper noise Pass by vehicles | Background noise quality Adaptation to background noise |
| Speech Quality (Receiving Direction) |  | Rating |
| Single Talk far-end | Maximum receiving volume New call | Speech level Speech sound quality Speech intelligibility / listening effort |

Table 14.2: Overview of Car to Car Tests

| Echo Canceller |  | Rating |
| --- | --- | --- |
| Single Talk | 3 typical driving scenarios (Germany: e.g. 0 km/h, 100 km/h, 130 km/h, 160 km/h) Fan (off or „worst case“) Receiving volume: nominal, maximum, varying Enclosure dislocation | Disturbance caused by echo Echo characteristics: - Intensity - Duration - Frequency of occurrence Background noise variation |
| Double Talk | 3 typical driving scenarios (Germany: e.g. 0 km/h, 100 km/h, 130 km/h, 160 km/h) Receiving volume: nominal, maximum, varying Enclosure dislocation | Disturbance caused by echo Echo characteristics: - Intensity - Duration - Frequency of occurrence Speech level variation Speech intelligibility / listening effort |
| System Stability (Car to Car) |  | Rating |
| Stability | Receiving volume: nominal, maximum Test signal: speech or impulse like excitation | Stability Echo characteristics: - Intensity - Duration - Frequency of occurrence |
| Speech and Background Noise Quality (Sending Direction) |  | Rating |
| Stationary noise | 3 typical driving scenarios (Germany: e.g. 0 km/h, 100 km/h, 130 km/h, 160 km/h) | Speech level Speech level fluctuation Speech sound quality Speech intelligibility / listening effort Background noise quality Signal to noise ratio |

Note : The driving condition selected should reflect the type of car and the typical range of driving speeds in the country where the car hands-free system is intended to be installed.

Test Environment and Equipment

Figure 14.1 and Figure 14.2 outline the test environment for the test scenarios car to landline and car to car. In both test scenarios the supervisor is located on the far end, i.e. landline or the observing car, respectively. He is guiding through the test procedure and doing most of the test evaluation.

Figure 14.1: Hands-free test car to landline

Figure 14.2: Hands-free test car to car

Required test equipment:

- Car equipped with the hands-free telephone system under test

- For car to car tests, both vehicles should be identical in terms of make, model, model year, and hands-free system.

- Appropriate recording equipment is needed at the landline station and the car for documentation purposes and if third-party listening tests are required later on.

Further requirements:

- The tests have to be conducted by expert listeners

- Male and female test persons should be in the car, quiet and loud talkers should be considered, different positions of the talkers with respect to the microphone should be considered

- The hands-free tests have to be conducted in areas with good cellular coverage

- In case of a Bluetooth connection between phone and hands-free system the phone should be placed in such a way the Bluetooth coverage and cellular coverage is good

- Some cell-phone networks have an influence on the hands-free performance (e.g. echo, AGC, noise reduction, etc). These networks should not be chosen for the tests if possible.

- Both the far-end and the near-end participant should be familiar with each other’s voice

- For the duration of the tests, there should be no change in the driver, front-passenger, or landline speaker.

Note: The network QoS parameters can be monitored by specific systems during the tests (based on 3GPP ) e.g. using TEMS investigation.

A description of the testing environment should be provided and should contain the following information:

- Car brand, model, model year, tires, type of road, engine type, interior trim (cloth or leather, sunroof…),

- Mobile telephone type and SW version

- Telematics microphone – description, location, orientation, distance to driver

- Network provider

- HW and SW version of the hands-free device under test

- HW and SW version of all devices which are part of the sound system

Background Noise and Driving Situations

For the tests several different driving and background noise scenarios should be considered which correspond to the main operating conditions of the hands-free system. Additionally some worst case scenarios might be regarded.

For the driving scenarios this implies that the scenarios might differ for different countries depending e.g. on the national speed limit for cars.

As example the following scenarios for different driving noise levels could be used (Germany):

- “low”:		    0 km/h

- “medium”: 	100 km/h

- „high“:		160 km/h

For countries with speed limit the maximum allowed speed might be used with additionally the climate control switched on to an appropriate level.

Additionally to the driving scenarios also some different settings of the fan/defrost/climate control/recirculation are useful background noise cases. As “worst case” scenario a setting of the fan might serve where the air stream directly flows to the hands-free microphone in the car.

Test Documentation

- After each test, a rating of the performance is done by referring to the given rating scales

- At the beginning of every test, one of the test participants announces the test number (this is done for recording purposes)

- Audio signals should be recorded for documentation purposes:

- On landline: Telephone audio recording  of uplink and downlink signal

- In the observing car: Binaural recording

- In the car under test: Binaural recording

Notes on Performance Rating

For the ratings the following items should be considered:

- The listeners who do the ratings have to be experienced with hands-free telephone systems.

- Limitations of the network have to be taken into account, e.g. for rating the hands-free system when the network’s voice codec quality depends on the telephone connection traffic (e.g. CDMA).

- The offered rating scales are of MOS type having grades 1 to 5 to be chosen from were 5 denotes “best” and 1 denotes “worst” quality.

- Some of the rating scales are designed for a more diagnostic purpose (e.g. the “echo duration”).

An example for a possible questionnaire applicable for this type of tests is given in Annex C.

# Corded Headsets

For further study

## Connector Type

## Connector Wiring and Electrical Specifications

## Headset Receive Characteristics

## Headset Transmit Characteristics

## Standard Behaviour in the Presence of Corded Headsets

# Wireless Headsets

For further study

## Wireless Headset Types

## Test Methodology for Verification of Standard Behaviour

## Standard Behaviour in the Presence of Wireless Headsets

### Associate Headset with Vehicle

### Enter the Vehicle

### Receive a Call

### Make a Call

### Terminate a Call

### Exit the Vehicle

### Listen to Other Audio Source

### Switch between Other Audio and Telephony

### Intra-vehicle Communications

Annex A: Speech Quality Measurements

In the following a testing method for determining the speech quality in sending and receiving is described. The speech quality expressed in TMOS and its relation to existing ITU-T terminology according to ITU-T Recommendation P.800.1 is as follows:

MOS related to listening-only situations

These MOS scores are applicable to a listening-only situation. Three different cases have to be distinguished.

MOS-LQS

The score has been collected in a laboratory test by calculating the arithmetic mean value of subjective judgments on a 5-point ACR quality scale, as it is defined in ITU-T Rec. P.800.

Subjective tests carried out according to ITU-T Recs P.830, P.835 and P.840 give results in terms of

MOS-LQS.

MOS-LQO

The score is calculated by means of an objective model which aims at predicting the quality for a

listening-only test situation. Objective measurements made using the model given in ITU-T Recs P.862.1 and P.862.2 give results in terms of MOS-LQO.

It should be noted that the method recommended by ITU-T in Recs P.862.1 and P.862.2 is validated between electrical interfaces only. Currently no ITU Recommendation exists which covers the measurement of listening quality including acoustical interfaces. Work on a new ITU-T Rec. P.OLQA which is intended to include acoustical interfaces is under progress.

MOS-LQO (acoustical)

This kind of measurement is performed at acoustical interfaces. In order to predict the listening quality as perceived by the user, this measurement includes the actual telephone set products provided by the manufacturer or vendor. In combination with the choice of the acoustical receiver in the lab test ("artificial ear"), there will be a more or less leaky condition between the handset's receiver and the artificial ear. The same constraints apply for hands-free telephony. Consequently, for more realistic test scenarios, there may be a degradation of the measured MOS value, while for more artificial test scenarios there may be a negligible difference. The TMOS is a MOS-LQO (acoustical) prediction.

|  | Listening-only | Conversational | Talking |
| --- | --- | --- | --- |
| Subjective | MOS-LQSy | MOS-CQSy | MOS-TQSy |
| Objective | MOS-LQOy | MOS-CQOy | MOS-TQOy |
| Estimated | MOS-LQEy | MOS-CQEy | MOS-TQEy |

NOTE – The letter "y" at the end of above acronyms is a placeholder for the descriptor of the respective audio bandwidth, see the following provisional instructions.

–			N for MOS scores obtained for narrow-band (300-3400 Hz) speech relative to a narrow-band high quality reference. This is applicable for instance to narrow-band only subjective tests or to P.862.1 scores.

–			W for MOS scores obtained for wideband (50-7000 Hz) speech relative to a wideband high quality reference. This is applicable for instance to wideband only subjective tests or to P.862.2 scores.

–			M for MOS scores obtained for narrow-band or wideband speech relative to a wideband high quality reference in a mixed bandwidths context. This is applicable for instance to mixed bandwidths subjective tests.

Further information can be found in ITU-T Rec. P.800.1[22]

## A.1	One Way Speech Quality in Sending

- The test signals used are the German test sequences as specified in ITU-T Recommendation P.501 [13] (2 male speakers, 2 female speakers, two sentences each). The test signal is equalized at the MRP, the test signal level is –4.7 dBPa at the MRP, the test signal level is measured as "active speech level" according to ITU-T Recommendation P.56 [15]. The speech activity should be between 30% and 70%. Finally the level at the HATS-HFRP is adjusted to –28.7 dBPa.

The original speech signal is used as the reference signal for the determination of the speech quality.

- The test arrangement is according to chapter 7.1. TMOS is determined using the settings “high quality handset“ and “narrowband” with TOSQA2001.

The calculation is made using the signal recorded at the electrical interface.

- The one way speech quality is determined as TMOS.

## A.2	One Way Speech Quality in Receiving

- The test signals used are the German test sequences as specified in ITU-T Recommendation P.501 [13] (2 male speakers, 2 female speakers, two sentences each). The test signal is -16 dBm0, the test signal level is measured as „active speech level“ according to ITU-T Recommendation P.56 [15]. The speech activity should be between 30% and 70%.

The original speech signal is used as the reference signal for the determination of the speech quality.

- The test arrangement is according to chapter 7.1. For the measurement the artificial head is free-field equalized according to ITU-T Recommendation P.581 [18]. The equalized output signal of the right ear is used for the measurements. TMOS is determined using the settings “high quality handset“ and “narrowband” with TOSQA2001.

The calculation is made using the signal recorded at the DRP of the right ear of the artificial head.

- The one way speech quality is determined as TMOS.

Note: This test method does not apply to systems including artificial bandwidth extension.

Annex B: Principles of Relative Approach

The Relative Approach [25] is an analysis method developed to model a major characteristic of human hearing. This characteristic is the much stronger subjective response to distinct patterns (tones and/or relatively rapid time-varying structure) than to slowly changing levels and loudnesses.

Figure B1: Block diagram of Relative Approach

The idea behind the Relative Approach analysis is based on the assumption that human hearing creates a running reference sound (an "anchor signal") for its automatic recognition process against which it classifies tonal or temporal pattern information moment-by-moment. It evaluates the difference between the instantaneous pattern in both time and frequency and the "smooth" or less-structured content in similar time and frequency ranges. In evaluating the acoustic quality of a complex "patterned" signal, the absolute level or loudness is almost without any significance. Temporal structures and spectral patterns are important factors in deciding whether a sound is judged as annoying or disturbing  [25].

Similar to human hearing and in contrast to other analysis methods the Relative Approach algorithm does not require any reference signal for the calculation. Only the signal under test is analyzed. Comparable to the human experience and expectation, the algorithm generates an "internal reference" which can be best described as a forward estimation. The Relative Approach algorithm objectifies pattern(s) in accordance with human perception by resolving or extracting them while largely rejecting pseudostationary energy. At the same time, it considers the context of the relative difference of the "patterned" and "non-patterned" magnitudes.

Figure B1 shows a block diagram of the Relative Approach. The time-dependent spectral pre-processing can either be done by a filter bank analysis (1/nth octave, typically 1/12th octave) or a Hearing Model spectrum versus time according to the Hearing Model of Sottek (see Bibliography). Both of them result in a spectral representation versus time. Both are calculating the spectrograph using only linear operation and their outputs are therefore directly comparable. The Hearing Model analysis parameters are fixed and based on the processing in human ears whereas the input parameters for the filter bank analysis can vary. The filter bank pre-processing approximates the Hearing Model version.

Two different variants of Relative Approach can be applied to the pre-processed signal. The first one applies a regression versus time for each frequency band in order to cover human expectation for each band within the next short period of time. Afterwards for each time slot a smoothing versus frequency is performed. The next step is a non-linear transformation according to the Hearing Model of Sottek  (see Bibliography). This output is compared to the source signal which is also Hearing Model transformed. Non-relevant components for human hearing are finally set to zero. This approach focuses on the detection of tonal components. The second version first smoothes versus frequency within a time slot and then applies the regression versus time. This output signal is transformed non-linear to the Hearing Model. It is compared to the output of the smoothing versus frequency which is also non-linearly transformed according to the Hearing Model. Finally non-relevant components for human hearing are again set to zero. Thus more transient structures are detected.

Via the factors 1 and 2 the weighting of Relative Approach for tonal and transient signals can be set. Typically 1= 0 and 2 = 1 are chosen. Thus, the model is tuned to detect time-variant transient structures.

The result of the Relative Approach analysis is a 3D spectrograph displaying the deviation from the "close to the human expectation" between the estimated and the current signal is displayed versus time and frequency. The Relative Approach uses a time resolution of t = 6,66 ms. The frequency range from 15 Hz to 24 kHz is divided into 128 frequency bands fm which corresponds to a 1/12th octave resolution. Due to the nonlinearity in the relationship between sound pressure and perceived loudness, the term "compressed pressure" in compressed Pascal (cPa) is used to describe the result of applying the nonlinear transform. The Relative Approach can determine how "close to the human expectation" a signal is, but not if this expectation is of a high or a low quality origin.

Annex C: Example for a Questionnaire for Subjective Testing

# C	Performance Rating

## C.1	Overview

The performance evaluation of the hands-free system covers categories like

- echo cancellation performance during single talk and double talk (echo during single talk, convergence after enclosure dislocation, echo during double talk, speech level variation during double talk, etc.)

- speech and background noise quality in sending direction (level, level variation, speech naturalness, etc)

- speech quality in receiving direction (level, level variation, speech naturalness)

- stability of the echo canceller for “closed loop” operation when doing car to car hands-free communication.

For the evaluation some ITU-T recommendations have served as guideline, such as [21], [22] and others from the P.800 series. The judgement is done according to rating scales given for each test case. The offered rating scales are of MOS type having grades 1 to 5 to be chosen from were 5 denotes “best” and 1 denotes “worst” quality. Some of the rating scales are designed for a more diagnostic purpose (e.g. “echo duration”).

Additional Notes on Performance Rating

For the ratings the following items should be considered:

- The evaluation has to be done by experts who are experienced with subjective testing of hands-free systems.

- Limitations of the network have to be taken into account and should be documented. For example for rating the hands-free system when using CDMA the rating could say “not better than 2 because of CDMA network”

- The offered rating scales are of MOS type having grades 1 to 5 to be chosen from were 5 denotes “best” and 1 denotes “worst” quality.

- Some of the rating scales are designed for a more diagnostic purpose (e.g. the “echo duration”).

## C.2	Test categories and rating types

The following table gives an overview of the test categories and the related rating scales.

Table C1: Test categories and rating types overview

| Test category | Test subcategory | conversation type | Rating side | Rating type | Rating Scales | Test condition / variation |
| --- | --- | --- | --- | --- | --- | --- |
| Speech and background noise quality in sending direction | Speech level | single talk near-end | far-end | Speech level | Loudness preference (office) | stationary background noise scenario: low/medium/high, fan/defrost, window |
|  | Speech quality | single talk near-end | far-end | Speech Quality | Speech level fluctuation Speech sound quality Speech naturalness Intelligibility/Listening effort | stationary background noise scenario: low/medium/high, fan/defrost, window |
|  | Background noise quality during near-end single talk | single talk near-end | far-end | Background noise quality | Signal to Noise Ratio Noise quality | stationary background noise scenario: low/medium/high, fan/defrost |
|  | Transient background noise quality | idle and single talk near-end | far-end | Transient background noise quality | Transient noise quality | transient background noise scenario: fan/defrost startup, wiper, indicator |
|  |  | idle | far-end | Adaptation to background noise | Adaptation to background noise | transient background noise scenario: noise jump, e.g. fan/defrost startup |
| Speech quality in receiving direction | Speech level | single talk far-end | near-end | Speech level (car, max.Vol.) | Loudness preference (car) for max. Vol. | only high background noise scenario |
|  |  | single talk far-end | near-end | Speech level (car, nominal. Vol., new Call) | Loudness preference (car) for nominal Volume, new Call | low background noise scenario |
|  | Speech quality | single talk far-end | near-end | Speech Quality | Speech sound quality Intelligibility/Listening effort | low background noise scenario |
| Echo cancellation | Echo during single talk | single talk far-end | far-end | Echo | Disturbance caused by Echo, Echo characterisitics (only to be rated if echo occurs): - Intensity, - Duration, - Frequency of occurrence - Echo Intelligibility | stationary background noise scenario: low/medium/high, fan/defrost; volume (car), movement of driver (enclosure dislocation) |
|  | Background noise quality during far-end single talk | single talk far-end | far-end | Background noise quality | Comfort noise quality (EC-Test) | stationary background noise scenario: medium/high, fan/defrost; |
|  | Echo during double talk | double talk | far-end | Echo | Disturbance caused by Echo, Echo characterisitics (only to be rated if echo occurs): - Intensity, - Duration, - Frequency of occurrence - Echo Intelligibility | stationary background noise scenario: low/medium/high, fan/defrost; volume (car), movement of driver (enclosure dislocation) |
|  | Speech quality at near-end during double talk | double talk | near-end | Speech Quality | Speech level variation during double talk Intelligibility/Listening effort during double talk | low background noise scenario |
|  | Speech quality at far-end during double talk | double talk | far-end | Speech Quality | Speech level variation during double talk Intelligibility/Listening effort during double talk | stationary background noise scenario: low/medium/high, fan/defrost, window |
| System stability | System stability | special test | far-end | Echo Convergence and Stability | System Stability: Speed of convergence of echo cancellation and robustness against echo back coupling | Car to Car, low background noise scenario, EC not adapted at start of test, max. Volume |

In addition to the tests given in Table C1 also some conversational tests can be performed as described in recommendations P.800 and P.831 (conversation opinion scale).

## C.3	Speech and Background Noise Quality in Sending Direction

Speech Level (office)

Description:

The rating scale is applied in test cases which evaluate the preferred speech level of the received signal at the far-end (office).

| Test category: | Speech and background noise quality in sending direction |
| --- | --- |
| Conversation type: | Single talk at near-end |
| Rating side: | Rated at far-end (office) |
| Test conditions: | stationary background noise scenarios of low/medium/high level; additional background noise scenarios: different fan/defrost settings, window open, etc. |
| Scale type: | Loudness-preference scale, P.800, ACR |

The rating scale is given in the following table.

Table C2: Loudness Preference (office)

| Rating description |  | Grade |
| --- | --- | --- |
| Much louder than preferred |  | 1 |
| Louder than preferred |  | 3 |
| Preferred |  | 5 |
| Quieter than preferred |  | 3 |
| Much quieter than preferred |  | 1 |

Speech Level Fluctuations

Description:

The rating scale is applied in test cases which evaluate level variations in speech in single-talk situations.

| Test category: | Speech and background noise quality in sending direction |
| --- | --- |
| Conversation type: | Single talk at near-end |
| Rating side: | Rated at far-end |
| Test conditions: | stationary background noise scenarios of low/medium/high level; additional background noise scenarios: different fan/defrost settings, window open, etc. |
| Scale type: | Special scale for HF-System diagnostic evaluation |

Level fluctuations to be examined for this evaluation are characterized by:

- level fading

- short drop-outs e.g. due to missing data packets

- cut-offs (missing word-ends or syllables)

- chopped voice

The rating scale is given in the following table.

Table C3: Speech Level Fluctuations

| Rating description |  | Grade |
| --- | --- | --- |
| No speech level variation audible |  | 5 |
| Slight level variations, just audible or very rare occurring |  | 4 |
| Moderate speech level variations, may occur frequently |  | 3 |
| Sometimes words or syllables are attenuated or missing |  | 2 |
| Many drop outs, cut offs, missing words or syllables, heavily chopped voice |  | 1 |

Speech Quality / Speech Naturalness

Description:

The rating scale is applied in test cases which evaluate speech naturalness received on the far-end for different background noise scenarios on the near-end. This evaluation includes possible impairments caused by signal distortion and band limiting effects which also degrade the speech naturalness. The best quality case for this evaluation would be a hand-set comparable voice quality.

| Test category: | Speech and background noise quality in sending direction, |
| --- | --- |
| Conversation type: | Single talk at near-end |
| Rating side: | Rated at far-end |
| Test conditions: | stationary background noise scenarios of low/medium/high level; additional background noise scenarios: different fan/defrost settings, window open, etc. |
| Scale type: | Special scale for HF-System diagnostic evaluation (considering also the degradation scale, P.800, Degradation Category Rating) |

Properties to be examined for this evaluation are

- synthetic/robotic sound

- speech signal distortion characterized by a scratchy sound.

- band limitation or filtering effects characterized by

- a shrill, sharp, thin, tinny or muffled sounding speech

- an emphasis on high frequencies or low frequencies.

The rating scale is given in the following table.

Table C4: Speech Quality / Speech Naturalness

| Rating description |  | Grade |
| --- | --- | --- |
| Speech sound is comparable to hand-set voice quality; Speech sounds clear and transparent | natural | 5 |
| Minor degradation to hand-set, still natural voice; Possibly slight distortions and/or slight band limitation effects | . | 4 |
| Maybe slight synthetic voice at times and/or low level distortion and/or moderate band limitation effects; | . | 3 |
| Very noticeable synthetic voice and/or heavy distortion and/or higher degree of band-limitation | . | 2 |
| Signal barely recognizable as voice | unnatural | 1 |

Intelligibility / Listening-effort

Description:

The rating table is applied to evaluate the effort required to understand the meaning of words and sentences. The applicable test cases are single talk and different background noise scenarios at the near-end and evaluation on the far-end.

| Test category: | Speech and background noise quality in sending direction, |
| --- | --- |
| Conversation type: | Single talk at near-end |
| Rating side: | Rated at far-end |
| Test conditions: | stationary background noise scenarios of low/medium/high level; additional background noise scenarios: different fan/defrost settings, window open, etc. |
| Scale type: | Listening-effort scale, P.800, ACR |

The rating scale is given in the following table.

The question heading this scale could be for example:

How would you judge the effort required to understand words and sentences of your remote partner?

Table C5: Intelligibility / Listening-effort

| Rating description |  | Grade |
| --- | --- | --- |
| Every word was clearly understood with no effort required. |  | 5 |
| Speech of the other side is understood with no appreciable effort required |  | 4 |
| Some words were hard to understand, moderate effort is required. |  | 3 |
| Many words were hard to understand, considerable effort required. |  | 2 |
| No meaning understood with any feasible effort. |  | 1 |

Signal to Noise Ratio for near-end single talk

Description:

The following rating scale is intended to evaluate the noise level compared to the speech level.

The evaluation is usually done in a high environmental noise test condition. The signal to noise ratio directly depends on the background noise level of the test scenario. The judgment therefore will get to a worse grade with the test scenario changing to a higher environmental noise. Therefore it is problematic for this test category to be used as absolute evaluation, it’s more applicable for comparison of different systems for the same environmental noise scenario.

| Test category: | Speech and background noise quality in sending direction, |
| --- | --- |
| Conversation type: | Single talk at near-end |
| Rating side: | Rated at far-end |
| Test conditions: | stationary background noise scenarios of low/medium/high level; additional background noise scenarios: different fan/defrost settings; |
| Scale type: | Scale is adapted from “Degradation category scale” (P.800, Annex D) and the extended “detectability scale” of the “Quantal-Response Detectability Test” (P.800, Annex C) |

The rating scale is given in the following table.

Table C6: Signal to Noise Ratio

| Rating description |  | Grade |
| --- | --- | --- |
| Noise very low, just audible |  | 5 |
| Noise audible, noise level clearly lower than speech level, noise is not disturbing |  | 4 |
| Medium noise level, lower than speech level, noise slightly disturbing |  | 3 |
| High noise level, almost same level as speech, clearly disturbing, but call would be continued. |  | 2 |
| Noise louder than speech, intolerably disturbing, call would be abandoned. |  | 1 |

Background Noise Quality

Description:

This scale is for evaluation of the sound quality of the near-end background noise examined at the far-end. For explicit evaluation of transient noise sources another rating scale is defined in the subsection following hereafter.

| Test category: | Speech and background noise quality in sending direction, |
| --- | --- |
| Conversation type: | Single talk at near-end |
| Rating side: | Rated at far-end |
| Test conditions: | stationary background noise scenarios of low/medium/high level; additional background noise scenarios: different fan/defrost settings; |
| Scale type: | Special scale for HF-System diagnostic evaluation |

The following sound characteristics have to be taken into account:

- Are there changes of noise-sound and level over time?

- Are there artefacts audible (clicks, pops, rattle) which cannot be matched to a natural source (e.g. road bumps) or don’t sound like their natural source?

- Naturalness:

  - does the noise sound like being part of the natural background?

  - does the noise sound synthetic (musical tones, watery sound)?

  - does the noise sound distorted?

Artefacts, synthetic sound, level and sound variation over time, and noise sounding not like being part of the natural background result in smaller (worse) grades.

The rating scale is given in the following table.

Table C7: ackground Noise Quality

| Rating description |  | Grade |
| --- | --- | --- |
| Comfortable, natural sound, constant in sound and level, no artefacts | natural | 5 |
| Slight distortion/synthetic sound, almost no artefacts, almost constant in sound and level | . | 4 |
| Moderate distortion/synthetic sound, or some artefacts/clicks/plops audible, or some moderate variation in sound and level | . | 3 |
| Clearly unnatural/distorted/synthetic sounding, or many artefacts/clicks/plops, or often variation in sound and level | . | 2 |
| Completely unnatural/distorted/synthetic sound, or permanent artefacts/clicks/plops, or permanent variations in sound and level, very uncomfortable to listen to | unnatural | 1 |

Noise Quality for Transient Noise Sources

Description:

This scale is for evaluation of the sound quality of transient near-end background noise examined at the far-end. Transient noise sources can be for example the car’s activated wiper, indicator, etc.

| Test category: | Speech and background noise quality in sending direction, |
| --- | --- |
| Conversation type: | Idle and single talk at near-end |
| Rating side: | Rated at far-end |
| Test conditions: | Transient noise sources active (indicator, wiper, etc.) |
| Scale type: | Special scale for HF-System diagnostic evaluation |

For the evaluation the naturalness of the noise sound has to be taken into account, for example

- does the noise sound like being part of the natural background?

- does the noise sound synthetic?

- does the noise sound distorted?

The rating scale is given in the following table.

Table C8: Transient Noise Quality

| Rating description |  | Grade |
| --- | --- | --- |
| Comfortable / natural sound; | natural | 5 |
| Almost natural sounding; Slight distortion/synthetic sound | . | 4 |
| Moderate unnatural sounding; Moderate distortion/synthetic sound | . | 3 |
| Clearly unnatural/distorted/synthetic sounding | . | 2 |
| Completely unnatural/distorted/synthetic sound | unnatural | 1 |

Adaptation to background noise

Description:

This scale is for evaluation of the speed of adaptation of the noise suppression to the near-end background noise after a noise level jump. The evaluation is done on the far-end.

| Test category: | Speech and background noise quality in sending direction, |
| --- | --- |
| Conversation type: | Idle |
| Rating side: | Rated at far-end |
| Test conditions: | Transient background noise: noise jump, e.g. fan/defrost startup |
| Scale type: | Special scale for HF-System diagnostic evaluation |

The test can be conducted for example by turning on the defrost/fan in the car to a high setting. When doing the test like this then the time the defrost/fan needs to run up has to be taken into account.

The rating scale is given in the following table.

Table C9: Adaptation to background noise

| Rating description |  | Grade |
| --- | --- | --- |
| immediate adaptation | very fast | 5 |
| adaptation time ≤ 1 second |  | 4 |
| adaptation time 2 … 3 seconds |  | 3 |
| adaptation time 3 … 10 seconds |  | 2 |
| adaptation time ≥ 10 seconds | very slow | 1 |

## C.4	Speech Quality in Receiving Direction (in the car under test)

Speech Sound Quality / Speech Naturalness (Receiving)

Description:

The rating scale is applied in test cases which evaluate speech naturalness received in the car under test (near-end). This evaluation includes possible impairments caused by signal distortion and band limiting effects which also degrade the speech naturalness. The best quality case for this evaluation would be a hand-set comparable voice quality. The evaluation is done in low background noise condition using nominal volume setting in the car.

| Test category: | Speech quality in receiving direction, |
| --- | --- |
| Conversation type: | Single talk at far-end |
| Rating side: | Rated at near-end |
| Test conditions: | low background noise condition |
| Scale type: | Special scale for HF-System diagnostic evaluation (considering also the degradation scale, P.800, Degradation Category Rating) |

Properties to be examined for this evaluation are

- synthetic/robotic sound

- speech signal distortion characterized by a scratchy sound.

- band limitation or filtering effects characterized by

- a shrill, sharp, thin, tinny or muffled sounding speech

- an emphasis on high frequencies or low frequencies.

For the rating scale see table C4.

Intelligibility / Listening-effort (Receiving)

Description:

The rating table is applied to evaluate the effort required to understand the meaning of words and sentences. The applicable test cases are single talk at the far-end examined in the car under test (near-end) in a low background noise condition using nominal volume setting in the car.

| Test category: | Speech quality in receiving direction |
| --- | --- |
| Conversation type: | Single talk at far-end |
| Rating side: | Rated at near-end |
| Test conditions: | low background noise condition |
| Scale type: | Listening-effort scale, P.800, ACR |

The question heading this scale could be for example:

How would you judge the effort required to understand words and sentences of your remote partner?

For the rating scale see table C5.

Speech Level (Receiving, maximum Volume)

Description:

The rating table is applied to evaluate the speech level heard from the loudspeakers in the car (near-end) when being in a high background noise condition and having the telephone volume set to maximum.

| Test category: | Speech quality in receiving direction |
| --- | --- |
| Conversation type: | Single talk at far-end |
| Rating side: | Rated at near-end |
| Test conditions: | high background noise condition |
| Scale type: | Loudness-preference scale, P.800, ACR |

The rating scale is given in the following table.

Table C10: Loudness Preference (Car) for maximum Volume Setting

| Rating description |  | Grade |
| --- | --- | --- |
| Much louder than preferred |  | 1 |
| Louder than preferred |  | 3 |
| Preferred (for maximum volume setting) |  | 5 |
| Quieter than preferred |  | 3 |
| Much quieter than preferred |  | 1 |

Speech Level for nominal Volume and new Call

Description:

The rating table is applied to evaluate the speech level heard from the loudspeakers in the car (near-end) when having a new call after having set the volume to nominal in the prior call.

| Test category: | Speech quality in receiving direction |
| --- | --- |
| Conversation type: | Single talk at far-end |
| Rating side: | Rated at near-end |
| Test conditions: | high background noise condition |
| Scale type: | Loudness-preference scale, P.800, ACR |

The rating scale is given in the following table.

Table C11: Loudness Preference for maximum Volume and new Call

| Rating description |  | Grade |
| --- | --- | --- |
| Much louder than preferred |  | 1 |
| Louder than preferred |  | 3 |
| Preferred |  | 5 |
| Quieter than preferred |  | 3 |
| Much quieter than preferred |  | 1 |

## C.5	Echo Cancellation Performance

For the evaluation of echo in a first step the disturbance caused by echo is judged. Only if echo is perceived then an additional evaluation of additional echo characteristics are performed. The evaluation of these echo characteristics are intended for diagnostic purposes.

The tests are intended to rate the perceived quality according to:

- Amount and nature of echo during single talk

- Amount and nature of echo during double talk

- Convergence characteristics of the EC to handle variation of the echo path 
(e.g. when the driver in the car is moving)

- Speech quality during double talk situations (e.g. intelligibility and speech level variation), this judgement is done on the near-end and the-far end

- Stability of the EC in car to car communication

The tests are performed for different background noise scenarios and driving conditions, to get some information about the EC robustness in high background noise conditions.

The scales given in this section can be applied for steady state conditions and initial convergence tests.

Disturbance caused by Echo

Description:

The rating scale is applied to evaluate the disturbance caused by echo examined at the far-end. The test scenarios should include different background noise conditions at the near-end.

| Test category: | Echo cancellation |
| --- | --- |
| Conversation type: | Single talk at far-end Double talk |
| Rating side: | Rated at far-end |
| Test conditions: | stationary background noise scenarios of low/medium/high level; additional background noise scenarios: different fan/defrost settings; enclosure dislocation due to movement of the driver; volume setting in the car: nominal, maximum |
| Scale type: | Conversation impairment scale, P.800 and P.831 |

The other echo rating scales (echo intensity, duration, frequency of occurrence and intelligibility) following hereafter are intended for diagnostic purposes. They are only used if echo is perceived.

The rating scale is given in the following table.

For the evaluation the participant has to answer a question like this for example:

How would you judge the degradation/impairment/disturbance from echo of your own voice during the test?

Table C12: Disturbance caused by echo

| Rating description |  | Grade |
| --- | --- | --- |
| Imperceptible |  | 5 |
| Perceptible but not annoying |  | 4 |
| Slightly annoying |  | 3 |
| Annoying |  | 2 |
| Very annoying |  | 1 |

Echo Intensity

Only for diagnostic purposes.

Description:

This rating is intended for diagnostic purposes and has only to be done if echo is perceived.

The scale is applied for evaluation of the echo level occurring in far-end single-talk and in double-talk test cases. The evaluation is done on the far-end. The test scenarios should include different background noise conditions at the near-end.

| Test category: | Echo cancellation |
| --- | --- |
| Conversation type: | Single talk at far-end Double talk |
| Rating side: | Rated at far-end |
| Test conditions: | stationary background noise scenarios of low/medium/high level; additional background noise scenarios: different fan/defrost settings; enclosure dislocation due to movement of the driver; volume setting in the car: nominal, maximum |
| Scale type: | Special scale for HF-System diagnostic evaluation |

The rating scale is given in the following table.

Table C13: Echo Intensity

| Rating description |  | Grade |
| --- | --- | --- |
| - |  | - |
| slight |  | 4 |
| moderate |  | 3 |
| loud |  | 2 |
| very loud |  | 1 |

Echo Duration

Only for diagnostic purposes.

Description:

This rating is intended for diagnostic purposes and has only to be done if echo is perceived.

The scale is applied for evaluation of the echo duration occurring in far-end single-talk and in double-talk test cases. The evaluation is done on the far-end. The test scenarios should include different background noise conditions at the near-end.

| Test category: | Echo cancellation |
| --- | --- |
| Conversation type: | Single talk at far-end Double talk |
| Rating side: | Rated at far-end |
| Test conditions: | stationary background noise scenarios of low/medium/high level; additional background noise scenarios: different fan/defrost settings; enclosure dislocation due to movement of the driver; volume setting in the car: nominal, maximum |
| Scale type: | Special scale for HF-System diagnostic evaluation |

The rating scale is given in the following table.

Table C14: Echo Duration

| Rating description |  | Grade |
| --- | --- | --- |
| very short |  |  |
| Short |  |  |
| Moderate |  |  |
| Long |  |  |
| very long / permanent |  |  |

Frequency of Echo Occurrence

Only for diagnostic purposes.

Description:

This rating is intended for diagnostic purposes and has only to be done if echo is perceived.

The scale characterizes the number of echo events occurring during the echo test, the test cases include far-end single-talk and double-talk. The evaluation is done on the far-end. The test scenarios should include different background noise conditions at the near-end.

| Test category: | Echo cancellation |
| --- | --- |
| Conversation type: | Single talk at far-end Double talk |
| Rating side: | Rated at far-end |
| Test conditions: | stationary background noise scenarios of low/medium/high level; additional background noise scenarios: different fan/defrost settings; enclosure dislocation due to movement of the driver; volume setting in the car: nominal, maximum |
| Scale type: | Special scale for HF-System diagnostic evaluation |

The rating scale is given in the following table.

Table C15: Frequency of Echo Occurrence

| Rating description |  | Grade |
| --- | --- | --- |
| only once during the test |  |  |
| only twice during the test |  |  |
| infrequently several times |  |  |
| echo occurs more often than not |  |  |
| permanent |  |  |

Echo Intelligibility

Only for diagnostic purposes.

Description:

This rating is intended for diagnostic purposes and has only to be done if echo is perceived.

The scale is applied for characterizing the type of sound of the echo occurring in far-end single-talk and in double-talk test cases (pure artefacts or the echoed voice of the talker). The evaluation is done on the far-end. The test scenarios should include different background noise conditions at the near-end.

| Test category: | Echo cancellation |
| --- | --- |
| Conversation type: | Single talk at far-end Double talk |
| Rating side: | Rated at far-end |
| Test conditions: | stationary background noise scenarios of low/medium/high level; additional background noise scenarios: different fan/defrost settings; enclosure dislocation due to movement of the driver; volume setting in the car: nominal, maximum |
| Scale type: | Special scale for HF-System diagnostic evaluation |

The rating scale is given in the following table.

Table C16: Echo Intelligibility

| Rating description |  | Grade |
| --- | --- | --- |
| pure artefacts |  |  |
| hardly recognizable as voice |  |  |
| distorted voice |  |  |
| slightly distorted voice |  |  |
| clear voice |  |  |

Comfort Noise Quality (EC Test)

Description:

The scale is for evaluation of the near-end background noise sound quality received at the far-end during far-end single talk. The evaluation gives information about the quality of comfort noise injection. Transient noise should be avoided at the near-end during the test.

| Test category: | Echo cancellation |
| --- | --- |
| Conversation type: | Single talk at far-end |
| Rating side: | Rated at far-end |
| Test conditions: | stationary background noise scenarios of low/medium/high level; additional background noise scenarios: different fan/defrost settings; |
| Scale type: | Special scale for HF-System diagnostic evaluation |

Following sound characteristics have to be taken into account:

- Are there changes of noise-sound and level over time (e.g. when changing from natural near-end background noise to comfort noise injection and vice versa)?

- Are artefacts audible (clicks, pops, rattle)?

- Naturalness:

  - Does the noise sound like being part of the natural background?

  - Does the noise sound synthetic (musical tones, watery sound)?

  - Does the noise sound distorted?

Artefacts, synthetic sound, level and sound variation over time, and noise sounding not like being part of the natural background will result in smaller (worse) rating grades.

The rating scale is given in the following table.

Table C17: Comfort Noise Quality (EC-Test)

| Rating description |  | Grade |
| --- | --- | --- |
| No difference between comfort noise and natural background noise perceivable; Comfortable, natural sound, constant in sound and level, no artefacts; | natural | 5 |
| Slight difference between comfort noise and natural background noise perceivable; Slight distorted/synthetic sound, almost no artefacts, almost constant in sound and level; | . | 4 |
| Moderate difference between comfort noise and natural background noise perceivable; Moderate distorted/synthetic sound, or some artefacts/clicks/plops audible, or some moderate variation in sound and level | . | 3 |
| Clear difference between comfort noise and natural background noise perceivable; Clearly unnatural/distorted/synthetic sounding, or many artefacts/clicks/plops, or often variation in sound and level | . | 2 |
| Comfort noise does not sound like the natural background noise at all; Very unnatural/distorted/synthetic sound, or permanent artefacts/clicks/plops, or permanent variations in sound and level, very uncomfortable to listen to | unnatural | 1 |

Speech Level Variation during Double Talk

Description:

The scale is applicable to evaluate speech level variations during double-talk. The evaluation is done on both sides, near-end and far-end. When the evaluation is done on the far-end then the test scenarios should include different background noise conditions at the near-end.

| Test category: | Echo cancellation |
| --- | --- |
| Conversation type: | Double talk |
| Rating side: | Rated at near-end Rated at far-end |
| Test conditions: | For rating at the near-end: - low background noise scenario For rating at the far-end: - stationary background noise scenarios of low/medium/high level; - additional background noise scenarios: different fan/defrost settings; - enclosure dislocation due to movement of the driver; - volume setting in the car: nominal, maximum |
| Scale type: | Special scale for HF-System diagnostic evaluation |

Level variations are characterized by:

- switching of an attenuation exactly during the double talk phases

- level fading

- short drop-outs or cut-offs (missing word-ends or syllables)

- chopped voice

The rating scale is given in the following table.

Table C18: Speech Level Variation during Double Talk

| Rating description |  | Grade |
| --- | --- | --- |
| No speech level variation audible |  | 5 |
| Slight level variations, just audible or very rare occurring |  | 4 |
| Moderate speech level variations may occur frequently, sometimes words or syllables might be attenuated or missing, or moderate constant attenuation being switched during the double talk phases |  | 3 |
| Many drop outs, cut offs, missing words or syllables, heavily chopped voice, or high constant attenuation being switched during the double talk phases |  | 2 |
| Not possible to hear the other side at all during Double Talk |  | 1 |

Intelligibility / Listening-effort during Double Talk

Description:

The rating scale is applied to evaluate the effort required to understand the meaning of words and sentences during double talk. The evaluation is done on both sides, near-end and far-end. When the evaluation is done on the far-end then the test scenarios should include different background noise conditions at the near-end.

| Test category: | Echo cancellation |
| --- | --- |
| Conversation type: | Double talk |
| Rating side: | Rated at near-end Rated at far-end |
| Test conditions: | For rating at the near-end: - low background noise scenario For rating at the far-end: - stationary background noise scenarios of low/medium/high level; - additional background noise scenarios: different fan/defrost settings; - enclosure dislocation due to movement of the driver; - volume setting in the car: nominal, maximum |
| Scale type: | listening-effort scale, P.800, ACR |

The rating scale is given in the following table.

Table C19: Intelligibility / Listening-effort during Double Talk

| Rating description |  | Grade |
| --- | --- | --- |
| Every word was clearly understood during double talk with no effort required. |  | 5 |
| Speech of the other side is understood during double talk with no appreciable effort required |  | 4 |
| Some words were hard to understand during double talk, moderate listening-effort is required |  | 3 |
| Many words were hard to understand during double talk, considerable listening-effort is required |  | 2 |
| No meaning understood with any feasible effort during double talk |  | 1 |

## C.6	Hands-free System Stability Tests (Car to Car)

The evaluation for system stability is intended to examine the convergence characteristic of the echo cancellation for “closed loop” operation when doing car to car hands-free communication. For the according tests the hands-free system under test is installed in both cars. And both systems having the echo cancellation filter not adapted when starting the test.

System Stability

Description:

The scale is applied for evaluation of the convergence of the echo cancellation and the robustness against back coupling of echo in car to car communication. In one test case single talk at the far-end is performed, in another test case an impulse like noise signal is generated close to the microphone on the far end.

In both cars the EC-filter is not adapted at the start of the test. The evaluation is done on the far-end.

As suggestion an appropriate test procedure could be as following. Both cars are in stand still and have the doors open and the volume set to nominal. Then single talk is performed in both cars one after the other to give the EC-filters the chance to adapt to this situation (or not to be adapted when the doors are closed afterwards). After that in both cars the volume is set to maximum. For generating an impulse like noise for example the doors of the cars could be slamed. Another possibility would be to close the doors quietly and generating the impulse like noise by clapping the hands close to the microphone.

| Test category: | System Stability |
| --- | --- |
| Conversation type: | - Single talk at far-end - Impulse like noise at far-end |
| Rating side: | Rated at far-end |
| Test conditions: | Initial state of EC-filter: not adapted Volume setting in the car: maximum |
| Scale type: | Special scale for HF-System diagnostic evaluation |

The rating scale is given in the following table.

Table C20: System Stability

| Rating description |  | Grade |
| --- | --- | --- |
| No echo is audible |  | 5 |
| Some echo can be heard, but disappears very quickly |  | 4 |
| The echo disappears slowly, the recurrences are audible for a few seconds |  | 3 |
| The echo disappears very slowly, the recurrences are audible for more than 10 seconds |  | 2 |
| The echo builds up like in an unstable feedback system |  | 1 |

______________

The near field is characterized by the distance (measured from the sound source) where the direct sound and the reflected sound are of equal intensity. In acoustics this distance is often referred as critical distance.