| INTERNATIONAL TELECOMMUNICATION UNION |  | Focus Group On "From/In/To Cars Communication II" |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | FG CarCom-C-23 |
|  |  | English only Original: English |
|  |  | Göteborg, 3-4 September 2009 |
| CONTRIBUTION |  |  |
| Source: | Chairman of the Focus Group |  |
| Title: | Input from Mats Forsen / Scott Pennock – Background noise |  |

Handling of the measurement microphone when doing electrical insert of background noise.

The current specification is still a little weak in this area.

Here are some suggestion how to improve the specification.

The general idea is to also do an electrical insert of recorded background noise of the measurement signal where noise is required (D-Value & Speech quality with background noise).

    - Playback of the Recorded Background Noise

Add the following text:

When using electrical or digital insert of background noise also the reference noise signals from measurement microphone shall be using electrical insert of noise.

      - Test

The test arrangement is according to chapter Error: Reference source not found. The microphone is positioned close to the hands-free microphone. In case of arrays the noise recording is made electrically; and the reference noise recording must be made during the noise collection process. The reference recording is made with a measurement microphone positioned close the center of the array.

Change the marked text to the following:

the reference noise recording must be made during the noise collection process and electrically inserted during the measurement.

____________