| INTERNATIONAL TELECOMMUNICATION UNION |  | Focus Group On Car Communication |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | FG CarCOM-C-36 |
|  |  | English only Original: English |
|  |  | Detroit, 16-17 July 2012 |
| CONTRIBUTION |  |  |
| Source: | Volkswagen AG, Technische Universität Braunschweig |  |
| Title: | Proposal of a Speech Component Quality Test for the Signal Enhancement Subsystem in the Send Path in Double Talk |  |

This contribution proposes the inclusion of a send path double talk test for the Signal Enhancement Subsystem described in the current draft of FG VSSR [1]. Aim of this extension is to allow for the measurement of the distortion of the speech component in the send path in double talk (as it was already the case in ITU-T Recommendations P.1100 [2] and P.1110 [3], Sections 8.2.4, respectively). This can be used for optimizations of the Signal Enhancement Subsystem with respect to speech quality during double talk. This test case uses the “adder property” (see DI-S2 and DI-S2inadd in the Figure to the left, from [2,3]) of the S2 interface to the Signal Enhancement Subsystem (see the Figure to the right and [1]).

The double talk test is then carried out in two steps:

- First, a near-end speech signal delivered by the acoustic subsystem is recorded via interface S2 (see Figure to the right).

- Secondly, in real-time processing, the Acoustic Subsystem should deliver the echo signal (or alternatively without Acoustic Subsystem: the echo signal is obtained by real-time convolution of the R2 output with some room impulse response), which is then superimposed by the recorded near-end speech using the signal adder. The resulting enhanced signal is then recorded via interface S3.

The analysis of the speech component distortion in double talk is done by means of the recorded signals and the approach as used in [2] and [3], employing MOS-LQON and MOS-LQOW, respectively.

References:

[1]	ITU-T, “FG CarCOM SubSystem Requirements for Automotive Speech Services”, ITU-T Draft 14 of FG. VSSR, Apr 2011.

[2]	ITU-T, “Narrow-band Hands-free Communication in Motor Vehicles”, ITU-T Recommendation P.1100, Mar 2011.

[3]	ITU-T, “Wideband Hands-free Communication in Motor Vehicles”, ITU-T Recommendation P.1110, Nov 2009.

Attachments:

______________