|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2017-2020 |  | SG12-TD619 |  |
| --- | --- | --- | --- | --- |
|  |  |  | STUDY GROUP 12 |  |
|  |  |  | Original: English |  |
| Question(s): |  | 17/12 | Geneva, 27 November – 6 December 2018 |  |
| TD |  |  |  |  |
| Source: |  | Editors of Y.1540 and new Annex A/Y.1540 |  |  |
| Title: |  | Revisions to Y.1540 and text of new Annex A with Initial Evaluation Plan |  |  |
| Purpose: |  | Proposal |  |  |
| Contact: |  | A.C.Morton, Co-Editor AT&T USA |  | Tel: +1 732-420-1571 Fax: +1 732-368-1192 E-mail: acm@research.att.com |
| Contact: |  | Rüdiger Geib, Co-Editor Deutsche Telekom AG Germany |  | Tel: +49 6151 5812747 E-mail: Ruediger.Geib@telekom.de |

| Keywords: | Capacity, Flow-related Parameters. |
| --- | --- |
| Abstract: | Revisions to incorporate IP Capacity and Flow-related Parameters (Throughput), and Initial Evaluation Plan |

Revisions to existing sections and new text follows:

Revisions to Y.1540, Clause 6.11

(Revisions are TBD)

## 6.11	Capacity parameters

An end-to-end IP packet transfer service traverses an ordered sequence of basic sections from a source host, to a destination host. The capacity parameters described below define properties for basic sections in terms of their ability to carry IP traffic, and corresponding properties for NSEs, also referred to as "paths". It is important to note that a basic section as well as a sequence of basic sections is associated with a direction. The direction is significant, as the properties of a sequence of sections in the forward direction need not be the same as in the reverse direction.

Note that, in contrast to the flow-related parameters defined in clause 6.12, the capacity-related parameters are not dependent on higher layer protocols on top of IP (e.g., TCP).

### 6.11.1 	Section metrics

#### 6.11.1.1	IP-layer bits transferred

For a given population of interest, the IP-layer bits transferred are defined as eight (8) times the number of octets in all IP packets generating successful IP packet transfer outcomes at an egress measurement point, from the first octet of the IP header to the last octet of the IP packet payload, inclusive.

Note that this definition is identical to the definition of IP-layer bits in [IETF RFC 5136]. Also note that the definition of IP-layer bits is IP-version agnostic.

#### 6.11.1.2	IP-layer section capacity

For a given population of interest, the IP-layer section capacity is:

where n0 is the highest number of IP-layer bits that can be transferred over a basic section generating successful IP packet transfer outcomes at the egress measurement point during a specified time interval [t, t + Δt].

Revisions to Y.1540, Clause 6.12 (from C-160)

## 6.12	Flow-related parameters

It is useful to characterize performance in terms of flow or throughput-related parameters that evaluate the ability of IP networks or sections to carry quantities of IP packets. It should be noted that a parameter intended to characterize the throughput of an IP application would not be equal to the amount of resources available to that application (as quantified in clause 6.11); this is because the higher layer protocols over IP (e.g., TCP flow control) also influence the IP-layer throughput experienced.

In the present version of this Recommendation, it is recommended that all flow- or throughput-related parameters should fulfil the following requirements:

1)	A parameter characterizing the throughput offered to an IP service should relate the amount of IP packets successfully transported by an IP network or section to the amount of IP packets that were delivered into this network or section.

2)	The throughput-related parameter should apply to an end-to-end IP network and to the IP transport across an EL, NS or NSE.

Some flow- or throughput-related parameters attempt to characterize the throughput capacity of an IP network, i.e., its ability to sustain a given IP packet transfer rate. It is recommended that any such parameters fulfil the following additional requirements:

1)	The traffic pattern offered to the IP network or section should be described, since the ability of the IP network or section to successfully deliver these packets depends on this traffic pattern.

2)	The rate at which traffic is offered should not exceed the capacity (in bits per second) of the link that connects the sections under test with the destination sections that are not under test.

3)	In any individual statement about throughput performance, the type of IP packet considered should be declared (where type includes the IP version, the presence of extension headers, the transport layer protocol, the other protocol headers, and any other relevant information, such as the length of the packets used).

It is also recommended to follow the guidelines for throughput-related parameters and their measurement found in the IETF RFC 3148 framework for bulk transfer capacity (BTC) metrics.

Parameters related to flow and throughput are specified in Annex ??.. Appendix IX describes how measurements using TCP do not meet the requirements of this clause.

Editor’s Note: Appendix III may be deleted or reconciled with the new Annex.

Annex A

IP-Based Flow-related Parameters and Methods of Measurement

(This annex forms an integral part of this Recommendation.)

#### A.1 Background

Standardisation of measurement architectures characterising “the Internet service” started in 2013. A consumer network access product supporting the Internet Protocol is a common way of connecting to the Internet. The majority of consumer applications communicate using TCP transport. TCP provides reliable datagram transmission. When trying to characterise an Internet service by standardised measurements, methods to characterise TCP Transport layer properties are often proposed for the judgement of Internet- or Link-Layer properties, like access bandwidth. Concerns have been raised, that this approach may not work properly and competing standards are proposed to characterise consumer Internet services based on measurements. This Annex specifies parameters for both purposes, access bandwidth measurement and reliable transport measurement.

SG12 is experienced in developing measurement- and modelling-standards for telecommunication. Further, expertise on statistical methodology and toolsets allowing an objective comparison between proposed measurements and modelling methods are available. Using SG12 expertise and methods to decide on the applicability of competing proposals related to the characterisation of  “consumer Internet service” is agreed as a good way to move from an exchange of views to a proof and comparison of concepts under reproducible testing conditions.[P.800] provides the following requirements, :

It is important that the conditions simulated in the test are correctly specified and set up, and measured accurately before and after each experiment; … and that faithful records of the output of each test are kept.

The IETF has approved and published RFC 8337, “Model-Based Metrics for Bulk Transport Capacity”, in March 2018 [RFC8337]. The Model-Based Metrics (MBM) work was the result of many years considering the problem of transport capacity measurement, primarily in the IETF IP Performance Metrics (IPPM) working group. The specification carefully describes the many issues and difficulties with repeatability when testing with standard-compliant TCP (section 4), and solves these problems principally by designing a method and a set of diagnostic tests where TCP’s flow-control is disabled. The method involves the evaluation of a target transport performance in terms of transmission rate and Round-trip time (RTT).

Testing conditions for relevant measurement methods are included at the end.

#### A.2 IP-Layer Access Bandwidth Parameters and Methods of Measurement (Consumer Internet Access)

## A.2.1	Parameter Definition

(see C-160 and Q17 Report)

## A.2.2	Method of Measurement

## A.2.2	Related Parameters and Methods

#### A.3 Flow-related Throughput Parameters and Methods of Measurement (Reliable delivery transport)

## A.3.1	Parameter Definition

(see C-160 and Q17 Report)

## A.3.2	Method of Measurement

## A.3.2	Related Parameters and Methods

#### A.4	Plan to qualify and compare access measurement methods

ITU-T Recommendation [P.800] and other recommendations of that series, although primarily aimed at voice, provide general guidance on how to set up, perform and evaluate measurement campaigns aiming to compare models based on measurement input with true conditions. To understand the capabilities and the limits of tests, tools and results designed to characterize Internet access types, a similar process is useful, as described below.

A set of various transmission conditions should be arranged for laboratory testing. The transmission conditions that are close to real operational conditions should first be defined. The conditions should be stable and verifiable during the tests, using laboratory measurement equipment. Then each method designed to characterize various types of Internet access can be tested against a set of various conditions. Measurement errors can be characterized. From the performance of each parameter and method, operational areas (conditions) can be identified where a measurement method has merits and where it is less reliable (and sources of measurement error can be quantified).

The testing conditions are listed below.

Editor’s Note: If there is technical interest of the interested parties and manpower available to pursue the process proposed here, then the combined group will first draft an improved set of conditions and plan.

## A.4.1	Measurement Points

[RFC7398] defines a reference path and measurement points for commonly used performance metrics.  Other similar measurement projects may also be able to use the extensions described here for measurement point location.  The purpose of [RFC7398] is to create an efficient way to describe the location of the measurement point(s) used to conduct a particular measurement, especially pointing-out when a measurement includes managed and non-managed (Private Network) subpaths.

Note that the measurement path, as delimited by the [RFC7398] measurement points, determines the applicability of subscription parameters, such as typical data rates offered and whether the subscription parameters inform the selection of MBM parameters such as target_data_rate.

## A.4.2	Laboratory testing conditions

All configurations and all background traffic behavior should be as close as possible to operational networking conditions. It is suggested to define a baseline test and then to vary testing parameters. This is the Phase 1 testing.

## A.4.2.1	Phase 1 Laboratory testing conditions

Following on from the BEREC Requirements, [BEREC] the first phase test will be with Shapers (and policers up to 10Mbps, [Google-Police]).  In addition, network impairments such as delay will be implemented and tested.

The Figure below illustrates two different utilities providing flexible traffic/rate control on a general purpose computing platform. The different traffic control utilities can be applied in three ways.

Figure 1   Three Alternative Test Paths on a General Purpose Computing Platform

In Figure 1, the test device is connected to a general purpose host via physical links at 10 Gbps. The test device is also a general purpose host, but it is completely isolated from the host performing traffic control, allowing each node to dedicate resources to their unique roles in the test environment. Different implementations of candidate measurement methods can be installed and tested through the traffic control node.

There are three alternatives to implement the traffic shaper function. The first on the left employs the linux kernel-equipped emulator, netem, which can emulate delay and assist with the traffic control after configuration for the correct NIC and physical interfaces. The vSwitch simply switches frames between its two ports. This configuration is commonly referred to as “phy2phy”.

The second alternative (middle) assumes that the Intel DPDK testpmd utility will be installed and configured to perform frame forwarding between the correct physical interfaces, along with control of bandwidth passing through the forwarding path.

The last alternative (right) uses testpmd installed in a Virtual Machine, or VM, and employs a vSwitch with configuration to connect the physical interfaces to the correct ports on the VM. Again, testpmd (or other utility running in the VM) controls the bandwidth of the path between the logical ports.

All three of these configurations are possible on the OPNFV VSPERF project pod, assigned for testing, development of, and evaluations using the VSPERF tool [Pod12].

Different conditions may then apply for single tests. In each test, only a single property is changed against the baseline test, all other configuration properties remain unchanged.

- The shaper/policer bandwidth are set to the rates as suggested by BEREC, up into the Gbps range, and will be tested.

- RTTs are set to the following values: 5, 10, 20, 40, 80, 160 ms.

- The burst tolerance of the shaper may be set to 0 or up to 10% of the shaper bandwidth. The value isn’t determined here and will be published when reporting the results.

- The methods of rate-limiting are shaping or policing.

- The Queue overflow strategy is tail drop.

- The tests will be executed without background traffic. An additional test may be performed with background traffic. The results and the results and the average background load will be published with the results.

- All test and background traffic will be best effort.

- The maximum measurement interval for an individual measurement is 30 seconds.

- For each individual measurement, tolerance of configured bandwidth as compared to a bandwidth measured during a test must be within 5% to accept a bandwidth as correctly measured by the candidate measurement system.

- The measurement software as used for the test must be available under an open source license. Also, the details of the development environment and required operating system must be provided.

Editor’s note: connect with the G.191 open source procedure.

- Results will be published as a supplement to this recommendation.

- The IP packet sizes correspond to a layer 2 frame size of 64, 256, 512 Bytes and in addition max IP MTU size of 1500 bytes.

Standard lab test equipment may be used to calibrate network properties [insert reference to TST 009] like maximum IP layer bandwidth, minimum and maximum round trip time, buffer depth and so on prior to every single measurement tool per network test condition. In this way, network performance benchmark information against which to compare measurement tool results and related evaluations can be captured.

The benchmark for correct shaper operation according to configuration is UDP CBR transmission, and ultimately for correct assessment of the channel conditions (consistent with [PAM-12]). If the UDP capacity and Shaper configuration differ, a third arbiter of correctness is packet capture of the UDP stream. Identification and discussion of any errors encountered is essential in benchmarking.

Editors note: preliminary laboratory tests for calibration and measurement method evaluation have been completed [AT&T TD from Lab_Eval.doc] [TST009]. Taking guidance from the existing text of Annex A/Y.1540 and the BEREC requirements for verifying measurement tools, AT&T tested one of the three alternative configurations using a general purpose compute platform that will meet the requirements (phy2phy). Key take-aways from this testing:

- The Bidirectional Throughput was 213.85 Mbps, fairly close to the configured value of 100 Mbps x2. The excess may be an artifact of the burst size allowed.

- The Binary Search with Loss verification was configured to allow a fairly large tolerance in Mbps to accept the result.  This tolerance should be re-considered when testing sub-Gbps rates.

## A.4.2.2	Phase 2 Laboratory testing conditions

Phase 2 tests will be conducted in a similar manner to the 2012 work by Goga and Teixeira [PAM-12].  From available knowledge of the measurement systems in widespread use today, none are using the method that has been shown in the past to produce the most accurate estimate of IP Capacity – UDP-based measurements [PAM-12]. Unfortunately, [PAM-12] was conducted when typical access capacities were <50 Mbps, and now (over 5 years later), this capacity range has even been exceeded on mobile Internet access. Key Take-away points from [PAM-12] are:

- iperf was used to emulate ﬂooding-based tools, because it allows us to set the number of parallel connections, the duration or the size of the transfer among other parameters.

- “Benchmark. The first line of Table. 1 Shows the UDP capacity obtained when flooding the link with iperf in UDP.…. The UDP capacity represents the maximum achievable IP rate for each line.”

- Spruce, pathload with large probes, and parallel TCP are the most accurate tools to estimate available bandwidth (a.k.a. residual capacity), but there are regions where parallel TCP tests fail to produce useful results, namely if unidentified parallel traffic is present.

The general drawbacks of existing measurement systems in use during the 2012 time-frame are analyzed in [MortonPQS].

Phase 2 tests should be conducted for each basic access type.

Basic access types:

Wireline: DSL consumer access, broadband cable access, fiber access, others.

Wireless: WiFi hot spot, UMTS, LTE wireless access, others.

Editor’s Note: <Table of conditions for each access type, across the world! This is where we could seek guidance from SG12 experts, BBF, 3GPP, CableLabs, etc., including the feasibility and value of the possible range of variables used in testing. This allows coverage assessment for all the metrics considered. >

The results and test paths used in Phase 1 are defined serving as a baseline for Phase 2. This can also be a basic access, e.g. an access having typical properties for a national telecommunication market. The same holds for all other parameters, e.g. a typical RTT for popular content should be assumed, background traffic should be typical and so on. Thus, the baseline access and networking conditions should be close to a real subscriber environment.

Different conditions may then apply for single tests. In each test, only a single property is changed against the baseline test, all other configuration properties remain unchanged.

- The wireline bandwidth may be lowered and raised to other bandwidths which are available.

- The RTTs may be decreased and increased.

- The burst tolerance of the access may be changed.

- The method of rate-limiting may be changed (shaping or policing).

- Active Queue Management may be activated.

- Background traffic may be added on the access (with or without Diffserv features, as may be expected in a particular market).

- Congestion may be added in another part of the network (which should however behave like in a real network, i.e., assume a buffer to be assigned to the congested network interface  and other traffic of varying throughput to be present).

Standard lab test equipment may be used in addition to benchmark network properties < either End-to-End, or the set of individual devices and their composed end-to-end performance> like maximum IP layer bandwidth, minimum and maximum round trip time, buffer depth and so on prior to every single measurement tool per network test condition. Thus we obtain network performance benchmark information against which to compare measurement tool results and related evaluations can be captured.

Needed: Specifications of all packet related aspects, beginning with packet size.

References for Annex A

P.1401, Methods, metrics and procedures for statistical evaluation, qualification and comparison of objective quality prediction models, ITU-T, 2012.

[P.800]  P.800,  Methods for objective and subjective assessment of quality, ITU-T, 1996.

[Google-Police] “An Internet-Wide Analysis of Traffic Policing”, Flach, Papageorge et al., University of Southern California and Google, 2016.

“A Deterministic TCP Bandwidth Sharing Model”, Lautenschlaeger, Bell-Labs Alacatel-Lucent
https://arxiv.org/ftp/arxiv/papers/1404/1404.4173.pdf

“Extending the TCP Steady-State Throughput Equation for Parallel TCP Flows”, Damjanovic, Welzl et al., University of Innsbruck, Budapest University of Technology.

http://heim.ifi.uio.no/~michawe/research/publications/mulPadhye-TechnicalReport.pdf

https://tools.ietf.org/html/rfc6582

https://www.rfc-editor.org/rfc/rfc8312.txt

“draft-ietf-quic-recovery-11”, Iyengar and Swett. work in progress, IETF 2018. https://datatracker.ietf.org/doc/draft-ietf-quic-recovery/

“draft-ietf-tsvwg-aqm-dualq-coupled-04”, Briscoe, De Schepper, et al., work in progress, IETF 2018. https://datatracker.ietf.org/doc/draft-ietf-tsvwg-aqm-dualq-coupled/

[RFC2330]  Paxson, V., Almes, G., Mahdavi, J., and M. Mathis, "Framework for IP Performance Metrics", RFC 2330, DOI 10.17487/RFC2330, May 1998. Available from  https://www.rfc-editor.org/info/rfc2330

[RFC6673]  Morton, A., "Round-Trip Packet Loss Metrics", RFC 6673,  DOI 10.17487/RFC6673, August 2012,  https://www.rfc-editor.org/info/rfc6673

[RFC7398]  Bagnulo, M., Burbridge, T., Crawford, S., Eardley, P., and A. Morton, "A Reference Path and Measurement Points for Large-Scale Measurement of Broadband Performance", RFC 7398, DOI 10.17487/RFC7398, February 2015. Available from  https://www.rfc-editor.org/info/rfc7398

[RFC8337] Mathis, M. and Morton, A. , IETF RFC 8337, Model-Based Metrics for Bulk Transport Capacity, March 2018. Available from  https://www.rfc-editor.org/info/rfc8337

[MortonPQS] Morton, A., “Improved Internet speed tests can enhance QoS and QoE”, Proceedings of the 4th International Workshop on Perceptual Quality of Systems (PQS 2013), held in Vienna, Austria, 2-4 September, 2013.  (Available from the author)

[Mou]  Mou, M. “Evaluating a TCP Model-Based Network Performance Measurement Method”, Masters Thesis at MIT, June 2017. Available from https://dspace.mit.edu/handle/1721.1/113177

[PAM-12] 	Oana Goga and Renata Teixeira, “Speed Measurements of Residential Internet Access”, Passive and Active Measurements Conference, PAM-12. Available from https://people.mpi-sws.org/~ogoga/papers/PAM12-speed.pdf

[TST009] https://docbox.etsi.org/ISG/NFV/Open/Drafts/TST009_NFVI_Benchmarks

[BEREC] BoR (18) 32: TENDER SPECIFICATIONS, No BEREC/2018/01/OT Net Neutrality Measurement Tool, March 2018

_______________________