|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2022-2024 |  |  | SG11-TD401/GEN |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | STUDY GROUP 11 |  |
|  |  |  |  | Original: English |  |
| Question(s): |  |  | 2/11 | Geneva, 10-19 May 2023 |  |
| TD |  |  |  |  |  |
| Source: |  |  | Editors |  |  |
| Title: |  |  | Output – initial baseline text of draft Recommendation ITU-T Q.TSCA “Procedure for issuing digital certificates for signalling security” (Geneva, 10-19 May 2023) |  |  |
| Contact: |  | Assaf Klinger Vaulto Technologies Ltd. Israel |  |  | Tel: +972502138133 E-mail: assaf.klinger@gmail.com |
| Contact: |  | Shi Minrui China Telecom China |  |  | Tel: +86-18918588657 E-mail: shimr@chinatelecom.cn |

| Abstract: | This document contains the baseline text of new draft Recommendation Q.TSCA “Procedure for issuing digital certificates for signalling security” which was presented at the Q2/11 meeting held in Geneva on 10-19 May 2023. |
| --- | --- |

This document is based on SG11-C112 and contains revisions discussed at the Q2/11 which was held during SG11 meeting (Geneva, 10-19 May 2023).

Background

Signalling protocols play a cornerstone role in providing different ICT services from the simple audio/video sessions to the complex digital financial services widely used over the globe. These protocols and telecommunication networks were designed without consideration for security and privacy. It enables attacks on ICT infrastructure including exploiting signalling protocols used for different ICT services.

While many different domains are using the Internet to build trustable connection among their customers. For instance, most of the financial institutions are widely using the Internet to give their customers more effective tools to control and manage their finances. The lack of security and privacy in existing ICT infrastructure does not enable such trustable connections. Furthermore, in developing countries, where access to financial services is limited only to legacy ICT infrastructure via over-the-top (OTT) applications we see an ever-growing increase of illegal usage of customers’ applications, thus resulting in the unlawful take-over of their assets.

Additionally, many people all over the globe experience the annoying phone calls or calls from parties pretending to be legitimate business ventures (e.g., representatives of banks, health insurance companies, etc.). Technically, these calling parties use the so-called spoofing number – which in essence is the way the calling party number can be replaced with the number of an official enterprise or anyone of trust. As a result, the spoofing numbers as well as robocalls, along with other similar attacks make lives of the customers uncomfortable and unsecure to say the least.

In summary, the signalling exchange level of security and privacy must match the level provided by the Internet to mitigate attacks on ICT infrastructure which breaks signalling protocols used for establishing different ICT services. Amongst the well-known attacks are telephone spam, spoofing numbers, location tracking, subscriber fraud, intercept calls and messages, DoS, infiltration attacks, routing attacks, etc. These attacks have become a major priority for different stakeholders, in particular the financial institutions and telecom operators. The ITU-T Study Group 11, which is the lead group on signalling, has been working on these issues since 2016.

Regarding spoofing calling party number, which is considered one of the major issues, ITU-T SG11 has revised Recommendations ITU-T Q.731.3, Q.731.4, Q.731.5 and Q.731.6, in order to specify an exceptional procedure for transit exchange connected to CPE (Customer Premises Equipment) with the aim of providing predefined calling party number by the originating operator.

To cope with issues related to intercepting messages and calls, including One-Time-Password which is widely used in the financial institutions, SG11 had developed new Recommendation ITU T Q.3057 (2020), which defines the signalling architecture and requirements for interconnection between trustable network entities in support of existing and emerging networks. This Recommendation describes the use of digital signature (digital certificates) in the signalling exchange which may guarantee the trustworthiness of the sender.

Furthermore, SG11 developed Recommendation ITU-T Q.3062 (2022) which defines algorithms for checking certificates for different protocols using Signalling Security Gateway (SSGW), which validates the signatures of other operator's certificates to allow or block the signalling packets.

Finally, SG11 developed standard ITU-T Q.3063 (2022) which identifies the signalling requirements of calling line identification authentication including codes and signalling procedure based on the mechanism defined in the ITU-T Q.3057.

To implement all the above-mentioned ITU-T Recommendations there is a need a trusted authority which may issue such certificates on the global level to be recognized by all parties.

More information about SG11 activities on signalling security are available on dedicated webpage at: https://itu.int/go/SIG-SECURITY.

Proposal

The implementation of the approaches highlighted in ITU-T Q.3057, Q.3062 and Q.3063 require to setup a global authority which may issue such certificates.

According to the Workshop and two Webinars (Episode 1 and Episode 2), which SG11 organized in 2021 and 2022, it was noted there is a need “…a globally trusted SDO, preferably one already in charge of numbering” (see key takeaways). Also, during the ITU Webinar Episode-2 (in November 2022), on the request received from the floor, “…Who would provide the global director of certification authorities? Could ITU do that?...” the answer was “…the trust anchor for the trust model for those tokens definitely can be ITU as UN specialized agency which is responsible for International Numbering Resources and it is globally trusted organization…” (see Q/A script).

In this regard, taking into consideration all the above-mentioned points as well as the set of ITU-T Recommendations (ITU-T Q.3057, Q.3062 and Q.3063), which require such mechanism, it is proposed to initiate a new work item – draft Recommendation Q.TSCA “Procedure for issuing digital certificates for signalling security”. The proposed draft Recommendation will require the trust anchor to maintain this procedure and define all required details, including application forms, etc.

The proposed baseline text of the draft Recommendation Q.TSCA “Procedure for issuing digital certificates for signalling security” is below in the Annex 1.

Annex 1

Draft Recommendation ITU-T Q.TSCA

Procedure for issuing digital certificates for signalling security

Summary

Signalling protocols play a cornerstone role in providing different ICT services from the simple audio/video sessions to the complex digital financial services widely used over the globe. These protocols and telecommunication networks were designed without consideration for security and privacy. It enables attacks on ICT infrastructure including exploiting signalling protocols used for different ICT services.

While many different domains are using the Internet to build trustable connection among their customers. For instance, most of the financial institutions are widely using the Internet to give their customers more effective tools to control and manage their finances. The lack of security and privacy in existing ICT infrastructure does not enable such trustable connections. Furthermore, in developing countries, where access to financial services is limited only to legacy ICT infrastructure via over-the-top (OTT) applications we see an ever-growing increase of illegal usage of customers’ applications, thus resulting in the unlawful take-over of their assets.

Additionally, many people all over the globe experience the annoying phone calls or calls from parties pretending to be legitimate business ventures (e.g., representatives of banks, health insurance companies, etc.). Technically, these calling parties use the so-called spoofing number – which in essence is the way the calling party number can be replaced with the number of an official enterprise or anyone of trust. As a result, the spoofing numbers as well as robocalls, along with other similar attacks make lives of the customers uncomfortable and unsecure to say the least.

In summary, the signalling exchange level of security and privacy must match the level provided by the Internet to mitigate attacks on ICT infrastructure which breaks signalling protocols used for establishing different ICT services. Amongst the well-known attacks are telephone spam, spoofing numbers, location tracking, subscriber fraud, intercept calls and messages, DoS, infiltration attacks, routing attacks, etc. These attacks have become a major priority for different stakeholders, in particular the financial institutions and telecom operators. To implement security for signalling messages and for calling line identifiers ITU-T SG11 developed recommendations ITU T Q.3057 (2020), ITU-T Q.3062 (2022) and ITU-T Q.3063 (2022). All of which rely on ITU-T X.509 digital certificates for proof of identity. These recommendations raised a for need a trusted authority and root certification authority which will issue and sign digital public key certificates on the global level to national or reginal certification authorities to create a global trust anchor to be recognized by all parties.

Keywords

Authentication, trust, signalling procedures, certification authority, digital signature, trust anchor, data integrity, public key certificate, end entity public key certificate.

Table of Contents

1	Scope	5

2	References	5

3	Definitions	5

3.1	Terms defined elsewhere:	5

3.2	Terms defined in this Recommendation:	5

4	Abbreviations and acronyms	6

5	Conventions	6

6	Procedure for issuing digital certificates	7

6.1	Procedure for verification of IEs for the issuance of a PEEC	8

6.2	Procedure for verification of IEs for the issuance of a VEEC	8

6.2.1	Request Information Verification – Overview	8

6.2.2	Verification of Applicant’s Legal Existence and Identity	8

6.2.3	Verification of Applicant’s Physical Existence	8

6.2.4	Verification of Applicant’s Operational Existence	8

6.2.5	Verification of Applicant’s CSP license	8

6.2.6	Verification of Name, Title, and Authority of Contract Signer	8

6.2.7	Verification of Signature on Applicant Agreement and EEC Requests	8

6.2.8	Other Verification Requirements	8

Appendix A Recommended list of verifiable CSP identifiers to be sent in the PEEC request	9

List of figures

Figure 1 - Signalling procedures of a trusted signalling certification authority	7

Draft Recommendation ITU-T Q.TSCA

Procedure for issuing digital certificates for signalling security

# Scope

This Recommendation specifies the procedure for the allocation of digital certificates for signalling security in support of existing and emerging networks.

# References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T X.509]	Recommendation ITU-T X.509 (2019), Information technology – Open Systems Interconnection – The directory: Public-key and attribute certificate frameworks.

[IETF RFC 4703]	Resolution of Fully Qualified Domain Name (FQDN) Conflicts among Dynamic Host Configuration Protocol (DHCP) Clients

[ITU-T Q.3057]	Recommendation ITU-T Q.3057 (2020) Signalling requirements and architecture for interconnection between trustable network entities.

[ITU-T Q.3062]	Recommendation ITU-T Q.3062 (2022) Signalling procedures and protocols for enabling interconnection between trustable network entities in support of existing and emerging networks.

# Definitions

## Terms defined elsewhere:

This Recommendation uses the following terms defined elsewhere:

3.1.1	authority [ITU-T X.509]: An entity, responsible for the issuance of certificates or of revocation lists.

3.1.2	certification authority (CA) [ITU-T X.509]: An authority trusted by one or more entities to create and digital sign public-key certificates. Optionally the certification authority may create the subjects' keys.

3.1.3	trust anchor [ITU-T X.509] An entity that is trusted by a relying party and used for validating public-key certificates.

3.1.4	trusted signalling certification authority (TSCA) [ITU-T Q.3057]: The root of trust for verifying digital signatures using the root certification authority (CA) model.

3.1.5.	fully qualified domain name (FQDN) [RFC 4703]: Network entity address indicator on Internet protocol interconnect.

## Terms defined in this Recommendation:

None

# Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

CA		Certification Authority

CSP		Communications Service Provider

DS		Digital Signature

DSA		Digital Signature Algorithm

ECDSA		Elliptic Curve Digital Signature Algorithm

EEC		End Entity public-key Certificate

FQDN		Fully Qualified Domain Name

IE		Information Element

PEEC		Provisional End Entity public-key Certificate

TSCA		Trusted Signalling Certification Authority

VEEC		Validated End Entity public-key Certificate

# Conventions

None

# Procedure for issuing digital certificates

ITU-T Q.3057 specifies the signalling procedure of the TSCA, shown in Figure 1

Figure 1 - Signalling procedures of a trusted signalling certification authority

The TSCA issues two types of digital certificates:

- PEEC – issued automatically upon verification of IEs sent in the certification request.
Note: PEEC will be granted after automatic verification by TSCA and it is valid for 6 months. It is noted that VEEC will follow and in the event of misalignment, it may result in revocation of PEEC.

- VEEC – issued upon verification of the requestor’s identity and their eligibility to obtain a digital certificate from the TSCA.
Note: Once the VEEC is granted, it is valid for two years.

The trust anchor has the responsibility for the allocation and management both the PEEC and VEEC. Both PEEC and VEEC should be ITU-T X.509 digital certificates.

CSPs who wish to obtain a VEEC should send a certificate request to the TSCA using the TSa interface defined in [ITU-T Q.3062] the certificate request should contain the list of trust anchor defined IEs, in addition to the CSPs public-key and be signed using the CSPs private-key as proof-of-possession.

## Procedure for verification of IEs for the issuance of a PEEC

Upon receiving a certificate request, from a requestor via the TSCA, the trust anchor will validate the IEs from the certificate request. The trust anchor will define the list of required IEs to be validated, a recommendation of such IEs can be found in Appendix A of this Recommendation. Once IEs have been verified, the TSCA shall issue the requestor a PEEC over the TSa interface defined in [ITU-T Q.3062].

## Procedure for verification of IEs for the issuance of a VEEC

Before issuing a VEEC, the trust anchor must ensure that all information submitted in the certificate request conforms to the requirements of, and has been verified in accordance with, the trust anchor’s guidelines and matches the information confirmed and documented by the TSCA pursuant to its verification processes (prior to issuing the PEEC). Such verification processes are intended to accomplish the following:

- Verify Applicant’s existence and identity, including:

  - Verify Applicant’s legal existence and identity.

  - Verify Applicant’s physical existence (business presence at a physical address).

  - Verify Applicant’s operational existence (business activity).

- Verify Applicant is a registered CSP, or has exclusive control of a CSP license, in the jurisdiction to be included in the VEEC.

- Verify Applicant’s authorization for the certificate request, including:

  - Verify the name, title, and authority of the Contract Signer.

  - Verify that Contract Signer signed the Applicant Agreement.

### Request Information Verification – Overview

As a general rule, the trust anchor is responsible for taking all verification steps reasonably necessary to satisfy each of the Verification Requirements set forth in the sections below. The acceptable methods of verification shall be defined by the trust anchor. In all cases, the trust anchor is responsible for taking any additional verification steps that may be reasonably necessary under the circumstances to satisfy the applicable verification requirement.

### Verification of Applicant’s Legal Existence and Identity

TBD

### Verification of Applicant’s Physical Existence

TBD

### Verification of Applicant’s Operational Existence

TBD

### Verification of Applicant’s CSP license

TBD

### Verification of Name, Title, and Authority of Contract Signer

TBD

### Verification of Signature on Applicant Agreement and EEC Requests

TBD

### Other Verification Requirements

TBD

Appendix A
Recommended list of verifiable CSP identifiers to be sent in the PEEC request

Following below is a recommendation of IEs to be sent to the TSCA in the certificate request, that can be verified automatically, to issue a PEEC to the requestor, the IEs recommended in this list are machine verifiable information elements about the applicant.

    - The applicant information should include, but is not limited to, the following details:

- Organization name. Include the formal legal organization name of the applicant in the EEC, as recorded with the incorporating or registration agency in the applicant’s jurisdiction of incorporation or registration (for private organizations), or as specified in the law of the political subdivision in which the government entity operates (for government entities), or as registered with the government business registration agency (for business entities).

- Assumed name (optional). Include the assumed name (e.g., “doing business as” appellation) of the applicant in the EEC, as recorded in the jurisdiction of Applicant’s Place of the place of business of the applicant, if requested by .the applicant.

- Security domain name (FQDN). Include the domain name(s) of the applicant in the EEC.

- Jurisdiction of incorporation or registration. Include the jurisdiction of incorporation or registration of the applicant in the EEC, consisting of:

  - city or town (if any).

  - state or province (if any).

  - country.

- Incorporating or registration agency. Include the name of Applicant’s the incorporating or registration agency of the applicant in the EEC.

- Registration number. Include the registration number assigned to applicant by the incorporating or registration agency in the jurisdiction of incorporation or registration of the applicant in the EEC. If the incorporating or registration agency does not issue registration numbers, then the date of incorporation or registration shall be recorded.

- Applicant address. Include the address of the place of business of the Applicant in the EEC, including:

- building number and street.

- city or town.

- state or province (if any)

- country.

- postal code (zip code).

- main telephone number.

- Public-key certificate requester. Include the name and contact information of the public-key certificate requester submitting the EEC request on behalf of the applicant.

- The applicant agreement includes the following elements.

  - General. Prior to the issuance of the EEC, the TSCA obtains the agreement of the applicant to a legally enforceable applicant agreement with the trust anchor for the express benefit of relying parties. The applicant agreement must be signed by an authorized contract signer acting on behalf of the applicant in accordance with local or peer SSGW law and must apply to the EEC to be issued pursuant to the EEC request. A separate Applicant agreement may be used for each EEC request, or a single applicant agreement may be used to cover multiple future EEC requests and resulting EEC(s), so long as each EEC that the TSCA issues to the applicant is clearly covered by an applicant agreement signed by an authorized contract signer acting on behalf of the applicant.

  - Agreement requirements. The applicant agreement must, at a minimum, specifically name both the applicant and the individual contract signer signing the agreement on behalf of the applicant and contain provisions imposing on the applicant the following obligations and warranties.

    - Accuracy of information: An obligation and warranty to provide accurate and complete information at all times to the TSCA, both in the EEC request and as otherwise requested by the TSCA in connection with the issuance of the EEC(s) to be supplied by the TSCA.

    - Protection of private key. An obligation and warranty by the applicant or a subcontractor (e.g., hosting provider) to take all reasonable measures to maintain sole control of, keep confidential and properly protect at all times the private key that corresponds to the public key to be included in the requested EEC(s) (and any associated access information or device, e.g., password or token).

    - Acceptance of EEC. An obligation and warranty that it will not install and use the EEC(s) until it has reviewed and verified the accuracy of the data in each EEC.

    - Use of EEC. An obligation and warranty to install the EEC only on a SSGW accessible at the security domain name listed on the EEC and to use the EEC solely in compliance with all applicable laws, solely for signalling and solely in accordance with the Applicant Agreement

    - Reporting and revocation. Upon compromise, an obligation and warranty to promptly cease using an EEC and its associated private key, and promptly request the TSCA to revoke the EEC, if:

      - any information in the EEC is or becomes incorrect or inaccurate, or there is any actual or suspected misuse or compromise of the private key of the applicant associated with the public key listed in the EEC.

Termination of use of EEC. An obligation and warranty to promptly cease all use of the private key corresponding to the public key listed in an EEC upon expiration or revocation of that EEC.

_____________________