| INTERNATIONAL TELECOMMUNICATION UNION |  | Focus Group On Future Networks |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | FG-FN OD-56 |
|  |  | Original: English |
|  |  | 6th FG-FN meeting: Geneva, Switzerland, 6–9 September 2010 |
| OUTPUT DOCUMENT 56 |  |  |
| Source: | Editors |  |
| Title: For: | Framework of network virtualization [Ed Note 1] Should be decided if “Future Networks” is to be included in the title later. |  |

0. Contents

1.	Scope	x

2.	References	x

3.	Definitions	x

3.1. Terms defined elsewhere	x

3.2. Terms defined in this document	x

4.	Abbreviations and acronyms	x

5.	Introduction	x

6.	Overview of network virtualization	x

6.1. LINP architecture in network virtualization framework	x

7.	Problem spaces	x

8.	Attributes of network virtualization	x

9.	Applicability	x

10.	Implementation examples	x

10.1. Router aspects	x

10.2. Application aspects	x

10.3. Mobility aspects	x

10.4. Other aspects	x

11.	Security considerations 	x

Annex A : Use cases 	x

[Ed Note 2]: It is necessary to keep consistency in using terminologies in the documents

1. Scope

The scope of this technical document includes:

- Definition of network virtualization

- Overview of network virtualization

- Problem spaces

- Attributes of network virtualization

- Applicability and use cases

- Implementation examples

[Ed Note 6] It is noted that whole procedures realizing network virtualization (provisioning, tool, etc) needs to be described in a separate section in order to complete the framework document. Further contributions are invited.

2. References

[1] GENI Design Principles, GDD-06-08, August 11, 2006.

[2] Susanna Avéssta, FIREworks D2.7 Draft Cooperation Concept, 2009.

[3] The GENI Project Office, “GENI (Global Environment for Network Innovations) System Requirements Document,” July 7, 2009.

[4] http://www.geni.net/

[5] “Design for Future Internet Service Infrastructures”, p. 227, Towards the Future Internet G. Tselentis et al. (Eds.) IOS Press, 2009

[6] “Proposal for a new project description on virtualization of multilayer transport network”FGFN C-xx, the third FGFN meeting

[7] S. Jeong, et al., “Draft Deliverable - Framework of Network Virtualization,” in the Proc. of ITU-T FG-FN, Salt Lake City Meeting, 2009.

[8] N.M. Mosharaf, et al., “A Survey of Network Virtualization,” Technical Report: CS-2008-25, October 15, 2008.

3. Definitions

3.1. Terms defined elsewhere

[TBD]

3.2. Terms defined in this document

3.2.1 network virtualization: A technology that enables the creation of logically isolated network partitions over shared physical network infrastructures so that multiple heterogeneous virtual networks can simultaneously coexist over the shared infrastructures. Also, network virtualization allows the aggregation of multiple resources and makes the aggregated resources appear as a single resource.

4. Abbreviations and acronyms

LINP	Logically Isolated Network Partition

FN	Future Network

5. Introduction

Future Network (FN) is a network which will be able to provide revolutionary services, capabilities, and facilities that are hard to support using existing network technologies. Also, it is expected that Future Network will overcome the limitations of the current networks. Future Network includes core technologies that are necessary for constructing future networking infrastructure and application service infrastructure. It is necessary that those core technologies are to be designed not bound to the current networking technologies, because the core technologies will be developed in order to overcome the limitations of the current networks. It is required to construct testbeds so as to design, develop, and evaluate the revolutionary core technologies. Especially, developing the core technologies may require complete new network environment from the current network architecture and the operation of one technology should not make unexpected effects to others. It is also considered one of the main promising technologies for construction of Future Networks, as reflected in the extensive current research carried out globally. Therefore, the testbeds should provide isolated network environment to the developers, providers, and users of the core technologies. Network virtualization is the prominent technology that can realize the isolation of networks and can be used to build the large scale testing infrastructure for Future Network technologies.

6. Overview of network virtualization

The virtual networks are completed isolated each other, so different virtual networks may use different protocols and packet formats. When combined with programmability in network elements, users of virtual networks can program the network elements on any layers from physical layer to application layer. They can even define new layering architecture without interfering the operation of other virtual networks. In other words, each virtual network can provide the corresponding user group with full network services similar to those provided by a traditional non-virtualized network. The users of virtual networks may not be limited to the users of services or applications, but may include service providers. For example, a service provider can lease a virtual network and can provide emerging services or technologies such as cloud computing service, and so on. The service providers can realize the emerging services as if they own dedicated physical network infrastructures. In order to facilitate the deployment of network virtualization, it may be necessary to provide control procedures such as creating virtual networks, monitoring the status of virtual network, measuring the performance, and so on.

Also, network virtualization can reduce the total cost by sharing network resources. One of motivation of network virtualization is to achieve better utilization of infrastructures in terms of reusing a single physical or logical resource for multiple other network instances, or to aggregate multiples of these resources to obtain more functionality. These resources can be not only network components, such as routers, switches, hosts, virtual machines, but also service elements, such as, operation and measurement services, instrumental services, and so on.

Network virtualization on its own is quite a useful technology, but to gain the greatest benefit it will be better to have a managed virtualization environment. Such management allows better control, monitoring, and analysis of the virtualized environments.

# 6.1 LINP architecture in network virtualization framework

Network virtualization is required to be capable of providing multiple LINP (Logically Isolated Network Partition) those are isolated each other. LINP may be created over the single physical infrastructure.

Figure 6-1 represents the architecture of LINP which creates multiple virtualized infrastructures in network virtualization framework. Each LINP is isolated each other and is programmable to satisfy the user’s demand on the functionality and amount. User’s demand is conveyed to slicing manager which is required to coordinate infrastructures so that appropriate LINP is provided to the user. LINP manager is recommended to handle user’s demand with real-time or scheduled.

Figure 6-1: LINP architecture in network virtualization framework

7. Problem spaces

This section describes the problem spaces of network virtualization.

7.1 Isolation

Legacy networks enable users to partially realize multiple network services over shared physical infrastructure in a way which allows services to be affected by other coexisting network services. How to provide secure isolation among the network services is an important issue. The isolation has various aspects including security isolation, performance isolation, management isolation, and so on. For example, since multiple network services coexist over shared physical infrastructures, security problems in a service may be spread out over the whole networks and may cause performance degradation of other services. Network virtualization can provide complete isolation of any LINP from all others, minimize the impact of behavior of LINPs to other networks, and support diversity of application, service and architectures.

7.2 Performance

In legacy networks, the network service providers hardly offer resources encompassing the physical capability of the resources. However, by leveraging network virtualization, it is possible to provide high performance resources for users by logically aggregating multiple resources into single resource. Therefore, a logical network consisting of requested resources can guarantee users’ performance requirements.

7.3 Scalability

It is actually restricted to scale physical resources according to users’ demands in legacy networks. Users of network virtualization, however, can add as many virtual resources to their LINPs as they need. That is, requested size of a LINP can be provided. In addition to the size, coexistence of multiple LINPs is one of the fundamental motivations behind network virtualization. Legacy networks hardly provide multiple networks, but multiple LINPs can be created in a shared physical network with the same resources.

[EdNote 8] the capability of accepting different network architecture depends on the architecture, implementation, etc. of network virtualization technologies, and each technology has its pros and cons (e.g., technology with specialized, limited capabilities usually have better performance). Investigation on such pros and cons should be discussed in this document, not necessary in this section. Further contributions are invited.

[EdNote 9] The investigation on pros and cons of network virtualization will be added in Applicability section based on further contributions.

7.4 Flexibility

In legacy networks, network providers have only to offer limited physical resources, and users only exploit restricted resources, e.g., nodes connected by physical links in a local area. However, in network virtualization, not only local resources but also remote resources can be provided and belong to a virtual network. Then users can use arbitrary network topology, forwarding or routing functions, and customized control protocols.

7.5 Evolvability

If network providers want to deploy a new network technology, such as new network layer protocols in the current network architecture, they need to construct a separate testbed so that the behavior of the new protocols does not affect the current services. After evaluating the new technology, the network provider deploys the technology to their network. However, this approach may lose legacy support or backward compatibility and users may be reluctant to adapt the new feature because the new technology may not support their existing services. By utilizing network virtualization, the network providers can integrate legacy support by allocating the existing networks to a logically isolated network partition. This partition will guarantee that the existing services and technologies may not be changed [9].

7.6 Management

Since each virtual network is independent from other virtual networks, it has to be managed independently from other virtual networks. At the same time, the management system for the virtual network has to collaborate with the management system of physical infrastructure. It is therefore necessary to carefully define which part of management can be done by the management system of the virtual network, and how to align it with that of physical infrastructure. Moreover, if the isolation is not perfect, alignment with the management systems of other virtual networks also becomes necessary.

[Ed Note 10]: More refined text will be provided by Takashi Egawa

[Ed Note 31]: Revise the text according to describe challenging tasks such as hierarchical management, isolation (or independent) management systems

7.7 Security

In the legacy network, failure or malfunction in one service or network can be spread over the whole network. Also, if a network element is compromised by a malicious user, the whole traffic traversing the element may be a target for attack. Since network virtualization ensures complete isolation among logical network partitions, the security problems do not be spread over the whole network partitions [9].

7.8 Benefits and overheads of network virtualization

Network virtualization may increase availability via maximizing utility of physical network infrastructures. However, availability becomes an issue because if one physical network element, which can be involved to several LINPs, were to go down, these LINPs will be also influenced by the failed physical network element. In other case, if a LINP management system were failed or to perform miss operation, all of relevant LINPs will also fail. Therefore most service provider for LINP has to carefully implement some failover technique, such as failure-resistant configuration using cluster physical network elements, to prevent such outage, even though each LINP is guaranteed to isolate their network services and has management systems to handle physical network infrastructure.

Scalability is difficult to deal with under fully utilization for capable physical network resources. LINP provides a way for corresponding user group to support full network services, so that a service provider can lease a LINP to provide emerging service according to fit user demand using restricted physical resources. The problem is that while service provider may implement a small scale of LINP, it could be requested to more size cause to quick grow of network services. Users or service provider of the LINP want to add as many physical resources as they need, but physical network elements, such as router, switch, and etc, cannot afford to be leased to add these resources any more in case of fully utilizations for capable physical network resources. So scalability and utilization are intertwined.

Additionally, the LINPs impose performance falling off from running an additional layer composed by programmable network elements above physical infrastructure. The performance degradation depends on virtualization techniques for isolation and management, because many software technologies are involved to achieve better isolation and management system. In other words, some traffic is handled by software as well as hardware forwarding.

8. Attributes of network virtualization

This section describes the attributes of network virtualization.

8.1. Programmability

Network virtualization allows the creation of logical network partitions by using not only local resources but also remote resources. The logical network partitions may be equipped with programmable control plane so that users can use arbitrary network topology, forwarding or routing functions, and customized protocols. In order to provide flexibility to the logical network partitions, they can be controlled with new control schemes on each logical network partitions. Programmability can support flexibility of control plane and make it possible to use different control schemes on logical network partitions.

8.2. Topology awareness

Since network virtualization allows aggregation of network elements distributed in the network infrastructures, it is necessary to discover those and to connect the network elements. Further, when constructing or re-constructing a logical network partition, good optimization may be required for effective use of infrastructure. For example, for the users who want low end-to-end delay, topology awareness may help to provide low delay by using the shortest route, whereas providing un-optimized but high bandwidth route for users who want high bandwidth only. Therefore, topology awareness is necessary so that network elements can seamlessly interact with each other during the construction of logical network partitions.

8.3. Quick reconfigurability

After creating LINPs according to users’ requirements, the capabilities of LINPs may need to be modified due to various reasons, for example the changes of users’ requirements, the status of networks, policies of resource owners and so on. Hence, each LINP should adjust its capability according to changes of requirements and the reconfiguring should be quickly done in order to minimize service disruption. Thus, the network virtualization should offer a method that the LINPs are capable of easily and rapidly creating their own network topologies and dynamically reconfiguring them.

8.4. Isolation

LINPs can be multiplexed over shared network infrastructures. However, this can be liable to restrict network performance and cause instability due to interference by other LINPs. The network virtualization should be capable of providing the complete performance isolation among LINPs.

8.5. Network abstraction

A logical network partition may accommodate a new architecture different from the current infrastructure architecture, so network elements should be controlled as abstracted resources. Network abstraction allows hiding the underlying characteristics of network elements from the way in which other network elements, applications, or users interact with those network elements and separates infrastructure instances and control frameworks of network virtualization. Network abstraction also allows the controlled opening of networks functions. In other words, it allows selective exposure of key network functionalities in network infrastructures. And Network abstraction will make possibility of infrastructure technology replacement with less impact to the framework of network virtualization. Therefore, it is necessary to support network abstraction methods to define levels of exposure and interfaces to other network elements.

8.6. Performance

It is known that some of emerging network services will require extremely high bandwidth of more than 10 Gbps with low latency and jitter. However, current packet-based technologies cannot satisfy such requirements. Each LINP should be capable of directly utilizing lower-layer transmission technologies such as wavelength and TDM (time-division multiplexing) provided by a transport network. Also, network virtualization is implemented by introducing virtualization layer or adaptation layer and the virtualization layer creates and manages the logical network partitions. Thus, the performance of the LINPs may not be as good as the non-virtualized network. Therefore, network virtualization should reduce the performance degradation due to the virtualization layer.

8.7. Security

To support diversified network services, the LINPs should retain the capability of customizing network control and operations independent from those in the physical network or other LINPs. At the same time, the LINP wishes to avoid complex physical network operations that are fully dependent on the types of network layers and equipment vendors. To disengage the LINP from the complexity of the physical network, the network virtualization should conceal a part of the physical network information and provides the simple interface for resource control to the LINPs.

8.8. Management

Considering the utility of customers, each LINP should be capable of using physical network resources and constructs a network topology. However, one possible problem is that some malicious LINP may occupy most of the resources, which deteriorates other LINPs’ performance due to network resource exhaustions. So, the network provider should have the capability of regulating the upper limit of bandwidth usage by each LINP to maintain the overall utility and performance.

[Ed Note 11] this section should be divided into two sections. One is Manageability that describes network operator’s viewpoint, and Programmability that provides user’s point of view.

[Ed Note 12] Further contributions regarding to Programmability is invited.

[Ed Note 13] In-system management approach is required in management section.

8.9 Mobility

When the user's demands is changed or network performance falls below Quality of Service thresholds, the network elements (e.g., resources (storages, computing powers, network interfaces, etc.) , system images, applications) can be moved to satisfy the user's demands and provide stable network performance. Therefore, network virtualization  should support not only mobility of one physical network to another but also mobility of the network elements including users among logical network partitions. The network elements are composed of resources, system images and applications, which are added for the improvement of network performance or removed for load balancing or energy saving purpose. Users can be dynamically attached or reattached to one of the virtualized networks depending on the desired application without any interruption of communication.

Note: this kind of mobility is sometimes called migration.

9. Applicability

This section describes the applicability of network virtualization.

The applicability of network virtualization may be classified into two broad categories according to the characteristics of virtualization; namely isolation and aggregation.

From the view point of isolation, network virtualization enables the complete isolation between each logical network partition. Typical use case of this category is that it is possible to create new business model by separating the conventional Internet service provider’s role into network provider and service provider. The network provider creates customized logical network partitions according to the service providers’ requirements, such as network bandwidth, the number and functional capability of network elements in the logical network partitions, total cost, and so on. The service provider can provide various services and applications for users without the burden of building its own network infrastructure. Moreover, the service provider can safely test an innovative pilot service or application that requires special features of network without affecting other existing services. From the perspective of network providers, they can increase the hardware utilization ratio and can reduce the operational cost of network infrastructure. From the viewpoint of service providers, it is possible to reduce the cost for building network infrastructure. Also, they can utilize the flexibility for the creation of network and application services.

The second applicability is aggregation. In the computing field, it is common to logically aggregate multiple computing resources into a single resource in order to support applications or services that require very high performance exceeding single resource’s capability. Building a high performance computing node by clustering many smaller nodes would be the typical use case of resource aggregation. Similar to this use case, network virtualization can allow building a logical network element whose capability is hard to be supported by a single one. The logical network element can support various functions and can easily expand its capability by aggregating multiple network elements.

[Ed Note 15] How to orchestrate different capability or functions of network virtualization should be investigated in applicability section.

[Ed Note 32] Current applicability seems to mandate specific service models. It is noted that the applicability may need to be more generally described.

10. Implementation examples

This section describes the implementation examples of network virtualization.

10.1. Router aspects

10.1.1. Functional architecture of virtualization support router

In general, the router architecture to support virtualization capability requires three layers, router hardware, router software, and control framework, respectively. Figure 10-1 shows the functional architecture of virtualization support router.

The router hardware typically consists of switching component for packet forwarding and flow table. The router software performs router’s main operations such as running routing protocols and building routing table, and so on. The router software may include virtualization layer in order to support virtualization in router. The virtualization layer typically creates and manages logically isolated virtual systems, which can run various components on the native hardware. Thus, the virtualization layer can support virtualization of router hardware resources by creating the isolated virtual systems, i.e. virtual router. The virtual router is a software implementation of a router that executes the same operations as a physical router. It is an isolated partition of a real router. Multiple virtual routers can coexist over the virtualization layer and each virtual routers are completely isolated each other so that one virtual router does not affect others. In order to provide the management of virtual routers, the virtualization layer can include the virtual machine monitor or hypervisor function. The control framework performs the interaction between the virtual or physical router and other network entities. Control framework defines interfaces, message types including basic protocols and required functions, message flows between router and network entities. The network entities may include networking elements such as routers, switches, and so on. It also may include logical entities such as registry for managing of LINPs, network resources, and user information. Each LINP and virtual routers are managed by resource manager that are implemented in the control framework of the physical router. The resource manager is in charge of creating and managing LINPs and virtual routers in the physical router.

Figure 10-1: Functional architecture of virtualization support router

[Ed Note 16] Mapping between LINP and virtual router should be provided. Further contribution necessary.

10.1.2. Control framework architecture

This section describes the architecture of control framework including the design of functional entities and interfaces between the entities.

[TBD]

10.1.3. Router software architecture

This section presents references or software implementation profiles in order to support virtualization in the router. It also defines interfaces between router software and control framework.

[TBD]

10.1.4. Router hardware architecture

This section provides considerations of hardware architecture in order to support virtualization in the router.

[TBD]

10.2. Application aspects

Figure 10-2 shows an example of relation between applications and LINPs. Each application accesses the LINP to control the functionality of the LINP, such as routing. Multiple applications may access the same LINP.

[Ed. Note 17] Terms and definitions of “physical network infrastructure”, “LINP”, and “virtualized network management” should be aligned with the previous sections.

Figure 10-2: Relationship between application and LINP

Figure 10-3 shows another example of relation between applications and LINPs. Here, application platform is introduced. The task of Application platform is to receive requests from the application and (1) access LINP to control the functionality of the LINP, (2) access virtualized network management to reconfigure LINP.

[Ed. Note 18] An explanation of large and small arrows in the figure should be added.

[Ed. Note 19] An explanation of why the application platform needs to reconfigure LINP (ex. to optimize resource usage) should be added.

Figure 10-3: Relationship between application and LINP

10.3 Mobility aspects

Figure 10-4 illustrates the relationship between LINPs and network elements. Each application can be moved from one LINP to another. In addition, a number of resources and system images can be added and removed according to the user's demand or requirement of the network performance. Therefore, the manager of LINPs should handle their mobility in real-time or in the scheduled time.

Figure 10-4: Relationship between LINPs and network elements

10.4. Other aspects

11. Security considerations

[TBD]

Annex A : Use cases

[Ed Note 20] use-cases should be described carefully; only indispensable use-cases should be written in the main body, and it should be short not to harm readability of normative part. With that, editors will massage the text here to fit into applicability section.

[Ed Note 21] in general use cases here are very specific. Generalizing it could be a good exercise to think applicability of these use cases, and the applicability of network virtualization technology.

[Ed Note 22] Further contributions about generalized use cases are invited.

[Ed Note 23] many people believe virtualized network is a part of the cloud computing, or computing technologies. We must be very careful of this perception, and a use case that clears this point is highly necessary.

[Ed Note 24] Further contribution investigating the use cases relationship between network virtualization and cloud computing is invited.

In this annex, the systematic knowledge of the use cases of network virtualization is described in order to clarify the purpose of network virtualization technology. The systematic description can be used as a guideline for describing the use cases, so further contributions regarding use cases should follow this guideline.

[Ed Note 25] the floor agreed that this is a good guideline to submit use cases. In the use-case appendices it should be mentioned that future contribution of use cases should follow this guideline.

In A.1, the list of use cases of network virtualization and the current status of network virtualization technology for each use case, and the categorization of use cases in A.2 are described.

A.1. List of use cases of network virtualization and current degree of maturation

A.1.1. Case1: the use case for feasibility experimentation of the new network architecture [1]

A.1.1.1 Use case

Who

- National research institutes

To whom

- Researchers interested in the new network architecture

Why

- The feasibility experimentation over at-scale experimental network is required to globally develop innovative new network architecture. Multiple experimentations are explored in one experimental network at the same time.

What

- Researcher wants the virtual network to consist of a virtual node and virtual links. The virtual node is required to work with the customized routing protocols.

When

- This service is needed now.

A.1.1.2 Current status of technology

There are on-going project developing open standard such as OpenFlow [x] that allows running of experimental protocols in programmable flow switches.

[Ed Note 26] Whether to include particular project name in FG-FN document or not is to be discussed FG-FN management level

Control frameworks for resource contention management are under development.

A.1.2 Case 2: network virtualization for application service provider

A.1.2.1 Use case

Who

- Application service provider who owns the computer systems and local area network infrastructures. The application service provider enables their customers to use the computer systems.

[Ed Note 27] the terms and concepts used in these use cases, e.g., application service provider, needs to be clarified.

[Ed Note 28] Further contribution is invited in order to clarify the meaning of application service provider.

To whom

- Application service providers

Why

- Recently, application service providers deploy computer systems in multiple locations to improve their service availability. When the systems in location (A) are out of order, all applications running on these systems are run on the systems in another site (B) instead. In this case, it is desirable that the application-user is not aware of this change. Thus, the systems deployed in different locations should be handled in the same manner as those located in the same location.

What

- One virtual computer system consists of multiple computer systems deployed in different locations, and one virtual network output link consists of multiple network output links deployed in different locations. The virtual computer system and virtual network output links should be synchronized.

When

- Application service provider requires these services now.

A.1.2.2 Current status of technology

A.1.3 Case 3: network virtualization for the network service provider who owns the network infrastructures

A.1.3.1 Use case

Today there are network service providers who own several different networks and provide several different services on each network. Networks should be operated using its management policy according to each service quality. For example, PSTN, dedicated line service, internet service, and mobile network service.

Several service networks must be composed from common physical network infrastructure.

Ex. multilayer network architecture [3]

A.1.3.2 Current status of technology

Logical router technology is implemented in commercial products. With this technology, multiple virtual routers can be created on one router, and IP routing protocol can be performed on virtual routers independently.

With programmable flow switching technology, the forwarding table of switch nodes can be controlled by the external servers.

The management framework for virtual networks, which consist of multiple-layer resources (ex. WDM-circuit and packet-switching networks) are under development. The virtual networks must be completely isolated from each other and the physical resource be optimally used.

A.1.4.  Use cases of mobility in virtualized network

A.1.4.1 User/Application Mobility

Figure A-1 shows use cases of user and application mobility in virtualized network.

Figure A-1. Uses cases of user and application mobility

Each virtualized network may provide different network resources. In this example, LINP 1 is created based on small network resources such as low power CPU, limited Memory, and low bandwidth. On the contrary, LINP 2 is created based on extra large network resources in order to process large amount of data, and support applications which require high computing power. In this case, User 1 is currently connected with LINP 1 to use APP1. When User 1 needs to use APP 2 which requires high computing power, the LINP manager discovers LINP 2 for User 1. In addition, if User 1 still needs to use APP1, APP1 can be accessed from LINP 2 or APP1 can be moved from LINP 1 to LINP 2 for User 1. Therefore, the LINP manager will handle not only the user mobility but also the mobility of application which is used by the user in the previous LINP.

A.1.4.2 Resource/System Image Mobility

Figure A-2 shows use cases of system image and resource mobility.

Figure A-2. Uses cases of system image and resource mobility

In this example, the system images can be moved from LINP 1 to LINP 2 according to user's demand, application type, and other reasons without any interruption. Furthermore, both system images and resources can be added to LINP 1, and removed from LINP 2 in order to achieve high network performance and energy saving. Therefore, the LINP manager should handle the mobility of system images and resources between LINPs.

A.1.5. Case 5: other possible cases

There are other possible use cases, and they will be added to this document.

A.2 Use case categories and their applicability

T.B.D.

____________