| INTERNATIONAL TELECOMMUNICATION UNION |  | Focus Group On Future Networks |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | FG-FN OD-57 |
|  |  | Original: English |
|  |  | 6th FG-FN meeting: Geneva, Switzerland, 6-9 September 2010 |
| OUTPUT DOCUMENT 57 |  |  |
| Source: | Editors |  |
| Title: | Draft Deliverable on “Overview of Energy Saving of Networks” |  |

This is a revised text of Draft Deliverable on “Overview of Energy Saving of Networks” based on contributions and discussions of 6th FG-FN meeting in Geneva, Switzerland, 6-9 September, 2010.

[C90] The proposed text was modified and added to “5. Introduction to Energy Saving of Networks”.

[C91] The proposed text was added to “7. Review of Energy Saving technologies”.

[C89] The proposed text was modified and added to “7. Review of Energy Saving technologies”.

[C98] The proposed text was added to a new “Living list” section.

[C88] The proposed text was added to new “Appendix II. Examples of energy saving technology”.

Overview of Energy Saving of Networks

[Editor's Note: should be decided if "Future" is to be included in the title later.]

Summary

This document describes the overview of energy saving of Future Networks.

Table of Contents

1. Scope

2. References

3. Definitions

3.1. Terms defined elsewhere

3.2. Terms defined in this document

4. Abbreviations and acronyms

5. Introduction to Energy Saving of Networks

6. Motivation and High Level Requirements

7.  Review of Energy Saving technologies

7.1. List of technologies and their levels

7.2. Device level technologies

7.3. System level technologies

7.4. Network level technologies

8. Capabilities

8.1. Considerations for energy saving

8.2. Classification of technologies from capabilities

8.3. Minimize device usage for traffic dynamics

8.4. Forward the traffic with less power

9. Impact analysis of Energy Saving

9.1 The effect of energy saving to be required or to be realized

9.2 Influences to other network performance

10. Overview of candidate functions

10.1. Management functions

10.2. Network devices

Appendix I. Examples of technology consideration

I.1. Environmental Token Consideration in Future Network Routing Capability

Appendix II. Examples of energy saving technology

II.1. ADSL2/ADSL2plus Low-Power Mode Guidelines

Living List

L.1. Candidate functions

Overview of Energy Saving of Networks

- 1	Scope

Energy saving of networks themselves is an important capability in future networks. In this document we call a network capability that use less energy as “energy saving of networks”, and

-  review energy saving technologies,

-  describe capabilities

-  describe impact analysis of energy saving

-  describe overview of candidate functions

-  describe high-level requirements

of energy saving of networks.

- 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this document. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this document are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is published regularly. The reference to a document within this document does not give it, as a stand-alone document, the status of a Recommendation.

[1] International Telecommunication Union, “ICT and Climate Change,” April 2008

[2] ISO 14067-1, “Carbon Footprints of Products – Part 1: Quantification”, under development

[3] ISO 14067-2, “Carbon Footprints of Products – Part 2: Communication”, under development

[4] Cisco, “Cisco Visual Networking Index: Forecast and Methodology, 2009-2014”, June 2010

[5] Green IT Promotion Council, 2008 Research Report, June 2009 (in Japanese)

[6] Global e-Sustainability Initiative (GeSI), Smart 2020 report, 2008

[7] “Annual Energy Outlook 2009 with Projections to 2030”, World Energy Outlook, DOE/EIA-0383, Mar. 2009.

[8] “ICT Energy Consumption – Trends and Challenges”, Gerhard Fettweis, Vodafone Chair Mobile Communication Systems.

[9] “http://www.greentouch.org/index.php?page=challenges-opportunities”

[10] “Green IT-A New Industry Shock Wave”, Gartner 2007.

[11] “Compact base stations: a new step in the evolution of base station design”, Senza Fili Consulting, 2010.

[12] “http://www.octasic.com/en/applications/wireless/small_cell.php”

[13] “http://www.instat.com/newmk.asp?ID=2810&SourceID=00000652000000000000”

[14] “BLI Research on Small Cells”, Davide Cherubini, Alcatel-Lucent.

[15] David Neilson, GreenTouch: Inventing Sustainable, Ultra Energy-Efficient ICT Networks,  GreenTouch presentation metrial, 2010.

[16] Hai Huang, Padmanabhan Pillai, Kang G. Shin, "Design and Implementation of Power-Aware Virtual Memory", usenix03, 2003

[17] ENERGY STAR computer specification Draft Version 5.0, 2008

[18] Broadband forum, TR 202, ADSL2/ADSL2plus Low-Power Mode Guidelines, issue 1, 2010

[19] NICC ND1424, Guidelines on DSL power saving models and non-stationary noise in metallic access networks, 2008

[20] ENERGY STAR Small Network Equipment Draft Specification Framework Document, October 2009

[21] IEEE P802.3az Energy Efficient Ethernet Task Force, http://www.ieee802.org/3/az/index.html

[22] IETF CAPWAP Working Group, http://www.ietf.org/html.charters/capwap-charter.html

[23] RFC 5412, “Lightweight Access Point Protocol”, Feb. 2010

[24] ETSI TR 102 530 V1.1.1, Environmental Engineering; The reduction of energy consumption in telecommunications equipment and related infrastructure, 2008.

[25] Broadband forum, TR 202, ADSL2/ADSL2plus Low-Power Mode Guidelines, issue 1, 2010

[26] ITU-T Rec. G.992.3 ADSL2, 2009

[27] ITU-T Rec. G.992.5 ADSL2plus 2009.

[28] NICC ND1424, Guidelines on DSL power saving models and non-stationary noise in metallic access networks, 2008

[29] ITU-T Rec. G.997.1 Physical Layer Management for DSL Transceivers, 2009

- 3	Definitions

[Editor's Note: This section will describe the definitions of terms to be concerned with energy saving of networks, devided in sub-sections 3.1 and 3.2]

- 3.1	Terms defined elsewhere

TBD

[Editor's Note: This sub-section will describe the definitions of terms to be used generally.]

- 3.2	Terms defined in this document

TBD

[Editor's Note: This sub-section will describe the definitions of terms to be used specifically in this document.]

- 4	Abbreviations and acronyms

TBD

- [Editor's Note: This section will list the abbreviations and acronyms to be concerned with energy saving of networks.]

- 5	Introduction to Energy Saving of Networks

After the monumental 4th IPCC report in 2007 that concluded the man-made climate change is ‘very likely’, various scientific evidence that show the seriousness and urgency of climate change has been added. To this end, it has become apparent that one of the key objectives of future networks is its contribution to combat climate change.

Contribution of ICT to combat climate change is often categorized into ‘by ICT’ and ‘of ICT’. ‘Of ICT’ is the challenge to reduce environmental load of ICT system itself such as PCs, servers, routers, and so on. On the other hand, ‘by ICT’ is the challenge to reduce environmental load by using ICT system; for example, telework and supply chain management (SCM). We can therefore categorize future networks’ contribution to climate change into two, ‘by future networks’ and ‘of future networks’.

- by future networks

Future networks must become a useful tool for other sectors to mitigate or to adapt climate change. Smart grid friendly networks architecture, QoS and cost suitable for ubiquitous sensor networks that monitor earth environment are a few example contributions by future networks.

- of future networks

Future networks must minimize its carbon footprint, the overall ecological impact from its production to use to disposal. For example, most of the ecological impact of mobile terminal arises from its production, so reducing the impact in production phase is a valuable contribution. Routers, switches and servers consume much electricity in its use phase, which makes energy saving a valuable contribution on these facilities.

Since energy consumption correlates with GHG emissions, both “by” and “of” Future Networks’ contributions to climate change are effective when Future Networks can save energy consumption. Energy consumption in ICT networks is rapidly increasing in part due to exponential network growth, especially with the explosion of wired data traffic as well as wireless data traffic [9]. The World Energy Outlook (WEO) 2009 estimated that the world’s energy consumption will be doubled in 2030 driven by the energy consumption of the networks [7][8]. Thus, the impact of world’s energy consumption caused by the networks will be more significant than now. Thus, reducing the energy consumption of networks becomes effective contribution to combat climate change due to the correlation of GHG emissions.   Therefore, energy saving is a critically important concept of Future Networks. In this document, Energy saving of networks means that total power consumption of network equipments such as routers and switches are systematically reduced.

- 6	Motivation and High Level Requirements

The mitigation of GHG emissions is a key requirement for Future Networks. Carbon footprint quantifies GHG emissions associated with goods and services. In general, the following stages are considered in carbon footprint:

- Production, manufacture, and transportation

- In-use (include service provisioning)

- Disposal/recycling

Networks are composed by equipment (products). For the network construction, products are transported from a manufacturer’s site to a construction place. After the end of life of equipment, the equipment should be disposed and/or recycled. Carbon footprint considers those activities and GHG emissions associated with these stages. In terms of networks, the stage of “in-use” is primarily considered, because GHG emissions associated with “in-use” activity can be controlled by network capabilities and architecture. For this stage, three categories of technologies are focused, i.e. device, system and network levels.

- Device level (e.g. LSI microfabrication, optical node, clock gating)

- System level (e.g. transmission, multiplexing, equipment configuration, system-level software)

- Network level (e.g. access, core, wireless, wireline, architecture-level software)

Technology associated with each level can be evolved. Future Networks should stimulate and flexibly adopt technological developments and evolutions. When network control parameters and mechanisms at each level of the category are standardized and supported by those categorized technologies, technological changes can be allowed and adopted by Future Networks. Therefore, high level requirements for energy saving aspect of networks will be management functions, which control parameters and mechanisms at each technological category. Management functions will include status monitoring and control of device, system, and network level of technologies.

- 7	Review of Energy Saving technologies

7.1. List of technologies and their levels

[Editor's Note: The granularity of following technologies is diverse. So, we need to fix it.]

[Editor's Note: Regarding technologies proposed by C-89, the level of description do not match with the existing text. For example, CAPWAP protocol and Ethernet issues can be an example of lightweight protocol. And ‘energy aware design’ is too generic. Developing appropriate text is for further study.]

Based on three levels described in section 6 (Device, System, and Network level), following technologies are listed.

- Device level Technology

LSI microfabrication, Optical node, Multi-core CPU, Clock gating, Power-aware virtual memory

- System(Equipment) level Technology

Sleep control, ALR/DVS, Circuit switch, Thermal design, Cache server, Filtering, Sorry server, Shaping, Compact base transceiver stations (BTSs)

- Network Technology

Energy-based routing/traffic engineering, Lightweight protocol, Transmission scheduling, CDN, Job scheduling, Small-cell design, Energy aware design

In sub-sections 7.2 - 7.4, the outline of each technology is described.

7.2. Device level technologies

7.2.1. LSI microfabrication

It is a device technology of LSI microfabrication and reduction of driving voltage. The more it advances, the less power is needed. Up to now, the ratio of improvement of it has progressed by 30% per year, but it seems slow down recently because of the increase of leak current.

7.2.2. Optical node

It is an implementation of optical transmission technology to a network node. Optical network has very large capacity compared with electrical network, which speed reaches Tbps or more. It can improve energy efficiency drastically that is defined by W/bps (necessary energy to get unit data rate) [15]. Because of its very high-capacity, it requires large amount of traffic aggregation. Otherwise, sufficient energy efficiency cannot be achieved. Moreover, optical packet switching is still difficult to realize.

7.2.3. Multi-core CPU

It is a technology to control power supply and clock rate to multi-core CPUs, so that necessary number of CPUs can operate at necessary clock rate. If CPUs are controlled adequately according to the load, necessary energy can be saved with it.

7.2.4. Clock gating

It is a technology to control clock supply to CPU(s) when necessary task does not exists. The longer the duration without clock is, the more energy can be saved. Nevertheless, if the transition between ON and OFF state of clock occurs frequently, the less energy saving effect is achieved.

7.2.5. Power-aware virtual memory

It is a technology to control the power state transitions of memory under software control not using hardware control [16]. Form the energy saving viewpoint, this technology brings the high energy-efficient power management of memories (such as buffer memory, cache memory) in a network node.

7.3. System level technologies

7.3.1. Sleep control

It is a technology to put equipments and functions to "sleep" when they are not used, and save the energy consumption. At system level such as routers and switches, sleep control is typically implemented to equipment itself and network interfaces. The effect of energy saving is depends on the traffic dynamics. The more the difference between maximum and minimum traffic, the more the energy saving effect can be achieved. The difficulty of this technology is that current routers must treat small control traffics such as routing information even if data traffic is not delivered, so it is a problem how to treat these control traffic. To solve this problem, “proxy” can be a candidate maintains continuous network presence during sleep mode operation for any network node, and responds to routine network traffic instead of network node. The details of this technology are described in the ENERGY STAR computer specification Version 5.0 [17]. To realize sleep control, the network node supports sleep control described in this section and network supports energy efficient Ethernet protocol developed by IEEE P802.3az Energy Efficient Ethernet Task Force [21]. Moreover, we can achieve energy savings of up to 2.9GWh/year per million lines by L2 power saving mode (e.g, low power mode) combined with existing ADSL2/ADSL2plus technology. One example of this technology is described in appendix II.1.

7.3.2. ALR/DVS

It is a sort of energy saving operation mode of network equipments. ALR implies Adaptive Link Rate, which controls link speed according to the amount of the traffic to be processed. DVS implies Dynamic Voltage Scaling, which controls driving voltage of CPU, hard disc, NIC, and so on, according to the amount of the traffic to be processed. The more scaling is accomplished, the more the energy saving can be achieved. The problem of this technology is how to treat burst traffic, and so on.

7.3.3. Circuit switch

Circuit switch consumes less energy than packet switch, because it is simple and does not need extra energy-consuming devices such as SRAM and CAM. Nevertheless, circuit switching cannot realize statistical multiplexing, so it may degrade network performance. It is a problem of circuit switch from the viewpoint of energy saving.

7.3.4. Thermal design

In a network node, cooling system consumes considerable amount of energy. Energy saving can be achieved by improvement of thermal design of a network node.

7.3.5. Cache server

This technology is the caching of web contents to reduce bandwidth usage, so it can achieve energy saving corresponding to the reduction. If the copy of the contents exists in this server at high probability, it is effective for energy saving. But, if this probability is low, the server needs to access to the original server frequently. It may costs more energy than the case without cache server.

7.3.6. Filtering

This technology blocks unnecessary data to be transmitted, so it can achieve energy saving corresponding to it. Filtering technology is not originally intended for energy saving, but using it in good manner can save energy.

7.3.7. Sorry server

It is a server which returns the alternative response to inform that it cannot provide services by some reason such as traffic congestion. Consequently, it can peak-shift the traffic on the time axis and ease the necessary traffic demand. So it can save energy.

7.3.8. Shaping

It is a technology that controls the output rate of packets according to the link speed. Because it controls the maximum data rate, it can save the energy of other nodes which is defined by the maximum data rate. Nevertheless, it may increase the delay.

7.3.9. Compact base transceiver stations (BTSs)

It is the latest base station system to be introduced for saving power consumption and cost reduction.  Unlike the conventional ground-based BTSs and distributed BTSs, compact BTSs do not require ground shelters and cooling equipment, so it consumes lower energy, less expensive equipment, and lower installation costs. Nevertheless, they are able to support high performance features such as multiple antennas per sector with multiple input, multiple output (MIMO), and beamforming [11].

7.4. Network level technologies

7.4.1. Energy-based routing/traffic engineering

It is a routing/traffic engineering technology to minimize network-wide energy consumption. It usually premises that sleep or ALR/DVS functions described in sub-section 6.3 can be utilized in network nodes. When sleep function can be utilized, this technology aggregates the traffic routes, and makes sleep unused nodes or links so that unnecessary energy can be saved. When ALR/DVS function can be utilized, this technology distributes the traffic routes so that each node treats minimum traffic, and makes link rate or voltage at the adequate level so that unnecessary energy can be saved. The problem of this technology is how to treat dynamics of the traffic. One example of this technology consideration is described in appendix I.1.

7.4.2. Lightweight protocol

This technology simplifies the network protocol and saves energy corresponding to it. Typically, there are two approaches to accomplish energy saving by it. One is to transfer the data traffic in the lower layer than IP, and the other is to make simple the transmission layer protocol. The former includes label switching such as MPLS, and the latter includes modified TCP that improves retransmission algorithm and CAPWAP protocol that reduces the signalling overhead for WLAN developed by IETF CAPWAP WG [22]. This technology can make the necessary functions of network nodes small, so it can save energy according to it.

7.4.3. Transmission scheduling

This technology premises the nodes with smaller buffer than usual. It controls the amount and the timing of packet transmission so that it can avoid output waits at each node. So it can save energy. The problem of this technology is the network performance. If transmission scheduling is controlled ineffectively, it degrades the network performance. Consequently, it may require extra network resources.

7.4.4. CDN (Contents Delivery Network)

It is an optimized network for delivery of web contents via the Internet. Optimized CDN can save energy because it can access the nearer server than the original one, so it can save energy corresponding to it [15]. The problem of this technology is as same as that of cache server. That is, if hit rate is small, this technology is not so effective for energy saving.

7.4.5. Job scheduling

This technology peak-shifts the traffic on the time axis as much as possible, and reduces the number of network equipments to be needed, in order to save energy. One method is to distribute broadcast content in advance.

7.4.6 Small-cell design

It is designed for the small-cell area (micro-cell, pico-cell, and femto-cell), where mobile demand is high such as city downtowns, shopping malls, airports, campuses, and large offices, to off-load macro-cell traffic. Typically, small-cell design can be implemented in the compact BTSs described in sub-section 6.3.9 with marcro BTSs, establishing the overlay cellular networks [12][13]. It needs less power to transmit the data traffic compared with that of macro-cell because its cell size is far small, so it can save energy. In addition, energy consumption for small-cell can be further reduced by Energy-efficient algorithms such as coverage-aware switch-offs, which detect existing spatial coverage and switch-off base stations, and traffic-aware sleep modes which detect User Equipment (UE) activity via carrier sniffing and temporarily switch-off idle small cells [14].

[Editor's Note: The above text was incorporated as is, but proof by peer-reviewed papers is necessary if smaller cell really reduce power consumption.]

7.4.7. Energy-aware network design

It is a part of energy saving technology in wired and wireless network. To reduce energy consumption, it considers the energy consumption at network. Until now, the designer is usually concerned about the performance and reliability and can neglect to improve energy efficiency which is just as important to reduce environmental impact. However, network design to real needs is important for optimization. Important is the installation data power consumption report, power utility mains outage reports (number of main losses, duration of outage, etc.), optimization should start with the highest power consumer at a site [15][24].

- 8	Capabilities

This section describes capabilities for energy saving of networks.

8.1 Considerations for energy saving

From the technologies listed in previous section, some considerations can be made regarding the direction for energy saving.

Target area

 ‘of future networks’ or ‘by future networks’

In order to reduce GHG emissions, both issues are important. But we focus on ‘of future networks’ in this document, as described in section 5.

 Stage in a lifecycle

We focus on the stages of “service provision” and “in-use”[4]. The former treats the static problem how smaller amount of network resources should be prepared for the given traffic demand, on the network design phase. The latter treats the dynamic problem how smaller amount of network resources should be used according to the traffic at the moment, on the network deployment phase.

 Level of technologies

Each level of technologies (device, system, and network) is important. Each technology doesn't stand alone, but cooperates each other among different levels. Our objective is to find a total solution which incorporates each level of technologies.

Possible capabilities

Considering technologies listed in section 6, capabilities for energy saving can be categorized as follows:

1) Restraint of increasing capacity needs of networks

1-1) Reduce the volume of traffic to be forwarded

1-2) Peak-shift the traffic as much as possible

2) Improvement of power consumption efficiency of the entire network

2-1) Control device operation for traffic dynamics

2-2) Forward the traffic with less power

Network capacity

Power consumption

(1

2)

Base point

W/bps

1)

Figure 7-1: Power consumption vs. Network capacity

Capability 1) is an approach from traffic manipulation. Power consumption of devices or equipments is defined by the maximum traffic they can accommodate. So, if maximum traffic to be accommodated could be reduced, power consumption would be also reduced according to it.

1-1) is a static solution. It is a technique to reduce the volume of traffic to be forwarded by devices or equipments. One example is providing a cache server at the entrance of networks so as to cache frequent-accessed content on it.

1-2) is a dynamic solution. It is a technique to move the data on time axis and to put down the peak traffic. One example is job scheduling, which allow broadcast content to be distributed in advance.

Capability 2) is an approach from architecture at various levels. Power consumption efficiency usually defined by necessary power to get unit data speed, that is, W/bps. So, if power consumption efficiency could be improved in some ways such as device evolution, total power consumption would be reduced according to it.

2-1) is a dynamic solution. Conventional network devices or equipments always operate at full spec and full speed. It is a technique to control the operation of devices or equipments according to the traffic. One example is sleep control.

2-2) is a static solution. Conventional data transmission is usually carried out on complex layered protocol. It is a technique to transmit data on simplified mechanism, using lower layer, light protocol, and so on. One example is Optical node.

Capabilities we should focus on

There are various methods to satisfy these capabilities. Some of them are technical, and some of them are non-technical. Both of them are important, but we will focus on technical methods in this document.

8.2. Classification of technologies from capabilities

From the viewpoint of the capabilities, technologies listed in section 6 are classified as follows:

[Editor's Note: The granularity of following technologies is diverse. So, we need to fix it.]

Table 1: Correspondence of technologies to capabilities

| Capability Level | 1) Restraint of increasing capacity needs |  | 2) Improvement of power efficiency |  |
| --- | --- | --- | --- | --- |
|  | 1-1) Reduce traffic | 1-2) Peak-shift | 2-1) Dynamic ctl. | 2-2) Less power |
| Device |  |  | - Multi-core CPU - Clock gating | - LSI - Optical node - Power aware virtual memory |
| System (Equipment) | - Cache server - Filtering | - Sorry server - Shaping | - Sleep - ALR/DVS | - Circuit switch - Thermal design - Compact BTSs |
| Network | - CDN | - Job scheduling | - Routing/TE - Energy-aware network design | - Light protocol - Txt. scheduling - Small-cell design |

## 8.3	Minimize device usage for traffic dynamics

Techniques to minimize device usage for traffic dynamics include sleep control and dynamic voltage scaling (DVS) control. Sleep control saves energy by putting a device to "sleep" when it is not in use, and DVS control works by reducing the traffic forwarding capacity of devices such as CPUs, line cards and network interface cards (NICs) when the traffic volume is low. For this reason, sleep control and DVS control are effective when they are applied to networks which characteristically have a large number of devices, low volume of traffic handled by individual networks and low device operating ratios.

From the view point of system technology, the internet traffic is characterized by dynamic variations in terms of time and space. Power consumption of routers, on the other hand, is almost independent of the volume of traffic forwarded and just driving routers consumes a certain amount of electric energy. Accordingly, putting routers to sleep when traffic does not flow into them provides a possible energy-saving technique. In the present situation, however, traffic is distributed for forwarding on a network according to the paths predefined by routing protocols or other methods, which means that traffic, if low volume, flows into each rouer even when the overall volume of traffic is low, resulting in the need to keep each router running all the time. This points to the technical challenge of how to control traffic paths in an entire networks in such a way that allows routers to be brought into a sleep mode.

## 8.4	Forward the traffic with less power

Techniques to forward the traffic with less power include use of evolving device technology and employment of optical nodes along with traffic aggregation. Device technology offers prospects for energy saving by LSI microfabrication and reduction of driving voltage. Use of optical nodes requires traffic aggregation as a prerequisite.

From the view point of system technology, the internet uses packet forwarding, which achieves efficient accommodation of applications by statistical multiplexing effect. The other side of the coin is that further increases in the speed and capacity of transmission lines in the future will require special types of memory that are high-speed, high-capacity and high-power-consumption (such as static random access memory [SRAM] and content addressable memory [CAM] for buffering and routing. However, video traffic, which is expected to increase in the future, is characterized by continuous generation in one direction while a session is maintained and routing is required only on a session-by-session basis. This eliminates the need to use CAM, which is intended for packet-by-packet routing and requires high power consumption, and packet buffers. Accordingly, one technological challenge is to build a new forwarding mechanism that does not require these types of memory to be used.

- 9	Impact analysis of Energy Saving

This section describes impact analysis  of energy saving of networks, especially performance aspects.

[Editor's Note:

Example items to be listed:

- The effect of energy saving to be required or to be realized

When applying technology of energy saving of networks, how much energy can be

saved ?

It may be difficult to describe quantitatively.

- The range of influences to other networks performance(e.g. QoS, security, and so on)

For example - Level-1: no degrade of service quality, Level-2: permissible degrade at

some level, and so on.]

## 9.1	The effect of energy saving to be required or to be realized

From the viewpoint to reduce CO2 emissions, it is important to reduce network power consumption.

[Editor’s Note: The effect of energy saving should be evaluated.]

## 9.2	Influences to other network performance

When introducing energy saving technologies to the networks, they may degrade other networks performance such as QoS, security, and so on, because they may be realized at the expense of traditional network services. It depends on the application services or network systems how much degrade can be admitted. For example, usual mail service can tolerate the delay of several seconds. So, influences to other network performance should be specified.

- 10	Overview of candidate functions

This section describes candidate functions for energy saving of networks.

Figure 1 illustrates a system model for energy saving of networks, which shows functional blocks to be needed and relationship between them.

[Editor's Note: This section will describe overview of candidate functions for energy saving of  networks. To be concrete, functional blocks and their relations(i.e., interfaces) should be described. It seems that the figure in OD-19(the figure above) would be the starting point.]

[Editor's Note: Network devices mean anything in network, so it can mean ‘equipment’. Better wording is necessary.]

[Editor's Note: This figure mention energy cost, not energy consumption. The selection of word, cost or consumption, relates the #1 objective of this document, environment/climate change or reduction of OPEX. Needs to be clarified in the future.]

## 10.1	Management functions

Management functions control network devices through device control interface in order to minimize total energy costs. Device control interface includes a variety of control parameters related to energy, such as routing paths, function usage of devices, and so on.

## 10.2	Network devices

Network devices are network equipments such as routers, switches, and so on. They should have device control interfaces. They could report their energy status to management functions through status monitoring interface. Status monitoring interface includes a variety of device parameters related to energy, such as traffic flow, device availability, and so on.

Appendix I. Examples of technology consideration

## I.1 Environmental Token Consideration in Future Network Routing Capability

Environmental token indicates a level of carbon-free power consumption at a network node, and includes a ratio (R) between carbon-free and total power consumptions at a node. A total of power consumption at a node is the sum of carbon-free and carbon-based-nuclear power consumptions. Environmental token also indicates GHG emissions, and includes carbon footprint (CF) of a node. Therefore, environmental token (ET) at one node can be represented as the following:

ET = (1 – R) x CF

A lower level of carbon-based power consumption would be preferable for mitigating GHG emissions. This means that a larger ratio value (R) would show less carbon-based power consumption and a better routing scheme. A value of carbon footprint will show an equivalent of GHG emission in consideration of Life Cycle Assessment at a network node. Consequently, a smaller value of ET will represent a preferable node in terms of environmental consideration. At each node with a routing capability, both carbon-free and carbon-based power consumptions are monitored, and the ratio R as well as ET are computed. Then, its value is notified to adjacent nodes via a central or distributed manner. (See Figure 1) A node with a routing capability will route information under consideration of the ratio/environmental token. In the figure, each RN will have a different energy saving capability, and small letters of ET will indicate less GHG emissions. Note that a routing capability would include bypassing and rerouting functions.

RN

RN: Network Node with Routing Capability

CCN: Central Control Node

Environmental Token Exchange in Distributed Manner

Environmental Token Exchange in Centralized Manner

RN

RN

RN

CCN

【ET】

【ET】

【ET】

Figure I.1 - Environmental Token Exchange

[Editor's Note: calculation of cost in global manner is an interesting issue for further study.]

## Appendix II. Examples of energy saving technology

## II.1 ADSL2/ADSL2plus Low-Power Mode Guidelines

The purpose of this appendix is to provide practical guidelines for network providers and service providers for the use of the existing low-power operational mode embodied in ITU-T Recommendations for ADSL2[26] and ADSL2plus[27] technology, while minimizing the risk that some emerging high speed services (e.g., IPTV) and delay sensitive services (e.g., VoIP) could be adversely impacted by the use of the ADSL2/2plus L2 mode. Furthermore, there is a need to use energy as efficiently as possible in order to minimize carbon footprint in a low-carbon economy. The use of low power mode(s) (e.g., L2) for ADSL2 and ADSL2plus is recognized as a key method of reducing the carbon footprint of the broadband ecosystem.

## II.1.1 Definitions

- L0 state: full power management state achieved after the initialization procedure has completed successfully (the ADSL link is fully functional)

- L2 state: low power management state (the ADSL link is active but a low power signal conveying background data is sent from the network operator to the CP

- L3 state: link state (Idle) at the start of the initialization procedure (there is no signal transmitting, the ADSL transceiver unit may be powered or unpowered)

- L0-TIME: minimum time between Exit from the L2 state and the next Entry into L2 state

- L2-TIME: minimum time between Entry into the L2 state and the first Power Trim in the L2 state and between two consecutive Power Trims in the L2 state

- L2-ATPR: Maximum aggregate transmit power reduction that can be performed at transition of L0 to L2 state

- L2-ATPRT: Total Maximum aggregate transmit power reduction that can be performed in an L2 state

## II.1.2 Recommended guidelines for key parameters

If L2 power saving mode is used in the access network it is recommended that the control parameters for it are set according to Table 2, and that any incumbent or local loop unbundling operator sharing the same metallic infrastructure use the same set of parameter values.

Table 2: Recommended values for L2 mode control parameters

| L2 mode parameter | Recommended value |
| --- | --- |
| L2-ATPR (dB) | 1 |
| L2-TIME (seconds) | ≥127 |
| L2-ATPRT (dB) | ≤10 |
| L0-TIME (seconds) | ≥127 |

The above parameters are defined in [25] and [29] as well as in sub-section II.1.1 of this appendix.

NOTE: Application of these recommended values will result in a power saving of the order 75% [28] of that achievable using the most aggressive power saving parameters but with a much reduced impact on DSL performance (by virtue of the fact of the increased damping in the control system which minimizes the fluctuating crosstalk).

NOTE: It may be appropriate also to set L2 “Minimum data rate in low power state” to a value lower than that necessary to support VoIP, so that at the start of a session the link immediately exits L2 state to ensure QoS and QoE for the duration of the call.

## II.1.3 Operation of L2 mode

In the description of these power saving modes L0 mode refers to the normal (full power operation), while L2 mode refers to the main mode of interest, a power saving mode that stops short of actually turning the transmit signal off altogether. The operation of this L2 mode is illustrated in Figure II-1.

Figure II.1 – How L2 mode works

In this figure it is assumed that initially the modem is operating in L0 mode and therefore transmitting at full power, when it is detected that little or no user data is being transmitted. As a result entry into L2 mode is enabled, resulting in an initial power reduction by L2-ATPR dB.

NOTE: The L2 power trim sent by the DSLAM may be rejected by the CPE due to an invalid or infeasible parameter. This can occur when there is no capacity left for further power reduction, or if further reduction of power might result in instability (negative noise margin on certain tones).

This level of power reduction shall be maintained for a period of at least L2-TIME seconds before further power reduction is permitted. Once this time has passed, if user data requirements still permit, a further reduction in power by L2-ATPR is enabled, and so on, provided that the total power reduction does not exceed L2-ATPRT. If at any time user data throughput demands a higher transmit power level then immediate return to L0 mode is the only option. Once in L0 mode again, this mode shall be maintained for at least L0-TIME seconds before L2 mode can be invoked again. The values of L2-ATPR, L2-TIME, L2-ATPRT and L0-TIME are all under control of the network operator and can be configured through the DSLAM or MSAN management system.

## Living List

## L.1 Candidate functions

The following is a proposed text as candidate functions in C-98. This contribution proposes a very important and interesting figure, but the maturity of it is not sufficient. So the materials of this contribution is added to a living list section here. The more detailed observation is for further study.

==== Start of proposed text ====

10 Overview of candidate functions

From the considerations of section 8 "Capabilities", Energy Saving of Networks should have the following as candidate functions, as shown in figure 10-x.

- 1st to 3rd columns represent transmit functions, and 4th column represents management functions. Transmit functions are also classified into three levels by rows: device, system (equipment), and network level.

- Among transmit functions, 1st column represents functions to forward the traffic with less power, and 2nd column represents functions to control device/system operation for traffic dynamics, and 3rd column represents functions to reduce or discard the useless traffic.

- Management functions monitor the status of transmit functions, and control them in order to minimize total electric power.

- Upper layer transmit functions may control or use lower layer transmit functions in order to control electric power.

Device level

System (Equipment) level

Network level

Low-power

device

Dynamic device

operation

Dynamic system

operation

Low-power

node

Light-weight

protocol

Energy-aware

routing

Packet

processing

Flow

processing

Control

Control

Monitor

Energy

Monitoring

Operation

Decision

Management

functions

Less power

functions

Dynamic ctl.

functions

Reduce traffic

functions

Control

Monitor

Monitor

Figure 10-x: Candidate functions

[Editor's Note: There are so many controversial points regarding the above figure, such as the necessity of "Monitor" arrows among transmit functions, and the necessity of "Control" and "Monitor" arrows between Device level and Management functions.]

10.1 Low-power device

It is a device function which consumes less power than conventional devices, such as LSI microfabrication and Clock gating.

10.2 Low-power node architecture

It is a function of node architecture which consumes less power than conventional nodes, such as circuit switch and thermal design.

10.3 Light-weight protocol

It is a function of simplified network protocol, such as MPLS.

10.4 Dynamic device operation

It is a function to control device operation  mode according to the necessary processing, such as dynamic voltage scaling (DVS) and dynamic clock operation technique.

10.5 Dynamic system operation

It is a function to control system operation mode according to the traffic, such as sleep mode and adaptive link rate (ALR).

10.6 Energy-aware routing

It is a routing/traffic engineering function to use a routing path with less energy, to aggregate the traffic routes and make sleep unused nodes, and so on.

10.7 Packet processing

It is a function not to transmit unnecessary traffic at appropriate nodes, such as filtering and cache server.

10.8 Flow handling

It is a function to optimize data flow at network level and reduce the traffic, such as Contents Delivery Network (CDN) and redirect.

10.9 Energy Monitoring

It is a function to monitor the energy which is consumed in transmit functions.

10.10 Operation Decision

It is a function to decide an operation mode for each transmit function and control it.

==== End of proposed text ====

_____________________

In global environment, IP traffic is estimated to increase by 34% every year [4]. If this rate of increase continues, IP traffic of 2025 will be 350 times that of 2005. Accordingly, It will cause estimated network power consumption in 2025 is predicted to be 6.2 times the 2005 level [5]. The energy consumption of the networks in 2007 is 3% among the overall energy consumption , it is increasing by 16~20 every year [8]

The Gartner estimated that global carbon footprint of networks in 2007 is 2&, which is similar percentage of the aviation [7].  and its percentage will be expected to increase by 2.3 times, from 2002 to 2020 [6].