RECOMMENDATION  ITU-R  BO.1445

IMPROVED  PATTERNS  FOR  FAST  ROLL-OFF  SATELLITE  TRANSMIT  ANTENNAS  
OF  THE  REGIONS  1  AND  3  BSS  PLANS  OF  RR  APPENDIX  S30

(Question ITU-R 93/11)

(2000)

Rec. ITU-R BO.1445

The ITU Radiocommunication Assembly,

considering

a)	that Resolution 532 (WRC-97) (World Radiocommunication Conference (Geneva, 1997)) invites ITU-R to study the possibilities to improve the efficiency of the Appendix S30 Plan of the RR by taking due account of the technical progress;

b)	that the existing RR Appendix S30 Regions 1 and 3 satellite transmitting antenna patterns with roll-off in the main beam can be improved to reflect technical progress;

c)	that the use of antennas with improved radiation patterns will lead to a more efficient use of the radio spectrum and the GSO,

recognizing

a)	that the adoption of improved satellite transmitting antenna patterns with fast roll-off in the main beam does not prevent the use of other antennas that have been coordinated or will be coordinated in the future on the basis of different patterns;

b)	that the next competent conference may include these patterns into Annex 5 of RR Appendix S30,

recommends

1	to use the improved satellite antenna co-polar and cross-polar patterns with fast roll-off in the main beam as provided in Annex 1, for elliptical beams of fast roll-off when such fast roll-off antenna is required, e.g. in studies in accordance with Resolution 532 (WRC-97).

ANNEX  1

FIGURE  1  [1445-01]

Curve A:	co-polar relative gain  ΔG=minΔG1,ΔG2 (dB):

where:

ΔG1=−12ϕϕ02	for 0  (/0)  0.5

ΔG1=−12ϕϕ0−xBminϕ02	for 0.5  (/0)  1.45ϕ0Bmin+x

ΔG1=−25.3	for 1.45ϕ0Bmin+x  (/0)  1.45

ΔG1=−22+20 logϕϕ0	for (0)  1.45

ΔG1=−Gon−axis	after intersection with Curve C

ΔG2=−12ϕϕ02	for 0    1.58 0

ΔG2=−30	for 1.58 0    3.16 0

ΔG2=−17.5+25 logϕϕ0	for   3.16 0

ΔG2=−Gon−axis	after intersection with Curve C

Curve B:	cross-polar relative gain (dB):

−40+40 logϕϕ0−1	for 0    0.33 0

−33	for 0.33 0    1.67 0

−40+40 logϕϕ0−1	for  > 1.67 0

−Gon−axis	after intersection with Curve C

Curve C:	minus the on-axis gain (Curve C in this Figure illustrates the particular case of an antenna with an on-axis gain of 42.773 dBi)

where:

:	off-axis angle (degrees)

0:	cross-sectional half-power beamwidth in the direction of interest (degrees)

Bmin:	0.6 for Regions 1 and 3

x=0.5 1−Bminϕ0

The relationship between the maximum gain of an antenna and the half-power beamwidth can be derived from the expression:

Gmax=27 843a b

where:

a and b are the angles (degrees) subtended at the satellite by the major and minor axes of the elliptical cross-section of the beam; an antenna efficiency of 55% was assumed.