RECOMMENDATION  ITU-R  BO.789-1*

DIGITAL  SOUND  BROADCASTING  TO  VEHICULAR,  PORTABLE  AND  FIXED
RECEIVERS  FOR  BROADCASTING-SATELLITE  SERVICE  (SOUND)
IN  THE  FREQUENCY  RANGE  500-3 000  MHz

(Question ITU-R 93/10)

(1992-1994)

Rec. ITU-R BO.789-1

The ITU Radiocommunication Assembly,

considering

a)	that there is an increasing requirement worldwide for suitable means of broadcasting a range of sound qualities up to high-quality stereophonic two-channel/multi-channel sound with subjective quality indistinguishable from high-quality consumer digital recorded media (“CD quality”) to vehicular, portable and fixed receivers;

b)	that technical developments in source and channel coding, modulation and advanced digital signal processing have demonstrated the technical feasibility and maturity of digital sound broadcasting systems;

c)	that a large series of demonstrations and field trials in various parts of the world have confirmed the technical and economic viability of digital sound broadcasting systems from a system design point of view;

d)	that a digital sound broadcasting system can provide better spectrum and power efficiency, as well as better performance in multipath environments, than conventional analogue systems;

e)	that satellite-based digital sound broadcasting systems can provide full coverage for sub-national, national or supra-national service areas;

f)	that the complementary use of satellite and terrestrial systems can result in better power and spectrum efficiency through the implementation of hybrid and mixed satellite/terrestrial digital sound broadcasting services;

g)	that a digital broadcasting system can be employed efficiently in both satellite and terrestrial applications using closely related emission signal parameters, thus allowing common receiver design with common processing VLSI circuits;

h)	that sound broadcasting has always used similar modulation techniques, such as AM or FM, and similar, if not identical, frequency bands, leading to a receiver that could be used worldwide, for the benefit of the listener,

further considering

j)	that  the World Administrative Radio Conference (Geneva,1979) (WARC-79) adopted Resolution No. 505 which encouraged technical experiments in the band 1 429-1 525 MHz and that the Second Session of the World Administrative Radio Conference on the Use of the Geostationary-Satellite Orbit and on the Planning of Space Services Utilizing It (Geneva, 1988) (WARC ORB-88) adopted Resolution No. 520 which proposed to give a mandate for a frequency allocation for one or more bands in the frequency range from 500-3 000 MHz to a competent ITU conference;

k)	that the World Administrative Radio Conference for Dealing with Frequency Allocations in Certain Parts of the Spectrum (Malaga-Torremolinos, 1992) (WARC-92) has allocated the band 1 452-1 492 MHz to the broadcasting-satellite service (BSS) (sound) and complementary terrestrial broadcasting service for the provision of digital audio broadcasting. Also, additional footnote allocations were included for specific countries in the band 2 310-2 360 MHz and in the band 2 535-2 655 MHz in Radio Regulations Nos. 750B and 757A,

recommends

that, when sound broadcasting from satellite and complementary terrestrial services, intended for vehicular, portable and fixed reception (see Note 1), are introduced into the frequency ranges allocated by WARC-92 to BSS (sound), digital sound broadcasting systems should be used and should have the following technical and operational characteristics and capabilities for the transmitter/receiver components of a total BSS/BS (sound) system:

Note 1 – It is understood that some administrations may wish to develop BSS(S) and BS systems that do not provide the entire range of characteristics listed under recommends. For example an administration may wish to have a service that provides the equivalent of monophonic FM audio intended primarily for reception by very low cost fixed or portable receivers, rather than vehicular-mounted receivers. Nevertheless, it is understood that such administrations would endeavour to develop digital sound broadcasting systems that conform, to the extent practicable, with the characteristics cited in this Recommendation.

1.	be capable of providing a range of audio qualities up to high-quality stereophonic two-channel/multi-channel sound with subjective quality indistinguishable from high-quality consumer digital recorded media (“CD quality”) to vehicular, portable and fixed receivers;

2.	provide better spectrum efficiency than conventional analogue FM systems;

3.	provide significantly improved performance in a multipath and shadowing environment through the use of the service concepts, systems and mitigation techniques outlined in relevant ITU-R texts;

4.	be capable of utilizing common signal processing in receivers for any satellite and terrestrial broadcasting applications;

5.	allow configuration/reconfiguration in order to transmit sound programmes with lower bit rates to trade-off quality and the number of sound programmes available;

6.	allow for a trade-off between extent of coverage for a given emission power, service quality and number of sound programmes and data services;

7.	be capable of allowing, with a common receiver, the use of all means of programme delivery, such as:

–	sub-national, national or supra-national UHF satellite services,

–	mixed/hybrid use of satellite and complementary terrestrial services,

–	local, sub-national and national VHF/UHF terrestrial services,

–	cable distribution networks;

8.	be capable of providing enhanced facilities for programme-related data (e.g. service identification, programme labelling, programme delivery control, copyright control, conditional access, dynamic programme linking, services for visually and hearing-impaired, etc.);

9.	be capable of providing value-added services with different data capacities (e.g. traffic message channels, business data, paging, still picture/graphics, future integrated services digital broadcasting (ISDB), low bit rate video/audio multiplex, etc.);

10.	allow for flexible assignment of services within a given multiplex;

11.	a system multiplex structure capable of complying with the layered International Organization for Standardization (ISO) open system interconnect model and permitting interfacing to information technology equipment and communications networks;

12.	allow manufacturing low-cost receivers and antennas through mass production.

Note 1 – An example of a digital sound broadcasting system (Digital System A) that meets the above technical and operational requirements is described in Annex 1. Another example is given in Annex 2 (Digital System B) which is under development and test in the United States of America. Technology in this area is developing rapidly. Accordingly, if additional systems meeting these requirements become available, they should be considered for inclusion as examples. Studies are being conducted in various parts of the world on several methods for digital sound broadcasting that take these requirements into account.

Note 2 – System and service characteristics as well as radio-frequency aspects of digital sound broadcasting systems are considered in detail in relevant ITU-R texts.

Note 3 – Recommendation ITU-R BS.774 is closely related to terrestrial digital sound broadcasting.

ANNEX  1

Digital System A

# 1.	Introduction

Digital System A is designed to provide high-quality, multi-service digital radio broadcasting for reception by vehicular, portable and fixed receivers. It is designed to operate at any frequency up to 3 000 MHz for terrestrial, satellite, hybrid (satellite and terrestrial), and cable broadcast delivery. The System is also designed as a flexible, general-purpose integrated services digital broadcasting (ISDB) system which can support a wide range of source and channel coding options, sound-programme associated data and independent data services, in conformity with the flexible and broad-ranging service and system requirements given in Recommendations ITU-R BO.789 and ITU-R BS.774, supported by ITU-R Reports BS.1203 and BO.955.

The system is a rugged, yet highly spectrum and power-efficient, sound and data broadcasting system. It uses advanced digital techniques to remove redundancy and perceptually irrelevant information from the audio source signal, then it applies closely-controlled redundancy to the transmitted signal for error correction. The transmitted information is then spread in both the frequency and time domains so that a high quality signal is obtained in the receiver, even when working in conditions of severe multipath propagation, whether stationary or mobile. Efficient spectrum utilization is achieved by interleaving multiple programme signals and a special feature of frequency re-use permits broadcasting networks to be extended, virtually without limit, using additional transmitters all operating on the same radiated frequency.

A conceptual diagram of the emission part of the System is shown in Fig. 1.

Digital System A has been developed by the Eureka 147 digital audio broadcasting (DAB) Consortium and is known as the Eureka DAB System. It has been actively supported by the EBU in view of introducing digital sound broadcasting services in Europe in 1995. Since 1988, the System has been successfully demonstrated and extensively tested in Europe, Canada, the United States of America and in other countries worldwide. In this Annex, Digital System A is referred to as “the System”. The full system specification will be available as a European Telecommunications Standard.

# 2.	Use of a layered model

The System is capable of complying with the International Organization for Standardization (ISO) open system interconnection (OSI) basic reference model described in ISO 7498 (1984). The use of this model is recommended in Recommendation ITU-R BT.807 and Report ITU-R BT.1207, and a suitable interpretation for use with layered broadcasting systems is given in the Recommendation. In accordance with this guidance, the System will be described in relation to the layers of the model, and the interpretation applied here is illustrated in Table 1.

Descriptions of many of the techniques involved are most easily given in relation to the operation of the equipment at the transmitter, or at the central point of a distribution network in the case of a network of transmitters.

The fundamental purpose of the System is to provide sound programmes to the radio listener, so the order of sections in the following description will start from the application layer (use of the broadcast information), and proceed downwards to the physical layer (the means of radio transmission).

# 3.	Application layer

This layer concerns the use of the System at the application level. It considers the facilities and audio quality which the System provides and which broadcasters can offer to their listeners, and the different transmission modes.

## 3.1	Facilities offered by the System

The System provides a signal which carries a multiplex of digital data, and this conveys several programmes at the same time. The multiplex contains audio programme data, and ancillary data comprising programme-associated data (PAD), multiplex configuration information (MCI) and service information (SI). The multiplex may also carry general data services which may not be related to the transmission of sound programmes.

FIGURE 1/BO.789-1...[D01] = 23 CM

TABLE  1

Interpretation of the OSI layered model

| Name of layer | Description | Features specific to the System |
| --- | --- | --- |

| Application layer | Practical use of the system | System facilities Audio quality Transmission modes |
| --- | --- | --- |

| Presentation layer | Conversion for presentation | Audio encoding & decoding Audio presentation Service information |
| --- | --- | --- |

| Session layer | Data selection | Programme selection Conditional access |
| --- | --- | --- |

| Transport layer | Grouping of data | Programme services Main service multiplex Ancillary data Association of data |
| --- | --- | --- |

| Network layer | Logical channel | ISO audio frames Programme associated data |
| --- | --- | --- |

| Data link layer | Format of the transmitted signal | Transmission frames Synchronization |
| --- | --- | --- |

| Physical layer | Physical (radio) transmission | Energy dispersal Convolutional encoding Time interleaving Frequency interleaving Modulation by 4-DPSK OFDM Radio transmission |
| --- | --- | --- |

In particular, the following facilities are made available to users of the System:

–	the audio signal (i.e. the programme) being provided by the selected programme service;

–	the optional application of receiver functions, for example dynamic range control, which may use ancillary data carried with the programme;

–	a text display of selected information carried in the SI. This may be information about the selected programme, or about others which are available for optional selection;

–	options which are available for selecting other programmes, other receiver functions, and other SI;

–	one or more general data services, for example a traffic message channel (TMC).

The System includes facilities for conditional access, and a receiver can be equipped with digital outputs for audio and data signals.

## 3.2	Audio quality

Within the capacity of the multiplex, the number of programme services and, for each, the presentation format (e.g. stereo, mono, surround-sound, etc.), the audio quality and the degree of error protection (and hence ruggedness) can be chosen to meet the needs of the broadcasters.

The following range of options is available for the audio quality:

–	very high quality, with audio processing margin,

–	subjectively transparent quality, sufficient for the highest quality broadcasting,

–	high quality, equivalent to good FM service quality,

–	medium quality, equivalent to good AM service quality,

–	speech-only quality.

The System provides full quality reception within the limits of transmitter coverage; beyond these limits reception degrades in a subjectively graceful manner.

## 3.3	Transmission modes

The System has three alternative transmission modes which allow the use of a wide range of transmitting frequencies up to 3 GHz. These transmission modes have been designed to cope with Doppler spread and delay spread, for mobile reception in presence of multipath echoes.

Table 2 gives the constructive echo delay and nominal frequency range for mobile reception. The noise degradation at the highest frequency and in the most critical multipath condition, occurring infrequently in practice, is equal to 1 dB at 100 km/h.

TABLE  2

| Parameter | Mode I | Mode II | Mode III |
| --- | --- | --- | --- |

| Guard interval duration (ms) | 246 | 62 | 31 |
| --- | --- | --- | --- |

| Constructive echo delay up to (ms) | 300 | 75 | - 37.5 |
| --- | --- | --- | --- |

| Nominal frequency range (for mobile reception) up to | 375 MHz | 1.5 GHz | 3 GHz |
| --- | --- | --- | --- |

From this table, it can be seen that the use of higher frequencies imposes a greater limitation on the maximum echo delay. Mode I is most suitable for a terrestrial single-frequency network (SFN), because it allows the greatest transmitter separations. Mode II is most suitable for local radio applications requiring one terrestrial transmitter, and hybrid satellite/terrestrial transmission up to 1.5 GHz. However, Mode II can also be used for a medium-to-large scale SFN (e.g. at 1.5 GHz) by inserting, if necessary, artificial delays at the transmitters and/or by using directive transmitting antennas. Mode III is most appropriate for satellite and complementary terrestrial transmission at all frequencies up to 3 GHz.

Mode III is also the preferred mode for cable transmission up to 3 GHz.

# 4.	Presentation layer

This layer concerns the conversion and presentation of the broadcast information.

## 4.1	Audio source encoding

The audio source encoding method used by the System is ISO/IEC MPEG-Audio Layer II, given in the ISO Standard 11172-3. This sub-band coding compression system is also known as the MUSICAM system.

The System accepts a number of PCM audio signals at a sampling rate of 48 kHz with PAD. The number of possible audio sources depends on the bit rate and the error protection profile. The audio encoder can work at 32, 48, 56, 64, 80, 96, 112, 128, 160 or 192 kbit/s per monophonic channel. In stereophonic or dual channel mode, the encoder produces twice the bit rate of a mono channel.

The different bit-rate options can be exploited by broadcasters depending on the intrinsic quality required and/or the number of sound programmes to be provided. For example, the use of bit-rates greater than or equal to 128 kbit/s for mono, or greater than or equal to 256 kbit/s for a stereo programme, provides not only very high quality, but also some processing margin, sufficient for further multiple encoding/decoding processes, including audio post-processing. For high-quality broadcasting purposes, a bit-rate of 128 kbit/s for mono or 256 kbit/s for stereo is preferred, giving fully transparent audio quality. Even the bit-rate of 192 kbit/s per stereo programme generally fulfils the EBU requirement for digital audio bit-rate reduction systems. A bit-rate of 96 kbit/s for mono gives good sound quality, and 48 kbit/s can provide roughly the same quality as normal AM broadcasts. For some speech-only programmes, a bit-rate of 32 kbit/s may be sufficient where the greatest number of services is required within the system multiplex.

A block diagram of the functional units in the audio encoder is given in Fig. 2. The input PCM audio samples are fed into the audio encoder. One encoder is capable of processing both channels of a stereo signal, although it may, optionally, be presented with a mono signal. A polyphase filter bank divides the digital audio signal into 32 sub-band signals, and creates a filtered and sub-sampled representation of the input audio signal. The filtered samples are called sub-band samples. A perceptual model of the human ear creates a set of data to control the quantizer and coding. These data can be different, depending on the actual implementation of the encoder. One possibility is to use an estimation of the masking threshold to obtain these quantizer control data. Successive samples of each sub-band signal are grouped into blocks, then in each block, the maximum amplitude attained by each sub-band signal is determined and indicated by a scale factor. The quantizer and coding unit creates a set of coding words from the sub-band samples. These processes are carried out during ISO audio frames, which will be described in the network layer.

FIGURE 2/BO.789-1...[D02] = 11 CM

## 4.2	Audio decoding

Decoding in the receiver is straightforward and economical using a simple signal processing technique, requiring only demultiplexing, expanding and inverse-filtering operations. A block diagram of the functional units in the decoder is given in Fig. 3.

The ISO audio frame is fed into the ISO/MPEG-Audio Layer II decoder, which unpacks the data of the frame to recover the various elements of information. The reconstruction unit reconstructs the quantized sub-band samples, and an inverse filter bank transforms the sub-band samples back to produce digital uniform PCM audio signals at 48 kHz sampling rate.

## 4.3	Audio presentation

Audio signals may be presented monophonically or stereophonically, or audio channels may be grouped for surround-sound. Programmes may be linked to provide the same programme simultaneously in a number of different languages. In order to satisfy listeners in both Hi-Fi and noisy environments, the broadcaster can optionally transmit a dynamic range control (DRC) signal which can be used in the receiver in a noisy environment to compress the dynamic range of the reproduced audio signal. Note that this technique can also be beneficial to listeners with impaired hearing.

FIGURE 3/BO.789-1...[D03] = 6 CM

## 4.4	Presentation of Service Information

With each programme transmitted by the System, the following elements of SI can be made available for display on a receiver:

–	basic programme label (i.e. the name of the programme),

–	time and date,

–	cross-reference to the same, or similar programme (e.g. in another language) being transmitted in another ensemble or being simulcast by an AM or FM service,

–	extended service label for programme-related services,

–	programme information (e.g. the names of performers),

–	language,

–	programme type (e.g. news, sport, music, etc.),

–	transmitter identifier,

–	traffic message channel (TMC, which may use a speech synthesizer in the receiver).

Transmitter network data can also be included for internal use by broadcasters.

# 5.	Session layer

This layer concerns the selection of, and access to, broadcast information.

## 5.1	Programme selection

So that a receiver can gain access to any or all of the individual services with a minimum overall delay, information about the current and future content of the multiplex is carried by the fast information channel (FIC). This information is the MCI, which is machine-readable data. Data in the FIC are not time-interleaved, so the MCI is not subject to the delay inherent in the time-interleaving process applied to audio and general data services. However, these data are repeated frequently to ensure their ruggedness. When the multiplex configuration is about to change, the new information, together with the timing of the change is sent in advance in the MCI.

The user of a receiver can select programmes on the basis of textual information carried in the SI, using the programme service name, the programme type identity or the language. The selection is then implemented in the receiver using the corresponding elements of the MCI.

If alternative sources of a chosen programme service are available and an original digital service becomes untenable, then linking data carried in the SI (i.e. the “cross reference”) may be used to identify an alternative (e.g. on an FM service) and switch to it. However, in such a case, the receiver will switch back to the original service as soon as reception is possible.

## 5.2	Conditional access

Provision is made for both synchronization and control of conditional access.

Conditional access can be applied independently to the service components (carried either in the MSC or FIC), services or the whole multiplex.

# 6.	Transport layer

This layer concerns the identification of groups of data as programme services, the multiplexing of data for those services and the association of elements of the multiplexed data.

## 6.1	Programme services

A programme service generally comprises an audio service component and optionally additional audio and/or data service components, provided by one service provider. The whole capacity of the multiplex may be devoted to one service provider (e.g. broadcasting five or six high-quality sound programme services), or it may be divided amongst several service providers (e.g. collectively broadcasting some twenty medium quality programme services).

## 6.2	Main service multiplex

With reference to Fig. 1, the data representing each of the programmes being broadcast (digital audio data with some ancillary data, and maybe also general data) are subjected to convolutional encoding (see § 9.2) and time-interleaving, both for error protection. Time-interleaving improves the ruggedness of data transmission in a changing environment (e.g. reception by a moving vehicular receiver) and imposes a predictable transmission delay. The interleaved and encoded data are then fed to the main service multiplexer where, each 24 ms, the data are gathered in sequence into the multiplex frame. The combined bit-stream output from the multiplexer is known as the main service channel (MSC) which has a gross capacity of 2.3 Mbit/s. Depending on the chosen code rate (which can be different from one service component to another), this gives a net bit rate ranging from approximately 0.8 to 1.7 Mbit/s, through a 1.5 MHz bandwidth. The main service multiplexer is the point at which synchronized data from all of the programme services using the multiplex are brought together.

General data may be sent in the MSC as an unstructured stream or organized as a packet multiplex where several sources are combined. The data rate may be any multiple of 8 kbit/s, synchronized to the System multiplex, subject to sufficient total multiplex capacity, taking into account the demand for audio services.

The FIC is external to the MSC and is not time-interleaved.

## 6.3	Ancillary data

There are three areas where ancillary data may be carried within the System multiplex:

–	the FIC, which has limited capacity, depending on the amount of essential MCI included,

–	there is special provision for a moderate amount of PAD to be carried within each audio channel,

–	all remaining ancillary data are treated as a separate service within the MSC. The presence of this information is signalled in the MCI.

## 6.4	Association of data

A precise description of the current and future content of the MSC is provided by the MCI, which is carried by the FIC. Essential items of SI which concern the content of the MSC (i.e. for program selection) must also be carried in the FIC. More extensive text, such as a list of all the day’s programs, must be carried separately as a general data service. Thus, the MCI and SI contain contributions from all of the programs being broadcast.

The PAD, carried within each audio channel, comprises mainly the information which is intimately linked to the sound program and therefore cannot be sent in a different data channel which may be subject to a different transmission delay.

# 7.	Network layer

This layer concerns the identification of groups of data as programmes.

## 7.1	ISO audio frames

The processes in the audio source encoder are carried out during ISO audio frames of 24 ms duration. The bit allocation, which varies from frame to frame, and the scale factors are coded and multiplexed with the sub-band samples in each ISO audio frame. The frame packing unit (see Fig. 2) assembles the actual bit stream from the output data of the quantizer and coding unit, and adds other information, such as header information, CRC words for error detection, and PAD, which travel along with the coded audio signal. Each audio channel contains a PAD channel having a variable capacity (generally at least 2 kbit/s), which can be used to convey information which is intimately linked to the sound program. Typical examples are lyrics, speech/music indication and dynamic range control (DRC) information.

The resulting audio frame carries data representing 24 ms duration of stereo (or mono) audio, plus the PAD, for a single programme and complies with the ISO 11172-3 Layer II format, so it can be called an ISO frame. This allows the use of an ISO/MPEG-Audio Layer II decoder in the receiver.

# 8.	Data link layer

This layer provides the means for receiver synchronization.

## 8.1	The transmission frame

In order to facilitate receiver synchronization, the transmitted signal is built up with a regular frame structure (see Fig. 4). The transmission frame comprises a fixed sequence of symbols. The first is a null symbol to provide a coarse synchronization (when no RF signal is transmitted), followed by a fixed reference symbol to provide fine synchronization, AGC, AFC and phase reference functions in the receiver; these symbols make up the synchronization channel. The next symbols are reserved for the FIC, and the remaining symbols provide the MSC. The total frame duration TF is either 96 ms or 24 ms, depending on the transmission mode as given in Table 3.

Each audio service within the MSC is allotted a fixed time slot in the frame.

FIGURE 4/BO.789-1...[D04] = 4 CM

TABLE  3

Transmission parameters of the System

|  | Mode I | Mode II | Mode III |
| --- | --- | --- | --- |

| Total frame duration, TF | 96 ms | 24 ms | 24 ms |
| --- | --- | --- | --- |
| Null symbol duration, TNULL | 1.297 ms | 324 ms | 168 ms |
| Overall symbol duration, Ts | 1.246 ms | 312 ms | 156 ms |
| Useful symbol duration, ts | 1 ms | 250 ms | 125 ms |
| Guard interval duration, D | 246 ms | 62 ms | 31 ms |
| (Ts = ts + D) |  |  |  |
| Number of radiated carrier, N | 1 536 | 384 | 192 |

# 9.	The physical layer

This layer concerns the means for radio transmission (i.e. the modulation scheme and the associated error protection).

## 9.1	Energy dispersal

In order to ensure appropriate energy dispersal in the transmitted signal, the individual sources feeding the multiplex are scrambled.

## 9.2	Convolutional encoding

Convolutional encoding is applied to each of the data sources feeding the multiplex to ensure reliable reception. The encoding process involves adding deliberate redundancy to the source data bursts (using a constraint length of 7). This gives “gross” data bursts.

In the case of an audio signal, greater protection is given to some source-encoded bits than others, following a pre-selected pattern known as the unequal error protection (UEP) profile. The average code rate, defined as the ratio of the number of source-encoded bits to the number of encoded bits after convolutional encoding, may take a value from 1/3 (the highest protection level) to 3/4 (the lowest protection level). Different average code rates can be applied to different audio sources, subject to the protection level required, and the bit-rate can be applied to different audio sources, subject to the protection level required and the bit-rate of the source-encoded data. For example, the protection level of audio services carried by cable networks may be lower than that of services transmitted in radio-frequency channels.

General data services are convolutionally encoded using one of a selection of uniform rates. Data in the FIC are encoded at a constant 1/3 rate.

## 9.3	Time interleaving

Time interleaving of interleaving depth of 16 frames is applied to the convolutionally encoded data in order to provide further assistance to a mobile receiver.

## 9.4	Frequency interleaving

In the presence of multipath propagation, some of the carriers are enhanced by constructive signals, while others suffer destructive interference (frequency selective fading). Therefore, the System provides frequency interleaving by a re-arrangement of the digital bit stream amongst the carriers, such that successive source samples are not affected by a selective fade. When the receiver is stationary, the diversity in the frequency domain is the prime means to ensure successful reception.

## 9.5	Modulation by 4-DPSK OFDM

The System uses 4-DPSK OFDM. This scheme meets the exacting requirements of high bit-rate digital broadcasting to mobile, portable and fixed receivers, especially in multipath environments.

The basic principle consists of dividing the information to be transmitted into a large number of bit-streams having low bit-rates individually, which are then used to modulate individual carriers. The corresponding symbol duration becomes larger than the delay spread of the transmission channel. In the receiver any echo shorter than the guard interval will not cause inter-symbol interference but rather contribute positively to the received power (see Fig. 5). The large number N of carriers is known collectively as an ensemble.

FIGURE 5/BO.789-1...[D05] = 8 CM

In the presence of multipath propagation, some of the carriers are enhanced by constructive signals, while others suffer destructive interference (frequency selective fading). Therefore, the System includes a redistribution of the elements of the digital bit stream in time and frequency, such that successive source samples are affected by independent fades. When the receiver is stationary, the diversity in the frequency domain is the only means to ensure successful reception; the time diversity provided by time-interleaving does not assist a static receiver. For the System, multipath propagation is a form of space-diversity and is considered to be a significant advantage, in stark contrast to conventional FM or narrow-band digital systems where multipath propagation can completely destroy a service.

In any system able to benefit from multipath, the larger the transmission channel bandwidth, the more rugged the system. In the System, an ensemble bandwidth of 1.5 MHz was chosen to secure the advantages of the wideband technique, as well as to allow planning flexibility. Table 3 also indicates the number of OFDM carriers within this bandwidth for each transmission mode.

A further benefit of using OFDM is that high spectrum and power efficiency can be obtained with single frequency networks for large area coverage and also for dense city area networks. Any number of transmitters providing the same programmes may be operated on the same frequency, which also results in an overall reduction in the required operating powers. As a further consequence distances between different service areas are significantly reduced.

Because echoes contribute to the received signal, all types of receiver (i.e. portable, home and vehicular) may utilize simple, non-directional antennas.

## 9.6	Spectrum of the RF signal

The spectrum of the system ensemble is shown in Fig. 6.

FIGURE 6/BO.789-1...[D06] = 14 CM

# 10.	RF performance characteristics of Digital System A

RF evaluation tests have been carried out on Digital System A using Mode I at 226 MHz and Mode II at 1 500 MHz for a variety of conditions representing mobile and fixed reception. Measurements of BER vs. C/N were made on a data channel using the following conditions:

D = 64 kbit/s, R = 0.5

D = 24 kbit/s, R = 0.375

where:

D :	source data rate and

R :	average channel code rate.

## 10.1	BER vs. C/N (in 1.5 MHz) in a Gaussian channel at 226 MHz

Additive, Gaussian white noise was added to set the C/N at the input of the receiver. The results are shown in Fig. 7. As an example, for R = 0.5, the measured results can be compared with those from a software simulation, to show the inherent performance of the system. It can be seen that an implementation margin of less than 0.5 dB is obtained at a bit-error ratio (BER) of 10-4.

FIGURE 7/BO.789-1...[D07] = 17 CM

## 10.2	BER vs. C/N (in 1.5 MHz) in a Rayleigh channel at 226 MHz

Measurements of BER vs. C/N were made on a data channel (D = 64 kbit/s, R = 0.5), using a fading channel simulator.

The results are shown in Fig. 8. For the example of a Rayleigh channel with a rural profile and the receiver travelling at 130 km/h, the measured results (curve B) may be compared with those of a software simulation (curve A). The difference is less than 3 dB at a BER of 10-4. Curve C illustrates typical urban performance at relatively low speed, but in a highly frequency dispersive channel. Curve D illustrates the performance in a representative single frequency network in bad conditions, where signals are received with delays up to 600 µs (corresponding to 180 km excess path length).

## 10.3	BER vs. C/N (in 1.5 MHz) in a Rayleigh channel at 1 500 MHz

Measurements of BER vs. C/N were made on a data channel using a fading channel simulator. The results are shown in Fig. 9.

## 10.4	Audio service availability

Provisional assessments of sound quality indicate that it is not perceptibly impaired if the BER is less than 10-4.

FIGURE 8/BO.789-1...[D08] = 17 CM

FIGURE 9/BO.789-1...[D09] = 17 CM

ANNEX  2

Digital System B

# 1.	Introduction

The Digital Sound Broadcasting System B is a flexible, bandwidth and power-efficient system for providing digital audio and ancillary digital data broadcasting for reception by indoor/outdoor, fixed and portable, and mobile receivers. System B is designed for satellite as well as terrestrial transmission.

## 1.1	System status

A complete implementation of System B, including the appropriate satellite and terrestrial propagation channel models, exists as a COMDISCO SPW simulation on a Sun work station. An engineering model of the system is currently being built. It will be tested in late 1993 and early 1994 under the auspices of the Electronic Industries Association (EIA), both in the laboratory and via satellite (NASA’s Tracking and Data Relay Satellite (TDRS)) under a variety of reception conditions.

## 1.2	System overview

System B is structured into two functional elements, a digital data transport core function (transmitter/receiver/
modem), and a data manipulation (audio compression/data multiplexing) function. This functional division is illustrated by the block diagram of the transmitter in Fig. 10.

FIGURE 10/BO.789-1...[D10] = 8 CM

Each System B link is designed to operate over a range of data rates from 32 kbit/s to 384 kbit/s. This enables a service provider independent access to a satellite transponder and allows each provider to supply a mix of one or more audio and data channels. It also allows each provider to use the power and bandwidth resources of the transmitter in proportion to each provider’s aggregate data rate.

A specific audio compression scheme for System B is not currently specified. Any scheme which falls within the systems’ data rate capabilities can be accommodated. Several audio compression schemes will be tested during the system testing process and recommendations for each type of service will be made on the basis of audio quality, and required data rate.

The functional division of System B at the receive side is shown in Fig. 11. The receiver is designed to operate over the range of link data rates allowed by the transmission channel. This structure enables access to a given provider’s services through demultiplexing of a single data stream, and access to other providers by tuning to another frequency channel.

FIGURE 11/BO.789-1...[D11] = 3 CM

The signal structure of System B allows a simple basic digital receiver implementation which will work well in a majority of reception environments. Additional signal processing functions which can be added to the basic receiver without impact to the signal structure have been developed to enhance operation in more difficult reception environments.

# 2.	Signal structure

The signal structure of the core system consists of convolutional encoding, followed by time interleaving, and QPSK modulation with pulse shaping for bandwidth efficiency. An outer code such as a Reed Solomon code can be added to the data stream of any of the data sources.

# 3.	Signal generation

The steps in the signal construction processes are shown in the block diagram of Fig. 12.

FIGURE 12/BO.789-1...[D12] = 4 CM

## 3.1	Synchronization

In order to resolve the QPSK ambiguity at the receiver and establish interleaver frame synchronization, a unique sync word is overwritten over part of the symbol stream prior to modulation.

# 4.	Receiver structure

A functional block diagram of the receiver is shown in Fig. 13. Each of its functional blocks is described below.

FIGURE 13/BO.789-1...[D13] = 18 CM

## 4.1	Core receiver

The core receiver consists of the RF to IF converter and demodulator/detector blocks in Fig. 13. The first block performs the functions of tuning the receiver to the proper RF frequency and converting the received signal down to a low IF frequency. The second block will in most cases be a completely digital implementation. Here the low IF output of the first block is sampled and processed to demodulate, detect, and decode the combined data stream.

### 4.1.1	Carrier and symbol synchronization loops

Carrier demodulation and data detection takes place in cross coupled QPSK carrier and bit synchronization tracking loops. The carrier loop would in most cases be implemented as a Costas loop. Another option to implement carrier demodulation is using a block phase estimator, which gives somewhat worse performance than a coherent demodulator, but may be useful in a rapid fading environment.

The symbol synchronization loop is a transition tracking loop which is used to establish symbol timing for all subsequent operations.

### 4.1.2	Symbol de-interleaving and ambiguity resolution

Detected symbols, in soft decision format, are fed into the time de-interleaver where their original time sequence is re-established. The symbols at the input to the de-interleaver are also correlated against a replica of the synchronization word which was overlaid on the symbol stream. Detection of the synchronization word or its inverse allows the establishment of de-interleaver frame synchronization as well as the resolution of the QPSK ambiguity.

### 4.1.3	Viterbi decoding

Viterbi decoding of the convolutional code takes place at either the 1/2 or 1/3 rate as chosen by the service provider. Integrated circuits which run at both rates are available so providing both rates is not a significant cost impact to the receiver. The choice of a rate 1/3 code does result in a bandwidth penalty which the service provider may want to absorb, especially if the service is intended for mobile reception in the presence of heavy shadowing.

## 4.2	Receiver enhancements

Several signal processing enhancements have been developed to improve receiver operation in difficult reception environments.

### 4.2.1	Channel state aided decoding

A Viterbi decoder will provide better average performance under fading signal conditions if its input can be assigned a zero weight during the time the signal is below threshold. This can be accomplished by doing a signal-to-noise estimation to detect when a fade occurs.

### 4.2.2	Antenna diversity

Indoor propagation measurements have shown that large standing waves can occur inside buildings. The peaks and troughs of these waves are fractions of a wavelength apart, so that close spaced antenna diversity is practical. There may be applications for antenna diversity for outdoor, especially mobile reception, conditions as well. The simplest approach is to switch to the other antenna when a signal drop is detected. Depending on conditions, there are probably more complex strategies that can be developed.

### 4.2.3	Equalization

Equalization is most useful when a signal is distorted by echoes from reflections. In most satellite applications, with reasonable satellite elevation angles, reflected signals are many dB below the direct signal. The problem of echoes is much more severe in terrestrial broadcasting, where several reflected signals can be close in power to the direct signal. The function of the equalizer is to sort out the competing signals, select the strongest one, and eliminate the effects of the others.

## 4.3	Application function implementation

This portion of the receiving system consists of the data demultiplexing, optional outer code decoding, and audio decoding functions. In some applications the use of an outer code is not necessary, or as is the case with some audio compression systems, additional error protection is provided within the encoder/decoder circuitry.

________________

* 	Reports ITU-R BO.955 and ITU-R BS.1203 were used as the basis for this Recommendation.