RECOMMENDATION  ITU-R  BR.1291

SCANNED  AREA  OF  SUPER  16  mm  FILM  FOR  PRODUCTION  
AND  POST-PRODUCTION  IN  16:9  TELEVISION  SYSTEMS

(Question ITU-R 240/11)

(1997)

Rec. ITU-R BR.1291

The ITU Radiocommunication Assembly,

considering

a)	that there is a need for a unique scanning standard for Super 16 mm film to tell the cameraman in the viewfinder which part of the captured image will be used for broadcasting;

b)	that for production and post-production the largest possible area of the camera image should be scanned;

c)	that there should be a small safety margin of about 2% between the exposed and scanned image areas;

d)	that there exists an International Organization for Standardization (ISO) Standard DIS 5768-1996 which is in agreement with the majority of film cameras in use;

e)	that the major telecine manufacturers are willing in the future to design their equipment according to ISO Standard DIS 5768;

f)	that there is a need for backwards compatibility with existing telecines which do not yet coincide with ISO Standard DIS 5768;

g)	that there is a test film available for the alignment of telecine scan settings to ISO Standard DIS 5768,

recommends

1	broadcasters to adopt the scanned area on Super 16 mm film as given in Fig. 1 for programme production and post-production (the dimensions of the area scanned for broadcasting are given in Recommendation ITU-R BR.1289).

FIGURE 1291-01