RECOMMENDATION  ITU-R  BR.1375-2*, **

High-definition television (HDTV) recording

(Question ITU-R 108/11)

(1998-2001-2002)

The ITU Radiocommunication Assembly,

considering

a)	that the common image format, having 1 080  1 920 square pixels, is now recommended as the image capture format for HDTV;

b)	that the digital broadcasting service for HDTV programmes is gaining in momentum. There are several services in operation or being planned in many countries;

c)	that various analogue and digital format video tape recorders (VTRs) for HDTV tape recording have been developed;

d)	that many countries hold large archives of valuable, irreplaceable HDTV programmes, based on the signal formats defined in Recommendation ITU-R BT.709, Part 1;

e)	that analogue HDTV open-reel recording formats are considered to be obsolete for use in programme production today, but the programmes recorded on those formats in the past are often valuable;

f)	that digital recording of HDTV programmes is now widely used in HDTV production and post-production;

g)	that there are several types of equipment available for digital HDTV recording today, that differ in the recording medium (e.g. open-reel tape or cassette), in recording format (e.g. use or not of bit-rate compression) and in recorder type (e.g. deck or portable camcorder);

h)	that bit-rate compression techniques based on DCT and VLC coding can provide highly efficient methods of recording HDTV programmes, whose source data rate exceeds 1 Gbit/s;

j)	that the operational and handling characteristics of digital HDTV VTRs have improved to the point that they can be used not only in the studios but also in the field, with a level of convenience similar to that of conventional VTRs;

k)	that HDTV cameras using two-million pixel CCD imaging devices and digital signal processing with advanced VLSI chip(s) can provide high quality HDTV pictures while offering the operating features and small size of conventional cameras;

l)	that the use of 24 frame1 formats will be used to supplement and augment film production,

recommends

1	that digital HDTV recording should be preferred for HDTV programme origination and production, and for storage for programme exchange;

2	that analogue HDTV tape recordings should preferably be transferred to a digital recording format for post-processing and archiving;

3	that the 1 080  1 920 common image format specified in Recommendation ITU-R BT.709, Part 2 shall be preferred for HDTV recording, while the other image formats specified in Recommendation ITU-R BT.709, Part 1 will retain their value especially in view of the need to ensure the possibility of reusing archived HDTV programme assets;

4	that HDTV programmes intended for extensive and complex post-processing should preferably be recorded on a digital format that does not use lossy compression, while HDTV programmes intended for limited or no post-processing should preferably be recorded on a digital format, which may use a modest amount of compression;

5	that for the purpose of international programme exchange2, operators should take into account that, at the present time and depending on the application, the two HDTV recording formats generally known as HDCAM and HD-D5 are the ones in most widespread use.3

NOTE 1 – The Tables in Appendix 1 recall specifications of commercially available recording devices for programme production in the HDTV formats specified in the latest version of Recommendation ITU-R BT.709.

Appendix 1

Among the various recording formats listed in the Tables of this Appendix, there are two predominant formats available for international exchange for HDTV recordings. These are those generally known as HDCAM and HD-D5.

Both formats have their own design objectives, as each data rate shows. A lower data rate gained from a higher compression ratio provides a longer recording time and is suitable for handy camcorder operations. A higher data rate provides a better picture quality suitable for subsequent video processing and is suitable for studio operation.

Some key parameters for these two formats are shown for information in Table 1. All formats are described in the other Tables of this Appendix.

TABLE  1

Major specifications of the recommended HDTV recording formats

| Manufacture specification |  |  | HDCAM |  |  |  |  | HD-D5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Video specification | Quantization |  | 8 |  |  |  |  | 10 or 8 |  |  |  |
|  | Compression |  | 1/7 |  |  |  |  | 1/5 for 10 bits 1/4 for 8 bits |  |  |  |
|  | Sampling structure |  | 3:1:1 |  |  |  |  | 4:2:2 |  |  |  |
|  | DCT (luminance) |  | 1/3.6 |  |  |  |  | 1/5 for 10 bits 1/4 for 8 bits |  |  |  |
|  | DCT (chrominance) |  | 1/7.2 |  |  |  |  | 1/5 for 10 bits 1/4 for 8 bits |  |  |  |
|  | Data rate (Mbit/s) |  | 140 |  |  |  |  | 235 |  |  |  |
| Audio specification | Bit/sample |  | 20 |  |  |  |  | 20 or 24 |  |  |  |
|  | Number of AES3(1) channels |  | 2 |  |  |  |  | 2 or 4 |  |  |  |
|  | Multi-channel audio and/or AES3/EBU non-audio data |  | Yes |  |  |  |  |  |  |  |  |
| Recording/ playback length | Size of cassette (mm) | L | 254 × 145 × 25 |  |  |  | 296 × 167 × 25 |  |  |  |  |
|  |  | M | None |  |  |  | 212 × 124 × 25 |  |  |  |  |
|  |  | S | 156 × 96 × 25 |  |  |  | 161 × 98 × 25 |  |  |  |  |
|  | System (see Recommendation ITU-R BT.709, Part 2) |  | 25/P | 30/P* | 50/i | 60/i* | 25/P |  | 30/P* | 50/i | 60/i* |
|  | Time (min) | L | 148 | 124 | 148 | 124 | 148 |  | 124 | 148 | 124 |
|  |  | M | -- | -- | -- | -- | 75 |  | 63 | 75 | 63 |
|  |  | S | 48 | 40 | 48 | 40 | 27 |  | 23 | 27 | 23 |
| (1) An AES3 channel may carry two linear PCM audio channels or it may carry data as indicated by status channel bit 1. NOTE 1 – The mark “*” shows inclusion of the frame or field frequency having those values divided by 1.001. S: small, M: medium, L: large. |  |  |  |  |  |  |  |  |  |  |  |

The following Tables describe the major features and specifications for the available HDTV storage devices:

Table 2:  Tape recorder for 1125/60 (59.94) – Overview

Table 3:  Disk recorder for 1125/60 (59.94) – Overview

Table 4:  HDTV digital VTR for 1125/60 (59.94) – Details

Table 5:  Tape recorder for 1125/50 – Overview

Table 6:  HDTV digital VTR for 1125/50 – Details

Table 7:  HDTV analogue VTR for 1125/60 – Details

TABLE  2

Tape recorder for 1125/60 (59.94) – Overview

| Digital/analogue |  |  | Digital |  |  |  |  |  | Analogue |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Compressed/non-compressed |  |  | Compressed |  |  |  | Non-compressed |  |  |
| Package type |  |  | Cassette |  |  |  | Cassette | Open reel | Cassette |
| Manufacture specification |  |  | HDCAM |  | HD-D5 | DVCPRO HD | D6 | HDD/HDDP | HDV, AU-HD |
| Recording/playback time (maximum) |  |  | 40/124 min |  | 23/63/124 min | 46 min | 8/28/64 min | 96 min | 63 min |
|  |  | Luminance (MHz) | 23 |  | 30 | 20 | 30 |  | 20 |
| Video bandwidth |  | Chrominance (MHz) | 7 |  | 15 | 10 | 15 |  | 7 |
|  |  | Number of lines | 1080 |  |  |  | 1080 | 1035 | 1036 |
|  |  | Digital or analogue | PCM |  |  |  | PCM |  | PCM |
| Audio specification |  | Number of AES3 channels | 2 | 2 or 4 |  | 4 | 5 | 4 (Digital)  1 (Analogue) | 2 |
|  |  | Sampling frequency | 48 kHz, 20 bits | 48 kHz, 20 or 24 bits |  | 48 kHz, 16 bits | 48 kHz, 20/24 bits | 48 kHz, 16 bits | 48 kHz, 16 bits |
| Ancillary data |  |  | – | 5.8 kbytes/Frame |  | 7.4 kbytes/Frame | 38.4 kbytes/Frame | 38.4 kbytes/Frame (5 VBI lines) |  |
| Media | Tape width (mm) |  | 12.65 |  |  | 6.35 | 19.01 | 25.4 | 12.65 |
|  | Size of cassette (mm) |  | S: 156  96 L: 254  145 | S: 161  98 M: 212  124 L: 296  167 |  | L: 125  78 | S: 172  109 M: 254  150 L: 366  206 | 14 in. reel | 205  121.5 |
|  | Substance |  | Metal particle |  |  |  | Metal particle |  | Metal particle |
| Application example |  |  | Camcorder | Portable |  | Camcorder | Studio deck |  | Portable studio deck |
| PCM: pulse code modulation VBI: vertical blanking interval. |  |  |  |  |  |  |  |  |  |

TABLE  3

Disk recorder for 1125/60 (59.94) – Overview

| Digital/analogue |  | Digital | Analogue |  |
| --- | --- | --- | --- | --- |
| Compressed/non-compressed |  | Compressed | Compressed |  |
| Handling |  | Removable | Removable |  |
| Recording and/or playback |  | Recording and playback | Playback only | Write-once |
| Media |  | MO | Optical | Optical wobbling |
| Manufacture specification |  |  | HDL-2000 | HDL-5800 |
| Recording/playback time |  | 32 min | 15 min (CLV) | 20 min (CLV) |
| Video bandwidth | Luminance | 44.55 MHz, 8 bits | 20 MHz |  |
|  | Chrominance | 14.85 MHz, 8 bits | 6 MHz |  |
| Audio specification | Number of lines | 1088 |  |  |
|  | Digital or analogue | PCM | PCM |  |
|  | Number of AES3 channels | 2 | 1 |  |
|  | Sampling frequency | 48 kHz, 20 bits | 48 kHz, 16 bits |  |
| Ancillary data |  | 256 kbytes/frame |  |  |
| Media | Size of caddy (mm) | 300 | 300 |  |
|  | Substance | Magnetic coat | Aluminium coated | Alloy coat |
| Notes |  | Intra frame 94 Mbit/s drive, double sided | Single sided |  |
| CLV: constant linear velocity MO: magneto optical. |  |  |  |  |

TABLE  4

HDTV digital VTR for 1125/60 (59.94) – Details

| Manufacture specification |  |  | HDD/HDDP | D6 | HDCAM | HD-D5 | DVCPRO HD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling frequency |  | Video (MHz) | 74.25 (74.25/1.001) |  |  |  |  |
|  |  | Audio (kHz) | 48 |  |  |  |  |
| Quantization |  | Video (bits) | 8 |  |  | 10/8 | 8 |
|  |  | Audio (bits) | 20 | 20/24 | 20 | 20/24 | 16 |
| Number of AES3 channels |  |  | 4 (Digital)  1 (Analogue) | 5 | 2 | 2 or 4 | 4 |
| Video specification | Compression |  | N.A. | N.A. | Intra field/ frame 1/7 | Intra field 1/5 (10 bits); 1/4 (8 bits) | Intra frame 1/10 |
|  | Sampling structure |  | 4:2:2 | 4:2:2 | 3:1:1 | 4:2:2 | 2.7:1.3:1.3 |
|  | DCT (luminance) |  | N.A. | N.A. | 1/3.6 | 1/5;1/4 | 1/6.6 |
|  | DCT (chrominance) |  | N.A. | N.A. | 1/7.2 | 1/5;1/4 | 1/6.6 |
| Channel coding |  |  | 8-8 map | 8-12 map | S-NRZI | 8-14 map | 24-25 I-NRZI |
| Total rate (Mbit/s) |  |  | 1 188 | 1 212 | 185 | 301 | 167 |
| Video rate (Mbit/s) |  |  | 958.5 | 995.3 | 140 | 235 | 100 |
| Number of recording RF channels |  |  | 8 |  | 4 (camcorder)/ 2 (studio) | 4 | 4 (camcorder)/ 2 (studio) |
| ECC |  | Inner | 110, 104 | 237, 221 | 231, 219 | 95, 87 | 85, 77 |
|  |  | Outer | 64, 60 | 254, 240 | 250, 226 | 128, 120 | 149, 138 |
| Drum diameter (mm) |  |  | 134.6 | 96.5 | 81.4 | 76.0 | 21.7 |
| Drum rotation (rps) |  |  | 120 | 150 | 45 (camcorder)/ 90 (studio) | 90 | 150 (camcorder)/ 300 (studio) |
| Number of tracks (/field) |  |  | 16 | 40 | 6 | 12 | 40/frame |
| Tape speed (mm/s) |  |  | 805.2 | 497 | 96.8 | 167.228 | 135.28 |
| Track pitch (m) |  |  | 37 | 22 | 21.7 | 20.0 | 18 |
| Minimum wavelength (m) |  |  | 0.69 | 0.81 | 0.49 | 0.63 | 0.49 |
| Tape width (mm) |  |  | 25.4 | 19.01 | 12.65 |  | 6.35 |
| Media substance |  |  | Metal particle |  |  |  |  |
| Tape Hc (kA/m) |  |  | 115 | 127 | 135 | 143 | 184 |
| Cassette size (mm) |  |  | 11.75/14 in. reel | S: 172  109 M: 254  150 L: 366  206 | S: 156  96 L: 254  145 | S: 161  98 M: 212  124 L: 296  167 | L: 125  78 |
| Recording time (min) |  |  | 63/94 | 8/28/64 | 40/124 | 32/63/124 | 46 |
| ECC: error correction code N.A.: not applicable. |  |  |  |  |  |  |  |

TABLE  5

Tape recorder for 1125/50 – Overview

| Digital/analogue |  | Digital |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Compressed/non-compressed |  | Compressed |  |  |  |  | Non-compressed |  |
| Package type |  | Cassette |  |  |  |  |  |  |
| Manufacture specification |  | HDCAM | HD-D5 |  | DVCPRO HD | D6(1) |  |  |
| Recording/playback time (maximum) |  | 48/148 min | 27/75/148 min |  | 46 min | 8/28/64 min |  |  |
| Video bandwidth | Luminance (MHz) | 23 | 30 |  | 23 | 30 |  |  |
|  | Chrominance (MHz) | 7 | 15 |  | 11 | 15 |  |  |
|  | Number of lines | 1080 |  |  |  | 1080 |  |  |
| Audio specification | Digital or analogue | PCM |  |  |  |  |  |  |
|  | Number of AES3 channels | 2 | 4 |  |  |  |  | 6 |
|  | Audio sampling | 48 kHz, 24 bits |  |  | 48 kHz, 16 bits |  |  | 48 kHz, 20/24 bits |
| Ancillary data |  | – |  | 5.8 kbytes/Frame | 8.9 kbytes/Frame |  |  | 23 kbytes/Frame |
| Media | Tape width (mm) | 12.65 |  | 12.65 | 6.35 |  |  | 19.01 |
|  | Size of cassette (mm) | S: 156  96 L: 254  145 |  | S: 161  98 M: 212  124 L: 296  167 | L: 125  78 |  |  | S: 172  109 M: 254  150 L: 366  206 |
|  | Substance | Metal particle |  |  |  |  |  |  |
| (1) The D6 Society for Motion Picture and Television Engineers (SMPTE) Document may not reflect the latest numbers in this Table. |  |  |  |  |  |  |  |  |

TABLE  6

HDTV digital VTR for 1125/50 – Details

| Manufacture specification |  |  | D6(1) | HDCAM | HD-D5 | DVCPRO HD |
| --- | --- | --- | --- | --- | --- | --- |
| Sampling frequency |  | Video (MHz) | 74.25 |  |  |  |
|  |  | Audio (kHz) | 48 |  |  |  |
| Quantization |  | Video (bits) | Y: 10, Cr/Cb: 8 | 8 | 8/10 | 8 |
|  |  | Audio (bits) | 20/24 | 20 | 24 | 16 |
| Number of AES3 channels |  |  | 6 | 2 | 4 |  |
| Video specification | Compression |  | N.A. | Intra field/frame 1/7 | Intra field 1/5 (10 bits); 1/4 (8 bits) | Intra frame 1/8.9 |
|  | Sampling structure |  | 4:2:2 | 3:1:1 | 4:2:2 | 3:1.5:1.5 |
|  | DCT (luminance) |  | N.A. | 1/3.6 | 1/5;1/4 | 1/6.6 |
|  | DCT (chrominance) |  | N.A. | 1/7.2 | 1/5;1/4 | 1/6.6 |
| Channel coding |  |  | 8-12 map | S-NRZI | 8-14 map | 24-25 I-NRZI |
| Total rate (Mbit/s) |  |  | 1 212 | 154 | 269 | 167 |
| Video rate (Mbit/s) |  |  | 933.1 | 117 | 196 | 100 |
| Number of recording RF channels |  |  | 8 | 4/2 | 4 | 4 (camcorder)/ 2 (studio) |
| ECC |  | Inner | 237, 221 | 231, 219 | 95, 87 | 85, 87 |
|  |  | Outer | 254, 240 | 250, 226 | 128, 120 | 149, 138 |
| Drum diameter (mm) |  |  | 96.5 | 81.4 | 76.0 | 21.7 |
| Drum rotation (rps) |  |  | 150 | 37.5/75 | 75 | 150 (camcorder)/ 300 (studio) |
| Number of tracks (/field) |  |  | 48 | 6 | 12 | 48/frame |
| Tape speed (mm/s) |  |  | 497 | 80.7 | 139.496 | 135.415 |
| Track pitch (m) |  |  | 22 | 21.7 | 20 | 18 |
| Minimum wavelength (m) |  |  | 0.81 | 0.49 | 0.59 | 0.49 |
| Tape width (mm) |  |  | 19.01 | 12.65 |  | 6.35 |
| Media substance |  |  | Metal particle |  |  |  |
| Tape Hc (kA/m) |  |  | 127 | 132 | 144 | 184 |
| Cassette size (mm) |  |  | S: 172  109 M: 254  150 L: 366  206 | S: 156  96 L: 254  145 | S: 161  98 M: 212  124 L: 296  167 | 125  78 |
| Recording time (min) |  |  | 8/28/64 | 48/148 | 27/75/148 | 46 |
| (1) The D6 SMPTE Document may not reflect the latest numbers in this Table. |  |  |  |  |  |  |

TABLE  7

HDTV analogue VTR for 1125/60 – Details

| Manufacture specification |  | 1 inch | UNIHI(1) |
| --- | --- | --- | --- |
| Video | Luminance (MHz) | 20 |  |
|  | Chrominance (MHz) | 10 | 7 |
| Audio sampling (kHz) |  | – | 48 |
| Number of audio channels |  | 2 analogue channels | 2 (AES3 channel) |
| Number of recording RF channels |  | 4 | 2 |
| Drum diameter (mm) |  | 134.6 | 76 |
| Drum rotation (rsp) |  | 60 | 90 |
| Number of tracks (/field) |  | 4 | 6 |
| Tape speed (mm/s) |  | 483 | 119.709 |
| Track pitch (m) |  | 89 | 24.8 |
| Tape width (mm) |  | 25.4 | 12.650 |
| Media substance |  | Metal particle |  |
| Tape Hc (kA/m) |  | 55.7 | 123.3 |
| Cassette size (mm) |  | – | 205  121.5 |
| Recording time (min) |  | 63 |  |
| (1) The UNIHI format is still widely used for the international exchange of HDTV programmes for viewing purpose. |  |  |  |

*	 	This Recommendation should be brought to the attention of the International Electrotechnical Commission (IEC).

*	*	Note by the Secretariat: This Recommendation was amended in April 2006 to correct Table 6 (ECC).

1	 	24 frame applications in HDTV recording will be considered in a separate new Recommendation.

2	 	International programme exchange is defined as the transmission of television or sound programme material (or components thereof) among professional parties in different countries. It should be based on internationally agreed and widely employed technical standards or operating practices, except by prior bilateral agreement among the parties involved.

3	 	This provision is not intended to exclude the possibility that other formats may be used in house, or for programme exchange subject to mutual agreement among the parties concerned.