RECOMMENDATION  ITU-R  BR.602-4

EXCHANGE  OF  TELEVISION  RECORDINGS  FOR
PROGRAMME  EVALUATION

(Question ITU-R 103/11)

(1982-1990-1992-1994-2000)

Rec. ITU-R BR.602-4

The ITU Radiocommunication Assembly,

considering

a)	that a significant number of television recordings are exchanged by broadcasting organizations for purposes of programme evaluation;

b)	that several formats, such as the VHS format (specified in the International Electrotechnical Commission (IEC) Publication 774) provide good interchangeability between video cassettes recorded on machines supplied by several manufacturers as well as attractive price, cassette size and playing time;

c)	that the chosen format should provide programme quality consistent in time and adequate for viewing purposes and programme evaluation;

d)	that some of those formats were originally designed for the consumer market; consequently:

–	the recorders and the cassettes are comparatively inexpensive;

–	the recorders can be easily operated;

–	they are comparatively reliable and robust;

–	they are quite widespread and easily available;

e)	that the use of tape cassettes offers advantages in handling and shipping,

recommends

1	that video cassettes conforming with the VHS format should continue to be preferred for use in the international exchange of recorded programmes for programme evaluation, in both the 625-lines, 50 field/s and the 525-lines, 60 field/s standards.

NOTE 1 – Several other formats may be used for the same application subject to prior agreement. These are, for instance, the S-VHS format (IEC/SC 60B (Secretariat) 199), the Video 8 format (IEC Publication 843), and the Hi8 format (IEC/SC 60B (Secretariat) 198). Annex 1 describes, for information, some different practices currently applied on a regional or local basis;

2	that the following operating practices applicable to VHS recordings should be used:

–	only the normal speed recording mode (standard play) should be used;

–	the sound should be recorded on the two longitudinal tracks;

–	for stereo sound, the left-channel signal should be recorded on track 1 (inner track) and the right-channel signal on track 2 (edge track).

ANNEX  1

Some regional or local practices used in the exchange of recordings 
for television programme evaluation

a)	Members of the Asia Pacific Broadcasting Union (ABU) regularly use VHS recordings for their viewing purposes, however some Members of the ABU also use Betamax recordings, S-VHS recordings or Hi8 recordings.

b)	Members of the North American Broadcasters Association (NABA) use U-matic recordings or Betacam recordings for their viewing purposes.

c)	Members of the Organización de Televisión Ibero-Americana (OTI) regularly use both VHS recordings and S-VHS recordings for their viewing purposes. They also exceptionally use U-matic recordings or Betacam recordings.

d)	Members of the Union des Radiodiffusions et Télévisions Nationales d'Afrique (URTNA) regularly use both VHS recordings and low-band U-matic recordings for their viewing purposes.