RECOMMENDATION  779

OPERATING  PRACTICES  FOR  4:2:2  DIGITAL  TELEVISION  RECORDING

(Question 103/11)

(1992)

Rec. 779

The CCIR

recommends

that for international exchange of television programmes conforming to the 4:2:2 digital standard (Recommendation 601), the operating practices should be as follows:

# 1.	Presentation of recordings

Recordings of a single programme of a duration up to the maximum cassette play time should be contained in one cassette.

Separate programmes should always be on separate cassettes.

# 2.	Programme identification

The content of a recorded digital television cassette should be identified by at least the following information, to be provided on a label attached to the cassette itself, and on another label attached to the cassette container:

a)	name of the organization which made the recording;

b)	title of the programme, or title, sub-title and episode number;

c)	library number (reference number) of the programme or of the cassette;

d)	total number of cassettes, and number of the cassette in the sequence, if the programme is on more than one cassette;

e)	total playing time, and playing time of the programme material recorded on each cassette;

f)	longitudinal time code address for the start of the programme;

g)	television scanning standard (625/50 or 525/60);

h)	monophonic or stereophonic programme sound, and allocation of the digital audio channels to additional programme sound components, if any.

It would be beneficial, in view of the implementation of fully automated television stations, if at least information items b), c) and d), were also provided in the form of a bar code to be printed on an appropriate label attached on each recorded cassette. A suitable form of bar code is under study.

The information required above should be provided in at least one of the official languages of the ITU.

# 3.	Leaders

Programme material recorded on digital television cassettes should be preceded and followed by appropriate leaders as shown below:

µ

|  | Duration | Picture and sound content |
| --- | --- | --- |
| Thread up leader Identification leader Cue-up leader Run-out trailer | 35 s 15 s 38 s 32 s 30 s | Blank tape Aural and/or visual identification Aural and/or visual count-down, 10 to 2 Black and silence PROGRAMME MATERIAL Black and silence (minimum) |

§

Information shown on the identification leader should match the one shown on the labels (§ 2).

The cue-up leader, the programme material and the run-out trailer should appear on the tape as an uninterrupted recording.

# 4.	Cue audio track

In the case of a complete programme, the longitudinal (editing) audio should preferably be a replica of the programme sound, complete with its identification and countdown leader; it may however be interspersed with additional cues to identify segments of the programme, as needed.

# 5.	Time and control code

The time address information to be used as reference for the exchange of recordings should be the one carried on the longitudinal time and control track. In the case of the exchange of a finished edited programme, such time address information should be continuous and monotonically increasing, furthermore, the same time addresses should preferably (but not mandatorily) appear also on the time and control code multiplexed with the video information, and on the time and control code multiplexed on the digital audio channel that carries the finished programme sound. Programme data carried in the user bits of the longitudinal time and control signals should match information shown on the programme identification label.

# 6.	Allocation of audio channels

The four audio channels offered by the digital television tape recording format should be allocated as shown in Annex 1.

PAGE BLANCHE POUR ANNEX 1  (T02)

# Case 1 – Single monophonic programme content

This is the case where a single monophonic sound accompanies the video content of the programme. In such a case, the monophonic programme content represents the complete programme sound mix but, for the purpose of international exchange, it can be accompanied by the so-called “international sound” - the complete monophonic mix of music, effects, etc. lacking only the speech which can be added in the dubbing process in order to obtain a complete monophonic programme sound mix in a language different from the original one.

# Case 2 – Single stereophonic programme content

This case is similar to case 1, i.e. a single complete stereophonic programme sound accompanies the video content of the programme. In this case, as in case 1, the stereophonic programme sound represents the complete programme sound mix and it can be accompanied by a complete stereophonic mix of music and all effects – a stereophonic international sound – which may be used by the receiving organization for dubbing.

# Case 3 – Two complete stereophonic programme contents

In this case, the video content of the recorded television programme is accompanied by two somewhat different complete stereophonic programme mix sounds. The difference may be the language or any other component.

Note 1 – In the case where two different stereophonic programme sounds are recorded with the same video programme content, the sending organization should provide written information on the nature of each of the two sounds.

# Case 4 – Monophonic programme with separate commentary

When original news or documentary recordings with monophonic sound are exchanged, it is always expected that it is possible to dub them in a different language; they should therefore contain a complete international sound, i.e. the sound recorded on the spot with all ambiance, original speech, etc. which can be mixed later, by the receiving organization, with a new commentary in its own language.

# Case 5 and Case 6 – Stereophonic programme with separate commentary

There is a similarity between these cases and case 4 but, since these involve stereophonic sound, all channels are used and the sound dubbing has to be done on a copy of the original recording where the complete stereophonic mix can be re-recorded on channels 1 and 2, or the stereophonic commentary in a different language and the stereophonic international sound can be recorded on channels 1, 2, 3 and 4 respectively.

# Case 7 – Single monophonic programme content non-mixed

In this case, the video content of the programme is accompanied by a non-mixed monophonic programme sound, i.e. the speech or commentary, the music and the effects are not mixed together.

Such a configuration permits mixing at a later stage during the re-recording or dubbing of that particular tape. In general, such a case may appear when unfinished programmes, or programming segments, are exchanged (for example: one broadcasting organization may collect inserts from different sources in order to assemble a combined programme).

# Case 8 – Stereophonic programme and second audio programme (SAP)

This case, which is mainly of interest in the United States of America, has been included in the table for completeness. A complete stereophonic programme is accompanied by a monophonic second audio programme. Under the circumstances of use of a second audio programme, the transmitted quality is of a lower quality than normal programme sound.

# Longitudinal cue audio channel

In all the cases described above, the cue audio channel should preferably contain a complete monophonic programme mix or, if this is not practicable, the content of audio channel 1.

_________________