RECOMMENDATION  784

EXCHANGE  OF  TELEVISION  PROGRAMMES  ON  16 mm  FILM
WITH  TWO  SYNCHRONOUS  SOUND  TRACKS  ON  A  SEPARATE  SUPPORT

(Question 41/11)

(1992)

Rec. 784

The CCIR,

recommends

that when television programmes are exchanged having two high-quality sound tracks, when the vision support is 16 mm film, the following specifications should apply:

# 1.	Sound support

When the vision component of the programme is on 16 mm film, the two sound components should be on a separate 16 mm magnetic film.

# 2.	Sound recording specifications

The dimensions and position of the tracks and the recording characteristics for the magnetic film should conform to Fig. 2 of CCIR Recommendation 265.

# 3.	Picture film specifications

The picture film should conform to CCIR Recommendation 265.

# 4.	Allocation of tracks

The allocation of tracks on the separate 16 mm magnetic film should be as follows:

–	for stereo sound:

centre track: left channel,

edge track: right channel;

–	for independent synchronous sounds:

centre track: dubbing or subtitling sound;

edge track: original transmission sound.

Note 1 – An example of a programme leader is given in EBU Document Tech. 3203.

Note 2 – An example of a programme label is given in EBU Document Tech. 3211.