Recommendation ITU-R BS.2076-0 (06/2015)

Audio Definition Model

BS Series Broadcasting service (sound)

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Annex 1 of Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from http://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Recommendations (Also available online at http://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radiowave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2015

ITU 2015

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION  ITU-R  BS.2076-0*

Audio Definition Model

(2015)

Scope

This Recommendation describes the structure of a metadata model that allows the format and content of audio files to be reliably described. This model, called the Audio Definition Model (ADM), specifies how XML metadata can be generated to provide the definitions of tracks in an audio file.

Keywords

ADM, Audio Definition Model, BWF, Metadata, Wave-file, WAVE, object-based, channel-based, scene-based, renderer, XML, XSD, format, immersive

The ITU Radiocommunication Assembly,

considering

a)	that Recommendation ITU-R BS.2051 – Advanced sound system for programme production, highlights the need for a file format that is capable of dealing with the requirements for future audio systems;

b)	that Recommendation ITU-R BS.1909 – Performance requirements for an advanced multichannel stereophonic sound system for use with or without accompanying picture, outlines the requirements for an advanced multichannel stereophonic sound system;

c)	that it is desirable that there is a single open standard for a metadata model for defining audio content that file and streaming formats could either adopt or become compatible with by means of suitable interfacing,

recommends

for the following use cases:

•	applications requiring a generic metadata model for, and a formalized description of, custom/proprietary audio formats and content (including codecs);

•	generating and parsing audio metadata with general-purpose tools, such as text editors;

•	an organization's internal production developments, where multi-purpose metadata needs to be added;

•	a human-readable and hand-editable file for describing audio configurations (such as describing a mixing studio channel configuration) in a consistent and translatable format is needed,

to use the Audio Definition Model (ADM) described in Annex 1 for metadata to describe audio formats used in programme production and international exchange.

Annex 1

Audio Definition Model

# 1	Introduction

Audio for broadcasting and cinema is evolving towards an immersive and interactive experience which requires the use of more flexible audio formats. A fixed channel-based approach is not sufficient to encompass these developments and so combinations of channel, object and scene-based formats are being developed. Reports ITU-R BS.2266 [1] and Recommendations ITU-R BS.1909 [2] and ITU-R BS.2051 [3] highlight these developments and the need for the production chain to accommodate them.

The central requirement for allowing all the different types of audio to be distributed, whether by file or by streaming, is that whatever file/stream format is used, metadata should co-exist to fully describe the audio. Each individual track within a file or stream should be able to be correctly rendered, processed or distributed according to the accompanying metadata. To ensure compatibility across all systems, the Audio Definition Model is an open standard that will make this possible.

# 2	Background

This purpose of this model is to formalise the description of audio. It is not a format for carrying audio. This distinction will help in the understanding of the model.

## 2.1	Cooking Analogy

To help explain what the ADM actually does, it may be useful to consider a cooking analogy. The recipe for a cake will contain a list of ingredients, instructions on how to combine those ingredients and how to bake the cake.

The ADM is like a set of rules for writing the list of ingredients; it gives a clear description of each item, for example: 2 eggs, 400g flour, 200g butter, 200g sugar.

The ADM provides the instructions for combining ingredients but does not tell you how to do the mixing or baking; in the audio world that is what the renderer does.

The ADM is in general compatible with wave-file based formats such as ITU-R BS.1352, the BWF as defined by the EBU in [4] and other wave based formats that support the usage of the needed additional chunks.

When used in the context of a BWF file according to [4], the <chna> chunk of the BWF file is like the bar code on the packet of each of the ingredients; this code allows us to look up the model’s description of each item. The bag containing the actual ingredients is like the 'data' chunk of the BWF file that contains the audio samples.

From a BWF file point of view, we would look at our bar codes on each ingredient in our bag, and use that to look up the description of each item in the bag. Each description follows the structure of the model. There might be ingredients such as breadcrumbs which could be divided into its own components (flour, yeast, etc.); which is like having an audio object containing multiple channels (e.g. ‘stereoʼ containing ‘left’ and ‘right’).

## 2.2	Brief overview

This model will initially use XML as its specification language, though it could be mapped to other languages such as JSON (JavaScript Object Notation) if required. When it is used with BWF files according to [4], the XML can be embedded in the <axml> chunk of the file.

The model is divided into two sections, the content part, and the format part. The content part describes what is contained in the audio, so will describe things like the language of any dialogue, the loudness and so on.

The format part describes the technical nature of the audio so it can be decoded or rendered correctly. Some of the format elements may be defined before we have any audio signals, whereas the content parts can usually only be completed after the signals have been generated.

While this model is based around  a wave-file based format, it is a more general model. However, examples are given using BWF according to the definition in [4] as this explains more clearly how the model works. It is also expected that the model’s parameters are added to in subsequent versions of this specification to reflect the progress in audio technology.

# 3	Description of the model

The overall diagram of the model is given in Fig. 1. This shows how the elements relate to each other and illustrates the split between the content and format parts. It also shows the <chna> chunk of a BWF file according to [4] and how it connects the tracks in the file to the model.

Where a BWF file according to [4] contains a number of audio tracks, it is necessary to know what each track is. The <chna> chunk contains a list of numbers corresponding to each track in the file. Hence, for a 6 track file, the list is at least 6 long. For each track there is an audioTrackFormatID number and an audioTrackUID number (notice the additional ‘U’ which stands for ‘unique’). The reason the list could be longer than the number of tracks is that a single track may have different definitions at different times so will require multiple audioTrackUIDs and references.

The audioTrackFormatID is used to look up the definition of the format of that particular track. The audioTrackFormatIDs are not unique; for example, if a file contains 5 stereo pairs, there will be 5 identical audioTrackFormatIDs to describe the ‘left’ channel, and 5 to describe the ‘right’ channel. Thus, only two different audioTrackFormatIDs will need to be defined. However, audioTrackUIDs are unique (hence the ‘U’), and they are there to uniquely identify the track. This use of IDs means that the tracks can be ordered in any way in the file; their IDs reveal what those tracks are.

Figure 1

Overall UML Model

## 3.1	Format

The audioTrackFormatID answers the question "What is the format of this track?" The audioTrackFormat will also contain an audioStreamFormatID, which allows identification of the combination of the audioTrackFormat and audioStreamFormat. An audioStreamFormat describes a decodable signal.

The audioStreamFormat is made up of one or more audioTrackFormats. Hence, the combination of audioStreamFormat and audioTrackFormat reveals whether the signal has to be decoded or not.

The next stage is to find out what type of audio the stream is; for example it may be a conventional channel (e.g. ‘front left’), an audio object (e.g. something named ‘guitar’ positioned at the front), 
a HOA (Higher Order Ambisonics) component (e.g. ‘X’) or a group of channels . Inside audioStreamFormat there will be a reference to either an audioChannelFormat or audioPackFormat that will describe the audio stream. There will only be one of these references.

If audioStreamFormat contains an audioChannelFormat reference (i.e. audioChannelFormatIDRef) then audioStreamFormat is one of several different types of audioChannelFormat. An audioChannelFormat is a description of a single waveform of audio. In audioChannelFormat there is a typeDefinition attribute, which is used to define what the type of channel is.

The typeDefinition attribute can be set to ‘DirectSpeakers’, ‘HOA’, ‘Matrix’ ‘Objects’ or ‘Binaural’. For each of those types, there is a different set of sub-elements to specify the static parameters associated with that type of audioChannelFormat. For example, the ‘DirectSpeakers’ type of channel has the sub-element ‘speakerLabel’ for allocating a loudspeaker to the channel.

To allow audioChannelFormat to describe dynamic channels (i.e. channels that change in some way over time), it uses audioBlockFormat to divide the channel along the time axis. The audioBlockFormat element will contain a start time (relative to the start time of the parent audioObject) and duration. Within audioBlockFormat there are time-dependent parameters that describe the channel which depend upon the audioChannelFormat type.

For example, the ‘Objects’ type of channel has the sub-elements ‘azimuth’, ‘elevation’ and ‘distance’ to describe the location of the sound. The number and duration of audioBlockFormats is not limited, there could be an audioBlockFormat for every sample if something moves rapidly, though that might be a bit excessive! At least one audioBlockFormat is required and so static channels will have one audioBlockFormat containing the channelʼs parameters.

If audioStreamFormat refers to an audioPackFormat, it describes a group of channels. An audioPackFormat element groups together one or more audioChannelFormats that belong together (e.g. a stereo pair). This is important when rendering the audio, as channels within the group may need to interact with each other.

The reference to an audioPackFormat containing multiple audioChannelFormats from an audioStreamFormat usually occurs when the audioStreamFormat contains non-PCM audio which carries several channels encoded together. AudioPackFormat would usually not be referred from audioStreamFormat for most channel and scene-based formats with PCM audio. Where this reference does exist, the function of audioPackFormat is to combine audioChannelFormats that belong together for rendering purposes.

For example, ‘stereo’, ‘5.1’, ‘1st order Ambisonics’ would all be examples of an audioPackFormat. Note that audioPackFormat just describes the format of the audio. For example, a file containing 5 stereo pairs will contain only one audioPackFormat to describe ‘stereo’. It is possible to nest audioPackFormats; a ‘2nd order HOA’ could contain a ‘1st order HOA’ audioPackFormat alongside audioChannelFormats for the R, S, T, U & V components.

## 3.2	Content

Using an audio scene with 5 stereo pairs as an example, the audioTrackFormat defines which audio tracks are left and right, not which ones belong together, nor what is represented in them. AudioObject is used to determine which tracks belong together and where they are in the file. This element links the actual audio data with the format, and this is where audioTrackUID comes in. For a stereo pair (in PCM), audioObject will contain references to two audioTrackUIDs; therefore, those two tracks will contain stereo audio. It will also contain a reference to audioPackFormat, which defines the format of those two tracks as a stereo pair.

As there are 5 stereo pairs in this example, 5 audioObject elements will be needed. Each one will contain the same reference to a stereo audioPackFormat, but will contain different reference to audioTrackUIDs, as each stereo pair is carrying different audio. The order of audioTrackUIDRefs is not important in an audioObject, as the format definition through audioTrack, audioStreamFormat, audioChannelFormat and audioPackFormat determines which track is which.

The audioObject element also contains startTime and duration attributes. This start time is the time when the signal for the object starts in a file or recording. Thus, if startTime is “00:00:10.00000”, the signal for the object will start 10 seconds into the track in the audio file.

As audioPackFormat can be nested, it follows that audioObjects can be nested. Therefore, the audioObject will contain not only references to the two audioTrackUIDs carrying the stream, but also references to two audioObjects, one for the 5.1 and one for the 2.0.

AudioObject is referred to by audioContent, which gives a description of the content of the audio; it has parameters such as language (if thereʼs dialogue) and the loudness parameters. Some of the values for these parameters can only be calculated after the audio has been generated, and this is why they are not in the format part.

AudioProgramme brings all the audioContent together; it combines them to make the complete ‘mix’.

For example:

–	an audioProgramme may contain audioContent for ‘narrator’ and another one for ‘background music’;

–	an audioProgramme for France may contain audioContents called ‘dialogue-fr’ and ‘backgroundMusic’, and another audioProgramme for the UK which contains audioContents called ‘dialogue-en’ and the same ‘backgroundMusic’.

Multiple audioProgramme elements can be defined in one ADM XML tree representation. This facilitates the description of a presentation that represents a predefined number of meaningful mixes that users can choose from. Each audioProgramme element may reference just a subset of audioContent elements of the ADM XML tree. This is one method to enable the ADM to describe personalized audio.

For example:

–	Following the previous example for audioProgramme, a single ADM XML tree can contain both French and English audioProgramme elements.

–	An ADM XML tree describing a sports program can contain audioProgramme elements for a home team and an away team. The home team audioProgamme may contain audioContent elements for a ‘home team biased commentary’, and another one for ‘ambience’. The away team audioProgramme may contain audioContent for an ‘away team biased commentary’ and the same ‘ambience’.

|  | Ambience | Neutral commentary | Home team biased commentary | Away team biased commentary |
| --- | --- | --- | --- | --- |
| Default mix |  |  |  |  |
| Home team |  |  |  |  |
| Away team |  |  |  |  |

# 4	Standard formats

For many situations, particularly in channel and scene-based work, many of the required formats will be common. For example, mono, stereo and 5.1 all have standard definitions and it would be inefficient to generate and carry a mass of XML every time one of these formats needs to be described. Therefore, the EBU plans to generate a set of standard format descriptions for many of the commonly used formats.

This set will be freely available in a reference XML file, which will be updated regularly. The reference file will not have to be included in a file using the ADM but can be externally referred to. Therefore a file will not need to carry the XML of the format if only standard formats are used. The occasions any ADM XML code will need to be carried in a file is when audioProgramme, audioContent and audioObject are used, or custom definitions are required.

# 5	ADM elements

Each of the elements within the ADM is described in the following subsections. The attributes and sub-elements marked with an asterisk (*) are already defined in the EBU Core Metadata set [5].

## 5.1	audioTrackFormat

The audioTrackFormat element corresponds to a single set of samples or data in a single track in a storage medium. It is used to describe what format the data is in, allowing a renderer to decode the signal correctly. It is referred from the audioStreamFormat element which is used to identify the combination of tracks required to decode the track data successfully.

For PCM audio an audioStreamFormat will refer to a single audioTrackFormat and so the two elements are effectively describing the same thing. For coded audio, multiple audioTrackFormats will have to be combined in a single audioStreamFormat to generate decodable data.

Software that parses the model can start from either audioTrackFormat or audioStreamFormat. To allow for this flexibility audioTrackFormat can also refer back to the audioStreamFormat. However, it is a strict requirement that if this reference is used, the audioTrackFormat must refer to the same audioStreamFormat that is referring back to it.

Figure 2

audioTrackFormat

### 5.1.1	Attributes

| Attribute | Description | Example |
| --- | --- | --- |
| audioTrackFormatID | ID for track | AT_00010001_01 |
| audioTrackFormatName | Name for track | PCM_FrontLeft |
| formatLabel | Descriptor of the format | 0001 |
| formatDefinition | Description of the format | PCM |
| formatLink | URI for the format (not currently used in the ADM) |  |
| formatLanguage | Language of the formatDefinition (not currently used in the ADM) |  |

### 5.1.2	Sub-elements

| Element | Description | Example | Quantity |
| --- | --- | --- | --- |
| audioStreamFormatIDRef | Reference to an audioStreamFormat | AS_00010001 | 0 or 1 |

### 5.1.3	Sample code

<audioTrackFormat audioTrackFormatID="AT_00010001_01" audioTrackFormatName="PCM_FrontLeft" formatDefinition="PCM" formatLabel="0001"> <audioStreamFormatIDRef>AS_00010001</audioStreamFormatIDRef> </audioTrackFormat>

## 5.2	audioStreamFormat

A stream is a combination of tracks (or one track) required to render a channel, object, HOA component or pack. The audioStreamFormat establishes a relationship between audioTrackFormats and the audioChannelFormats or audioPackFormat. Its main use is to deal with non-PCM encoded tracks, where one or more audioTrackFormats must be combined to represent a decodable signal that covers several audioChannelFormats (by referencing an audioPackFormat).

Figure 3

audioStreamFormat

### 5.2.1	Attributes

| Attribute | Description | Example |
| --- | --- | --- |
| audioStreamFormatID | ID for the stream | AS_00010001 |
| audioStreamFormatName | Name of the stream | PCM_FrontLeft |
| formatLabel* | Descriptor of the format | 0001 |
| formatDefinition* | Description of the format | PCM |
| formatLink* | URI for the format (not currently used in the ADM) |  |
| formatLanguage* | Language of format description (not currently used in the ADM) |  |

### 5.2.2	Sub-elements

| Element | Description | Example |
| --- | --- | --- |
| audioChannelFormatIDRef | Reference to audioChannelFormat | AC_00010001 |
| audioPackFormatIDRef | Reference to audioPackFormat | AP_00010003 |
| audioTrackFormatIDRef | Reference to audioTrackFormat | AT_00010001_01 |

### 5.2.3	Sample code

<audioStreamFormat audioStreamFormatID="AT_00010001" audioStreamFormatName="PCM_FrontLeft" formatDefinition="PCM" formatLabel="0001"> <audioTrackFormatIDRef>AT_00010001_01</audioTrackFormatIDRef> <audioChannelFormatIDRef>AC_00010001</audioChannelFormatIDRef> </audioStreamFormat>

## 5.3	audioChannelFormat

An audioChannelFormat represents a single sequence of audio samples on which some action may be performed, such as movement of an object which is rendered in a scene. It is sub-divided in the time domain into one or more audioBlockFormats.

Figure 4

audioChannelFormat

### 5.3.1	Attributes

| Attribute | Description | Example |
| --- | --- | --- |
| audioChannelFormatName | Name of the channel | FrontLeft |
| audioChannelFormatID | ID of the channel, see § 6 for the use of the audioChannelFormatID in typical channel configurations | AC_00010001 |
| typeLabel* | Descriptor of the type of channel | 0001 |
| typeDefinition* | Description of the type of channel | DirectSpeakers |
| typeLink* | URI for the type (not currently used in the ADM) |  |
| typeLanguage* | Language of the typeDefinition (not currently used in the ADM) |  |

The typeDefinition of the audioChannel Format specifies the type of audio it is describing, and also determines which parameters are used within its audioBlockFormat children.

Currently, there are five different typeDefinitions:

| typeDefinition | typeLabel | Description |
| --- | --- | --- |
| DirectSpeakers | 0001 | For channel-based audio, where each channel feeds a speaker directly |
| Matrix | 0002 | For channel-based audio where channels are matrixed together, such as Mid-Side, Lt/Rt |
| Objects | 0003 | For object-based audio where channels represent audio objects (or parts of objects), so include positional information |
| HOA | 0004 | For scene-based audio where Ambisonics and HOA are used |
| Binaural | 0005 | For binaural audio, where playback is over headphones |

### 5.3.2	Sub-elements

| Element | Description | Attributes | Quantity |
| --- | --- | --- | --- |
| audioBlockFormat | Time division of channel containing dynamic metadata | See § 5.4 | 1...* |
| frequency | Sets a high or low cut-off frequency for the audio in Hz | typeDefinition = "lowPass" or "highPass" | 0...2 |

The optional frequency parameter allows a frequency range of the audio to be specified. This can be either low-pass or high-pass, or by combining both to achieve band-pass and band-stop. The mostly common use of this is for LFE channels where a low-pass frequency (e.g. 200 Hz) can be specified.

### 5.3.3	Sample code

<audioChannelFormat audioChannelFormatID="AC_00010001" audioChannelFormatName="FrontLeft" typeDefinition="DirectSpeakers"> <audioBlockFormat ...> ... </audioBlockFormat> </audioChannelFormat>

## 5.4	audioBlockFormat

An audioBlockFormat represents a single sequence of audioChannelFormat samples with fixed parameters, including position, within a specified time interval.

### 5.4.1	Attributes

| Attribute | Description | Example |
| --- | --- | --- |
| audioBlockFormatID | ID for block | AB_00010001_00000001 |
| rtime | Start time of block (relative to the start time of the parent audioObject) | 00:00:00.00000 |
| duration | Duration of block | 00:00:05.00000 |

The sub-elements within audioBlockFormat are dependent upon the typeDefinition or typeLabel of the parent audioChannelFormat element.

Currently, there are five different typeDefinitions:

| typeDefinition | typeLabel | Description |
| --- | --- | --- |
| DirectSpeakers | 0001 | For channel-based audio, where each channel feeds a speaker directly |
| Matrix | 0002 | For channel-based audio where channels are matrixed together, such as Mid-Side, Lt/Rt |
| Objects | 0003 | For object-based audio where channels represent audio objects (or parts of objects) and so include positional information |
| HOA | 0004 | For scene-based audio where Ambisonics and HOA are used |
| Binaural | 0005 | For binaural audio, where playback is over headphones |

### 5.4.2	Sample code

<audioBlockFormat audioBlockFormatID="AB_00010001_00000001" rtime="00:00:00.00000" duration="00:00:05.00000"> ... </audioBlockFormat>

### 5.4.3	Sub-elements

#### 5.4.3.1	If audioChannelFormat.typeDefinition == "DirectSpeakers"

For channel-based systems, this is the metadata used to describe the channel. If the channel is intended to be played out through a specific loudspeaker, then use speakerLabel to indicate the label of that speaker. While both the maximum and minimum values for the three position elements are available (using the bound attribute), they should be avoided as the exact position should normally be specified by omitting the bound attribute.

Figure 5

audioBlockFormat (DirectSpeakers)

| Element | Attribute | Bound attribute | Description | Units | Example | Quantity |
| --- | --- | --- | --- | --- | --- | --- |
| speakerLabel |  | N/A | A reference to the label of the speaker position | – | M-30 | 0...* |
| position | coordinate=“azimuth” |  | Exact azimuth location of sound | Degrees | −30.0 | 1 |
| position | coordinate=“azimuth” | max | Max. azimuth location of sound | Degrees | −22.5 | 0 or 1 |
| position | coordinate=“azimuth” | min | Min. azimuth location of sound | Degrees | −30.0 | 0 or 1 |
| position | coordinate=“elevation” |  | Exact elevation location of sound | Degrees | 0.0 | 1 |
| position | coordinate=“elevation” | max | Max. elevation location of sound | Degrees | 5.0 | 0 or 1 |
| position | coordinate=“elevation” | min | Min. elevation location of sound | Degrees | 0.0 | 0 or 1 |
| position | coordinate=“distance” |  | Exact normalized distance from origin | Normalized to 1 | 1.0 | 0 or 1 |
| position | coordinate=“distance” | max | Max. normalized distance from origin | Normalized to 1 | 0.8 | 0 or 1 |
| position | coordinate=“distance” | min | Min. normalized distance from origin | Normalized to 1 | 0.9 | 0 or 1 |
| position | screenEdgeLock |  | Defines a speaker position at a screen edge | Left, right, top, bottom | left | 0 or 1 |

The screenEdgeLock attribute allows a speaker to be positioned on the edge of the screen. The attribute can be used in combination with the coordinate=“elevation” and/or the coordinate=“azimuth” attribute and it is set to a string stating at which edge of the screen to the speaker position should be assumed to be (if screen-size information is available), so it is either “left”, “right”, “top”, “bottom”. The coordinate attribute must still be included so it is clear which dimension is being set, and to provide an alternative position should the screen not exist or no screen-size information be available.

The example XML code below illustrates how a speaker positioned on the right edge of the screen can be defined (with an alternative position of −29.0 degrees should the screen not exist).

<audioBlockFormat ...> <speakerLabel>M-SC</speakerLabel> <position coordinate="azimuth" screenEdgeLock=”right”>-29.0</position> <position coordinate="elevation">0.0</position> <position coordinate="distance">1.0</position> </audioBlockFormat>

The distance measure is normalized, as absolute speaker distances from the origin are rarely used, but an absolute reference distance is available in audioPackFormat. These coordinates are based on the polar system, as this is the common way of describing channel and speaker locations. However it is also possible to use the Cartesian coordinate system by using different coordinate attributes (‘X’, ‘Y’ and ‘Z’); and this system is described in more detail in § 8.

##### 5.4.3.1.1	Sample code

<audioBlockFormat ...> <speakerLabel>M-30</speakerLabel> <position coordinate="azimuth">-30.0</position> <position coordinate="elevation">0.0</position> <position coordinate="distance">1.0</position> </audioBlockFormat>

#### 5.4.3.2	If audioChannelFormat.typeDefinition == “Matrix”

This is for channel-based matrix channels, such as mid-side and Lt/Rt. The matrix element contains a list of coefficient sub-elements which each refer to other channels and a multiplication factor. All the coefficients in this list should be added together to generate the matrix equation.

Figure 6

audioBlockFormat (Matrix)

For example, the matrix element of a ‘Side’ channel will contain two coefficient sub-elements, one with the value 1.0 referring to "Left" and the other with a value of −1.0 referring to ‘Right’; this gives Side=Left-Right.

The values for gain and phase shift can either be constants (using gain and phase) or they may be variables (using gainVar and phaseVar) that allow the renderer to decide the value, maybe via another source of metadata. This type of channel can also be used to specify down-mixes of channels, such as Lo/Ro.

| Element | Sub-elements | Description | Quantity |
| --- | --- | --- | --- |
| matrix | coefficient | Contains the coefficients for combining other channels | 1 |

| Sub-element | Attribute | Description | Units | Example | Quantity |
| --- | --- | --- | --- | --- | --- |
| coefficient | gain | Multiplication factor of another channel. Constant value. | Ratio | −0.5 | 1...* |
| coefficient | gainVar | Multiplication factor of another channel. Variable. | Ratio | csel | 1...* |
| coefficient | phase | Phase shift of another channel. Constant value. | Degrees | 90 | 1...* |
| coefficient | phaseVar | Phase shift of another channel. Variable. | Degrees | ph | 1...* |
| coefficient |  | Reference to other channel definition |  | AC_00010001 | 1...* |

##### 5.4.3.2.1	Sample code

<audioBlockFormat ...> <matrix> <coefficient gain="1.0">AC_00010001</coefficient> <coefficient gain="-1.0">AC_00010002</coefficient> </matrix> </audioBlockFormat>

#### 5.4.3.3	If audioChannelFormat.typeDefinition == "Objects"

This is for object-based audio where the position of the audio object may change dynamically. As well as the polar coordinates of the object, there are parameters for the object’s size, and whether it is a diffuse or coherent sound.

The channelLock parameter will inform a renderer to send the object’s audio to the nearest speaker or channel, rather than the usual panning, interpolation, etc. The jumpPosition parameter will ensure the renderer does not perform any temporal interpolation of the position values, so the object will jump in space rather than move smoothly to the next position.

The position elements use the coordinate attribute to specify which axis is used. The primary coordinate system is the Polar coordinate system, which uses azimuth, elevation and distance axes. However, it is possible to specify other axes for other coordinates such as X, Y and Z for the Cartesian coordinate system. This is described in more detail in § 8.

The position and object size parameters definitions depend upon the coordinate system used, so they are each described in the two following tables.

For a polar/spherical coordinate system:

| Sub-element | Attribute | Description | Units | Example | Quantity | Default |
| --- | --- | --- | --- | --- | --- | --- |
| position | coordinate= "azimuth" | azimuth “theta” of sound location | Degrees (−180 ≤ theta ≤ 180) | −22.5 | 1 |  |
| position | coordinate= "elevation" | elevation “phi” of sound location | Degrees (−90 ≤ phi ≤ 90) | 5.0 | 1 |  |
| position | coordinate= "distance" | distance “r” from origin | abs(r) ≤ 1 | 0.9 | 0 or 1 | 1.0 |
| width |  | horizontal extent | Degrees | 45 | 0 or 1 | 0.0 |
| height |  | vertical extent | Degrees | 20 | 0 or 1 | 0.0 |
| depth |  | distance extent | Ratio | 0.2 | 0 or 1 | 0.0 |

For a Cartesian coordinate system:

| Sub-element | Attribute | Description | Units | Example | Quantity | Default |
| --- | --- | --- | --- | --- | --- | --- |
| position | coordinate=“X” | left/right dimension | Normalized Units (abs(X) ≤ 1) | −0.2 | 1 |  |
| position | coordinate=“Y” | back/front dimension | Normalized Units (abs(Y) ≤ 1) | 0.1 | 1 |  |
| position | coordinate="Z” | bottom/top dimension | Normalized Units abs(Z) ≤ 1 | −0.5 | 0 or 1 | 0.0 |
| width |  | X-width | Normalized Units 0 ≤ X ≤ 1 | 0.03 | 0 or 1 | 0.0 |
| depth |  | Y-width | Normalized Units 0 ≤ Y ≤ 1 | 0.05 | 0 or 1 | 0.0 |
| height |  | Z-width | Normalized Units 0 ≤ Z ≤ 1 | 0.07 | 0 or 1 | 0.0 |

The screenEdgeLock attribute also exists with the position element, which is described in § 5.4.3.1.

The following parameters are independent of the coordinates system used:

| Sub-element | Attribute | Description | Units | Example | Quantity | Default |
| --- | --- | --- | --- | --- | --- | --- |
| cartesian |  | Specifies coordinate system, if the flag is set to 1 the Cartesian coordinate system is used, otherwise spherical coordinates are used. | 1/0 flag | 1 | 0 or 1 | 0 |
| gain |  | Apply a gain to the audio in the object | linear gain value | 0.5 | 0 or 1 | 1.0 |
| diffuse |  | Describes the diffuseness of an audioObject (if it is diffuse or direct sound) | 0.0 to 1.0 | 0.5 | 0 or 1 | 0 |
| channelLock | maxDistance | If set to 1 a renderer can lock the object to the nearest channel or speaker, rather than normal rendering. The optional maxDistance attribute defines the radius of a sphere around the object’s position. If one or more speakers exist in the defined sphere or on its surface, the object snaps to the nearest speaker. If maxDistance is undefined, a default value of infinity is assumed, meaning that the object should snap to the nearest of all speakers (unconditioned channelLock). | 1/0 flag for channelLock, float value for maxDistance in the range from 0.0 to 2.0 | 1, 1.0 | 0 or 1 | 0 (channelLock), infinity (maxDistance) |
| objectDivergence | azimuthRange | Adjusts the balance between the object’s specified position and two other positions specified by the azimuthRange value (symmetrical on both sides of the object at the object’s position +/– azimuthRange). A value of 0 for the objectDivergence means no divergence. | 0 to 1.0 for objectDivergence, 0.0 to 180.0 (angle) for azimuthRange | 0.5, 60.0 | 0 or 1 | 0.0, 0.0 |
| jumpPosition | interpolationLength | If set to 1 the position will be interpolated over a period set by the attribute interpolationLength. If set to 0 then interpolation will take the entire length of the block. An interpolationLength value of zero will mean the object jumps without interpolation. | 1/0 flag for jumpPosition seconds (5d.p) for interpolationLength | 1, 0.05125 | 0 or 1 for jumpPosition | 0 |
| zoneExclusion (“zone” sub-elements) |  | Indicates which speaker/room zones the object should not be rendered through. | see “zone” sub-elements |  | 0 or 1 |  |
| zone (sub-element of zoneExclusion) | minX maxX minY maxY minZ maxZ | Specifies the corner points of a cuboid in the 3D space that will be excluded from rendering. Multiple zone elements can be used to specify more complex exclusion shapes. | –1.0 to 1.0 float for each attribute, string for a label to describe the exclusion zone | minX=–1.0 maxX=1.0 minY=–1.0 maxY=0.0 minZ=–1.0 maxZ=1.0 “Rear half” | 1..* |  |
| screenRef |  | Indicates whether the object is screen-related (flag is equal to 1) or not (flag is equal to 0) | 1/0 flag | 0 | 0 or 1 | 0 |
| importance |  | Importance of an object. Similar to definition for audioPack, except specifically for an object. | 0 to 10 | 10 | 0 or 1 | 10 |

Figure 7

audioBlockFormat (Objects)

##### 5.4.3.3.1	Sample code

<audioBlockFormat ...> <position coordinate="azimuth">-22.5</position> <position coordinate="elevation">5.0</position> <position coordinate="distance">0.9</position> <depth>0.2</depth> </audioBlockFormat>

#### 5.4.3.4	If audioChannelFormat.typeDefinition == “HOA”

This is for scene-based channels (or components) such as Ambisonics/HOA. The component can be described by either a combination of degree and order values, or an equation. The different versions of Ambisonics (such as N3D and FuMa) are specified by providing suitable names for the parent audioChannelFormat and audioPackFormat elements. Each version should be assigned a range of ID values to accommodate a sufficient number of channels. It is recommended that C-style mathematical notation be used for the equation element (e.g. ‘cos(A)*sin(E)’).

Figure 8

audioBlockFormat (HOA)

| Sub-element | Description | Type | Example | Quantity |
| --- | --- | --- | --- | --- |
| equation | An equation to describe the HOA component | string |  | 0 or 1 |
| degree | Degree of the HOA component | int | 1 | 0 or 1 |
| order | Order of the HOA component | int | 1 | 0 or 1 |

##### 5.4.3.4.1	Sample code

<audioBlockFormat ...> <degree>1</degree> <order>1</order> </audioBlockFormat>

#### 5.4.3.5	If audioChannelFormat.typeDefinition == “Binaural”

This is for binaural representation of audio. Given that binaural consists of two channels, the left and right ear, this is rather simple. As the name of the audioChannelFormat will be either “leftEar” or “rightEar” there is no other metadata required in audioBlockFormat.

Figure 9

audioBlockFormat (Binaural)

#### 5.4.3.6	Sample code

<audioBlockFormat .../>

## 5.5	audioPackFormat

The audioPackFormat groups together one or more audioChannelFormats that belong together.

Examples of audioPackFormats are ‘stereo’ and ‘5.1’ for channel-based formats. It can also contain references to other packs to allow nesting. The typeDefinition is used to define the type of channels described within the pack. The typeDefinition/typeLabel must match those in the referred audioChannelFormats.

Figure 10

audioPackFormat

### 5.5.1	Attributes

| Attribute | Description | Example |
| --- | --- | --- |
| audioPackFormatID | ID for the pack, see § 6 for the use of the audioPackFormatID in typical channel configurations | AP_00010001 |
| audioPackFormatName | Name for the pack | stereo |
| typeLabel | Descriptor of the type of channel | 0001 |
| typeDefinition | Description of the type of channel | DirectSpeakers |
| typeLink | URI for the type (not currently used in the ADM) |  |
| typeLanguage | Language of the typeDefinition (not currently used in the ADM) |  |
| importance | Importance of a pack. Allows a renderer to discard a pack below a certain level of importance. 10 is the most important, 0 is the least. | 10 |

There are five different typeDefinitions:

| typeDefinition | typeLabel | Description |
| --- | --- | --- |
| DirectSpeakers | 0001 | For channel-based audio, where each channel feeds a speaker directly |
| Matrix | 0002 | For channel-based audio where channels are matrixed together, such as Mid-Side, Lt/Rt |
| Objects | 0003 | For object-based audio where channels represent audio objects (or parts of objects), so include positional information |
| HOA | 0004 | For scene-based audio where Ambisonics and HOA are used |
| Binaural | 0005 | For binaural audio, where playback is over headphones |

### 5.5.2	Sub-elements

| Element | Description | Example | Quantity |
| --- | --- | --- | --- |
| audioChannelFormatIDRef | Reference to an audioChannelFormat | AC_00010001 | 0...* |
| audioPackFormatIDRef | Reference to an audioPackFormat | AP_00010002 | 0...* |
| absoluteDistance | Absolute distance in metres | 4.5 | 0 or 1 |

There is an overall absolute distance parameter which can be used with the normalized distance parameters specified with the audioBlockFormats, to give absolute distances to each block.

### 5.5.3	Sample code

<audioPackFormat audioPackFormatID="AP_000010002" audioPackFormatName="stereo" typeLabel="0001"> <audioChannelIDRef>AC_00010001</audioChannelIDRef> <audioChannelIDRef>AC_00010002</audioChannelIDRef> </audioPackFormat>

## 5.6	audioObject

An audioObject establishes the relationship between the content, the format via audio packs, and the assets using the track UIDs. AudioObjects can be nested and so they can refer to other audioObjects.

Figure 11

audioObject

### 5.6.1	Attributes

| Attribute | Description | Example |
| --- | --- | --- |
| audioObjectID | ID of the object | AO_1001 |
| audioObjectName | Name of the object | dialogue_stereo |
| start | Start time for the object, relative to the start of the programme | 00:00:00.00000 |
| duration | Duration of object | 00:02:00.00000 |
| dialogue | If the audio is not dialogue set a value of 0; if it contains only dialogue a value of 1; if it contains both then a value of 2. | 0 |
| importance | Importance of an object. Allows a renderer to discard an object below a certain level of importance. 10 is most important, 0 least. | 10 |
| interact | Set to 1 if a user can interact with the object, 0 if not. | 1 |
| disableDucking | Set to 1 to disallow automatic ducking of object, 0 to allow ducking | 0 |

### 5.6.2	Sub-elements

| Element | Description | Example |
| --- | --- | --- |
| audioPackIDRef | Reference to an audioPack for format description | AP_00010001 |
| audioObjectIDRef | Reference to another audioObject | AO_1002 |
| audioComplementaryObjectIDRef | Reference to another audioObject that is complementary to the object, e.g. to describe mutually exclusive languages. | AO_1003 |
| audioTrackUIDRef | Reference to an audioTrackUID (when using a BWF file according to [4] this is listed in the <chna> chunk) | ATU_00000001 |
| audioObjectInteraction | Specification of possible user interaction with the object. |  |

If the value of audioTrackUIDRef is set to ATU_00000000 then it does not refer to a track in the file, but refers to a silent or empty track. This could useful for multichannel formats where some of the channels are not being used, so instead of storing zero value samples in the file, this silent track is used instead thus saving space in the file.

### 5.6.3	audioComplementaryObjectIDRef

The audioComplementaryObjectIDRef element contains a reference to another audioObject that is complementary to the parent audioObject. A list of audioComplementaryObjectIDRefs can therefore be used to describe mutually exclusive content, e.g. language tracks that contain the same dialogue in different dub versions (“XOR” relationship).

To avoid cross-references between audioComplementaryObjectIDRefs of several audioObjects, the audioComplementaryObjectIDRef sub-element should only be included in one corresponding parent audioObject for each set of mutually exclusive contents. The parent audioObject with the audioComplementaryObjectIDRefs should be the one that contains the default version of the set of mutually exclusive contents.

### 5.6.4	audioObjectInteraction sub-element

An audioObjectInteraction element describes any possible user interaction with the corresponding parent audioObject. It should be present only if the “Interact” attribute of the parent audioObject is set to 1. In case the “Interact” attribute of the parent audioObject is set to 0, any audioObjectInteraction element should be ignored. The audioObjectInteraction element has the following attributes and sub-elements.

| Attribute | Description | Example |
| --- | --- | --- |
| onOffInteract | Set to 1 if a user can switch the object on or off, 0 if not. | 1 |
| gainInteract | Set to 1 if a user can change the gain of the object, 0 if not. | 1 |
| positionInteract | Set to 1 if a user can change the position of the object, 0 if not. | 0 |

| Element | Coordinate attribute | Bound attribute | Description | Units | Example |
| --- | --- | --- | --- | --- | --- |
| gainInteractionRange | N/A | min | Minimum gain factor of possible user gain interaction (gainMin = gain (or 1.0 if not defined) * gainInteractionRangeMin) | linear gain value | 0.5 |
|  | N/A | max | Maximum gain factor of possible user gain interaction (gainMax = gain (or 1.0 if not defined) * gainInteractionRangeMax) | linear gain value | 1.5 |
| positionInteractionRange | azimuth | min | Minimum azimuth offset value of possible user position interaction | Degrees | –30.0 |
|  | azimuth | max | Maximum azimuth offset value of possible user position interaction | Degrees | +30.0 |
|  | elevation | min | Minimum elevation offset value of possible user position interaction | Degrees | –15.0 |
|  | elevation | max | Maximum elevation offset value of possible user position interaction | Degrees | +15.0 |
|  | distance | min | Minimum normalized distance of possible user position interaction | 0 to 1 | 0.5 |
|  | distance | max | Maximum normalized distance of possible user position interaction | 0 to 1 | 0.5 |
| positionInteractionRange | X | min | Minimum X-axis offset value of possible user position interaction | Normalized Units (abs(X) ≤ 1) | –0.5 |
|  | X | max | Maximum X-axis offset value of possible user position interaction | Normalized Units (abs(X) ≤ 1) | +0.5 |
|  | Y | min | Minimum Y-axis offset value of possible user position interaction | Normalized Units (abs(Y) ≤ 1) | –0.2 |
|  | Y | max | Maximum Y-axis offset value of possible user position interaction | Normalized Units (abs(Y) ≤ 1) | 0.0 |
|  | Z | min | Minimum Z-axis offset value of possible user position interaction | Normalized Units (abs(Z) ≤ 1) | 0.1 |
|  | Z | max | Maximum Z-axis offset value of possible user position interaction | Normalized Units (abs(Z) ≤ 1) | 0.4 |

#### 5.6.4.1	Sample code

<audioObjectInteraction onOffInteract="1" gainInteract="1" positionInteract="1"> <gainInteractionRange bound="min">0.5</gainInteractionRange> <gainInteractionRange bound="max">2.0</gainInteractionRange> <positionInteractionRange coordinate="elevation" bound="min"> -10.0 </positionInteractionRange> <positionInteractionRange coordinate="elevation" bound="max"> +10.0 </positionInteractionRange> <positionInteractionRange coordinate="azimuth" bound="min"> -30.0 </positionInteractionRange> <positionInteractionRange coordinate="azimuth" bound="max"> +30.0 </positionInteractionRange> </audioObjectInteraction>

### 5.6.5	Sample code

<audioObject audioObjectID="AO_1001" audioObjectName="Dialogue_stereo"> <audioPackIDRef>AP_00010001</audioPackIDRef> <audioTrackUIDRef>ATU_00000001</audioTrackUIDRef> <audioTrackUIDRef>ATU_00000002</audioTrackUIDRef> </audioObject>

## 5.7	audioContent

An audioContent element describes the content of one component of a programme (e.g. background music), and refers to audioObjects to tie the content to its format. This element includes loudness metadata.

Figure 12

audioContent

### 5.7.1	Attributes

| Attribute | Description | Example |
| --- | --- | --- |
| audioContentID | ID of the content | ACO_1001 |
| audioContentName | Name of the content | Music |
| audioContentLanguage | Language of the content | en |

### 5.7.2	Sub-elements

| Element | Description | Example |
| --- | --- | --- |
| audioObjectIDRef | Reference to audioObject | AO_1001 |
| loudnessMetadata | See § 5.7.3 |  |
| dialogue | If the audio is not dialogue set a value of 0; if it contains only dialogue set a value of 1; if it contains both then set a value of 2. | 0 |

### 5.7.3	dialogue

This element specifies the kind of content that is included in the parent audioContent. The Dialogue sub-element can take the values 0 (no dialogue), 1 (pure dialogue) or 2 (mixed). It has an attribute that specifies the type of content using defined lists (enumerators) of content kinds.

The attribute is dependent on the value of the Dialogue element.

| Value of dialogue | Attribute | Description | Example |
| --- | --- | --- | --- |
| 0 | nonDialogueContentKind | ID of the contained content kind (enumerator, see specification below) | 0 |
| 1 | dialogueContentKind | ID of the contained content kind (enumerator, see specification below) | 0 |
| 2 | mixedContentKind | ID of the contained content kind (enumerator, see specification below) | 0 |

| nonDialogueContentKind | Description |
| --- | --- |
| 0 | undefined |
| 1 | music |
| 2 | effect |
| dialogueContentKind | Description |
| 0 | undefined |
| 1 | (storyline) dialogue |
| 2 | voiceover |
| 3 | spoken subtitle |
| 4 | audio description/visually impaired |
| 5 | commentary |
| 6 | emergency |
| mixedContentKind | Description |
| 0 | undefined |
| 1 | complete main |
| 2 | mixed |
| 3 | hearing impaired |

### 5.7.4	Loudness attributes and sub-elements

| Attribute | Description | Example |
| --- | --- | --- |
| loudnessMethod | The method or algorithm used to calculate the loudness. | “BS.1770” |
| loudnessRecType | The RecType indicates which regional recommended practice was followed in the loudness correction of the audio | “R128” |
| loudnessCorrectionType | The correction type is used to indicate what correction the audio, for example, file-based or real-time. | “File-based” |

The audio could be measured by various means, relating to loudness algorithm, regional recommended practice followed, and by what correction type. The loudnessMethod or algorithm used will typically be BS.1770, but in the future, there could be newer methods. The RecType indicates the regional recommended practice that was followed as a character string, such as, “EBU R128”, “ATSC A/85”, “ARIB TR B32” or “FreeTV OP59”. The CorrectionType specifies how the audio has been correlated: in an off-line file-based or a real-time process.

| Element | Description | Units | Example |
| --- | --- | --- | --- |
| integratedLoudness | Integrated loudness value | LKFS/LUFS | −23.0 |
| loudnessRange | Loudness range | LU | 10.0 |
| maxTruePeak | Maximum true-peak | dBTP | −2.3 |
| maxMomentary | Maximum momentary loudness | LKFS/LUFS | −19.0 |
| maxShortTerm | Maximum short term loudness | LKFS/LUFS | −21.2 |
| dialogueLoudness | Loudness of the average dialogue | LKFS/LUFS | −24.0 |

NOTE – ITU-R BS.1770 uses LKFS for loudness units, and the EBU uses LUFS. Both units are identical, and the model does not require the units to be expressed in the metadata.

### 5.7.5	Sample code

<audioContent audioContentID="ACO_1001" audioContentName="Music"> <audioObjectIDRef>AO_1001</audioObjectIDRef> <loudnessMetadata> <integratedLoudness>-23.0</integratedLoudness> <maxTruePeak>-2.3</maxTruePeak> </loudnessMetadata> </audioContent>

## 5.8	audioProgramme

An audioProgramme element refers to a set of one or more audioContents that are combined to create a full audio programme. It contains start and end timecodes for the programme which can be used for alignment with video timecodes. Loudness metadata is also included to allow the programme’s loudness to be recorded.

Figure 13

audioProgramme

### 5.8.1	Attributes

| Attribute | Description | Example |
| --- | --- | --- |
| audioProgrammeID | ID of the programme | APR_1001 |
| audioProgrammeName | Name of the programme |  |
| audioProgrammeLanguage | Language of the dialogue | fr |
| start | Start timecode for the programme | 00:00:10.00000 |
| end | End timecode for the programme | 00:10:00.00000 |
| typeGroup (Label, Definition, Link, Language) | (not currently used) |  |
| formatGroup (Label, Definition, Link, Language) | (not currently used) |  |
| maxDuckingDepth | Indicates the maximum amount of automatic ducking allowed for every audioObject in the programme. Range is 0 to −62 dB |  |

### 5.8.2	Sub-elements

| Element | Description | Example |
| --- | --- | --- |
| audioContentIDRef | Reference to content | ACO_1001 |
| loudnessMetadata | See § 5.8.3 |  |
| audioProgrammeReferenceScreen | Specification of a reference/production/monitoring screen size for the audioProgramme, see § 5.8.4. If the reference screen-size is not given, a default screen-size is implicitly defined (see § 9.9). |  |

### 5.8.3	audioProgrammeReferenceScreen

An audioProgrammeReferenceScreen element describes a reference/production/monitoring screen that was used by the content creator during the production of the content of this audioObject.

| Attribute | Description | Example |
| --- | --- | --- |
| aspectRatio | Aspect ratio of the screen (proportional relationship between its width and its height (with respect to the image dimensions)) | 1.78, 1.6 |

| Element | Coordinate Attribute | Description | Units | Example |
| --- | --- | --- | --- | --- |
| screenCentrePosition | azimuth | Azimuth angle of the centre of the screen | Degrees | +30.0 |
|  | elevation | Elevation angle of the centre of the screen | Degrees | −15.0 |
|  | distance | Distance to the centre of the screen. Default is 1.0 | 0.0 to 1.0 | 1.0 |
|  | X | X-coordinate of the centre of the screen | Normalized units (abs(X) ≤ 1) | −0.3 |
|  | Y | Y-coordinate of the centre of the screen | Normalized units (abs(Y) ≤ 1) | −0.2 |
|  | Z | Z-coordinate of the centre of the screen | Normalized units (abs(Z) ≤ 1) | 1.0 |
| screenWidth | azimuth | Width of the screen in polar coordinates (azimuth opening angle theta) | Degrees (0 < theta ≤180) | +58.0 |
|  | X | Width of the screen in Cartesian coordinates (width of the screen on the X-axis) | 0 < X <= 2 | 0.8 |

### 5.8.4	Loudness attributes and sub-elements

| Attribute | Description | Example |
| --- | --- | --- |
| loudnessMethod | The method or algorithm used to calculate the loudness. | “BS.1770” |
| loudnessRecType | The RecType indicates which regional recommended practice was followed in the loudness correction of the audio | “R128” |
| loudnessCorrectionType | The correction type is used to indicate what correction the audio, for example, file-based or real-time. | “File-based” |

The audio could be corrected or normalized by numerous means, relating to loudness algorithm, regional recommended practice followed, and by what correction type. The loudnessMethod or algorithm used will typically be “BS.1770” as defined in Recommendation ITU-R BS.1770 [6], but in the future, there could be newer methods. The RecType indicates the regional recommended practice that was followed as a character string, such as “EBU R128”, “ATSC A/85”, “ARIB TR B32” or “FreeTV OP59”. The CorrectionType specifies how the audio has been correlated: in an off-line file-based or a real-time process.

| Element | Description | Units | Example |
| --- | --- | --- | --- |
| integratedLoudness | Integrated loudness value | LKFS/LUFS | −23.0 |
| loudnessRange | Loudness range | LU | 10.0 |
| maxTruePeak | Maximum true-peak | dBTP | −2.3 |
| maxMomentary | Maximum momentary loudness | LKFS/LUFS | −19.0 |
| maxShortTerm | Maximum short term loudness | LKFS/LUFS | −21.2 |
| dialogueLoudness | Loudness of the average dialogue | LKFS/LUFS | −24.0 |

NOTE – ITU-R BS.1770 uses LKFS for loudness units, and the EBU uses LUFS. Both units are identical, and the model does not require the units to be expressed in the metadata.

### 5.8.5	Sample code

<audioProgramme audioProgrammeID="APR_1001" audioProgrammeName="Documentary"> <audioContentIDRef>ACO_1001</audioContentIDRef> <audioContentIDRef>ACO_1002</audioContentIDRef> </audioProgramme>

## 5.9	audioTrackUID

The audioTrackUID uniquely identifies a track or asset within a file or recording of an audio scene. This element contains information about the bit-depth and sample-rate of the track. It also contains sub-elements that allow the model to be used for non-BWF applications by performing the job of the <chna> chunk. When using the model with MXF files the audioMXFLookUp sub-element (which contains sub-elements to refer to the audio essences in the file) is used.

Figure 14

audioTrackUID

### 5.9.1	Attributes

| Attribute | Description | Example |
| --- | --- | --- |
| UID | The actual UID value | ATU_00000001 |
| sampleRate | Sample rate of track in Hz | 48000 |
| bitDepth | Bit-depth of track in bits | 24 |

### 5.9.2	Sub-elements

| Element | Description | Example |
| --- | --- | --- |
| audioMXFLookUp | See § 5.9.3 |  |
| audioTrackFormatIDRef | Reference to an audioTrackFormat description | AT_00010001_01 |
| audioPackFormatIDRef | Reference to an audioPackFormat description | AP_00010002 |

### 5.9.3	MXF sub-elements

MXF has different meanings for the terms ‘track’ and ‘channel’ from their use in the ADM. In MXF ‘track’ is the storage medium containing audio or video, and for audio this ‘track’ can be sub-divided into ‘channels’.

| Element | Description | Type | Example |
| --- | --- | --- | --- |
| packageUIDRef | Reference to an MXF package | UMID string | urn:smpte:umid: 060a2b34.01010105.01010f20.13000000. 540bca53.41434f05.8ce5f4e3.5b72c985 |
| trackIDRef | Reference to an MXF track | int | MXFTRACK_3 |
| channelIDRef | Reference to a channel track | Int | MXFCHAN_1 |

### 5.9.4	Sample code

<audioTrackUID UID="ATU_00000001" sampleRate="48000" bitDepth="24"/>

## 5.10	audioFormatExtended

AudioFormatExtended is the parent element, containing all the ADM elements.

### 5.10.1	Sub-elements

| Element | Description |
| --- | --- |
| audioProgramme | Description of the whole audio programme. |
| audioContent | Description of the content of some audio within the programme. |
| audioObject | The link between the actual audio tracks and their format. |
| audioPackFormat | A description of a pack of channels that relate together. |
| audioChannelFormat | A description of an audio channel. |
| audioStreamFormat | A description of an audio stream. |
| audioTrackFormat | A description of an audio track. |
| audioTrackUID | The unique identifier for an actual audio track. |

# 6	Use of IDs

The ID attributes in each of the elements have three main purposes: to allow the elements to reference each other, to provide a unique identification for each defined element, and to provide a logical numerical representation of the contents of the element. The IDs for each element follows the following format:

| Element | ID format |
| --- | --- |
| audioPackFormat | AP_yyyyxxxx |
| audioChannelFormat | AC_yyyyxxxx |
| audioBlockFormat | AB_yyyyxxxx_zzzzzzzz |
| audioStreamFormat | AS_yyyyxxxx |
| audioTrackFormat | AT_yyyyxxxx_zz |
| audioProgramme | APR_wwww |
| audioContent | ACO_wwww |
| audioObject | AO_wwww |

The yyyy part is a four digit hexadecimal number that represents the type of element it is, by using the typeLabel values. Currently there are 5 defined type label values:

| typeDefinition | typeLabel | Description |
| --- | --- | --- |
| DirectSpeakers | 0001 | For channel-based audio, where each channel feeds a speaker directly |
| Matrix | 0002 | For channel-based audio where channels are matrixed together, such as Mid-Side, Lt/Rt |
| Objects | 0003 | For object-based audio where channels represent audio objects (or parts of objects), so include positional information |
| HOA | 0004 | For scene-based audio where Ambisonics and HOA are used |
| Binaural | 0005 | For binaural audio, where playback is over headphones |

The xxxx part is a four digit hexadecimal number which identifies the description within a particular type. Values in the range 0001-0FFF are reserved for standard definition such as ‘FrontLeft’ or ‘Stereo’. Values in the range 1000-FFFF are for custom definitions, which will be particularly used in object-based audio where all the objects will be custom definitions.

The audioChannelFormatID values in the range 0001-0FFF specify the channel with respect to the channel label and channel configuration. The following audioChannelFormatIDs should be used for typical speaker configurations such as Stereo, 5.1 and 22.2.

Example of Channel Labels

| Attribute | ID of channel | Name of channel |
| --- | --- | --- |
| audioChannelFormatID | AC_00010001 | FrontLeft |
| audioChannelFormatID | AC_00010002 | FrontRight |
| audioChannelFormatID | AC_00010003 | FrontCentre |
| audioChannelFormatID | AC_00010004 | LFE-1 |
| audioChannelFormatID | AC_00010005 | LeftSurround |
| audioChannelFormatID | AC_00010006 | RightSurround |
| audioChannelFormatID | AC_00010101 | FrontLeftCentre |
| audioChannelFormatID | AC_00010102 | FrontRightCentre |
| audioChannelFormatID | AC_00010103 | BackLeft |
| audioChannelFormatID | AC_00010104 | BackRight |
| audioChannelFormatID | AC_00010105 | BackCentre |
| audioChannelFormatID | AC_00010106 | LeftSurroundDirect |
| audioChannelFormatID | AC_00010107 | RightSurroundDirect |
| audioChannelFormatID | AC_00010108 | SideLeft |
| audioChannelFormatID | AC_00010109 | SideRight |
| audioChannelFormatID | AC_0001010A | FrontLeftWide |
| audioChannelFormatID | AC_0001010B | FrontRightWide |
| audioChannelFormatID | AC_0001010C | TopFrontLeft |
| audioChannelFormatID | AC_0001010D | TopFrontRight |
| audioChannelFormatID | AC_0001010E | TopFrontCentre |
| audioChannelFormatID | AC_0001010F | TopBackLeft |
| audioChannelFormatID | AC_00010110 | TopBackRight |
| audioChannelFormatID | AC_00010111 | TopBackCentre |
| audioChannelFormatID | AC_00010112 | TopSideLeft |
| audioChannelFormatID | AC_00010113 | TopSideRight |
| audioChannelFormatID | AC_00010114 | TopCentre |
| audioChannelFormatID | AC_00010115 | LFE-2 |
| audioChannelFormatID | AC_00010116 | BottomFrontLeft |
| audioChannelFormatID | AC_00010117 | BottomFrontRight |
| audioChannelFormatID | AC_00010118 | BottomFrontCentre |
| audioChannelFormatID | AC_00010119 | TopLeftSurround |
| audioChannelFormatID | AC_0001011A | TopRightSurround |
| audioChannelFormatID | AC_0001011B | LeftScreenEdge |
| audioChannelFormatID | AC_0001011C | RightScreenEdge |

The audioPackFormatID specifies the channel configuration. The following standard values for typical channel configurations should be used where applicable:

Examples of Channel Configurations

| Attribute | ID of pack | Name of pack |
| --- | --- | --- |
| audioPackFormatID | AP_00010001 | Mono |
| audioPackFormatID | AP_00010002 | Stereo |
| audioPackFormatID | AP_00010003 | 5.1 |
| audioPackFormatID | AP_00010004 | 7.1Top or 2/0/0+3/0/2+0/0/0.1 |
| audioPackFormatID | AP_00010005 | 9.1Top or 2/0/2+3/0/2+0/0/0.1 |
| audioPackFormatID | AP_00010007 | 10.2 or 2/0/1+3/2/2+0/0/0.2 |
| audioPackFormatID | AP_00010008 | 13.1 or 2/0/2+5/2/2+0/0/0.1 |
| audioPackFormatID | AP_00010009 | 22.2 or 3/3/3+3/2/2+2/0/0.2 |

In audioBlockFormat the zzzzzzzz part is an 8 digit hexadecimal number that acts as an index/counter for the blocks within the channel. The yyyyxxxx values should match those of the parent audioChannelFormat ID.

In audioTrackFormat the zz part is a 2 digit hexadecimal number that acts as an index/counter for the tracks within the stream. The yyyyxxxx values should match those of the reference audioStreamFormat ID.

The audioProgramme, audioContent and audioObject do not have a type and so have no yyyy values. As there is initially no intention to have standard definitions for these elements the values for wwww will be in the hexadecimal range 1000-FFFF because they will always be custom values. However, keeping the standard range of values (0000-0FFF) set aside for now may be useful in future; for example, EBU R 123 configurations may use them.

# 7	<chna> Chunk

While the ADM is designed to be a general model, its relationship with the BW64 file is important to explain. The following describes how a BW64 file will access the ADM metadata via a new RIFF chunk called <chna>. An overview of this new chunk is given here.

The ADM is linked to the BW64 file using the audioTrackFormat, audioPackFormat and audioObject (via audioTrackUID) elements. The BW64 file will contain a new chunk called <chna> (short for ‘channel allocation’), which will contain a set of IDs for each track in the file. These IDs will either refer to elements, or be referred to from an element.

Each track in the chunk contains the following IDs:

•	audioTrackFormatID – the ID of the description of a particular audioTrackFormat element. As audioTrackFormat also refers to audioStreamFormat and either audioPackFormat or audioChannelFormat, this ID is enough to describe the format for a particular track.

•	audioPackFormatID – the ID of the description of a particular audioPackFormat. As most audioChannelFormats need to be assigned to an audioPackFormat (e.g. ‘FrontLeft’ channel in ‘5.1’ pack), it must be specified in the <chna> chunk with this ID.

•	audioTrackUID – the unique ID that identifies the track. The content descriptor audioObject requires knowledge of which tracks in the file are being described, so contains a list of audioTrackUID references which correspond to audio tracks in the file.

To enable tracks to contain more than one audioTrackFormatID, in order to allow different formats in the track at different times, the track number can be allocated multiple IDs. An example of such as allocation is below:

| Track No | audioTrackUID | audioTrackFormatID | audioPackFormatID |
| --- | --- | --- | --- |
| 1 | 00000001 | 00010001_01 | 00010001 |
| 2 | 00000002 | 00031001_01 | 00031001 |
| 2 | 00000003 | 00031002_01 | 00031002 |

Here, track number two has two audioTrackUIDs as the audioTrackFormats and audioPackFormats assigned to it are used at different times in the file. The times of allocation would have to be found be inspecting the audioObject elements that cover those audioTrackUIDs. An example of this is a programme where tracks 1 and 2 contain the theme tune which lasts for the first minute of the file. These tracks are free after this first minute, so some audio objects from the main body of the programme are stored in them subsequently. As the theme tune and the audio objects have completely different formats and contents they require different audioTrackUIDs.

# 8	Coordinate system

The position elements in audioBlockFormat, for both the ‘DirectSpeakers’ and ‘Objects’ typeDefinitions, allow different axes to be specified in the coordinate attribute. The primary coordinate system used is the polar system which uses azimuth, elevation and distance. To ensure consistency when specifying positions each of the polar axes should be based on these guidelines:

•	The origin is in the centre, where the sweet-spot would be (although some systems do not have a sweet-spot, so the centre of the space should be assumed).

•	Azimuth – angle in the horizontal plane with 0 degrees as straight ahead, and positive angles to the left (or anti-clockwise) when viewed from above.

•	Elevation – angle in the vertical plane with 0 degrees horizontally ahead, and positive angles going up.

•	Distance – a normalized distance, where 1.0 is assumed to be the default radius of the sphere.

It is also possible to specify Cartesian coordinates by using X, Y and Z as the coordinate attributes. It is recommended that normalized values be used here, where the values 1.0 and −1.0 are on the surface of the cube, with the origin being the centre of the cube.

The direction of each axis should be:

•	X – left to right, with positive values to the right.

•	Y – front to back, with positive values to the front.

•	Z – top to bottom, with positive values to the top.

If normalized distances are used in the coordinate system they can be scaled to an absolute distance by multiplying by the absoluteDistance parameter in the audioPackFormat.

In Ambisonics and HOA, the coordinate system is also Cartesian based, but the axes are different. In this case the direction of each axis is:

•	X – front to back, with positive values to the front.

•	Y – left to right, with positive values to the left.

•	Z – top to bottom, with positive values to the top.

To avoid confusion with the other Cartesian system, it is recommended the axes be labelled ‘X_HOA’, ‘Y_HOA’ & ‘Z_HOA’. However, the HOA component definitions are unlikely to include coordinate information and so this information is primarily to ensure the rendering is correctly done.

# 9	Object-based parameter descriptions

## 9.1	gain

The gain parameter is a linear gain and controls the level of the audio signal in the object. At rendering the level of signal will be multiplied by the gain value. If the gain parameter is not set a value of 1.0 is assumed, so the audio signal’s level is not adjusted.

## 9.2	diffuse

The diffuse value between 0.0 and 1.0 describes the diffuseness of a sound.

## 9.3	channelLock

If the channelLock flag is set to 1 then the renderer will send the audio signal to the nearest (in terms of 3D position) channel or speaker position. A typical application for this is where the exact location of the object is not critical, but the need for un-processed reproduction of that signal takes priority.

The optional maxDistance attribute defines the radius r, 0 ≤ r ≤ 2, of a sphere around the object’s position. If one or more speakers exist in the defined sphere or on its surface, the object snaps to the nearest speaker. If maxDistance is undefined, a default value of infinity is assumed, meaning that the object should snap to the nearest of all speakers (unconditioned channelLock).

## 9.4	jumpPosition and interpolationLength

If the jumpPosition flag is set to 0 then the renderer will interpolate a moving object between positions over the full duration of the block. If it is set to 1 it will use its interpolationLength attribute to set the interpolation period. If it is set to 0 the interpolationLength will be ignored.

The interpolationLength parameter allows the interpolation of a moving object to be done over a shorter time period than the next update time. This allows the control of the crossfading of objects that may be desirable due to processing done to objects. If the value is set to zero then the object will jump position without interpolation. If this attribute is not included when jumpPosition is set to 1, then the interpolation length will be set to 0.

## 9.5	zoneExclusion

The zoneExclusion parameter is used to dynamically reconfigure the object renderer to “mask out” certain speaker zones during playback. This guarantees that no loudspeaker belonging to the masked zones will be used for rendering the applicable object. Typical zone masks used in production today include sides and rear. Multiple zone sub-elements within zoneExclusion can be set simultaneously to mask out more than one zone. The default is that all zones are enabled and when zoneExclusion is set to one or more of the indicated zones, those are “masked out” during playback. The sub-element zone is used to define the coordinates of the zone in the unit-cuboid.

Zones are defined by the sub-element zone by specifying the corner points of a unit-cuboid in 3D space by: minX, maxX, minY, maxY, minZ, maxZ.

For example: minX= −1.0 maxX=1.0 minY= −1.0 maxY= –1.0 minZ= −1.0 maxZ=1.0 specifies the rear wall.

## 9.6	objectDivergence

The objectDivergence parameter (0.0 to 1.0) indicates the amount an object is split symmetrically into a pair of virtual objects, so that a phantom object is created in the position of the original object. The spread of the signal between the virtual objects should not create an image shift from the original object position and should be power preserving across virtual objects and the original. The azimuthRange attribute allows the angle of virtual objects to be specified. Thus, a value of 45 degrees would place virtual objects 45 degrees to the left and right of the specified object. The default angle is 45 degrees if this attribute is not used. The values of objectDivergence should be interpreted as:

| Value | Description |
| --- | --- |
| 0 | No divergence with only the original object being present. |
| 1 | Maximum divergence where this would represent virtual objects being created azimuthRange degrees on either side of the original position. |

Example: With an LCR loudspeaker configuration and the object positioned directly at the C position, and the LR virtual objects specified by using an azimuthRange of 30 degrees. An objectDivergence value of 0 indicating no divergence, only the centre speaker would be firing. A value of 0.5 would have all three (LCR) loudspeakers firing equally, and a value of 1 would have the L and R loudspeakers firing equally.

## 9.7	screenRef

The screenRef flag is used to indicate whether the object is screen-related or not.

The screenRef flag can be used by a renderer for a special processing of all screen-related objects taking into account the size of a local reproduction screen compared to the production screen-size.

If a renderer uses the screenRef flag to enable a special processing, it should use the reference/monitoring/production screen-size of the currently rendered audioProgramme as the reference screen.

If the flag is set and no audioProgrammeReferenceScreen element is included in the corresponding currently rendered audioProgramme, the reference production/monitoring screen is implicitly defined on the basis of Recommendation ITU-R BT.1845-1 – Guidelines on metrics to be used when tailoring television programmes to broadcasting applications at various image quality levels, display sizes and aspect ratios [8].

A UHDTV-1 (3 840 ×2 160) production/monitoring display and an optimal viewing distance are assumed. Based on the resolution of the production screen and the optimal viewing distance, an optimal horizontal viewing angle can be derived. In case of a UHDTV-1 display it results in an optimal horizontal viewing angle of 58°. The size of the production/monitoring screen is defined as:

| Azimuth of left bottom corner of screen | 29.0° |
| --- | --- |
| Elevation of the left bottom corner of screen | −17.5° |
| Aspect ratio | 1.6 |
| Width of the screen | 58° |

These spherical values can be transferred to Cartesian coordinates assuming a reference distance of 1.0 by first transferring the values above to the “standard” azimuth/elevation convention (0° azimuth is in front of the right ear, positive values are counted counter-clockwise; 0° elevation is directly above the head, positive values are counted downwards to the front) and then using the trigonometric functions to gain the Cartesian coordinates. This results in the following values (orientation of the Cartesian coordinate axes as in § 8):

| X-coordinate of the centre of the screen | 0.0 |
| --- | --- |
| Y-coordinate of the centre of the screen | 0.8341 |
| Z-coordinate of the centre of the screen | 0.0 |
| Aspect ratio | 1.6 |
| Width of the screen | 0.9428 |

## 9.8	importance

The importance parameter allows a renderer to discard objects below a certain level of importance, with 10 being the most, and 0 the least.

# 10	References

[1]	Report ITU-R BS.2266 – Framework of future audio broadcasting systems

[2]	Recommendation ITU-R BS.1909 – Performance requirements for an advanced multichannel stereophonic sound system for use with or without accompanying picture

[3]	Recommendation ITU-R BS.2051 – Advanced sound system for programme production

[4]	EBU Tech 3285 – Specification of the Broadcast Wave Format (BWF)

[5]	EBU Tech 3293 – EBU Core Metadata Set (Version 1.5, April 2014)

[6]	Recommendation ITU-R BS.1770 – Algorithms to measure audio programme loudness and true-peak audio level

[7]	EBU R 123 – EBU Audio Track Allocation for File Exchange (referenced in the Attachment to this Annex)

[8]	Recommendation ITU-R BT.1845-1 – Guidelines on metrics to be used when tailoring television programmes to broadcasting applications at various image quality levels, display sizes and aspect ratios (03/2010)

Attachment to Annex 1

Examples of ADM usage

This section contains a selection of examples of metadata that uses the ADM. These are to help illustrate how the ADM is used, but should not be considered as references for audio definitions.

## A.1	Channel-based example

The most common use of audio is still channel-based, where tracks within a file each represent a static audio channel. This example demonstrates how to define two tracks, streams and channels; and a pack for stereo. The track and stream definitions are for PCM audio. Two objects are defined, both stereo, but containing different content so there are 4 tracks used. This example uses a programme called ‘Documentary’ containing ‘Music’ and ‘Speech’ each defined as separate stereo objects.

The format-related elements in this example represent a tiny subset of the standard reference set of definitions. In practice, this XML code would be part of the standard reference file and would not have to be included in the BWF file. All that would be required is a <chna> chunk with the references to the audioTrackFormats and audioPackFormats and any extra XML required for audioObject, audioContent and audioProgramme.

### A.1.1	Summary of elements

These are the elements in the format part of the description:

| Element | ID | Name | Description |
| --- | --- | --- | --- |
| audioTrackFormat | AT_00010001_01 | PCM_FrontLeft | Defines track as PCM |
| audioTrackFormat | AT_00010002_01 | PCM_FrontRight | Defines track as PCM |
| audioStreamFormat | AS_00010001 | PCM_FrontLeft | Defines stream as PCM |
| audioStreamFormat | AS_00010002 | PCM_FrontRight | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00010001 AB_00010001_00000001 | FrontLeft | Describes channel as front left with a position and speaker reference |
| audioChannelFormat & audioBlockFormat | AC_00010002 AB_00010002_00000001 | FrontRight | Describes channel as front right with a position and speaker reference |
| audioPackFormat | AP_00010002 | Stereo | Defines a stereo pack referring to two channels. |

These are the elements in the content part of the description:

| Element | ID | Name | Description |
| --- | --- | --- | --- |
| audioObject | AO_1001 | Music | Object for ‘Music’, stereo format |
| audioObject | AO_1002 | Speech | Object for ‘Speech’, stereo format |
| audioContent | ACO_1001 | Music | Music content |
| audioContent | ACO_1002 | Speech | Speech content |
| audioProgramme | APR_1001 | Documentary | Programme ‘Documentary’ containing ‘Music’ and ‘Speech’ content |

### A.1.2	Diagram

The diagram shows how the defined elements relate to each other. The top half of the diagram covers the elements that describe the 2 channel stereo format. The <chna> chunk in the middle shows how the four tracks are connected to the format definitions. The content definition elements are at the bottom of the diagram, with the audioObject elements containing the track UID references to the UIDs in the <chna> chunk.

### A.1.3	Sample code

This XML sample code does not include the audioFormatExtended parent element and the XML header for clarity.

The first excerpt of code covers the format elements, which could be contained within the standard reference file:

<!-- ############ --> <!-- PACKS --> <!-- ############ --> <audioPackFormat audioPackFormatID="AP_00010002" audioPackFormatName="Stereo" typeLabel="0001" typeDefinition="DirectSpeakers"> <audioChannelFormatIDRef>AC_00010001</audioChannelFormatIDRef> <audioChannelFormatIDRef>AC_00010002</audioChannelFormatIDRef> </audioPackFormat> <!-- ############ --> <!-- CHANNELS --> <!-- ############ --> <audioChannelFormat audioChannelFormatID="AC_00010001" audioChannelFormatName="FrontLeft" typeLabel="0001" typeDefinition="DirectSpeakers"> <audioBlockFormat audioBlockFormatID="AB_00010001_00000001"> <speakerLabel>M+30</speakerLabel> <position coordinate="azimuth">30.0</position> <position coordinate="elevation">0.0</position> <position coordinate="distance">1.0</position> </audioBlockFormat> </audioChannelFormat> <audioChannelFormat audioChannelFormatID="AC_00010002" audioChannelFormatName="FrontRight" typeLabel="0001" typeDefinition="DirectSpeakers"> <audioBlockFormat audioBlockFormatID="AB_00010002_00000001"> <speakerLabel>M-30</speakerLabel> <position coordinate="azimuth">-30.0</position> <position coordinate="elevation">0.0</position> <position coordinate="distance">1.0</position> </audioBlockFormat> </audioChannelFormat> <!-- ############ --> <!-- STREAMS --> <!-- ############ --> <audioStreamFormat audioStreamFormatID="AS_00010001" audioStreamFormatName="PCM_FrontLeft" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00010001</audioChannelFormatIDRef> <audioTrackFormatIDRef>AT_00010001_01</audioTrackFormatIDRef> </audioStreamFormat> <audioStreamFormat audioStreamFormatID="AS_00010002" audioStreamFormatName="PCM_FrontRight" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00010002</audioChannelFormatIDRef> <audioTrackFormatIDRef>AT_00010002_01</audioTrackFormatIDRef> </audioStreamFormat> <!-- ############ --> <!-- AUDIO TRACKS --> <!-- ############ --> <audioTrackFormat audioTrackFormatID="AT_00010001_01" audioTrackFormatName="PCM_FrontLeft" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00010001</audioStreamFormatIDRef> </audioTrackFormat> <audioTrackFormat audioTrackFormatID="AT_00010002_01" audioTrackFormatName="PCM_FrontRight" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00010002</audioStreamFormatIDRef> </audioTrackFormat>

The second excerpt covers the content part, which would have to be included in the <axml> chunk of the BWF file:

<!-- ############ --> <!-- PROGRAMMES --> <!-- ############ --> <audioProgramme audioProgrammeID="APR_1001" audioProgrammeName="Documentary"> <audioContentIDRef>ACO_1001</audioContentIDRef> <audioContentIDRef>ACO_1002</audioContentIDRef> </audioProgramme> <!-- ############ --> <!-- CONTENTS --> <!-- ############ --> <audioContent audioContentID="ACO_1001" audioContentName="Music"> <audioObjectIDRef>AO_1001</audioObjectIDRef> <loudnessMetadata> <integratedLoudness>-28.0</integratedLoudness> </loudnessMetadata> </audioContent> <audioContent audioContentID="ACO_1002" audioContentName="Speech"> <audioObjectIDRef>AO_1002</audioObjectIDRef> <loudnessMetadata> <integratedLoudness>-23.0</integratedLoudness> </loudnessMetadata> </audioContent> <!-- ############ --> <!-- OBJECTS --> <!-- ############ --> <audioObject audioObjectID="AO_1001" audioObjectName="Music" start="00:00:00.00"> <audioPackFormatIDRef>AP_00010002</audioPackFormatIDRef> <audioTrackUIDRef>ATU_00000001</audioTrackUIDRef> <audioTrackUIDRef>ATU_00000002</audioTrackUIDRef> </audioObject> <audioObject audioObjectID="AO_1002" audioObjectName="Speech" start="00:00:00.00"> <audioPackFormatIDRef>AP_00010002</audioPackFormatIDRef> <audioTrackUIDRef>ATU_00000003</audioTrackUIDRef> <audioTrackUIDRef>ATU_00000004</audioTrackUIDRef> </audioObject> <!-- ############ --> <!—- AUDIO TRACK UIDs --> <!-- ############ --> <audioTrackUID UID="ATU_00000001"> <audioTrackFormatIDRef>AT_00010001_01</audioTrackFormatIDRef> <audioPackFormatIDRef>AP_00010002</audioPackFormatIDRef> </audioTrackUID> <audioTrackUID UID="ATU_00000002"> <audioTrackFormatIDRef>AT_00010002_01</audioTrackFormatIDRef> <audioPackFormatIDRef>AP_00010002</audioPackFormatIDRef> </audioTrackUID> <audioTrackUID UID="ATU_00000003"> <audioTrackFormatIDRef>AT_00010001_01</audioTrackFormatIDRef> <audioPackFormatIDRef>AP_00010002</audioPackFormatIDRef> </audioTrackUID> <audioTrackUID UID="ATU_00000004"> <audioTrackFormatIDRef>AT_00010002_01</audioTrackFormatIDRef> <audioPackFormatIDRef>AP_00010002</audioPackFormatIDRef> </audioTrackUID>

## A.2	Object-based example

To demonstrate how the ADM can be used in object-based audio here is a simple example using a single object. This example uses multiple audioBlockFormats within an audioChannelFormat to describe the dynamic properties of an object called “Car”. The audioBlockFormats uses the start and duration attributes to frame the time dependent metadata, thus allowing the object’s position to move in space.

### A.2.1	Summary of elements

These are the elements in the format part of the description:

| Element | ID | Name | Description |
| --- | --- | --- | --- |
| audioTrackFormat | AT_00031001_01 | PCM_Car1 | Defines track as PCM |
| audioStreamFormat | AS_00031001 | PCM_Car1 | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00031001 AB_00031001_00000001 AB_00031001_00000002 AB_00031001_00000003 | Car1 | Describes channel as an object type containing 3 blocks with different positional metadata in each. |
| audioPackFormat | AP_00031001 | Car | Defines a pack referring to one channel. |

These are the elements in the content part of the description:

| Element | ID | Name | Description |
| --- | --- | --- | --- |
| audioObject | AO_1001 | Car | Object for ‘Car, stereo format |
| audioContent | ACO_1001 | Cars | ‘Cars’ content |
| audioProgramme | APR_1001 | CarsSounds | Programme ‘CarsSounds’ containing ‘Cars’ content |

### A.2.2	Diagram

The diagram shows how the defined elements relate to each other. The top half of the diagram covers the elements that describe the single channel object containing 3 blocks. The <chna> chunk in the middle shows how the single track is connected to the format definitions. The content definition elements are at the bottom of the diagram, with the audioObject element containing the track UID references to the UID in the <chna> chunk.

### A.2.3	Sample code

This XML sample code does not include the audioFormatExtended parent element and the XML header for clarity. The excerpt of code covers both the format and content elements:

<!-- ############ --> <!-- PROGRAMMES --> <!-- ############ --> <audioProgramme audioProgrammeID="APR_1001" audioProgrammeName="CarsSounds"> <audioContentIDRef>ACO_1001</audioContentIDRef> </audioProgramme> <!-- ############ --> <!-- CONTENTS --> <!-- ############ --> <audioContent audioContentID="ACO_1001" audioContentName="Cars"> <audioObjectIDRef>AO_1001</audioObjectIDRef> <loudnessMetadata> <integratedLoudness>-23.0</integratedLoudness> </loudnessMetadata> </audioContent> <!-- ############ --> <!-- OBJECTS --> <!-- ############ --> <audioObject audioObjectID="AO_1001" audioObjectName="Car" start="00:00:00.00000"> <audioPackFormatIDRef>AP_00031001</audioPackFormatIDRef> <audioTrackUIDRef>ATU_00000001</audioTrackUIDRef> </audioObject> <!-- ############ --> <!-- PACKS --> <!-- ############ --> <audioPackFormat audioPackFormatID="AP_00031001" audioPackFormatName="Car" typeLabel="0003" typeDefinition="Objects"> <audioChannelFormatIDRef>AC_00031001</audioChannelFormatIDRef> </audioPackFormat> <!-- ############ --> <!-- CHANNELS --> <!-- ############ --> <audioChannelFormat audioChannelFormatID="AC_00031001" audioChannelFormatName="Car1" typeLabel="0003" typeDefinition="Objects"> <audioBlockFormat audioBlockFormatID="AB_00031001_00000001" rtime="00:00:00.00000" duration="00:00:05.00000"> <position coordinate="azimuth">-22.5</position> <position coordinate="elevation">5.0</position> <position coordinate="distance">1.0</position> </audioBlockFormat> <audioBlockFormat audioBlockFormatID="AB_00031001_00000002" rtime="00:00:05.00000" duration="00:00:10.00000"> <position coordinate="azimuth">-24.5</position> <position coordinate="elevation">6.0</position> <position coordinate="distance">0.9</position> </audioBlockFormat> <audioBlockFormat audioBlockFormatID="AB_00031001_00000003" rtime="00:00:15.00000" duration="00:00:20.00000"> <position coordinate="azimuth">-26.5</position> <position coordinate="elevation">7.0</position> <position coordinate="distance">0.8</position> </audioBlockFormat> </audioChannelFormat> <!-- ############ --> <!-- STREAMS --> <!-- ############ --> <audioStreamFormat audioStreamFormatID="AS_00031001" audioStreamFormatName="PCM_Car1" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00031001</audioChannelFormatIDRef> <audioTrackFormatIDRef>AT_00031001_01</audioTrackFormatIDRef> </audioStreamFormat> <!-- ############ --> <!-- AUDIO TRACKS --> <!-- ############ --> <audioTrackFormat audioTrackFormatID="AT_00031001_01" audioTrackFormatName="PCM_Car1" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00031001</audioStreamFormatIDRef> </audioTrackFormat>

## A.3	Scene-based example

The other main type of audio is scene-based where the audio channels are representing Ambisonics/HOA components. Their use is very similar to that of the channel-based approach with the main difference being the parameters used within audioBlockFormat. This example shows a simple 1st order Ambisonics (using the N3D method) configuration using 4 channels mapped onto 4 tracks. Like the channel-based approach, the format elements would be defined in a standard reference file so in practice would not need to be included in the BWF file itself.

### A.3.1	Summary of elements

These are the elements in the format part of the description:

| Element | ID | Name | Description |
| --- | --- | --- | --- |
| audioTrackFormat | AT_00040001_01 | PCM_N3D_ACN_0 | Defines track as PCM |
| audioTrackFormat | AT_00040002_01 | PCM_N3D_ACN_1 | Defines track as PCM |
| audioTrackFormat | AT_00040003_01 | PCM_N3D_ACN_2 | Defines track as PCM |
| audioTrackFormat | AT_00040004_01 | PCM_N3D_ACN_3 | Defines track as PCM |
| audioStreamFormat | AS_00040001 | PCM_N3D_ACN_0 | Defines stream as PCM |
| audioStreamFormat | AS_00040002 | PCM_N3D_ACN_1 | Defines stream as PCM |
| audioStreamFormat | AS_00040003 | PCM_N3D_ACN_2 | Defines stream as PCM |
| audioStreamFormat | AS_00040004 | PCM_N3D_ACN_3 | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00040001 AB_00040001_00000001 | N3D_ACN_0 | Describes channel as ACN0 HOA component |
| audioChannelFormat & audioBlockFormat | AC_00040002 AB_00040002_00000001 | N3D_ACN_1 | Describes channel as ACN1 HOA component |
| audioChannelFormat & audioBlockFormat | AC_00040003 AB_00040003_00000001 | N3D_ACN_2 | Describes channel as ACN2 HOA component |
| audioChannelFormat & audioBlockFormat | AC_00040004 AB_00040004_00000001 | N3D_ACN_3 | Describes channel as ACN3 HOA component |
| audioPackFormat | AP_00040001 | HOA_N3D_1st | Defines a 1st order HOA pack referring to four ACN channels. |

These are the elements in the content part of the description:

| Element | ID | Name | Description |
| --- | --- | --- | --- |
| audioObject | AO_1001 | BackgroundHOA | Object for ‘BackgroundHOA’, 1st order HOA format |
| audioContent | ACO_1001 | Background | ‘Background’ content |
| audioProgramme | APR_1001 | HOADemo | ‘HOADemo’ containing a ‘Background’content |

### A.3.2	Diagram

The diagram shows how the defined elements relate to each other. The top half of the diagram covers the elements that describe the 4 channels of the 1st order HOA (N3D method). The <chna> chunk in the middle shows how the four tracks are connected to the format definitions. The content definition elements are at the bottom of the diagram, with the audioObject element containing the track UID references to the UIDs in the <chna> chunk.

### A.3.3	Sample code

This XML sample code does not include the audioFormatExtended parent element and the XML header for clarity. The first excerpt of code covers the format elements, which could be contained within the standard reference file:

<!-- ############ --> <!-- PACKS --> <!-- ############ --> <audioPackFormat audioPackFormatID="AP_00040001" audioPackFormatName="HOA_N3D_1st" typeLabel="0004" typeDefinition="HOA"> <audioChannelFormatIDRef>AC_00040001</audioChannelFormatIDRef> <audioChannelFormatIDRef>AC_00040002</audioChannelFormatIDRef> <audioChannelFormatIDRef>AC_00040003</audioChannelFormatIDRef> <audioChannelFormatIDRef>AC_00040004</audioChannelFormatIDRef> </audioPackFormat> <!-- ############ --> <!-- CHANNELS --> <!-- ############ --> <audioChannelFormat audioChannelFormatID="AC_00040001" audioChannelFormatName="N3D_ACN_0" typeDefinition="HOA"> <audioBlockFormat audioBlockFormatID="AB_00040001_00000001"> <equation>1</equation> <degree>0</degree> <order>0</order> </audioBlockFormat> </audioChannelFormat> <audioChannelFormat audioChannelFormatID="AC_00040002" audioChannelFormatName="N3D_ACN_1" typeDefinition="HOA"> <audioBlockFormat audioBlockFormatID="AB_00040002_00000001"> <equation>sqrt(3)*cos(E)</equation> <degree>1</degree> <order>-1</order> </audioBlockFormat> </audioChannelFormat> <audioChannelFormat audioChannelFormatID="AC_00040003" audioChannelFormatName="N3D_ACN_2" typeDefinition="HOA"> <audioBlockFormat audioBlockFormatID="AB_00040003_00000001"> <equation>sqrt(3)*sin(E)</equation> <degree>1</degree> <order>0</order> </audioBlockFormat> </audioChannelFormat> <audioChannelFormat audioChannelFormatID="AC_00040004" audioChannelFormatName="N3D_ACN_3" typeDefinition="HOA"> <audioBlockFormat audioBlockFormatID="AB_00040004_00000001"> <equation>sqrt(3)*cos(E)*cos(A)</equation> <degree>1</degree> <order>1</order> </audioBlockFormat> </audioChannelFormat> <!-- ############ --> <!-- STREAMS --> <!-- ############ --> <audioStreamFormat audioStreamFormatID="AS_00040001" audioStreamFormatName="PCM_N3D_ACN_0" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00040001</audioChannelFormatIDRef> <audioTrackFormatIDRef>AT_00040001_01</audioTrackFormatIDRef> </audioStreamFormat> <audioStreamFormat audioStreamFormatID="AS_00040002" audioStreamFormatName="PCM_N3D_ACN_1" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00040002</audioChannelFormatIDRef> <audioTrackFormatIDRef>AT_00040002_01</audioTrackFormatIDRef> </audioStreamFormat> <audioStreamFormat audioStreamFormatID="AS_00040003" audioStreamFormatName="PCM_N3D_ACN_2" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00040003</audioChannelFormatIDRef> <audioTrackFormatIDRef>AT_00040003_01</audioTrackFormatIDRef> </audioStreamFormat> <audioStreamFormat audioStreamFormatID="AS_00040004" audioStreamFormatName="PCM_N3D_ACN_3" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00040004</audioChannelFormatIDRef> <audioTrackFormatIDRef>AT_00040004_01</audioTrackFormatIDRef> </audioStreamFormat> <!-- ############ --> <!-- AUDIO TRACKS --> <!-- ############ --> <audioTrackFormat audioTrackFormatID="AT_00040001_01" audioTrackFormatName="PCM_N3D_ACN_0" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00040001</audioStreamFormatIDRef> </audioTrackFormat> <audioTrackFormat audioTrackFormatID="AT_00040002_01" audioTrackFormatName="PCM_N3D_ACN_1" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00040002</audioStreamFormatIDRef> </audioTrackFormat> <audioTrackFormat audioTrackFormatID="AT_00040003_01" audioTrackFormatName="PCM_N3D_ACN_2" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00040003</audioStreamFormatIDRef> </audioTrackFormat> <audioTrackFormat audioTrackFormatID="AT_00040004_01" audioTrackFormatName="PCM_N3D_ACN_3" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00040004</audioStreamFormatIDRef> </audioTrackFormat>

The second excerpt covers the content part, which would have to be included in the <axml> chunk of the BWF file:

<!-- ############ --> <!-- PROGRAMMES --> <!-- ############ --> <audioProgramme audioProgrammeID="APR_1001" audioProgrammeName="HOADemo"> <audioContentIDRef>ACO_1001</audioContentIDRef> </audioProgramme> <!-- ############ --> <!-- CONTENTS --> <!-- ############ --> <audioContent audioContentID="ACO_1001" audioContentName="Background"> <audioObjectIDRef>AO_1001</audioObjectIDRef> </audioContent> <!-- ############ --> <!-- OBJECTS --> <!-- ############ --> <audioObject audioObjectID="AO_1001" audioObjectName="BackgroundHOA"> <audioPackFormatIDRef>AP_00040001</audioPackFormatIDRef> <audioTrackUIDRef>ATU_00000001</audioTrackUIDRef> <audioTrackUIDRef>ATU_00000002</audioTrackUIDRef> <audioTrackUIDRef>ATU_00000003</audioTrackUIDRef> <audioTrackUIDRef>ATU_00000004</audioTrackUIDRef> </audioObject> <!-- ############ --> <!—- AUDIO TRACK UIDs --> <!-- ############ --> <audioTrackUID UID="ATU_00000001"> <audioTrackFormatIDRef>AT_00040001_01</audioTrackFormatIDRef> <audioPackFormatIDRef>AP_00040001</audioPackFormatIDRef> </audioTrackUID> <audioTrackUID UID="ATU_00000002"> <audioTrackFormatIDRef>AT_00040002_01</audioTrackFormatIDRef> <audioPackFormatIDRef>AP_00040001</audioPackFormatIDRef> </audioTrackUID> <audioTrackUID UID="ATU_00000003"> <audioTrackFormatIDRef>AT_00040003_01</audioTrackFormatIDRef> <audioPackFormatIDRef>AP_00040001</audioPackFormatIDRef> </audioTrackUID> <audioTrackUID UID="ATU_00000004"> <audioTrackFormatIDRef>AT_00040004_01</audioTrackFormatIDRef> <audioPackFormatIDRef>AP_00040001</audioPackFormatIDRef> </audioTrackUID>

## A.4	Material exchange format mapping example

The ADM has been designed to not only allow BWF files according to [4] to become a flexible multichannel file format, but also be incorporated by other file formats. Currently, material exchange format ((MXF) – SMPTE 377M), which carries both video and audio, has a rather limited capability in terms of specifying its audio format. The ADM could be used by MXF files in a similar way to BWF files allowing a comprehensive format description of the audio.

MXF files often use EBU R 123 [7] (“EBU Audio Track Allocation for File Exchange”) audio track configurations. This is a set of channel and matrix-based track allocations for between 2 and 16 track files or streams. This example will show how a particular R123 configuration can be represented by the ADM that’s suitable for MXF.

This example will demonstrate how the 4a R 123 configuration can be represented by the ADM. This configuration uses 4 tracks:

| Track Number | Track Use | Group |
| --- | --- | --- |
| 1 | Stereo Left (PCM) | PCM Stereo pair |
| 2 | Stereo Right (PCM) |  |
| 3 | MCA (Dolby E) | Multichannel audio Dolby E stream |
| 4 | MCA (Dolby E) |  |

## A.4.1	Summary of elements

These are the elements in the format part of the description:

| Element | ID | Name | Description |
| --- | --- | --- | --- |
| audioTrackFormat | AT_00010001_01 | PCM_FrontLeft | Defines track as PCM |
| audioTrackFormat | AT_00010002_01 | PCM_FrontRight | Defines track as PCM |
| audioTrackFormat | AT_00020001_01 | DolbyE1 | Defines track as containing Dolby E data |
| audioTrackFormat | AT_00020001_02 | DolbyE1 | Defines track as containing Dolby E data |
| audioStreamFormat | AS_00010001 | PCM_FrontLeft | Defines stream as PCM |
| audioStreamFormat | AS_00010002 | PCM_FrontRight | Defines stream as PCM |
| audioStreamFormat | AS_00020001 | DolbyE_5.1 | Defines stream as Dolby E |
| audioChannelFormat & audioBlockFormat | AC_00010001 AB_00010001_00000001 | FrontLeft | Describes channel as front left with a position and speaker reference |
| audioChannelFormat & audioBlockFormat | AC_00010002 AB_00010002_00000001 | FrontRight | Describes channel as front right with a position and speaker reference |
| audioChannelFormat & audioBlockFormat | AC_00010003 AB_00010003_00000001 | FrontCentre | Describes channel as front centre with a position and speaker reference |
| audioChannelFormat & audioBlockFormat | AC_00010004 AB_00010004_00000001 | LFE | Describes channel as LFE with a position and speaker reference |
| audioChannelFormat & audioBlockFormat | AC_00010005 AB_00010005_00000001 | SurroundLeft | Describes channel as front right with a position and speaker reference |
| audioChannelFormat & audioBlockFormat | AC_00010006 AB_00010006_00000001 | SurroundRight | Describes channel as front right with a position and speaker reference |
| audioPackFormat | AP_00010002 | Stereo | Defines a stereo pack referring to two channels. |
| audioPackFormat | AP_00010003 | 5.1 | Defines a 5.1 pack referring to 6 channels. |

These are the elements in the content part of the description:

| Element | ID | Name | Description |
| --- | --- | --- | --- |
| audioObject | AO_0041 | R123_4a | Object for R123 4a configuration |
| audioObject | AO_0002 | R123_Stereo | Object for stereo |
| audioObject | AO_0004 | R123_5.1 | Object for 5.1 |

### A.4.2	Diagram

The diagram shows how the defined elements relate to each other. The top half of the diagram covers the elements that describe the 2 channel stereo PCM format and the 6 channel Dolby E 5.1 encoded format. In the Dolby E part, two audioTrackFormats refer to a single audioStreamFormat as Dolby E requires that two tracks are combined to decode the audio signals. The Dolby E audioStreamFormat refers to an audioPackFormat as it is representing a group of channels rather than a single one. This 5.1 audioPackFormat refers to the 6 audioChannelFormats that describe each channel.

The R123 4a configuration is represented by an audioObject (named ‘R123_4a’) which refers to two further audioObjects (for the stereo and 5.1 groups) which contain the references to the audioTrackUIDs. This demonstrates the nesting feature of audioObjects.

As MXF does not feature a <chna> chunk, it uses sub-elements of audioTrackUID to generate references to the essences within the MXF file. The audioMXFLookUp sub-element is designed to facilitate these relationships.

### A.4.3	Sample code

This XML sample code does not include the audioFormatExtended parent element and the XML header for clarity. The first excerpt of code covers the format elements, which could be contained within the standard reference file:

<!-- ############ --> <!-- PACKS --> <!-- ############ --> <audioPackFormat audioPackFormatID="AP_00010002" audioPackFormatName="Stereo" typeLabel="0001" typeDefinition="DirectSpeakers"> <audioChannelFormatIDRef>AC_00010001</audioChannelFormatIDRef> <audioChannelFormatIDRef>AC_00010002</audioChannelFormatIDRef> </audioPackFormat> <audioPackFormat audioPackFormatID="AP_00010003" audioPackFormatName="5.1" typeLabel="0001" typeDefinition="DirectSpeakers"> <audioChannelFormatIDRef>AC_00010001</audioChannelFormatIDRef> <audioChannelFormatIDRef>AC_00010002</audioChannelFormatIDRef> <audioChannelFormatIDRef>AC_00010003</audioChannelFormatIDRef> <audioChannelFormatIDRef>AC_00010004</audioChannelFormatIDRef> <audioChannelFormatIDRef>AC_00010005</audioChannelFormatIDRef> <audioChannelFormatIDRef>AC_00010006</audioChannelFormatIDRef> </audioPackFormat> <!-- ############ --> <!-- CHANNELS --> <!-- ############ --> <audioChannelFormat audioChannelFormatID="AC_00010001" audioChannelFormatName="FrontLeft" typeLabel="0001" typeDefinition="DirectSpeakers"> <audioBlockFormat audioBlockFormatID="AB_00010001_00000001"> <speakerLabel>M+30</speakerLabel> <position coordinate="azimuth">30.0</position> <position coordinate="elevation">0.0</position> <position coordinate="distance">1.0</position> </audioBlockFormat> </audioChannelFormat> <audioChannelFormat audioChannelFormatID="AC_00010002" audioChannelFormatName="FrontRight" typeLabel="0001" typeDefinition="DirectSpeakers"> <audioBlockFormat audioBlockFormatID="AB_00010002_00000001"> <speakerLabel>M-30</speakerLabel> <position coordinate="azimuth">-30.0</position> <position coordinate="elevation">0.0</position> <position coordinate="distance">1.0</position> </audioBlockFormat> </audioChannelFormat> <audioChannelFormat audioChannelFormatID="AC_00010003" audioChannelFormatName="FrontCentre" typeLabel="0001" typeDefinition="DirectSpeakers"> <audioBlockFormat audioBlockFormatID="AB_00010003_00000001"> <speakerLabel>M+00</speakerLabel> <position coordinate="azimuth">0.0</position> <position coordinate="elevation">0.0</position> <position coordinate="distance">1.0</position> </audioBlockFormat> </audioChannelFormat> <audioChannelFormat audioChannelFormatID="AC_00010004" audioChannelFormatName="LFE" typeLabel="0001" typeDefinition="DirectSpeakers"> <frequency typeDefinition="lowPass">200</frequency> <audioBlockFormat audioBlockFormatID="AB_00010004_00000001"> <speakerLabel>LFE+00</speakerLabel> <position coordinate="azimuth">0.0</position> <position coordinate="elevation">-20.0</position> <position coordinate="distance">1.0</position> </audioBlockFormat> </audioChannelFormat> <audioChannelFormat audioChannelFormatID="AC_00010005" audioChannelFormatName="SurroundLeft" typeLabel="0001" typeDefinition="DirectSpeakers"> <audioBlockFormat audioBlockFormatID="AB_00010005_00000001"> <speakerLabel>M+110</speakerLabel> <position coordinate="azimuth">110.0</position> <position coordinate="elevation">0.0</position> <position coordinate="distance">1.0</position> </audioBlockFormat> </audioChannelFormat> <audioChannelFormat audioChannelFormatID="AC_00010006" audioChannelFormatName="SurroundRight" typeLabel="0001" typeDefinition="DirectSpeakers"> <audioBlockFormat audioBlockFormatID="AB_00010006_00000001"> <speakerLabel>M-110</speakerLabel> <position coordinate="azimuth">-110.0</position> <position coordinate="elevation">0.0</position> <position coordinate="distance">1.0</position> </audioBlockFormat> </audioChannelFormat> <!-- ############ --> <!-- STREAMS --> <!-- ############ --> <audioStreamFormat audioStreamFormatID="AS_00010001" audioStreamFormatName="PCM_FrontLeft" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00010001</audioChannelFormatIDRef> <audioTrackFormatIDRef>AT_00010001_01</audioTrackFormatIDRef> </audioStreamFormat> <audioStreamFormat audioStreamFormatID="AS_00010002" audioStreamFormatName="PCM_FrontRight" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00010002</audioChannelFormatIDRef> <audioTrackFormatIDRef>AT_00010002_01</audioTrackFormatIDRef> </audioStreamFormat> <audioStreamFormat audioStreamFormatID="AS_00020001" audioStreamFormatName="DolbyE_5.1" formatLabel="DolbyE" formatDefinition="DolbyE"> <audioPackFormatIDRef>AP_00010003</audioPackFormatIDRef> <audioTrackFormatIDRef>AT_00020001_01</audioTrackFormatIDRef> <audioTrackFormatIDRef>AT_00020001_02</audioTrackFormatIDRef> </audioStreamFormat> <!-- ############ --> <!-- AUDIO TRACKS --> <!-- ############ --> <audioTrackFormat audioTrackFormatID="AT_00010001_01" audioTrackFormatName="PCM_FrontLeft" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00010001</audioStreamFormatIDRef> </audioTrackFormat> <audioTrackFormat audioTrackFormatID="AT_00010002_01" audioTrackFormatName="PCM_FrontRight" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00010002</audioStreamFormatIDRef> </audioTrackFormat> <audioTrackFormat audioTrackFormatID="AT_00020001_01" audioTrackFormatName="DolbyE1" formatLabel="0002" formatDefinition="data"> <audioStreamFormatIDRef>AS_00020001</audioStreamFormatIDRef> </audioTrackFormat> <audioTrackFormat audioTrackFormatID="AT_00020001_02" audioTrackFormatName="DolbyE2" formatLabel="0002" formatDefinition="data"> <audioStreamFormatIDRef>AS_00020001</audioStreamFormatIDRef> </audioTrackFormat>

The second excerpt (below) covers the content part, in this case audioObjects and audioTrackUIDs, which should be contained within the MXF file. The audioTrackUIDs contain the audioMXFLoopUp elements which locate the essence within the MXF file.

<!-- ############ --> <!-- OBJECTS --> <!-- ############ --> <audioObject audioObjectID="AO_0041" audioObjectName="R123_4a"> <audioObjectIDRef>AO_0002</audioObjectIDRef> <audioObjectIDRef>AO_0004</audioObjectIDRef> </audioObject> <audioObject audioObjectID="AO_0002" audioObjectName="R123_Stereo"> <audioPackFormatIDRef>AP_00010002</audioPackFormatIDRef> <audioTrackUIDRef>ATU_00000001</audioTrackUIDRef> <audioTrackUIDRef>ATU_00000002</audioTrackUIDRef> </audioObject> <audioObject audioObjectID="AO_0004" audioObjectName="R123_5.1coded"> <audioPackFormatIDRef>AP_00010003</audioPackFormatIDRef> <audioTrackUIDRef>ATU_00000003</audioTrackUIDRef> <audioTrackUIDRef>ATU_00000004</audioTrackUIDRef> </audioObject> <!-- ############ --> <!—- AUDIO TRACK UIDs --> <!-- ############ --> <audioTrackUID UID="ATU_00000001"> <audioMXFLookUp> <packageUIDRef>urn:smpte:umid:060a2b34.01010105.01010f20.13000000.540bca53.41434f05.8ce5f4e3.5b72c985</packageUIDRef> <trackIDRef>MXFTRACK_3</trackIDRef> <channelIDRef>MXFCHAN_1</channelIDRef> </audioMXFLookUp> <audioTrackFormatIDRef>AT_00010001_01</audioTrackFormatIDRef> <audioPackFormatIDRef>AP_00010002</audioPackFormatIDRef> </audioTrackUID> <audioTrackUID UID="ATU_00000002"> <audioMXFLookUp> <packageUIDRef>urn:smpte:umid:060a2b34.01010105.01010f20.13000000.540bca53.41434f05.8ce5f4e3.5b72c985</packageUIDRef> <trackIDRef>MXFTRACK_3</trackIDRef> <channelIDRef>MXFCHAN_2</channelIDRef> </audioMXFLookUp> <audioTrackFormatIDRef>AT_00010002_01</audioTrackFormatIDRef> <audioPackFormatIDRef>AP_00010002</audioPackFormatIDRef> </audioTrackUID> <audioTrackUID UID="ATU_00000003"> <audioMXFLookUp> <packageUIDRef>urn:smpte:umid:060a2b34.01010105.01010f20.13000000.540bca53.41434f05.8ce5f4e3.5b72c985</packageUIDRef> <trackIDRef>MXFTRACK_3</trackIDRef> <channelIDRef>MXFCHAN_1</channelIDRef> </audioMXFLookUp> <audioTrackFormatIDRef>AT_00020001_01</audioTrackFormatIDRef> </audioTrackUID> <audioTrackUID UID="ATU_00000004"> <audioMXFLookUp> <packageUIDRef>urn:smpte:umid:060a2b34.01010105.01010f20.13000000.540bca53.41434f05.8ce5f4e3.5b72c985</packageUIDRef> <trackIDRef>MXFTRACK_3</trackIDRef> <channelIDRef>MXFCHAN_1</channelIDRef> </audioMXFLookUp> <audioTrackFormatIDRef>AT_00020001_02</audioTrackFormatIDRef> </audioTrackUID>

## A.5	Personalized Audio Example

To demonstrate how the ADM can be used to describe personalized audio, here is an example using a combination of channel-based audio for the ambience/bed and object-based audio for the commentator objects. This example uses multiple audioProgramme elements that represent five different preset mixes for a sports programme: default mix, just the action, clear commentary, home team, and away team. The corresponding ADM XML tree contains four different audioContent elements to choose from: ambience, main commentary, home team biased commentary, and away team biased commentary.

|  | Ambience | Main commentary 1 | Main commentary 2 | Home team biased commentary | Away team biased commentary |
| --- | --- | --- | --- | --- | --- |
| Default mix |  |  |  |  |  |
| Just the action |  |  |  |  |  |
| Clear commentary |  |  |  |  |  |
| Home team |  |  |  |  |  |
| Away team |  |  |  |  |  |

### A.5.1	Summary of elements

These are the elements in the format part of the description:

| Element | ID | Name | Description |
| --- | --- | --- | --- |
| audioTrackFormat | AT_00010001_01 | PCM_FrontLeft | Defines track as PCM |
| audioStreamFormat | AS_00010001 | PCM_FrontLeft | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00010001 AB_00010001_00000001 | FrontLeft | Describes channel as front left with a position and speaker reference |
| audioTrackFormat | AT_00010002_01 | PCM_FrontRight | Defines track as PCM |
| audioStreamFormat | AS_00010002 | PCM_FrontRight | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00010002 AB_00010002_00000001 | FrontRight | Describes channel as front right with a position and speaker reference |
| audioTrackFormat | AT_00010003_01 | PCM_FrontCentre | Defines track as PCM |
| audioStreamFormat | AS_00010003 | PCM_FrontCentre | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00010003 AB_00010003_00000001 | FrontCentre | Describes channel as front centre with a position and speaker reference |
| audioTrackFormat | AT_00010004_01 | PCM_LFE | Defines track as PCM |
| audioStreamFormat | AS_00010004 | PCM_LFE | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00010004 AB_00010004_00000001 | LFE | Describes channel as LFE with a position and speaker reference |
| audioTrackFormat | AT_00010005_01 | PCM_SurroundLeft | Defines track as PCM |
| audioStreamFormat | AS_00010005 | PCM_SurroundLeft | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00010005 AB_00010005_00000001 | SurroundLeft | Describes channel as surround left with a position and speaker reference |
| audioTrackFormat | AT_00010006_01 | PCM_SurroundRight | Defines track as PCM |
| audioStreamFormat | AS_00010006 | PCM_SurroundRight | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00010006 AB_00010006_00000001 | SurroundRight | Describes channel as surround right with a position and speaker reference |
| audioPackFormat | AP_00010003 | 5.1 | Defines a 5.1 pack referring to six channels. |
| audioTrackFormat | AT_00031001_01 | PCM_Main_Comm1 | Defines track as PCM |
| audioStreamFormat | AS_00031001 | PCM_Main_Comm1 | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00031001 AB_00031001_00000001 | Main_Comm1 | Describes channel as an object type containing a single block with positional metadata. |
| audioTrackFormat | AT_00031002_01 | PCM_Main_Comm2 | Defines track as PCM |
| audioStreamFormat | AS_00031002 | PCM_Main_Comm2 | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00031002 AB_00031002_00000001 | Main_Comm2 | Describes channel as an object type containing a single block with positional metadata. |
| audioTrackFormat | AT_00031003_01 | PCM_Home_Comm | Defines track as PCM |
| audioStreamFormat | AS_00031003 | PCM_Home_Comm | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00031003 AB_00031003_00000001 | Home_Comm | Describes channel as an object type containing a single block with positional metadata. |
| audioTrackFormat | AT_00031004_01 | PCM_Away_Comm | Defines track as PCM |
| audioStreamFormat | AS_00031004 | PCM_Away_Comm | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00031004 AB_00031004_00000001 | Away_Comm | Describes channel as an object type containing a single block with positional metadata. |
| audioPackFormat | AP_00031001 | MainComm1 | Defines a pack referring to one channel. |
| audioPackFormat | AP_00031002 | MainComm2 | Defines a pack referring to one channel. |
| audioPackFormat | AP_00031003 | HomeComm | Defines a pack referring to one channel. |
| audioPackFormat | AP_00031004 | AwayComm | Defines a pack referring to one channel. |

These are the elements in the content part of the description:

| Element | ID | Name | Description |
| --- | --- | --- | --- |
| audioObject | AO_1001 | Ambience | Object for ‘Ambience’, 5.1 format |
| audioContent | ACO_1001 | Ambience | ‘Ambience’ content |
| audioObject | AO_1002 | Main_Comm1 | Object for ‘Main_Comm1’, mono format |
| audioObject | AO_1003 | Main_Comm2 | Object for ‘Main_Comm2’, mono format |
| audioContent | ACO_1002 | Main_Comm | ‘Main_Comm’ content |
| audioObject | AO_1004 | Home_Comm | Object for ‘Home_Comm’, mono format |
| audioContent | ACO_1003 | Home_Comm | ‘Home_Comm’ content |
| audioObject | AO_1005 | Away_Comm | Object for ‘Away_Comm’, mono format |
| audioContent | ACO_1004 | Away_Comm | ‘Away_Comm’ content |
| audioProgramme | APR_1001 | DefaultMix | Programme ‘DefaultMix’ containing ‘Ambience’ and ‘Main_Comm” content |
| audioProgramme | APR_1002 | JustTheAction | Programme ‘JustTheAction’ containing only ‘Ambience’ content |
| audioProgramme | APR_1003 | ClearCommentary | Programme ‘ClearCommentary’ containing only ‘Main_Comm’ content |
| audioProgramme | APG_1004 | HomeTeam | Programme ‘HomeTeam’ containing ‘Ambience’ and ‘Home_Comm’ content |
| audioProgramme | APG_1005 | AwayTeam | Programme ‘AwayTeam’ containing ‘Ambience’ and ‘Away_Comm’ content |

### A.5.2	Diagram

The diagram shows how the defined elements relate to each other. The top half of the diagram covers the elements that describe the 5.1 channel ambience/bed and the 4 mono objects. The <chna> chunk in the middle shows how the tracks are connected to the format definitions. The content definition elements are at the bottom of the diagram, with the audioObject element containing the track UID references to the UID in the <chna> chunk.

### A.5.3	Sample code

This XML sample code does not include the audioFormatExtended parent element and the XML header for clarity. The excerpt of code covers both the format and content elements:

<!-- ############ --> <!-- PROGRAMMES --> <!-- ############ --> <audioProgramme audioProgrammeID="APR_1001" audioProgrammeName="DefaultMix"> <audioContentIDRef>ACO_1001</audioContentIDRef> <audioContentIDRef>ACO_1002</audioContentIDRef> </audioProgramme> <audioProgramme audioProgrammeID="APR_1002" audioProgrammeName="JustTheAction"> <audioContentIDRef>ACO_1001</audioContentIDRef> </audioProgramme> <audioProgramme audioProgrammeID="APR_1003" audioProgrammeName="ClearCommentary"> <audioContentIDRef>ACO_1002</audioContentIDRef> </audioProgramme> <audioProgramme audioProgrammeID="APR_1004" audioProgrammeName="HomeTeam"> <audioContentIDRef>ACO_1001</audioContentIDRef> <audioContentIDRef>ACON_1003</audioContentIDRef> </audioProgramme> <audioProgramme audioProgrammeID="APR_1005" audioProgrammeName="AwayTeam"> <audioContentIDRef>ACO_1001</audioContentIDRef> <audioContentIDRef>ACO_1004</audioContentIDRef> </audioProgramme> <!-- ############ --> <!-- CONTENTS --> <!-- ############ --> <audioContent audioContentID="ACO_1001" audioContentName="Ambience"> <audioObjectIDRef>AO_1001</audioObjectIDRef> <loudnessMetadata> <integratedLoudness>-23.0</integratedLoudness> </loudnessMetadata> </audioContent> <audioContent audioContentID="ACO_1002" audioContentName="Main_Comm"> <audioObjectIDRef>AO_1002</audioObjectIDRef> <audioObjectIDRef>AO_1003</audioObjectIDRef> <loudnessMetadata> <integratedLoudness>-23.0</integratedLoudness> </loudnessMetadata> </audioContent> <audioContent audioContentID="ACO_1003" audioContentName="Home_Comm"> <audioObjectIDRef>AO_1004</audioObjectIDRef> <loudnessMetadata> <integratedLoudness>-23.0</integratedLoudness> </loudnessMetadata> </audioContent> <audioContent audioContentID="ACO_1004" audioContentName="AwayComm"> <audioObjectIDRef>AO_1005</audioObjectIDRef> <loudnessMetadata> <integratedLoudness>-23.0</integratedLoudness> </loudnessMetadata> </audioContent> <!-- ############ --> <!-- OBJECTS --> <!-- ############ --> <audioObject audioObjectID="AO_1001" audioObjectName="Ambience"> <audioPackFormatIDRef>AP_00010003</audioPackFormatIDRef> <audioTrackUIDRef>ATU_00000001</audioTrackUIDRef> <audioTrackUIDRef>ATU_00000002</audioTrackUIDRef> <audioTrackUIDRef>ATU_00000003</audioTrackUIDRef> <audioTrackUIDRef>ATU_00000004</audioTrackUIDRef> <audioTrackUIDRef>ATU_00000005</audioTrackUIDRef> <audioTrackUIDRef>ATU_00000006</audioTrackUIDRef> </audioObject> <audioObject audioObjectID="AO_1002" audioObjectName="Main_Comm1" start="00:00:00.00000"> <audioPackFormatIDRef>AP_00031001</audioPackFormatIDRef> <audioTrackUIDRef>ATU_00000007</audioTrackUIDRef> </audioObject> <audioObject audioObjectID="AO_1003" audioObjectName="Main_Comm2" start="00:00:00.00000"> <audioPackFormatIDRef>AP_00031002</audioPackFormatIDRef> <audioTrackUIDRef>ATU_00000008</audioTrackUIDRef> </audioObject> <audioObject audioObjectID="AO_1004" audioObjectName="Home_Comm" start="00:00:00.00000"> <audioPackFormatIDRef>AP_00031003</audioPackFormatIDRef> <audioTrackUIDRef>ATU_00000009</audioTrackUIDRef> </audioObject> <audioObject audioObjectID="AO_1005" audioObjectName="Away_Comm" start="00:00:00.00000"> <audioPackFormatIDRef>AP_00031004</audioPackFormatIDRef> <audioTrackUIDRef>ATU_0000000a</audioTrackUIDRef> </audioObject> <!-- ############ --> <!-- PACKS --> <!-- ############ --> <audioPackFormat audioPackFormatID="AP_00010003" audioPackFormatName="5.1" typeLabel="0001" typeDefinition="DirectSpeakers"> <audioChannelFormatIDRef>AC_00010001</audioChannelFormatIDRef> <audioChannelFormatIDRef>AC_00010002</audioChannelFormatIDRef> <audioChannelFormatIDRef>AC_00010003</audioChannelFormatIDRef> <audioChannelFormatIDRef>AC_00010004</audioChannelFormatIDRef> <audioChannelFormatIDRef>AC_00010005</audioChannelFormatIDRef> <audioChannelFormatIDRef>AC_00010006</audioChannelFormatIDRef> </audioPackFormat> <audioPackFormat audioPackFormatID="AP_00031001" audioPackFormatName="MainComm1" typeLabel="0003" typeDefinition="Objects"> <audioChannelFormatIDRef>AC_00031001</audioChannelFormatIDRef> </audioPackFormat> <audioPackFormat audioPackFormatID="AP_00031002" audioPackFormatName="MainComm2" typeLabel="0003" typeDefinition="Objects"> <audioChannelFormatIDRef>AC_00031002</audioChannelFormatIDRef> </audioPackFormat> <audioPackFormat audioPackFormatID="AP_00031003" audioPackFormatName="HomeComm" typeLabel="0003" typeDefinition="Objects"> <audioChannelFormatIDRef>AC_00031003</audioChannelFormatIDRef> </audioPackFormat> <audioPackFormat audioPackFormatID="AP_00031004" audioPackFormatName="AwayComm" typeLabel="0003" typeDefinition="Objects"> <audioChannelFormatIDRef>AC_00031004</audioChannelFormatIDRef> </audioPackFormat> <!-- ############ --> <!-- CHANNELS --> <!-- ############ --> <audioChannelFormat audioChannelFormatID="AC_00010001" audioChannelFormatName="FrontLeft" typeLabel="0001" typeDefinition="DirectSpeakers"> <audioBlockFormat audioBlockFormatID="AB_00010001_00000001"> <speakerLabel>M+30</speakerLabel> <position coordinate="azimuth">30.0</position> <position coordinate="elevation">0.0</position> <position coordinate="distance">1.0</position> </audioBlockFormat> </audioChannelFormat> <audioChannelFormat audioChannelFormatID="AC_00010002" audioChannelFormatName="FrontRight" typeLabel="0001" typeDefinition="DirectSpeakers"> <audioBlockFormat audioBlockFormatID="AB_00010002_00000001"> <speakerLabel>M-30</speakerLabel> <position coordinate="azimuth">-30.0</position> <position coordinate="elevation">0.0</position> <position coordinate="distance">1.0</position> </audioBlockFormat> </audioChannelFormat> <audioChannelFormat audioChannelFormatID="AC_00010003" audioChannelFormatName="FrontCentre" typeLabel="0001" typeDefinition="DirectSpeakers"> <audioBlockFormat audioBlockFormatID="AB_00010003_00000001"> <speakerLabel>M+00</speakerLabel> <position coordinate="azimuth">0.0</position> <position coordinate="elevation">0.0</position> <position coordinate="distance">1.0</position> </audioBlockFormat> </audioChannelFormat> <audioChannelFormat audioChannelFormatID="AC_00010004" audioChannelFormatName="LFE" typeLabel="0001" typeDefinition="DirectSpeakers"> <frequency typeDefinition="lowPass">200</frequency> <audioBlockFormat audioBlockFormatID="AB_00010004_00000001"> <speakerLabel>LFE+00</speakerLabel> <position coordinate="azimuth">0.0</position> <position coordinate="elevation">-20.0</position> <position coordinate="distance">1.0</position> </audioBlockFormat> </audioChannelFormat> <audioChannelFormat audioChannelFormatID="AC_00010005" audioChannelFormatName="SurroundLeft" typeLabel="0001" typeDefinition="DirectSpeakers"> <audioBlockFormat audioBlockFormatID="AB_00010005_00000001"> <speakerLabel>M+110</speakerLabel> <position coordinate="azimuth">110.0</position> <position coordinate="elevation">0.0</position> <position coordinate="distance">1.0</position> </audioBlockFormat> </audioChannelFormat> <audioChannelFormat audioChannelFormatID="AC_00010006" audioChannelFormatName="SurroundRight" typeLabel="0001" typeDefinition="DirectSpeakers"> <audioBlockFormat audioBlockFormatID="AB_00010006_00000001"> <speakerLabel>M-110</speakerLabel> <position coordinate="azimuth">-110.0</position> <position coordinate="elevation">0.0</position> <position coordinate="distance">1.0</position> </audioBlockFormat> </audioChannelFormat> <audioChannelFormat audioChannelFormatID="AC_00031001" audioChannelFormatName="MainComm1" typeLabel="0003" typeDefinition="Objects"> <audioBlockFormat audioBlockFormatID="AB_00031001_00000001" rtime="00:00:00.00000" duration="00:05:00.00000"> <position coordinate="X">-1.0</position> <position coordinate="Y">1.0</position> <position coordinate="Z">0.0</position> </audioBlockFormat> </audioChannelFormat> <audioChannelFormat audioChannelFormatID="AC_00031002" audioChannelFormatName="MainComm2" typeLabel="0003" typeDefinition="Objects"> <audioBlockFormat audioBlockFormatID="AB_00031002_00000001" rtime="00:00:00.00000" duration="00:05:00.00000"> <position coordinate="X">1.0</position> <position coordinate="Y">1.0</position> <position coordinate="Z">0.0</position> </audioBlockFormat> </audioChannelFormat> <audioChannelFormat audioChannelFormatID="AC_00031003" audioChannelFormatName="HomeComm" typeLabel="0003" typeDefinition="Objects"> <audioBlockFormat audioBlockFormatID="AB_00031003_00000001" rtime="00:00:00.00000" duration="00:05:00.00000"> <position coordinate="X">0.0</position> <position coordinate="Y">1.0</position> <position coordinate="Z">0.0</position> </audioBlockFormat> </audioChannelFormat> <audioChannelFormat audioChannelFormatID="AC_00031004" audioChannelFormatName="AwayComm" typeLabel="0003" typeDefinition="Objects"> <audioBlockFormat audioBlockFormatID="AB_00031004_00000001" rtime="00:00:00.00000" duration="00:05:00.00000"> <position coordinate="X">0.0</position> <position coordinate="Y">1.0</position> <position coordinate="Z">0.0</position> </audioBlockFormat> </audioChannelFormat> <!-- ############ --> <!-- STREAMS --> <!-- ############ --> <audioStreamFormat audioStreamFormatID="AS_00010001" audioStreamFormatName="PCM_FrontLeft" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00010001</audioChannelFormatIDRef> <audioTrackFormatIDRef>AT_00010001_01</audioTrackFormatIDRef> </audioStreamFormat> <audioStreamFormat audioStreamFormatID="AS_00010002" audioStreamFormatName="PCM_FrontRight" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00010002</audioChannelFormatIDRef> <audioTrackFormatIDRef>AT_00010002_01</audioTrackFormatIDRef> </audioStreamFormat> <audioStreamFormat audioStreamFormatID="AS_00010003" audioStreamFormatName="PCM_FrontCentre" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00010003</audioChannelFormatIDRef> <audioTrackFormatIDRef>AT_00010003_01</audioTrackFormatIDRef> </audioStreamFormat> <audioStreamFormat audioStreamFormatID="AS_00010004" audioStreamFormatName="PCM_LFE" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00010004</audioChannelFormatIDRef> <audioTrackFormatIDRef>AT_00010004_01</audioTrackFormatIDRef> </audioStreamFormat> <audioStreamFormat audioStreamFormatID="AS_00010005" audioStreamFormatName="PCM_SurroundLeft" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00010005</audioChannelFormatIDRef> <audioTrackFormatIDRef>AT_00010005_01</audioTrackFormatIDRef> </audioStreamFormat> <audioStreamFormat audioStreamFormatID="AS_00010006" audioStreamFormatName="PCM_SurroundRight" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00010006</audioChannelFormatIDRef> <audioTrackFormatIDRef>AT_00010006_01</audioTrackFormatIDRef> </audioStreamFormat> <audioStreamFormat audioStreamFormatID="AS_00031001" audioStreamFormatName="PCM_MainComm1" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00031001</audioChannelFormatIDRef> <audioTrackFormatIDRef>AT_00031001_01</audioTrackFormatIDRef> </audioStreamFormat> <audioStreamFormat audioStreamFormatID="AS_00031002" audioStreamFormatName="PCM_MainComm2" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00031002</audioChannelFormatIDRef> <audioTrackFormatIDRef>AT_00031002_01</audioTrackFormatIDRef> </audioStreamFormat> <audioStreamFormat audioStreamFormatID="AS_00031003" audioStreamFormatName="PCM_HomeComm" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00031003</audioChannelFormatIDRef> <audioTrackFormatIDRef>AT_00031003_01</audioTrackFormatIDRef> </audioStreamFormat> <audioStreamFormat audioStreamFormatID="AS_00031004" audioStreamFormatName="PCM_AwayComm" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00031004</audioChannelFormatIDRef> <audioTrackFormatIDRef>AT_00031004_01</audioTrackFormatIDRef> </audioStreamFormat> <!-- ############ --> <!-- AUDIO TRACKS --> <!-- ############ --> <audioTrackFormat audioTrackFormatID="AT_00010001_01" audioTrackFormatName="PCM_FrontLeft" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00010001</audioStreamFormatIDRef> </audioTrackFormat> <audioTrackFormat audioTrackFormatID="AT_00010002_01" audioTrackFormatName="PCM_FrontRight" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00010002</audioStreamFormatIDRef> </audioTrackFormat> <audioTrackFormat audioTrackFormatID="AT_00010003_01" audioTrackFormatName="PCM_FrontCentre" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00010003</audioStreamFormatIDRef> </audioTrackFormat> <audioTrackFormat audioTrackFormatID="AT_00010004_01" audioTrackFormatName="PCM_LFE" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00010004</audioStreamFormatIDRef> </audioTrackFormat> <audioTrackFormat audioTrackFormatID="AT_00010005_01" audioTrackFormatName="PCM_SurroundLeft" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00010005</audioStreamFormatIDRef> </audioTrackFormat> <audioTrackFormat audioTrackFormatID="AT_00010006_01" audioTrackFormatName="PCM_SurroundRight" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00010006</audioStreamFormatIDRef> </audioTrackFormat> <audioTrackFormat audioTrackFormatID="AT_00031001_01" audioTrackFormatName="PCM_MainComm1" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00031001</audioStreamFormatIDRef> </audioTrackFormat> <audioTrackFormat audioTrackFormatID="AT_00031002_01" audioTrackFormatName="PCM_MainComm2" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00031002</audioStreamFormatIDRef> </audioTrackFormat> <audioTrackFormat audioTrackFormatID="AT_00031003_01" audioTrackFormatName="PCM_HomeComm" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00031003</audioStreamFormatIDRef> </audioTrackFormat> <audioTrackFormat audioTrackFormatID="AT_00031004_01" audioTrackFormatName="PCM_AwayComm" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00031004</audioStreamFormatIDRef> </audioTrackFormat> <!-- ############ --> <!—- AUDIO TRACK UIDs --> <!-- ############ --> <audioTrackUID UID="ATU_00000001"> <audioTrackFormatIDRef>AT_00010001_01</audioTrackFormatIDRef> <audioPackFormatIDRef>AP_00010003</audioPackFormatIDRef> </audioTrackUID> <audioTrackUID UID="ATU_00000002"> <audioTrackFormatIDRef>AT_00010002_01</audioTrackFormatIDRef> <audioPackFormatIDRef>AP_00010003</audioPackFormatIDRef> </audioTrackUID> <audioTrackUID UID="ATU_00000003"> <audioTrackFormatIDRef>AT_00010003_01</audioTrackFormatIDRef> <audioPackFormatIDRef>AP_00010003</audioPackFormatIDRef> </audioTrackUID> <audioTrackUID UID="ATU_00000004"> <audioTrackFormatIDRef>AT_00010004_01</audioTrackFormatIDRef> <audioPackFormatIDRef>AP_00010003</audioPackFormatIDRef> </audioTrackUID> <audioTrackUID UID="ATU_00000005"> <audioTrackFormatIDRef>AT_00010005_01</audioTrackFormatIDRef> <audioPackFormatIDRef>AP_00010003</audioPackFormatIDRef> </audioTrackUID> <audioTrackUID UID="ATU_00000006"> <audioTrackFormatIDRef>AT_00010006_01</audioTrackFormatIDRef> <audioPackFormatIDRef>AP_00010003</audioPackFormatIDRef> </audioTrackUID> <audioTrackUID UID="ATU_00000007"> <audioTrackFormatIDRef>AT_00031001_01</audioTrackFormatIDRef> <audioPackFormatIDRef>AP_00031001</audioPackFormatIDRef> </audioTrackUID> <audioTrackUID UID="ATU_00000008"> <audioTrackFormatIDRef>AT_00031002_01</audioTrackFormatIDRef> <audioPackFormatIDRef>AP_00031002</audioPackFormatIDRef> </audioTrackUID> <audioTrackUID UID="ATU_00000009"> <audioTrackFormatIDRef>AT_00031003_01</audioTrackFormatIDRef> <audioPackFormatIDRef>AP_00031003</audioPackFormatIDRef> </audioTrackUID> <audioTrackUID UID="ATU_0000000a"> <audioTrackFormatIDRef>AT_00031004_01</audioTrackFormatIDRef> <audioPackFormatIDRef>AP_00031004</audioPackFormatIDRef> </audioTrackUID>

## A.6	22.2 multichannel programme with an alternative dialogue example

### A.6.1	Summary of elements

These are the elements in the format part of the description:

| Element | ID | Name | Description |
| --- | --- | --- | --- |
| audioTrackFormat | AT_00010001_01 | PCM_FrontLeft | Defines track as PCM |
| audioStreamFormat | AS_00010001 | PCM_FrontLeft | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00010001 AB_00010001_00000001 | FrontLeft | Describes channel as front left with a position and speaker reference |
| audioTrackFormat | AT_00010002_01 | PCM_FrontRight | Defines track as PCM |
| audioStreamFormat | AS_00010002 | PCM_FrontRight | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00010002 AB_00010002_00000001 | FrontRight | Describes channel as front right with a position and speaker reference |
| audioTrackFormat | AT_00010003_01 | PCM_FrontCentre | Defines track as PCM |
| audioStreamFormat | AS_00010003 | PCM_FrontCentre | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00010003 AB_00010003_00000001 | FrontCentre | Describes channel as front centre with a position and speaker reference |
| audioTrackFormat | AT_00010004_01 | PCM_LFE1 | Defines track as PCM |
| audioStreamFormat | AS_00010004 | PCM_LFE1 | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00010004 AB_00010004_00000001 | LFE1 | Describes channel as LFE1 with a position and speaker reference |
| audioTrackFormat | AT_00010007_01 | PCM_BackLeft | Defines track as PCM |
| audioStreamFormat | AS_00010007 | PCM_BackLeft | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00010007 AB_00010007_00000001 | BackLeft | Describes channel as surround left with a position and speaker reference |
| audioTrackFormat | AT_00010008_01 | PCM_BackRight | Defines track as PCM |
| audioStreamFormat | AS_00010008 | PCM_BackRight | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00010008 AB_00010008_00000001 | BackRight | Describes channel as surround right with a position and speaker reference |
| audioTrackFormat | AT_00010008_01 | PCM_FrontLeftCentre | Defines track as PCM |
| audioStreamFormat | AS_00010009 | PCM_FrontLeftCentre | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00010009 AB_00010009_00000001 | FrontLeftCentre | Describes channel as front left centre with a position and speaker reference |
| audioTrackFormat | AT_00010009_01 | PCM_FrontRightCentre | Defines track as PCM |
| audioStreamFormat | AS_0001000a | PCM_FrontRightCentre | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_0001000a AB_0001000a_00000001 | FrontRightCentre | Describes channel as front right centre with a position and speaker reference |
| audioTrackFormat | AT_0001000b_01 | PCM_BackCentre | Defines track as PCM |
| audioStreamFormat | AS_0001000b | PCM_BackCentre | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_0001000b AB_0001000b_00000001 | BackCentre | Describes channel as back centre with a position and speaker reference |
| audioTrackFormat | AT_0001000c_01 | PCM_LFE2 | Defines track as PCM |
| audioStreamFormat | AS_0001000c | PCM_LFE2 | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_0001000c AB_0001000c_00000001 | LFE2 | Describes channel as LFE2 with a position and speaker reference |
| audioTrackFormat | AT_0001000d_01 | PCM_SideLeft | Defines track as PCM |
| audioStreamFormat | AS_0001000d | PCM_SideLeft | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_0001000d AB_0001000d_00000001 | SideLeft | Describes channel as surround right with a position and speaker reference |
| audioTrackFormat | AT_0001000e_01 | PCM_SideRight | Defines track as PCM |
| audioStreamFormat | AS_0001000e | PCM_SideRight | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_0001000e AB_0001000e_00000001 | SideRight | Describes channel as side right with a position and speaker reference |
| audioTrackFormat | AT_0001000f_01 | PCM_TopFrontLeft | Defines track as PCM |
| audioStreamFormat | AS_0001000f | PCM_TopFrontLeft | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_0001000f AB_0001000f_00000001 | TopFrontLeft | Describes channel as top front left with a position and speaker reference |
| audioTrackFormat | AT_00010010_01 | PCM_TopFrontRight | Defines track as PCM |
| audioStreamFormat | AS_00010010 | PCM_TopFrontRight | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00010010 AB_00010010_00000001 | TopFrontRight | Describes channel as top front right with a position and speaker reference |
| audioTrackFormat | AT_00010011_01 | PCM_TopFrontCentre | Defines track as PCM |
| audioStreamFormat | AS_00010011 | PCM_TopFrontCentre | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00010011 AB_00010011_00000001 | TopFrontCentre | Describes channel as top front centre with a position and speaker reference |
| audioTrackFormat | AT_00010012_01 | PCM_TopCentre | Defines track as PCM |
| audioStreamFormat | AS_00010012 | PCM_TopCentre | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00010012 AB_00010012_00000001 | TopCentre | Describes channel as top centre with a position and speaker reference |
| audioTrackFormat | AT_00010013_01 | PCM_TopBackLeft | Defines track as PCM |
| audioStreamFormat | AS_00010013 | PCM_TopBackLeft | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00010013 AB_00010013_00000001 | TopBackLeft | Describes channel as top back left with a position and speaker reference |
| audioTrackFormat | AT_00010014_01 | PCM_TopBackRight | Defines track as PCM |
| audioStreamFormat | AS_00010014 | PCM_TopBackRight | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00010014 AB_00010014_00000001 | TopBackRight | Describes channel as top back right with a position and speaker reference |
| audioTrackFormat | AT_00010015_01 | PCM_TopSideLeft | Defines track as PCM |
| audioStreamFormat | AS_00010015 | PCM_TopSideLeft | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00010015 AB_00010015_00000001 | TopSideLeft | Describes channel as top side left with a position and speaker reference |
| audioTrackFormat | AT_00010016_01 | PCM_TopSideRight | Defines track as PCM |
| audioStreamFormat | AS_00010016 | PCM_TopSideRight | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00010016 AB_00010016_00000001 | TopSideRight | Describes channel as top side right with a position and speaker reference |
| audioTrackFormat | AT_00010017_01 | PCM_TopBackCentre | Defines track as PCM |
| audioStreamFormat | AS_00010017 | PCM_TopBackCentre | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00010017 AB_00010017_00000001 | TopBackCentre | Describes channel as top back centre with a position and speaker reference |
| audioTrackFormat | AT_00010018_01 | PCM_BottomFrontCentre | Defines track as PCM |
| audioStreamFormat | AS_00010018 | PCM_BottomFrontCentre | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00010018 AB_00010018_00000001 | BottomFrontCentre | Describes channel as bottom front centre with a position and speaker reference |
| audioTrackFormat | AT_00010019_01 | PCM_BottomFrontLeft | Defines track as PCM |
| audioStreamFormat | AS_00010019 | PCM_BottomFrontLeft | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00010019 AB_00010019_00000001 | BottomFrontLeft | Describes channel as bottom front left with a position and speaker reference |
| audioTrackFormat | AT_0001001a_01 | PCM_BottomFrontRight | Defines track as PCM |
| audioStreamFormat | AS_0001001a | PCM_BottomFrontRight | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_0001001a AB_0001001a_00000001 | BottomFrontRight | Describes channel as bottom front right with a position and speaker reference |
| audioPackFormat | AP_00010009 | 22.2 | Defines a 22.2 pack referring to 24 channels. |
| audioObject | AO_1001 | MainLanguage | Object for 'MainLanguage', 22.2 format |
| audioObject | AO_1002 | AlternativeLanguage | Object for 'AlternativeLanguage', 22.2 format |
| audioContent | ACN_1001 | MainLanguage | 'MainLanguage' content |
| audioContent | ACN_1002 | AlternativeLanguage | 'AlternativeLanguage' content |
| audioProgramme | APG_1001 | MainLanguage | Programme 'MainLanguage' containing 'MainLanguage' content |
| audioProgramme | APG_1002 | AlternativeLanguage | Programme 'AlternativeLanguage' containing 'AlternativeLanguage' content |

### A.6.2	Diagram

The diagram shows how the defined elements relate to each other. The top half of the diagram covers the elements that describe the 22.2 channel and the one alternative dialogue object. The <chna> chunk in the middle shows how the tracks are connected to the format definitions. The content definition elements are at the bottom of the diagram, with the audioObject element containing the track UID references to the UID in the <chna> chunk.

### A.6.3	Sample code

This XML sample code does not include the audioFormatExtended parent element and the XML header for clarity. The excerpt of code covers both the format and content elements:

<!-- ############ -->

<!-- PROGRAMMES -->

<!-- ############ -->

<audioProgramme audioProgrammeID="APG_1001" audioProgrammeName="Main_Language">

<audioContentIDRef>ACN_1001</audioContentIDRef>

</audioProgramme>

<audioProgramme audioProgrammeID="APG_1002" audioProgrammeName="Alternative_Language">

<audioContentIDRef>ACN_1002</audioContentIDRef>

</audioProgramme>

<!-- ############ -->

<!-- CONTENTS -->

<!-- ############ -->

<audioContent audioContentID="ACN_1001" audioContentName="Main_Language">

<audioObjectIDRef>AO_1001</audioObjectIDRef>

<loudnessMetadata>

<integratedLoudness>-24.0</integratedLoudness>

</loudnessMetadata>

</audioContent>

<audioContent audioContentID="ACN_1002" audioContentName="Alternative_Language">

<audioObjectIDRef>AO_1002</audioObjectIDRef>

<loudnessMetadata>

<integratedLoudness>-24.0</integratedLoudness>

</loudnessMetadata>

</audioContent>

<!-- ############ -->

<!-- OBJECTS -->

<!-- ############ -->

<audioObject audioObjectID="AO_1001" audioObjectName="Main_Language">

<audioPackFormatIDRef>AP_00010009</audioPackFormatIDRef>

<audioTrackUIDRef>ATU_00000001</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000002</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000003</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000004</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000005</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000006</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000007</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000008</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000009</audioTrackUIDRef>

<audioTrackUIDRef>ATU_0000000a</audioTrackUIDRef>

<audioTrackUIDRef>ATU_0000000b</audioTrackUIDRef>

<audioTrackUIDRef>ATU_0000000c</audioTrackUIDRef>

<audioTrackUIDRef>ATU_0000000d</audioTrackUIDRef>

<audioTrackUIDRef>ATU_0000000e</audioTrackUIDRef>

<audioTrackUIDRef>ATU_0000000f</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000010</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000011</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000012</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000013</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000014</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000015</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000016</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000017</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000018</audioTrackUIDRef>

</audioObject>

<audioObject audioObjectID="AO_1002" audioObjectName="Alternative_Language">

<audioPackFormatIDRef>AP_00010009</audioPackFormatIDRef>

<audioTrackUIDRef>ATU_00000001</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000002</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000019</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000004</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000005</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000006</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000007</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000008</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000009</audioTrackUIDRef>

<audioTrackUIDRef>ATU_0000000a</audioTrackUIDRef>

<audioTrackUIDRef>ATU_0000000b</audioTrackUIDRef>

<audioTrackUIDRef>ATU_0000000c</audioTrackUIDRef>

<audioTrackUIDRef>ATU_0000000d</audioTrackUIDRef>

<audioTrackUIDRef>ATU_0000000e</audioTrackUIDRef>

<audioTrackUIDRef>ATU_0000000f</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000010</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000011</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000012</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000013</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000014</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000015</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000016</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000017</audioTrackUIDRef>

<audioTrackUIDRef>ATU_00000018</audioTrackUIDRef>

</audioObject>

<!-- ############ -->

<!-- PACKS -->

<!-- ############ -->

<audioPackFormat audioPackFormatID="AP_00010009" audioPackFormatName="22.2" typeLabel="0001"  typeDefinition="DirectSpeakers">

<audioChannelFormatIDRef>AC_00010001</audioChannelFormatIDRef>

<audioChannelFormatIDRef>AC_00010002</audioChannelFormatIDRef>

<audioChannelFormatIDRef>AC_00010003</audioChannelFormatIDRef>

<audioChannelFormatIDRef>AC_00010004</audioChannelFormatIDRef>

<audioChannelFormatIDRef>AC_00010007</audioChannelFormatIDRef>

<audioChannelFormatIDRef>AC_00010008</audioChannelFormatIDRef>

<audioChannelFormatIDRef>AC_00010009</audioChannelFormatIDRef>

<audioChannelFormatIDRef>AC_0001000a</audioChannelFormatIDRef>

<audioChannelFormatIDRef>AC_0001000b</audioChannelFormatIDRef>

<audioChannelFormatIDRef>AC_0001000c</audioChannelFormatIDRef>

<audioChannelFormatIDRef>AC_0001000d</audioChannelFormatIDRef>

<audioChannelFormatIDRef>AC_0001000e</audioChannelFormatIDRef>

<audioChannelFormatIDRef>AC_0001000f</audioChannelFormatIDRef>

<audioChannelFormatIDRef>AC_00010010</audioChannelFormatIDRef>

<audioChannelFormatIDRef>AC_00010011</audioChannelFormatIDRef>

<audioChannelFormatIDRef>AC_00010012</audioChannelFormatIDRef>

<audioChannelFormatIDRef>AC_00010013</audioChannelFormatIDRef>

<audioChannelFormatIDRef>AC_00010014</audioChannelFormatIDRef>

<audioChannelFormatIDRef>AC_00010015</audioChannelFormatIDRef>

<audioChannelFormatIDRef>AC_00010016</audioChannelFormatIDRef>

<audioChannelFormatIDRef>AC_00010017</audioChannelFormatIDRef>

<audioChannelFormatIDRef>AC_00010018</audioChannelFormatIDRef>

<audioChannelFormatIDRef>AC_00010019</audioChannelFormatIDRef>

<audioChannelFormatIDRef>AC_0001001a</audioChannelFormatIDRef>

</audioPackFormat>

<!-- ############ -->

<!-- CHANNELS -->

<!-- ############ -->

<audioChannelFormat audioChannelFormatID="AC_00010001" audioChannelFormatName="FrontLeft"  typeLabel="0001" typeDefinition="DirectSpeakers">

<audioBlockFormat audioBlockFormatID="AB_00010001_00000001">

<speakerLabel>M+060</speakerLabel>

<position coordinate="azimuth">60.0</position>

<position coordinate="elevation">0.0</position>

<position coordinate="distance">1.0</position>

</audioBlockFormat>

</audioChannelFormat>

<audioChannelFormat audioChannelFormatID="AC_00010002" audioChannelFormatName="FrontRight"  typeLabel="0001" typeDefinition="DirectSpeakers">

<audioBlockFormat audioBlockFormatID="AB_00010002_00000001">

<speakerLabel>M-060</speakerLabel>

<position coordinate="azimuth">-60.0</position>

<position coordinate="elevation">0.0</position>

<position coordinate="distance">1.0</position>

</audioBlockFormat>

</audioChannelFormat>

<audioChannelFormat audioChannelFormatID="AC_00010003" audioChannelFormatName="FrontCentre" typeLabel="0001" typeDefinition="DirectSpeakers">

<audioBlockFormat audioBlockFormatID="AB_00010003_00000001">

<speakerLabel>M+000</speakerLabel>

<position coordinate="azimuth">0.0</position>

<position coordinate="elevation">0.0</position>

<position coordinate="distance">1.0</position>

</audioBlockFormat>

</audioChannelFormat>

<audioChannelFormat audioChannelFormatID="AC_00010004" audioChannelFormatName="LFE1" typeLabel="0001" typeDefinition="DirectSpeakers">

<frequency typeDefinition="lowPass">200</frequency>

<audioBlockFormat audioBlockFormatID="AB_00010004_00000001">

<speakerLabel>LFE+045</speakerLabel>

<position coordinate="azimuth">45.0</position>

<position coordinate="elevation">-30.0</position>

<position coordinate="distance">1.0</position>

</audioBlockFormat>

</audioChannelFormat>

<audioChannelFormat audioChannelFormatID="AC_00010007" audioChannelFormatName="BackLeft"  typeLabel="0001" typeDefinition="DirectSpeakers">

<audioBlockFormat audioBlockFormatID="AB_00010007_00000001">

<speakerLabel>M+135</speakerLabel>

<position coordinate="azimuth">135.0</position>

<position coordinate="elevation">0.0</position>

<position coordinate="distance">1.0</position>

</audioBlockFormat>

</audioChannelFormat>

<audioChannelFormat audioChannelFormatID="AC_00010008" audioChannelFormatName="BackRight"  typeLabel="0001" typeDefinition="DirectSpeakers">

<audioBlockFormat audioBlockFormatID="AB_00010008_00000001">

<speakerLabel>M-135</speakerLabel>

<position coordinate="azimuth">-135.0</position>

<position coordinate="elevation">0.0</position>

<position coordinate="distance">1.0</position>

</audioBlockFormat>

</audioChannelFormat>

<audioChannelFormat audioChannelFormatID="AC_00010009" audioChannelFormatName="FrontLeftCentre"  typeLabel="0001" typeDefinition="DirectSpeakers">

<audioBlockFormat audioBlockFormatID="AB_00010009_00000001">

<speakerLabel>M+030</speakerLabel>

<position coordinate="azimuth">30.0</position>

<position coordinate="elevation">0.0</position>

<position coordinate="distance">1.0</position>

</audioBlockFormat>

</audioChannelFormat>

<audioChannelFormat audioChannelFormatID="AC_0001000a" audioChannelFormatName="FrontRightCentre"  typeLabel="0001" typeDefinition="DirectSpeakers">

<audioBlockFormat audioBlockFormatID="AB_0001000a_00000001">

<speakerLabel>M-030</speakerLabel>

<position coordinate="azimuth">-30.0</position>

<position coordinate="elevation">0.0</position>

<position coordinate="distance">1.0</position>

</audioBlockFormat>

</audioChannelFormat>

<audioChannelFormat audioChannelFormatID="AC_0001000b" audioChannelFormatName="BackCentre" typeLabel="0001" typeDefinition="DirectSpeakers">

<audioBlockFormat audioBlockFormatID="AB_0001000b_00000001">

<speakerLabel>M+180</speakerLabel>

<position coordinate="azimuth">180.0</position>

<position coordinate="elevation">0.0</position>

<position coordinate="distance">1.0</position>

</audioBlockFormat>

</audioChannelFormat>

<audioChannelFormat audioChannelFormatID="AC_0001000c" audioChannelFormatName="LFE1" typeLabel="0001" typeDefinition="DirectSpeakers">

<frequency typeDefinition="lowPass">200</frequency>

<audioBlockFormat audioBlockFormatID="AB_0001000c_00000001">

<speakerLabel>LFE-045</speakerLabel>

<position coordinate="azimuth">-45.0</position>

<position coordinate="elevation">-30.0</position>

<position coordinate="distance">1.0</position>

</audioBlockFormat>

</audioChannelFormat>

<audioChannelFormat audioChannelFormatID="AC_0001000d" audioChannelFormatName="SideLeft"  typeLabel="0001" typeDefinition="DirectSpeakers">

<audioBlockFormat audioBlockFormatID="AB_00010009_00000001">

<speakerLabel>M+090</speakerLabel>

<position coordinate="azimuth">90.0</position>

<position coordinate="elevation">0.0</position>

<position coordinate="distance">1.0</position>

</audioBlockFormat>

</audioChannelFormat>

<audioChannelFormat audioChannelFormatID="AC_0001000e" audioChannelFormatName="SideRight"  typeLabel="0001" typeDefinition="DirectSpeakers">

<audioBlockFormat audioBlockFormatID="AB_0001000a_00000001">

<speakerLabel>M-090</speakerLabel>

<position coordinate="azimuth">-90.0</position>

<position coordinate="elevation">0.0</position>

<position coordinate="distance">1.0</position>

</audioBlockFormat>

</audioChannelFormat>

<audioChannelFormat audioChannelFormatID="AC_0001000f" audioChannelFormatName="TopFrontLeft"  typeLabel="0001" typeDefinition="DirectSpeakers">

<audioBlockFormat audioBlockFormatID="AB_0001000f_00000001">

<speakerLabel>U+045</speakerLabel>

<position coordinate="azimuth">45.0</position>

<position coordinate="elevation">45.0</position>

<position coordinate="distance">1.0</position>

</audioBlockFormat>

</audioChannelFormat>

<audioChannelFormat audioChannelFormatID="AC_00010010" audioChannelFormatName="TopFrontRight"  typeLabel="0001" typeDefinition="DirectSpeakers">

<audioBlockFormat audioBlockFormatID="AB_00010010_00000001">

<speakerLabel>U-045</speakerLabel>

<position coordinate="azimuth">-45.0</position>

<position coordinate="elevation">45.0</position>

<position coordinate="distance">1.0</position>

</audioBlockFormat>

</audioChannelFormat>

<audioChannelFormat audioChannelFormatID="AC_00010011" audioChannelFormatName="TopFrontCentre"  typeLabel="0001" typeDefinition="DirectSpeakers">

<audioBlockFormat audioBlockFormatID="AB_00010011_00000001">

<speakerLabel>U+000</speakerLabel>

<position coordinate="azimuth">0.0</position>

<position coordinate="elevation">45.0</position>

<position coordinate="distance">1.0</position>

</audioBlockFormat>

</audioChannelFormat>

<audioChannelFormat audioChannelFormatID="AC_00010012" audioChannelFormatName="TopCentre"  typeLabel="0001" typeDefinition="DirectSpeakers">

<audioBlockFormat audioBlockFormatID="AB_00010012_00000001">

<speakerLabel>T+000</speakerLabel>

<position coordinate="azimuth">0.0</position>

<position coordinate="elevation">90.0</position>

<position coordinate="distance">1.0</position>

</audioBlockFormat>

</audioChannelFormat>

<audioChannelFormat audioChannelFormatID="AC_00010013" audioChannelFormatName="TopBackLeft"  typeLabel="0001" typeDefinition="DirectSpeakers">

<audioBlockFormat audioBlockFormatID="AB_00010013_00000001">

<speakerLabel>U+135</speakerLabel>

<position coordinate="azimuth">135.0</position>

<position coordinate="elevation">45.0</position>

<position coordinate="distance">1.0</position>

</audioBlockFormat>

</audioChannelFormat>

<audioChannelFormat audioChannelFormatID="AC_00010014" audioChannelFormatName="TopBackRight"  typeLabel="0001" typeDefinition="DirectSpeakers">

<audioBlockFormat audioBlockFormatID="AB_00010014_00000001">

<speakerLabel>U-135</speakerLabel>

<position coordinate="azimuth">-135.0</position>

<position coordinate="elevation">45.0</position>

<position coordinate="distance">1.0</position>

</audioBlockFormat>

</audioChannelFormat>

<audioChannelFormat audioChannelFormatID="AC_00010015" audioChannelFormatName="TopSideLeft"  typeLabel="0001" typeDefinition="DirectSpeakers">

<audioBlockFormat audioBlockFormatID="AB_00010015_00000001">

<speakerLabel>U+090</speakerLabel>

<position coordinate="azimuth">90.0</position>

<position coordinate="elevation">45.0</position>

<position coordinate="distance">1.0</position>

</audioBlockFormat>

</audioChannelFormat>

<audioChannelFormat audioChannelFormatID="AC_00010016" audioChannelFormatName="TopSideRight"  typeLabel="0001" typeDefinition="DirectSpeakers">

<audioBlockFormat audioBlockFormatID="AB_00010016_00000001">

<speakerLabel>U-090</speakerLabel>

<position coordinate="azimuth">-90.0</position>

<position coordinate="elevation">45.0</position>

<position coordinate="distance">1.0</position>

</audioBlockFormat>

</audioChannelFormat>

<audioChannelFormat audioChannelFormatID="AC_00010017" audioChannelFormatName="TopBackCentre"  typeLabel="0001" typeDefinition="DirectSpeakers">

<audioBlockFormat audioBlockFormatID="AB_00010017_00000001">

<speakerLabel>U+180</speakerLabel>

<position coordinate="azimuth">180.0</position>

<position coordinate="elevation">45.0</position>

<position coordinate="distance">1.0</position>

</audioBlockFormat>

</audioChannelFormat>

<audioChannelFormat audioChannelFormatID="AC_00010018" audioChannelFormatName="BottomFrontCentre"  typeLabel="0001" typeDefinition="DirectSpeakers">

<audioBlockFormat audioBlockFormatID="AB_00010018_00000001">

<speakerLabel>B+000</speakerLabel>

<position coordinate="azimuth">0.0</position>

<position coordinate="elevation">-30.0</position>

<position coordinate="distance">1.0</position>

</audioBlockFormat>

</audioChannelFormat>

<audioChannelFormat audioChannelFormatID="AC_00010019" audioChannelFormatName="BottomFrontLeft"  typeLabel="0001" typeDefinition="DirectSpeakers">

<audioBlockFormat audioBlockFormatID="AB_00010019_00000001">

<speakerLabel>B+045</speakerLabel>

<position coordinate="azimuth">45.0</position>

<position coordinate="elevation">-30.0</position>

<position coordinate="distance">1.0</position>

</audioBlockFormat>

</audioChannelFormat>

<audioChannelFormat audioChannelFormatID="AC_0001001a" audioChannelFormatName="BottomFrontRight"  typeLabel="0001" typeDefinition="DirectSpeakers">

<audioBlockFormat audioBlockFormatID="AB_0001001a_00000001">

<speakerLabel>B-045</speakerLabel>

<position coordinate="azimuth">-45.0</position>

<position coordinate="elevation">-30.0</position>

<position coordinate="distance">1.0</position>

</audioBlockFormat>

</audioChannelFormat>

<!-- ############ -->

<!-- STREAMS -->

<!-- ############ -->

<audioStreamFormat audioStreamFormatID="AS_00010001" audioStreamFormatName="PCM_FrontLeft" formatLabel="0001" formatDefinition="PCM">

<audioChannelFormatIDRef>AC_00010001</audioChannelFormatIDRef>

<audioTrackFormatIDRef>AT_00010001_01</audioTrackFormatIDRef>

</audioStreamFormat>

<audioStreamFormat audioStreamFormatID="AS_00010002" audioStreamFormatName="PCM_FrontRight" formatLabel="0001" formatDefinition="PCM">

<audioChannelFormatIDRef>AC_00010002</audioChannelFormatIDRef>

<audioTrackFormatIDRef>AT_00010002_01</audioTrackFormatIDRef>

</audioStreamFormat>

<audioStreamFormat audioStreamFormatID="AS_00010003" audioStreamFormatName="PCM_FrontCentre" formatLabel="0001" formatDefinition="PCM">

<audioChannelFormatIDRef>AC_00010003</audioChannelFormatIDRef>

<audioTrackFormatIDRef>AT_00010003_01</audioTrackFormatIDRef>

</audioStreamFormat>

<audioStreamFormat audioStreamFormatID="AS_00010004" audioStreamFormatName="PCM_LFE1" formatLabel="0001" formatDefinition="PCM">

<audioChannelFormatIDRef>AC_00010004</audioChannelFormatIDRef>

<audioTrackFormatIDRef>AT_00010004_01</audioTrackFormatIDRef>

</audioStreamFormat>

<audioStreamFormat audioStreamFormatID="AS_00010007" audioStreamFormatName="PCM_BackLeft" formatLabel="0001" formatDefinition="PCM">

<audioChannelFormatIDRef>AC_00010007</audioChannelFormatIDRef>

<audioTrackFormatIDRef>AT_00010007_01</audioTrackFormatIDRef>

</audioStreamFormat>

<audioStreamFormat audioStreamFormatID="AS_00010008" audioStreamFormatName="PCM_BackRight" formatLabel="0001" formatDefinition="PCM">

<audioChannelFormatIDRef>AC_00010008</audioChannelFormatIDRef>

<audioTrackFormatIDRef>AT_00010008_01</audioTrackFormatIDRef>

</audioStreamFormat>

<audioStreamFormat audioStreamFormatID="AS_00010009" audioStreamFormatName="PCM_FrontLeftCentre" formatLabel="0001" formatDefinition="PCM">

<audioChannelFormatIDRef>AC_00010009</audioChannelFormatIDRef>

<audioTrackFormatIDRef>AT_00010009_01</audioTrackFormatIDRef>

</audioStreamFormat>

<audioStreamFormat audioStreamFormatID="AS_0001000a" audioStreamFormatName="PCM_FrontRightCentre" formatLabel="0001" formatDefinition="PCM">

<audioChannelFormatIDRef>AC_0001000a</audioChannelFormatIDRef>

<audioTrackFormatIDRef>AT_0001000a_01</audioTrackFormatIDRef>

</audioStreamFormat>

<audioStreamFormat audioStreamFormatID="AS_0001000b" audioStreamFormatName="PCM_BackCentre" formatLabel="0001" formatDefinition="PCM">

<audioChannelFormatIDRef>AC_0001000b</audioChannelFormatIDRef>

<audioTrackFormatIDRef>AT_0001000b_01</audioTrackFormatIDRef>

</audioStreamFormat>

<audioStreamFormat audioStreamFormatID="AS_0001000c" audioStreamFormatName="PCM_LFE2" formatLabel="0001" formatDefinition="PCM">

<audioChannelFormatIDRef>AC_0001000c</audioChannelFormatIDRef>

<audioTrackFormatIDRef>AT_0001000c_01</audioTrackFormatIDRef>

</audioStreamFormat>

<audioStreamFormat audioStreamFormatID="AS_0001000d" audioStreamFormatName="PCM_SideLeft" formatLabel="0001" formatDefinition="PCM">

<audioChannelFormatIDRef>AC_0001000d</audioChannelFormatIDRef>

<audioTrackFormatIDRef>AT_0001000d_01</audioTrackFormatIDRef>

</audioStreamFormat>

<audioStreamFormat audioStreamFormatID="AS_0001000e" audioStreamFormatName="PCM_SideRight" formatLabel="0001" formatDefinition="PCM">

<audioChannelFormatIDRef>AC_0001000e</audioChannelFormatIDRef>

<audioTrackFormatIDRef>AT_0001000e_01</audioTrackFormatIDRef>

</audioStreamFormat>

<audioStreamFormat audioStreamFormatID="AS_0001000f" audioStreamFormatName="PCM_TopFrontLeft" formatLabel="0001" formatDefinition="PCM">

<audioChannelFormatIDRef>AC_0001000f</audioChannelFormatIDRef>

<audioTrackFormatIDRef>AT_0001000f_01</audioTrackFormatIDRef>

</audioStreamFormat>

<audioStreamFormat audioStreamFormatID="AS_00010010" audioStreamFormatName="PCM_TopFrontRight" formatLabel="0001" formatDefinition="PCM">

<audioChannelFormatIDRef>AC_00010010</audioChannelFormatIDRef>

<audioTrackFormatIDRef>AT_00010010_01</audioTrackFormatIDRef>

</audioStreamFormat>

<audioStreamFormat audioStreamFormatID="AS_00010011" audioStreamFormatName="PCM_TopFrontCentre" formatLabel="0001" formatDefinition="PCM">

<audioChannelFormatIDRef>AC_00010011</audioChannelFormatIDRef>

<audioTrackFormatIDRef>AT_00010011_01</audioTrackFormatIDRef>

</audioStreamFormat>

<audioStreamFormat audioStreamFormatID="AS_00010012" audioStreamFormatName="PCM_TopCentre" formatLabel="0001" formatDefinition="PCM">

<audioChannelFormatIDRef>AC_00010012</audioChannelFormatIDRef>

<audioTrackFormatIDRef>AT_00010012_01</audioTrackFormatIDRef>

</audioStreamFormat>

<audioStreamFormat audioStreamFormatID="AS_00010013" audioStreamFormatName="PCM_TopBackLeft" formatLabel="0001" formatDefinition="PCM">

<audioChannelFormatIDRef>AC_00010013</audioChannelFormatIDRef>

<audioTrackFormatIDRef>AT_00010013_01</audioTrackFormatIDRef>

</audioStreamFormat>

<audioStreamFormat audioStreamFormatID="AS_00010014" audioStreamFormatName="PCM_TopBackRight" formatLabel="0001" formatDefinition="PCM">

<audioChannelFormatIDRef>AC_00010014</audioChannelFormatIDRef>

<audioTrackFormatIDRef>AT_00010014_01</audioTrackFormatIDRef>

</audioStreamFormat>

<audioStreamFormat audioStreamFormatID="AS_00010015" audioStreamFormatName="PCM_TopSideLeft" formatLabel="0001" formatDefinition="PCM">

<audioChannelFormatIDRef>AC_00010015</audioChannelFormatIDRef>

<audioTrackFormatIDRef>AT_00010015_01</audioTrackFormatIDRef>

</audioStreamFormat>

<audioStreamFormat audioStreamFormatID="AS_00010016" audioStreamFormatName="PCM_TopSideRight" formatLabel="0001" formatDefinition="PCM">

<audioChannelFormatIDRef>AC_00010016</audioChannelFormatIDRef>

<audioTrackFormatIDRef>AT_00010016_01</audioTrackFormatIDRef>

</audioStreamFormat>

<audioStreamFormat audioStreamFormatID="AS_00010017" audioStreamFormatName="PCM_TopBackCentre" formatLabel="0001" formatDefinition="PCM">

<audioChannelFormatIDRef>AC_00010017</audioChannelFormatIDRef>

<audioTrackFormatIDRef>AT_00010017_01</audioTrackFormatIDRef>

</audioStreamFormat>

<audioStreamFormat audioStreamFormatID="AS_00010018" audioStreamFormatName="PCM_BottomFrontCentre" formatLabel="0001" formatDefinition="PCM">

<audioChannelFormatIDRef>AC_00010018</audioChannelFormatIDRef>

<audioTrackFormatIDRef>AT_00010018_01</audioTrackFormatIDRef>

</audioStreamFormat>

<audioStreamFormat audioStreamFormatID="AS_00010019" audioStreamFormatName="PCM_BottomFrontLeft" formatLabel="0001" formatDefinition="PCM">

<audioChannelFormatIDRef>AC_00010019</audioChannelFormatIDRef>

<audioTrackFormatIDRef>AT_00010019_01</audioTrackFormatIDRef>

</audioStreamFormat>

<audioStreamFormat audioStreamFormatID="AS_0001001a" audioStreamFormatName="PCM_BottomFrontRight" formatLabel="0001" formatDefinition="PCM">

<audioChannelFormatIDRef>AC_0001001a</audioChannelFormatIDRef>

<audioTrackFormatIDRef>AT_0001001a_01</audioTrackFormatIDRef>

</audioStreamFormat>

<!-- ############ -->

<!-- AUDIO TRACKS -->

<!-- ############ -->

<audioTrackFormat audioTrackFormatID="AT_00010001_01" audioTrackFormatName="PCM_FrontLeft"  formatLabel="0001" formatDefinition="PCM">

<audioStreamFormatIDRef>AS_00010001</audioStreamFormatIDRef>

</audioTrackFormat>

<audioTrackFormat audioTrackFormatID="AT_00010002_01" audioTrackFormatName="PCM_FrontRight"  formatLabel="0001" formatDefinition="PCM">

<audioStreamFormatIDRef>AS_00010002</audioStreamFormatIDRef>

</audioTrackFormat>

<audioTrackFormat audioTrackFormatID="AT_00010003_01" audioTrackFormatName="PCM_FrontCentre"  formatLabel="0001" formatDefinition="PCM">

<audioStreamFormatIDRef>AS_00010003</audioStreamFormatIDRef>

</audioTrackFormat>

<audioTrackFormat audioTrackFormatID="AT_00010004_01" audioTrackFormatName="PCM_LFE1"  formatLabel="0001" formatDefinition="PCM">

<audioStreamFormatIDRef>AS_00010004</audioStreamFormatIDRef>

</audioTrackFormat>

<audioTrackFormat audioTrackFormatID="AT_00010007_01" audioTrackFormatName="PCM_BackLeft"  formatLabel="0001" formatDefinition="PCM">

<audioStreamFormatIDRef>AS_00010007</audioStreamFormatIDRef>

</audioTrackFormat>

<audioTrackFormat audioTrackFormatID="AT_00010008_01" audioTrackFormatName="PCM_BackRight"  formatLabel="0001" formatDefinition="PCM">

<audioStreamFormatIDRef>AS_00010008</audioStreamFormatIDRef>

</audioTrackFormat>

<audioTrackFormat audioTrackFormatID="AT_00010009_01" audioTrackFormatName="PCM_FrontLeftCentre"  formatLabel="0001" formatDefinition="PCM">

<audioStreamFormatIDRef>AS_00010009</audioStreamFormatIDRef>

</audioTrackFormat>

<audioTrackFormat audioTrackFormatID="AT_0001000a_01" audioTrackFormatName="PCM_FrontRightCentre"  formatLabel="0001" formatDefinition="PCM">

<audioStreamFormatIDRef>AS_0001000a</audioStreamFormatIDRef>

</audioTrackFormat>

<audioTrackFormat audioTrackFormatID="AT_0001000b_01" audioTrackFormatName="PCM_BackCentre"  formatLabel="0001" formatDefinition="PCM">

<audioStreamFormatIDRef>AS_0001000b</audioStreamFormatIDRef>

</audioTrackFormat>

<audioTrackFormat audioTrackFormatID="AT_0001000c_01" audioTrackFormatName="PCM_LFE2"  formatLabel="0001" formatDefinition="PCM">

<audioStreamFormatIDRef>AS_0001000c</audioStreamFormatIDRef>

</audioTrackFormat>

<audioTrackFormat audioTrackFormatID="AT_0001000d_01" audioTrackFormatName="PCM_SideLeft"  formatLabel="0001" formatDefinition="PCM">

<audioStreamFormatIDRef>AS_0001000d</audioStreamFormatIDRef>

</audioTrackFormat>

<audioTrackFormat audioTrackFormatID="AT_0001000e_01" audioTrackFormatName="PCM_SideRight"  formatLabel="0001" formatDefinition="PCM">

<audioStreamFormatIDRef>AS_0001000e</audioStreamFormatIDRef>

</audioTrackFormat>

<audioTrackFormat audioTrackFormatID="AT_0001000f_01" audioTrackFormatName="PCM_TopFrontLeft"  formatLabel="0001" formatDefinition="PCM">

<audioStreamFormatIDRef>AS_0001000f</audioStreamFormatIDRef>

</audioTrackFormat>

<audioTrackFormat audioTrackFormatID="AT_00010010_01" audioTrackFormatName="PCM_TopFrontRight"  formatLabel="0001" formatDefinition="PCM">

<audioStreamFormatIDRef>AS_00010010</audioStreamFormatIDRef>

</audioTrackFormat>

<audioTrackFormat audioTrackFormatID="AT_00010011_01" audioTrackFormatName="PCM_TopFrontCentre"  formatLabel="0001" formatDefinition="PCM">

<audioStreamFormatIDRef>AS_00010011</audioStreamFormatIDRef>

</audioTrackFormat>

<audioTrackFormat audioTrackFormatID="AT_00010012_01" audioTrackFormatName="PCM_TopCentre"  formatLabel="0001" formatDefinition="PCM">

<audioStreamFormatIDRef>AS_00010012</audioStreamFormatIDRef>

</audioTrackFormat>

<audioTrackFormat audioTrackFormatID="AT_00010013_01" audioTrackFormatName="PCM_TopBackLeft"  formatLabel="0001" formatDefinition="PCM">

<audioStreamFormatIDRef>AS_00010013</audioStreamFormatIDRef>

</audioTrackFormat>

<audioTrackFormat audioTrackFormatID="AT_00010014_01" audioTrackFormatName="PCM_TopBackRight"  formatLabel="0001" formatDefinition="PCM">

<audioStreamFormatIDRef>AS_00010014</audioStreamFormatIDRef>

</audioTrackFormat>

<audioTrackFormat audioTrackFormatID="AT_00010015_01" audioTrackFormatName="PCM_TopSideLeft"  formatLabel="0001" formatDefinition="PCM">

<audioStreamFormatIDRef>AS_00010015</audioStreamFormatIDRef>

</audioTrackFormat>

<audioTrackFormat audioTrackFormatID="AT_00010016_01" audioTrackFormatName="PCM_TopSideRight"  formatLabel="0001" formatDefinition="PCM">

<audioStreamFormatIDRef>AS_00010016</audioStreamFormatIDRef>

</audioTrackFormat>

<audioTrackFormat audioTrackFormatID="AT_00010017_01" audioTrackFormatName="PCM_TopBackCentre"  formatLabel="0001" formatDefinition="PCM">

<audioStreamFormatIDRef>AS_00010017</audioStreamFormatIDRef>

</audioTrackFormat>

<audioTrackFormat audioTrackFormatID="AT_00010018_01" audioTrackFormatName="PCM_BottomFrontCentre"  formatLabel="0001" formatDefinition="PCM">

<audioStreamFormatIDRef>AS_00010018</audioStreamFormatIDRef>

</audioTrackFormat>

<audioTrackFormat audioTrackFormatID="AT_00010019_01" audioTrackFormatName="PCM_BottomFrontLeft"  formatLabel="0001" formatDefinition="PCM">

<audioStreamFormatIDRef>AS_00010019</audioStreamFormatIDRef>

</audioTrackFormat>

<audioTrackFormat audioTrackFormatID="AT_0001001a_01" audioTrackFormatName="PCM_BottomFrontRight"  formatLabel="0001" formatDefinition="PCM">

<audioStreamFormatIDRef>AS_0001001a</audioStreamFormatIDRef>

</audioTrackFormat>

<!-- ############ -->

<!-- AUDIO TRACK UIDs -->

<!-- ############ -->

<audioTrackUID UID="ATU_00000001">

<audioTrackFormatIDRef>AT_00010001_01</audioTrackFormatIDRef>

<audioPackFormatIDRef>AP_00010009</audioPackFormatIDRef>

</audioTrackUID>

<audioTrackUID UID="ATU_00000002">

<audioTrackFormatIDRef>AT_00010002_01</audioTrackFormatIDRef>

<audioPackFormatIDRef>AP_00010009</audioPackFormatIDRef>

</audioTrackUID>

<audioTrackUID UID="ATU_00000003">

<audioTrackFormatIDRef>AT_00010003_01</audioTrackFormatIDRef>

<audioPackFormatIDRef>AP_00010009</audioPackFormatIDRef>

</audioTrackUID>

<audioTrackUID UID="ATU_00000004">

<audioTrackFormatIDRef>AT_00010004_01</audioTrackFormatIDRef>

<audioPackFormatIDRef>AP_00010009</audioPackFormatIDRef>

</audioTrackUID>

<audioTrackUID UID="ATU_00000005">

<audioTrackFormatIDRef>AT_00010007_01</audioTrackFormatIDRef>

<audioPackFormatIDRef>AP_00010009</audioPackFormatIDRef>

</audioTrackUID>

<audioTrackUID UID="ATU_00000006">

<audioTrackFormatIDRef>AT_00010008_01</audioTrackFormatIDRef>

<audioPackFormatIDRef>AP_00010009</audioPackFormatIDRef>

</audioTrackUID>

<audioTrackUID UID="ATU_00000007">

<audioTrackFormatIDRef>AT_00010009_01</audioTrackFormatIDRef>

<audioPackFormatIDRef>AP_00010009</audioPackFormatIDRef>

</audioTrackUID>

<audioTrackUID UID="ATU_00000008">

<audioTrackFormatIDRef>AT_0001000a_01</audioTrackFormatIDRef>

<audioPackFormatIDRef>AP_00010009</audioPackFormatIDRef>

</audioTrackUID>

<audioTrackUID UID="ATU_00000009">

<audioTrackFormatIDRef>AT_0001000b_01</audioTrackFormatIDRef>

<audioPackFormatIDRef>AP_00010009</audioPackFormatIDRef>

</audioTrackUID>

<audioTrackUID UID="ATU_0000000a">

<audioTrackFormatIDRef>AT_0001000c_01</audioTrackFormatIDRef>

<audioPackFormatIDRef>AP_00010009</audioPackFormatIDRef>

</audioTrackUID>

<audioTrackUID UID="ATU_0000000b">

<audioTrackFormatIDRef>AT_0001000d_01</audioTrackFormatIDRef>

<audioPackFormatIDRef>AP_00010009</audioPackFormatIDRef>

</audioTrackUID>

<audioTrackUID UID="ATU_0000000c">

<audioTrackFormatIDRef>AT_0001000e_01</audioTrackFormatIDRef>

<audioPackFormatIDRef>AP_00010009</audioPackFormatIDRef>

</audioTrackUID>

<audioTrackUID UID="ATU_0000000d">

<audioTrackFormatIDRef>AT_0001000f_01</audioTrackFormatIDRef>

<audioPackFormatIDRef>AP_00010009</audioPackFormatIDRef>

</audioTrackUID>

<audioTrackUID UID="ATU_0000000e">

<audioTrackFormatIDRef>AT_00010010_01</audioTrackFormatIDRef>

<audioPackFormatIDRef>AP_00010009</audioPackFormatIDRef>

</audioTrackUID>

<audioTrackUID UID="ATU_0000000f">

<audioTrackFormatIDRef>AT_00010011_01</audioTrackFormatIDRef>

<audioPackFormatIDRef>AP_00010009</audioPackFormatIDRef>

</audioTrackUID>

<audioTrackUID UID="ATU_00000010">

<audioTrackFormatIDRef>AT_00010012_01</audioTrackFormatIDRef>

<audioPackFormatIDRef>AP_00010009</audioPackFormatIDRef>

</audioTrackUID>

<audioTrackUID UID="ATU_00000011">

<audioTrackFormatIDRef>AT_00010013_01</audioTrackFormatIDRef>

<audioPackFormatIDRef>AP_00010009</audioPackFormatIDRef>

</audioTrackUID>

<audioTrackUID UID="ATU_00000012">

<audioTrackFormatIDRef>AT_00010014_01</audioTrackFormatIDRef>

<audioPackFormatIDRef>AP_00010009</audioPackFormatIDRef>

</audioTrackUID>

<audioTrackUID UID="ATU_00000013">

<audioTrackFormatIDRef>AT_00010015_01</audioTrackFormatIDRef>

<audioPackFormatIDRef>AP_00010009</audioPackFormatIDRef>

</audioTrackUID>

<audioTrackUID UID="ATU_00000014">

<audioTrackFormatIDRef>AT_00010016_01</audioTrackFormatIDRef>

<audioPackFormatIDRef>AP_00010009</audioPackFormatIDRef>

</audioTrackUID>

<audioTrackUID UID="ATU_00000015">

<audioTrackFormatIDRef>AT_00010017_01</audioTrackFormatIDRef>

<audioPackFormatIDRef>AP_00010009</audioPackFormatIDRef>

</audioTrackUID>

<audioTrackUID UID="ATU_00000016">

<audioTrackFormatIDRef>AT_00010018_01</audioTrackFormatIDRef>

<audioPackFormatIDRef>AP_00010009</audioPackFormatIDRef>

</audioTrackUID>

<audioTrackUID UID="ATU_00000017">

<audioTrackFormatIDRef>AT_00010019_01</audioTrackFormatIDRef>

<audioPackFormatIDRef>AP_00010009</audioPackFormatIDRef>

</audioTrackUID>

<audioTrackUID UID="ATU_00000018">

<audioTrackFormatIDRef>AT_0001001a_01</audioTrackFormatIDRef>

<audioPackFormatIDRef>AP_00010009</audioPackFormatIDRef>

</audioTrackUID>

<audioTrackUID UID="ATU_00000019">

<audioTrackFormatIDRef>AT_00010003_01</audioTrackFormatIDRef>

<audioPackFormatIDRef>AP_00010009</audioPackFormatIDRef>

</audioTrackUID>

## A.7	Example of the use of the Matrix type

### A.7.1	Summary of elements

These are the elements in the format part of the description:

| Element | ID | Name | Description |
| --- | --- | --- | --- |
| audioTrackFormat | AT_00010001_01 | PCM_FrontLeft | Defines track as PCM |
| audioTrackFormat | AT_00010002_01 | PCM_FrontRight | Defines track as PCM |
| audioTrackFormat | AT_00010003_01 | PCM_Centre | Defines track as PCM |
| audioTrackFormat | AT_00010004_01 | PCM_LFE | Defines track as PCM |
| audioTrackFormat | AT_00010005_01 | PCM_SurroundLeft | Defines track as PCM |
| audioTrackFormat | AT_00010006_01 | PCM_SurroundRight | Defines track as PCM |
| audioTrackFormat | AT_00020003_01 | PCM_Lo | Defines track as PCM |
| audioTrackFormat | AT_00020004_01 | PCM_Ro | Defines track as PCM |
| audioStreamFormat | AS_00010001 | PCM_FrontLeft | Defines stream as PCM |
| audioStreamFormat | AS_00010002 | PCM_FrontRight | Defines stream as PCM |
| audioStreamFormat | AS_00010003 | PCM_Centre | Defines stream as PCM |
| audioStreamFormat | AS_00010004 | PCM_LFE | Defines stream as PCM |
| audioStreamFormat | AS_00010005 | PCM_SurroundLeft | Defines stream as PCM |
| audioStreamFormat | AS_00010006 | PCM_SurroundRight | Defines stream as PCM |
| audioStreamFormat | AS_00020003 | PCM_Lo | Defines stream as PCM |
| audioStreamFormat | AS_00020004 | PCM_Ro | Defines stream as PCM |
| audioChannelFormat & audioBlockFormat | AC_00010001 AB_00010001_00000001 | FrontLeft | Describes channel as front left with a position and speaker reference |
| audioChannelFormat & audioBlockFormat | AC_00010002 AB_00010002_00000001 | FrontRight | Describes channel as front right with a position and speaker reference |
| audioChannelFormat & audioBlockFormat | AC_00010003 AB_00010003_00000001 | FrontCentre | Describes channel as front centre with a position and speaker reference |
| audioChannelFormat & audioBlockFormat | AC_00010004 AB_00010004_00000001 | LFE | Describes channel as LFE with a position and speaker reference |
| audioChannelFormat & audioBlockFormat | AC_00010005 AB_00010005_00000001 | SurroundLeft | Describes channel as surround left with a position and speaker reference |
| audioChannelFormat & audioBlockFormat | AC_00010006 AB_00010006_00000001 | SurroundRight | Describes channel as surround right with a position and speaker reference |
| audioChannelFormat & audioBlockFormat | AC_00020003 AB_00020003_00000001 | Lo | Describes channel as Lo with a matrix combination of FL, C and SL channels |
| audioChannelFormat & audioBlockFormat | AC_00020004 AB_00020004_00000001 | Ro | Describes channel as Ro with a matrix combination of FR, C and SR channels |
| audioPackFormat | AP_00020002 | LoRo | Defines a LoRo pack referring to two matrix channels. |
| audioPackFormat | AP_00010003 | 5.1 | Defines a 5.1 pack referring to 6 channels. |

These are the elements in the content part of the description:

| Element | ID | Name | Description |
| --- | --- | --- | --- |
| audioObject | AO_1001 | 5.1 | Object for 5.1 mix |
| audioObject | AO_1002 | 5.1_LoRo_Downmix | Object for 5.1 to Lo/Ro down-mix |

### A.7.2	Diagram

The diagram shows how the defined elements relate to each other. The top half of the diagram covers the elements that the 5.1 channels and the Lo/Ro down-mix channel. The <chna> chunk in the middle shows how the tracks are connected to the format definitions. The content definition elements are at the bottom of the diagram, with the audioObject element containing the track UID references to the UID in the <chna> chunk.

### A.7.3	Sample code

This XML sample code does not include the audioFormatExtended parent element and the XML header for clarity. The first excerpt of code covers the format elements, which could be contained within the standard reference file:

<!-- ############ --> <!-- OBJECTS --> <!-- ############ --> <audioObject audioObjectID="AO_1001" audioObjectName="5.1"> <audioPackFormatIDRef>AP_00010003</audioPackFormatIDRef> <audioTrackUIDRef>ATU_00000001</audioTrackUIDRef> <audioTrackUIDRef>ATU_00000002</audioTrackUIDRef> <audioTrackUIDRef>ATU_00000003</audioTrackUIDRef> <audioTrackUIDRef>ATU_00000004</audioTrackUIDRef> <audioTrackUIDRef>ATU_00000005</audioTrackUIDRef> <audioTrackUIDRef>ATU_00000006</audioTrackUIDRef> </audioObject> <audioObject audioObjectID="AO_1002" audioObjectName="5.1_LoRo_Downmix"> <audioPackFormatIDRef>AP_00020002</audioPackFormatIDRef> <audioTrackUIDRef>ATU_00000007</audioTrackUIDRef> <audioTrackUIDRef>ATU_00000008</audioTrackUIDRef> </audioObject> <!-- ############ --> <!-- PACKS --> <!-- ############ --> <audioPackFormat audioPackFormatID="AP_00010003" audioPackFormatName="5.1" typeLabel="0001" typeDefinition="DirectSpeakers"> <audioChannelFormatIDRef>AC_00010001</audioChannelFormatIDRef> <audioChannelFormatIDRef>AC_00010002</audioChannelFormatIDRef> <audioChannelFormatIDRef>AC_00010003</audioChannelFormatIDRef> <audioChannelFormatIDRef>AC_00010004</audioChannelFormatIDRef> <audioChannelFormatIDRef>AC_00010005</audioChannelFormatIDRef> <audioChannelFormatIDRef>AC_00010006</audioChannelFormatIDRef> </audioPackFormat> <audioPackFormat audioPackFormatID="AP_00020002" audioPackFormatName="LoRo" typeLabel="0002" typeDefinition="Matrix"> <audioChannelFormatIDRef>AC_00020003</audioChannelFormatIDRef> <audioChannelFormatIDRef>AC_00020004</audioChannelFormatIDRef> </audioPackFormat> <!-- ############ --> <!-- CHANNELS --> <!-- ############ --> <audioChannelFormat audioChannelFormatID="AC_00010001" audioChannelFormatName="FrontLeft" typeLabel="0001" typeDefinition="DirectSpeakers"> <audioBlockFormat audioBlockFormatID="AB_00010001_00000001"> <speakerLabel>M+30</speakerLabel> <position coordinate="azimuth">30.0</position> <position coordinate="elevation">0.0</position> <position coordinate="distance">1.0</position> </audioBlockFormat> </audioChannelFormat> <audioChannelFormat audioChannelFormatID="AC_00010002" audioChannelFormatName="FrontRight" typeLabel="0001" typeDefinition="DirectSpeakers"> <audioBlockFormat audioBlockFormatID="AB_00010002_00000001"> <speakerLabel>M-30</speakerLabel> <position coordinate="azimuth">-30.0</position> <position coordinate="elevation">0.0</position> <position coordinate="distance">1.0</position> </audioBlockFormat> </audioChannelFormat> <audioChannelFormat audioChannelFormatID="AC_00010003" audioChannelFormatName="FrontCentre" typeLabel="0001" typeDefinition="DirectSpeakers"> <audioBlockFormat audioBlockFormatID="AB_00010003_00000001"> <speakerLabel>M+00</speakerLabel> <position coordinate="azimuth">0.0</position> <position coordinate="elevation">0.0</position> <position coordinate="distance">1.0</position> </audioBlockFormat> </audioChannelFormat> <audioChannelFormat audioChannelFormatID="AC_00010004" audioChannelFormatName="LFE" typeLabel="0001" typeDefinition="DirectSpeakers"> <frequency typeDefinition="lowPass">200</frequency> <audioBlockFormat audioBlockFormatID="AB_00010004_00000001"> <speakerLabel>LFE+00</speakerLabel> <position coordinate="azimuth">0.0</position> <position coordinate="elevation">-20.0</position> <position coordinate="distance">1.0</position> </audioBlockFormat> </audioChannelFormat> <audioChannelFormat audioChannelFormatID="AC_00010005" audioChannelFormatName="SurroundLeft" typeLabel="0001" typeDefinition="DirectSpeakers"> <audioBlockFormat audioBlockFormatID="AB_00010005_00000001"> <speakerLabel>M+110</speakerLabel> <position coordinate="azimuth">110.0</position> <position coordinate="elevation">0.0</position> <position coordinate="distance">1.0</position> </audioBlockFormat> </audioChannelFormat> <audioChannelFormat audioChannelFormatID="AC_00010006" audioChannelFormatName="SurroundRight" typeLabel="0001" typeDefinition="DirectSpeakers"> <audioBlockFormat audioBlockFormatID="AB_00010006_00000001"> <speakerLabel>M-110</speakerLabel> <position coordinate="azimuth">-110.0</position> <position coordinate="elevation">0.0</position> <position coordinate="distance">1.0</position> </audioBlockFormat> </audioChannelFormat> <audioChannelFormat audioChannelFormatID="AC_00020003" audioChannelFormatName="Lo" typeLabel="0002" typeDefinition="Matrix"> <audioBlockFormat audioBlockFormatID="AB_00020003_00000001"> <speakerLabel>M+30</speakerLabel> <matrix> <coefficient gain="1.0">AC_00010001</coefficient> <coefficient gainVar="clev">AC_00010003</coefficient> <coefficient gainVar="slev">AC_00010005</coefficient> </matrix> </audioBlockFormat> </audioChannelFormat> <audioChannelFormat audioChannelFormatID="AC_00020004" audioChannelFormatName="Ro" typeLabel="0002" typeDefinition="Matrix"> <audioBlockFormat audioBlockFormatID="AB_00020004_00000001"> <speakerLabel>M-30</speakerLabel> <matrix> <coefficient gain="1.0">AC_00010002</coefficient> <coefficient gainVar="clev">AC_00010003</coefficient> <coefficient gainVar="slev">AC_00010006</coefficient> </matrix> </audioBlockFormat> </audioChannelFormat> <!-- ############ --> <!-- STREAMS --> <!-- ############ --> <audioStreamFormat audioStreamFormatID="AS_00010001" audioStreamFormatName="PCM_FrontLeft" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00010001</audioChannelFormatIDRef> <audioTrackFormatIDRef>AS_00010001_AT_01</audioTrackFormatIDRef> </audioStreamFormat> <audioStreamFormat audioStreamFormatID="AS_00010002" audioStreamFormatName="PCM_FrontRight" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00010002</audioChannelFormatIDRef> <audioTrackFormatIDRef>AS_00010002_AT_01</audioTrackFormatIDRef> </audioStreamFormat> <audioStreamFormat audioStreamFormatID="AS_00010003" audioStreamFormatName="PCM_FrontCentre" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00010003</audioChannelFormatIDRef> <audioTrackFormatIDRef>AS_00010003_AT_01</audioTrackFormatIDRef> </audioStreamFormat> <audioStreamFormat audioStreamFormatID="AS_00010004" audioStreamFormatName="PCM_LFE" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00010004</audioChannelFormatIDRef> <audioTrackFormatIDRef>AS_00010004_AT_01</audioTrackFormatIDRef> </audioStreamFormat> <audioStreamFormat audioStreamFormatID="AS_00010005" audioStreamFormatName="PCM_SurroundLeft" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00010005</audioChannelFormatIDRef> <audioTrackFormatIDRef>AS_00010005_AT_01</audioTrackFormatIDRef> </audioStreamFormat> <audioStreamFormat audioStreamFormatID="AS_00010006" audioStreamFormatName="PCM_SurroundRight" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00010006</audioChannelFormatIDRef> <audioTrackFormatIDRef>AS_00010006_AT_01</audioTrackFormatIDRef> </audioStreamFormat> <audioStreamFormat audioStreamFormatID="AS_00020003" audioStreamFormatName="PCM_Lo" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00020003</audioChannelFormatIDRef> <audioTrackFormatIDRef>AS_00020003_AT_01</audioTrackFormatIDRef> </audioStreamFormat> <audioStreamFormat audioStreamFormatID="AS_00020004" audioStreamFormatName="PCM_Ro" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00020004</audioChannelFormatIDRef> <audioTrackFormatIDRef>AS_00020004_AT_01</audioTrackFormatIDRef> </audioStreamFormat> <!-- ############ --> <!-- AUDIO TRACKS --> <!-- ############ --> <audioTrackFormat audioTrackFormatID="AS_00010001_AT_01" audioTrackFormatName="PCM_FrontLeft" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00010001</audioStreamFormatIDRef> </audioTrackFormat> <audioTrackFormat audioTrackFormatID="AS_00010002_AT_01" audioTrackFormatName="PCM_FrontRight" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00010002</audioStreamFormatIDRef> </audioTrackFormat> <audioTrackFormat audioTrackFormatID="AS_00010003_AT_01" audioTrackFormatName="PCM_FrontCentre" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00010003</audioStreamFormatIDRef> </audioTrackFormat> <audioTrackFormat audioTrackFormatID="AS_00010004_AT_01" audioTrackFormatName="PCM_LFE" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00010004</audioStreamFormatIDRef> </audioTrackFormat> <audioTrackFormat audioTrackFormatID="AS_00010005_AT_01" audioTrackFormatName="PCM_SurroundLeft" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00010005</audioStreamFormatIDRef> </audioTrackFormat> <audioTrackFormat audioTrackFormatID="AS_00010006_AT_01" audioTrackFormatName="PCM_SurroundRight" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00010006</audioStreamFormatIDRef> </audioTrackFormat> <audioTrackFormat audioTrackFormatID="AS_00020003_AT_01" audioTrackFormatName="PCM_Lo" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00020003</audioStreamFormatIDRef> </audioTrackFormat> <audioTrackFormat audioTrackFormatID="AS_00020004_AT_01" audioTrackFormatName="PCM_Ro" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00020004</audioStreamFormatIDRef> </audioTrackFormat> <audioTrackUID UID="ATU_00000001"/> <audioTrackUID UID="ATU_00000002"/> <audioTrackUID UID="ATU_00000003"/> <audioTrackUID UID="ATU_00000004"/> <audioTrackUID UID="ATU_00000005"/> <audioTrackUID UID="ATU_00000006"/> <audioTrackUID UID="ATU_00000007"/> <audioTrackUID UID="ATU_00000008"/>

* 	Radiocommunication Study Group 6 made editorial amendments to this Recommendation in July 2015 in accordance with Resolution ITU-R 1.