RECOMMENDATION  ITU-R  BT.1203*

User requirements for generic bit-rate reduction coding
of digital TV signals (SDTV, EDTV and HDTV)
for an end-to-end television system

(1995)

The ITU Radiocommunication Assembly,

considering

a)	that rapid progress is being made in bit-rate reduction coding techniques;

b)	that bit-rate reduction coding of digital SDTV, EDTV and HDTV signals will find wide applications for studio production, for contribution, for both primary and secondary distribution and for emission by terrestrial means and satellite;

c)	that in the total chain of broadcasting, a number of codecs will be used in cascade which leads to a loss of picture quality;

d)	that ITU-T Recommendation J.81 is already used for codecs for SNG, contribution and primary distribution;

e)	that there could be advantages in having a generic (i.e. related) bit-rate reduction coding in the various applications if maximum commonality among various applications can be utilized;

f)	that interoperability between different video formats and distribution media should be achieved;

g)	that digital and analogue broadcasting systems will both exist during a transition period,

recommends

1	that same picture format or closely related signal formats should be used as far as possible, through a total broadcasting chain;

2	that the picture format of both input and output signals for coding and decoding should be the same;

3	that the same or closely related coding schemes should be used as much as possible for terrestrial and satellite emission and secondary distribution in order to minimize the receiver cost and the quality degradation;

4	that the encoder should as far as possible allow non-changing parameters which may be used in subsequent coding processes, e.g. motion information to be down-loaded into the subsequent codecs;

5	that the interface between the codecs should be simple;

6	that the coding scheme can be used by composite and component signals without a perceptible loss of picture quality,

further recommends

1	that the values listed in Table 1 should be used for the input of the encoder and for the output of the decoder;

2	that the functional and operational requirements described in Table 4 should be satisfied;

3	that the benefits of generic coding applied to the total or to parts of the total broadcasting chain be studied in terms of ease of operation, equipment cost and picture quality.

Annex 1

TABLE  1

Signal format for codecs

| Items |  | SNG(1) |  | Contribution | Studio production | Primary distribution | Primary distribution | Terrestrial | Satellite | Secondary distribution |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Mode 1 | Mode 2 |  |  | Case 1(2) | Case 2(3) |  |  |  |
| No. of samples/line | HDTV EDTV SDTV | Examples are listed in Tables 2 and 3 |  |  |  |  |  |  |  |  |
| Interlace ratio | HDTV EDTV SDTV | Interlace format in the current state with transition to progressive format in the future |  |  |  |  |  |  |  |  |
| No. of lines/frame | HDTV EDTV SDTV | Examples are listed in Tables 2 and 3 |  |  |  |  |  |  |  |  |
| Colour format | HDTV EDTV SDTV | 4:2:2 processing required |  |  |  | 4:2:2 should be used for the digital interface 4:2:0 may be used for internal coding |  |  |  |  |
| (1) Mode 1: good transmission conditions. Mode 2: poor transmission conditions. (2) Case 1: digital primary distribution followed by analogue secondary distribution or emission. (3) Case 2: digital primary distribution followed by digital secondary distribution or emission. |  |  |  |  |  |  |  |  |  |  |

TABLE  2

Examples of picture formats

|  | 50 Hz environment |  | 60 Hz environment |  |
| --- | --- | --- | --- | --- |
| HDTV (16:9) | 1 920  1 152 1 440  1 152 | I I | 1 920  1 035 1 920  1 080 1 920  1 080 1 440  1 080 1 280  720 | I P I I P |
| EDTV (16:9) | 960  576 960  576 720  576 | P I I | 960  483(1) 960  483(1) 720  483(1) | P I I |
| SDTV (4:3) | 720  576 | I | 720  483(1) | I |
| I: interlaced scans P: progressive scans (1) The number of coded lines can be 480 in the case of emission and secondary distribution applications, but the aspect ratio should be defined using 483 active lines. |  |  |  |  |

TABLE  3

Upper bounds for sampling density and luminance pel rate specified in MPEG-2

| Level | Layer |  | Simple | Main | SNR | Spatial | High 4:2:2 | High 4:2:0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| High | Enhancement | pel/H line/V frame/s pel rate(1) |  |  |  |  | 1 920 1 152 60 83.5584 | 1 920 1 152 60 62.6688 |
|  | Base | pel/H line/V frame/s pel rate |  | 1 920 1 152 60 62.6688 |  |  | 960 576 30 19.6608 | 960 576 30 14.7456 |
| High-1 440 | Enhancement | pel/H line/V frame/s pel rate |  |  |  | 1 440 1 152 60 47.0016 | 1 440 1 152 60 62.6688 | 1 440 1 152 60 47.0016 |
|  | Base | pel/H line/V frame/s pel rate |  | 1 440 1 152 60 47.0016 |  | 720 576 30 10.3680 | 720 576 30 14.7456 | 720 576 30 11.0592 |
| Main | Enhancement | pel/H line/V frame/s pel rate |  |  |  |  | 720 576 30 14.7456 | 720 576 30 11.0952 |
|  | Base | pel/H line/V frame/s pel rate | 720 576 30 10.3680 | 720 576 30 10.3680 | 720 576 30 10.3680 |  | 352 288 30 | 352 288 30 3.04128 |
| Low | Enhancement | pel/H line/V frame/s pel rate |  |  |  |  |  |  |
|  | Base | pel/H line/V frame/s pel rate |  | 352 288 30 3.04128 | 352 288 30 3.04128 |  |  |  |
| (1) Unit of pel rate: Msample/s. |  |  |  |  |  |  |  |  |

Annex 2

TABLE  4

Functional and operational requirements for generic codecs

| Item | SNG Mode 1 | SNG Mode 2 | Contribution | Studio production | Primary distribution Case 1 | Primary distribution Case 2 | Terrestrial | Satellite | Secondary distribution |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. of audio channels HDTV EDTV SDTV | Minimum 2 Minimum 2 |  | Maximum 8 Maximum 6 |  | Maximum 6 Maximum 6 |  |  |  |  |
| Range of bit rates HDTV EDTV SDTV | Up to 140 Mbit/s Further study Up to 34 or 45 Mbit/s |  |  |  | Corresponds to SNG and contribution bit rates | Corresponds to secondary distribution | Up to 80 Mbit/s Further study Up to 15 Mbit/s |  |  |
| Prediction mode | I, P |  |  |  | (I, B, P) and (I, P) are used in non-live and live broadcasting, respectively |  |  |  |  |
| Picture quality | 12% | 36% | 12% |  | 12% | 18% |  |  |  |
| Compatibility | Not required |  |  |  |  | Desirable |  |  |  |
| Hierarchical coding | Not required |  |  |  |  | Required only for the graceful degradation system |  |  |  |
| Scalability | Not required, however if needed then lower quality can be obtained with a standards converter |  |  |  |  | Desirable, needed for hierarchical coding |  |  |  |
| Interoperability | Not required |  |  |  |  | A decoder should decode bit streams with any picture format for SDTV or EDTV or HDTV, but not necessarily all |  |  |  |

TABLE  4 (end )

| Item | SNG Mode 1 | SNG Mode 2 | Contribution | Studio production | Primary distribution Case 1 | Primary distribution Case 2 | Terrestrial | Satellite | Secondary distribution |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Editability |  |  |  | Required in the bit stream domain |  |  |  |  |  |
| Bit-rate flexibility | A decoder should decode bit streams at any bit rate described in the item of “range of bit rates” |  |  |  |  |  |  |  |  |
| Codec delay | An overall delay of less than 300 ms would be desirable for interactive talk-back applications |  |  |  |  |  |  |  |  |
| Recovery time (after a break of 50 ms) |  1 s |  |  500 ms |  |  500 ms |  |  |  |  |
| Acquisition time | The major contributions to acquisition time are the decoding delay and the interval between I pictures. A desirable figure for this value is less than 500 ms |  |  |  |  |  |  |  |  |
| Error concealment | Required, a decoder should support this functionality and should also provide a signalling function of error conditions for studio applications |  |  |  | Desirable |  |  |  |  |
| Graceful degradation | Not required |  |  |  |  | Desirable, essential for mobile and portable reception |  |  |  |
| Channel hopping latency | Not required |  |  |  |  | Less than 550 ms |  |  |  |
| Relative delay between sound and vision |  2 ms per codec |  |  |  |  |  | Under study |  |  |
| I: interframe P: P frame B: B frame. |  |  |  |  |  |  |  |  |  |

Annex 3

Definition and explanation of items listed in Tables 1, 3 and 4

Generic coding: digital coding of pictures based on family of related coding methods.

No. of samples/line: number of luminance samples per active line.

Interlace ratio: 1:1 is for a progressive format, where the input signal is frame-structured. 2:1 is for an interlaced format, where the input signal is field-structured.

No. of lines/frame: number of vertical lines per active frame.

Colour format: ratio between the number of the luminance pixels and the number of the co-sited chroma difference pixels or the ratio between the colour pixels R, G and B.

No. of audio channels: total number of sound channels per programme, together with a description how these channels can be combined for different applications.

Range of bit rates: minimum and maximum encoder output bit rates for several input formats.

Prediction mode: type of prediction used inside the encoder. This influences very strongly the maximal achievable picture quality of following codecs.

Picture quality: results of the subjective evaluation of the encoding and decoding performance in an error-free channel.

Compatibility: description whether the bit stream syntax allows the separate signal processing of parts of the total bit stream in subsequent codecs.

Hierarchical coding: method to achieve different resolution layers on the decoder side.

Scalability: access to several picture qualities in a single bit stream.

Interoperability: description of the grade of commonality between different bit streams inside the broadcasting chain.

Editability: ability to edit a programme taking into account the structure of the encoder output data.

Bit-rate flexibility: the coding algorithm may allow the use of either CBR (constant bit rate) – or VBR (variable bit rate) – coding.

Codec delay: the delay introduced by the coding/decoding algorithm.

Recovery time: the time period between a physical interruption inside the broadcasting chain and the achievement of full functionality.

Acquisition time: the maximal acceptable waiting time from start of the decoding process until the display of the picture. This might influence the choice of the generic coding scheme.

Error concealment: possibility of the decoder to react in a specified way to alarm signals coming from the FEC part of the decoder.

Graceful degradation: to avoid an abrupt degradation of the picture quality on the decoder side, the output of scalable encoders can be protected by different FEC schemes or by non-uniform modulation schemes. A combination of both methods is also possible.

Channel hopping: necessity to switch as quickly as possible between different programmes.

*	 	Radiocommunication Study Group 6 made editorial amendments to this Recommendation in 2003 in accordance with Resolution ITU-R 44.