RECOMMENDATION ITU-R BT.1208

VIDEO  CODING  FOR  DIGITAL  TERRESTRIAL  TELEVISION  BROADCASTING

(Question ITU-R 121/11)

(1995)

Rec. ITU-R BT.1208

The ITU Radiocommunication Assembly,

considering

a)	that digital terrestrial television broadcasting will be introduced in the VHF/UHF bands;

b)	that the source signals for this service may be of standard, extended or high-definition quality and of different scanning formats;

c)	that digital emission might enable scalable television broadcasting by means of hierarchical coding;

d)	that compatibility with the scanning formats for conventional television emission systems might be desirable;

e)	that the maximum bit rate available is limited by the bandwidth of the channels (6-8 MHz) in the VHF/UHF bands;

f)	that it is desirable that the bit-rate reduction method used for terrestrial emissions is based on a generic coding algorithm;

g)	that maximum commonality is also desirable between digital systems for different emission and secondary distribution media (e.g. to the home receivers);

h)	that reception under adverse conditions might be made possible with reduced but acceptable picture quality;

j)	that portable or mobile reception might be made possible with reduced but acceptable picture quality;

k)	that methods for subjective assessment of the quality of television pictures is the subject of Recommendations ITU-R BT.500, ITU-R BT.710 and ITU-R BT.811,

recommends

1	that digital terrestrial television broadcasting systems should use the video-coding standards described in ISO/IEC 13818-2 or ITU-T Recommendation H.262 (see Annex 1 for information leading to this Recommendation);

2	that the encoding parameters and methods should generate bit streams which conform to the sets of profiles and levels specified in ISO/IEC 13818-2 or ITU-T Recommendation H.262.

ANNEX  1

Considerations for the selection of an MPEG-2 video-coding conformance
point for digital terrestrial television broadcasting

The MPEG-2 family of video-coding standards provides the capability of a universal coding and compression method to serve compatibly various broadcasting requirements due to the ability of the higher profiles and levels to decode lower profiles and levels. It is therefore possible to choose an appropriate profile and level for a receiver that could universally decode SDTV, EDTV and HDTV broadcasts. Furthermore, it would still provide the flexibility to broadcasters to provide different functionalities for SDTV, EDTV and HDTV to meet local technical and service requirements and constraints.

The family of MPEG-2 standards is shown in Table 1.

TABLE  1

MPEG-2 profiles and levels

| Levels | Profiles |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Simple 4:2:0 | Main 4:2:0 | SNR scalability 4:2:0 | Spatial scalability 4:2:0 | High scalability 4:2:2 |
| High 1920 ´ 1152 | Undefined | 80 Mbit/s | Undefined | Undefined | 100 Mbit/s |
| High-1440 1440 ´ 1152 | Undefined | 60 Mbit/s | 60 Mbit/s | 60 Mbit/s | 80 Mbit/s |
| Main 720 ´ 576 | 15 Mbit/s | 15 Mbit/s | 15 Mbit/s | 15 Mbit/s | 20 Mbit/s |
| Low 352 ´ 288 | Undefined | 4 Mbit/s | 4 Mbit/s | 4 Mbit/s | Undefined |

At this point in time consideration is given to the main profile/main level and SNR scalability profile/main level for the provision of SDTV and EDTV digital terrestrial television broadcasting services including multi-programme television broadcasting services. For HDTV services consideration is being given to the main profile/high level and high-1440 level as well as the spatial scalability profile/high-1440 level.

In order for a television receiver to be able to decode all these various television services it has to have functionality of the highest profile and highest level proposed for these services. This leads for the universal television receiver decoder to the choice of the spatial scalability profile at the high level as the conformance point in the MPEG-2 standard.

This profile and level is able to decode all lower levels of this profile as well as all the levels of the SNR scalability, main and simple profiles. However, a functionality at the simple profile and the low level at all profiles is thought unlikely to be attractive to broadcasting. The shaded area in Table 1 reflects these considerations.

Since there is no obligation at the encoder to use all the functionalities as permitted by the MPEG-2 spatial scalability profile/high level standard, only a subset of the functionalities may be used to provide SDTV, EDTV or HDTV. Therefore, it is possible to provide digital terrestrial television broadcasting services with different functionalities.

The choice of a conformance point for the receiver video decoder at a profile and level as high as possible provides a high level of flexibility to the broadcaster. A broadcast service may be started with very modest functionality and can be upgraded when required without consideration having to be given to already existing receivers. It is also possible to use different functionalities (resulting in different qualities) depending on the time and nature of the programme.

In the following the expected implementation for SDTV, EDTV and HDTV will be discussed.

###### SDTV and EDTV

Developments in many countries indicate that the MPEG-2 main profile at the main level and the SNR scalability at the main level are the considered candidates, which fall within the recommended boundaries in Table 1. Consequently, SDTV and EDTV can be decoded by the tentatively proposed universal receiver. First evaluations of MPEG-2 codec simulations suggest that SDTV quality is likely to be achieved at the order of 5 Mbit/s and EDTV quality at the order of 9 Mbit/s. Further evaluations should continue since there are many factors that affect quality.

###### HDTV

Different functionalities for this service are considered in different countries. Countries which do not consider spatial scalability as part of the required functionalities may implement television broadcast encoders at the lower profiles such as the SNR scalability or main profiles at the high or high-1440 level.

It is expected that the choices for implementing HDTV services will be narrowed down during 1995 after further simulation and hardware evaluations have been completed.

See Appendix 1 for further information and considerations.

APPENDIX  1

TO  ANNEX  1

Note on video-decoding standard for digital terrestrial broadcasting

Various countries are seriously considering implementing digital terrestrial television broadcasting in the near future. All or some of the possible qualities for digital television, that is SDTV, EDTV and HDTV, will be offered by several countries. While there appears to be a general consensus that video codecs should comply with the ISO/IEC MPEG-2 family of standards, different profiles and levels of this standard are considered for different quality of TV services, and, furthermore, the particular choice of a standard for a given service.

The World Broadcasting Union has clearly expressed strong support for the use of a standard that could lead to a universal digital TV receiver that would be able to decode digital television services of varying qualities in any country.

The ISO/IEC MPEG-2 family of standards is an ideal vehicle to meet this requirement, provided that the appropriate profile and level in this standard is chosen.

The philosophy of this family of standards is to define for each profile and level certain functionalities from a large but limited set of functionalities (syntax elements). The lowest functionalities, using the lowest number of the available syntax elements, are associated with low profiles and low levels. As one moves from low profiles and levels to higher profiles and levels increasingly more functionalities are added. Consequently higher profile level decoders can decode signals that are at the same level but lower profile or at a lower level in the same or lower profile. This capability indicates the advantage of using a standard for the decoder at as high a profile and level as possible. Since the transmission, that is the encoding, can be done at any lower functionality than the receiver decoder is capable to decode, the broadcaster has the freedom to provide services at different qualities and functionalities without being restricted by the receiver capabilities. Therefore broadcasters could provide services that may be changed (improved) over time, as well as offering services with different functionalities in different countries. The lower the profile and level for the receiver decoder is chosen, the more limited the service functionality choices for the broadcaster will be.

There is no doubt that the higher the proposed profile and level is, the more complex and initially costly the receiver decoder will be. However, it should be kept in mind that receiver cost studies that have been carried out (for example, in North America) indicate in the case of an HDTV receiver that the cost of the electronics circuits is only a small percentage of the overall receiver cost. Furthermore, since digital terrestrial television broadcasting services are expected to start in a few years’ time, improvements in solid state devices and cost reductions can be expected. Since digital television services have yet to be introduced, and should be provided for a long time, the standard should allow for improvements and evolution in the future and should not be constrained by short-term concerns.

It may, therefore, be advantageous to give consideration to further studies to investigate the degree of commonality that would be desirable in digital television receiver video decoders.

_________________