RECOMMENDATION  ITU-R  BT.1550

MPEG-2 recoding data set for the preservation of picture quality
in cascade of MPEG-2 codecs compressed stream format

(Question ITU-R 41/6)

(2001)



The ITU Radiocommunication Assembly,

considering

a)	that digital television broadcasts make use of ISO/IEC 13818-2:2000 MPEG-coded compressed signals;

b)	that there are economic and operational advantages in handling television signals in compressed form in acquisition and production;

c)	that unless special steps are taken, the cascading of ISO/IEC 13818-2:2000 MPEG codecs may lead to degradations of picture quality;

d)	that the original ISO/IEC 13818-2:2000 MPEG coding parameters can be extracted in an MPEG decoder;

e)	that near- loss -less cascading is achievable by any recoding process which is able to fully match the parameters that were used at a previous encoding stage;

f)	that Recommendation ITU-R BT.1532 defines the data set for near- loss -less cascading;

g)	that a compressed form of the data set is also useful for studio applications,

recommends

1	that the compressed data set described in SMPTE Standard 329M-2000 – MPEG-2 Video Recoding Data Set – Compressed Stream Format, be used for carrying coding parameter information from an ISO/IEC 13818-2:2000 MPEG decoder to any subsequent re-encoder.

Summary of SMPTE Standard 329M-2000

This Standard specifies the content of the picture related recoding data set for the representation of ISO/IEC 13818-2 MPEG coding information for the purpose of optimally cascading decoders and recoders at any bit rate or structure of group of pictures. The coding information is as derived from an ISO/IEC 13818 compliant MPEG bit stream during the picture decoding process, as described in ISO/IEC 13818-2. The scope and operation of this standard are defined by the content of a sufficient recoding data set in compressed form which may be derived in decoders that

comply with ISO/IEC 13818-2, including all non-scalable profiles defined in ISO/IEC 13818-2. To allow the re-synchronization of the video and its associated audio or data after processing, a mechanism using some additional information derived from ISO/IEC 13818-1 is also included in this Standard. This sufficient compressed data set may be transported by various means defined in other SMPTE standards and ITU-R Recommendations.

The principal application of this Standard is to preserve the quality of the video signal when cascading MPEG-2 decoders and coders (including transcoding) by feeding forward previous coding decisions. The MPEG-2 recoding compressed data set is described as sufficient when it contains the data required that, in combination with an MPEG-2 decoded or partially decoded picture, allows bit accurate recreation of the previously picture coded bit stream. The various methods whereby the data set is transported alongside the MPEG-2 decoded or partially decoded picture information are the subject of other standards.

NOTE 1 – SMPTE Standard 329M-2000 is given in Annex 1. This document references the MPEG version of 1996, however the recoding data set technical specifications do not change with the update to the version of 2000. SMPTE Standard 329M-2000 and its summary refer to Version 2000 only, which is the version approved by Administrations of Member States of the ITU and Radiocommunication Sector Members participating in the work of Radiocommunication Study Group 6 in application of Resolution ITU-R 45 on 23 December 2001. By agreement between ITU and SMPTE, this Version was provided and authorized for use by SMPTE and accepted by ITU-R for inclusion in this Recommendation. Any subsequent version of SMPTE Standard 329M, which has not been accepted and approved by Radiocommunication Study Group 6 is not part of this Recommendation. For subsequent versions of SMPTE Documents, the reader should consult the SMPTE website: http://www.smpte.org/.

Note by the Secretariat: The SMPTE Standard 329M-2000, previously referred to as a web link site in electronic form, has been annexed to the text of this Recommendation.