RECOMMENDATION  ITU-R  BT.1619

Vertical ancillary data mapping for
serial digital interface

(Questions ITU-R 42/6 and ITU-R 20/6)

(2003)

The ITU Radiocommunication Assembly,

considering

a)	that many countries are installing digital television production facilities based on the use of digital video components conforming to Recommendations ITU-R BT.601, ITU-R BT.656, ITU-R BT.709, ITU-R BT.799 and ITU-R BT.1120;

b)	that there exists the capacity within a signal conforming to Recommendation ITU-R BT.656, ITU-R BT.799 or ITU-R BT.1120 for additional data signals to be multiplexed with the video data signal itself;

c)	that there are operational and economic benefits to be achieved by the multiplexing of ancillary data signals with the video data signal;

d)	that the operational benefits are increased if a minimum of different formats are used for ancillary data signals;

e)	that formatting of ancillary data packets is specified in Recommendation ITU-R BT.1364;

f)	that carriage of data broadcast services intended for the public as well as broadcaster internal control and communications within the serial digital interface (SDI) benefit broadcast transmission operations,

recommends

1	that the ancillary data formatting described in SMPTE Standard 334M-2000  Vertical Ancillary Data Mapping for Bit-Serial Interface, be used as a method for carrying communications and control information in the SDI.

NOTE 1  SMPTE 334M includes a normative reference to SMPTE 292M  Bit-Serial Digital Interface for High-Definition Television Systems. The 1250-line, 50 Hz format listed in Table 1, column C of SMPTE 292M shall not be considered part of this Recommendation.

Summary of SMPTE Standard 334M-2000

This Recommendation defines a method of coding which allows data services to be carried in the vertical ancillary data space of a bit-serial component television signal conforming with Recommendation ITU-R BT.656 or ITU-R BT.1120 and formatted as specified in Recommendation ITU-R BT.1364.

This includes data broadcast services intended for the public as well as broadcaster internal control and communications. Despite the reference to the bit-serial interface, nothing in this specification precludes its use in a parallel digital interface for component digital video signals. The data described in this Standard may also be transported in key length value format according to Recommendation ITU-R BT.1563 (SMPTE 336M-2001), or via other means.

NOTE 1 – SMPTE Standards 334M-2000 and 292M-1998 are given in Annexes 1 and 2. SMPTE Standards 334M-2000 and 292M-1998 refer to Versions 2000 and 1998 respectively only, which are the versions approved by Administrations of Member States of the ITU in application of Resolution ITU-R 1-3 on 3-05-03. By agreement between ITU and SMPTE, these Versions were provided and authorized for use by SMPTE and accepted by ITU-R for inclusion in this Recommendation. Any subsequent versions of SMPTE Standards 334M and 292M, which have not been accepted and approved by Radiocommunication Study Group 6 are not part of this Recommendation. For subsequent versions of SMPTE documents, the reader should consult the SMPTE website: http://www.smpte.org/.