Recommendation ITU-R BT.709-5 (04/2002)

Parameter values for the HDTV standards for production and international programme exchange

BT Series Broadcasting service (television)

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Annex 1 of Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from http://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Recommendations (Also available online at http://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radiowave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2009

 ITU 2009

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION  ITU-R  BT.709-5

Parameter values for the HDTV** standards for production
and international programme exchange

(Question ITU-R 27/11)

(1990-1994-1995-1998-2000-2002)

Scope

This Recommendation defines the image format parameters and values for HDTV. Part 1 of this Recommendation is not in current use and is included to capture historic evolution of the Recommendation. Part 2 of the Recommendation is widely deployed; implementers of broadcasting technologies should apply the values contained in Part 2.

The ITU Radiocommunication Assembly,

considering

a)	that for many years HDTV programmes have been produced in several countries;

b)	that parameter values for HDTV production standards should have maximum commonality;

c)	that two HDTV scanning standards, 1125/60/2:1 and 1250/50/2:1, were previously developed for that purpose, having a significant number of parameters which have been agreed on a worldwide basis, and for which some equipment remains in use;

d)	that a HDTV common image format of 1 920 pixels by 1080 lines providing square pixel sampling and a number of interlace and progressive picture rates has been designed for digital television, computer imagery and other applications (in this Recommendation, the term pixel is used to describe a picture element in the digital domain);

e)	that the parameters defined for all these systems meet the quality goals set for HDTV;

f)	that film productions are an important programme source for HDTV broadcasting and, conversely, the use of HDTV production systems has significant benefits for film programme production;

g)	that high-quality conversion between the various HDTV systems, as well as down-conversion to 525/625 television systems, has been successfully implemented;

h)	that programmes produced and archived will not become obsolete using these standards,

recommends

1	that for HDTV programme production and international exchange, one of the systems described in Part 2 of this Recommendation, should be used.

Signal parameter values for the 1125/60/2:1 system
and the 1250/50/2:1 system

Part  1

HDTV systems related to conventional television

(The areas in bold characters in the Tables below denote parameter values which have been agreed to on a worldwide basis.)

# 1	Opto-electronic conversion

| Item | Parameter | Value |  |
| --- | --- | --- | --- |
|  |  | 1125/60/2:1 | 1250/50/2:1 |
| 1.1 | Opto-electronic transfer characteristics before non-linear precorrection | Assumed linear |  |
| 1.2 | Overall opto-electronic transfer characteristics at source | V  1.099 L0.45 – 0.099 for 1  L  0.018 V  4.500 L for 0.018  L  0 where: L: luminance of the image 0  L  1 V: corresponding electrical signal |  |
| 1.3 | Chromaticity coordinates (CIE, 1931) Primary – Red (R) – Green (G) – Blue (B) | x | y |
|  |  | 0.640 0.300 0.150 | 0.330 0.600 0.060 |
| 1.4 | Assumed chromaticity for equal primary signals (Reference white) ER  EG  EB | D65 |  |
|  |  | x | y |
|  |  | 0.3127 | 0.3290 |

# 2	Picture characteristics

| Item | Parameter | Value |  |
| --- | --- | --- | --- |
|  |  | 1125/60/2:1 | 1250/50/2:1 |
| 2.1 | Aspect ratio | 16:9 |  |
| 2.2 | Sample per active line | 1920 |  |
| 2.3 | Sampling lattice | Orthogonal |  |
| 2.4 | Active lines per picture | 1035 | 1152 |

# 3	Picture scanning characteristics

| Item | Parameter | Value |  |
| --- | --- | --- | --- |
|  |  | 1125/60/2:1 | 1250/50/2:1 |
| 3.1 | Order of sample scanning | Left to right, top to bottom 1st line of field 1 above 1st line of field 2 |  |
| 3.2 | Interlace ratio | 2:1 |  |
| 3.3 | Picture rate (Hz) | 30 | 25 |
| 3.4 | Total number of lines | 1125 | 1250 |
| 3.5 | Field frequency (Hz) | 60 | 50 |
| 3.6 | Line frequency (Hz) | 33 750  0.001% | 31 250  0.0001% |

# 4	Signal format

The terms R, G, B, Y, CB, CR, are often used and are generally understood to refer to the signals ER', EG',EB',EY',ECB',ECR' respectively (i.e. they correspond to gamma pre-corrected signals).

| Item | Parameter | Value |  |
| --- | --- | --- | --- |
|  |  | 1125/60/2:1 | 1250/50/2:1 |
| 4.1 | Conceptual non-linear precorrection of primary signals |   0.45 (see item 1.2) |  |
| 4.2 | Derivation of luminance signal EY(1)' | EY' 0.2126ER' 0.7152EG' 0.0722EB' | EY' 0.299ER' 0.587EG' 0.114EB' |
| 4.3 | Derivation of colour-difference signal (analogue coding)(1) | ECB' 0.5389 (EB'–EY') ECR' 0.6350 (ER'–EY') | ECB'= 0.564 (EB'–EY') ECR'= 0.713 (ER'–EY') |
| 4.4 | Derivation of colour-difference signal (digital coding) CB, CR | Digitally scaled from the values of item 4.3 |  |
| (1) The coefficients for the equations have been calculated following the rules laid down in SMPTE RP177-1993. |  |  |  |

# 5	Analogue representation

Levels are specified in millivolts (mV) measured across a matched 75  termination.

| Item | Parameter | Value |  |
| --- | --- | --- | --- |
|  |  | 1125/60/2:1 | 1250/50/2:1 |
| 5.1 | Nominal level (mV) ER', EG', EB', EY' | Reference black: 0 Reference white: 700 (see Fig. 1) |  |
| 5.2 | Nominal level (mV) ECB',ECR' |  350 (see Fig. 1) |  |
| 5.3 | Form of synchronizing signal | Tri-level bipolar (see Fig. 2) |  |
| 5.4 | Line sync timing reference | OH (see Fig. 2) |  |
| 5.5 | Sync level (mV) |  300  2% |  |
| 5.6 | Sync signal timing | (See Table 1 and Fig. 3) Sync on all components | (See Fig. 4) – rise time 50  10 ns (between 10-90%) – see also footnote (1) |
| 5.7 | Inter-component timing accuracy | Not applicable |  2 ns |
| 5.8 | Blanking interval | (See Table 1 and Fig. 5) | (See Tables 2 and 3) |
| 5.9 | Nominal signal bandwidth (MHz) | 30 (for all components) |  |
| (1) When using R, G, B signals, the use of syncs on at least the green channel is advised; transmission of separate syncs is also acceptable. When using Y, CB, CR signals the Y signal at least carries sync. |  |  |  |

TABLE  1

Level and timing specification of synchronizing signal of the 1125/60/2:1 system
(see Figs. 3 and 5)

| Symbol | Parameter | Nominal value | Reference clock intervals | Tolerance |
| --- | --- | --- | --- | --- |
| a | Negative line sync width | 0.593 s | 44 |  0.040 s |
| b | End of active video | 1.185 s | 88 |  0.080 s/– 0 s |
| c | Positive line sync width | 0.593 s | 44 |  0.040 s |
| d | Clamp period | 1.778 s | 132 |  0.040 s |
| e | Start of active video | 2.586 s | 192 |  0.080 s/– 0 s |
| f | Rise/fall time | 0.054 s | 4 |  0.020 s |
| t2 – t1 | Symmetry of rising edge | – | – |  0.002 s |
| Sm | Amplitude of negative pulse | 300 mV | – |  6 mV |
| Sp | Amplitude of positive pulse | 300 mV | – |  6 mV |
| V | Amplitude of video signal | 700 mV | – | – |
| – | Field-blanking interval | 45 H/field | 99 000 | – |

TABLE  2

Line timing details for the 1250/50/2:1 system
(See Figs. 4, 6 and 7)

| Item | Parameter | Time (s) | 2.25 MHz samples | 72 MHz samples |
| --- | --- | --- | --- | --- |
| 1 | Total line length | 32 | 72 | 2 304 |
| 2 | Active line length(1) – digital – analogue | 26.67 26.00 | 60 (58.5) | 1 920 1 872 |
| 3 | Line blanking(2) – digital – analogue | 5.33 6.00 | 12 (13.5) | 384 432 |
| 4 | Front porch(2) | 0.89 | 2 | 64 |
| 5 | Back porch(2) | 2.67 | 6 | 192 |
| 6 | Tri-level sync half width (T-sync) | 0.89 | 2 | 64 |
| 7 | Field pulse | 8.00 | 18 | 576 |
| (1) Relative disposition of analogue and digital active lines assumed to be as per scaled version of Recommendation ITU-R BT.601 (Annex 1, Part A) (i.e. symmetrical). The analogue active line is measured from the half-height of signal after line blanking. Rise and fall times assumed to be 15 ns but subject to ratification. Analogue blanking should preferably be applied at the studio or playout output. (2) Front porch is defined as the interval between the end of active video and the half-height of the leading negative edge of the tri-level sync pulse. Similarly back porch is the interval between the half-height of the trailing negative edge of the tri-level sync and the start of active video (see Fig. 6). |  |  |  |  |

TABLE  3

Field timing details for the 1250/50/2:1 system
(See Figs. 7 and 8)

| Item | Parameter | Value/Description |
| --- | --- | --- |
| 1 | Total number of lines per frame | 1250 |
| 2 | Total number of lines per field | 625 |
| 3 | Active lines per frame | 1152 |
| 4 | Active lines per field | 576 |
| 5 | Frame reference OV | OH on line 1 |
| 6 | Frame indication | Line 1250 |
| 7 | Field indication | Line 625 |
| 8 | Active lines field 1 | Lines 45 ... 620 inclusive |
| 9 | Active lines field 2 | Lines 670 ... 1245 inclusive |
| 10 | Field blanking | Lines 1246 ... 44 and 621 ... 669 inclusive |

# 6	Digital representation

| Item | Parameter | Value |  |
| --- | --- | --- | --- |
|  |  | 1125/60/2:1 | 1250/50/2:1 |
| 6.1 | Coded signal | R, G, B, or Y, CB, CR |  |
| 6.2 | Sampling lattice – R, G, B, Y | Orthogonal, line and picture repetitive |  |
| 6.3 | Sampling lattice signal – CB, CR | Orthogonal, line and picture repetitive co-sited with each other and with alternate(1) Y samples |  |
|  |  | (Multiples of 2.25 MHz) |  |
| 6.4 | Sampling frequency (MHz) – R, G, B, Y | 74.25  0.001% (33  2.25) | 72  0.0001% (32  2.25) |
|  |  | (Half of luminance sampling frequency) |  |
| 6.5 | Sampling frequency (MHz) – CB, CR | 37.125  0.001% (33/2  2.25) | 36  0.0001% (32/2  2.25) |
| 6.6 | Number of samples per full line – R, G, B, Y – CB, CR | 2 200 1 100 | 2 304 1 152 |
| 6.7 | Active number of samples per line – R, G, B, Y – CB, CR | 1 920 960 |  |
| 6.8 | Coding format | Linear, 8- or 10-bit/component |  |
| 6.9 | Timing relationship between the analogue synchronizing reference OH and video data (in clock periods) | 192 | 256 |
| 6.10 | Quantization levels(2) | 8- bit coding |  |
|  | – Black level R, G, B, Y – Achromatic CB, CR – Nominal peak – R, G, B, Y – CB, CR | 16 128 235 16 and 240 |  |
| 6.11 | Quantization level assignment(3) | 8- bit coding |  |
|  | – Video data – Timing references(2) | 1 through 254 0 and 255 |  |
| 6.12 | Filter characteristics(4) – R, G, B, Y – CB, CR | See Fig. 9A See Fig. 9B | See Fig. 10A See Fig. 10B |
| (1) The first active colour-difference samples being co-sited with the first active luminance sample. (2) For 1125/60/2:1 – In the case of 10-bit representation the two LSBs are ignored. (3) For 1125/60/2:1 – For 10-bit coding two LSBs are added to the 8-bit code words. For 1250/50/2:1 – 10-bit representation is under study. (4) These filter templates are defined as guidelines. |  |  |  |

Part  2

HDTV system with square pixel common image format

Introduction

The common image format (CIF) is defined to have common picture parameter values independent of the picture rate. The following picture rates are specified: 60 Hz, 50 Hz, 30 Hz, 25 Hz and 24 Hz. For the 60, 30 and 24 Hz systems, picture rates having those values divided by 1.001 are also specified. The parameter values for these systems, as referred to in the Table of § 6, are presented in parentheses.

Pictures are defined for progressive (P) capture and interlace (I) capture. Progressive captured pictures can be transported with progressive (P) transport or progressive segmented frame (PsF) transport. Interlace captured pictures can be transported with interlace (I) transport. Refer to Annex 1 for a description of segmented frame transport.

This results in the following combinations of picture rates and transports:

| System | Capture (Hz) | Transport |
| --- | --- | --- |
| 60/P | 60 progressive | Progressive |
| 30/P | 30 progressive | Progressive |
| 30/PsF | 30 progressive | Segmented frame |
| 60/I | 30 interlace | Interlace |
| 50/P | 50 progressive | Progressive |
| 25/P | 25 progressive | Progressive |
| 25/PsF | 25 progressive | Segmented frame |
| 50/I | 25 interlace | Interlace |
| 24/P | 24 progressive | Progressive |
| 24/PsF | 24 progressive | Segmented frame |

In cases where a progressive captured image is transported as a segmented frame, or a segmented frame signal is processed in a progressive format, the following rules shall be observed (see Fig. 11):

–	line numbering from the top of the captured frame to the bottom of the captured frame shall be sequential;

–	active line 1 and active line 1080 of the progressive captured image shall be mapped onto total line 42 and total line 1121, respectively, of the 1125 total lines;

–	odd active lines of the progressive captured image (1, 3, ..., 1079) shall be mapped onto total lines 21 through 560 of the segmented frame interface;

–	even active lines of the progressive captured image (2, 4, ..., 1080) shall be mapped onto total lines 584 through 1123 of the segmented frame interface.

With these rules, segmented frame transport has the same line numbering as that of interlace transport.

# 1	Opto-electronic conversion

| Item | Parameter | System Values |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 60/P | 30/P | 30/PsF | 60/I | 50/P | 25/P | 25/PsF | 50/I | 24/P | 24/PsF |
| 1.1 | Opto-electronic transfer characteristics before non-linear pre-correction | Assumed linear |  |  |  |  |  |  |  |  |  |
| 1.2 | Overall opto-electronic transfer characteristics at source | V  1.099 L0.45 – 0.099 for 1  L  0.018 V  4.500 L for 0.018 > L  0 where: L : luminance of the image 0  L  1 V : corresponding electrical signal |  |  |  |  |  |  |  |  |  |
| 1.3 | Chromaticity coordinates (CIE, 1931) | x |  |  |  |  | y |  |  |  |  |
|  | Primary – Red (R) – Green (G) – Blue (B) | 0.640 0.300 0.150 |  |  |  |  | 0.330 0.600 0.060 |  |  |  |  |
| 1.4 | Assumed chromaticity for equal primary signals (Reference white) | D65 |  |  |  |  |  |  |  |  |  |
|  |  | x |  |  |  |  | y |  |  |  |  |
|  | ER  EG  EB | 0.3127 |  |  |  |  | 0.3290 |  |  |  |  |

2	Picture characteristics

| Item | Parameter | System Values |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 60/P | 30/P | 30/PsF | 60/I | 50/P | 25/P | 25/PsF | 50/I | 24/P | 24/PsF |
| 2.1 | Aspect ratio | 16:9 |  |  |  |  |  |  |  |  |  |
| 2.2 | Samples per active line | 1 920 |  |  |  |  |  |  |  |  |  |
| 2.3 | Sampling lattice | Orthogonal |  |  |  |  |  |  |  |  |  |
| 2.4 | Active lines per picture | 1080 |  |  |  |  |  |  |  |  |  |
| 2.5 | Pixel aspect ratio | 1:1 (square pixels) |  |  |  |  |  |  |  |  |  |

# 3	Signal format

| Item | Parameter | System Values |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 60/P | 30/P | 30/PsF | 60/I | 50/P | 25/P | 25/PsF | 50/I | 24/P | 24/PsF |
| 3.1 | Conceptual non-linear pre-correction of primary signals |   0.45 (see item 1.2) |  |  |  |  |  |  |  |  |  |
| 3.2 | Derivation of luminance signal EY' | EY'  0.2126ER'  0.7152EG'  0.0722EB' |  |  |  |  |  |  |  |  |  |
| 3.3 | Derivation of color-difference signal (analogue coding) | ECB'=EB'−EY'1.8556 =− 0.2126 ER'−0.7152 EG'+0.9278 EB'1.8556 ECR'=ER'−EY'1.5748 =0.7874 ER'−0.7152 EG'−0.0722 EB'1.5748 |  |  |  |  |  |  |  |  |  |
| 3.4 | Quantization of RGB, luminance and colour-difference signals(1), (2) | DR'=INT219 ER'+16 ⋅2n−8 DG'=INT219 EG'+16 ⋅2n−8 DB'=INT219 EB'+16 ⋅2n−8 DY'=INT219 EY'+16 ⋅2n−8 DCB'=INT224 ECB'+128 ⋅2n−8 DCR'=INT224 ECR'+128 ⋅2n−8 |  |  |  |  |  |  |  |  |  |
| 3.5 | Derivation of luminance and colour-difference signals via quantized RGB signals | DY'=INT 0.2126 DR'+0.7152 DG'+0.0722 DB' DCB'=INT −0.21261.8556DR'−0.71521.8556DG'+0.92781.8556DB'⋅224219+2n−1DCR'=INT0.78741.5748DR'−0.71521.5748DG'−0.07221.5748DB'⋅224219+2n−1 |  |  |  |  |  |  |  |  |  |
| (1) “n” denotes the number of the bit length of the quantized signal. (2) The operator INT returns the value of 0 for fractional parts in the range of 0 to 0.4999... and +1 for fractional parts in the range of 0.5 to 0.9999..., i.e. it rounds up fractions above 0.5. |  |  |  |  |  |  |  |  |  |  |  |

# 4	Analogue representation

| Item | Parameter | System Values |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 60/P | 30/P | 30/PsF | 60/I | 50/P | 25/P | 25/PsF | 50/I | 24/P | 24/PsF |
| 4.1 | Nominal level (mV) ER', EG', EB', EY' | Reference black: 0 Reference white: 700 (see Fig. 13B) |  |  |  |  |  |  |  |  |  |
| 4.2 | Nominal level (mV) ECB',ECR' |  350 (see Fig. 13B) |  |  |  |  |  |  |  |  |  |
| 4.3 | Form of synchronizing signal | Tri-level bipolar (see Fig. 13A) |  |  |  |  |  |  |  |  |  |
| 4.4 | Line sync timing reference | OH (see Fig. 13A) |  |  |  |  |  |  |  |  |  |
| 4.5 | Sync level (mV) |  300  2% |  |  |  |  |  |  |  |  |  |
| 4.6 | Sync signal timing | Sync on all components (see Table 4, Figs. 12 and 13) |  |  |  |  |  |  |  |  |  |
| 4.7 | Blanking interval | (see Table 4, Figs. 12 and 13) |  |  |  |  |  |  |  |  |  |

# 5	Digital representation

| Item | Parameter | System Values |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 60/P | 30/P | 30/PsF | 60/I | 50/P | 25/P | 25/PsF | 50/I | 24/P | 24/PsF |
| 5.1 | Coded signal | R, G, B or Y, CB, CR |  |  |  |  |  |  |  |  |  |
| 5.2 | Sampling lattice – R, G, B, Y | Orthogonal, line and picture repetitive |  |  |  |  |  |  |  |  |  |
| 5.3 | Sampling lattice – CB, CR | Orthogonal, line and picture repetitive co-sited with each other and with alternate(1) Y samples |  |  |  |  |  |  |  |  |  |
| 5.4 | Number of active samples per line – R, G, B, Y – CB, CR | 1 920 960 |  |  |  |  |  |  |  |  |  |
| 5.5 | Coding format | Linear 8 or 10 bits/component |  |  |  |  |  |  |  |  |  |
| 5.6 | Quantization levels |  |  |  |  |  |  |  |  |  |  |
|  |  | 8-bit coding |  |  |  |  | 10-bit coding |  |  |  |  |
|  | – Black level R, G, B, Y – Achromatic CB, CR – Nominal peak – R, G, B, Y – CB, CR | 16 128 235 16 and 240 |  |  |  |  | 64 512 940 64 and 960 |  |  |  |  |
| 5.7 | Quantization level assignment | 8-bit coding |  |  |  |  | 10-bit coding |  |  |  |  |
|  | – Video data – Timing reference | 1 through 254 0 and 255 |  |  |  |  | 4 through 1 019 0-3 and 1 020-1 023 |  |  |  |  |
| 5.8 | Filter characteristics(2) – R, G, B, Y – CB, CR | See Fig. 14A See Fig. 14B |  |  |  |  |  |  |  |  |  |
| (1) The first active color-difference samples being co-sited with the first active luminance sample. (2) These filter templates are defined as guidelines. |  |  |  |  |  |  |  |  |  |  |  |

# 6	Picture scanning characteristics

| Item | Parameter | System Values |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 60/P | 30/P | 30/PsF | 60/I | 50/P | 25/P | 25/PsF | 50/I | 24/P | 24/PsF |
| 6.1 | Order of sample presentation in a scanned system | Left to right, top to bottom For interlace and segmented frame systems, 1st active line of field 1 at top of picture |  |  |  |  |  |  |  |  |  |
| 6.2 | Total number of lines | 1125 |  |  |  |  |  |  |  |  |  |
| 6.3 | Field/frame/segment frequency (Hz) | 60 (60/1.001) | 30 (30/1.001) | 60 (60/1.001) |  | 50 | 25 | 50 |  | 24 (24/1.001) | 48 (48/1.001) |
| 6.4 | Interlace ratio | 1:1 |  |  | 2:1 | 1:1 |  |  | 2:1 | 1:1 |  |
| 6.5 | Picture rate (Hz) | 60 (60/1.001) | 30 (30/1.001) |  |  | 50 | 25 |  |  | 24 (24/1.001) |  |
| 6.6 | Line frequency(1) (Hz) | 67 500 (67 500/1.001) | 33 750 (33 750/1.001) |  |  | 56 250 | 28 125 |  |  | 27 000 (27 000/1.001) |  |
| 6.7 | Samples per full line – R, G, B, Y – CB, CR | 2 200 1 100 |  |  |  | 2 640 1 320 |  |  |  | 2 750 1 375 |  |
| 6.8 | Nominal analogue signal bandwidths(2) (MHz) | 60 | 30 |  |  | 60 | 30 |  |  |  |  |
| 6.9 | Sampling frequency – R, G, B, Y (MHz) | 148.5 (148.5/1.001) | 74.25 (74.25/1.001) |  |  | 148.5 | 74.25 |  |  | 74.25 (74.25/1.001) |  |
| 6.10 | Sampling frequency(3) – CB, CR (MHz) | 74.25 (74.25/1.001) | 37.125 (37.125/1.001) |  |  | 74.25 | 37.125 |  |  | 37.125 (37.125/1.001) |  |

(1) The tolerance on frequencies is  0.001%. (2) Bandwidth is for all components. (3) CB, CR sampling frequency is half of luminance sampling frequency.

TABLE  4

Level and line timing specification
(See Figs. 12 and 13)

| Symbol | Parameter | System Values |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 60/P | 30/P |  | 30/PsF | 60/I | 50/P | 25/P |  | 25/PsF | 50/I | 24/P |  | 24/PsF |
| T | Reference clock interval (s) | 1/148.5 (1.001/148.5) | 1/74.25 (1.001/74.25) |  |  |  | 1/148.5 | 1/74.25 |  |  |  | 1/74.25 (1.001/74.25) |  |  |
| a | Negative line sync width(1) (T) | 44  3 |  |  |  |  |  |  |  |  |  |  |  |  |
| b | End of active video(2) (T) | 88  6 – 0 |  |  |  |  | 528  6 – 0 |  |  |  |  | 638  6 – 0 |  |  |
| c | Positive line sync width (T) | 44  3 |  |  |  |  |  |  |  |  |  |  |  |  |
| d | Clamp period (T) | 132  3 |  |  |  |  |  |  |  |  |  |  |  |  |
| e | Start of active video (T) | 192  6 – 0 |  |  |  |  |  |  |  |  |  |  |  |  |
| f | Rise/fall time (T) | 4  1.5 |  |  |  |  |  |  |  |  |  |  |  |  |
| – | Active line interval (T) | 1 920  0 – 12 |  |  |  |  |  |  |  |  |  |  |  |  |
| Sm | Amplitude of negative pulse (mV) | 300  6 |  |  |  |  |  |  |  |  |  |  |  |  |
| Sp | Amplitude of positive pulse (mV) | 300  6 |  |  |  |  |  |  |  |  |  |  |  |  |
| V | Amplitude of video signal (mV) | 700 |  |  |  |  |  |  |  |  |  |  |  |  |
| H | Total line interval (T) | 2200 |  |  |  |  | 2640 |  |  |  |  | 2750 |  |  |
| g | Half line interval (T) | 1100 |  |  |  |  | 1320 |  |  |  |  | 1375 |  |  |
| h | Vertical sync width (T) | 1 980  3 |  | 880  3 |  |  | 1 980  3 |  | 880  3 |  |  | 1 980  3 | 880  3 |  |
| k | End of vertical sync pulse (T) | 88  3 |  |  |  |  | 528  3 |  | 308  3 |  |  | 638  3 | 363  3 |  |
| (1) “T ” denotes the duration of a reference clock or the reciprocal of the clock frequency. (2) A “line” starts at line sync timing reference OH (inclusive), and ends just before the subsequent OH (exclusive). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Annex  1

to  Part  2

Segmented frame
(See Note 1)

NOTE 1 – The term segmented frame in the context of this Recommendation is intended to indicate that a picture has been captured in a progressive mode, and transported as two segments. One segment containing the odd lines of the progressive image, the second segment containing the even lines of the progressive image.

# 1	Background

The television systems in current use have typically used interlace capture (acquisition) and transmission. The frame/field rates of these systems have been 50/60 Hz, a rate that when presented on cathode ray tube (CRT) display devices did not require any associated picture flicker correction. Television systems of the future will support both interlace and progressive capture and display technology.

In addition to the support of interlace and progressive capture and display, there will be extended frame rates to be supported, along with new display technology. For a number of years there will be a mix of the old and new technologies.

Specifically, the PsF technology is intended to be implemented only when frames rates of 30 Hz and lower are being used.

A large percentage of television programming is produced on film that has a frame rate of 24 frames/s and sometimes 30 frames/s. Past practice was to perform post production by editing the film to produce a complete programme on film. The final film could be transferred to 60 Hz video by employing the 3:2 pull down technique. For 25 Hz release the film could be transferred by running the 24 frame film at 25 frames/s.

It is common practice to transfer the film to 60 Hz (field) interlace video for post production. Once the film is transferred, edit decision lists are created based on the 60 Hz (field) video rate, not the original 24 Hz original film frame rate. The conversion process from 24 Hz film to 60 Hz (field) video results in a number of operational impediments, such as tracking of 3:2 pull down, editing of split fields etc. In addition when 25 Hz video copies of the material are required, either reconforming is necessary, or standards conversion 30 Hz to 25 Hz, with a loss of quality.

Equipment is now available that will permit the transfer, post production and worldwide distribution of film originated material with the original frame rate of 24 frames/s.

# 2	24-frame/s production

Using the CIF of 1920  1080, film material may be transferred using progressive capture. This transfer will provide the highest resolution capture, with no 3:2 pull-down artifacts, moreover both 30 Hz frame rate and 25 Hz frame rate versions may be created from a single master with no quality loss.

The 30 Hz frame rate copy may be created by playing the 24-frame/s original and inserting the 3:2 pull-down during the replay process. This process also has the advantage of maintaining the 3:2 pull-down sequence during the replay process such that any downstream picture processing, such as an MPEG encoder, will not be affected by any 3:2 discontinuities.

The 25 Hz frame rate copy may be created by simply playing back the 24 Hz film rate original at the slightly faster 25 Hz rate; there is no picture quality loss.

In addition to simply transferring film originated material it is expected that electronic capture of images will occur at a 24-frame/s rate; this will provide the production community with yet another tool for seamless integration of images from various sources.

# 3	Progressive/interlace compatibility

The post production world has a need to cater for both progressive and interlace television signal formats for the foreseeable future. Therefore any new signal format such as 24P, the original film frame rate, will need to coexist with interlace formats of 25 Hz and 30 Hz systems. One of the constraints in monitoring the 24-frame/s systems is the picture flicker that is present when displaying a 24-frame/s signal on a CRT display. Interlace systems minimize this flicker by refreshing the CRT phosphors every 60th/50th of a second. There are at least two solutions to the flicker created by the 24-frame/s systems, install a frame store in every monitor, or provide to the monitor a signal that emulates the interlace refresh rate.

24PsF/25PsF/30PsF are transmission formats that will provide monitoring devices with signal refresh rates that will permit direct monitoring of the original frame rate of the material.

It should be noted that in some cases users may want to monitor 24-frame/30-frame material at other than the original frame rates.

The use of 24PsF/25PsF/30PsF does not in any way limit the monitoring of the signal by the newer flat panel displays.

A second potential use of the 24PsF/25PsF/30PsF transmission format is in the area of digital post production switchers. A common switcher design handling both interlace and progressive signals is economically possible, and addresses the requirements of end users who have a requirement to work in interlace and progressive formats with common equipment. The digital interface of an interlace signal and a PsF signal are common, only the signal content is different.

# 4	Signal mapping

The 24PsF/25PsF/30PsF transmission format maps a progressive image onto the interlace digital serial interface as defined in this Recommendation (see Fig. 11).

Line numbering convention for the image capture and image transmission is contained in the introduction of Part 2 (see also Fig. 11).

The same line numbers of an interlace picture are used by the PsF to carry the segmented frame format.

The sF format is not related to any interlace format characteristics. It is a way to convey a progressive image that has been captured at a 24/25/30 Hz rate. Capture at these low frequencies may require special monitoring considerations. The sF transmission format is intended to provide an economical solution while still retaining the compatibility with interlace systems.

Radiocommunication Study Group 6 made editorial amendments to this Recommendation in 2009 in accordance with Resolution ITU-R 1.

**	“A high-definition system is a system designed to allow viewing at about three times the picture height, such that the system is virtually, or nearly, transparent to the quality of portrayal that would have been perceived in the original scene or performance by a discerning viewer with normal visual acuity.” Report ITU-R BT.801.

This Part 1 specifies parameter values used in the early days of analogue high definition television implementation. It is provided here in recognition of the fact that it is no longer in use today, but vintage HDTV programs of historical values still exist in the form of analogue video tape recordings, that may occasionally need to be played back on appropriate equipment.