RECOMMENDATION  801

TEST  SIGNALS  FOR  DIGITALLY  ENCODED  COLOUR  TELEVISION  SIGNALS
CONFORMING  WITH  RECOMMENDATIONS  601  AND  656

(Question 25/11)

(1992)

Rec. 801

The CCIR,

considering

a)	that digital television systems operate in very different ways from analogue systems with the consequence that a quite different set of picture impairments may be introduced;

b)	that impairments may occur both from the conversions to and from the digital domain (which include filtering, sampling and quantization), and by degradations of the digital signal itself (such as individual digit errors, timing jitter or loss of frame synchronization);

c)	that for measurements of such impairments it is necessary to provide the test signals,

recommends

that for measurements of quantization errors and timing errors between analogue and digital active lines in conversion process from and to the digital signals conforming with Recommendation 601, using 8-bit quantization, and for verifying the conformity of the multiplex format with Recommendation 656, and checking for the correct operation of the associated interfaces, test signals used should be selected from the following list.

The test signals are listed in Table 1 and its brief description and precise sample values are annexed in Annexes 1 and 2, respectively.

TABLE  1

List of test signals

| No. | Title |
| --- | --- |
| 11 | grey |
| 12 | alternating white/black at 0.1 Hz |
| 13 | end-of-line pulses |
| 14 | black/white ramp |
| 15 | yellow/grey ramp |
| 16 | grey/blue ramp |
| 17 | cyan/grey ramp |
| 18 | grey/red ramp |
| 19 | CB, Y, CR, Y ramp |
| 10 | white, end-of-line porches |
| 11 | blue, end-of-line porches |
| 12 | red, end-of-line porches |
| 13 | yellow, end-of-line porches |
| 14 | cyan, end-of-line porches |
| 15 | digital colour-bar |

ANNEX  1

Brief description of test signals

The formulae corresponding to the test signals are defined in § 1, and the waveforms are illustrated in § 2.

# 1.	Formulae*

In cases where sample values are derived by computation, an addition of 0.5 is included in the formula to ensure that the appropriate level is obtained by rounding the result.

## 1.1	Test signal No. 1: grey

The active video lines of this signal are defined by:

Y(i)  =  Al(i),	CR  =  CB  =  128.

This signal is critical for transmission via a parallel interface, since each of the 8 interface data binary signals then contains a succession of bits 0, 1, 0, 1, 0, 1 . . . and attains maximum power concentration at high frequencies (multiples of 13.5 MHz) which often prove difficult to preserve in practical transmission links.

## 1.2	Test signal No. 2: alternating white/black at 0.1 Hz

This signal produces alternately:

–	for 5 s, pictures containing “white” digital active video lines defined by:

Y(i)  =  A2(i),	CR  =  CB  =  128;

–	for 5 s, pictures containing “black” digital active video lines defined by:

Y  =  16,	CR  =  CB  =  128.

This signal produces a variation of the black level in the corresponding analogue video signals, owing to the suppression of continuous components and very low frequencies by the analogue transmission links. It provides a means of checking the compensation for this variation, as well as black stability and accuracy in digital coding.

## 1.3	Test signal No. 3: end-of-line pulses

The signal’s digital active video lines are defined by:

Y(i)  =  A3(i),	CR  =  CB  =  128.

This four-pulse signal can be used to check the position of the digital active line in relation to the analogue reference, as well as the activity of samples situated at the end of the digital active line. The outside edges of the two internal pulses coincide with the ends of the line, in the 625/50 system.

## 1.4	Test signal No. 4: black/white ramp

The digital active video lines of this signal are defined by:

Y(i)  =  int (A4(i)),	CR  =  CB  =  128.

This signal may be used to test the existence and position of quantization levels 1 to 254 of the luminance signal.

## 1.5	Test signal No. 5: yellow/grey ramp

The digital active lines of this signal are defined by:

CB(i)	=  int (A5(i))

CR(i)	=  int (128.5 – (0.114 / 0.701) (A5(i) – 128))

Y(i)		=  int (126 – (169 / 224) (A5(i) – 128))

This signal can be used to test the existence and position of quantization levels 1 to 128 of the colour difference signal CB.

## 1.6	Test signal No. 6: grey/blue ramp

The digital active video lines of this signal are defined by the same formulae as in § 1.5, replacing A5 by A6.

This signal can be used to test the existence and position of quantization levels 128 to 254 of the colour difference signal CB.

## 1.7	Test signal No. 7: cyan/grey ramp

The digital active video lines of this signal are defined by:

CB(i)	=  int (128.5 – (0.299 / 0.886) (A5(i) – 128))

CR(i)	=  int (A5(i))

Y(i)		=  int (126 – (88 / 224) (A5(i) – 128))

This signal may be used to test the existence and position of quantization levels 1 to 128 of the colour difference signal CR.

## 1.8	Test signal No. 8: grey/red ramp

The digital active video lines of this signal are defined by the same formulae as in § 1.7, replacing A5 by A6.

This signal may be used to test the existence and position of quantization levels 128 to 254 of the colour difference signal CR.

## 1.9	Test signal No. 9: CB , Y, CR , Y ramp

The active video lines of this signal are defined by A7(i) in Table 2 for 1 440 samples of the digital active line multiplex.

This signal is useful for testing the conformity of the digital video signal format at the output of the digital processing equipment carrying out demultiplexing and remultiplexing operations on the components of the digital video signal.

Note 1 – This signal produces spurious colours in the R, G, B field.

## 1.10	Test signal No. 10: white, end-of-line porches

The active video lines of this signal are defined by:

Y(i)  =  A8(i),	CB  =  CR  =  128.

This signal has no shaping of the transitions on Y at the ends of the digital active line and is useful for observing the analogue shaping of the line blankings by the 4:2:2 decoders.

Two integral transitions of the Blackman pulse with a rise time of 300 ns are placed 3 ms from the leading and trailing edges of analogue line blankings for 625-line systems, permitting comparative observation of the transitions and verification of the conformity of the digital-analogue time correspondence on Y.

## 1.11	Test signal No. 11: blue, end-of-line porches

The active video lines of this signal are defined by:

Y  =  41,	CB(i)  =  A9(i),	CR  =  110.

This signal can be used to make the observations described in § 1.10 for high transitions on CB.

## 1.12	Test signal No. 12: red, end-of-line porches

The active video lines of this signal are defined by:

Y  =  81,	CB  =  90,	CR  =  A9(i).

This signal can be used to make the observations described in § 1.10 for high transitions on CR.

## 1.13	Test signal No. 13: yellow, end-of-line porches

The active video lines of this signal are defined by:

Y  =  210,	CB(i)  =  A10(i),	CR  =  146.

This signal can be used to make the observations described in § 1.10 for low transitions on CB.

## 1.14	Test signal No. 14: cyan, end-of-line porches

The active video lines of this signal are defined by:

Y  =  170,	CB  =  166,	CR(i)  =  A10(i).

This signal can be used to make the observations described in § 1.10 for low transitions on CR.

## 1.15	Digital colour bar signals

The frequent use of colour bar signals in analogue television suggests the need to define such encoded signals for digital, in order to monitor levels and phasing between components after 4:2:2 decoding.

Tables 3a) and 3b) give a description of 100/0/100/0 and 100/0/75/0 colour bars calculated by means of mathematical equations with the following characteristics:

–	shaping of transitions by integral of the Blackman impulse;

–	rise time 10% to 90% for Y  =  150 ns;

–	rise time 10% to 90% for CB and CR  =  300 ns.

# 2.	Waveforms of test signals

Figures as follows indicate sample levels.

## 2.1	Grey: A1

Figure  [D01]  =  1.5 cm

## 2.2	White: A2

Figure  [D02]  =  2.5 cm

## 2.3	End-of-line pulses: A3

Figure  [D03]  =  2.5 cm

## 2.4	Black/white ramp: A4

Figure  [D04]  =  2.5 cm

## 2.5	Yellow/grey and cyan/grey ramp: A5

Figure  [D05]  =  2 cm

## 2.6	Grey/blue and grey/red ramp: A6

Figure  [D06]  =  2 cm

## 2.7	CB , Y, CR , Y ramp: A7

Figure  [D07]  =  2.5 cm

## 2.8	White, end-of-line porches: A8

Figure  [D08]  =  2.5 cm

## 2.9	Blue and red, end-of-line porches: A9

Figure  [D09]  =  2 cm

## 2.10	Yellow and cyan, end-of-line porches: A10

Figure  [D10]  =  2 cm

ANNEX  2

Sample values corresponding to test signal

TABLE  2

Table of values used for defining digital test signals

A1: Grey

| i | 0 to 19 | 20 | 21 | 22 | 23 | 24 | 25 to 693 | 694 | 695 | 696 | 697 | 698 | 699 to 719 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A1(i) | 16 | 18 | 33 | 72 | 110 | 125 | 127 | 125 | 110 | 72 | 33 | 18 | 16 |

A2: White

| i | 0 to 19 | 20 | 21 | 22 | 23 | 24 | 25 to 693 | 694 | 695 | 696 | 697 | 698 | 699 to 719 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A2(i) | 16 | 19 | 50 | 126 | 201 | 232 | 235 | 232 | 201 | 126 | 50 | 19 | 16 |

A3: End-of-line pulses

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 to 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 to 705 | 706 | 707 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A3(i) | 16 | 44 | 154 | 235 | 154 | 44 | 16 | 17 | 64 | 185 | 229 | 121 | 31 | 16 | 17 | 64 |

| i | 708 | 709 | 710 | 711 | 712 | 713 | 714 | 715 | 716 | 717 | 718 | 719 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A3(i) | 185 | 229 | 121 | 31 | 16 | 16 | 44 | 154 | 235 | 154 | 44 | 16 |

A4: Black/white ramp

| i | 0 to 20 | 21 | 22 | 23 | 24 to 59 | 60 to 87 | 88 to 99 | 100 to 535 | 536 to 549 | 550 to 585 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A4(i) | 16 | 14 | 9 | 3 | 1 | ((i – 56) / 2) | 16 | ((i – 66) / 2) | 235 | ((i – 78) / 2) |

| i | 586 to 599 | 600 | 601 | 602 | 603 | 604 | 605 to 719 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A4(i) | 254 | 250 | 217 | 135 | 53 | 20 | 16 |

i : sample number and takes on values from 0 to 719.

TABLE  2  (continued)

A5: Yellow/grey and cyan/grey ramp

| i | 0 to 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 to 39 | 40 to 95 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A5(i) | 128 | 126 | 120 | 108 | 89 | 65 | 40 | 21 | 9 | 3 | 1 | ((i – 32) / 4) |

| i | 96 to 119 | 120 to 563 | 564 to 719 |
| --- | --- | --- | --- |
| A5(i) | 16 | ((i – 52) / 4) | 128 |

A6: Grey/blue and grey/red ramp

| i | 0 to 19 | 20 to 563 | 564 to 579 | 580 to 631 | 632 to 659 | 660 | 661 | 662 | 663 | 664 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A6(i) | 128 | ((i + 396) / 4) | 240 | ((i + 384) / 4) | 254 | 252 | 246 | 234 | 215 | 191 |

| i | 665 | 666 | 667 | 668 | 669 to 719 |
| --- | --- | --- | --- | --- | --- |
| A6(i) | 167 | 148 | 136 | 130 | 128 |

A7: CB, Y, CR, Y ramp

| i | 0 to 253 | 254 to 507 | 508 to 761 | 762 to 1 015 | 1 016 to 1 269 | 1 270 to 1 439 |
| --- | --- | --- | --- | --- | --- | --- |
| A7(i) | i + 1 | 508 – i | i – 507 | 1 016 – i | i – 1 015 | 1 524 – i |

A8: White, end-of-line porches

| i | 0 to 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 to 667 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A8(i) | 235 | 232 | 218 | 187 | 139 | 86 | 46 | 24 | 17 | 16 |

| i | 668 | 669 | 670 | 671 | 672 | 673 | 674 | 675 | 676 to 719 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A8(i) | 19 | 33 | 64 | 112 | 165 | 205 | 227 | 234 | 235 |

TABLE  2  (continued)

A9: Blue and red, end-of-line porches

| i | 0 to 23 | 24 | 25 | 26 | 27 to 333 | 334 | 335 | 336 | 337 | 338 to 359 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A9(i) | 240 | 232 | 191 | 143 | 128 | 130 | 152 | 204 | 236 | 240 |

A10: Yellow and cyan, end-of-line porches

| i | 0 to 23 | 24 | 25 | 26 | 27 to 333 | 334 | 335 | 336 | 337 | 338 to 359 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A10(i) | 16 | 24 | 65 | 113 | 128 | 126 | 104 | 52 | 20 | 16 |

TABLE  3

Description of encoded colour-bar signals according to the 4:2:2 level of Recommendation 601

a)	Designation:  100/0/100/0 colour bars

Definition of Y for digital active line with rise time  =  150 ns

| i | 0 to 13 | 14 | 15 | 16 | 17 | 18 | 19 to 99 | 100 | 101 | 102 | 103 | 104 | 105 to 185 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y(i) | 16 | 16 | 39 | 126 | 212 | 235 | 235 | 235 | 232 | 223 | 213 | 210 | 210 |

| i | 186 | 187 | 188 | 189 | 190 | 191 to 271 | 272 | 273 | 274 | 275 | 276 | 277 to 357 | 358 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y(i) | 210 | 206 | 190 | 174 | 170 | 170 | 169 | 167 | 157 | 147 | 145 | 145 | 144 |

| i | 359 | 360 | 361 | 362 | 363 to 443 | 444 | 445 | 446 | 447 | 448 | 449 to 529 | 530 | 531 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y(i) | 141 | 126 | 110 | 107 | 106 | 106 | 104 | 94 | 84 | 82 | 81 | 81 | 77 |

| i | 532 | 533 | 534 | 535 to 615 | 616 | 617 | 618 | 619 | 620 | 621 to 719 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y(i) | 61 | 45 | 41 | 41 | 41 | 38 | 28 | 19 | 16 | 16 |

i : sample number and takes on values from 0 to 719.

TABLE  3  (continued)

Definition of CR for digital active line with rise time  =  300 ns

| i | 0 to 5 | 6 | 7 | 8 | 9 | 10 | 11 to 48 | 49 | 50 | 51 | 52 | 53 | 54 to 91 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR(i) | 128 | 128 | 128 | 128 | 128 | 128 | 128 | 128 | 130 | 137 | 144 | 146 | 146 |

| i | 92 | 93 | 94 | 95 | 96 | 97 to 134 | 135 | 136 | 137 | 138 | 139 | 140 to 177 | 178 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR(i) | 146 | 133 | 81 | 29 | 16 | 16 | 16 | 18 | 25 | 32 | 34 | 34 | 35 |

| i | 179 | 180 | 181 | 182 | 183 to 220 | 221 | 222 | 223 | 224 | 225 | 226 to 263 | 264 | 265 | 266 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR(i) | 54 | 128 | 202 | 221 | 222 | 222 | 224 | 231 | 238 | 240 | 240 | 240 | 227 | 175 |

| i | 267 | 268 | 269 to 306 | 307 | 308 | 309 | 310 | 311 | 312 to 359 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR(i) | 123 | 110 | 110 | 110 | 112 | 119 | 126 | 128 | 128 |

Definition of CB for digital active line with rise time  =  300 ns

| i | 0 to 5 | 6 | 7 | 8 | 9 | 10 | 11 to 48 | 49 | 50 | 51 | 52 | 53 | 54 to 91 | 92 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CB(i) | 128 | 128 | 128 | 128 | 128 | 128 | 128 | 128 | 116 | 72 | 28 | 16 | 16 | 16 |

| i | 93 | 94 | 95 | 96 | 97 to 134 | 135 | 136 | 137 | 138 | 139 | 140 to 177 | 178 | 179 | 180 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CB(i) | 31 | 91 | 150 | 166 | 166 | 166 | 154 | 110 | 65 | 54 | 54 | 54 | 69 | 128 |

| i | 181 | 182 | 183 to 220 | 221 | 222 | 223 | 224 | 225 | 226 to 263 | 264 | 265 | 266 | 267 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CB(i) | 187 | 202 | 202 | 202 | 191 | 146 | 102 | 90 | 90 | 90 | 106 | 165 | 225 |

| i | 268 | 269 to 306 | 307 | 308 | 309 | 310 | 311 | 312 to 359 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CB(i) | 240 | 240 | 240 | 228 | 184 | 140 | 128 | 128 |

TABLE  3  (continued)

b)	Designation:  100/0/75/0 colour bars

Definition of Y for digital active line with rise time  =  150 ns

| i | 0 to 13 | 14 | 15 | 16 | 17 | 18 | 19 to 99 | 100 | 101 | 102 | 103 | 104 | 105 to 185 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y(i) | 16 | 16 | 39 | 126 | 212 | 235 | 235 | 235 | 227 | 198 | 169 | 162 | 162 |

| i | 186 | 187 | 188 | 189 | 190 | 191 to 271 | 272 | 273 | 274 | 275 | 276 | 277 to 357 | 358 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y(i) | 161 | 158 | 146 | 134 | 131 | 131 | 131 | 129 | 122 | 114 | 112 | 112 | 112 |

| i | 359 | 360 | 361 | 362 | 363 to 443 | 444 | 445 | 446 | 447 | 448 | 449 to 529 | 530 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y(i) | 109 | 98 | 87 | 84 | 84 | 84 | 82 | 74 | 67 | 65 | 65 | 65 |

| i | 531 | 532 | 533 | 534 | 535 to 615 | 616 | 617 | 618 | 619 | 620 | 621 to 719 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y(i) | 62 | 50 | 38 | 35 | 35 | 35 | 33 | 25 | 18 | 16 | 16 |

Definition of CR for digital active line with rise time  =  300 ns

| i | 0 to 5 | 6 | 7 | 8 | 9 | 10 | 11 to 48 | 49 | 50 | 51 | 52 | 53 | 54 to 91 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR(i) | 128 | 128 | 128 | 128 | 128 | 128 | 128 | 128 | 129 | 135 | 140 | 142 | 142 |

| i | 92 | 93 | 94 | 95 | 96 | 97 to 134 | 135 | 136 | 137 | 138 | 139 | 140 to 177 | 178 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR(i) | 141 | 132 | 93 | 54 | 44 | 44 | 44 | 45 | 51 | 56 | 58 | 58 | 58 |

| i | 179 | 180 | 181 | 182 | 183 to 220 | 221 | 222 | 223 | 224 | 225 | 226 to 263 | 264 | 265 | 266 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR(i) | 72 | 128 | 184 | 198 | 198 | 198 | 200 | 205 | 211 | 212 | 212 | 212 | 202 | 163 |

| i | 267 | 268 | 269 to 306 | 307 | 308 | 309 | 310 | 311 | 312 to 359 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR(i) | 124 | 115 | 114 | 114 | 116 | 121 | 127 | 128 | 128 |

TABLE  3  (continued)

Definition of CB for digital active line with rise time  =  300 ns

| i | 0 to 5 | 6 | 7 | 8 | 9 | 10 | 11 to 48 | 49 | 50 | 51 | 52 | 53 | 54 to 91 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CB(i) | 128 | 128 | 128 | 128 | 128 | 128 | 128 | 128 | 119 | 86 | 53 | 44 | 44 |

| i | 92 | 93 | 94 | 95 | 96 | 97 to 134 | 135 | 136 | 137 | 138 | 139 | 140 to 177 | 178 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CB(i) | 44 | 56 | 100 | 145 | 156 | 156 | 156 | 148 | 114 | 81 | 73 | 72 | 73 |

| i | 179 | 180 | 181 | 182 | 183 to 220 | 221 | 222 | 223 | 224 | 225 | 226 to 263 | 264 | 265 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CB(i) | 84 | 128 | 172 | 183 | 184 | 183 | 175 | 142 | 108 | 100 | 100 | 100 | 111 |

| i | 266 | 267 | 268 | 269 to 306 | 307 | 308 | 309 | 310 | 311 | 312 to 359 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CB(i) | 156 | 200 | 212 | 212 | 212 | 203 | 170 | 137 | 128 | 128 |

_________________

*	Y, CR, CB sample numbering is in accordance with Recommendation 656.

These digital waveforms are made up of pulses in uniform ranges, ramps between two uniform ranges, and transitions between two uniform ranges, shaped by a filter whose impulse response R(t) is defined as a function of time t as follows:

–     for – 3T < t < 3T,	R(t)  =  0.42  +  0.50 cos(p t / 3T)  +  0.08 cos(2p t / 3T)

–     otherwise R(t)  =  0

(R(t) : Blackman window).

The value of T is 74 ns for digital waveforms A1, A2, A3 and A4 and 148 ns for A5 and A6.