Recommendation ITU-R F.1333-1 (05/1999)

Estimation of the actual elevation angle from a station in the fixed service towards a space station taking into account atmospheric refraction

F Series Fixed service

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Annex 1 of Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from http://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Recommendations (Also available online at http://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radiowave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2010

 ITU 2010

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION  ITU-R  F.1333-1*

ESTIMATION  OF  THE  ACTUAL  ELEVATION  ANGLE  FROM  A  STATION
IN  THE  FIXED  SERVICE  TOWARDS  A  SPACE  STATION  TAKING
INTO  ACCOUNT  ATMOSPHERIC  REFRACTION

(1997-1999)

Rec. ITU-R F.1333-1

Scope

This Recommendation provides a method of calculating the actual elevation angle as seen from a station in the fixed service towards a space station where the elevation angle towards the station is known only under free-space propagation conditions in a vacuum. The method is developed using the mean annual global reference atmosphere given in Recommendation ITU-R P.835.

The ITU Radiocommunication Assembly,

considering

a)	that in some frequency sharing studies between the fixed service and space radiocommunication services, including fixed-satellite, broadcasting-satellite and space science services, it is necessary to estimate various factors related to propagation such as slant path attenuation by atmospheric gases and attenuation due to Fresnel zone blockage;

b)	that the above attenuation is a function of the actual elevation angle as seen from a station in the fixed service towards a space station (geostationary or non-geostationary);

c)	that it is necessary to establish a simplified calculation method to estimate the actual elevation angle towards a space station taking into account atmospheric refraction where the elevation angle of a space station is known only in free-space propagation conditions in a vacuum;

d)	that it is generally appropriate to estimate the actual elevation angle of a space station under the condition of the mean annual global reference atmosphere given in Recommendation ITU-R P.835,

recommends

1	that the method for estimating the actual elevation angle as seen from a station in the fixed service towards a space station (geostationary or non-geostationary) under the condition of the mean annual global reference atmosphere in Recommendation ITU-R P.835 as described in Annex 1 should be used where the elevation angle towards a space station is known only in free-space propagation conditions in vacuum (see Notes 1 and 2);

2	that for an atmospheric refractivity model other than the mean annual global reference atmosphere as given in Recommendation ITU-R P.835, a different numerical formula should be derived corresponding to the specific atmospheric refractivity model (see Note 3).

NOTE 1 – This Recommendation can be used, for example, for estimation of slant path attenuation by atmospheric gases and Fresnel zone blockage attenuation required in Recommendation ITU-R F.1249.

NOTE 2 – Annex 1 to Recommendation ITU-R F.1108 gives an example method for calculating the elevation angle towards a non-geostationary space station in free-space propagation conditions, and Annex 2 to Recommendation ITU-R F.1249 gives a method for calculating the elevation angle towards a geostationary space station in free-space propagation conditions.

NOTE 3 – Annex 2 to Recommendation ITU-R SF.765 (and also Annex 2 to Recommendation ITU-R F.1249) gives an example of the numerical formulae for refraction corrections corresponding to the exponential model of N0  400 and N  – 68 (for maximum refraction correction) and the exponential model of N0  250 and N  – 30 (for minimum refraction correction), where N0 is the sea-level radio refractivity and N is the gradient (difference between sea level and 1 km altitude), which are used for determination of the separation angle between the fixed service antenna main beam direction and the direction towards the geostationary-satellite orbit (GSO) (or a specific orbital location in the GSO).

ANNEX  1

Actual elevation angle where the elevation angle of a space station
is known only in free-space propagation conditions

# 1	Introduction

In some cases of sharing studies, the elevation angle of a space station (geostationary or non-geostationary), as seen from a fixed service station, is known only in free-space propagation conditions in a vacuum. In such cases, it is necessary to estimate the actual elevation angle taking account of atmospheric refraction. This annex gives a calculation method for such a need.

# 2	Visibility of space station

Under a quasi-exponential model of atmosphere for refraction, such as in Recommendation ITU-R P.835, the radio beam emitted from a fixed service station (h (km) altitude above sea level and  (degrees) elevation angle) is bent towards the Earth due to the effect of atmospheric refraction. This refraction correction,  (degrees), can be evaluated by the following integral:

t=−h¥n'(x)n(x)⋅tanjdx	(1)

where  is determined as follows on the basis of Snell's law in polar coordinates:

cos     c(r+x)⋅n(x)	(2)

c    (r    h)    n(h)    cos 	(3)

where:

r :	Earth's radius (6 370 km).

The function n(x) is the atmospheric refractive index at altitude x (km), and can be calculated by using the formula for the radio refractive index in Recommendation ITU-R P.453 based on the mean annual global reference atmosphere in Recommendation ITU-R P.835. In addition, n'(x) is the derivative of n(x).

The values of  (h, ) (degree) have been evaluated under the condition of the mean annual global reference atmosphere and it was found that the following numerical formula gives a good approximation:

 (h, )    1 / [1.283    0.7491     0.01986 2    h (0.3114    0.07020 )]	(4)

The above formula has been derived as an approximation for 0  h  3 km and m    90°, where m is the angle at which the radio beam is just intercepted by the surface of the Earth and is given by:

m    – arc cos  rr+hn0nh	(5)

or, approximately, m  – 0.875  degrees.

Now, assume that the elevation angle of a space station is 0 (degrees) under free-space propagation conditions. The minimum elevation angle from a fixed service station for which the radio beam is not intercepted by the surface of the Earth is m. The refraction correction corresponding to m is  (h, m). Therefore, the space station is visible only when the following inequality holds:

m  –  (h, m)   0	(6)

# 3	Estimation of the actual elevation angle

When the inequality in equation (6) holds, the actual elevation angle,  (degrees), taking account of atmospheric refraction can be calculated by solving the following equation:

  –  (h, )    0	(7)

We define that the solution of equation (7) is given as follows:

    0    s(h, 0)	(8)

where the values of s (h, 0) are identical with those of  (h, ), but are expressed as a function of 0.

The function s (h, 0) (degrees) can be closely approximated by the following numerical formula:

s (h, 0)	  1 / [1.712    0.5507 0    0.03424 02  

  h (0.2584    0.07940 0    0.01034 02)]	(9)

Now, the value of  calculated by equation (8) is the actual elevation angle to be used in the estimation of various factors such as slant path attenuation and attenuation due to Fresnel zone blockage.

# 4	Summary of calculations

Step 1:	The elevation angle of a space station in free-space propagation conditions in a vacuum is designated as 0.

Step 2:	By using equations (4) and (5), examine whether the inequality in equation (6) holds or not. If the answer is no, the satellite is not visible and, therefore, no further calculations are required.

Step 3:	If the answer in Step 2 is yes, calculate  by using equations (8) and (9). This is the actual elevation angle to be used for estimation of various factors such as slant path attenuation and attenuation due to Fresnel zone blockage.

*	Radiocommunication Sudy Group 5 made editorial amendments to this Recommendation in December 2009 in accordance with Resolution ITU-R 1.