RECOMMENDATION  ITU-R  F.1491

ERROR  PERFORMANCE  OBJECTIVES  FOR  REAL  DIGITAL  RADIO  LINKS  USED
IN  THE  NATIONAL  PORTION  OF  A  27 500  km  HYPOTHETICAL
REFERENCE  PATH  AT  OR  ABOVE  THE  PRIMARY  RATE

(Question ITU-R 210/9)

(2000)

Rec. ITU-R F.1491

The ITU Radiocommunication Assembly,

considering

a)	that ITU-T has specified in ITU-T Recommendation G.826 the error performance parameters and objectives for national constant bit rate (CBR) digital paths at or above the primary rate;

b)	that ITU-T has specified in ITU-T Recommendation G.829 the error performance events and the block structures for synchronous digital hierarchy (SDH) multiplex and regenerator sections;

c)	that Recommendation ITU-R F.1189 provides the error performance objectives for CBR digital path at or above the primary rate carried by digital radio-relay systems which may form part of the national portion of a 27 500 km hypothetical reference path (HRP);

d)	that any real path/link for digital data transmission at or above the primary rate may be realized using a linear and/or a redundant topology, depending on the needs of network providers;

e)	that there is a need to establish the performance objectives for real digital radio links in order to allow a proper engineering of the radio links;

f)	that for the purpose of this Recommendation, the national portion of a 27 500 km HRP can be subdivided into three basic sections (see Fig. 1),

FIGURE 1 / F.1491-01] = 11 CM

recommends

1	that the error-performance objectives for the access and short haul sections shall make use only of the block allocation contribution specified in ITU-T Recommendation G.826 for the national portion, and that the long haul section shall make use of the distance-based allocation and part of the fixed block allowance allocation;

2	that for the error performance objectives applicable to each direction of any real radio link of length Llink belonging to the long haul inter-exchange network sections of the national portion of HRP the values in Table 1 should apply. The lower limit of Llink, used to scale the objectives to the real case, is Lmin  50 km.

TABLE  1

Error performance objectives for real radio-relay links belonging to the long haul
inter-exchange network section of the national portion of the HRP
at or above the primary rate

| Rate (Mbit/s) | 1,5 to 5 |  5 to 15 |  15 to 55 |  55 to 160 |  160 to 3 500 |
| --- | --- | --- | --- | --- | --- |
| Errored second ratio (ESR) | 0.04 A | 0.05 A | 0.075 A | 0.16 A | For further study |
| Severely errored second ratio (SESR) | 0.002 A | 0.002 A | 0.002 A | 0.002 A | 0.002 A |
| Background block error ratio (BBER) | 2 A  10– 4 (see Note 10) | 2 A 10– 4 | 2 A 10– 4 | 2 A 10– 4 | 1 A 10– 4 |

where:

A  (A1  0.01) Llink/500	for   50 km  Llink  500 km

A  =  A1  2  10–5 Llink	for   500 km  Llink

A1 has provisionally been agreed to be in the range of 0.01 to 0.02 (1% to 2%) (see Notes 3 and 4);

3	that for the error performance objectives applicable to each direction of any real radio link forming all of the short haul network sections of the national portion of the HRP the values given in Table 2 should apply.

TABLE  2

Error performance objectives for radio-relay links forming all of the short haul
inter-exchange network section of the national portion of the HRP
at or above the primary rate

| Rate (Mbit/s) | 1.5 to 5 |  5 to 15 |  15 to 55 |  55 to 160 |  160 to 3 500 |
| --- | --- | --- | --- | --- | --- |
| ESR | 0.04 B | 0.05 B | 0.075 B | 0.16 B | For further study |
| SESR | 0.002 B | 0.002 B | 0.002 B | 0.002 B | 0.002 B |
| BBER | 2 B 10– 4 (see Note 10) | 2 B 10– 4 | 2 B 10– 4 | 2 B 10– 4 | 1 B 10– 4 |

The value of B has provisionally been agreed to be in the range of 0.075 to 0.085 (7.5% to 8.5%) (see Notes 3 and 4);

4	that for the error performance objectives applicable to each direction of any real radio link forming all of the access network sections of the national portion of the HRP the values given in Table 3 should apply.

TABLE  3

Error performance objectives for radio-relay links forming all of
the access network section  of the national portion of
the HRP at or above the primary rate (see Note 6)

| Rate (Mbit/s) | 1.5 to 5 |  5 to 15 |  15 to 55 |  55 to 160 |  160 to 3 500 |
| --- | --- | --- | --- | --- | --- |
| ESR | 0.04 C | 0.05 C | 0.075 C | 0.16 C | For further study |
| SESR | 0.002 C | 0.002 C | 0.002 C | 0.002 C | 0.002 C |
| BBER | 2 C 10– 4 (see Note 10) | 2 C 10– 4 | 2 C 10– 4 | 2 C 10– 4 | 1 C 10– 4 |

The value of C has provisionally been agreed to be in the range of 0.075 to 0.085 (7.5% to 8.5%) (see Notes 3 and 4);

5	that for the error performance objectives evaluation in recommends 1, the error performance parameters for any real link are defined as follows:

–	SESR is the ratio of severely errored second (SES) events to total seconds in the available time during a fixed measurement interval;

–	BBER is the ratio of background block error (BBE) events to total blocks in the available time during a fixed measurement interval. The count of total blocks excludes all blocks during SESs;

–	ESR is the ratio of errored second (ES) events to total seconds in the available time during a fixed measurement interval.

- Examples of real links in the national portion of HRP and application to real cases are given in Annex 1.

NOTE 1 – The error performance objectives apply only when the system is considered to be available. The entry and exit criteria into and from the unavailable state are defined in Annex A of ITU-T Recommendation G.826.

NOTE 2 – The objectives given in this Recommendation are understood to be long-term objectives to be met over an evaluation period of typically 30 consecutive days (1 month). These objectives should be respected for any month.

NOTE 3 – The sum of the percentages A1%  B%  C% shall not exceed 17.5%, in accordance with the allocations to the national portion of an international CBR path given in ITU-T Recommendation G.826.

NOTE 4 – The provisional values agreed for B%  C% are in the range 15.5% to 16.5%.

NOTE 5 – Depending on national network configurations administrations may reallocate the A%, B% and C% block allowances among the sections of the national portion of a radio path.

NOTE 6 – There is a great variety in the architecture of access networks in different countries. If the radio path forms only part of the short haul or access network section, it is at the discretion of administrations to make an appropriate apportionment of the objectives given in Tables 2 and 3 as a block allowance to the elements forming the short haul or access network section.

NOTE 7 – In the case of multi-hop links the objectives derived according to this Recommendation apply to the overall links; the allocation of the objectives to each hop is under the responsibility of the network operators.

NOTE 8 – The SES, BBE and ES events and the block structure for SDH multiplex and regenerator sections are defined in ITU-T Recommendation G.829, the SES, BBE and ES events and the block structure for paths are defined in ITU-T Recommendation G.826 (see Annex B).

NOTE 9 – The ESR objective for higher bit rate (160 to 3 500 Mbit/s) paths is still under study.

NOTE 10 – For systems designed prior to 1996, the BBER objective is 3  10– 4

NOTE 11 – The effect of interference and all other sources of performance degradations are included in Tables 1, 2 and 3.

ANNEX  1

Examples of calculation for real links

The examples of calculation below refer to a generic national portion configuration, as shown in Fig. 2.

FIGURE 1 / F.1491-02] = 8 CM

Example 1:  the Access portion of the network is 20 km long and it is formed by a single link:

Link L1  20 km

Capacity: 2 Mbit/s.

In this case the objectives are length independent; if C  0.075 is assumed (see recommends 4) we have:

SESR  0.002 C  1.5  10– 4 (equivalent to 389 SES/month)

ESR  0.04 C  3  10–3 (equivalent to 7 776 ES/month)

BBER  2  10– 4  B  1.5  10–5 (equivalent to 77 760 EB/month)

EB :  errored block.

Example 2:  the short haul portion of the network is 80 km long and it is formed by a single link:

Link L2  80 km

Capacity: 34 Mbit/s.

In this case the objectives are length independent; if B = 0.075 is assumed (see recommends 3) we have:

SESR  0.002 B  1.5  10– 4 (equivalent to 389 SES/month)

ESR  0.075 B  5.625  10–3 (equivalent to 14 580 ES/month)

BBER  2  10– 4  B  1.5  10–5 (equivalent to 311 040 EB/month)

Example 3:  real link in long haul portion of the network:

Link L3  105 km

SDH transmission rate: synchronous transport module, STM-1 (155.52 Mbit/s):

SESR  0.002 A  0.002 (A1  0.01)  105/500

ESR  0.16 A  0.16 (A1  0.01)  105/500

BBER  0.0002 A  0.0002 (A1  0.01)  105/500

In this case the objectives are length dependent; in Table 4 the minimum and the maximum limits (A1  0.01 and A1  0.02) are shown:

TABLE  4

Values for the objectives

| A1 value | SESR | ESR | BBER |
| --- | --- | --- | --- |
| 0.01 | 8  10–6 ( 22 SES/month) | 6.72  10– 4 ( 1 742 ES/month) | 8.4  10–7 ( 17 418 EB/month) |
| 0.02 | 1.26  10–5 ( 33 SES/month) | 1.008  10–3 ( 2 613 ES/month) | 1.26  10–6 ( 26 127 EB/month) |

NOTE 1 – Rounding to nearest integer has been used for fractional results.

ANNEX  2

Error performance events for different SDH radio-relay link configurations

# 1	Introduction

In a radio link the link end-points, which are represented by the radio terminal at the two sides of the link, may terminate a path, multiplex section and regenerator section. All configurations are possible according to definition given in ITU-T Recommendation G.783 and Recommendation ITU-R F.750.

The examples below show the relationship between the main practicable configuration for SDH link and the estimation of error performance events (SES, ES, BBE). The radio specific performance monitoring in presence of a protection switching is defined in Recommendation ITU-R F.750.

# 2	Link-end points are path end-points with and without frequency diversity

The B3 or V5 byte, dealing with high order path and low order path respectively, are calculated/evaluated in both link end-points.

Error performance events are defined in ITU-T Recommendation G.826. The values of relative error performance parameters, obtained by means of B3 or V5 byte according to ITU-T Recommendations G.826 and G.783, are fully compatible with Recommendations ITU-R F.1092 and ITU-R F.1189.

# 3	Link end-points are SDH multiplex section (MS) end-points

## 3.1	One hop MS without frequency diversity protection

The B2 byte is calculated/evaluated in both link end-points, while the B3 and V5 byte are passed transparently through the link end-points without modification.

Error performance events are defined in ITU-T Recommendation G.829. The comparison of the values of relative error performance parameters, obtained by means of B2 byte according to ITU-T Recommendations G.829 and G.783, with the objectives defined in Recommendations ITU-R F.1092 and ITU-R F.1189 could be considered as an estimation. The accuracy depends on the number of errors per burst.

## 3.2	One hop MS with frequency diversity protection

The error performance monitoring functionalities of the protected section, i.e. the section outside the protection switching depends on the allocation of SDH radio protection switching (RPS) function blocks, as described in Recommendation ITU-R F.750.

In case of type C allocation, defined in Recommendation ITU-R F.750, the B1 byte is calculated/evaluated in both link end-points per each channel (i.e. working channels and protecting channel). The B2 is calculated/evaluated in both link end-points outside the protection section, so it gives directly the performance of the protected section. The B3 and V5 byte are passed through the link end-points without modification.

Error performance events are defined in ITU-T Recommendation G.829. The comparison of the values of relative error performance parameters, obtained by means of B2 byte according to ITU-T Recommendations G.829 and G.783, with the objectives defined in Recommendations ITU-R F.1092 and ITU-R F.1189 could be considered as an estimation. The accuracy depends on the number of errors per burst.

In case of type B allocation, defined in Recommendation ITU-R F.750, the B1 and B2 bytes are calculated/evaluated in both link end-points per each channel (i.e. working channels and protecting channel). The B3 and V5 byte are passed through the link end-points without modification.

Two processes may, in this case, be possible for the radio protected section quality:

–	the first one is to evaluate separately the quality of the STM-N signal at the input and at the output from the protected radio section by means of not intrusive monitoring of B3 bytes and to let management system provide the difference;

–	the second one is to send forward, through a media dependent byte of the regenerator section overhead (RSOH) passed transparently through any intermediate repeater acting as MS, the bit interleaved parity, BIP-8 equivalent information of input EB towards the far end terminal. The far-end terminal may evaluate the difference with the output quality and directly providing management system with the actual quality of the protected radio section.

This methodology of MS tandem connections monitoring, is in principle similar to the high order tandem connections monitoring defined by ITU-T Recommendations G.707 and G.783, but no parity recovery algorithm, as that of N1 byte of VC-4 path overhead (PoH), is required. More details on performance monitoring of radio implementing a protection switch can be found in Recommendation ITU-R F.750.

# 4	Link end-points are SDH regenerator section (RS) end-points

## 4.1	One hop RS without frequency diversity protection

The B1 byte is calculated/evaluated in both link end-points, while the B2, B3 and V5 byte are passed through the link end-points without modification.

Error performance events are defined in ITU-T Recommendation G.829. The values of relative error performance parameters, obtained by means of B1 byte according to ITU-T Recommendations G.829 and G.783, are fully compatible with Recommendations ITU-R F.1092 and ITU-R F.1189.

## 4.2	RS with frequency diversity protection switching

The error performance monitoring functionalities of the protected section, i.e. the section outside the protection switching depends on the allocation of SDH RPS function blocks, as described in Recommendation ITU-R F.750.

The B1 byte is calculated/evaluated in both link end-points per each channel (i.e. working channels and protecting channel), while the B2, B3 and V5 bytes are passed through the link end-points without modification.

The error performance monitoring should be performed using the same methodologies described in § 3.2 based on not intrusive monitoring of B2 byte or by means of an RS tandem connection monitoring methodology.

Error performance events are defined in ITU-T Recommendation G.829. The compatibility with Recommendations ITU-R F.1092 and ITU-R F.1189 of the values of relative error performance parameters, obtained by means of one of the two previous methods according to Recommendation ITU-R F.750, need further investigations.

## 4.3	Multihop RS without frequency diversity protection

The B1 byte is calculated/evaluated in both link end-points per each channel (i.e. working channels and protecting channel), while the B2, B3 and V5 bytes are passed through the link end-points without modification.

Error performance events are defined in ITU-T Recommendation G.829. The quality of the whole link may be evaluated using the same methodologies described in § 3.2.

# 5	Link end-points are combinations from the previous ones

The evaluation of error performance events is possible only for the relevant section terminated in both link end-points.