Recommendation ITU-R F.1501 (05/2000)

Coordination distance for systems in the fixed service (FS) involving high-altitude platform stations (HAPs) sharing the frequency bands 47.2-47.5 GHz and 47.9-48.2 GHz with other systems in the fixed service

F Series Fixed service

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Annex 1 of Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from http://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Recommendations (Also available online at http://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radiowave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2010

 ITU 2010

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION ITU-R F.1501*

COORDINATION  DISTANCE  FOR  SYSTEMS  IN  THE  FIXED  SERVICE  (FS)
INVOLVING  HIGH-ALTITUDE  PLATFORM  STATIONS  (HAPSs)  SHARING
THE  FREQUENCY  BANDS  47.2-47.5  GHz  AND  47.9-48.2  GHz  WITH
OTHER  SYSTEMS  IN  THE  FIXED  SERVICE

(2000)

Rec. ITU-R F.1501

Scope

This Recommendation provides calculation methods to determine coordination distances between the fixed service using high altitude platform stations (HAPS) and other systems in the fixed service in the bands 47.2-47.5 GHz and 47.9-48.2 GHz.

The ITU Radiocommunication Assembly,

considering

a)	that WRC-97 designated the bands 47.2-47.5 GHz and 47.9-48.2 GHz for use by HAPSs in the FS;

b)	that these bands are also allocated for use by the fixed, fixed-satellite and mobile services;

c)	that frequency sharing for fixed-service systems using HAPSs and other systems in the FS is being considered in ITU-R;

d)	that there may be a requirement to establish criteria for determining coordination distances within which detailed sharing arrangements would need to be considered, or that there may be a requirement to specify predetermined coordination distances,

recommends

1	that Annex 1 should be used for the determination of coordination distance or as a basis for establishing a predetermined coordination distance between stations operating in a high-altitude platform network (HAPN) and other stations in the FS.

ANNEX  1

Coordination distance for systems involving the use of HAPSs

# 1	Geometrical coordination distance

For an initial assessment, it may be appropriate to use a coordination distance based on the above-the-horizon visibility (including atmospheric refraction) between a HAPS and other stations in the conventional FS or in HAPN.

The following predetermined coordination distance (measured from the sub-HAPS point) should be adopted between a HAPS and ground terminals of other systems in the conventional FS or in different HAPNs:

150 + (141.6 − 0.274 h) h km	(1)

where h is the HAPS altitude (km) above sea level in the 20-50 km range.

The following predetermined coordination distance (measured from the sub-HAPS point) should be adopted between HAPSs in different systems:

(141.6 − 0.274 h1) h1 + (141.6 − 0.274 h2) h2 km	(2)

where h1 and h2 are the HAPS altitude (km) above sea level of two systems, both in the 20-50 km range.

# 2	Coordination distance using typical system parameters and representative propagation conditions for the bands of 47.2-47.5 GHz and 47.9-48.2 GHz

## 2.1	Minimum propagation attenuation due to atmospheric gases

### 2.1.1	Attenuation between a HAPS and ground terminals

In the path between a HAPS and ground terminals of other systems in the conventional FS or in different HAPN, atmospheric gases including water vapour cause attenuation, which depends on the distribution along the path of meteorological parameters such as temperature, pressure and humidity, and thus varies with the geographic location of the site, the month of the year, the height of a ground terminal above sea level, the elevation angle of the slant path and the operating frequency.

According to Recommendation ITU-R SF.1395, the following numerical formulae can be used for estimating minimum slant path attenuation in the 47 GHz band where:

AL (h, ), AM (h, ) and AH (h, ): total atmospheric absorption loss (dB) for the low-latitude (within 22.5 of the Equator), mid-latitude (greater than 22.5, but less than 45 from the Equator) and high-latitude (45 or more from the Equator) areas, respectively;

h and :	ground terminal antenna altitude above sea level (km) and elevation angle (degrees), respectively.

The approximation was carried out for 0  h  3 km and 0    90. The actual elevation angle may be determined from the elevation angle developed under free space propagation conditions using the method in Recommendation ITU-R F.1333. For actual elevation angles below 0, the attenuation for 0 should be used.

Frequency band 47.2-47.5 GHz

In this frequency band, the attenuation is larger at higher frequencies and, therefore, the following formulae give the attenuation at 47.2 GHz.

AL (h, θ)52.43[1 + 0.7364 θ + 0.03601 θ2 – 0.001099 θ3 + 0.8024 × 10–5 θ4+ h (0.2624 + 0.2479 θ) + h2 (0.08130 + 0.02637 θ)]	(3a)

AM (h, θ)47.00[1 + 0.7004 θ + 0.03568 θ2 – 0.001081 θ3 + 0.7878 × 10–5 θ4+ h (0.2527 + 0.1970 θ) + h2 (0.05539 + 0.03239 θ)]	(3b)

AH (h, θ) = 46.70[1 + 0.6872 θ + 0.03637 θ2 – 0.001105 θ3 + 0.8087 × 10–5 θ4+ h (0.2472 + 0.1819 θ) + h2 (0.04858 + 0.03221 θ)]	(3c)

Frequency band 47.9-48.2 GHz

In this frequency band, the attenuation is larger at higher frequencies and, therefore, the following formulae give the attenuation at 47.9 GHz.

AL (h, θ) = 57.90[1 + 0.7262 θ + 0.03534 θ2 – 0.001074 θ3 + 0.7826 × 10–5 θ4+ h (0.2576 + 0.2382 θ) + h2 (0.07645 + 0.02443 θ)]	(4a)

AM (h, θ) = 53.06[1 + 0.6962 θ + 0.03555 θ2 – 0.001076 θ3 + 0.7840 × 10–5 θ4+ h (0.2495 + 0.1940 θ) + h2 (0.05420 + 0.03176 θ)]	(4b)

AH (h, θ) = 53.21[1 + 0.6864 θ + 0.03632 θ2 – 0.001103 θ3 + 0.8073 × 10–5 θ4+ h (0.2476 + 0.1812 θ) + h2 (0.04791 + 0.03191 θ)]	(4c)

### 2.1.2	Attenuation between two HAPSs

If two HAPSs use two frequency bands for opposite directions of transmission, there is a possibility of unacceptable interference between them. If the distance between sub-HAPS points is sufficiently large, the path may be subject to attenuation due to atmospheric gases mainly in the lower atmosphere. The method of estimating the minimum value of this attenuation is described below.

First, calculate the average altitude, h0 (km), of the altitudes h1 and h2 of two HAPS platforms as follows:

h0    (h1    h2)/2	(5)

It is assumed that h1 and h2 may be different, but the difference is not significant (see Note 2).

Then from Table 1 for 20  h0  30 km, calculate the minimum altitude of the path, h (km), between two HAPSs. For those values of h0 and the distance which are not given in Table 1, the minimum altitude of the path can be estimated by means of an appropriate interpolation.

Table 1 was produced for the atmospheric refractivity model of maximum refraction as defined in Recommendation ITU-R SF.765.

TABLE  1

Minimum altitude of the path between two HAPSs

| Sub-HAPS distance (km) | Minimum altitude of the path, h (km) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | For average altitude of two HAPSs, h0 |  |  |  |  |  |
|  | 20 km | 22 km | 24 km | 26 km | 28 km | 30 km |
| 350 400 450 500 550 600 650 700 750 800 850 900 950 1 000 1 050 1 100 1 150 1 200 1 250 1 300 1 350 1 400 1 450 | 17.63 16.91 16.10 15.20 14.22 13.16 12.03 10.84 9.61 8.36 7.11 5.89 4.73 3.64 2.66 1.78 1.00 0.32 – – – – – | – – – 17.16 16.16 15.08 13.92 12.69 11.41 10.09 8.75 7.42 6.13 4.91 3.77 2.75 1.84 1.04 0.35 – – – – | – – – – – 17.03 15.85 14.59 13.26 11.89 10.47 9.05 7.64 6.29 5.01 3.84 2.79 1.89 1.05 0.35 – – – | – – – – – – 17.79 16.51 15.16 13.74 12.27 10.77 9.26 7.79 6.37 5.05 3.85 2.78 1.84 1.02 0.32 – – | – – – – – – – – 17.08 15.63 14.12 12.56 10.97 9.39 7.85 6.38 5.03 3.80 2.72 1.77 0.96 0.26 – | – – – – – – – – – 17.55 16.01 14.40 12.75 11.08 9.43 7.83 6.33 4.95 3.71 2.62 1.68 0.87 0.18 |

When h  17 km, the attenuation is negligibly small. If 0  h  17 km, the minimum propagation attenuation can be estimated by the following numerical formulae, where:

AL (h), AM (h), AH (h):	total atmospheric absorption loss (dB) for the low-latitude, mid-latitude and high-latitude areas, respectively.

Frequency band 47.2-47.5 GHz

AL (h)104.36(1 + 0.25960 h + 0.092795 h2 – 0.0047598 h3+ 0.00018436 h4 + 0.000031666 h5)	(6a)

AM (h)93.94(1 + 0.28813 h + 0.010729 h2 – 0.018033 h3– 0.0024068 h4 + 0.00014071 h5)	(6b)

AH (h)93.39(1 + 0.27156 h + 0.023900 h2 + 0.0096081 h3– 0.0013613 h4 + 0.00012031 h5)	(6c)

Frequency band 47.9-48.2 GHz

AL (h) = 115.28(1 + 0.25520 h + 0.085840 h2 – 0.0041978 h3 + 0.00016894 h4 + 0.000030414 h5)	(7a)

AM (h)106.07(1 + 0.28529 h + 0.0097223 h2 + 0.017834 h3– 0.0023697 h4 + 0.00013852 h5)	(7b)

AH (h)106.44(1 + 0.27253 h + 0.023020 h2 + 0.0095858 h3– 0.0013468 h4 + 0.00011928 h5)	(7c)

NOTE 1 – When 0  h  3 km, the values of equations (6a)-(7c) are almost twice the values of equations (3a)-(4c) for the same altitude and   0.

NOTE 2 – For the same average altitude h0, when h1 and h2 are unequal, the minimum altitude of the path becomes slightly lower, resulting in slightly larger atmospheric absorption loss. Table 2 shows the minimum altitude (km) of the path for the case of h0  25 km. The effects of unequal HAPS altitudes are small.

TABLE  2

Minimum altitude of the path for the case of h0  25 km

| Sub-HAPS distance (km) | Minimum altitude of the path (km) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | For h1 and h2 |  |  |  |  |  |
|  | 25/25 km | 24/26 km | 23/27 km | 22/28 km | 21/29 km | 20/30 km |
| 700 900 1 100 1 300 | 15.55 9.90 4.43 0.68 | 15.52 9.88 4.42 0.67 | 15.45 9.84 4.40 0.66 | 15.32 9.77 4.36 0.64 | 15.14 9.66 4.31 0.62 | 14.90 9.53 4.24 0.58 |

## 2.2	Coordination distance using typical system parameters

This matter requires further study.

*	Study Group 5 made editorial amendments to this Recommendation (on 7-8 December 2009) in accordance with Resolution ITU-R 1.