RECOMMENDATION  ITU-R  F.747*

Radio-frequency channel arrangements for fixed wireless
systems operating in the 10 GHz band

(Question 136/9)

(1992)

The ITU Radiocommunication Assembly,

considering

a)	that the 10.5 to 10.68 GHz frequency band is allocated to the fixed and mobile services;

b)	that this band is advantageous for various digital fixed wireless system applications;

c)	that such systems are already in extensive use and are expected to be used more extensively in the future;

d)	that differing applications for various administrations may require different frequency plans;

e)	that several services with various transmission signal characteristics and capacities may be in simultaneous use in this frequency band;

f)	that the specific band allocated to each service or even to each administration may vary from one country to another;

g)	that the applications in this frequency band may require differing channel bandwidths;

h)	that a high degree of compatibility between radio-frequency channels of different arrangements can be achieved by selecting all channel centre frequencies from a homogeneous basic pattern;

j)	that the differing digital hierarchies used in various countries or regions may require the use of homogeneous basic patterns with differing intervals,

recommends

1	that frequency plans for the 10.5 to 10.68 GHz band should be based on homogeneous patterns;

2	that the homogeneous pattern with a preferred 3.5 MHz interval be defined by the relation:

fn    fr  –  1 200.5    3.5 n               MHz

where the reference frequency of the homogeneous pattern is:

fr    11 701 MHz

and the number of radio-frequency channels is:

1    n    50

(see Annex 1);

3	that the homogeneous pattern with a preferred 1.25 MHz interval be defined by the relation:

fn    fr  –  1 151    1.25 n               MHz

where the reference frequency of the homogeneous pattern is:

fr    11 701               MHz

and the number of radio-frequency channels is:

1    n    103

(see Annex 2);

4	that the channel spacings, the centre gap, and the distance to the lower and upper band limits, the reference frequency, as well as any further subdivision or combination of radio-frequency channels should be agreed by the administrations concerned, dependent on the application and channel capacity envisaged.

Annex 1

Description of a radio-frequency channel arrangement
using the homogeneous pattern described in recommends 2

The channel arrangement is as follows:

–	lower half of the band:	fn		fr  –  1 204    7 m               MHz

–	upper half of the band:	fn'		fr  –  1 113    7 m               MHz

The 12 values of m from 1 to 12 will provide the twelve channel frequencies of each half-band.

The reference frequency fr is:

fr    11 701               MHz

This arrangement is illustrated in Fig. 1.

Annex 2

Description of a radio-frequency channel arrangement
using the homogeneous pattern described in recommends 3

The channel arrangement assigns separate frequency band portions to point-to-point and to point-to-multipoint transmission, respectively, as shown in Fig. 2. Two different channel bandwidths are available for each of the two services: 1.25 and 2.5 MHz for the point-to-point service, 2.5 and 5.0 MHz for the digital electronic message service (DEMS). Each assigned radio-frequency channel may be subdivided as desired for greater usage versatility and higher spectral efficiency.

*	Radiocommunication Study Group 9 made editorial amendments to this Recommendation in 2002 in accordance with Resolution ITU-R 44.