Recommendation ITU-R F.748-4 (05/2001)

Radio-frequency arrangements for systems of the fixed service operating in the 25, 26 and 28 GHz bands

F Series Fixed service

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Annex 1 of Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from http://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Recommendations (Also available online at http://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radiowave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2009

 ITU 2009

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION  ITU-R  F.748-4*

Radio-frequency arrangements for systems of the fixed service
operating in the 25, 26 and 28 GHz bands

(1992-1994-1995-1999-2001)

Scope

This Recommendation provides specifications for radio-frequency channel arrangements for systems in the fixed service with channel separations ranging from 2.5 to 112 MHz in the bands 24.5-26.5 GHz, 27.5-29.5 GHz, 24.25-25.25 GHz and 25.27-26.98 GHz. One Annex (Annex 3) includes block-based arrangements with bandwidths of 40 MHz and 60 MHz in the frequency range 24.25 to 26.98 GHz.

The ITU Radiocommunication Assembly,

considering

a)	that the bands 24.25-25.25 GHz, 25.25-27.5 GHz and 27.5-29.5 GHz are allocated to fixed and other services;

b)	that some administrations use digital systems for point-to-point (P-P) and/or point-to-multipoint (P-MP) applications;

c)	that the bands are used for differing applications by various administrations and that these applications require different frequency plans;

d)	that the bands may also be used for fixed wireless access (FWA) systems;

e)	that several types of service with various capacities may be in simultaneous use in these frequency bands;

f)	that the band allocated to each service or even to each administration may vary from one country to another;

g)	that the applications in these frequency bands may require differing channel bandwidths;

h)	that a high degree of compatibility between different systems and between radio-frequency (RF) channels of different arrangements can be achieved by selecting all channel centre frequencies from a homogeneous basic pattern;

j)	that allocation of frequency blocks to FWA systems allows flexible deployment of various technologies including provision for inter-system/services operation and overall spectrum efficiency,

recommends

1	that the preferred RF channel arrangement for the 24.25-25.25 GHz, 25.25-27.5 GHz and 27.5-29.5 GHz bands should be based on homogeneous patterns;

2	that the homogeneous pattern with a preferred 3.5 MHz interval be defined by the relation:

fp  fr  3.5 p

where:

1		p		285	for the band 24.25-25.25 GHz

287		p		928	for the band 25.25-27.5 GHz

930		p		1 500	for the band 27.5-29.5 GHz

fr :	reference frequency of the homogeneous pattern;

3	that the homogeneous pattern with a preferred 2.5 MHz interval be defined by the relation:

fp  fr  2  2.5 p

where:

1		p		399	for the band 24.25-25.25 GHz

401		p		1 299	for the band 25.25-27.5 GHz

1 301		p		2 099	for the band 27.5-29.5 GHz

fr :	reference frequency of the homogeneous pattern;

4	that the reference frequency of the homogeneous pattern for international connections should be:

fr   248               MHz

5	that all go channels should be in one half of any bidirectional band, and all return channels in the other;

6	that the channel spacings, XS, centre gap, YS, and the lower and upper band limits, Z1S, Z2S, should be agreed by the administrations concerned, dependent on the application and the channel capacity envisaged (see Recommendation ITU-R F.746 for definitions of XS, YS and ZS);

7	that allocated blocks should result from aggregation of contiguous channels in accordance with the homogeneous pattern.

NOTE 1 – Due regard has to be taken that, in certain countries, a 3.5 MHz homogeneous pattern, interleaved by 1.75 MHz from that referred in recommends 2, is used in conjunction with the main pattern.

NOTE 2 – Examples of channel arrangements based on this Recommendation are described in Annexes 1 and 2.

NOTE 3 – Examples of FWA system block (sub-band) arrangements are described in Annex 3.

ANNEX  1

Radio-frequency channel arrangements for some CEPT administrations
in the band 24.5-26.5 GHz in accordance with recommends 2

An example of the RF channel arrangement based on this Recommendation for carrier spacings of 112 MHz, 56 MHz, 28 MHz, 14 MHz, 7 MHz and 3.5 MHz is derived as follows (see Fig. 1):

Let	f0	be the reference frequency of 25 501 MHz    fr    (358    3.5) MHz,

fn	be the centre frequency of a RF channel in the lower half of the band,

fn'	be the centre frequency of a RF channel in the upper half of the band,

then the centre frequencies of individual channels are expressed by the following relationships:

a)	for systems with a carrier spacing of 112 MHz:

lower half of band:	fn    f0  –  1 008    112 n	MHz

upper half of band:	fn'	  f0    112 n		MHz

where:

n  1, 2, 3, . . . 8

b)	for systems with a carrier spacing of 56 MHz:

lower half of band:	fn    f0  –  980    56 n	MHz

upper half of band:	fn'	  f0    28    56 n		MHz

where:

n  1, 2, 3, . . . 16

c)	for systems with a carrier spacing of 28 MHz:

lower half of band:	fn    f0  –  966    28 n	MHz

upper half of band:	fn'	  f0    42    28 n		MHz

where:

n  1, 2, 3, . . . 32

d)	for systems with a carrier spacing of 14 MHz:

lower half of band:	fn    f0  –  959    14 n	MHz

upper half of band:	fn'	  f0    49    14 n		MHz

where:

n  1, 2, 3, . . . 64

(See Note 1)

e)	for systems with a carrier spacing of 7 MHz:

lower half of band:	fn    f0  –  955.5    7 n	MHz

upper half of band:	fn'	  f0    52.5    7 n	MHz

where:

n  1, 2, 3, . . . 128

f)	for systems with a carrier spacing of 3.5 MHz:

lower half of band:	fn  f0  –  953.75    3.5 n	MHz

upper half of band:	fn'	f0  54.25    3.5 n	MHz

where:

n  1, 2, 3, . . . 256.

NOTE 1 – The RF channel arrangements of § a) to e) of this Annex use channel centre frequencies fn and fn' selected from the homogeneous pattern of recommends 2. The arrangement of § f) above uses frequencies spaced by 3.5 MHz but interleaved between the homogeneous pattern of recommends 2 with an offset of 1.75 MHz.

NOTE 2 – Figure 1 gives occupied spectrum in the 24.5-26.5 GHz band. The centre and edge guardbands may be reduced, by agreement between administrations, to allow the use of an increased number of lower capacity systems, by the addition of extra channels using frequencies derived from the homogeneous pattern of recommends 2.

NOTE 3 – The RF channel arrangements of § a) to f) of this Annex may be used in digital systems for P-P and/or P-MP applications.

ANNEX  2

RF channel arrangements for some CEPT administrations
in the band 27.5-29.5 GHz in accordance with recommends 2

An example of the RF channel arrangement based on this Recommendation for carrier spacings of 112 MHz, 56 MHz, 28 MHz, 14 MHz, 7 MHz and 3.5 MHz is derived as follows:

Let	f0	be the centre frequency of 28 500.5 MHz    fr    (1 215    3.5) MHz,

fn	be the centre frequency of a RF channel in the lower half of the band,

fn'	be the centre frequency of a RF channel in the upper half of the band,

then the centre frequencies of individual channels are expressed by the following relationships:

a)	for systems with a carrier spacing of 112 MHz:

lower half of band:	fn    f0  –  1 008    112 n	MHz

upper half of band:	fn'	  f0    112 n		MHz

where:

n  1, 2, 3, . . . 8

b)	for systems with a carrier spacing of 56 MHz:

lower half of band:	fn    f0  –  980    56 n	MHz

upper half of band:	fn'	  f0    28    56 n		MHz

where:

n  1, 2, 3, . . . 16

c)	for systems with a carrier spacing of 28 MHz:

lower half of band:	fn    f0  –  966    28 n	MHz

upper half of band:	fn'	  f0    42    28 n		MHz

where:

n  1, 2, 3, . . . 32

d)	for systems with a carrier spacing of 14 MHz:

lower half of band:	fn    f0  –  959    14 n	MHz

upper half of band:	fn'	  f0    49    14 n		MHz

where:

n  1, 2, 3, . . . 64

e)	for systems with a carrier spacing of 7 MHz:

lower half of band:	fn    f0  –  955.5    7 n	MHz

upper half of band:	fn'	  f0    52.5    7 n	MHz

where:

n  1, 2, 3, . . . 128

(See Note 1)

f)	for systems with a carrier spacing of 3.5 MHz:

lower half of band:	fn  f0  –  953.75    3.5 n	MHz

upper half of band:	fn'	f0    54.25    3.5 n	MHz

where:

n  1, 2, 3, . . . 256.

NOTE 1 – The RF channel arrangements of § a) to e) of this Annex use channel centre frequencies fn and fn' selected from the homogeneous pattern of recommends 2. The arrangement of § f) above uses frequencies spaced by 3.5 MHz but interleaved between the homogeneous pattern of recommends 2 with an offset of 1.75 MHz.

NOTE 2 – Figure 2 gives occupied spectrum in the 27.5-29.5 GHz band. The centre and edge guardbands may be reduced, by agreement between administrations, to allow the use of an increased number of lower capacity systems, by the addition of extra channels using frequencies derived from the homogeneous pattern of recommends 2.

NOTE 3 – The RF channel arrangements of § a) to f) of this Annex may be used in digital systems for P-P and/or P-MP applications.

ANNEX  3

RF block arrangements in the bands 24.25-25.25 GHz
and 25.27-26.98 GHz in accordance with recommends 7

# 1	Arrangement based on a 40 MHz frequency block

The following arrangements can be referred to for use by administrations wishing to implement arrangements based on frequency blocks.

## 1.1	RF block arrangement description

The band 24.25-25.25 GHz is divided into five paired frequency blocks (40 MHz  40 MHz) as follows:

| 1.2 Usage Paired block | Lower frequency block (MHz) | Upper frequency block (MHz) |
| --- | --- | --- |
| A/A' | 24 250-24 290 | 25 050-25 090 |
| B/B' | 24 290-24 330 | 25 090-25 130 |
| C/C' | 24 330-24 370 | 25 130-25 170 |
| D/D' | 24 370-24 410 | 25 170-25 210 |
| E/E' | 24 410-24 450 | 25 210-25 250 |

–	The block arrangement is for both P-P and P-MP systems.

–	Frequency blocks are paired to facilitate frequency division duplex (FDD) systems. Preference is given to the lower frequency blocks for downlink operation and to the upper frequency blocks for uplink operation. Time division duplex (TDD) systems may operate in either the lower or upper frequency blocks.

–	Operators may subdivide the 40 MHz blocks according to their needs.

–	Larger frequency blocks can be made available through the aggregation of 40 MHz paired blocks.

# 2	Arrangement based on a 60 MHz frequency block

## 2.1	RF block arrangement description

The band 25.27-26.98 GHz is divided into 13 paired frequency blocks (60 MHz  60 MHz) as follows.

| 2.2 Usage Paired block | Lower frequency block (MHz) | Upper frequency block (MHz) |
| --- | --- | --- |
| D1/D'1 | 25 270-25 330 | 26 125-26 185 |
| D2/D'2 | 25 330-25 390 | 26 185-26 245 |
| D3/D'3 | 25 390-25 450 | 26 245-26 305 |
| B1/B'1 | 25 450-25 510 | 26 305-26 365 |
| B2/B'2 | 25 510-25 570 | 26 365-26 425 |
| B3/B'3 | 25 570-25 630 | 26 425-26 485 |
| B4/B'4 | 25 630-25 690 | 26 485-26 545 |
| B5/B'5 | 25 690-25 750 | 26 545-26 605 |
| B6/B'6 | 25 750-25 810 | 26 605-26 665 |
| B7/B'7 | 25 810-25 870 | 26 665-26 725 |
| D4/D'4 | 25 945-26 005 | 26 800-26 860 |
| D5/D'5 | 26 005-26 065 | 26 860-26 920 |
| D6/D'6 | 26 065-26 125 | 26 920-26 980 |

–	The block arrangement is for both P-P and P-MP systems.

–	Frequency blocks are paired to facilitate FDD systems. Preference is given to the lower frequency blocks for uplink operation and to the upper frequency blocks for downlink operation. TDD systems may operate in either the lower or upper frequency blocks.

–	Operators may subdivide the 60 MHz blocks according to their needs.

–	Larger frequency blocks can be made available through the aggregation of 60 MHz paired blocks.

*	Radiocommunication Study Group 5 made editorial amendments to this Recommendation in 2009 in accordance with Resolution ITU-R 1.