9E5:	HF data transmission

RECOMMENDATION  ITU-R  F.763-1

DATA  TRANSMISSION  OVER  HF  CIRCUITS  USING  PHASE-SHIFT  KEYING

(Question ITU-R 145/9)

(1992-1994)

Rec. ITU-R F.763-1

The ITU Radiocommunication Assembly,

considering

a)	that there is an increasing demand for high-rate data transmission;

b)	that to compensate for the unfavourable nature of the transmission medium, the following techniques are available:

–	various forms of dual diversity operation including separate SSB emissions or a single ISB emission;

–	several levels of in-band frequency diversity;

–	error detection and error correction coding combined with time interleaving;

–	variable data rate to adapt the system to the channel capacity;

–	introduction of guard times between frames to combat multipath propagation and group-delay distortion,

recommends

1.	that for data transmission at binary data rates up to 2 400 bit/s using frequency-division multiplex phase-shift systems, the system described in Annex 1 is preferred;

2.	that reference should be made to Annex 2 for additional information concerning generalized phase-shift keying.

ANNEX  1

Data transmission at 2 400/1 200/600/300/150/75 bit/s over HF circuits
using multi-channel voice-frequency telegraphy
and phase-shift keying

# 1.	System description

1.1	A receiving/transmitting terminal of the system consists of:

–	a sender and receiver of digital information (e.g. computer);

–	a modem, the primary function of which is the conversion of information from digital to analogue form compatible with the input to a radio transmitter and conversion of the analogue information at a radio receiver output into digital data compatible with the digital receiver input.

This modem also performs various coding functions and effects diversity combination;

–	RF receiving and transmitting equipment connected to antennas.

1.2	At the transmit side, the 2 400 bit/s incoming data stream is fed to a serial-to-parallel converter. At 32-bit intervals (i.e. 13.33 ms intervals) the content of this converter is transferred in parallel to a 32-bit memory device, the output of which is connected to a QPSK modulator.

The modem generates in transmission a composite audio signal consisting of a set of 18 tones in the band 300 to 3 000 Hz.

Of these tones, 16 have a spacing of 110 Hz (935 to 2 585 Hz) and are modulated in DE-QPSK mode (differentially encoded quaternary phase shift keying), each at 75 Bd, thus permitting a data rate of 16 ´ 75 ´ 2 = 2 400 bit/s.

The tone at 605 Hz is used for the correction of end-to-end frequency errors, including any Doppler effect. The tone at 2 915 Hz (or 825 Hz) is used for system synchronization.

The dual diversity combiner can accept inputs either from two receivers operating in space, frequency or polarization diversity mode or from one receiver operating in ISB mode.

When the data rate is a sub-multiple of the transmission speed, various in-band diversity arrangements can be implemented. As an example, a data rate of 1 200 bit/s provides a dual diversity (1 200 ´ 2), a data rate of 600 bit/s, a quadruple diversity (600 ´ 4) and so forth, all with a transmission speed of 2 400 bit/s. Utilization of the maximum possible diversity, both in-band and between independent channels, can thus be made according to the data rate selected. Provision is made for 75/150/300/600/1 200 bit/s.

In addition to a choice of coded/uncoded operation, with selectable data rate and diversity mode, this modem also allows setting of the interleaving interval thus providing a flexible communication system as summarized in Table 1.

TABLE  1

Data rates/modes (independently selectable for transmission and reception)

|  | Uncoded modes |  |  | Coded modes |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Data rate (bit/s) | Diversity modes |  |  | Time interleaving Available time spread | Additional diversity modes |  |  |
|  | In-band | Channel | Total | (transmitter and receiver) (s) | In-band | Channel | Total |
| 2 400 | – | X2 | X2 |  |  |  |  |
| 1 200 | X2 | X2 | X4 | 0-12.8 | – | X2 | X2 |
| 2 600 | X4 | X2 | X8 | 0-25.6 | X2 | X2 | X4 |
| 2 300 | X8 | X2 | X16 | 0-51.2 | X4 | X2 | X8 |
| 2 150 | X16 | X2 | X32 | 0-102.5 | X8 | X2 | X16 |
| 2 175 |  |  |  | 0-205 | X16 | X2 | X32 |

The transmission signal consists of frames whose duration is 13.33 ms. This includes a time guard (4.2 ms) which is introduced to offset the effects of multipath propagation.

The modem uses two techniques to reduce signal impairments, particularly those caused by impulsive noise and flat fading:

–	error correction code;

–	time interleaving.

A form of BCH cyclic block code (16, 8) is used. The BCH codewords are stored in a memory to be extracted during the interleaving process. Interleaving is obtained by considering:

–	the first bit of the last stored word,

–	the second bit of the “(m) word stored before”,

–	the third bit of the “(2 m) word stored before” ...,

–	the 16th bit of the “(15 m) word stored before”.

The interleaving level (m codewords) can be chosen according to the propagation conditions of the radio path from 0 (no interleaving), 1, 2, 4, 8, 16, 32, or 64, corresponding to a data reception delay ranging from a few milliseconds to tens of seconds. As the wrong bits do not belong to the same coded word, a better protection against burst errors is achieved.

In Fig. 1, the performance of the modem with Gaussian distributed noise is given in terms of bit error probability (Pe) as a function of S/N, for both with coding and without coding modes, in a 250-3 000 Hz bandwidth.

The effects of coding become prominent at the higher values of signal-to-noise ratio.

The curves were obtained with an experimental test set-up in which the modem was fed with a test pattern to produce the audio frequency tones. The output of the modem was summed with Gaussian noise, filtered and applied to the receiving input of another modem from which the test pattern was retrieved at the output. The test pattern was then fed to a data error analyzer to enable the bit-error ratio to be determined.

Figure 2 indicates the results of a computer simulation of the modem performance in a fading channel.

A fading channel was simulated in which two equi-amplitude paths carry signals separated by a multipath delay of 1 ms and differing in frequency by 1 Hz, in order to obtain fades which ran through the passband rather than remaining at certain fixed frequencies.

From Fig. 2, it can be seen that the performance is improved by using a combination of the various types of diversity (in-band and out-of-band), error correcting codes and interleaving techniques for 600, 1 200 and 2 400 bit/s rates.

The modem is currently in experimental use as part of an HF link between two radio stations located in central and southern Italy, and separated by approximately 800 km (500 miles).

1.3	The RF equipment performs, in transmission, operations relative to channel modulation and produces an emission having suitable radio frequency and power characteristics. Reverse operations, relative to frequency conversion, are carried out in reception so as to obtain the composite audio signal to be conveyed to the modem.

The RF equipment has the following specific characteristics:

–	phase jitter: less than 5° for 10 ms time interval (100 samples);

–	group delay distortion: 500 ms in transmission, 500 ms in reception;

–	intermodulation: 36 dB below peak envelope power.

FIGURE  1/F.763...[D01] = 3 CM  PLEINE  PAGE

FIGURE  2/F.763...[D02] = 3 CM  PLEINE  PAGE

ANNEX  2

Transmission systems using phase-shift keying

# 1.	Introduction

In HF channels information at bit rates of over 200 bit/s is normally transmitted using multistate methods and complex signals. This generally involves a combination of frequency-shifted orthogonal subcarriers with binary PSK. With the latter technique a bit rate twice that obtainable with FSK can be achieved in the same frequency band and the redundancy can be used to increase noise immunity. Apart from multi-frequency PSK, practical interest attaches to a more general type of modulation – generalized PSK, in which the information to be transmitted is contained not in the differences between the instantaneous phases of the sine-wave signals but in the difference between the phase spectra of complex orthogonal signals. The amplitude spectra of such signals coincide and may be matched with the channel frequency characteristic (or the interference spectrum) without violating the conditions of mutual orthogonality. On this basis it is possible to consider the construction of adaptive modems with a higher noise immunity or traffic capacity.

The practical application of generalized PSK has been held up in the past by the well-known difficulties involved in the synthesis and processing of complex signals. The basic problems have now been solved, thanks to the theory of synthesis which has been developed, and the availability of microelectronic modules with a high degree of integration, which has removed the obstacle of technical circuit complexity. This Annex sets forth the main principles governing the design of modems with generalized PSK, describes a variant which has been developed and gives a number of test results.

# 2.	Theoretical questions

## 2.1	Selection of signals

As was pointed out by Shannon, in order to achieve a transmission rate equal to the communication capacity in channels with a frequency characteristic of Y(w) and a Gaussian noise of N(w), use has to be made of signals characterized by a steady state Gaussian process with a power of P and a power spectrum of:

µ §	(1)

where the integration range W is determined from the condition F(w) ³ 0 and the constant B depends on the power of the signals. Since in practice there are always standards governing the permissible limits for the delay in information being transmitted, the maximum signal duration and the number of signals have to be limited. Under these conditions finite-dimensional combinations of determined orthogonal signals, the squares of whose spectral density moduli coincide with F(w), may be considered as being close to optimum. However from (1) it follows that F(w) = 0 at all frequencies where B < Y(w)/N(w), i.e. mutual orthogonality has to be preserved when individual portions of the spectrum are rejected. The multi-frequency signals used in existing modems do not possess this property. Moreover, their orthogonal spectrum shape is optimum only for channels with a flat frequency characteristic and white noise type interference. Calculations show that failure to meet these conditions may result in information transmission rate losses of up to 40% of the channel’s communication capacity.

Another criterion for assessing the optimum nature of orthogonal signal combinations is the requirement as regards the shape of their autocorrelation function. For example, to ensure stability in the operation of a synchronization system, the main lobe of this function has to be narrow enough and the side lobes must not exceed a given level. In this case mutual orthogonality must be assured at a given signal amplitude spectrum which does not necessarily satisfy condition (1).

In view of the above, in order to achieve generalized PSK, a special class of signals based on the use of complex systems of functions with double orthogonality was developed. Their spectral densities may be represented as follows:

Sk (w)  =  ïS(w)ïe j [K y(w) + a(w)]	(2)

where:

ïS(w)ï2  =  A

A :	constant factor

a(w) :	arbitrary bounded function.

For a given amplitude spectrum, it is therefore possible to define the phase spectrum of the signals and therefore their spectral density. Further synthesis involves finding samples of spectral signal densities with different serial numbers and transforming them using an FFT into time samples. Synthesis of the signals may be combined with coding of the signals in the time domain using the Reed-Solomon code; for this purpose a number of zero samples has to be added beforehand to the spectral density samples and only then can the FFT be performed. It should be noted that this type of mixed coding (orthogonal in the frequency domain and Reed-Solomon code in the time domain) is most effective for HF channels.

## 2.2	Selection of a processing algorithm

In the case of multi-state methods for the transmission of information, it is best to process the signals to be received using the optimum algorithm for reception “as a whole”. The simplest way of implementing such an algorithm is to use component demodulators; for this purpose the following conditions have to be met:

–	the multi-state signals have to be component-type, i.e. they have to be made up of the sum of the elementary signals;

–	each elementary signal must contain information about the corresponding element of a code word bi, k;

–	the interference affecting the elementary signals must be mutually independent.

In this case the decision rule is as follows:

µ §	(3)

where:

ei, k :	sign coefficient which takes the value	+1 when bi, k = 1 and
	–1 when bi, k = 0

yk  =  ln

where:

Zk :	complex input signal (see Fig. 1)

W (Zk /1) :	probability of Zk being 1

W (Zk /0) :	probability of Zk being 0.

Optimality in this case is determined by the extent to which the signals used meet the conditions listed above. The first two involve the possibility of using a component demodulator. For these conditions to be met, it is sufficient for each spectral density sample (or its components) to contain information about the sign of the corresponding binary symbol. The condition stipulating the mutual independence of interference can be reduced to a condition stipulating the independence of the projections of the received signal vector onto the system on the basis of Fourier transform functions. This condition is fulfilled in the case of independent fades in individual frequency bands, invariance of the basic functions to time shifts and interference with a flat power spectrum. In practice it is impossible for the various requirements listed above to be met fully, so that the noise immunity of the component demodulator will be lower than the potential noise immunity although much higher than in the case of separate signal element reception.

The block diagram of the receiving section of the modem which implements decision rule (3) consists of the following units (see Fig. 3): a unit for calculating the logarithm of the likelihood ratio yk; a unit for calculating the linear forms Li; a decision element determining the number of the linear form with the maximum value; a digital converter which compares with each number its own combination of binary symbols; and this provides an assessment of the information sequence transmitted.

FIGURE  3/F.763...[D03] = 5 CM

# 3.	Description of the system

A block diagram of the system is shown in Fig. 4. It consists of the following elements: the user terminals; the signal conversion unit (modem) located either in the immediate vicinity of the terminals or in a separate communication control unit; the receiving and transmitting SSB equipment and the corresponding antennas. When the modem is installed in the control unit, communication with the terminal is established via tone frequency channels.

When it is set up in the immediate vicinity of the terminal, it can be connected up by d.c. circuits.

FIGURE  4/F.763...[D04] = 10 CM

A block diagram of the modem embodying the principles considered above is shown in Fig. 5. The modem is designed to transmit digital information at 600-1 200 bit/s. For lower bit rates, an additional codec has to be used. A bit rate of 2 400 bit/s is obtained by increasing the number of signals used and switching over to separate signal element reception. The modem transmitter consists of a coupling device (CD) and an orthogonal signal synthesizer (OSS).

FIGURE  5/F.763...[D05] = 16 CM

The CD is designed to match the modem with the user’s terminal via the tone frequency channels or the d.c. circuits and to control the synthesizer. It includes a tone frequency amplifier-rectifier, a regenerator, and a logic control circuit.

The OSS shapes the analogue signals and amplifies them to the requisite level. It consists of a coding unit, a ROM, a digital to analogue converter (DAC), an LF filter and a power amplifier. A specific feature of the operation of the OSS is that time samples of all the signals to be used for the transmission of information are already entered in its ROM. These samples were calculated beforehand on a computer in accordance with the rules laid down in the previous section.

In an initial stage, in order to verify the basic principles used, a set of 16 bi-orthogonal signals was synthesized; these had a flat amplitude spectrum in the range 1.1-2.42 kHz and an effective band of 0.66-2.86 kHz. Their spectral densities were represented by means of four complex samples, each of which could provide information on the signs of two binary symbols. To transpose the spectrum to these samples, two zero samples were added and after a Fourier transform an additional multiplication by a complex component was carried out.

The time samples of the signals calculated in this way were entered in an 8 bit grid in the ROM and, after they had been read at a timing frequency of 8.8 kHz, it was possible to obtain at the output of the DAC unit analogue signals with a duration of 3.33 ms and an orthogonality interval of 2.27 ms.

The sequence of operations in the modem transmitter is as follows: the binary information signals from the terminal are regenerated, combined to form 4-bit code words and are then fed to the input of the relative coding circuit which controls the selection from the ROM of one of the 16 forms of the signal. From the output of the ROM the samples are converted by the DAC unit into an analogue signal which after being amplified is fed along the tone frequency channel to the input of the SSB transmitter.

As shown in Fig. 3 the receiving section of the modem consists of the following elements: a unit which calculates the logarithms of the likelihood ratio (correlation detectors (CDT); a phase difference calculator (PDC)); a matrix matched filter (MF) which calculates all the linear forms Li; a decision element (DE) which determines the number of the maximum form; and a digital conversion unit (DCU). It also contains a synchronization device (SD) and a control unit (CU). Provision is made for a single or a dual operating mode with space or polarization diversity.

The transformation of the analogue signals into spectral density samples is carried out by the correlation detectors which calculate the in-phase and the quadrature component of each sample. The initial uncertainty relating to the phase of the channel is then eliminated using the PDC and the phase spectrum of the signal received is calculated. The matched filter is a matrix adder and each of its columns is adjusted for an appropriate sample selection using inverting amplifiers. The DE looks for the column with the maximum output voltage and, using the DCU, transmits the corresponding 4 element sequence of binary symbols which is fed to the input terminal either directly or via the tone-frequency keying unit (TKU).

The control unit (CU) works on the principle that the voltages at the output buses of the matched filter coincide precisely to within a constant factor with the distribution of a priori probabilities. It is clear that the performance of the channel will be better the “steeper” this distribution is since in the ideal case the voltage must appear only on one of the output buses of the matched filter. The difference between the maximum voltage and the voltage whose level is closest to it at the other bus may be used to evaluate the quality of the channel in the information transmission process.

# 4.	Experimental investigations

The laboratory tests on the modem were carried out using a modelling test bench which included the following elements: the SSB receiver; a two-ray channel simulator; a noise generator; and a frequency meter to calculate the number of errors. A pseudo-random sequence (PRS) from a generator built into the modem was used as a test combination. Three modes of operation were analysed: a channel with constant parameters and white noise; a single-ray channel with Rayleigh fading; and a two-ray channel with a difference in the ray propagation time of 1 ms, identical ray amplitudes and Rayleigh fading. The results of the tests are shown in Figs. 6 and 7. By way of comparison, Fig. 6 shows curves for the noise immunity of a multi-frequency modem as described in Annex 1 to this Recommendation for the same transmission speed. As the curves show, the modem studied has a better noise immunity. Comparison of curves A and B in Fig. 7a) show that the modem has a higher noise immunity in the two-ray channel than in the one-ray channel. The reasons for this is that in the case of flat fades, decision rule (3) is no longer optimum. In a two-ray channel a predominant role is played by frequency selective fades which the modem can combat more effectively. The dotted line in the figure shows the theoretical curve for the noise immunity of an optimum non-coherent separate signal element reception using binary PSK in the the case of Rayleigh fading.

FIGURE  6/F.763...[D06] = 12.5 CM

FIGURE  7/F.763...[D07] = 3 CM  PLEINE  PAGE

Link tests on the modem were carried out on 3 600 km and 4 300 km latitudinal paths. Use was made of a 15 kW SSB transmitter, rhombic transmitting antennas and fishbone receiving antennas (dual reception). Tests on the first path were carried out during the day and during the night on one frequency. On the second path two frequencies were used. The information bit rate was 1 200 bit/s. On the basis of five minute measurements, integral curves were plotted showing the error rate distribution; these are shown in Fig. 7b).

# 5.	Conclusions

The use of generalized PSK in combination with reception “as a whole” opens up additional possibilities for increasing noise immunity in the transmission of digital information. A modem developed to serve as a practical example of how the generalized PSK method may be implemented uses signals with a flat spectrum and from this point of view is similar to the modems described in Annex 1 to this Recommendation. Tests have shown that for links over 3 000-4 000 km it guarantees a bit rate of 1 200 bit/s with an error rate not exceeding 10–4 to 10–3 for 95 to 98% of the time.

_________________