RECOMMENDATION  ITU-R M.493-6*

digital  selective-calling  system  for  use  in  the  maritime  mobile service

(Question ITU-R 9/8)

(1974-1978-1982-1986-1990-1992-1994)

Rec. ITU-R M.493-6

The ITU Radiocommunication Assembly,

considering

a)	that selective calling in the shore-to-ship, ship-to-ship and ship-to-shore directions would expedite the handling of traffic in the maritime mobile service;

b)	that the International Maritime Organization (IMO) has listed a number of operational requirements that should be taken into account when designing a general purpose selective-calling system;

c)	that Chapter IV of the International Convention for the Safety of Life at Sea, 1974, as amended in 1988, requires the use of digital selective-calling for distress alerting and safety calling in the Global Maritime Distress and Safety System;

d)	that neither the selective-calling system described in Recommendation ITU-R M.257, nor that forming part of the systems described in Recommendations ITU-R M.476 and ITU-R M.625, can fully meet the IMO recommended performance standards;

e)	that several administrations have indicated an urgent need for a general purpose selective-calling system;

f)	that several administrations have been developing different systems;

g)	that the system should be applicable to the maritime mobile service, both for international and national needs;

h)	that it is desirable that the selective-calling system fulfil the requirements of all types of vessels desiring to use it;

j)	that Radio Regulations Appendix 43 adopted by the World Administrative Radio Conference for the Mobile Services (Geneva, 1983) (WARC MOB-83) has provided for the use of maritime mobile service identities by all administrations,

recommends

1.	that where there is need for a general purpose digital selective-calling system, the system should be designed in accordance with the characteristics given in Annex 1;

2.	that where there is need for simplified versions of DSC equipment, they should be designed in accordance with Annex 2.

ANNEX  1

General purpose equipment characteristics

# 1.	General

1.1	The system is a synchronous system using characters composed from a ten-bit error-detecting code as listed in Table 1.

1.1.1	The first seven bits of the ten-bit code of Table 1 are information bits. Bits 8, 9 and 10 indicate, in the form of a binary number, the number of B elements that occur in the seven information bits, a Y element being a binary number 1 and a B element a binary number zero. For example, a BYY sequence for bits 8, 9 and 10 indicates 3 (0 ´ 4 + 1 ´ 2 + 1 ´ 1) B elements in the associated seven information bit sequence; and a YYB sequence indicates 6 (1 ´ 4 + 1 ´ 2 + 0 ´ 1) B elements in the associated seven information bit sequence. The order of transmission for the information bits is least significant bit first but for the check bits it is most significant bit first.

TABLE  1

Ten-bit error-detecting code

| Symbol No. | Emitted signal and bit position 1 2 3 4 5 6 7 8 9 10 |  | Symbol No. | Emitted signal and bit position 1 2 3 4 5 6 7 8 9 10 |  | Symbol No. | Emitted signal and bit position 1 2 3 4 5 6 7 8 9 10 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 00 01 02 03 04 05 06 07 08 09 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 38 39 40 41 42 | BBBBBBBYYY YBBBBBBYYB BYBBBBBYYB YYBBBBBYBY BBYBBBBYYB YBYBBBBYBY BYYBBBBYBY YYYBBBBYBB BBBYBBBYYB YBBYBBBYBY BYBYBBBYBY YYBYBBBYBB BBYYBBBYBY YBYYBBBYBB BYYYBBBYBB YYYYBBBBYY BBBBYBBYYB YBBBYBBYBY BYBBYBBYBY YYBBYBBYBB BBYBYBBYBY YBYBYBBYBB BYYBYBBYBB YYYBYBBBYY BBBYYBBYBY YBBYYBBYBB BYBYYBBYBB YYBYYBBBYY BBYYYBBYBB YBYYYBBBYY BYYYYBBBYY YYYYYBBBYB BBBBBYBYYB YBBBBYBYBY BYBBBYBYBY YYBBBYBYBB BBYBBYBYBY YBYBBYBYBB BYYBBYBYBB YYYBBYBBYY BBBYBYBYBY YBBYBYBYBB BYBYBYBYBB |  | 43 44 45 46 47 48 49 50 51 52 53 54 55 56 57 58 59 60 61 62 63 64 65 66 67 68 69 70 71 72 73 74 75 76 77 78 79 80 81 82 83 84 85 | YYBYBYBBYY BBYYBYBYBB YBYYBYBBYY BYYYBYBBYY YYYYBYBBYB BBBBYYBYBY YBBBYYBYBB BYBBYYBYBB YYBBYYBBYY BBYBYYBYBB YBYBYYBBYY BYYBYYBBYY YYYBYYBBYB BBBYYYBYBB YBBYYYBBYY BYBYYYBBYY YYBYYYBBYB BBYYYYBBYY YBYYYYBBYB BYYYYYBBYB YYYYYYBBBY BBBBBBYYYB YBBBBBYYBY BYBBBBYYBY YYBBBBYYBB BBYBBBYYBY YBYBBBYYBB BYYBBBYYBB YYYBBBYBYY BBBYBBYYBY YBBYBBYYBB BYBYBBYYBB YYBYBBYBYY BBYYBBYYBB YBYYBBYBYY BYYYBBYBYY YYYYBBYBYB BBBBYBYYBY YBBBYBYYBB BYBBYBYYBB YYBBYBYBYY BBYBYBYYBB YBYBYBYBYY |  | 186 187 188 189 190 191 192 193 194 195 196 197 198 199 100 101 102 103 104 105 106 107 108 109 110 111 112 113 114 115 116 117 118 119 120 121 122 123 124 125 126 127 | BYYBYBYBYY YYYBYBYBYB BBBYYBYYBB YBBYYBYBYY BYBYYBYBYY YYBYYBYBYB BBYYYBYBYY YBYYYBYBYB BYYYYBYBYB YYYYYBYBBY BBBBBYYYBY YBBBBYYYBB BYBBBYYYBB YYBBBYYBYY BBYBBYYYBB YBYBBYYBYY BYYBBYYBYY YYYBBYYBYB BBBYBYYYBB YBBYBYYBYY BYBYBYYBYY YYBYBYYBYB BBYYBYYBYY YBYYBYYBYB BYYYBYYBYB YYYYBYYBBY BBBBYYYYBB YBBBYYYBYY BYBBYYYBYY YYBBYYYBYB BBYBYYYBYY YBYBYYYBYB BYYBYYYBYB YYYBYYYBBY BBBYYYYBYY YBBYYYYBYB BYBYYYYBYB YYBYYYYBBY BBYYYYYBYB YBYYYYYBBY BYYYYYYBBY YYYYYYYBBB |
| B = 0 Y = 1 | Order of bit transmission: bit 1 first. |  |  |  |  |  |  |

1.2	Time diversity is provided in the call sequence as follows:

1.2.1	Apart from the phasing characters, each character is transmitted twice in a time-spread mode; the first transmission (DX) of a specific character is followed by the transmission of four other characters before the re-transmission (RX) of that specific character takes place, allowing for a time-diversity reception interval of:

1.2.1.1	400 ms for HF and MF channels, and

1.2.1.2	331/3 ms for VHF radio-telephone channels.

1.2.2	For a call which includes repetition(s) of the call sequence (see § 11.1), Fig. 1c) shows the transition between the end of one call sequence and the start of the following call sequence.

1.3	The classes of emission, frequency shifts and modulation rates are as follows:

1.3.1	F1B or J2B 170 Hz and 100 Bd for use on HF and MF channels. When frequency-shift keying is effected by applying audio signals to the input of single-sideband transmitters (J2B), the centre of the audio-frequency spectrum offered to the transmitter is 1 700 Hz.

1.3.2	Frequency modulation with a pre-emphasis of 6 dB/octave (phase modulation) with frequency-shift of the modulating sub-carrier for use on VHF channels:

–	frequency-shift between 1 300 Hz and 2 100 Hz; the sub-carrier being at 1 700 Hz;

–	the frequency tolerance of the 1 300 Hz and 2 100 Hz tones is ± 10 Hz;

–	the modulation rate is 1 200 Bd;

–	the index of modulation is 2.0 ± 10%.

1.3.3	The radio-frequency tolerances of new designs of both transmitters and receivers in the MF and HF bands should be:

–	Coast station ± 10 Hz

–	Ship station ± 10 Hz

–	Receiver bandwidth should not exceed 300 Hz.

1.4	The higher frequency corresponds to the B-state and the lower frequency corresponds to the Y-state of the signal elements.

1.5	The information in the call is presented as a sequence of seven-bit combinations constituting a primary code.

1.5.1	The seven information bits of the primary code express a symbol number from 00 to 127, as shown in Table 1, and where:

1.5.1.1	the symbols from 00 to 99 are used to code two decimal figures according to Table 2;

1.5.1.2	the symbols from 100 to 127 are used to code service commands (see Table 3).

1.6	The receiver decoder should provide maximum utilization of the received signal, including use of the error-check character.

1.7	Where the call repetitions described in § 11.1 apply, the following conditions are considered necessary:

1.7.1	the transmitter encoder must provide repetitive transmission of the call sequence in accordance with § 11.1; and

1.7.2	the receiver decoder should provide maximum utilization of the received signal, including use of the error-check character and by using an iterative decoding process with adequate memory provision.

# 2.	Technical format of a call sequence

2.1	The technical format of the call sequence is:

DIAGRAMME...[D01] = 4 CM

2.2	Examples of typical call sequences and the construction of the transmission format are given in Tables 4-7, and in Figs. 1-4.

2.3	The flow charts illustrating the operation of the digital selective-calling system are shown in Figs. 5a and 5b.

# 3.	Dot pattern and phasing

3.1	The phasing sequence provides information to the receiver to permit correct bit phasing and unambiguous determination of the positions of the characters within a call sequence.

Note 1 – Acquisition of character synchronization should be achieved by means of character recognition rather than, for example, by recognizing a change in the dot pattern, in order to reduce false synchronization caused by a bit error in the dot pattern.

3.2	The phasing sequence consists of specific characters in the DX and RX positions transmitted alternatively. Six DX characters are transmitted.

3.2.1	The phasing character in the DX position is symbol No. 125 of Table 1.

3.2.2	The phasing characters in the RX position specify the start of the information sequence (i.e. the format specifier) and consist of the symbol Nos. 111, 110, 109, 108, 107, 106, 105 and 104 of Table 1, consecutively.

3.3	Phasing is considered to be achieved when two DXs and one RX, or two RXs and one DX, or, if practical three RXs in the appropriate DX or RX positions, respectively, are successfully received.

3.4	To provide appropriate conditions for earlier bit synchronization and to allow for scanning methods to monitor several HF and MF frequencies by ship stations, the phasing sequence should be preceded by a dot pattern (i.e. alternating B-Y sequence bit synchronization signals) with duration of:

### 3.4.1	200 bits

At HF and MF, for “distress”, “distress acknowledgement”, “distress relay” and “distress relay acknowledgement” calls and for all calling sequences to ship stations.

### 3.4.2	20 bits

At HF and MF, for all acknowledgement sequences (except distress acknowledgements and distress relay acknowledgements – see § 3.4.1) and for all calling sequences to coast stations (except distress relay calls – see § 3.4.1). At VHF for all calls.

Note 1 – In exceptional circumstances and only on national working frequencies the 200 bit dot pattern could also be included in acknowledgement sequences to ship stations.

# 4.	Format specifier

4.1	The format specifier characters which are transmitted twice in both the DX and RX positions (see Fig. 1) are:

4.1.1	symbol No. 112 for a “distress” call RR No. 3086); or

4.1.2	symbol No. 116 for an “all ships” call; or

4.1.3	symbol No. 114 for a selective call to a group of ships having a common interest (e.g. belonging to one particular country, or to a single shipowner, etc.); or

4.1.4	symbol No. 120 for a selective call to a particular individual station; or

4.1.5	symbol No. 102 for a selective call to a group of ships in a particular geographic area; or

4.1.6	symbol No. 123 for a selective call to a particular individual station using the semi-automatic/automatic service.

4.2	It is considered that receiver decoders must detect the format specifier character twice for “distress” calls and “all ships” calls to effectively eliminate false alerting. For other calls, the address characters provide additional protection against false alerting and, therefore, single detection of the format specifier character is considered satisfactory (see Table 8).

# 5.	Address

5.1	“Distress” calls and “all ships” calls do not have addresses since these calls are implicitly addressed to all stations (ship stations and coast stations).

5.2	For a selective call directed to an individual ship, to a coast station or to a group of stations having a common interest, the address consists of the characters corresponding to the station’s maritime mobile service identity, the sequence consisting of characters coded in accordance with Table 2.

Note 1 – According to RR Appendix 43, maritime mobile service identities are formed of a series of nine digits, consisting of three digits of the Maritime Identification Digits (MID) and six more digits.

These identifications are included in the address and self-identification parts of the call sequence and are transmitted as five characters C5C4C3C2C1, comprising the ten digits of:

(X1, X2) (X3, X4) (X5, X6) (X7, X8) and (X9, X10)

respectively, whereas digit X10 is always the figure 0 unless the equipment is also designed in accordance with Recommendation ITU-R M.1080.

Example:

MID X4 X5 X6 X7 X8 X9 being the ship station identity is transmitted by the digital selective-calling equipment as:

(M, I) (D, X4) (X5, X6) (X7, X8) (X9, 0)

5.3	For a selective call directed to a group of ships in a particular geographic area a numerical geographic coordinates address consisting of ten digits (i.e. 5 characters), is constructed as follows (see Fig. 6):

Note 1 – In order to comply with commonly accepted practice, the order of entry and read-out should be: first latitude and then longitude.

5.3.1	the designated geographic area will be a rectangle in Mercator projection;

5.3.2	the upper left-hand (i.e. North-West) corner of the rectangle is the reference point for the area;

5.3.3	the first digit indicates the azimuth sector in which the reference point is located, as follows:

5.3.3.1	quadrant NE is indicated by the digit “0”,

5.3.3.2	quadrant NW is indicated by the digit “1”,

5.3.3.3	quadrant SE is indicated by the digit “2”,

5.3.3.4	quadrant SW is indicated by the digit “3”;

5.3.4	the second and third digits indicate the latitude of the reference point in tens and units of degrees;

5.3.5	the fourth, fifth and sixth digits indicate the longitude of the reference points in hundreds, tens and units of degrees;

5.3.6	the seventh and eighth digits indicate the vertical (i.e. North-to-South) side of the rectangle, Dj, in tens and units of degrees;

5.3.7	the ninth and tenth digits indicate the horizontal (i.e. West-to-East) side of the rectangle, Dl, in tens and units of degrees.

# 6.	Category

6.1	The “category” information is coded as shown in Table 9 and defines the degree of priority of the call sequence.

6.2	For a “distress” call the priority is defined by the format specifier and no category information is included in the call sequence.

6.3	For safety related calls, the “category” information specifies:

6.3.1	distress (RR No. 3134); or

6.3.2	urgency; or

6.3.3	safety.

6.4	For other calls, the “category” information specifies:

6.4.1	ship’s business; to cater for shore-to-ship communications having priority category 6 as defined in RR No. 4441. Some coast stations do not use the ship’s business priority category;

6.4.2	routine.

# 7.	Self-identification

7.1	The maritime mobile service identity (MMSI) assigned to the calling station, coded as indicated in § 5.2, is used for self-identification. The MMSI should be stored in the DSC unit and it should not be possible for the user to easily change it.

# 8.	Messages

The messages that are included in a call sequence contain the following message elements, which are listed in the order in which they would appear in each message:

8.1	For a “distress” call (see Table 4 and Fig. 4a)) the distress information is contained in four messages in the following order:

8.1.1	Message 1 is the “nature of distress” message, coded as shown in Table 10, i.e.:

8.1.1.1	fire, explosion;

8.1.1.2	flooding;

8.1.1.3	collision;

8.1.1.4	grounding;

8.1.1.5	listing, in danger of capsizing;

8.1.1.6	sinking;

8.1.1.7	disabled and adrift;

8.1.1.8	undesignated distress;

8.1.1.9	abandoning ship;

8.1.1.10	EPIRB emission.

8.1.2	Message 2 is the “distress coordinates” message, consisting of ten digits indicating the location of the vessel in distress, coded on the principles described in Table 2, in pairs starting from the first and second digits:

Note 1 – In order to comply with commonly accepted practice, the order of entry and read-out should be: first latitude and then longitude.

8.1.2.1	The first digit indicates the quadrant in which the incident has occurred, as follows:

8.1.2.1.1	quadrant NE is indicated by the digit “0”,

8.1.2.1.2	quadrant NW is indicated by the digit “1”,

8.1.2.1.3	quadrant SE is indicated by the digit “2”,

8.1.2.1.4	quadrant SW is indicated by the digit “3”.

8.1.2.2	The next four figures indicate the latitude in degrees and minutes.

8.1.2.3	The next five figures indicate the longitude in degrees and minutes.

8.1.2.4	If “distress coordinates” cannot be included, the 10 digits following the “nature of distress” should be automatically transmitted as the digit  9 repeated 10 times.

8.1.2.5	DSC equipment with facilities for automatic position updating should be provided with an NMEA 0183 (or IEC-1162) interface for input of data from a navigation equipment.

8.1.3	Message 3 is the time indication (UTC) when the coordinates were valid consisting of four digits coded on the principles described in Table 2, in pairs starting from the first and second digits.

8.1.3.1	The first two digits indicate the time in hours.

8.1.3.2	The third and fourth digits indicate the part of the hours in minutes.

8.1.3.3	If the time cannot be included the four time indicating digits should be transmitted automatically as “8 8 8 8”.

8.1.4	Message 4 is a single character to indicate the type of communication (telephone or teleprinter) which is preferred by the station in distress for subsequent exchange of distress traffic (RR No. 3134). This character is coded as shown in Table 11.

8.2	For other types of calls (see Table 5 and Figs. 2 and 3) except “distress relay”, “distress relay acknowledgement” and “distress acknowledgement” calls (see § 8.3), two messages are included in the following order:

8.2.1	Message 1 is the “telecommand” information and consists of 2 characters (first and second telecommand) coded as shown in Tables 11 and 12;

8.2.1.1	if no information additional to that conveyed by the first telecommand character is required, then the second telecommand signal should be symbol No. 126 (no information) – see Table 12;

8.2.1.2	if no telecommand information is used, symbol No. 126 is transmitted twice.

8.2.2	Message 2 may contain two “channel or frequency message” elements, each of which always consists of three characters, “character 1”, “character 2” and “character 3”, indicating the proposed working frequency (in the F1B/J2B mode the assigned frequency should be used) in multiples of 100 Hz or the channel number (coded in accordance with Table 13) or the ship’s position. The first frequency element (the RX field) in the call indicates the called station receive frequency and the second frequency element (the TX field) indicates the called station transmit frequency. In acknowledgements the RX and TX fields indicate the receive and transmit frequency of the acknowledging station respectively (see also Fig. 2).

Note 1 – If only one channel or frequency message element is used, this indicates the called station receive channel or frequency or a two-frequency (paired) channel. A second channel or frequency message element may be used to designate the called station transmit channel or frequency. If the calling station indicates only the called station receive frequency (for broadcast mode transmissions) then the symbol No. 126 repeated three times should be transmitted instead of the called station transmit channel or frequency message element. If no “channel or frequency message” elements are used, the symbol No. 126 is transmitted six times. For calls using the semi-automatic/automatic VHF service (see Table 7) then only one “channel or frequency message” element is transmitted which indicates the paired channel number. In the absence of this element the symbol No. 126 should be transmitted three times.

#### 8.2.2.1	Frequency information

The frequency (in the F1B/J2B mode the assigned frequency should be used) in multiples of 100 Hz may only be indicated as such when the frequency is below 30 MHz. The three characters provide for the required six decimal digits. Character 1 represents the units (U) and tens (T) of 100 Hz, character 2 the hundreds (H) and thousands (M) and character 3 the tens of thousands (TM) and hundreds of thousands (HM) of 100 Hz.

#### 8.2.2.2	Channel information

##### 8.2.2.2.1	HF and MF channels

If the HM digit is 3, this indicates that the number represented by the digits TM, M, H, T and U is the HF/MF working channel number (either single frequency or two frequency channels).

##### 8.2.2.2.2	VHF channels

If the HM digit is 9, this indicates that the number represented by the values of the digits M, H, T and U is the VHF working channel number.

#### 8.2.2.3	Ship’s position information

8.2.2.3.1	Message 2 may contain the ship’s position, consisting of the digit 5 repeated two times and ten digits (five characters) indicating this position, coded in accordance with § 8.1.2 to § 8.1.2.3 (see Table 14).

8.2.2.3.2	If a reply to a calling sequence requesting ship’s position is required (see Fig. 3d) then message 2 consists of twelve digits (six symbols), the first of which should be coded in accordance with § 8.1.2 to § 8.1.2.3 followed by one symbol No. 126.

8.2.2.3.3	Message 3 follows message 2 in this case and contains the time (UTC) when the coordinates were valid, coded as indicated in § 8.1.3 to § 8.1.3.3.

8.2.3	Message 3 follows message 2 when using the DSC system for calls initiated by ship stations requiring a semi-automatic or automatic connection (see Table 7) and contains the public switched network number (e.g. telephone number). In this case the format specifier used is symbol No. 123.

8.2.3.1	This number is coded by up to nine symbols in a manner similar to that shown in Table 2, except that the first character transmitted should be either symbol No. 105 or No. 106 to indicate whether the network number contains an odd or even number of significant digits. As an example, the number 0012345 would be coded as symbol numbers 105 00 01 23 45 whereas the number 00123456 should be coded as symbol numbers 106 00 12 34 56.

8.3	For “distress relay” including shore-to-ship alerts, “distress relay acknowledgement” and “distress acknowledgement” calls, the message formats are indicated in Figs. 4b) and 4c) respectively.

8.3.1	For a distress relay where the identity of the station in distress is unknown, the “identification of the station in distress” should be automatically transmitted as the symbol No. 126 five times.

8.4	For test calls on the exclusive distress and safety calling frequencies on MF and HF, the call sequence is given in Table 6 (see also Recommendation ITU-R M.541, Annex 1). Technical means should be included to prevent the transmission of this sequence on VHF. Furthermore, the first telecommand symbol No. 118 (see Table 3) should only be capable of being inserted into the sequence given in Table 6.

# 9.	End of sequence

The “end of sequence” character is transmitted three times in the DX position and once in the RX position (see Fig. 1b)). It is one of the three unique characters corresponding to symbol Nos. 117, 122 and 127 as follows:

9.1	symbol No. 117 if the call requires acknowledgement (Acknowledge RQ);

9.2	symbol No. 122 if the sequence is an answer to a call that requires acknowledgement (Acknowledge BQ);

9.3	symbol No. 127 for all other calls.

# 10.	Error-check character

10.1	The error-check character is the final character transmitted and it serves to check the entire sequence for the presence of errors which are undetected by the ten-unit error-detecting code and the time diversity employed.

10.2	The seven information bits of the error-check character shall be equal to the least significant bit of the modulo-2 sums of the corresponding bits of all information characters (i.e. even vertical parity). The format specifier and the end of sequence characters are considered to be information characters. The phasing characters shall not be considered to be information characters. Only one format specifier character and one end of sequence character should be used in constructing the error-check character. The error-check character shall also be sent in the DX and RX positions. Automatic acknowledgement transmissions should not start unless the error-check character is received and decoded correctly.

# 11.	Call repetitions

11.1	Normally a call attempt should consist of a single call sequence transmission (see Fig. 1b)). However, in exceptional circumstances and only using national DSC frequencies (see Recommendation ITU-R M.541), up to five transmissions of the same call sequence may be transmitted in accordance with Fig. 1c).

11.2	Distress calls must be transmitted as single calls preceded by a dot pattern. However, where a distress call attempt contains more than one consecutive distress call on the same frequency (see Recommendation ITU-R M.541, Annex 1, § 3.1.3), these consecutive calls may be transmitted with no gap between the end of one call and the start of the dot pattern of the following call to enable bit synchronization to be maintained.

# 12.	Audible alarm

An audible alarm and visual indicator should be provided upon reception of a distress call or a call with category distress (see Recommendation ITU-R M.541, § 3).

FIGURE 1/M.493-6...[D02] = PLEINE PAGE

FIGURE 2/M.493-6...[D03] = PLEINE PAGE

FIGURE 3/M.493-6...[D04] = PLEINE PAGE

FIGURE 4/M.493-6...[D05] = PLEINE PAGE

FIGURE 5a/M.493-6...[D06] = PLEINE PAGE

FIGURE 5b/M.493-6...[D07] = PLEINE PAGE

FIGURE 6/M.493-6...[D08] = PLEINE PAGE

TABLE  2

Packing table for decimal numbers into ten-bit characters

The digits for the

| Thousands of millions D2 | Hundreds of millions D1 | Tens of millions D2 | Millions D1 | Hundreds of thousands D2 | Tens of thousands D1 | Thousands D2 | Hundreds D1 | Tens D2 | Units D1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Character 5 | Character 4 | Character 3 | Character 2 | Character 1 |
| --- | --- | --- | --- | --- |

Note 1 – Character 1 is the last character transmitted. The digit sequence D2-D1 varies from 00 to 99 inclusive in each character (character 1 to 5 inclusive). The character that represents a particular two-decimal figure is transmitted as the symbol number (see Table 1) that is identical to that particular two-decimal figure. When the number consists of an odd number of decimal digits, a zero shall be added in front of the most significant position to provide an integral number of ten-bit characters.

TABLE  3

Use of symbol Nos. 100 to 127

| Symbol No. | Phasing and unique functions | Format specifier | Category | Nature of distress | First telecommand | Second telecommand |
| --- | --- | --- | --- | --- | --- | --- |

| 100 |  |  | Routine | Fire, explosion | F3E/G3E simplex TP | No reason given(1) |
| --- | --- | --- | --- | --- | --- | --- |

| 101 |  |  |  | Flooding | F3E/G3E duplex TP | Congestion at maritime switching centre |
| --- | --- | --- | --- | --- | --- | --- |

| 102 |  | Geographical area |  | Collision | (2) | Busy(1) |
| --- | --- | --- | --- | --- | --- | --- |

| 103 |  | (3) | (3) | Grounding | Polling | Queue indication(1) |
| --- | --- | --- | --- | --- | --- | --- |

| 104 | Phasing RX-0 position |  |  | Listing, in danger of capsizing | Unable to comply | Station barred(1) |
| --- | --- | --- | --- | --- | --- | --- |

| 105 | Phasing RX-1 position |  |  | Sinking | End of call(4) | No operator available(1) |
| --- | --- | --- | --- | --- | --- | --- |

| 106 | Phasing RX-2 position |  | Ship’s business | Disabled and adrift | Data | Operator temporarily unavailable(1) |
| --- | --- | --- | --- | --- | --- | --- |

| 107 | Phasing RX-3 position |  |  | Undesignated distress | (2) | Equipment disabled(1) |
| --- | --- | --- | --- | --- | --- | --- |

| 108 | Phasing RX-4 position |  | Safety | Abandoning ship | (2) | Unable to use proposed channel(1) |
| --- | --- | --- | --- | --- | --- | --- |

| 109 | Phasing RX-5 position |  |  |  | J3E TP | Unable to use proposed mode(1) |
| --- | --- | --- | --- | --- | --- | --- |

| 110 | Phasing RX-6 position | (5) | Urgency |  | Distress acknowledgement | Ships and aircraft according to Resolution No. 18 (Mob-83) |
| --- | --- | --- | --- | --- | --- | --- |

| 111 | Phasing RX-7 position |  |  |  | H3E TP | Medical transport (as defined in 1949 Geneva Convention and additional Protocols) |
| --- | --- | --- | --- | --- | --- | --- |

TABLE  3 (continued)

| Symbol No. | Phasing and unique functions | Format specifier | Category | Nature of distress | First telecommand | Second telecommand |
| --- | --- | --- | --- | --- | --- | --- |

| 112 |  | Distress | Distress | EPIRB emission | Distress relay | Pay-phone/public call office |
| --- | --- | --- | --- | --- | --- | --- |

| 113 |  |  |  |  | F1B/J2B TTY-FEC | Facsimile/Data according to Recommendation ITU-R M.1081 |
| --- | --- | --- | --- | --- | --- | --- |

| 114 |  | Ships having common interest |  |  | (2) | (2) |
| --- | --- | --- | --- | --- | --- | --- |

| 115 |  |  |  |  | F1B/J2B TTY-ARQ | Data V.21 |
| --- | --- | --- | --- | --- | --- | --- |

| 116 |  | All ships |  |  | F1B/J2B TTY | Data V.22 |
| --- | --- | --- | --- | --- | --- | --- |

| 117 | Ack. RQ (EOS) |  |  |  | (6) | (6) |
| --- | --- | --- | --- | --- | --- | --- |

| 118 |  |  |  |  | Test(7) | Data V.22 bis |
| --- | --- | --- | --- | --- | --- | --- |

| 119 |  |  |  |  | F1B/J2B TTY | Data V.23 |
| --- | --- | --- | --- | --- | --- | --- |

| 120 |  | Individual stations |  |  | A1A Morse TR | Data V.26 bis |
| --- | --- | --- | --- | --- | --- | --- |

| 121 |  | Reserved for national non-calling purposes e.g. Report ITU-R M.1159 |  |  | Ship position or location registration updating | Data V.26 ter |
| --- | --- | --- | --- | --- | --- | --- |

| 122 | Ack. BQ (EOS) |  |  |  | (6) | (6) |
| --- | --- | --- | --- | --- | --- | --- |

| 123 |  | Individual station semi-automatic/ automatic service |  |  | A1A Morse | Data V.27 ter |
| --- | --- | --- | --- | --- | --- | --- |

| 124 |  | (5) |  |  | F1C/F2C/F3C FAX | Data V.32 |
| --- | --- | --- | --- | --- | --- | --- |

| 125 | Phasing DX position |  |  |  | (6) | (6) |
| --- | --- | --- | --- | --- | --- | --- |

| 126 | * |  |  |  | No information(8) | No information(8) |
| --- | --- | --- | --- | --- | --- | --- |

| 127 | EOS |  |  |  | (6) | (6) |
| --- | --- | --- | --- | --- | --- | --- |

| - TR : tape recorder |  |
| --- | --- |

| - TP : telephony |  |
| --- | --- |

| - TTY : direct printing |  |
| --- | --- |

| - ARQ : Rec. ITU-R M.476 or Rec. ITU-R M.625 equipment |  |
| --- | --- |

| - FAX : facsimile |  |
| --- | --- |

| - RLS : emergency position-indicating radio beacon |  |
| --- | --- |

| - EOS : end of sequence |  |
| --- | --- |

| - * Symbol transmitted in place of unused message information. |  |
| --- | --- |

|  | (1) Currently unassigned when used with first telecommands other than symbol No. 104 – for future use. (2) Currently unassigned – for future use. (3) Used for selective call to a group of ships in a specified VTS area (Rec. ITU-R M.825). Should not be used in any future expansion. (4) Only used for semi-automatic/automatic service. (5) Used in the automatic VHF/UHF service (Rec. ITU-R M.586). Should not be used in any future expansion. (6) Should not be used in any future expansion. (7) See § 8.4. (8) See § 8.2.1.1 and 8.2.1.2. |
| --- | --- |

TABLE  4

Call sequences of “distress call” and “all ships call”

| (2) Format | (5) Address | (1) Category | (5) Self- | Message | (1)* EOS | (1) ECC |
| --- | --- | --- | --- | --- | --- | --- |

| specifier |  |  | identification | 1 | 2 | 3 | 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Distress call 112 | ______ | ________ | 00-------99 | (1) Nature of distress 100---------124 | (5) Distress coordinates 00-----------99 | (2) Time | (1) ** | 127 | ECC* |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| All ships call 116 | ______ | Distress 112 Urgency 110 Safety 108 | 00-------99 | (2) Telecommand 100---------126 except 117, 122 and 125 | (6) Frequency or channel 00-----------99 | not used | not used | EOS 127 | ECC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- ( ) : number of characters

- ECC : error-check character

- EOS : end of sequence

- * See § 9. ** Type of subsequent communication, see Table 11 and § 8.1.4.

TABLE  5

Call sequences of selective calls

| (2) Format | (5) Address | (1) Category | (5) Self- | Message | (1)* EOS | (1) ECC |
| --- | --- | --- | --- | --- | --- | --- |

| specifier |  |  | identification | 1 | 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Geographical area call 102 | 00------99 | Distress 112 Urgency 110 Safety 108 |  | (2) | (6) | EOS 127 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Ships having common interest call 114 | See § 5 of Annex 1 | Ship’s business 106 | 00-------99 | Telecommand 100---------126 except 117, 122 and 125 | Frequency, channel or ship’s position 00-----------99 | Ack. RQ 117 or Ack. BQ 122 | ECC |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Individual call 120 |  | Routine 100 |  |  |  | or EOS 127 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

| - ( ) : number of characters |  |
| --- | --- |

| - ECC : error-check character |  |
| --- | --- |

| - EOS : end of sequence |  |
| --- | --- |

| * See § 9. |  |
| --- | --- |

TABLE  6

Call sequence of selective calls for testing the equipment used
for distress and safety calls

| (2) Format | (5) Address | (1) Category | (5) Self- | Message |  | (1) * EOS | (1) ECC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| specifier |  |  | identification | 1 | 2 |  |  |
| Individual call 120 | 00------99 | Safety 108 | 00----------99 | (2) First telecommand 118; second telecommand 126 | (6) Frequency or channel 126 transmitted six times | Ack. RQ 117 or Ack. BQ 122 | ECC |

- ( ) : number of characters

- ECC : error-check character

- EOS : end of sequence

* See § 9.

TABLE  7

Call sequence of semi-automatic/automatic ship-to-shore call

| (2) Format | (5) Address | (1) Category | (5) Self- | Message | (1) * EOS | (1) ECC |
| --- | --- | --- | --- | --- | --- | --- |

| specifier |  |  | identification | 1 | 2 | 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 123 | 00-----99 | Routine 100 | 00---------99 | (2) VHF calls – first telecommand 100, 101, 104, 105, 106, 121 or 124; MF/HF calls – first telecommand 102 ... 124, except 110, 112, 117 or 122; second telecommand in accordance with Table 12 | (6) ** Frequency, channel or ship’s position 00---------99 | (2-9) Selection information 105 or 106, followed by 00----------99 see § 8.2.3.1 | Ack. RQ 117 or Ack. BQ 122 | ECC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

( ) : number of characters

ECC : error-check character

EOS : end of sequence

* See § 9. ** Only 3 for VHF calls.

TABLE  8

Format specifier

| Symbol No. | Format specifier |
| --- | --- |

| 112 | Distress call |
| --- | --- |

| 116 | All ships call |
| --- | --- |

|  | Selective call to: |
| --- | --- |

| 120 | – Individual stations |
| --- | --- |

| 102 | – Ships in a particular geographic area |
| --- | --- |

| 114 | – Ships having a common interest |
| --- | --- |

| 123 | Semi-automatic/automatic service |
| --- | --- |

TABLE  9

Category

| Symbol No. | Category |
| --- | --- |

|  | Safety related: |
| --- | --- |
| 112 | Distress |
| 110 | Urgency |
| 108 | Safety |
|  | Others: |
| 106 | Ship’s business |
| 100 | Routine |

TABLE  10

Nature of distress

| Symbol No. | Nature of distress |
| --- | --- |

| 100 | Fire, explosion |
| --- | --- |

| 101 | Flooding |
| --- | --- |

| 102 | Collision |
| --- | --- |

| 103 | Grounding |
| --- | --- |

| 104 | Listing, in danger of capsizing |
| --- | --- |

| 105 | Sinking |
| --- | --- |

| 106 | Disabled and adrift |
| --- | --- |

| 107 | Undesignated distress |
| --- | --- |

| 108 | Abandoning ship |
| --- | --- |

| 112 | EPIRB emission |
| --- | --- |

TABLE  11

First telecommand character

| Symbol No. (1) | Use and/or mode | Terminal equipment |
| --- | --- | --- |

| 100 101 102 103 104 105 106 107 108 109 110 111 112 113 114 115 116 118 119 120 121 123 124 126 | F3E/G3E simplex F3E/G3E duplex (2) Polling Unable to comply (3) End of call (4) Data (5) (2) (2) J3E Distress acknowledgement H3E Distress relay F1B/J2B CED (2) F1B/J2B ARQ F1B/J2B receive Test (7) F1B/J2B A1A Morse Ship position or location registration updating A1A Morse F1C/F2C/F3C No information (8) | - Telephone Telephone – – – – Modem – – Telephone – Telephone – Teleprinter (6) – Telex/teleprinter (6) Teleprinter – Teleprinter Tape recorder – Morse key/head-set Facsimile machine – |
| --- | --- | --- |

- (1) Symbols 117, 122, 125 and 127 should not be used.

- (2) Currently unassigned – for future use.

- (3) One of second telecommand symbols 100-109 must follow (see Table 12).

- (4) Only used for semi-automatic/automatic service.

- (5) One of second telecommand symbols 115-124 should follow (see Table 12).

- (6) Equipment according to Recommendation ITU-R M.476 or Recommendation ITU-R M.625.

- (7) See § 8.4.

- (8) See § 8.2.1.2.

TABLE  12

Second telecommand character

| Symbol No. (1) | Meaning | For use with the following first telecommand signals |
| --- | --- | --- |

| 100 101 102 103 104 105 106 107 108 109 | No reason given (2) Congestion at maritime switching centre (2) Busy (2) Queue indication (2) Station barred (2) No operator available (2) Operator temporarily unavailable (2) Equipment disabled (2) Unable to use proposed channel (2) Unable to use proposed mode (2) | 104 (Unable to comply) (3) |
| --- | --- | --- |

| 110 111 | Ships and aircraft according to Resolution No. 18 (Mob-83) Medical transport (as defined in 1949 Geneva Convention and additional Protocols) | Any except 104, 105, 106, 110, 112 or 118 |
| --- | --- | --- |

| 112 113 114 | Pay-phone public call office Facsimile/data according to Recommendation ITU-R M.1081 (4) | 100, 101, 109, 115 or 124 106 – |
| --- | --- | --- |

| 115 116 118 119 120 121 123 124 | Data V.21 (5) Data V.22 (5) Data V.22 bis (5) Data V.23 (5) Data V.26 bis (5) Data V.26 ter (5) Data V.27 ter (5) Data V.32 (5) | 106 (Data) |
| --- | --- | --- |

| 126 | No information (6) | Any except 104, 110 or 112 |
| --- | --- | --- |

- (1) Symbols 117, 122, 125 and 127 should not be used.

- (2) Currently unassigned when used with first telecommands other than symbol No. 104 – for future use.

- (3) When second telecommands 100-109 are given alternative assignments (see ((2)), they may be used with first telecommands other than symbol No. 104.

- (4) Currently unassigned – for future use.

- (5) Data communication in accordance with these ITU-T Recommendations may require special provision at coast stations and may not be practicable in all frequency bands.

- (6) See § 8.2.1.1.

TABLE  13

Frequency or channel information

| Frequency | 0 1 2 | X X X | X X X | X X X | X X X | X X X | The frequency in multiples of 100 Hz as indicated by the figures for the digits HM, TM, M, H, T, U. |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | 3 | X | X | X | X | X | The HF/MF working channel number indicated by the values of the digits TM, M, H, T and U. |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Channels | 8 | X | X | X | X | X | Only used for Recommendation ITU-R M.586 equipment. |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | 9 | O | X | X | X | X | The VHF working channel number indicated by the values of the digits M, H, T and U. |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | HM | TM | M | H | T | U |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | Character 3 | Character 2 | Character 1 |  |
| --- | --- | --- | --- | --- |

Note 1 – Character 1 is the last character transmitted.

TABLE  14

Position information (Annex 1, § 8.2.2.3)

|  | Quadrant digit NE = 0 | Latitude | Longitude |
| --- | --- | --- | --- |

|  | NW = 1 SE = 2 SW = 3 | Tens of degrees | Units of degrees | Tens of minutes | Units of minutes | Hundreds of degrees | Tens of degrees | Units of degrees | Tens of minutes | Units of minutes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 55 | X | X | X | X | X | X | X | X | X | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Character 6 | Character 5 | Character 4 | Character 3 | Character 2 | Character 1 |
| --- | --- | --- | --- | --- | --- |
| Note 1 – Character 1 is the last character transmitted. |  |  |  |  |  |

ANNEX  2

Equipment classes

1.	Class A equipment, which includes all the facilities defined in Annex 1, will comply with the IMO GMDSS carriage requirements for MF/HF installations.

Class B equipment providing minimum facilities for equipment on ships not required to use Class A equipment and complying with the minimum IMO GMDSS carriage requirements for MF and/or VHF installations.

Class D equipment is intended to provide minimum facilities for VHF DSC distress, urgency and safety as well as routing calling and reception, not necessarily in full accordance with IMO GMDSS carriage requirements for VHF installations.

Class E equipment is intended to provide minimum facilities for MF and/or HF DSC distress, urgency and safety as well as routine calling and reception, not necessarily in full accordance with IMO GMDSS carriage requirements for MF/HF installations.

Class F equipment is intended to provide for VHF DSC distress, urgency and safety calling and also for reception of acknowledgements to its own distress calls (in order to terminate the transmission).

Note 1 – Class C equipment as defined in Recommendation ITU-R M.493-5 has been replaced by Class F equipment.

2.	The technical requirements for Class B, D, E and F are given in § 3, 4, 5 and 6 below.

# 3.	Class B (MF and/or VHF only)

## 3.1	Transmit capabilities

3.1.1	Format specifier:	Distress call
All ships call
Individual station call
Semi-automatic/automatic service call.

3.1.2	The numerical identification of the called station (address).

3.1.3	Category:	Distress
Urgency
Safety
Routine.

3.1.4	Self-identification (automatically inserted).

### 3.1.5	Messages

3.1.5.1	For distress calls:

Message 1:	Nature of distress, defaulting to undesignated distress

Message 2:	Distress coordinates

Message 3:	Time for last position update

Message 4:	Type of subsequent communication:

MF: H3E or J3E

VHF: F3E/G3E simplex

3.1.5.2	For distress relay calls:

First telecommand:	Distress relay

Identification of the ship:	As defined in Annex 1

Messages 1 to 4:	As § 3.1.5.1 above

3.1.5.3	For distress acknowledgement calls:

First telecommand:	Distress acknowledgement

Identification of the ship:	As defined in Annex 1

Messages 1 to 4:	As § 3.1.5.1 above

3.1.5.4	For all other calls:

First telecommand:

MF:	for individual station calls H3E, J3E or “test” (see Annex 1 § 8.4);

for calls using the semi-automatic/automatic MF-services H3E, J3E or “end of call”.

VHF:	for individual station calls F3E/G3E simplex or duplex;

for calls using the semi-automatic/automatic VHF-services F3E/G3E simplex or duplex or “end of call”.

Second telecommand:		No information

Frequency/channel or ship's position:	As defined in Annex 1

Selection information (semi-automatic/automatic service):	Telephone number of public telephone subscriber

3.1.6	End of sequence character:

As defined in Annex 1.

## 3.2	Receive capabilities

3.2.1	Receive and be capable of displaying all the information in calls listed in § 3.1 plus all distress relay calls having the format specifier “geographical area calls”, all distress acknowledgement calls and all “unable to comply“ calls.

3.2.2	Audible alarm upon reception of any DSC call.

# 4.	Class D (VHF only)

## 4.1	Transmit capabilities

4.1.1	Format specifier:	Distress call
All ships call
Individual station call.

4.1.2	The numerical identification of the called station (address).

4.1.3	Category:	Distress
Urgency
Safety
Routine.

4.1.4	Self-identification (automatically inserted).

### 4.1.5	Messages

4.1.5.1	For distress calls:

Message 1:	Nature of distress, defaulting to undesignated distress

Message 2:	Distress coordinates

Message 3:	Time for last position update

Message 4:	Type of subsequent communication: F3E/G3E simplex.

4.1.5.2	For all other calls:

First telecommand:				F3E/G3E simplex

Unable to comply

Second telecommand:			No information

Frequency/channel information:	VHF working channel, defaulting to channel 16 for urgency and safety calls.

4.1.6	End of sequence character: as defined in Annex 1.

## 4.2	Receive capabilities

Receive and be capable of displaying all the information in calls listed in § 4.1 plus all distress relay calls except those having the format specifier “geographical area calls”, all distress acknowledgement calls and all “unable to comply” calls.

# 5.	Class E (MF and/or HF only)

## 5.1	Transmit capabilities

5.1.1	Format specifier:	Distress call
		All ships call
		Individual station call.

5.1.2	The numerical identification of the called station (address).

5.1.3	Category:	Distress
	Urgency
	Safety
	Routine.

5.1.4	Self-identification (automatically inserted).

### 5.1.5	Messages

5.1.5.1	For distress calls:

Message 1:	Nature of distress, defaulting to undesignated distress

Message 2:	Distress coordinates

Message 3:	Time for last position update

Message 4:	Type of subsequent communication: H3E or J3E

5.1.5.2	For all other calls:

First telecommand:	J3E telephony
	Unable to comply
	No information

Second telecommand:		No information

Frequency/channel information:	MF/HF working channel, on MF defaulting to 2 182 kHz for urgency and safety calls.

5.1.6	End of sequence character: as defined in Annex 1.

## 5.2	Receive capabilities

Receive and be capable of displaying all the information in calls listed in § 5.1 plus all distress relay calls having the format specifier “geographical area calls”, all distress acknowledgement calls and all “unable to comply” calls.

# 6.	Class F (VHF only)

## 6.1	Transmit capabilities

6.1.1	Format specifier:	Distress call
All ships call.

6.1.2	Category:	Distress
Urgency
Safety.

6.1.3	Self-identification (automatically inserted).

### 6.1.4	Messages

6.1.4.1	For distress calls:

Message 1:	Undesignated distress

Message 2:	Distress coordinates: 9999999999

Message 3:	Time indication: 8888

Message 4:	F3E/G3E simplex.

6.1.4.2	For all other calls:

First telecommand:		F3E/G3E simplex

Second telecommand:	No information

Frequency/channel information:	channel 16

## 6.2	Receive capabilities

The equipment should be able to receive acknowledgements to its own distress calls.

_________________

*	The Director, Radiocommunication Bureau, is requested to bring this Recommendation to the attention of the International Maritime Organization (IMO).