RECOMMENDATION  ITU-R  P.1407

MULTIPATH  PROPAGATION  AND  PARAMETERIZATION
OF  ITS  CHARACTERISTICS

(Question ITU-R 203/3)

(1999)

Rec. ITU-R P.1407

The ITU Radiocommunication Assembly,

considering

a)	the necessity of estimating the effects of multipath on services employing digital systems;

b)	that it is desirable to standardize the terminology and expressions used to characterize multipath,

recommends

1	that, to describe the concepts of multipath in a consistent manner, the terms and definitions given in Annex 1 should be employed.

ANNEX  1

# 1	Introduction

In radio systems with low antenna heights, there are often multiple indirect paths between the transmitter and receiver due to reflections from surrounding objects, in addition to the direct path when there is line-of-sight. Such multipath propagation is particularly significant in urban environments, where the sides of buildings and paved road surfaces provide strong reflections. As a result, the received signal consists of the summation of several components having various amplitudes, phase angles and directions of arrival.

The resulting spatial variability of signal strength can be viewed as having two regimes:

a)	rapid fading which varies over distances of the order of a wavelength due primarily to changes in phase angles of different signal components;

b)	slow fading which varies over larger distances due primarily to changes in shadowing loss by surrounding objects.

In addition, the various signal components can be Doppler shifted by different amounts due to the movement of the mobile or of reflecting objects such as vehicles.

The multipath mobile channel can be characterized in terms of its impulse response which varies at a rate dependent on the speed of the mobile and/or the scatterers. Therefore, a receiver has to be able to cope with the signal distortion arising from echoes in the channel as well as the rapid changes in the nature of this distortion. Such characteristics of the mobile radio channel are described by the power delay profiles and the Doppler spectra which are obtained from wideband channel sounding measurements.

Signals transmitted to and from moving vehicles in urban or forested environments exhibit extreme variations in amplitude due to multiple scattering. Fades of 30 dB or more below the mean level are common. The instantaneous field strength when measured over distances of a few tens of wavelengths is approximately Rayleigh-distributed. The mean values of these small sector distributions vary widely from area to area, depending on the height, density and distribution of hills, trees, buildings and other structures.

Multipath propagation characteristics are a major factor in controlling the quality of digital mobile communications. Physically, multipath propagation characteristics imply multipath number, amplitude, path-length difference (delay), and arrival angle. These can be characterized by the transfer function of the propagation path (amplitude-frequency characteristics), and the correlation bandwidth.

A time-varying linear channel can be characterized by a linear transversal filter. The output of this filter contains a sum of delayed, attenuated and Doppler shifted versions of the input signal. The channel is then represented by the delay-Doppler-spread function, sometimes referred to as the scattering function. This function represents the multipath phenomenon in the three dimensions of excess delay, Doppler frequency and power density. This formulation is particularly suitable for realizing a hardware simulator in the form of a dynamic transversal filter.

# 2	Multipath parameters

The appropriate parameters for the statistical description of multipath effects are given below and can be computed either from instantaneous power delay profiles or from profiles averaged over a few wavelengths.

The average delay is the power weighted-average of the excess delays measured and is given by the first moment of the impulse response.

The r.m.s. delay spread is the power weighted standard deviation of the excess delays and is given by the second moment of the impulse response. It provides a measure of the variability of the mean delay.

The delay window is the length of the middle portion of the power profile containing a certain percentage of the total energy found in that impulse response.

The delay interval is defined as the length of the impulse response between two values of excess delay which mark the first time the amplitude of the impulse response exceeds a given threshold, and the last time it falls below it.

The correlation bandwidth is defined as the frequency for which the autocorrelation function of the transfer function falls below a given threshold.

The total energy, Pm, of the impulse response is:

Pm=t0t3P(t)dt	(1)

where:

P(t) :	power density of the impulse response

t :	excess delay

t0 :	instant when P(t) exceeds the cut-off level for the first time

t3 :	instant when P(t) exceeds the cut-off level for the last time.

The average delay, TD, is given by the first moment of the impulse response:

TD=1PmtLOSt3(t±tLOS)P(t)dt	(2)

where tLOS is the delay of the line-of-sight (LOS) path. The impulse response cannot commence earlier than tLOS, it may, however, start at (t0  tLOS).

The delays may be determined from the following relationship:

ti(ms)=3.3rikm

where ri is the sum of the distances from the transmitter to the multipath reflector, and from the reflector to the receiver, or is the total distance from the transmitter to receiver for tLOS.

The r.m.s. delay spread, S, is defined by the square root of the second central moment:

S=1Pmt0t3t2P(t)dt−1Pmt0t3tP(t)dt2	(3)

or, in discrete form:

S=Piti2Pi−Piti2Pi2	(4)

where:

P1 :	received power from the direct signal

P2...N :	received powers from the reflected signals

t1 :	propagation delay from the direct signal

t2...N :	propagation delays from the reflected signals.

The delay window, Wq, is the length of the middle portion of the impulse response containing a certain percentage, q, of the total energy:

Wq    (t2  –  t1)	(5)

whereby the boundaries t1 and t2 are defined by:

t1t2P(t)dt=q100t0t3P(t)dt=q100Pm	(6)

and the energy outside of the window is split into two equal parts 100−q200Pm.

The delay interval, Ith, is defined as the time difference between the instant t4 when the amplitude of the impulse response first exceeds a given threshold Pth, and the instant t5 when it falls below that threshold for the last time:

Ith    (t5  –  t4)	(7)

The Fourier transform of the power density of the impulse response provides the autocorrelation C( f ) of the transfer function:

C(f)=t0t3P(t) exp(±j2pft)dt	(8)

The correlation bandwidth Bx is defined as the frequency for which |C( f )| equals x% of C( f  0).

Delay windows for 50%, 75% and 90% energy, delay intervals for thresholds of 9, 12 and 15 dB below the peak and correlation bandwidth for 50% and 90% of correlation are recommended for analysis of data. It is worth noting that the effects of noise and spurious signals in the system (from RF to data processing) can be very significant. Therefore, it is important to determine the noise and/or spurious threshold of the systems accurately and to allow a safety margin on top of that. A safety margin of 3 dB is recommended, and in order to ensure the integrity of results, it is recommended that a minimum peak-to-spurious ratio of, for example, 15 dB (excluding the 3 dB safety margin) is used as an acceptance criterion before an impulse response is included in the statistics.

An example of the use of some of these terms is given in Fig. 1.

FIGURE 1/P.1407 = 15 CM