RECOMMENDATION  ITU-R  PN.835-1

REFERENCE  STANDARD  ATMOSPHERE  FOR  GASEOUS  ATTENUATION

(Question ITU-R 201/3)

(1992-1994)

Rec. ITU-R PN.835-1

The ITU Radiocommunication Assembly,

considering

a)	the necessity for a reference standard atmosphere for use in calculating gaseous attenuation along an Earth-space path,

recommends

1.	that the standard atmosphere in Annex 1 be used to determine temperature, pressure and water-vapour pressure as a function of altitude, for calculating gaseous attenuation, except when more reliable local data are available.

ANNEX  1

# 1.	Temperature and pressure

The reference standard atmosphere is based on the United States Standard Atmosphere, 1976, in which the atmosphere is divided into seven successive layers showing linear variation with temperature, as given in Fig. 1.

FIGURE  1/PN.835...[D01] = 12 CM

The temperature T at height h is given by:

(1)

where:

(2)

and Li is the temperature gradient starting at altitude Hi and is given in Table 1.

TABLE  1

| Subscript, i | Altitude, Hi (km) | Temperature gradient, Li (K/km) |
| --- | --- | --- |
| 0 | 00 | –6.5 |
| 1 | 11 | –0.0 |
| 2 | 20 | +1.0 |
| 3 | 32 | +2.8 |
| 4 | 47 | –0.0 |
| 5 | 51 | –2.8 |
| 6 | 71 | –2.0 |
| 7 | 85 |  |

When the temperature gradient Li ¹ 0, pressure is given by the equation:

(3)

and when the temperature gradient Li = 0, pressure is obtained from the equation:

(4)

The ground-level standard temperature and pressure are:

(5)

Note that above about 85 km altitude, local thermodynamic equilibrium of the atmosphere starts to break down, and the hydrostatic equation, on which the above equations are based, is no longer valid.

# 2.	Water-vapour pressure

The distribution of water vapour in the atmosphere is generally highly variable, but may be approximated by the equation:

r(h)  =  r0 exp (–h / h0)mmmmmg/m3	(6)

where the scale height h0 = 2 km, and the standard ground-level water-vapour density is:

(7)

Vapour pressure is obtained from the density using the equation (see Recommendation ITU-R PN.453):

(8)

Water-vapour density decreases exponentially with increasing altitude, up to an altitude where the mixing ratio e(h)/P(h) =  2 ´ 10–6. Above this altitude, the mixing ratio is assumed to be constant.

# 3.	Dry atmosphere for attenuation calculations

The profile of the density of atmospheric gases other than water vapour (the “dry atmosphere”) may be found from the temperature and pressure profiles given in § 1 above.

For attenuation calculations, this density profile may be approximated by an exponential profile according to equation (6) with:

h0  =  6 km	(9)

_________________