Recommendation ITU-R P.835-5 (02/2012)

Reference standard atmospheres

P Series Radiowave propagation

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Annex 1 of Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from http://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Recommendations (Also available online at http://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radiowave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2013

ITU 2013

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION  ITU-R  P.835-5

Reference  standard  atmospheres

(Question ITU-R 201/3)

(1992-1994-1997-1999-2005-2012)

Scope

Recommendation ITU-R P.835 provides expressions and data for reference standard atmospheres required for the calculation of gaseous attenuation on Earth-space paths.

The ITU Radiocommunication Assembly,

considering

a)	the necessity for a reference standard atmosphere for use in calculating gaseous attenuation along an Earth-space path,

recommends

1	that the standard atmospheres in Annex 1 be used to determine temperature, pressure and water-vapour pressure as a function of altitude, for calculating gaseous attenuation when more reliable local data are not available;

2	that the experimental data in Annexes 2 and 3 be used for the locations of interest when seasonal and monthly variations are concerned.

Annex 1

# 1	Mean annual global reference atmosphere

The following reference standard atmosphere reflects the annual mean profiles when averaged across the globe.

## 1.1	Temperature and pressure

The reference standard atmosphere is based on the United States Standard Atmosphere, 1976, in which the atmosphere is divided into seven successive layers showing linear variation with temperature, as given in Fig. 1.

The temperature T at height h is given by:

T(h)    Ti    Li (h  –  Hi)               K	(1)

where:

Ti    T(Hi)	(2)

and Li is the temperature gradient starting at altitude Hi and is given in Table 1.

TABLE 1

| Subscript, i | Altitude, Hi (km) | Temperature gradient, Li (K/km) |
| --- | --- | --- |
| 0 | 0 | –6.5 |
| 1 | 11 | 0.0 |
| 2 | 20 | 1.0 |
| 3 | 32 | 2.8 |
| 4 | 47 | 0.0 |
| 5 | 51 | –2.8 |
| 6 | 71 | –2.0 |
| 7 | 85 |  |

figure 1

Reference profile of atmospheric temperature

When the temperature gradient Li  0, pressure is given by the equation:

(3)

and when the temperature gradient Li  0, pressure is obtained from the equation:

(4)

The ground-level standard temperature and pressure are:

(5)

Note that above about 85 km altitude, local thermodynamic equilibrium of the atmosphere starts to break down, and the hydrostatic equation, on which the above equations are based, is no longer valid.

## 1.2	Water-vapour pressure

The distribution of water vapour in the atmosphere is generally highly variable, but may be approximated by the equation:

(h)    0 exp (–h / h0)               g/m3	(6)

where the scale height h0  2 km, and the standard ground-level water-vapour density is:

0    7.5               g/m3	(7)

Vapour pressure is obtained from the density using the equation (see Recommendation ITU-R P.453):

(8)

Water-vapour density decreases exponentially with increasing altitude, up to an altitude where the mixing ratio e (h)/P(h)  2  10–6. Above this altitude, the mixing ratio is assumed to be constant.

## 1.3	Dry atmosphere for attenuation calculations

The profile of the density of atmospheric gases other than water vapour (the “dry atmosphere”) may be found from the temperature and pressure profiles given in § 1.1.

For attenuation calculations, this density profile may be approximated by an exponential profile according to equation (6) with:

h0    6 km	(9)

# 2	Low-latitude annual reference atmosphere

For low latitudes (smaller than 22°) the seasonal variations are not very important and a single annual profile can be used.

The temperature T (K) at height h (km) is given by:

T(h)  300.4222 – 6.3533 h  0.005886 h2	for	 0  h  17

T(h)  194  (h – 17) 2.533	for	17  h  47

T(h)  270		for	47  h  52

T(h)  270 – (h – 52) 3.0714	for	52  h  80

T(h)  184		for	80  h  100

while the pressure P (hPa):

P(h)  1012.0306 – 109.0338 h  3.6316 h2	for	 0  h  10

P(h)  P10 exp [–0.147 (h – 10)]	for	10  h  72

P(h)  P72 exp [–0.165 (h – 72)]	for	72  h  100

where P10 and P72 are the pressures at 10 and 72 km respectively.

For water vapour (g/m3):

(h) 	19.6542 exp [–0.2313 h – 0.1122 h2  0.01351 h3

– 0.0005923 h4]	for	0  h  15

(h) 	0		for	h  15

# 3	Mid-latitude reference atmosphere

For mid-latitudes (between 22 and 45) the following profiles may be used for the summer and winter.

## 3.1	Summer mid-latitude

The temperature T (K) at height h (km) is given by:

T(h)  294.9838 – 5.2159 h – 0.07109 h2	for	 0  h  13

T(h)  215.15	for	13  h  17

T(h)  215.15 exp [(h – 17) 0.008128]	for	17  h  47

T(h)  275		for	47  h  53

T(h)  275  {1 – exp [(h – 53) 0.06] } 20	for	53  h  80

T(h)  175		for	80  h  100

while the pressure P (hPa):

P(h)  1012.8186 – 111.5569 h  3.8646 h2	for	 0  h  10

P(h)  P10 exp [– 0.147 (h – 10)]	for	10  h  72

P(h)  P72 exp [– 0.165 (h – 72)]	for	72  h  100

where P10 and P72 are the pressures at 10 and 72 km respectively.

For water vapour (g/m3):

(h) 	14.3542 exp [– 0.4174 h – 0.02290 h2

 0.001007 h3]	for	0  h  15

(h) 	0		for	h  15

## 3.2	Winter mid-latitude

The temperature T (K) at height h (km) is given by:

T(h)  272.7241 – 3.6217 h – 0.1759 h2	for	 0  h  10

T(h)  218		for	10  h  33

T(h)  218  (h – 33) 3.3571	for	33  h  47

T(h)  265		for	47  h  53

T(h)  265 – (h – 53) 2.0370	for	53  h  80

T(h)  210		for	80  h  100

while the pressure P (hPa):

P(h)  1018.8627 – 124.2954 h  4.8307 h2	for	 0  h  10

P(h)  P10 exp [– 0.147 (h – 10)]	for	10  h  72

P(h)  P72 exp [– 0.155 (h – 72)]	for	72  h  100

where P10 and P72 are the pressures at 10 and 72 km respectively.

For water vapour (g/m3):

(h) 	3.4742 exp [– 0.2697 h – 0.03604 h2

 0.0004489 h3]	for	0  h  10

(h) 	0		for	h  10

# 4	High latitude reference atmosphere

For high latitudes (higher than 45) the following profiles may be used for the summer and winter.

## 4.1	Summer high latitude

The temperature T (K) at height h (km) is given by:

T(h)  286.8374 – 4.7805 h – 0.1402 h2	for	 0  h  10

T(h)  225		for	10  h  23

T(h)  225 exp [(h – 23) 0.008317]	for	23  h  48

T(h)  277		for	48  h  53

T(h)  277 – (h – 53) 4.0769	for	53  h  79

T(h)  171		for	79  h  100

while the pressure P (hPa):

P(h)  1008.0278 – 113.2494 h  3.9408 h2	for	 0  h  10

P(h)  P10 exp [–0.140 (h – 10)]	for	10  h  72

P(h)  P72 exp [–0.165 (h – 72)]	for	72  h  100

where P10 and P72 are the pressures at 10 and 72 km respectively.

For water vapour (g/m3):

(h) 	8.988 exp [– 0.3614 h – 0.005402 h2

– 0.001955 h3]	for	0  h  15

(h) 	0		for	h  15

## 4.2	Winter high latitude

The temperature T (K) at height h (km) is given by:

T(h)  257.4345  2.3474 h – 1.5479 h2  0.08473 h3	for	 0  h  8.5

T(h)  217.5	for	 8.5  h  30

T(h)  217.5  (h – 30) 2.125	for	30  h  50

T(h)  260		for	50  h  54

T(h)  260 – (h – 54) 1.667	for	54  h  100

while the pressure P (hPa):

P(h)  1010.8828 – 122.2411 h  4.554 h2	for	 0  h  10

P(h)  P10 exp [–0.147 (h – 10)]	for	10  h  72

P(h)  P72 exp [–0.150 (h – 72)]	for	72  h  100

where P10 and P72 are the pressures at 10 and 72 km respectively.

For water vapour (g/m3):

(h)  1.2319 exp [0.07481 h – 0.0981 h2  0.00281 h3]	for	0  h  10

(h)  0			for	h  10

Annex 2

# 1	Experimental data of atmospheric vertical profiles

Monthly averages of vertical profiles of temperature, pressure and relative humidity were calculated for 353 locations over the world, using 10 years (1980-1989) of radiosonde observations. This dataset (DST.STD) is available from ITU/BR and contains the mean monthly vertical profiles, for both 00.00 UTC and 12.00 UTC, of pressure, temperature and relative humidity. These profiles, calculated in the absence of rain, range from 0 to 16 km with a step of 500 m. The mean monthly profiles are contained in ASCII files named <WMO_code>.dat, where WMO_code is the code name of the site according to the World Meteorological Organization (e.g.: 03496.dat, 03496 is the WMO station code for Hemsby-in-Norfolk). An example of one profile is given in Table 2. The list of locations is contained in an ASCII file (using the comma separated value file, CSV, file format) called dst_std_lst.csv. Each record of this file contains the following field: WMO_CODE, Station Name, Country, Latitude, Longitude, Altitude above sea level. An example of such a record is given in Table 3.

Above the maximum altitude, extrapolation can be performed by using the reference profiles given in Annex 1. To translate the relative humidity into absolute values of water-vapour density, the formulae contained in Recommendation ITU-R P.453 should be used.

TABLE 2

DST.STD data format – Example of month average profile
(station 10410)

| YYMMDDHH NL |  |  |  |
| --- | --- | --- | --- |
| 99 199 0 33 |  |  |  |
| Press (hPa) | Z (km) | Temp (K) | RH (%/100) |
| 1 016.905 | 0.00 | 273.62 | 0.864E+00 |
| 956.686 | 0.50 | 273.33 | 0.830E+00 |
| 898.555 | 1.00 | 271.74 | 0.754E+00 |
| 844.014 | 1.50 | 269.59 | 0.665E+00 |
| 791.860 | 2.00 | 267.15 | 0.591E+00 |
| 742.661 | 2.50 | 264.56 | 0.518E+00 |
| 696.285 | 3.00 | 261.89 | 0.470E+00 |
| 651.977 | 3.50 | 258.94 | 0.458E+00 |
| 610.086 | 4.00 | 255.88 | 0.448E+00 |
| 570.467 | 4.50 | 252.69 | 0.445E+00 |
| 533.076 | 5.00 | 249.33 | 0.451E+00 |
| 497.767 | 5.50 | 245.90 | 0.453E+00 |
| 464.123 | 6.00 | 242.32 | 0.450E+00 |
| 432.441 | 6.50 | 238.75 | 0.450E+00 |
| 402.414 | 7.00 | 235.16 | 0.443E+00 |
| 374.177 | 7.50 | 231.59 | 0.437E+00 |
| 347.236 | 8.00 | 228.12 | 0.433E+00 |
| 322.281 | 8.50 | 224.88 | 0.427E+00 |
| 298.474 | 9.00 | 221.89 | 0.421E+00 |
| 276.492 | 9.50 | 219.27 | 0.416E+00 |
| 255.527 | 10.00 | 217.08 | 0.411E+00 |
| 236.297 | 10.50 | 215.62 | 0.402E+00 |
| 218.415 | 11.00 | 214.79 | 0.393E+00 |
| 201.366 | 11.50 | 214.14 | 0.348E+00 |
| 186.214 | 12.00 | 214.02 | 0.205E+00 |
| 172.093 | 12.50 | 214.24 | 0.104E+00 |
| 158.709 | 13.00 | 214.66 | 0.368E-01 |
| 146.492 | 13.50 | 214.94 | 0.351E-02 |
| 135.813 | 14.00 | 214.88 | 0.120E-02 |
| 125.690 | 14.50 | 214.50 | 0.117E-02 |
| 116.027 | 15.00 | 214.01 | 0.113E-02 |
| 106.798 | 15.50 | 213.56 | 0.110E-02 |
| 98.291 | 16.00 | 213.26 | 0.107E-02 |

Legend to Table 2: YY = Year (99 for mean monthly profiles) MM = Month (1 = January, 2 = February, ...) DD = Day of the month (99 for mean monthly profiles) HH = Hour of the day (UTC) NL = Number of vertical levels (NL = 33 for STD.DST) Press (hPa) = Atmospheric total pressure Z (km) = Height above the Earth’s surface Temp (K) = Air temperature RH (%/100) = Relative humidity (as a fraction) NOTE 1 – The level values of Temp and Press may be set to zero if unrecorded.

TABLE  3

DST_STD_LST.CSV station information file – Example of record structure

| WMO code | Station name | Country | Latitude (degrees) | Longitude (degrees) | Asl (m) |
| --- | --- | --- | --- | --- | --- |
| 10 410 | ESSEN | DL | 51.4 | 6.967 | 153 |
| NOTE 1 – Latitude and longitude values are in decimal degrees (i.e. 51.4 = 51º 24'). |  |  |  |  |  |

Annex 3

# 1	Numerical weather prediction data of atmospheric vertical profiles

Monthly averages, conditioned to the hour of the day, of vertical profiles of temperature, pressure and water-vapour density were calculated using the ECMWF 15-year data set (ERA15) from the re-analysis project. This dataset (ESA_STD_PROF) is available from ITU/BR and contains the mean monthly vertical profiles, for 00.00, 06.00, 12.00 and 18.00 UTC, of total air pressure, air temperature and water-vapour density. These profiles extend from a reference height located around the local Earth’s surface up to about 30 km above the Earth’s surface and contains 32 levels derived from ERA15 model levels. The data are from 0 to 360 in longitude and from +90 to −90 in latitude, with a resolution of 1.5º in both latitude and longitude. All the data are stored in files using the IEEE floating point single precision standard (4 bytes, 32 bits) in a Big-Endian format.

The mean monthly profiles of each meteorological parameter are contained in binary files named <param>_<hh>.bin, where param is the name of the meteorological parameter (pres = total air pressure [hPa], temp = air temperature [K], vapd = water-vapour density [g/m3]) and hh is the hour of the day (i.e. 00, 06, 12 and 18 [UTC]). The heights of the profile levels are contained in a separate binary file, named hght.bin, in which vertical profiles of mean monthly level heights are contained. An example of the data contained in the database for a specific is given in Table 4.

TABLE 4

ESA_STD_PROF content – Example of profile at grid point
(latitude = 45 (degrees) and longitude = 9 (degrees))
at 12 UTC of July

| Height (m) | Press (hPa) | Temp (K) | Vapd (g/m3) |
| --- | --- | --- | --- |
| 668.309 | 939.255 | 298.373 | 9.823 |
| 701.645 | 935.673 | 298.125 | 9.617 |
| 819.406 | 923.092 | 296.598 | 9.302 |
| 1 029.200 | 900.957 | 294.292 | 8.811 |
| 1 312.119 | 871.693 | 291.459 | 8.099 |
| 1 653.510 | 837.298 | 288.287 | 6.992 |
| 2 042.286 | 799.373 | 285.107 | 5.706 |
| 2 470.212 | 759.191 | 282.116 | 4.555 |
| 2 931.283 | 717.723 | 279.045 | 3.641 |
| 3 421.197 | 675.691 | 275.934 | 2.692 |
| 3 937.159 | 633.633 | 272.913 | 1.855 |
| 4 477.475 | 591.936 | 269.707 | 1.286 |
| 5 040.996 | 550.876 | 266.183 | 0.911 |
| 5 627.126 | 510.656 | 262.354 | 0.636 |
| 6 235.769 | 471.427 | 258.213 | 0.428 |
| 6 867.105 | 433.307 | 253.687 | 0.277 |
| 7 521.528 | 396.390 | 248.780 | 0.173 |
| 8 199.571 | 360.767 | 243.521 | 0.103 |
| 8 901.801 | 326.527 | 237.971 | 0.058 |
| 9 629.047 | 293.764 | 232.319 | 0.034 |
| 10 382.883 | 262.580 | 226.984 | 0.019 |
| 11 167.396 | 233.064 | 222.845 | 0.009 |
| 11 990.928 | 205.263 | 220.483 | 0.003 |
| 12 864.380 | 179.195 | 219.279 | 0.001 |
| 13 799.389 | 154.827 | 218.154 | 0.001 |
| 14 812.536 | 132.043 | 217.057 | 0.001 |
| 15 934.765 | 110.604 | 216.026 | 0.000 |
| 17 228.709 | 90.110 | 215.674 | 0.000 |
| 18 821.158 | 70.037 | 216.262 | 0.000 |
| 20 964.607 | 50.038 | 219.300 | 0.000 |
| 24 270.756 | 30.039 | 223.166 | 0.000 |
| 31 430.756 | 10.320 | 232.854 | 0.000 |

Legend to Table 4: Z (m) = Height with respect to sea level Press (hPa) = Atmospheric total pressure Temp (K) = Air temperature Vapd (g/m3) = Water-vapour density NOTE 1 – The Matlab and Fortran procedures to access the ESA_STD_ PROF dataset are available from the ITU-R website dealing with Radiocommunication Study Group 3.

______________