RECOMMENDATION  ITU-R  S.1062

ALLOWABLE  ERROR  PERFORMANCE  FOR  A  HYPOTHETICAL  REFERENCE
DIGITAL  PATH  OPERATING  AT  OR  ABOVE  THE  PRIMARY  RATE

(Question ITU-R 75/4)

(1994)

Rec.  ITU-R S.1062

The ITU Radiocommunication Assembly,

considering

a)	that satellites operating in the fixed-satellite service play an important role in providing reliable international digital communications;

b)	that satellite link performance must be sufficient to allow compliance with overall end-to-end performance objectives and in turn end-user quality objectives;

c)	that satellite link performance is generally distance independent;

d)	that Recommendation ITU-R S.614 specifies satellite link performance objectives which comply with the objectives specified in ITU-T Recommendation G.821;

e)	that the error performance for connections operating at or above the primary rate has been specified by the ITU-T in ITU-T Recommendation G.826;

f)	that in defining error performance criteria, it is necessary to take into account all foreseeable error-inducing mechanisms, especially time-varying propagation conditions and interference;

g)	that satellite systems can be designed to meet a wide range of performance requirements,

recommends

1.	that future and, wherever possible, existing satellite links within the public switched network operating at or above the primary rate should be designed to at least meet the specifications set forth in ITU-T Recommendation G.826. An example set of masks derived from G.826 parameters is presented in Note 1;

2.	that the methodology explained in Annex 1 can be used to generate the necessary bit-error probability (BEP) (see Note 4) performance masks specified in Note 1. The same methodology can be used at a 155 Mbit/s rate to derive the mask in Note 2;

3.	that the following Notes should be regarded as part of the Recommendation:

Note 1 – In order to fully comply with the requirements of ITU-T Recommendation G.826, the bit-error ratio (BER) divided by the average number of errors per burst (BER/a, see § 3 of Annex 1) at the output (i.e. at either end of a two-way connection) of a satellite hypothetical reference digital path (HRDP) forming part of an international connection operating at or above the primary rate including 155 Mbit/s should not exceed during the total time (worst month) the performance masks defined by the values given below and also in the BEP masks given in Fig. 1.

Note 2 – Although Note 1 assures full compliance with ITU-T Recommendation G.826, a more stringent mask may be desirable or necessary for certain services. An example mask follows:

| Bit rate (Mbit/s) | - Percentage of total time (worst month) | BER/a |
| --- | --- | --- |

| - 1.5 | - 0 0.2 2.0 10.0 | - 7 × 10–7 3 × 10–8 5 × 10–9 |
| --- | --- | --- |

| - 2.0 | - 0.2 2.0 10.0 | - 7 × 10–6 2 × 10–8 2 × 10–9 |
| --- | --- | --- |

| - 6.0 | - 0.2 2.0 10.0 | - 8 × 10–7 1 × 10–8 1 × 10–9 |
| --- | --- | --- |

| - 51.0 | - 0.2 2.0 10.0 | - 4 × 10–7 2 × 10–9 2 × 10–10 |
| --- | --- | --- |

| - 155,0 | - 0.2 2.0 10.0 | - 1 × 10–7 1 × 10–9 1 × 10–10 |
| --- | --- | --- |

The BER at the output (i.e. at either end of a two-way connection) of a satellite HRDP operating at or above the primary rate up to and including 155 Mbit/s should not exceed during the total time (worst month) the performance mask defined by the values given below:

| Percentage of total time (worst month) | BER/a | For a = 10 (BER) |
| --- | --- | --- |

| - 00.2 02 10 | - 1 × 10–7 1 × 10–9 1 × 10–10 | - 1 × 10–6 1 × 10–8 1 × 10–9 |
| --- | --- | --- |

Note 3 – The HRDP referred to in this Recommendation is specified in Recommendation ITU-R S.521.

Note 4 – The BERs given in Notes 1 and 2 should be measured over a sufficiently long period of time in order to ensure that they provide a good estimate of the BEP. A method for measuring BERs as a function of percentage of time is given in Annex 1 of Recommendation ITU-R S.614.

Note 5 – For ease of application of this Recommendation the values for the objectives given in Notes 1 and 2 are given in terms of total time and represent the limits of a bit-error rate performance model utilizing the method outlined in Annex 1. In arriving at the objectives given in Notes 1 and 2 the errors occurring during the unavailable time have been excluded from the calculation of the objectives. An explanation of the relationship between available time and total time is given in Note 7. The objectives for BERs given in Note 1 are not unique in meeting the requirements of ITU-T Recommendation G.826. Other sets of objectives may be used by the designer where appropriate as long as the sets of objectives satisfy ITU-T Recommendation G.826.

Note 6 – This Recommendation will find its primary application in satellite systems operating below 15 GHz. The extension of the performance requirements given in this Recommendation to systems operating at higher frequencies is the subject of further study.

Note 7 – A period of unavailable time begins at the onset of ten consecutive severely errored seconds (SES) events. These 10 s are considered to be part of unavailable time. A new period of available time begins at the onset of ten consecutive non-SES events. These 10 s are considered to be part of available time. For rates at or above the primary rate, an SES is defined as a second which contains ³ 30% errored blocks or at least one severely disturbed period (see ITU-T Recommendation G.826, § 3.1.1 and Annex 1). Bit-error probabilities corresponding to the available/unavailable state threshold are given in Table 4.

Note 8 – The objectives given in Notes 1 and 2 are given in terms of percentage of the worst month. These monthly percentages correspond to the following yearly percentages:

–	10% of worst month		4.0% of year

–	2% of worst month		0.6% of year

–	0.2% of worst month	0.04% of year.

Note 9 – In order to comply with Notes 1 and 2 at frequencies greater than 10 GHz, it may be advantageous to make use of fade countermeasures including adaptive forward error-correction (FEC) coding, power control or site diversity. Information on site diversity operation is given in Annex 1, Recommendation ITU-R S.522.

Note 10 – The preferred method of verifying digital satellite performance is on the basis of in-service measurements (ISM). These measurements would utilize the block error detection schemes which are related to the inherent block size and structure of the transmission system. FEC, scrambling and differential encoding have an impact on interpretation of the measurements (see Annex 1, § 3).

Note 11 – The error performance described in Notes 1 and 2 was developed based on the use of an HRDP in the international portion of the link (e.g. switched international gateway-to-switched international gateway). Other applications of the HRDP within the connection are possible (e.g. end office-to-end office) and the error performance objectives can be adjusted accordingly.

Note 12 - Existing and future satellite links within the public switched network should, whenever possible, be designed/upgraded to the performance objectives specified in this Recommendation. However, it may not always be practical to upgrade an existing system which was designed to comply with Recommendation ITU-R S.614.

Note 13 – The methods described in this Recommendation can be applied to the design of satellite links in private networks, although the BER masks may not be appropriate depending on the configuration of the network and the services carried.

Note 14 – The performance objectives shall be met for the relevant end-to-end transmission having the maximum rate rather than the ostensible transmission rate. For instance, if the transmission rate over a satellite link is 6 Mbit/s and the maximum transmission rate involved between the end points is 2 Mbit/s, the performance objectives for 2 Mbit/s transmission shall be applied in designing the satellite link.

ANNEX  1

# 1.	ITU-T Recommendation G.826 – Definitions, parameters and objectives

Consistent with ITU-T Recommendation G.821, the requirements of ITU-T Recommendation G.826 are given in terms of errored intervals (EI). The terminology between the two Recommendations is similar but the definitions of the parameters are different. For ITU-T Recommendation G.826, the EI are defined in terms of errored blocks (EB) as opposed to individual bit errors. The purpose of this specification is to allow the verification of adherence to the performance requirements of ITU-T Recommendation G.826 on an in-service basis. The specification of performance in terms of block errors instead of bit errors has important consequences for systems where the errors tend to occur in groups, such as systems employing scrambling and FEC. The block used in ITU-T Recommendation G.826 is that group of contiguous bits that normally makes up the inherent monitoring block or frame of the transmission system being employed.

## 1.1	Definitions

Events

–	Errored block (EB)

A block in which one or more bits are in error.

–	Errored second (ES)

A 1 s period with one or more errored blocks. SES defined below are a subset of ES.

–	Severely errored second (SES)

A 1 s period which contains ³ 30% errored blocks (see Note 1) or at least one severely disturbed period (SDP) (see Note 2).

For out-of-service (OOS) measurements, an SDP occurs when, over a minimum period of time equivalent to four contiguous blocks or 1 ms, whichever is larger, either all the contiguous blocks are affected by a high binary error density of ³ 10–2, or a loss of signal information is observed. For in-service monitoring purposes, an SDP is estimated by the occurrence of a network defect. The term defect is defined in the relevant Annexes (2, 3 or 4 of ITU-T Recommendation G.826) for the different network fabrics: plesiochronous digital hierarchy (PDH), synchronous digital hierarchy (SDH) or cell-based, respectively.

Note 1 – For historical reasons, SESs on some PDH systems are defined with a different percentage of errored blocks (see Annex 2 of ITU-T Recommendation G.826).

For maintenance purposes, values different from 30% may be used and these values may vary with transmission rate.

Note 2 – SDP events may persist for several seconds and may be precursors to periods of unavailability, especially when there are no restoration/protection procedures in use. SDPs persisting for T s, where 2 £T < 10 (some network operators refer to these events as “failures”) can have a severe impact on service, for example the disconnection of switched services. The only way ITU-T Recommendation G.826 limits the frequency of these events is through the limit for the severely errored seconds ratio (SESR).

–	Background block error (BBE)

An errored block not occurring as part of an SES.

## 1.2	Parameters

Error performance should only be evaluated while the path is in the available state. For a definition of the entry/exit criteria for the unavailable state see Annex 1 of ITU-T Recommendation G.826.

–	Errored second ratio (ESR)

The ratio of ES to total seconds in available time during a fixed measurement interval.

–	Severely errored seconds ratio (SESR)

The ratio of SES to total seconds in available time during a fixed measurement interval.

–	Background block error ratio (BBER)

The ratio of errored blocks to total blocks during a fixed measurement interval, excluding all blocks during SES and unavailable time.

## 1.3	Performance objectives

The end-to-end objectives of ITU-T Recommendation G.826 are given in Table 1. The performance objectives are given as a function of transmission system bit rate. The ranges of block sizes accommodated at these bit rates are also given. As stated above, the block size will be that associated with the frame structure of the transmission system. Ranges of block size are given so as not to prejudice the development of future transmission systems. These objectives are specified for available time.

TABLE  1

End-to-end performance objectives for a 27 500 km international digital
connection at, or above, the primary rate

| Rate (Mbit/s) | 1.5 to 5 | > 5 to 15 | > 15 to 55 | > 55 to 160 | > 160 to 3 500 | > 3 500 |
| --- | --- | --- | --- | --- | --- | --- |

| Bits/block | 2 000-8 000(1) | 2 000-8 000 | 4 000-20 000 | 6 000-20 000 | 15 000-30 000(2) | For further study |
| --- | --- | --- | --- | --- | --- | --- |

| ESR | 0.04 | 0.05 | 0.075 | 0.16 | (3) | For further study |
| --- | --- | --- | --- | --- | --- | --- |

| SESR | 0.002 | 0.002 | 0.002 | 0.002 | 0.002 | For further study |
| --- | --- | --- | --- | --- | --- | --- |

| BBER | 3 × 10–4 | 2 × 10–4 | 2 × 10–4 | 2 × 10–4 | 10–4 | For further study |
| --- | --- | --- | --- | --- | --- | --- |

- (1) VC-11 and VC-12 (ITU-T Recommendation G.709) paths are defined with a number of bits/block of 832 and 1 120 respectively, i.e., outside of the recommended range for 1.5 to 5 Mbit/s paths. For these block sizes, the BBER objective for VC-11 and VC-12 is 2 × 10–4.

- (2) Because bit-error ratios are not expected to decrease dramatically as the bit rates of transmission systems increase, the block sizes (bits) used in evaluating very high bit-rate paths should remain within the range of 15 000 to 30 000 bits/block. Preserving a constant block size for very high bit-rate paths results in relatively constant BBER and SESR objectives for these paths.

- As currently defined, VC-4-4c (ITU-T Recommendation G.709) is a 601 Mbit/s path with a block size of 75 168 bits/block. Since this exceeds the maximum recommended block size for a path of this rate, VC-4-4c paths should not be estimated in service using this table. The BBER objective for VC-4-4c using the 75 168 bit block size is taken to be 4 x 10–4. There are currently no paths defined for bit rates greater than VC-4-4c (> 601 Mbit/s). Digital sections are defined for higher bit rates and guidance on evaluating the performance of digital sections can be found below.

- (3) Due to the lack of information on the performance of paths operating above 160 Mbit/s, no ESR objectives are recommended at this time. Nevertheless, ESR processing should be implemented within any error performance measuring devices operating at these rates for maintenance or monitoring purposes.

Independent of the actual distance spanned, any satellite hop in the international or national portion receives a 35% allocation of the end-to-end objectives. The performance objectives for a satellite HRDP are given in Table 2 for transmission rates between 1.5 and 3 500 Mbit/s.

TABLE  2

Satellite hypothetical reference digital path performance objectives for an international 
or national digital connection operating at, or above, primary rate

| Rate (Mbit/s) | 1.5 to 5 | > 5 to 15 | > 15 to 55 | > 55 to 160 | > 160 to 3 500 |
| --- | --- | --- | --- | --- | --- |

| ESR | 0.014 | 0.0175 | 0.0262 | 0.056 | (1) |
| --- | --- | --- | --- | --- | --- |

| SESR | 0.0007 | 0.0007 | 0.0007 | 0.0007 | 0.0007 |
| --- | --- | --- | --- | --- | --- |

| BBER | 1.05 × 10–4 | 0.7 × 10–4 | 0.7 × 10–4 | 0.7 × 10–4 | 0.35 × 10–4 |
| --- | --- | --- | --- | --- | --- |

- (1) Due to the lack of information on the performance of paths operating above 160 Mbit/s, no ESR objectives are recommended at this time. Nevertheless, ESR processing should be implemented within any error performance measuring devices operating at these rates for maintenance or monitoring purposes.

## 1.4	Monitoring blocks

The block used in ITU-T Recommendation G.826 is the inherent monitoring block of the transmission system employed. Table 3 shows the block size and number of blocks/s for various transmission rates.

TABLE  3

Relationship between bit rate, block size and number of blocks per second

| Bit rate (Mbit/s) | Block size (bits) | Number of blocks/s |
| --- | --- | --- |

| - 1.544 | - 4 632 | 333 1/3 |
| --- | --- | --- |

| - 2.048 | - 2 048 | 1 000 |
| --- | --- | --- |

| - 6.312 | - 3 156 | 2 000 |
| --- | --- | --- |

| - 44.736 | - 4 760 | 9 398 63/119 |
| --- | --- | --- |

| - 51.840 | - 6 480 | 8 000 |
| --- | --- | --- |

| - 155.520 | - 19 440 | 8 000 |
| --- | --- | --- |

# 2.	Bit-error probability masks

The set of parameters and objectives as defined in ITU-T Recommendation G.826 is not suitable for the transmission system design. It must be transformed into a bit-error probability versus percentage-of-time distribution, also called a bit-error probability mask, in such a way that any digital transmission designed to meet the mask would also meet the objectives of the Recommendation. The transform, however, does not result in a “unique” mask.

Two approaches for determining bit-error masks are provided here. Approach 1, based on the Poisson distribution, is pragmatic and provides useful BER masks with minimal computational complexity. Approach 2, based on the Neyman – A contagious distribution which considers the distribution of bit errors within a burst, is computationally complex. Either approach leads to BER masks that are, for practical transmission design purposes, identical.

## 2.1	Approach 1: Poisson distribution

### 2.1.1	Probability of the basic events

On a satellite link, the probability of an errored block is a function of time because the BEP depends on the propagation conditions. Hence, the probability of an errored block, PEB(t), assuming a Poisson distribution of error bursts, can be expressed as:

µ §

where:

BEPCRC (t)  =  BEP(t) / a

a :	average number of errors per burst (a ³ 1), and

NB :	number of bits per block.

The probability of an errored second, PES (t), can be expressed as:

µ §

where:

n :	 number of blocks/s.

Since the probability of k errored blocks in a total of n blocks, Pn,k, (t), is given by:

µ §

then, the probability of an SES, PES (t), is:

PSES (t)  =

### 2.1.2	Generation of masks

Assuming a general form of the mask given in Fig. 1, and using the probability formula given above one can compute the ESR. The ESR defined as the total errored seconds (seconds with one or more errored blocks) divided by the total available seconds, Ta, is given by:

µ §

Similarly, the SESR is given by:

µ §

If PES (t) and PSES (t) are assumed to be piece-wise constant in time, then ESR and SESR can be expressed as:

and		µ §

Where M is the total number of time intervals, PESi (t) and PSESi (t) are the probability of an ES and SES respectively in the i-th time interval, and Dti is the i-th time interval divided by Ta.

BBER is defined as the ratio between errored blocks to the total blocks during available seconds, excluding all blocks during SES. Thus:

BBER  =

If we set  µ §, then BBER can be expressed as:

The above method will obviously result in an infinite number of models meeting the ITU-T Recommendation G.826 performance objectives. Therefore, the following assumptions are made:

1.	The mask is a 3-point mask with characteristic points at 0.2%, 2% and 10% of the time.

2.	In the region t < 0.2% the mask is represented by a straight line of a slope such that ESR, SESR and BBER are at their highest values.

3.	The BEP/a at 2% and BEP/a at 10% differ by one decade.

4.	The unavailability threshold (Tth) is defined by PSES = 0.5.

The values of BEPth /a which satisfy the above assumptions and the condition BEP £ 10–3 can be found from Fig. 2. These values, at various bit rates, are listed in Table 4. Alternatively, the BEPth /a can be computed from the formula:

µ §

FIGURE 1/S.1062...[D01] = 9 cm

FIGURE 2/S.1062...[D02] = 15,5 CM

TABLE  4

| Bit rate (Mbit/s) | BEPth/a |
| --- | --- |

| 001.544 | 8 × 10–5 |
| --- | --- |

| 002.048 | 2 × 10–4 |
| --- | --- |

| 006.432 | 1 × 10–4 |
| --- | --- |

| 051.84 | 6 × 10–5 |
| --- | --- |

| 155.52 | 2 × 10–5 |
| --- | --- |

The following process is followed to ensure that assumption 2 is met:

Step 1	–	Set the mask values at 10%, 2% and 0.2% of the time.

Step 2	–	Determine the value BEPth /a from Table 4.

Step 3	–	Assume a straight line of an arbitrary slope, in the mask region t < 0.2%.

Step 4	–	Determine the time percentage Tth that corresponds to BEPth /a.

Step 5	–	Calculate ESR, SESR and BBER incorporating 10% of Tth into the available time. (This fraction of time is incorporated into the available time to account for the case where the BEP is worse than the BEPth but recovers in less than 10 s.)

Step 6	–	Select a new slope and repeat steps 3 through 5 until the highest values of ESR, SESR and BBER are found.

If the criteria for ESR, SESR and BBER are satisfied for all slopes, then the defined mask is considered to meet the ITU-T Recommendation G.826.

FIGURE 3/S.1062...[D03] = 13 CM

By an iterative process, repeating the computations of ESR, SESR and BBER, a mask can be generated which satisfies the objectives of ITU-T Recommendation G.826. A set of masks, for various transmission rates is given in Fig. 3. In developing the masks, assumption 3 was changed for the 1.5 Mbit/s data rate in order to achieve a smooth mask. Another set of the masks was generated with assumption 3 modified for each transmission rate and is shown in Fig. 3a.

FIGURE 3a/S.1062...[D04] = 12.5 CM

## 2.2	Approach 2: Neyman – A Contagious distribution

A set of masks was also developed using the Neyman – A Contagious distribution. The results were very similar to those shown in Fig. 3a. The procedure for using this method along with an example of how to generate masks is given in the following sections.

### 2.2.1	Procedure

The ESR, SER and BBER, are expressed in terms of EB, thus probability of EB must be calculated first. In doing so one must ensure that the bursty nature of errors are taken into account. One statistical model that handles the random occurrences of bursts, and the random occurrences of errors, within the bursts is the Neyman – A Contagious distribution. The Neyman – A Contagious distribution is given by the following equation:

µ §

where:

P(n) :	probability that n errors occur in N transmitted bits

a :		expected number of errors within a burst

Abs :		average number of bursts per block.

Since an EB is defined as a block having one or more bits in error, then the probability of an EB, PB, can be written as:

PB(EB)   =   1  –  P(0)

with:

P(0) :	probability of zero errors in N transmitted bits (N = number of bits in a block) and based on the Neyman – A Contagious distribution it is given by:

µ §

The following parameters were used with the Neyman method to calculate PB (EB).

a 	=  10

Abs 	=  (BER*N) / a

The value of a = 10 is used in this example and corresponds to the rate 3/4 FEC as in INTELSAT’s IDR standard (see Fig. 3).

Once PB (EB) is calculated, then probability of errored second, P(ES), probability of severely errored second, P(SES) and background block error ratio, BBER, can be determined, and since PB (EB) is a function of BER, all the above named parameters are functions of BER and they can be determined as follows:

#### 2.2.1.1	Determination of P(ES)

An ES is by definition a second which has one or more errored blocks. Based on this definition the probability of an ES can be written as:

µ §

where:

PB (EB) :	probability of errored block

M :		number of blocks in a second.

#### 2.2.1.2	Determination of P(SES)

An SES, is defined as a 1 s period which contains ³ 30% errored blocks, or at least one severely disturbed period (SDP), where an SDP is defined as occurring when over a minimum period of time equivalent to four contiguous blocks, either all the contiguous blocks are affected by a high binary error density of > 1–2, or a loss of signal information is observed. The probability of an SES can then be written as:

P(SES)  =  P(³ 30% EB)  +  P(³ 1 SDP)  –  P(³ 30% EB and ³ 1 SDP)

but P(³ = 1 SDP) is small and negligible compared to P(³ = 30% EB) for the BER values of interest, so in our analysis we have used:

P(SES)  =  P(³ 30% EB)

Based on the definition and assumption given above, the probability of an SES, P(SES), can be written as:

µ §

where:

PB  =  PB (EB): probability of errored block

M :	number of blocks/s.

#### 2.2.1.3	Determination of BBER

A background block error, BBE, is defined as an errored block that is neither part of a severely errored second nor is in the unavailable time. Background block error ratio (BBER), is then defined as the ratio between BBE and the total number of blocks not including blocks in a severely errored second or blocks in unavailable time. It then follows that:

µ §

where PB, P(SES) and M are as defined above, and provided that the second is not during the unavailable time.

### 2.2.2	Generation of masks

Since P(ES), P(SES), and BBER are functions of BER, one can plot graphs of these quantities versus BER. A set of sample graphs for 6.312 Mbit/s is shown in Figs. 4 to 6.

From the graph of P(ES) versus BER, Fig. 4, it can be seen that on a log-log scale, P(ES) varies linearly with BER for up to a certain BER, after which the curve becomes flat, i.e. P(ES) becomes 1. This BER value is named BERt1. Therefore, one can write:

P(ES)  =  k1 * BER   for BER  <  BERt1

where k1 is the slope of the line and is data-rate dependent.

The graph of P(SES) versus BER, Fig. 5, shows a brick wall behaviour, i.e. below a certain BER, BERt2, P(SES) is 1.

The graph of BBER versus BER, Fig. 6, shows properties similar to that of P(ES) versus BER, i.e. BBER varies linearly (on a log-log scale) with BER. This is true for BER < BERt2, therefore BBER can be written as:

BBER  =  k2 * BER   for BER  <  BERt2

where k2 is the slope of the line and it depends on the data rate.

FIGURE 4/S.1062...[D05] = 12CM

FIGURE 5/S.1062...[D06] = 12 CM

FIGURE 6/S.1062...[D07] = 9,5 CM

Values of BERt1, BERt2, k1 and k2 are tabulated below.

TABLE  5

BERt1, BERt2, k1 and k2 at 6 Mbit/s

| Data rate (Mbit/s) | 6.312 |
| --- | --- |

| - BERt1 | 1.5 × 10–6 |
| --- | --- |

| - BERt2 | 1.0 × 10–3 |
| --- | --- |

| - k1 | 6.7 × 105 |
| --- | --- |

| - k2 | 2.85 × 102 |
| --- | --- |

To develop a mask, it can be shown that:

BER = a * tb

where:

t is the percentage of time and a and b are given as:

b = log (BER1/BER2)/log (t1/t2)

a = BER1/(t1)b

with:

(t1, BER1) and (t2, BER2) being the coordinates of the two ends of the straight line segment on the graph of log t versus log BER.

From this we can easily derive expressions to determine the contributions of the line segment between times t1 and t2 to ESR and BBER:

µ §

similarly,

µ §

As far as the contribution of the line segments to SES is concerned, we know that SES is only significant when BER is very close to BERt2. A straight line approximation in the vicinity of BERt2 will be used to determine the percentage of time for which the system is experiencing SES. The following example illustrates the development of a typical mask.

### 2.2.3	Example mask

The following is an example of the development of a typical mask for 6.312 Mbit/s, with related BERt1, BERt2, k1 and k2 values used from Table 5.

The following three break-points (BP), along with their BER values are used:

BP1 : t1 = 0.2%		BER1 = 1 × 10–5

BP2 : t2 = 2%			BER2 = 1 × 10–7

*BP3 : t3 = 10%			BER3 = 1 × 10–8

The BER values above are selected with the following points in mind:

a)	BER is selected such that BER1 < BERt2, this will provide some available time beyond 0.2% of the time;

b)	BER2 is selected such that it is much less than BER1, and BER3 is taken to be one tenth of BER2.

Once the break-points along with the BER values have been selected, tests have to be performed to see whether these values would satisfy the ESR, SESR and BBER objectives set by ITU-T Recommendation G.826. These tests are done in the following manner:

#### ESR test

Using the equation developed above and the related parameters, Table 6 can be constructed:

TABLE  6

Percentage ESR calculation results

| t1 (%) | t2 (%) | BER1 | BER2 | ESR (%) |
| --- | --- | --- | --- | --- |

| - 10 | - 100 | 1 × 10–8 | 1 × 10–8 | 0.603 |
| --- | --- | --- | --- | --- |

| - 0 2 | - 010 | 1 × 10–7 | 1 × 10–8 | 0.156 |
| --- | --- | --- | --- | --- |

| 0 0.2 (see Note 1) | - 002 | 1 × 10–5 | 1 × 10–7 | 0.701 |
| --- | --- | --- | --- | --- |

| - 0 0.02 | - 000.2 | 1 × 10–3 | 1 × 10–5 | 0.180 |
| --- | --- | --- | --- | --- |

Now summing up the contributions of the mask segments to % ESR, we get the total % ESR to be 1.64% which is less than the prescribed objective of 1.75%. Therefore, this mask does meet the ESR objective.

Note 1 – Since the BER value at 0.2% of the time is greater than BERt1, and knowing that P(ES) = 1, at BER > BERt1, then we must find the percentage time tx on the mask which corresponds to a BER value equal to BERt1. Using simple mathematics it can be shown that tx is given by the following expression:

µ §

Using this expression it is found that tx = 0.52, so the contribution of the line segment between time percentages 0.52 and 2 to the % ESR is calculated in the usual manner, whereas the contribution of the segment between 0.52% and the unavailable time (arbitrarily chosen to be 0.02%) is simply the difference between tx and the unavailable time, i.e., 0.52 – 0.02 = 0.50 in this case.

#### BBER test

Using the expression for calculating BBER, developed earlier, and the appropriate parameters, the contribution of the mask to BBER can be calculated. The results of these calculations are given in Table 7.

TABLE  7

Percentage BBER calculation results

| t1 (%) | t2 (%) | BER1 | BER2 | BBER (%) |
| --- | --- | --- | --- | --- |

| - 10.00 | - 100 | 1 × 10–8 | 1 × 10–8 | 2.565 × 10–4 |
| --- | --- | --- | --- | --- |

| - 2.00 | - 010 | 1 × 10–7 | 1 × 10–8 | 6.617 × 10–5 |
| --- | --- | --- | --- | --- |

| - 0.20 | - 002 | 1 × 10–5 | 1 × 10–5 | 5.130 × 10–4 |
| --- | --- | --- | --- | --- |

| - 0.02 | - 000.2 | 1 × 10–3 | 1 × 10–5 | 5.130 × 10–3 |
| --- | --- | --- | --- | --- |

Summing up the contributions of the different segments to BBER, we obtain the total contribution of the mask which is 5.966 × 10–3. This number is less than the prescribed objective of 7.0 × 10–3, hence the BBER objective is also met.

#### SESR test

The contribution of the mask to SESR is only significant when BER is close to BERt2. From Fig. 5 it can be seen that for BER values less than 9.0 × 10–4, P(SES) rapidly becomes insignificant, hence it is only needed to determine the contribution of the portion of time for which BERt2 > BER > 9 × 10–4. Following a similar method as specified in Note 1 above, the contribution of the mask to SESR is found to be 1.2 × 10–2%, which is well within the required objective of 7.0 × 10–2%. This means that the mask meets the SESR objective as well.

This concludes the testing of the mask and verifies the fact that the mask defined above will meet the performance objectives set by ITU-T Recommendation G.826 at 6.312 Mbit/s.

It is worth mentioning that had the mask failed to pass any of the above three tests, then the mask would have to be redefined and tested.

# 3.	Relationship between bit-error ratio and error-event ratio

It is well known that errors on satellite links employing FEC and scrambler schemes tend to occur in clusters. The appearance of the clusters, which can also be called error events, is random following a Poisson distribution. The resulting block error rate is the same as if it were caused by randomly (Poisson distributed) occurring bit errors with a bit-error ratio BER/a, where a (used in § 2.1.1.1 to account for the burstiness of errors) is the average number of errored bits within a cluster, a also represents the ratio between the bit-error ratio and error-event ratio.

Statistical properties of the clusters of errors are dependent on the FEC/scrambler scheme used. Computer simulations and measurements of various FEC schemes (without scrambler or differential encoding) were used to determine the factor a. These results are given in Table 8.

TABLE  8

Factor for various FEC schemes

|  | Without FEC | With FEC |
| --- | --- | --- |

| Bit rate (Mbit/s) |  | 1/2 | 3/4 | 7/8 |
| --- | --- | --- | --- | --- |

| - 1.544 | 1.0 | 2.7 | 5.1 | 6.6 |
| --- | --- | --- | --- | --- |

| - 2.048 | 1.0 | 3.4 | 6.8 | 8.2 |
| --- | --- | --- | --- | --- |

| - 6.312 | 1.0 | 2.6 | 5.1 | 7.0 |
| --- | --- | --- | --- | --- |

| - 51.840 | 1.0 | 2.8 | 5.4 | 7.2 |
| --- | --- | --- | --- | --- |

| - 155.520 | 1.1 | 2.8 | 4.9 | 7.2 |
| --- | --- | --- | --- | --- |

Laboratory measurements of the INTELSAT IDR type digital transmissions (FEC R 3/4 plus scrambler) led to an a = 10 over the range of BER 10–4 to 10–11. An a = 5 was determined in the same measurements for the INTELSAT IBS type digital transmissions (FEC R = 1/2 plus scrambler).

From Table 8 and the results of the measurements it appears that a could be in a range of 1 to 10 for the cases investigated. Further studies of other types of FEC/scrambler schemes are required. The impact of parameter a on the performance model could be assessed as follows.

The masks in Figs. 1 and 2 were generated using a = 10. If, for example, no FEC/scrambler (a = 1) were used, the models would be shifted by one decade and the BER requirements would be more stringent (by one decade).

# 4.	Conclusions

The results of studies have shown that the masks required for meeting the ITU-T Recommendation G.826 are transmission rate dependent. The masks are also dependent on the error distribution which in turn are influenced by the FEC/scrambler scheme applied.

Service requirements also need to be taken into account in deriving the allowable error performance masks.

_________________