RECOMMENDATION  ITU-R  S.1149

NETWORK  ARCHITECTURE  AND  EQUIPMENT  FUNCTIONAL  ASPECTS
OF  DIGITAL  SATELLITE  SYSTEMS  IN  THE  FIXED-SATELLITE
SERVICE  FORMING  PART  OF  SYNCHRONOUS  DIGITAL
HIERARCHY  TRANSPORT  NETWORKS

(Question ITU-R 201/4)

(1995)

Rec. ITU-R S.1149

The lTU Radiocommunication Assembly,

considering

a)	that ITU-T Recommendations G.707, 708 and 709 specify bit rates, multiplexing structures and detailed mappings of tributaries in a new digital multiplex technique, known as the synchronous digital hierarchy (SDH);

b)	that digital satellite systems will remain as constituent elements of public/private networks in which SDH-based transmission technologies and SDH transport networks are being introduced;

c)	the on-going ITU-T studies on a technology-independent functional architecture of transport networks;

d)	that ITU-T Recommendation G.803 defines the architecture of SDH transport networks;

e)	that ITU-T Recommendation G.780 defines the terminology of SDH and that ITU-T Recommendations G.781, 782 and 783 specify the general characteristics and functions of synchronous multiplexing equipment;

f)	that ITU-T Recommendations G.703 and G.957 respectively define the physical parameters of the electrical and optical interfaces of SDH equipment;

g)	that ITU-T Recommendations G.831, G.784 and G.774 define the management capabilities of SDH transport networks, the management functions and protocols of SDH networks and the management information model from the network element viewpoint;

h)	that integration with the Telecommunications Management Network TMN, defined in ITU-T Recommendation M.30, is required;

j)	the on-going ITU-T studies on principles and guidelines for the integration of satellite and radio systems in SDH Transport Networks, including the concept of point-to-multipoint topologies, hereafter referred to as the “ITU-T Recommendation G.861 radio and satellite SDH integration guidelines”;

k)	that the multidestination and multipoint capabilities of satellite systems are in common use because of their operational advantages;

l)	that Recommendations ITU-R F.750 and ITU-R F.751 define architectures, functional aspects, and transmission characteristics and performance of radio-relay systems in SDH based networks;

m)	that ITU-T Recommendation G.832 defines the method of transporting SDH elements over PDH networks but that interconnection and interworking between Plesiochronous Digital Hierarchy PDH and SDH networks may be handled differently within a satellite system providing that the external interfaces and functionalities are compatible with terrestrial systems;

n)	the on-going ITU-R studies on transmission systems, its performance, multiple access methods and OAM aspects of digital satellite systems in the fixed-satellite service (FSS) forming part of SDH transport networks,

recommends

1	that digital satellite systems in the FSS comply with the network architecture and equipment functional requirements described in this Recommendation to permit integration into SDH transport networks.

CONTENTS

Page

µ

1	Introduction		10

1.1	Scope		10

1.2	Abbreviations		11

1.3	Definition of satellite specific terms		13

2	SDH transport network description		13

2.1	SDH multiplexing techniques		13

2.1.1	Basic structure		13

2.1.2	Sub-STM-1 multiplex signals for radio systems		14

2.2	SDH layered network modelling		15

3	Applications of FSS in SDH transport networks		16

3.1	Service aspects		16

3.2	Network management aspects		16

3.2.1	General		16

3.2.2	SDH equipment and management functional blocks		16

3.3	FSS system operational aspects		16

3.3.1	Multiplexing flexibility and efficiency		16

3.3.2	Timing		16

3.3.3	Satellite connection availability, automatic protection switching		19

4	FSS-SDH network scenarios, modelling and description		19

4.1	Digital sections (scenario 1)		19

4.1.1	Description		19

4.1.2	Layered network model		19

4.2	Wide-area single-rate cross-connect (scenario 2)		19

4.2.1	Description		19

4.2.2	Layered network model		20

4.3	Wide-area multi-rate cross-connect (scenario 3)		21

4.3.1	Description		21

4.3.2	Layered network model		22

5	FSS-SDH synchronous baseband equipment		23

5.1	SBE for SDH digital section (scenario 1)		23

5.1.1	Digital section at 155.52 Mbit/s (STM-1)		23

5.1.2	PDH embedded transmission of SDH signals		23

5.1.3	Satellite digital section at 51.84 Mbit/s (STM-0)		24

5.1.4	AU pointer processing and Doppler buffers		24

5.1.5	Bit interleaving		24

5.1.6	Alarm conditions and consequent actions		25

5.2	SBE for wide-area single-rate cross-connect (scenario 2)		26

5.2.1	SSOH functions including multidestination for S-IOs		26

5.2.2	Section layer frame format and multiplex structure		28

5.2.3	AU pointer processing and Doppler buffers		28

5.2.4	Bit interleaving		28

5.2.5	Alarm conditions and consequent actions		29

5.3	SBE for wide-area multi-rate cross-connect (scenario 3)		29

5.3.1	SSOH functions including multidestination for satellite intra-office sections		31

5.3.2	Satellite tributary unit groups 1/2 multiplex structures		33

5.3.3	Satellite section layer frame structures		35

5.3.4	SSOH allocation		35

5.3.5	Control channel mechanism		36

5.3.6	Section bit rates		38

5.3.7	Pointer processing and Doppler buffers		38

5.3.8	TU pointer byte interleaving		39

5.3.9	Bit interleaving		39

5.3.10	Alarm conditions and consequent actions		39

Annex 1 – SSOH DCC serial channel protocol 		41

# 1	Introduction

## 1.1	Scope

This Recommendation addresses network architecture and equipment functions relevant for the design of synchronous digital satellite systems in the fixed-satellite service (FSS) forming parts of, or providing synchronous interconnections between, transport networks based on the Synchronous Digital Hierarchy (SDH).

A general architectural diagram of FSS systems with SDH network transport capability is shown in Fig. 1. The basic requirement is transparent transport of SDH signal elements (virtual containers) through the satellite system.

This Recommendation focuses on SDH functionalities in the satellite system synchronous baseband equipment (SBE) necessary to realize three different scenarios for satellite system integration in SDH transport networks:

Scenario 1:	SDH digital section

Scenario 2:	wide-area single-rate cross-connect (common internal section bit rate of 51.84 Mbit/s)

Scenario 3:	wide-area multi-rate cross-connect (a range of internal section bit rates <51.84 Mbit/s)

This Recommendation follows the “ITU-T Recommendation G.861 radio and satellite SDH integration guidelines”, as well as other relevant G-series SDH Recommendations.

FIGURE 1...[D01] = 8 CM

NOTE 1 – Synchronous baseband equipment and satellite transmission equipment are illustrated as separate units for the purpose of explanation; they may be implemented physically as one unit.

NOTE 2 – Reference point A: open EI for scenario 1; NNI and NNRP for scenarios 2 and 3.

NOTE 3 – Reference point B: SRP and open SEI for scenarios 1 and 2, SRP and closed SEI for scenario 3 (see the “ITU-T Recommendation G.861 radio and satellite SDH integration guidelines”).

The basic functions of the SBE are one or more of the following, depending on the scenario:

–	perform SDH multiplex set of functions, adapted to satellite applications and dependent on scenario;

–	conversion and adaptation of section and path layers to satellite system internal interfaces or reference points;

–	establishment of section and possibly path layer asymmetrical multidestination connections, and support point-to-point symmetrical path layer cross-connected connections;

–	perform specific section functions for signal transmission over the satellite medium;

–	integration of SDH pointer and satellite system timing;

–	SBE SDH multiplex based management functions.

The basic functions of the satellite transmission equipment are:

–	modem functions;

–	multiple access system protocols, procedures and their management functions;

–	timing preservation.

## 1.2	Abbreviations

APS:	automatic protection switching

AU:	administrative unit

AUG:	administrative unit group

BIP:	bit interleaved parity

DCC:	data communication channel

DXC:	digital cross connect

EI:	equipment interface

FDMA:	frequency division multiple access

HOVC:	higher order virtual container

HPA:	higher order path adaptation

HPC:	higher order path connection

HPOM:	higher order path overhead monitor

HSPA:	higher order satellite path adaptation

HSPT:	higher order satellite path termination

HSSA:	higher order satellite section adaptation

HSUG:	higher order supervisory unequipped generator

HSUM:	higher order supervisory unequipped monitor

IOS:	intra-office section

ISI:	internal satellite interface

LOVC:	lower order virtual container

LPA:	lower order path adaptation

LPC:	lower order path connection

LPOM:	lower order path overhead monitor

LPT:	lower order path termination

LSSA:	lower order satellite section adaptation

LSUG:	lower order supervisory unequipped generator

LSUM:	lower order supervisory unequipped monitor

LT:	line termination

MCF:	message communications function

MDSS:	multi-destination satellite server

MSOH:	multiplex section overhead

MSA:	multiplex section adaptation

MST:	multiplex section termination

MSP:	multiplex section protection

NNI:	network node interface

NNRP:	network node reference point

OAM:	operation, administration and maintenance

OHA:	overhead access

PDH:	plesiochronous digital hierarchy

RDI:	remote defect indication

REI:	remote error indication

RSOH:	regenerator section overhead

RST:	regenerator section termination

SBE:	synchronous baseband equipment

SDH:	synchronous digital hierarchy

SEI:	satellite equipment interface

SETPI:	synchronous equipment timing physical interface

SETS:	synchronous equipment timing source

S-IOS:	satellite intra-office section

SOH:	section overhead

SPI:	synchronous physical interface

SRP:	satellite reference point

SRT:	satellite regenerator termination

SSOH:	satellite section overhead

SSPI:	synchronous satellite physical interface

SST:	satellite section termination

SSTM-n:	satellite synchronous transport module-n

STM-N:	synchronous transport module-N

STUG-ij:	satellite tributary unit group ij

TDMA:	time division multiple access

TMN:	telecommunication management network

TU:	tributary unit

TUG:	tributary unit group

VC:	virtual container

## 1.3	Definition of satellite specific terms

HSPA:	higher order satellite path adaptation – a higher order adaptation function within the satellite system synchronous baseband equipment.

HSPT:	higher order satellite path termination – a higher order path termination function within the satellite system synchronous baseband equipment.

HSSA:	higher order satellite section adaptation – a higher order section adaptation function within the satellite system synchronous baseband equipment.

ISI:	internal system interface – a system specific internal interface which is not subject to standardization.

LSP:	lower order satellite path – a lower order path through the satellite system.

LSSA:	lower order satellite section adaptation – a lower order section adaptation function within the satellite system synchronous baseband equipment.

MDSS:	multidestination satellite server layer – the layer in the model which represents the multidestination capability of the satellite transmission system.

SFCOH:	satellite frame complementary overhead – an overhead to the baseband composite signal to accommodate satellite transmission system’s OAM functions (e.g. modem alarms and VOW).

S-IOS:	satellite intra office section – an SDH section which is internal to the satellite system and which may have multipoint topology. It may cover a wide geographical area.

SLT:	satellite line terminal – performs normal RST, MST and MS APS functions on the satellite side of the NNI.

SRP:	satellite reference point – the point between the synchronous baseband equipment and the satellite transmission equipment.

SSOH:	satellite section overhead – the overhead used within the satellite section between the synchronous baseband equipment.

SSPI:	satellite synchronous physical interface – the physical interface between the synchronous baseband equipment and the satellite transmission equipment.

SST:	satellite section termination – the function where the section overhead may be manipulated.

SS-TDMA:	satellite switched time-division multiple access – a fixed TDMA system with on-board burst-to-burst cyclic inter-beam connection reconfigurations.

SSTM-n:	satellite synchronous transport module order n – similar to STM-n of terrestrial transport systems but consisting of STUG payload multiplex and satellite section specific overhead.

STUG-ij:	satellite tributary unit group ij – similar to a terrestrial tributary unit group TUG but represents a new multiplexing stage in between the G.708 TUG-2 level and TUG-3/VC-3 level.

# 2	SDH transport network description

## 2.1	SDH multiplexing techniques

### 2.1.1	Basic structure

The basic SDH multiplexing techniques (bit rates, frame formats and structures) are described in ITU-T Recommendations G.707, G.708 and G.709. The SDH multiplex hierarchy has a level 1 bit rate of 155.52 Mbit/s (STM-1) as well as higher levels currently defined as STM-4 (622 080 Mbit/s) and STM-16 (2.48832 Gbit/s) formed by byte interleaving hierarchically lower synchronous multiplex signals.

Figure 2 shows the basic 125 µs frame structure of the STM-1 signal in a 270 ´ 9 bytes matrix format.

FIGURE 2...[D02] = 8 CM

The smallest SDH synchronous tributary signal element defined in the STM-1 multiplex is VC-12 (virtual container-12) having a payload capacity of 2.240 Mbit/s and a 64 kbit/s capacity per frame for its 4-byte POH, which is spread over a 4-frame multiframe. End-to-end transparency at the VC level is available to the SDH network users. Lower order paths (LOVC) and higher order path (HOVC) are defined, which can be flexibly configured and set up, through remote management operation, on a network-edge-to-network-edge, or node-to-node basis. HOVCs are either VC-4s or VC-3s. The supported LOVCs are then VC-3, VC-2 and VC-12, or VC-2 and VC-12 respectively.

Network nodes in SDH are interconnected by multiplex sections, which in turn may consist of a number of regenerator sections. Internode management is facilitated by functions communicated within multiplex section overhead (MSOH) and regenerator section overhead (RSOH). The allocation of section overhead (SOH) capacity and byte allocation to various management functions and the functions and protocols are described in ITU-T Recommendations G.708 and G.784.

### 2.1.2	Sub-STM-1 multiplex signals for radio systems

The construction of lower than STM-1 SDH multiplex signal bit rates for satellite system transmission follows the interworking multiplexing tree of Fig. 3, derived from the “ITU-T Recommendation G.861 radio and satellite SDH integration guidelines”, with the addition of satellite tributary unit groups (STUGs). STUGs signal elements (multiplex groups) of different sizes are used as baseband signals of the satellite transmission equipment (FDMA or TDMA).

In all cases, the following principles shall be adhered to in constructing sub-STM-1 satellite multiplex signals:

–	byte orientation of the multiplexed signals;

–	transparency to the lowest order basic SDH signal elements (VCs, TUs) as a minimum;

–	provision of SDH section overhead functionalities, including OAM and network management across the satellite transmission paths and between the SDH baseband units.

FIGURE 3...[D03] = 22 CM

## 2.2	SDH layered network modelling

ITU-T Recommendation G.803 SDH Network Functional Description defines the concept of a layered model of the transport network. Its use by all SDH network participants (designers, operators, equipment vendor) is essential for compatibility in design, operation and management. ITU-T Recommendation G.803 defines two groups of network

layers in the transport network, path layer and section layer. The SDH basic frame structure in Fig. 2 corresponds to the organization of the network in logical layers:

a)	the path layer, consisting of:	–	the lower-order VC (LOVC) layer
–	the higher-order VC (HOVC) layer

b)	the section layer, consisting of:	–	the multiplex section (MS) layer
–	the regenerator section (RS) layer

The RS layer is media dependent and the MS layer may be media dependent. The HOVC and LOVC are media independent and are components to construct, within their respective layers, G.803 subnetworks with a wide variety of topologies.

The G.803 layered network modelling is used, with suitable modifications, in the description of the three FSS-SDH network scenarios in § 4.

# 3	Applications of FSS in SDH transport networks

## 3.1	Service aspects

FSS systems in any of the three scenarios make up a part of the SDH transport network, thus contributing to the provision of efficient transmission network services to the SDH network users.

## 3.2	Network management aspects

### 3.2.1	General

Incorporation of SDH multiplex equipment functions as part of the satellite system synchronous baseband equipment facilitate commonality, accessibility, uniformity and integration in the overall SDH network management, and thus of the enhanced management functionality and of management automation.

### 3.2.2	SDH equipment and management functional blocks

The description methodology of ITU-T Recommendations G.782, G.783 and G.784 “Functional Reference Model” is used. The satellite system SDH equipment is defined as a collection of functional blocks, logically partitioned to facilitate the description of functions, operation and management of the whole system. It does not impose nor imply any physical partitioning of implementations along the block boundaries. A generic functional block diagram of a satellite system SDH multiplex equipment including its management and timing functional blocks is given in Figs. 4a and 4b (derived from Fig. 2-1/G.783).

These figures include all functions required for the transport and management of user’s traffic from one or more external ingress interfaces to one or more external egress interfaces.

Decomposition of FSS-SDH synchronous baseband equipment into selected functional blocks for the three network scenarios is described in § 5.1, 5.2 and 5.3.

## 3.3	FSS system operational aspects

### 3.3.1	Multiplexing flexibility and efficiency

Use of the flexible SDH multiplexing techniques in the satellite system facilitates the realization of efficient multidestination operation.

### 3.3.2	Timing

Satellite motion buffers to remove Doppler timing variations are in practice combined with plesiochronous buffers. Table 1 gives the required satellite motion (Doppler) buffer sizes as a function of satellite orbit inclination. Complete removal of Doppler timing variations is only possible when these can be separated from clock drifts (the received satellite signals show the combined phase effects). This is possible when real-time satellite position information is available at the receive and possibly transmit ends (e.g., in SS-TDMA systems).

FIGURE 4a...[D04] = 24 CM

FIGURE 4b...[D05] = 16 CM

TABLE  1

Doppler buffer size as a function of satellite orbit inclination

| Inclination (degrees) | Maximum Doppler (relative frequency) | Minimum buffer size (ms) |
| --- | --- | --- |
| 0.1 0.5 1.0 1.5 2.0 2.5 3.0 | ± 1.8 ´ 10-8 ± 4.0 ´ 10-8 ± 6.7 ´ 10-8 ± 9.4 ´ 10-8 ± 1.2 ´ 10-7 ± 1.5 ´ 10-7 ± 1.6 ´ 10-7 | 1.2 2.2 3.6 5.2 6.6 8.2 9.6 |

The SDH AU and TU pointer processing guarantees payload data integrity during controlled timing slips (plesiochronous interfacing) between two digital networks with different primary clock references. The benefit of SDH payload data integrity during controlled slips is gained by integrating the SDH pointer processing with Doppler processing, as described in § 5.1, 5.2 and 5.3.

### 3.3.3	Satellite connection availability, automatic protection switching

Subject to economic trade-off, availability of satellite systems which are subject to propagation degradation and outages could be maintained above a certain design limit, using the SDH network self-healing feature within the satellite system, i.e., SDH automatic protection mechanisms at path or section layers or both. This is for further study.

# 4	FSS-SDH network scenarios, modelling and description

## 4.1	Digital sections (scenario 1)

### 4.1.1	Description

The SDH transport network perspective of this scenario is given in Fig. 1 of ITU-T Recommendation G.861.

As a point-to-point regenerator section operating at STM-1 (155.52 Mbit/s), access to the standard RSOH functions, such as BIP-8 error monitoring, DCCR and voice order wires can be gained at the system SBE. Transparency to MSOH K1/K2 bytes permits multimedia automatic network level protection of mixed-media multiplex sections. The MST and MSP functions are at the extreme (terrestrial) ends of the interoffice MSs. Reference point A (Fig. 1) is an open interface (G.957 optical and G.703 electrical) at STM-1 rate, with the SBE acting as an SDH (satellite) regenerator terminals (SRTs).

The “ITU-T Recommendation G.861 radio and satellite SDH integration guidelines” also allow radio and satellite synchronous digital sections operating at 51.84 Mbit/s, as well as within existing 140 Mbit/s PDH facilities through a G.732 type interworking multiplexer. The SDH signal conversion from STM-1 to reduced bit rate synchronous structures is a line terminal (LT) functionality. Multipoint/multidestination satellite operation is not part of this scenario.

### 4.1.2	Layered network model

The G.803 layered network model of the FSS-SDH digital section is given in Fig. 5. The model presentation is adapted to reflect transparency of the system to one or more layers of the interconnected terrestrial networks. The RS is shown terminated by the satellite transmission system, and the MS and all LOVC and HOVC signals are transported transparently.

## 4.2	Wide-area single-rate cross-connect (scenario 2)

### 4.2.1	Description

This scenario employs the wide-area visibility of the satellite to create a cross-connect function which operates internally at a single bit rate of 51.84 Mbit/s. The SDH transport network perspective of this scenario is given in Fig. 2 of ITU-T Recommendation G.861.

The cross-connect and other SDH equipment functions are replicated and distributed over various earth station locations SBEs, and system interfacing to the terrestrial SDH networks is through standard NNIs.

The SBE permits efficient byte-oriented (asymmetric) add/drop functions of payload traffic (VC-12, TUG-2) to/from SDH multiplex signals to support multidestination satellite operation. The internal cross-connect synchronous signal transport at STM-0 (51.84 Mbit/s) supports “medium” satellite routes, and implements both point-to-point and point-to-multipoint S-IOS. In multidestination operation, each SBE handles its SDH payload traffic within a single transmit and several receive unidirectional S-IOSs to/from multiple correspondents. The SBE is based on 51.84 Mbit/s synchronous multiplex equipment, modified to allow asymmetrical working across the satellite equipment interfaces (SEI at reference point B in Fig. 1). Across the terrestrial NNI (reference point A), the SBE as a SLT can perform normal RST, MST and MS APS functions. Transported client signals are VC-12, VC-2 and VC-3 point-to-point path layer connections. Standard path status monitoring, end-to-end tracing and BIP-2/8 error monitoring in the POH are carried transparently through the cross-connect.

FIGURE 5...[D06] = 13 CM

### 4.2.2	Layered network model

The G.803 layered network model of the wide-area cross-connect is given in Fig. 6. The model presentation is adapted to reflect transparency of the system to one or more layers of the interconnected terrestrial networks. A MDSS layer is introduced to aid the modelling of system internal multi-destination connections at STM-0. The MDSS layer extends strictly from the bottom satellite medium sublayer, to the (sub)-layer boundary with the client layer network.

MD trails in the MDSS layer provide link connections with simple point-to-point topology in the path layer networks. MD trails also lend themselves as servers of (future) complex point-to-multipoint connection topologies in the path layer networks.

SDH signal handling within the MDSS layer is characterized by:

–	SDH signal element groupings to one common satellite route size (STM-0);

–	asymmetries in number of directional satellite sections between communicating earth stations;

–	restricted visibility from the externally terminated SDH network server trails.

Satellite networks formed within the MDSS layer are restricted to the FSS-SDH system, but provides transparency to G.803 type subnetworks in the path layer networks.

The highest server network sublayer with externally terminated trails is the VC-4 HOVC sublayer. RS, MS and VC-4 trails are terminated. Within the MDSS layer, VC-3 payload signals (and possibly lower order VCs) are unbundled, regrouped, and re-multiplexed into satellite section frames for multidestinational IOS transmission at STM-0 (51.84 Mbit/s) rate.

FIGURE 6...[D07] = 13 CM

## 4.3	Wide-area multi-rate cross-connect (scenario 3)

### 4.3.1	Description

The SDH transport network perspective of this scenario is given in Fig. 2 of ITU-T Recommendation G.861.

This scenario has similar characteristics to scenario 2, i.e., wide-area replication and distribution of cross-connect and other SDH equipment functions over the systems’ SBEs. Reference points A (Fig. 1) are standard NNIs, across which the SBE as a SLT performs normal RST, MST and MS APS functions. Internally across reference point B, the SBE supports asymmetrical VC-12, TUG-2 add/drop connection functions and S-IOS connections.

The cross-connect internal synchronous transport is over both point-to-point and point-to-multipoint S-IOSs, operating at a range of sub-STM-0 rates (carriage capability 1, 2, 3, 6, 9, 12, 15, 18 ´ VC-12s) defined in § 2.1.2 for “thin-route” satellite traffic. In multidestination operation, each SBE handles its SDH payload traffic within a single transmit and several receive unidirectional S-IOSs to/from multiple correspondents.

The SBE is a new generation of synchronous multiplex equipment developed for satellite systems. The SEIs (at reference point B in Fig. 1) are internal interfaces not defined in ITU-T Recommendations. Section 5.3 defines the SEI multiplex format and structure, and reduced S-IOS OH functionalities and allocation.

Transported client signals are VC-12 point-to-point path layer connections. Standard path status monitoring, end-to-end tracing, BIP-2 error monitoring in the POH are carried transparently through the cross-connect.

### 4.3.2	Layered network model

The G.803 layered network model of the wide-area cross-connect is given in Fig. 7. The model presentation is adapted to reflect transparency of the system to one or more layers of the interconnected terrestrial networks and to accommodate the internal MDSS layer.

FIGURE 7...[D08] = 13 CM

SDH signal handling within the MDSS layer is characterized by:

–	SDH signal element groupings optimized to satellite traffic route sizes (STUG payloads);

–	asymmetries in number and sizes of directional satellite sections between communicating earth stations;

–	restricted visibility from the externally terminated SDH network server trails.

Satellite networks formed within the MDSS layer are restricted to the FSS-SDH system, but provides transparency to G.803 type subnetworks in the path layer networks.

In Fig. 7, the highest server network sublayer, at which external trails are terminated is the HOVC sublayer. RS, MS, VC-4 and VC-3 trails are terminated. Within the MDSS layer their LOVC payload signals are unbundled, regrouped, and re-multiplexed into satellite section frames for multidestinational IOS transmission at lower rates, defined in § 5.3.

# 5	FSS-SDH synchronous baseband equipment

## 5.1	SBE for SDH digital section (scenario 1)

The SBE generally consists of SDH physical interface termination, regenerator and multiplex section termination and HSSA. MS protection switching capability for the terrestrial links accessing the SBE is not a requirement in this scenario. HOVC and LOVC path tandem connection monitoring using the Z5 byte (VC-3, VC-4), and the Z6 byte (VC-2) is an optional capability.

### 5.1.1	Digital section at 155.52 Mbit/s (STM-1)

In terms of equipment functional blocks defined in ITU Recommendations G.782 and G.783, the minimum SBE functional configuration shall be as given in Fig. 8, which shows a select set of functional blocks of the generic block diagram (see Fig. 4a). As a regenerator section as part of an MS, the satellite system in principle does not terminate MS (MST is depicted as optional). RST function shall be used to locate and position AU pointer bytes in the frame to facilitate AU pointer processing in the HSSA for timing purposes (see § 5.1.3).

FIGURE 8...[D09] = 4 CM

A modem function operating at the STM-1 rate of 155.52 Mbit/s is required for the satellite transmission equipment. No elastic Doppler buffer function is required in the modem equipment (see § 5.1.4).

### 5.1.2	PDH embedded transmission of SDH signals

Satellite systems with PDH transmission capabilities at 34, 45 and 140 Mbit/s rates can be used to transport SDH signals embedded in PDH structures according to ITU-T Recommendation G.832. In terms of equipment functional blocks, the G.832 based SBE minimum configuration shall be as given in Fig. 9, which shows a select set of functional blocks of the generic block diagram (see Fig. 4a).

FIGURE 9...[D10] = 6 CM

Elastic (Doppler) buffer operation is preferably part of the HSSA (see § 5.1.4), and its inclusion in standard PDH satellite modem equipment is for further study.

### 5.1.3	Satellite digital section at 51.84 Mbit/s (STM-0)

Annex A to ITU-T Recommendation G.708 defines the frame structure of a 51.84 Mbit/s synchronous signal for use in radio and satellite systems. The SBE functional blocks for the STM-N/STM-0 converter (an LT functionality) are generally similar to the G.832 SBE shown in Fig. 9, including the HSSA functionality described in § 5.1.4.

A modem operating at the STM-0 information rate of 51.84 Mbit/s, plus possible satellite function complementary overhead (SFCOH), is required. Elastic Doppler buffer operation is part of the HSSA, and thus is required in the modem function (see § 5.1.4).

### 5.1.4	AU pointer processing and Doppler buffers

Data (SDH payload) loss due to plesiochronous clock differences between interconnected synchronous digital networks is avoided by the SDH justification mechanism controlled by the AU pointer processing. Motion buffers for the removal/reduction of phase variations due to satellite Doppler shall be integrated with the AU pointer regeneration in the SBE (receive side) as shown in Fig. 10 for the STM-1 case.

FIGURE 10...[D11] = 10 CM

Although the figure shows the pointer buffer (PB) and motion buffer (MB) as functionally distinct entities, they can be more efficiently implemented by a common physical element.

The basic algorithm of AU pointer processing is given in ITU-T Recommendation G.783.

SBEs based on ITU-T Recommendation G.832 may not be equipped with satellite Doppler compensation. In this case, the G.703 SEI shall include a dedicated interface to extend timing from the AU pointer processing in the SBE for Doppler buffer read-out in the satellite transmission equipment (modem). This is for further study.

### 5.1.5	Bit interleaving

Bit interleaving/de-interleaving on the multiplex signal shall be performed in the SST functional block to provide protection against satellite transmission error bursts.

For STM-1 structures, the bit interleaving operation shall not be performed on the first nine SOH bytes (A1, A2, C1 and two reserved bytes) so as not to affect frame synchronization. The algorithm and details of bit interleaving the remaining 2 421 bytes of the frame are for further study.

The SDH scrambling technique is for further study.

### 5.1.6	Alarm conditions and consequent actions

FIGURE 11...[D12] = 4 CM

The SBE shall detect loss of signal (LOS) or loss of frame (LOF) on the satellite section and indicate multiplex section alarm indication signal (MS AIS) to the downstream multiplexer of the terrestrial SDH network. In response to the MS AIS, the downstream multiplexer will send a multiplex section remote defect indication (MS RDI) signal in the backward direction to inform the upstream multiplexer of the far-end terrestrial SDH network that it has received a section AIS from the connecting satellite network.

The fault conditions and the appropriate consequent actions for scenario 1 are summarized in Table 2. LOS, LOF, AU-LOP, MS AIS and AU-AIS are defined in ITU-T Recommendation G.783.

TABLE  2

Fault conditions and consequent actions taken by the SBE in scenario 1

|  |  | Consequent actions taken by the SBE(1) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Interface | Failure condition | Signal generated toward local terrestrial network |  | Signal generated toward local SEMF | Signal generated toward remote SBE |
|  |  | MS AIS | AU-AIS |  | MS AIS |
| Local terrestrial network interface | LOS/LOF |  |  | Yes | Yes |
| Satellite network | LOS/LOF | Yes |  | Yes |  |
| interface | AU-LOP |  | Yes | Yes |  |
| (1) A “Yes” in a table entry signifies that the specified action shall be taken by the SBE as a consequence of the fault condition. A blank in a table entry signifies that the specified action shall not be taken by the SBE because either the fault condition is not visible to the SBE or no action is required from the SBE as a consequence of the fault condition. |  |  |  |  |  |

The time interval between the detection of LOS/LOF and the transmission of section AIS, the duration of section AIS, and the time interval between the termination of LOS/LOF and the removal of section AIS are for further study.

## 5.2	SBE for wide-area single-rate cross-connect (scenario 2)

The SBE generally consists of SDH physical interface termination, RST and MST, HSSA, higher and lower order path connection functions (HPC and LPC). MS protection switching capability shall be provided on the terrestrial side of the SBE. HOVC and LOVC path tandem connection monitoring using the Z5 byte (VC-3, VC-4), and the Z6 byte (VC-2) is an optional capability.

The SBE shall extract HOVCs (VC-3s) and provide cross-connection of VC-3 signals under the control of network management. The SBE shall also terminate the higher order path overhead, and shall extract the lower order VC-12s from the tributary unit groups for some signals. The SBE will support cross-connection of the VC-12 signals under the control of network management. An example SBE asymmetric multidestination configuration with different destination traffic sizes is shown in Fig. 12. Multidestinational SBEs may have multiple transmit ports, which is an implementation matter.

FIGURE 12...[D13] = 5 CM

Figure. 13 shows the SBE internal functional blocks. Multiplexing and demultiplexing of LOVCs (VC-12, VC-2) and possibly HOVC (VC-3) and asymmetry reconciliation are accomplished in the HSPAs. The satellite section physical interfaces at 51.84 Mbit/s is FFS. (An alternative is to use the SONET 51.84 Mbit/s physical and electrical interface characteristics contained in Bellcore standard TR-TSY-000253.)

The HSSA functions shall be:

–	generation of AU pointers to form AU-3 signals according to ITU-T Recommendation G.709;

–	adaptation of the outgoing signal to 51.84 Mbit/s synchronous signal for transport over the S-IOS;

–	recovery of VC-3 signals and associated frame offset information from the received AU pointers;

–	buffering AU-3 signals to remove/reduce Doppler effects without data loss.

### 5.2.1	SSOH functions including multidestination for S-IOs

The SSOH will support the following overhead functions for the S-IOS across the satellite subnetwork:

–	error monitoring;

–	remote error indication for multiple connections;

–	remote defect indication for multiple connections;

–	data communications channel (DCC) for multiple connections;

–	VOW channel for multiple connections and conference calls;

–	500 µs multiframe alignment.

FIGURE 13...[D14] = 23 CM

### 5.2.2	Section layer frame format and multiplex structure

The S-IOS frame structure of the STM-0 synchronous signal module is based on ITU-T Recommendation G.708 Annex A.

Figure 14 shows STM-0 SOH byte re-allocation to accommodate addressing information for backward indications (RDI and REI) at the S-IOS layer for multidestination satellite operation.

FIGURE 14...[D15] = 11 CM

Each status or S byte in the header is assigned to a particular correspondent via management interactions. This byte shall carry the far end receive failure indication as well as four bits for far end block error reporting. This will allow a station to report to its correspondent the number of interleaved parity bits that do not meet the parity check in the BIP-N block of each frame.

The DCC protocol and message multiplexing mechanism as well as the method of addressing the DCC to a specific destination are described in Annex 1 to this Recommendation.

### 5.2.3	AU pointer processing and Doppler buffers

The same requirements as in § 5.1.3 apply for each individual receive side HSSA.

### 5.2.4	Bit interleaving

Bit interleaving/de-interleaving on the multiplex signal shall be performed in the SST functional block to provide protection against satellite transmission error bursts.

The bit interleaving operation shall not be performed on the first three SOH bytes (A1, A2, S1) so as not to affect frame synchronization. The method and algorithm of bit interleaving the remaining 807 bytes of the frame are for further study.

The SDH scrambling technique is for further study.

### 5.2.5	Alarm conditions and consequent actions

FIGURE 15...[D16] = 8 CM

The SBE shall detect LOS/LOF on the satellite section, and shall indicate to the downstream multiplexer of the terrestrial SDH network either the MS AIS, AU-AIS, or TU-AIS depending upon the number of AU/TUs affected; and shall indicate to the upstream SBE the satellite section S-IOS RDI signal.

The SBE shall detect loss of pointer (LOP) on the satellite section, and shall indicate to the downstream multiplexer of the terrestrial SDH network either the AU-AIS, or TU-AIS, and shall indicate to the upstream SBE either the HO-path or LO-path RDI signal depending upon the number of AU/TUs affected.

In multi-destination operation, the MS RDI signal is received by more than one upstream SBE; therefore, addressing of the upstream SBE, for which the MS RDI signal is intended, is necessary to ensure proper operation.

The fault conditions and consequent actions for scenario 2 are summarized in Table 3.

LOS, LOF, LOP and MS AIS are defined in ITU-T Recommendation G.783.

MS RDI, HO-RDI, LO-RDI, AU-AIS and TU-AIS are defined in ITU-T Recommendation G.709.

The time interval between the detection of LOS/LOF/LOP and the transmission of AIS and RDI signal, the duration of AIS and RDI signal, and the time interval between the termination of LOS/LOF/LOP and the removal of AIS and RDI signal are for further study.

## 5.3	SBE for wide-area multi-rate cross-connect (scenario 3)

The SBE generally consists of SDH physical interface termination, RST and MST, HSSA, higher order path termination and lower order path connection functions. MS protection switching capability shall be provided on the terrestrial side of the SBE.

HOVC and LOVC path tandem connection monitoring using the Z5 byte (VC-3, VC-4), and the Z6 byte (VC-2) is an optional capability.

TABLE  3

Fault conditions and consequent actions taken by the SBE in scenario 2

|  |  | Consequent actions taken by the SBE(1) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interface | Failure condition | Signal generated toward local terrestrial network |  |  |  |  |  | Signal generated toward local SEMF | Signal generated toward remote SBE |  |  |  |  |  |
|  |  | MS AIS | MS RDI | AU-AIS | HO-RDI | TU-AIS | LO-RDI |  | S-IOS AIS | S-IOS RDI | AU-AIS | HO-RDI | TU-AIS | LO-RDI |
|  | LOS/LOF |  | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |
|  | MS AIS |  | Yes |  |  |  |  | Yes |  |  |  |  |  |  |
|  | MS RDI |  |  |  |  |  |  | Yes |  |  |  |  |  |  |
| Local terrestrial | AU-LOP |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |
| network interface | AU-AIS |  |  |  | Yes |  |  |  |  |  |  |  | Yes |  |
|  | HO-RDI |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | TU-LOP |  |  |  |  |  | Yes | Yes |  |  |  |  | Yes |  |
|  | TU-AIS |  |  |  |  |  |  |  |  |  |  |  | Yes |  |
|  | LO-RDI |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | LOS/LOF | Yes(2) |  | Yes(3) |  | Yes(4) |  | Yes |  | Yes(5) |  |  |  |  |
|  | S-IOS AIS |  |  |  |  |  |  | Yes |  | Yes(5) |  |  |  |  |
|  | S-IOS RDI |  |  |  |  |  |  | Yes |  |  |  |  |  |  |
| Satellite network | AU-LOP |  |  | Yes(3) |  | Yes(4) |  | Yes |  |  |  | Yes |  |  |
| interface | AU-AIS |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | HO-RDI |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | TU-LOP |  |  |  |  | Yes |  | Yes |  |  |  |  |  | Yes |
|  | TU-AIS |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | LO-RDI |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (1) A “Yes” in a table entry signifies that the specified action shall be taken by the SBE as a consequence of the fault condition. A blank in a table entry signifies that the specified action shall not be taken by the SBE because either the fault condition is not visible to the SBE or no action is required from the SBE as a consequence of the fault condition. (2) In multi-destination, applied when LOS/LOF occurs on all received signals. (3) In multi-destination, applied on relevant AU signals. (4) In multi-destination, applied on relevant TU signals. (5) Includes addressing of the upstream SBE for which the MS RDI is intended (in multi-destination operation). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The SBE shall terminate the higher order path overhead, and shall extract the lower order VC-12s from the tributary unit groups. The SBE will support cross-connection of the VC-12 signals under the control of network management. An example SBE asymmetric multidestination configuration with different destination traffic sizes is shown in Fig. 16. Multi-destinational SBEs may have multiple transmit ports, which is an implementation matter.

FIGURE 16...[D17] = 8 CM

Figure 17 shows the SBE functional blocks. Multiplexing and demultiplexing of LOVCs to/from STUGs of different sizes, and MD asymmetry reconciliation are accomplished by the LSSAs.

SSPISs are either system dependent or may appear as open interfaces, i.e. the SBE and satellite transmission equipment may be integrated. In such cases, the SST interfaces directly with the multiple access system through ISIs.

The LSSA functions shall be:

–	generate TU pointers to form TU-12 signals (identical to the TU pointer processing in the HPA function defined in ITU-T Recommendation G.783), and multiplex TUs into STUG;

–	adapt the outgoing tributary unit group (STUG) signal to the SSTM-1/2n synchronous signals for transport over the S-IOS across the satellite network;

–	recover VC-12 signals and their associated frame offset information by interpreting TU pointers from individual received STUG tributaries;

–	buffer received TU-12 signals to remove effects of satellite Doppler without data loss.

### 5.3.1	SSOH functions including multidestination for satellite intra-office sections

The SSOH will support the following overhead functions for the S-IOS across the satellite system:

–	error monitoring,

–	far end block error reporting for multiple connections,

–	far end receive failure reporting for multiple connections,

–	DCC for multiple connections,

–	source trail tracing,

–	voice order wire (VOW) for multiple connections and conference calls,

–	VC-12 payload type,

–	500 µs multiframe alignment.

FIGURE 17...[D18] = 17 CM

### 5.3.2	Satellite tributary unit groups 1/2 multiplex structures

The STUG-1n (n = 1,2) signals are made up of one and two TU-12 signals respectively. The STUG-11 is equivalent to a TU-12 and the STUG-12 is made up of two byte multiplexed TU-12 signals as shown in Fig. 18.

FIGURE 18...[D19] = 17 CM

The STUG-2n (n = 1-6) signals are made up of from one to six TUG-2 signals byte multiplexed to form the STUG-2n. Byte multiplexing of TU-12s to form TUG-2s and byte multiplexing of TUG-2s to form an STUG-2n is shown in Fig. 19.

FIGURE 19...[D20] = 18 CM

### 5.3.3	Satellite section layer frame structures

FIGURE 20 et 21...[D21] = 5 CM

### 5.3.4	SSOH allocation

FIGURE 22 ..[D22] = 10 CM

The SSOH contains 2 bytes transmitted every 125 ms. The allocation of bits within the SSOH is as follows:

Frame alignment word FAW (32 bits, 4 bits per frame)

The FAW synchronizes the receiving SST to the start of the SSTM frame. Particular 4 bits patterns of the FAW are used to indicate the start of a 1 ms multiframe (CC frame), used for the control channel and source trail trace, and the start of the 500 ms multiframe pointing to the first V1 byte of the STUG multiplex payload.

NOTE 1 – The exact method of synchronization and FAW patterns are for further study.

Source trail trace (8 bits, 1 bit per frame)

The source trail trace indicates to the receiving destination(s) the identity of the origination station. Thus the receiving station can verify its continued connection to the intended transmitter. This information element employs a 1 ms frame (8 ´ 125 ms) over which the origin trail trace bits are collected. Every 125 ms a single bit is transmitted creating an 8 bit address unique to each station within the network. The addresses are assigned as part of the network configuration process.

Control channel (24 bits, 3 bits per frame)

The control channel conveys alarm and service messages, and enables payload reconfiguration. The control channel mechanism is described in § 5.3.5.

Voice order wire (VOW) (1 bit per frame)

This bit is used for VOW communication providing a 8 kbit/s encoded voice channel. The VOW channel is addressed to one or more destinations to support conferencing facility. The call set-up and clearing including addressing are performed using the service messages supported by the control channel.

DCC (2 bits per frame)

These bits are used to form a 16 kbit/s DCC. The DCC is addressed to one or more destinations. The DCC protocol and message multiplexing mechanism as well as the method of addressing the DCC to a specific destination are described in Annex 1.

For future use (1 bit per frame)

1 bit of the SSOH is reserved for future use.

BIP-4 (4 bits per frame)

These bits are used to transmit a BIP-4 value computed over all bits of the previous SSTM frame.

### 5.3.5	Control channel mechanism

The control channel (CC) contains 3 bits transmitted every 125 ms. The control channel employs a 1 ms CC frame (8 ´ 125 ms) over which bits are collected. A complete CC message consists of 24 bits.

Each CC message specifies an action to be taken at the following 1 ms CC frame boundary. If no new action is to be taken at the next CC frame, a refresh mechanism shall refresh the last commands transmitted on a per destination basis.

NOTE 1 – The exact definition of the refresh mechanism is for further study.

The information content of one CC message collected over a period of 1 ms CC frame is organized as follows:

FIGURE 23 ..[D23] = 6 CM

Alarm destination number (bits 1-6)

This field conveys the destination number for which alarm information is transferred. Destinations from 1 to 63 can be addressed although currently no more than 18 destinations can be active (SSTM-26). The number 00000 specifies that all destinations are addressed (broadcast). The type of alarm is transmitted in the next field.

In each 1 ms control frame one alarm for one destination can be transmitted. If more than one alarm exists, the alarms shall be transmitted in the order of their detection or in ascending destination number.

Alarm message type (bits 7-8)

Up to four different alarm types can be specified. Currently the following messages are used:

00	REI start

01	REI stop

10	RDI start

11	RDI stop

Service destination number (bits 9-14)

This field conveys the destination number for which a service message is transferred. Destinations from 1 to 63 can be addressed although currently no more than 18 destinations can be active (SSTM-26). The number 00000 specifies that all destinations are addressed (broadcast). The type of service message is transmitted in the next field.

In each 1 ms control frame one service message for one destination can be transmitted. If more than one service message is required, the service messages shall be transmitted in ascending destination number.

Service message type (bits 15-16)

Up to four different service message types can be specified. Currently the following messages are used:

00	VOW call start

11	Stop

01	Reserved

10	Reserved

VC-12 payload number (bits 17-22)

This field conveys the VC-12 payload number within an SSTM-1n or SSTM-2k frame. Payload numbers from 1 to 63 can be specified although currently no more than 18 payloads can exist (SSTM-26). The number 00000 indicates that all payloads are specified. A VC-12 payload can be set to various types of transport thus enabling future applications such as compression, billing information, etc.

VC-12 payload type (bits 23-24)

Up to four different VC-12 payload types are specified.

00	Set VC-12 payload to equipped

01	Set VC-12 payload to un-equipped

10	Set VC-12 payload to ATM cells

11	Reserved

### 5.3.6	Section bit rates

TABLE  4

Sub-STM-1 synchronous signal, payload, SSOH and bit rates

| Module | Payload |  | SSOH | S-IOS |
| --- | --- | --- | --- | --- |
| designation | Composition | Rate (kbit/s) | Rate (kbit/s) | Rate (kbit/s) |
| SSTM-11 | 1 ´ TU-12 | - 2 304 | 128 | - 2 432 |
| - SSTM-12 | 2 ´ TU-12 | - 4 608 | 128 | - 4 736 |
| - SSTM-21 | 1 ´ TUG-2 | - 6 912 | 128 | - 7 040 |
| - SSTM-22 | 2 ´ TUG-2 | - 13 824 | 128 | - 13 952 |
| - SSTM-23 | 3 ´ TUG-2 | - 20 736 | 128 | - 20 864 |
| - SSTM-24 | 4 ´ TUG-2 | - 27 684 | 128 | - 27 812 |
| - SSTM-25 | 5 ´ TUG-2 | - 34 560 | 128 | - 34 688 |
| - SSTM-26 | 6 ´ TUG-2 | - 41 472 | 128 | - 41 600 |
| - NOTE 1 – The need for a higher maximum SSTM-2n is for further study. |  |  |  |  |

### 5.3.7	Pointer processing and Doppler buffers

Data (SDH payload) loss due to plesiochronous clock differences between interconnected synchronous digital networks is avoided by the SDH justification mechanism controlled by the TU pointer processing. Motion buffers for the removal/reduction of phase variations due to satellite Doppler shall be integrated with the SDH TU pointer processing in the SBE (receive side) as shown in Fig. 24.

FIGURE 24 ..[D24] = 10 CM

Although the figure shows the pointer buffer (PB) and motion buffer (MB) as functionally distinct entities, they can be more efficiently implemented by a common physical element.

The basic algorithm of TU pointer processing is given in ITU-T Recommendation G.783.

### 5.3.8	TU pointer byte interleaving

Additional protection against satellite transmission error bursts shall be provided by byte interleaving the TU pointer bytes in the LSSA. The interleaving algorithm is for further study.

### 5.3.9	Bit interleaving

Bit interleaving/de-interleaving on the multiplex signal shall be performed in the SST functional block to provide protection against satellite transmission error bursts.

The bit interleaving operation shall not be performed on the first SSOH bytes containing the FAW so as not to affect frame synchronization. The method and algorithm of bit interleaving of the remaining bytes of the frame are for further study.

The SDH scrambling technique is for further study.

### 5.3.10	Alarm conditions and consequent actions

FIGURE 25 ..[D25] = 8 CM

The SBE shall detect LOS/LOF on the satellite section, and shall indicate to the downstream multiplexer of the terrestrial SDH network either the MS AIS, AU-AIS, or TU-AIS depending upon the number of AU/TUs affected, and shall indicate to the upstream SBE the satellite section S-IOS RDI signal.

The SBE shall detect LOP on the satellite section, and shall indicate to the downstream multiplexer of the terrestrial SDH network the TU-AIS, and shall indicate to the upstream SBE the LO-RDI signal.

In multi-destination operation, the satellite section RDI signal is received by more than one upstream SBE; therefore, addressing of the upstream SBE for which the satellite section RDI signal is intended, is necessary to ensure proper operation.

The fault conditions and consequent actions for scenario 3 are summarized in Table 5. LOS, LOF, LOP and MS AIS are defined in ITU-T Recommendation G.783. MS RDI, HO-RDI, LO-RDI, AU-AIS, TU-AIS are defined in ITU-T Recommendation G.709.

The time interval between the detection of LOS/LOF/LOP and the transmission of AIS and RDI signal, the duration of AIS and RDI signals, and the time interval between the termination of LOS/LOF/LOP and the removal of AIS and RDI signal are for further study.

TABLE  5

Fault conditions and consequent actions taken by the SBE in scenario 3

|  |  | Consequent actions taken by the SBE(1) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interface | Failure condition | Signal generated towards local terrestrial network |  |  |  |  |  | Signal generated towards local SEMF | Signal generated towards remote SBE |  |  |  |
|  |  | MS AIS | MS RDI | AU-AIS | HO-RDI | TU-AIS | LO-RDI |  | S-IOS AIS | S-IOS RDI | TU-AIS | LO-RDI |
|  | LOS/LOF |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  | MS AIS |  | Yes |  |  |  |  | Yes |  |  |  |  |
|  | MS RDI |  |  |  |  |  |  | Yes |  |  |  |  |
| Local terrestrial | AU-LOP |  |  |  | Yes |  |  | Yes |  |  | Yes |  |
| network interface | AU-AIS |  |  |  | Yes |  |  |  |  |  | Yes |  |
|  | HO-RDI |  |  |  |  |  |  |  |  |  |  |  |
|  | TU-LOP |  |  |  |  |  | Yes | Yes |  |  | Yes |  |
|  | TU-AIS |  |  |  |  |  |  |  |  |  | Yes |  |
|  | LO-RDI |  |  |  |  |  |  |  |  |  |  |  |
|  | LOS/LOF | Yes(2) |  |  |  | Yes(3) |  | Yes |  | Yes(4) |  |  |
|  | S-IOS AIS |  |  |  |  |  |  | Yes |  | Yes(4) |  |  |
| Satellite network | S-IOS RDI |  |  |  |  |  |  | Yes |  |  |  |  |
| interface | TU-LOP |  |  |  |  | Yes(3) |  | Yes |  |  |  | Yes |
|  | TU-AIS |  |  |  |  |  |  |  |  |  |  |  |
|  | LO-RDI |  |  |  |  |  |  |  |  |  |  |  |
| (1) A “Yes” in a table entry signifies that the specified action shall be taken by the SBE as a consequence of the fault condition. A blank in a table entry signifies that the specified action shall not be taken by the SBE because either the fault condition is not visible to the SBE or no action is required from the SBE as a consequence of the fault condition. (2) In multi-destination, applied when LOS/LOF occurs on all received signals. (3) In multi-destination, applied on relevant TU signals. (4) Includes addressing of the upstream SBE for which the MS RDI is intended (in multi-destination operation). |  |  |  |  |  |  |  |  |  |  |  |  |

ANNEX  1

SSOH DCC serial channel protocol

# Introduction

An SDH transport system may pass through several network operator management domains as illustrated in Fig. 26 with the DCC carried management messages being terminated at each network management boundary. These management messages pass between manager systems and management agents. The management agents translate these messages into network element level measurements or actions. Thus the management view of a network is just a collection of management intelligences and their associated management information bases (MIBs) which contain the description of the objects under management, as shown in Fig. 27.

# 1	The serial channel protocol

To provide more flexible support for the management features of SDH transport networks, the DCC bytes are employed to form a serial communications channel to carry management messages.

Such a channel requires a protocol stack. A lower layer protocol to correct errors, provide flow control and support addressing and a higher layer protocol to support statistical multiplexing and identify message types.

Higher layer control functions such as the message queueing algorithm and the concatenation of longer messages strings are outside the scope of this Recommendation.

Given that the transmission media will be a geostationary satellite and that the transmission speed may increase, as more facilities are added, to several hundred kbit/s, then it is recommended that the service specific connection oriented protocol (SSCOP), is employed as the layer 2 protocol. SSCOP is defined in ITU-T Recommendation Q.2110 (previously Q.SAAL).

To be consistent with the way SSCOP is used in the new Q.2100-series of ITU-T Recommendations, a special simple version of the service specific coordination function (SSCF) is defined for this SDH application based on the contents of ITU-T Recommendation Q.2130 SSCF for the UNI of the B-ISDN.

The signals between the SSCF element and the SSCOP element as illustrated in Fig. 28 are as follows:

| Signal name | Function | Contents |
| --- | --- | --- |
| ESTABLISH | To establish assured information transfer | None |
| RELEASE | Termination of connection | None |
| DATA | Assured transfer of data | Data string for transfer |
| RESYNCH | Resynchronises one direction of transmission |  |
| ERROR | A response indicating that SSCOP needs help | Cause number |
| UNIT DATA | To transfer non-assured data | Data string for transfer e.g. J0 |
| RELEASE BUFF | A command to clear all buffers |  |

Unless otherwise indicated all these signals can be either requests or responses. Full details of signal structures, state tables and SDLs are given in ITU-T Recommendations Q.2110 and Q.2130. UNIT DATA transfers do not require initialization of the protocol via ESTABLISH/RELEASE states.

FIGURE 26 ..[D26] = PAGE PLAINE

FIGURE 27 ..[D27] = PAGE PLAINE

FIGURE 28 ..[D28] = 14 cm

### 1.1	Physical layer

SSCOP is designed to operate on top of various different physical layers. In this application the physical layer is a non-contiguous string of bytes which have no restrictions on their contents, and with byte synchronization provided by the SSOH structure. Onto this base has to be applied a method for forming and synchronizing a frame structure and a method for detecting errors. Thus it is recommended that the frame structure and the frame delimiting process, using unique flag bytes, as defined in ISO Standards 7776 and 8885 for the XID frame are adopted including the use of the optional 32 bit CRC for error detection. The XID format identifier shall be 84H which is allocated in ISO 4335 (see Fig. 29).

FIGURE 29 ..[D29] = 5 cm

SSCOP replaces the related protocol procedures defined in ISO 4335. For example, there is no need for a SABME/UA exchange to initialize a link layer. This is consistent with the normal usage of XID frames which may be exchanged “prior to data link establishment” and avoids duplication of the SSCOP establishment process.

The XID frame is composed of a number of fields which are defined in the following table.

| Field name | Value | Usage |
| --- | --- | --- |
| FLAG | 01111110 | At least one flag between frames |
| ADDRESS | 11100111 | Mandatory |
| CONTROL | 11110101 | Mandatory |
| FORMAT ID | 00100001 (84H) | Mandatory |
| INFORMATION | SSCOP information | At least 8 bytes of SSCOP address information |
| CRC_32 | Dependent on frame contents | Mandatory |

The order of bit transmission is least significant bit (LSB) first.

## 1.2	Special SSCF for SDH transport systems

### 1.2.1	The message primitives supported at the interface to the top of the SSCF are:

| Signal name | Function | Contents |
| --- | --- | --- |
| ESTABLISH | Establish connection to distant terminal | None |
| RELEASE | Release connection | None |
| DATA | Transfer data in assured mode | Data string |
| UNIT-DATA | Broadcast data in non-assured mode | Data string e.g. J0 |

### 1.2.2	SSCOP timers

| SSCOP timer | Duration (s) |
| --- | --- |
| TIMER_POLL | 0.7 |
| TIMER_NO-RESPONSE | 2 |
| TIMER_KEEP-ALIVE | 2 |
| TIMER_IDLE | - 10 |
| - TIMER_CC | 1.5 |

#### 1.2.3	SSCOP parameters

| SSCOP parameter | Description | Value |
| --- | --- | --- |
| Max SD/UD/MD-PDU length | Maximum number of bytes per PDU information field | 2 048 |
| Max UU field length | Maximum number of bytes in UU field | 1 024 |
| MaxCC | Maximum number of retransmissions of BGN, END, ER, or RS PDUs | 20 |
| MaxPD | Max of VT(CC) before sending a poll | 16 |
| MaxSTAT | Max number of list elements in a STATPDU before it is segmented | 67 (default) |
| Clear-buffers | Permission to clear buffers on connection release | Yes |
| Credit | Management message | Yes initially |

# 2	Multipoint operation

To extend the definition of the serial channel to include point-to-multipoint topology with each branch having its own assured data transfer service requires that SSCOP supports multiple state-machines, one for each leaf. This mode of operation is not yet defined for SSCOP but it only requires the addition of two address fields in the SSCOP protocol data units (PDUs), one for the source address and the other for the destination address. The interface definition is extended by simply including an address field in the ESTABLISH, RELEASE and DATA messages:

| Signal name | Function | Contents |
| --- | --- | --- |
| ESTABLISH | Initialize connection to given address list | Address list |
| RELEASE | Release connection to given address list | Address list |
| DATA | Transfer data in assured mode to given address list. Note this address list must be within the establishment address list | Address list + data |

The address length is 4 bytes and employs T.51 alphanumeric codes.

The SSCOP PDU is 32 bit aligned and trailer oriented, i.e., the protocol control information is at the end of the PDU.

The source address 32 bit field and destination address 32 bit field are the first two fields in that order in the PDU information field.

# 3	Message structure for the serial channel

## 3.1	Presentation standard

The serial channel will employ 8 bit bytes coded according to ITU-T Recommendation T.51.

The LSB is transmitted first.

## 3.2	Message structures

The message structure will be based on ITU-T Recommendation X.209 which defines a simple type, length, variable (TLV) style of message. ITU-T Recommendation X.209 also defines a content description byte which is not required in this application. Thus all messages will start with a two byte header; type, length, followed by the variable part of the message.

Examples of the type code tables for the current generation of SDH are shown in Table 6.

TABLE  6

SOH type codes (example)

| 00 | Reserved and protected from access by end users |
| --- | --- |
| 01 | Reserved and protected from access by end users |
| 02 | J0 byte content |
| 03 | Reserved and protected from access by end users |
| 04 | S1 byte bits 1-4 content |
| 05 to 0C | Reserved and protected from access by end users |
| 0D | D bytes content (this code may never be used as each individual kind of traffic which uses a serial channel should have its own TYPE code) |
| 0E to FF | Reserved and protected from access by end users |

A single byte “length byte” is defined as this limits the length of messages and avoids blocking of the serial channel by overly long messages. Multibyte length fields are used in other applications but are not allowed here. It is the responsibility of the transmitter to control the priority of the messages. There is nothing in the message protocol to indicate priorities.

The type byte is the first byte of the contents of the SSCOP PDU. In this multipoint version of SSCOP this will be the byte after the destination address.

The speed of the serial channels (the number of bytes involved) does not effect the message structures or the protocol.

_________________