RECOMMENDATION  ITU-R  S.1656

Outline of a software specification for automating the examination of satellite network filings for compliance with Article 5 of the Radio Regulations

(Question ITU-R 230/4)

(2003)

The ITU Radiocommunication Assembly,

considering

a)	that the problem of the backlog in the Radiocommunication Bureau’s (Bureau) processing of satellite network filings was first discussed at the Plenipotentiary Conference (Kyoto, 1994), resulting first in the adoption of Resolution 18 (Kyoto, 1994) and then in decisions at WRC-97 that provided a partial solution to this problem;

b)	that Council-98 provided additional resources to the Bureau to address this problem, but the unused funds remaining at the end of the 1998-1999 budget period could not be carried over to the 2000-2001 budget period, and this problem remains unresolved;

c)	that in January 2000 a correspondence group was established by the Director of the Bureau to identify approaches for addressing this problem which could be considered at WRC-2000, but that Conference adopted only a few of the approaches identified;

d)	that, at Council-2000, the Director established a Software Experts Group to identify approaches for improving the software used in processing satellite network filings, but the work of that group has not yet resolved any significant portion of this problem;

e)	that the backlog in the Bureau’s processing of satellite network filings continues to be a very significant factor affecting administrations’ and operators’ ability to comply with the time-frames specified in various provisions of the Radio Regulations (RR);

f)	that the backlog in the Bureau’s processing of satellite network filings is having an adverse impact on the Bureau’s ability to continue providing other services to administrations, including those services provided to developing countries;

g)	that the annual reports to the Council submitted by the Director of the Bureau described the continually increasing growth of the backlog in the Bureau’s processing of satellite network filings;

h)	that the reports to the Council submitted by the Director of the Bureau, which specifically respond to Resolution 74 (Minneapolis, 1998) describe possible approaches for resolving this problem;

j)	that the report from the Working Group on Reform concludes that the backlog in the Bureau’s processing of satellite network filings has been a critical problem for the Union, and that the elimination of this problem will require the systematic development and implementation of an action plan for addressing numerous contributing factors;

k)	that at the 13-14 November 2000 meeting of the Informal Software Expert Group, it was identified that in the processing of satellite network filings many manual functions and manual interfaces were required for the regulatory examinations and some of these manual tasks were done at the validation stage as well;

l)	that examination for compliance with the Table of Frequency Allocations is a manual process;

m)	that automation of these manual checks could reduce the time required by the Bureau in processing satellite network filings, and may assist administrations to check the compliance of their own satellite network filings with the provisions of RR Article 5 before submitting them to the Bureau,

recommends

1	that Annex 1 to this Recommendation, comprising a software requirements outline, should be used in the development of software to automate the examination of satellite network filings for compliance with the Table of Frequency Allocations (RR Article 5) and associated footnotes,

invites

1	the Bureau to assure that software developed on the basis of this Recommendation and offered for use by the Bureau accurately reflects the provisions of RR Article 5 and is efficiently integrated into the satellite network filing processing procedures,

2	the Bureau to identify what additional manual checks may still be necessary and which of those can be automated to assure that satellite network filings are completely checked for compliance with RR Article 5 and the Rules of Procedure.

NOTE 1 – For the purpose of commercial development of software, a more detailed specification, based on the outline in Annex 1, needs to be prepared.

NOTE 2 – The software to be developed on the basis of this Recommendation should be designed to ensure easy integration with existing Bureau software and software tools used by administrations.

Annex 1

General software requirements to examine the compliance of
satellite network filings with RR Article 5

# 1	Reasons for developing a software requirement

This requirements Recommendation outlines a procedure and the parameters used for the automated checking against the provisions of RR Article 5 of radio-frequency notices drawn up in accordance with RR Appendix 4, in the interests of enhancing the efficiency with which notices are examined by the Bureau and reducing the number of errors made by administrations when completing them. This involves the development of an RR Article 5 database and software for the automated examination of notices.

# 2	Anticipated users and significance of the checking software

It is anticipated that the software may be used by the Bureau in checking satellite network filings for compliance with the provisions of RR Article 5. In addition, such software may be used by administrations in checking their satellite network filings before submission to the Bureau. Furthermore, the RR Article 5 relational database could be used as a reference system for the rapid retrieval of information on frequency band allocations.

# 3	Input file

The input for the examination of a satellite network filing shall be in the electronic format defined by the Bureau for the electronic space notification described in the Circular Letter CR/58 and subsequent amendments published on the Space Radiocommunications Stations on CD-ROM (SRS-on-CD-ROM) (see also Resolution 55 (WRC-2000)). The input database may contain one or more networks distinguished by the notice identification number (“ntc_id” as currently defining the field name within a table).

The current implementation of the satellite network filing maintains, to some extent, a hierarchical structure, some information relates to the complete network, some to a specific beam and some to a particular group of frequency assignments.

In addition, data might have been provided in graphical format and stored in the Graphical Interference Management System (GIMS) database, outside the alphanumerical database. In most cases, the process of automated examination must begin with the identification of the service areas, since this is usually provided in the form of graphical data for each beam. If the service area table associated with a filing is not populated, then it may be possible to use the GIMS software tool to extract a list of territories within a given service area.

The data submitted are all listed in RR Appendix 4:

–	The date of bringing into use (A2).

–	The assigned frequency (or frequencies) and bandwidth (C2, C3, C7).

–	The class(es) of station (C4).

–	The direction of the signal (B2).

–	Whether GSO or non-GSO (indicator for B3 or B4).

–	The service area or areas (C11).

–	The power characteristics of the transmission (C8) might also be required for further examinations due to e.i.r.p. or pfd limitations given in footnotes.

–	(Additional data might be required for some services, for example for services that can be either active or passive services, deep space, etc. or for particular cases where the date of receipt by the Bureau might be of importance.)

NOTE 1 – When this Recommendation is revised, further parameters may be added.

# 4	RR Article 5 reference database

The software will use a database of RR Article 5 that represents the allocations and constraints contained in the RR. This database contains a number of interrelated tables describing the allocations in terms of radiocommunication services, frequency bands, regions (or subregions), date of entry-into-force and associated footnotes. The format of the database is shown in Appendix 1.

Also included in the database is a set of tables representing the footnotes to the RR Article 5 Table of Frequency Allocations, broken down into their individual components. These components include, for example, additional allocations, constraints and regional variations. The format of this Table of the database is shown in Appendix 2.

# 5	Examination for compliance with the Table of Frequency Allocations and footnotes

The conformity software will check a particular notice through a sequence of steps that will depend on the structure of the notice itself and the complexity of the allocation as given in the Table of Frequency Allocations of RR Article 5 and the applicable footnotes within this or these allocation(s). This sequence might require a number of iterations:

–	Frequency band identified under C2a: This will be checked against the frequency range given for each allocation in the Table of Frequency Allocations, until the proper range is identified. In some cases, (e.g. radio astronomy service) it might be necessary to use other related parameters, C2b and C3. If the allocation is worldwide (i.e. global), the identification of the radiocommunication service will take place.

–	Allocated service: The correlation between the classes of station identified under C4, for each group and each beam of the notice and the radiocommunication service(s) of the Table of Frequency Allocations will be checked using a reference table that provides this correlation.

–	Direction of transmission: The direction of the signal (identified by item B2) will have to correspond to the direction that might restrict the use of the allocation.

–	Regional allocation: If it is a regional allocation, it will be necessary first to identify the territories intended to be served by the space network, in other words, the service area identified under C11. Since a service area could be defined in various ways, it might require accessing the graphical data and in this case perform a surfaces overlap between the graphical service area and the close contour that defined the radiocommunication Region examined. A simple case is a list of territories defining the service area that will have to be inside the close contour of the Region. Footnotes usually identify a list of territories. The correlation between classes of station and services will take place again, followed by other service-related checks (direction, GSO, etc.).

Once the allowed radiocommunication service is identified, the protection status is identified: either the service is primary, secondary, or “undefined” (no harmful interference; no protection).

–	Date of bringing into use: This will be checked against the period of applicability of the Table of Frequency Allocations of RR Article 5. This date is actually defined for each group, regardless of its location in RR Appendix 4. The same date is checked when a footnote specifies dates (until, after) or a period of validity.

Of course, optimization will need to be implemented while doing the navigation within and between the two databases (RR Article 5 and satellite network filing(s)). If an allocation has a service footnote attached, it is necessary to perform checks against the footnote before the examination is completed.

An allocation to a particular radiocommunication service may extend over a number of adjacent sub-bands of the Table of Frequency Allocations. In such a case, the allocation should be treated as a contiguous allocation for this service.

An example of an algorithm to check compliance is shown in Appendix 3.

Note that the filing contains a “class of station” for each assignment, and this must be mapped to the appropriate radiocommunication service(s) for comparison with the entry in the Table of Frequency Allocations – these Tables are part of the RR Article 5 database, and are shown in Appendix 4.

The software will examine the input file for compliance with applicable footnotes for each assignment. These may be advisory footnotes, or other footnotes that do not require an examination to be carried out by the Bureau. In most cases, however, the footnotes modify an allocation shown in the Table, or add additional constraints to its use. A list of the main examinations to check compliance with the footnotes is shown in Appendix 5.

# 6	Output information

The software will generate a data table that contains the results of each examination. The table will contain the following fields for each frequency range in the assignment:

–	the network identity (ntc_id);

–	group identity (grp_id);

–	assigned frequency;

–	compliance with service check (positive/negative);

–	indication of allocation status (secondary/primary);

–	compliance with permitted direction of transmission (positive/negative);

–	compliance with permitted service area (positive/negative);

–	compliance with permitted date of application (positive/negative);

–	a list of applicable footnotes;

–	indication of any applicable footnote that could not be automatically examined.

An example of the format of the output report is shown in Appendix 6.

This data table may be used by other processes (such as other automated examinations), or may be displayed directly to the user. For clarity, however, a user report should contain only those entries from the notice for which inconsistencies with RR Article 5 (the Table or footnotes thereto) were detected, or for which it was not possible, in automatic mode, to determine conformity with RR Article 5.

Consideration should be given to providing user-interfaces in multiple languages.

Appendix 1
to Annex 1

Format of RR Article 5 reference database

| Frequency band |  |  |  | Region | Footnote | Radiocommunication service | Service footnote | WRC | Decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FREQ_L |  | FREQ_U |  |  | FootNoteID (GENERAL) | SERVICE | FootNoteID (SERVICE) |  |  |
| 1930 | - | 1970 | MHz | XR1 |  |  |  |  |  |
|  |  |  |  |  |  | FIXED |  | 2000 | MOD |
|  |  |  |  |  |  | MOBILE | 5.388A | 2000 | MOD |
|  |  |  |  |  | 5.388 |  |  | 2000 | MOD |
| 1930 | - | 1970 | MHz | XR2 |  |  |  |  |  |
|  |  |  |  |  |  | FIXED |  | 2000 | MOD |
|  |  |  |  |  |  | MOBILE | 5.388A | 2000 | MOD |
|  |  |  |  |  |  | Mobile-satellite (Earth-to-space) |  | 2000 | MOD |
|  |  |  |  |  | 5.388 |  |  | 2000 | MOD |
| 1930 | - | 1970 | MHz | XR3 |  |  |  |  |  |
|  |  |  |  |  |  | FIXED |  | 2000 | MOD |
|  |  |  |  |  |  | MOBILE | 5.388A | 2000 | MOD |
|  |  |  |  |  | 5.388 |  |  | 2000 | MOD |

Appendix 2
to Annex 1

Format of RR Article 5 footnote table

Appendix 3
to Annex 1

Example algorithm to check compliance with 
RR Table of Frequency Allocations

while (grp_id X ≤ grp_id MAX) do	! For each group within a network filing

while (seq_no Y ≤ seq_no MAX) do	! For each sequence within a group

while (stn_cls X,Z ≤ stn_cls X,MAX) do	! For each station class within a group

! Check frequency and service

if (freq_min Y ≥ FREQ_L and freq_max Y ≤ FREQ_U and stn_cls X,Z = SERVICE1) then

[freq Y : service = OK]	! If yes, then denote in output file

if (ALLOC_TYPE = “secondary”) then	! Check secondary status

[freq Y : secondary = OK]	! If yes, then denote

end-if

else

[freq Y : service = NOK]	! If allocation or service mismatch, then denote

end-if

! Check direction of transmission

if (emi_rcp X = “E” and (DXN = “s-E” or DXN = “s-s”)) or (emi_rcp X = “R” and (DXN = “E-s” or DXN = “s-s”)) then

[freq Y : direction = OK]	! If correct, then denote

else

[freq Y : direction = NOK]	! If incorrect, then denote

end-if

if (REGION = “MW”) or (ctry Y = REGION) then	! Check if global or regional allocation

[freq Y : service_area = OK]	! If service area correct, then denote

else

[freq Y : service_area = NOK]	! If service area incorrect, then denote

end-if

if (d_rcv Y ≥ EFF_FROM) then	! Check date of receipt against RR

[freq Y : date-of-application = OK]	! If allocation in force, then denote

else

[freq Y : date-of-application = NOK]	! If allocation not in force, then denote

end-if

end-while

end-while

Appendix 4
to Annex 1

Mapping of class of station with radiocommunication service

Class of station/AP5 Code

| Class of station code | Station indicator | Related classes of station code | Class of station description | AP5 | AP5 Rev code |
| --- | --- | --- | --- | --- | --- |
| EA | Space station | TA | Space station in the amateur-satellite service |  | ASS |
| EB | Space station | UB | Space station in the broadcasting-satellite service (sound broadcasting) | BSS | BSS |
| EC | Space station | TC | Space station in the fixed-satellite service | FSS | FSS |
| ED | Space station | TD,UD | Space telecommand space station in the space operation service |  | SOSS |
| EE | Space station | UE,UY | Space station in the standard frequency and time signal-satellite service |  | SFTSS |
| EF | Space station | TF,TL | Space station in the radiodetermination-satellite service | RDSS | RDSS |
| EG | Space station | TG,TI | Space station in the maritime mobile-satellite service | MMSS | MMSS |
| EH | Space station | TH,UH | Space station in the space research service |  | SRSS |
| EI | Space station | TE,VA | Space station in the mobile-satellite service | MSS | MSS |
| EJ | Space station | TB,TJ | Space station in the aeronautical mobile-satellite service | AMSS | AMSS |
| EK | Space station | TK,UK | Space tracking space station in the space operation service |  | SOSS |
| EM | Space station | TM,UM | Space station in the meteorological-satellite service |  | MTSS |
| EN | Space station | TN,UN | Space station in the radionavigation-satellite service |  | RDNSS |
| EO | Space station | TO,TZ | Space station in the aeronautical radionavigation-satellite service |  | ARDNSS |
| EQ | Space station | TQ,TX | Space station in the maritime radionavigation-satellite service |  | MRDNSS |
| ER | Space station | TR,UR | Space telemetering space station in the space operation service |  | SOSS |
| ES | Space station | -- | Space station in the inter-satellite service |  | ISS |
| ET | Space station | TT,UT | Space station in the space operation service |  | SOSS |
| EU | Space station | TU,TY | Space station in the land mobile-satellite service | LMSS | LMSS |

Class of station/AP5 Code (continued)

| Class of station code | Station indicator | Related classes of station code | Class of station description | AP5 | AP5 Rev code |
| --- | --- | --- | --- | --- | --- |
| EV | Space station | UV | Space station in the broadcasting-satellite service (television) | BSS | BSS |
| EW | Space station | TW,UW | Space station in the Earth exploration-satellite service |  | EESS |
| EY | Space station | UE,UY | Space station in the standard frequency and time signal-satellite service |  | SFTSS |
| RA | Earth station | -- | Radio astronomy station |  | RA |
| TA | Earth station | EA | Space operation earth station in the amateur-satellite service |  | ATSS |
| TB | Earth station | EJ | Aeronautical earth station, providing a feeder link for the aeronautical mobile-satellite service | AMSS | AMSS |
| TC | Earth station | EC | Earth station in the fixed-satellite service | FSS | FSS |
| TD | Earth station | ED | Space telecommand earth station in the space operation service |  | SOSS |
| TE | Earth station | EI | Mobile earth station, i.e. earth station in the mobile-satellite service the emissions of which are intended to facilitate search and rescue operations (satellite EPIRB) | MSS | MSS |
| TF | Earth station | EF | Fixed earth station in the radiodetermination-satellite service | RDSS | RDSS |
| TG | Earth station | EG | Ship earth station, i.e. mobile earth station in the maritime mobile-satellite service | MMSS | MMSS |
| TH | Earth station | EH | Earth station in the space research service |  | SRSS |
| TI | Earth station | EG | Coast earth station, providing a feeder link for the maritime mobile-satellite service | MMSS | MMSS |
| TJ | Earth station | EJ | Aircraft earth station, i.e. mobile earth station (aircraft) in the aeronautical mobile-satellite service | AMSS | AMSS |
| TK | Earth station | EK | Space tracking earth station in the space operation service |  | SOSS |
| TL | Earth station | EF | Mobile earth station in the radiodetermination-satellite service | RDSS | RDSS |
| TM | Earth station | EM | Earth station in the meteorological-satellite service |  | MTSS |
| TN | Earth station | EN | Fixed earth station in the radionavigation-satellite service |  | RNSS |
| TO | Earth station | EO | Mobile earth station in the aeronautical radionavigation-satellite service |  | ARNSS |

Class of station/AP5 Code (end)

| Class of station code | Station indicator | Related classes of station code | Class of station description | AP5 | AP5 Rev code |
| --- | --- | --- | --- | --- | --- |
| TQ | Earth station | EQ | Mobile earth station in the maritime radionavigation-satellite service |  | MRNSS |
| TR | Earth station | ER | Space telemetering earth station in the space operation service |  | SOSS |
| TT | Earth station | ET | Earth station in the space operation service |  | SOSS |
| TU | Earth station | EU | Land mobile earth station, i.e. mobile earth station in the land mobile-satellite service | LMSS | LMSS |
| TW | Earth station | EW | Earth station in the Earth exploration-satellite service |  | EESS |
| TX | Earth station | EQ | Fixed earth station in the maritime radionavigation-satellite service |  | MRNSS |
| TY | Earth station | EU | Base earth station, providing a feeder link for the land mobile-satellite service | LMSS | LMSS |
| TZ | Earth station | EO | Fixed earth station in the aeronautical radionavigation-satellite service |  | ARNSS |
| UA | Earth station | EI | Mobile earth station, i.e. earth station in the mobile-satellite service intended to be used while in motion or during halts at unspecified points | MSS | MSS |
| UB | Earth station | EB | Earth station in the broadcasting-satellite service (sound broadcasting) | BSS | BSS |
| UD | Earth station | ED | Space telecommand mobile earth station in the space operation service |  | SOSS |
| UE | Earth station | EE | Earth station in the standard frequency-satellite service |  | TMSS |
| UH | Earth station | EH | Mobile earth station in the space research service |  | SRSS |
| UK | Earth station | EK | Space tracking mobile earth station in the space operation service |  | SOSS |
| UM | Earth station | EM | Mobile earth station in the meteorological-satellite service |  | MTSS |
| UN | Earth station | EN | Mobile earth station in the radionavigation-satellite service |  | RNSS |
| UR | Earth station | ER | Space telemetering mobile earth station in the space operation service |  | SOSS |
| UT | Earth station | ET | Mobile earth station in the space operation service |  | SOSS |
| UV | Earth Station | EV | Earth station in the broadcasting-satellite service (television) | BSS | BSS |
| UW | Earth station | EW | Mobile earth station in the Earth exploration-satellite service |  | EESS |
| UY | Earth station | EY | Earth station in the time signal-satellite service |  | SFTSS |
| VA | Earth station | EI | Land earth station, providing a feeder link for the mobile-satellite service | MSS | MSS |

AP5 Code/Radiocommunication service

| Ap5 Rev code | Radiocommunication service | Direction | Allocation basis | Domain | Rec. ITU-R SM.1413 | Ap5 |
| --- | --- | --- | --- | --- | --- | --- |
| NA | (Not allocated) |  | Not allocated | Not allocated |  |  |
| AMTS | AERONAUTICAL MOBILE |  | Primary basis | Terrestrial services | MA |  |
| AMTS | Aeronautical Mobile |  | Secondary basis | Terrestrial services | MA |  |
| AOMTS | AERONAUTICAL MOBILE (OR) |  | Primary basis | Terrestrial services | MAO |  |
| ARMTS | AERONAUTICAL MOBILE (R) |  | Primary basis | Terrestrial services | MAR |  |
| ARMSS | AERONAUTICAL MOBILE-SATELLITE (R) (Earth-to-space) | (Earth-to-space) | Primary basis | Space services | SMAR |  |
| ARMSS | AERONAUTICAL MOBILE-SATELLITE (R) (space-to-Earth) | (space-to-Earth) | Primary basis | Space services | SMAR |  |
| ARDNTS | AERONAUTICAL RADIONAVIGATION |  | Primary basis | Terrestrial services | DNA |  |
| ARDNTS | Aeronautical Radionavigation |  | Secondary basis | Terrestrial services | DNA |  |
| ATS | AMATEUR |  | Primary basis | Terrestrial services | Z |  |
| ATS | Amateur |  | Secondary basis | Terrestrial services | Z |  |
| ASS | AMATEUR-SATELLITE |  | Primary basis | Space services | SZ |  |
| ASS | Amateur-Satellite |  | Secondary basis | Space services | SZ |  |
| ASS | Amateur-Satellite (space-to-Earth) | (space-to-Earth) | Secondary basis | Space services | SZ |  |
| BTS | BROADCASTING |  | Primary basis | Terrestrial services | B |  |
| BSS | BROADCASTING-SATELLITE |  | Primary basis | Space services | SB | BSS |
| EESS | EARTH EXPLORATION-SATELLITE |  | Primary basis | Space services | SX |  |
| EESS | Earth Exploration-Satellite |  | Secondary basis | Space services | SX |  |
| EEASS | Earth Exploration-Satellite (active) |  | Secondary basis | Space services | SX |  |
| EESS | EARTH EXPLORATION-SATELLITE (Earth-to-space) | (Earth-to-space) | Primary basis | Space services | SX |  |
| EESS | Earth Exploration-Satellite (Earth-to-space) | (Earth-to-space) | Secondary basis | Space services | SX |  |
| EESS | EARTH EXPLORATION-SATELLITE (Earth-to-space) (space-to-space) | (Earth-to-space) (space-to-space) | Primary basis | Space services | SX |  |
| EEPSS | EARTH EXPLORATION-SATELLITE (passive) |  | Primary basis | Space services | SX |  |
| EEPSS | Earth Exploration-Satellite (passive) |  | Secondary basis | Space services | SX |  |
| EESS | EARTH EXPLORATION-SATELLITE (space-to-Earth) | (space-to-Earth) | Primary basis | Space services | SX |  |

AP5 Code/Radiocommunication service (continued)

| Ap5 Rev code | Radiocommunication service | Direction | Allocation basis | Domain | Rec. ITU-R SM.1413 | Ap5 |
| --- | --- | --- | --- | --- | --- | --- |
| EESS | Earth Exploration-Satellite (space-to-Earth) | (space-to-Earth) | Secondary basis | Space services | SX |  |
| EESS | EARTH EXPLORATION-SATELLITE (space-to-Earth) (space-to-space) | (space-to-Earth) (space-to-space) | Primary basis | Space services | SX |  |
| FTS | FIXED |  | Primary basis | Terrestrial services | F |  |
| FTS | Fixed |  | Secondary basis | Terrestrial services | F |  |
| FSS | FIXED-SATELLITE (Earth-to-space) | (Earth-to-space) | Primary basis | Space services | SF | FSS |
| FSS | FIXED-SATELLITE (Earth-to-space) (space-to-Earth) |  | Primary basis | Space services | SF | FSS |
| FSS | FIXED-SATELLITE (space-to-Earth) | (space-to-Earth) | Primary basis | Space services | SF | FSS |
| FSS | FIXED-SATELLITE (space-to-Earth) (Earth-to-space) |  | Primary basis | Space services | SF | FSS |
| ISS | INTER-SATELLITE |  | Primary basis | Space services | SI |  |
| LMTS | LAND MOBILE |  | Primary basis | Terrestrial services | MT |  |
| LMTS | Land Mobile |  | Secondary basis | Terrestrial services | MT |  |
| LMSS | LAND MOBILE-SATELLITE (Earth-to-space) | (Earth-to-space) | Primary basis | Space services | SMT | LMSS |
| LMSS | Land Mobile-Satellite (Earth-to-space) | (Earth-to-space) | Secondary basis | Space services | SMT | LMSS |
| LMSS | LAND MOBILE-SATELLITE (space-to-Earth) | (space-to-Earth) | Primary basis | Space services | SMT | LMSS |
| LMSS | Land Mobile-Satellite (space-to-Earth) | (space-to-Earth) | Secondary basis | Space services | SMT | LMSS |
| MMTS | MARITIME MOBILE |  | Primary basis | Terrestrial services | MM |  |
| MMTS | Maritime Mobile |  | Secondary basis | Terrestrial services | MM |  |
| MMDTS | MARITIME MOBILE (distress and calling) |  | Primary basis | Terrestrial services | MM |  |
| MMSS | MARITIME MOBILE-SATELLITE (Earth-to-space) | (Earth-to-space) | Primary basis | Space services | SMM | MMSS |
| MMSS | MARITIME MOBILE-SATELLITE (space-to-Earth) | (space-to-Earth) | Primary basis | Space services | SMM | MMSS |
| MRDN | MARITIME RADIONAVIGATION |  | Primary basis | Terrestrial services | MM |  |
| MRDN | MARITIME RADIONAVIGATION (radio beacons) |  | Primary basis | Terrestrial services | MM |  |
| MRDN | Maritime Radionavigation (radio beacons) |  | Secondary basis | Terrestrial services | MM |  |
| MTTS | METEOROLOGICAL AIDS |  | Primary basis | Terrestrial services | W |  |
| MTTS | Meteorological Aids |  | Secondary basis | Terrestrial services | W |  |

AP5 Code/Radiocommunication service (continued)

| Ap5 Rev code | Radiocommunication service | Direction | Allocation basis | Domain | Rec. ITU-R SM.1413 | Ap5 |
| --- | --- | --- | --- | --- | --- | --- |
| MTSS | METEOROLOGICAL-SATELLITE (Earth-to-space) | (Earth-to-space) | Primary basis | Space services | SW |  |
| MTSS | Meteorological-Satellite (Earth-to-space) | (Earth-to-space) | Secondary basis | Space services | SW |  |
| MTSS | METEOROLOGICAL-SATELLITE (space-to-Earth) | (space-to-Earth) | Primary basis | Space services | SW |  |
| MTSS | Meteorological-Satellite (space-to-Earth) | (space-to-Earth) | Secondary basis | Space services | SW |  |
| MTS | MOBILE |  | Primary basis | Terrestrial services | M |  |
| MTS | Mobile |  | Secondary basis | Terrestrial services | M |  |
| MDTS | MOBILE (distress and calling) |  | Primary basis | Terrestrial services | M |  |
| XAMTS | MOBILE except aeronautical mobile |  | Primary basis | Terrestrial services | MX |  |
| XAMTS | Mobile except aeronautical mobile |  | Secondary basis | Terrestrial services | MX |  |
| XARMTS | MOBILE except aeronautical mobile (R) |  | Primary basis | Terrestrial services | MXR |  |
| XARMTS | Mobile except aeronautical mobile (R) |  | Secondary basis | Terrestrial services | MXR |  |
| MSS | MOBILE-SATELLITE |  | Primary basis | Space services | SM | MSS |
| MSS | MOBILE-SATELLITE (Earth-to-space) | (Earth-to-space) | Primary basis | Space services | SM | MSS |
| MSS | Mobile-Satellite (Earth-to-space) | (Earth-to-space) | Secondary basis | Space services | SM | MSS |
| MSS | MOBILE-SATELLITE (space-to-Earth) | (space-to-Earth) | Primary basis | Space services | SM | MSS |
| MSS | Mobile-Satellite (space-to-Earth) | (space-to-Earth) | Secondary basis | Space services | SM | MSS |
| XAMSS | Mobile-Satellite except Aeronautical mobile-satellite (Earth-to-space) | (Earth-to-space) | Secondary basis | Space services | SMX |  |
| RA | RADIO ASTRONOMY |  | Primary basis | Radio astronomy service | SA | RA |
| RA | Radio Astronomy |  | Secondary basis | Radio astronomy service | SA | RA |
| RDSS | RADIODETERMINATION-SATELLITE (Earth-to-space) | (Earth-to-space) | Primary basis | Space services | SD | RDSS |
| RDSS | Radiodetermination-Satellite (Earth-to-space) | (Earth-to-space) | Secondary basis | Space services | SD | RDSS |
| RDSS | RADIODETERMINATION-SATELLITE (space-to-Earth) | (space-to-Earth) | Primary basis | Space services | SD | RDSS |
| RDSS | Radiodetermination-Satellite (space-to-Earth) | (space-to-Earth) | Secondary basis | Space services | SD | RDSS |

AP5 Code/Radiocommunication service (continued)

| Ap5 Rev code | Radiocommunication service | Direction | Allocation basis | Domain | Rec. ITU-R SM.1413 | Ap5 |
| --- | --- | --- | --- | --- | --- | --- |
| RDLTS | RADIOLOCATION |  | Primary basis | Terrestrial services | DL |  |
| RDLTS | Radiolocation |  | Secondary basis | Terrestrial services | DL |  |
| RDLSS | RADIOLOCATION-SATELLITE (Earth-to-space) | (Earth-to-space) | Primary basis | Space services | SDL |  |
| RDNTS | RADIONAVIGATION |  | Primary basis | Terrestrial services | DN |  |
| RDNSS | RADIONAVIGATION-SATELLITE |  | Primary basis | Space services | SDN |  |
| RDNSS | Radionavigation-Satellite |  | Secondary basis | Space services | SDN |  |
| RDNSS | RADIONAVIGATION-SATELLITE (space-to-Earth) | (space-to-Earth) | Primary basis | Space services | SDN |  |
| SOSS | SPACE OPERATION (Earth-to-space) | (Earth-to-space) | Primary basis | Space services | SO |  |
| SOSS | SPACE OPERATION (Earth-to-space) (space-to-space) | (Earth-to-space) (space-to-space) | Primary basis | Space services | SO |  |
| SOISS | SPACE OPERATION (satellite identification) |  | Primary basis | Space services | SO |  |
| SOSS | SPACE OPERATION (space-to-Earth) | (space-to-Earth) | Primary basis | Space services | SO |  |
| SOSS | Space Operation (space-to-Earth) | (space-to-Earth) | Secondary basis | Space services | SO |  |
| SOSS | SPACE OPERATION (space-to-Earth) (space-to-space) | (space-to-Earth) (space-to-space) | Primary basis | Space services | SO |  |
| SRSS | SPACE RESEARCH |  | Primary basis | Space services | SR |  |
| SRSS | Space Research |  | Secondary basis | Space services | SR |  |
| SRASS | Space Research (active) |  | Secondary basis | Space services | SR |  |
| SRSS | Space Research (deep space) | (deep space) | Secondary basis | Space services | SR |  |
| SRSS | SPACE RESEARCH (deep space) (Earth-to-space) | (deep space) (Earth-to-space) | Primary basis | Space services | SR |  |
| SRSS | Space Research (deep space) (Earth-to-space) | (deep space) (Earth-to-space) | Secondary basis | Space services | SR |  |
| SRSS | SPACE RESEARCH (deep space) (space-to-Earth) | (deep space) (space-to-Earth) | Primary basis | Space services | SR |  |
| SRSS | Space Research (deep space) (space-to-Earth) | (deep space) (space-to-Earth) | Secondary basis | Space services | SR |  |
| SRSS | SPACE RESEARCH (Earth-to-space) | (Earth-to-space) | Primary basis | Space services | SR |  |

AP5 Code/Radiocommunication service (end)

| Ap5 Rev code | Radiocommunication service | Direction | Allocation basis | Domain | Rec. ITU-R SM.1413 | Ap5 |
| --- | --- | --- | --- | --- | --- | --- |
| SRSS | SPACE RESEARCH (Earth-to-space) (space-to-space) | (Earth-to-space) (space-to-space) | Primary basis | Space services | SR |  |
| SRSS | SPACE RESEARCH (passive) |  | Primary basis | Space services | SR |  |
| SRSS | Space Research (passive) |  | Secondary basis | Space services | SR |  |
| SRSS | SPACE RESEARCH (space-to-Earth) | (space-to-Earth) | Primary basis | Space services | SR |  |
| SRSS | Space Research (space-to-Earth) |  | Secondary basis | Space services | SR |  |
| SRSS | SPACE RESEARCH (space-to-Earth) (space-to-space) | (space-to-Earth) (space-to-space) | Primary basis | Space services | SR |  |
| SRSS | Space Research (space-to-space) | (space-to-space) | Secondary basis | Space services | SR |  |
| SFTTS | STANDARD FREQUENCY AND TIME SIGNAL |  | Primary basis | Terrestrial services | H |  |
| SFTTS | STANDARD FREQUENCY AND TIME SIGNAL (10 000 kHz) |  | Primary basis | Terrestrial services | H |  |
| SFTTS | STANDARD FREQUENCY AND TIME SIGNAL (15 000 kHz) |  | Primary basis | Terrestrial services | H |  |
| SFTTS | STANDARD FREQUENCY AND TIME SIGNAL (2 500 kHz) |  | Primary basis | Terrestrial services | H |  |
| SFTTS | STANDARD FREQUENCY AND TIME SIGNAL (20 000 kHz) |  | Primary basis | Terrestrial services | H |  |
| SFTTS | STANDARD FREQUENCY AND TIME SIGNAL (20 kHz) |  | Primary basis | Terrestrial services | H |  |
| SFTTS | STANDARD FREQUENCY AND TIME SIGNAL (25 000 kHz) |  | Primary basis | Terrestrial services | H |  |
| SFTTS | STANDARD FREQUENCY AND TIME SIGNAL (5 000 kHz) |  | Primary basis | Terrestrial services | H |  |
| SFTSS | Standard Frequency and Time Signal-Satellite (space-to-Earth) | (space-to-Earth) | Secondary basis | Space services | SH |  |
| SFTSS | STANDARD FREQUENCY AND TIME SIGNAL-SATELLITE (400.1 MHz) |  | Primary basis | Space services | SH |  |
| SFTSS | Standard Frequency and Time Signal-Satellite (Earth-to-space) | (Earth-to-space) | Secondary basis | Space services | SH |  |
| SFTSS | Standard Frequency and Time Signal-Satellite (space-to-Earth) | (space-to-Earth) | Secondary basis | Space services | SH |  |
| EEASS | EARTH EXPLORATION-SATELLITE (active) |  | Primary basis | Space services | SX |  |
| SRASS | SPACE RESEARCH (active) |  | Primary basis | Space services | SR |  |

Appendix 5
to Annex 1

A list of main examinations to check compliance with RR footnotes

| Footnote constraint type | Examination | Footnote sub-tables |
| --- | --- | --- |
| Allocation | Does assignment comply with corresponding footnote allocation? | Footnotes (sub-divided) Normalized frequency band Allocation Category |
| Service area | Does service area of beam comply with corresponding footnote service area restriction? | Footnotes (sub-divided) Normalized frequency band Geographical area |
| Coordination | (Advisory, denote in output file against assignment) | Footnotes (sub-divided) Normalized frequency band Reference RR Restriction |
| Protection | (Advisory, denote in output file against assignment) | Footnotes (sub-divided) Normalized frequency band Restriction |
| Class of station/service | Does the associated earth station comply with corresponding footnote restriction on service type? | Footnotes (sub-divided) Normalized frequency band Allocation Restriction |
| Date | Does the date of use of the assignment comply with corresponding footnote restriction on date? | Footnotes (sub-divided) Normalized frequency band Restriction |
| Power/pfd limit | (Flag limit and possibly link to pfd and power examination software) | Footnotes (sub-divided) Normalized frequency band Restriction |
| Antenna size | (Advisory, denote in output file against assignment) | Footnotes (sub-divided) Normalized frequency band Restriction |

Appendix 6
to Annex 1

Example of output data required

| Network | Group | Assigned frequency | Service | Category of service | Direction | Service area | Date of application | Additional allocation | Footnote | Additional footnote fields |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ntc_id 1 | grp_id 1 | freq 1 | OK |  | OK | OK | OK | OK | 5.XXX 5.XYX | … … |  |
| . | . | … | OK | OK | OK | OK | OK |  |  | … |  |
| . | . | … | OK | OK | NOK | OK | OK |  |  | … | (1) |
| . | . | … | OK | OK | OK | OK | OK | OK | 5.XXY | … |  |
| . | grp_id 1 | freq N | OK | OK | OK | NOK | OK |  |  | … |  |
| . | grp_id 2 | freq 1 | OK | OK | OK | OK | OK | OK | 5.XYX | … |  |
| . | … | … | OK | OK | OK | OK | OK |  | 5.XYZ | … |  |
| . | … | … | OK | OK | NOK | OK | OK |  |  | … |  |
| . | grp_id MAX | freq MAX | OK | OK | OK | OK | OK |  |  | … |  |
| (1) Example: Space allocation exists for this band (Service OK), allocation is compliant (CoS OK), direction of transmission is incorrect (Direction NOK), service area is compliant (Service area OK), and the date-of-receipt is compliant (Date of Application OK). |  |  |  |  |  |  |  |  |  |  |  |

1	Noting the definition of the space operations service, assignments filed for this service may be permitted in bands allocated to other services. Therefore the following checks may be involved:

If the space operations service assignment does not use a space operations allocation then either:

a)	check automatically that the space operations frequency is in a frequency band allocated to the main service in which the space station is operating (e.g. FSS); or

b)	flag that the allocation must be checked, and carry out the above check manually.