Recommendation ITU-R S.1857 (01/2010)

Methodologies to estimate the off-axis e.i.r.p. density levels and to assess the interference towards adjacent satellites resulting from pointing errors of vehicle-mounted earth stations in the 14 GHz frequency band

S Series Fixed-satellite service

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Annex 1 of Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from http://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Recommendations (Also available online at http://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radiowave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2010

 ITU 2010

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION ITU-R  S.1857

Methodologies to estimate the off-axis e.i.r.p. density levels and to assess the interference towards adjacent satellites resulting from pointing errors of vehicle-mounted earth stations in the 14 GHz frequency band

(Question ITU-R 208/4)

(2010)

Scope

This Recommendation presents the general antenna pointing error characteristics of vehicle-mounted earth stations with active antenna tracking systems and provides a method to estimate the statistics of off-axis e.i.r.p. variations due to pointing errors. Furthermore, it provides a methodology to assess the potential interference towards adjacent satellites operating in the GSO, FSS systems.

The ITU Radiocommunication Assembly,

considering

a)	that FSS GSO satellites are well suited to provide Internet and data services through a wide range of network configurations;

b)	that there is an increasing need to support user mobility and broadband services to end-users;

c)	that vehicle-mounted earth station (VMES) terminals can provide a wide range of communication services over FSS satellites in the 14 GHz frequency band;

d)	that it is necessary to protect networks of the FSS from any potential interference from these VMES terminals;

e)	that efficient use of the radio-frequency spectrum and the GSO by VMES terminals can be accomplished through use of a model for the off-axis e.i.r.p. density and interference from such terminals;

f)	that VMES require statistical approaches to determine their off-axis e.i.r.p. density levels and interference to adjacent satellites;

g)	that satellite networks using VMES can be designed to comply with the interference limits required by adjacent satellite system operators;

h)	that it would be useful to have methodologies for assessing the interference levels and impact on link availability of victim satellite networks resulting from variations in off-axis e.i.r.p. density levels of VMES antennas that are too small in diameter to be meaningfully assessed using currently available methods,

noting

a)	that maximum permissible levels of off-axis e.i.r.p. density from very small aperture terminals (VSATs) are provided in Recommendation ITU-R S.728;

b)	that maximum permissible levels of inter-network interference caused by the earth and space station emissions of all other satellite networks operating in the same frequency band are provided in Recommendation ITU-R S.1323,

recommends

1	that the methodology and associated model given in Annex 1 can be used to estimate the off-axis e.i.r.p. density levels caused by antenna pointing errors of VMES;

2	that the methodology given in Annex 2 can be used to assess the interference levels resulting from variations in off-axis e.i.r.p. density levels of VMES;

3	that the methodology given in Annex 2 can be used to assess the impact to the link unavailability of the interfered system in situations where time-varying antenna pointing errors from VMES antennas of the type described in Note 2 are significant;

4	that the Notes 1 to 5 should be regarded as part of this Recommendation:

NOTE 1 – The methodology given in Annex 2 may be used to assess potential interference impacts of VMES.

NOTE 2 − The methodologies presented in this Recommendation were developed for VMES with directional reflector antennas having equivalent diameters ranging from 0.3 m to 1.0 m; mechanical or electronic tracking systems, and support vehicle speeds up to 100 km/h. However, the methodologies can be applied to other antenna sizes and vehicle speeds.

NOTE 3 − The parameters and the examples provided in the annexes represent some systems that operate in the 14 GHz frequency band.

NOTE 4 − The methodology described in this Recommendation applies when the VMES tracking system is locked to its target satellite.

NOTE 5 – To use this Recommendation it is necessary to know the representative values of  and c, as used in § 2 of Annex 1.

Annex 1

A model to estimate off-axis e.i.r.p. density levels caused
by antenna pointing errors of VMES

# 1	Introduction

Recent demand for on-the-move communication applications has generated interest in a new type of satellite terminal. These terminals, which are mounted on vehicles, generally consist of small, high-performance antennas, tracking systems with servo controllers and positioners, and include the respective intermediate-frequency (IF) and RF equipment. The antenna size and other transmission parameters are selected to provide two-way communications under various terrains and operational conditions. The terminals considered in this annex will operate over FSS in the 14 GHz frequency band. Currently these terminals are being tested for use in terrestrial vehicles and trains.

The terminals mounted on vehicles, as detailed in this contribution, may cause additional interference to adjacent satellites due to motion-induced antenna pointing errors. From a satellite operator’s perspective, this interference should be maintained at a minimum level. On the other hand, service providers will seek to design their systems such that the terminals provide enough transmit power to support end-user applications at reasonable data rates. This annex addresses these conflicting demands, i.e. the need to transmit sufficient power to support reasonable data rates while maintaining an interference level that is acceptable to the satellite operators.

In on-the-move communication applications, because of the motion of the antenna platform, errors in the antenna pointing and tracking system can lead to antenna pointing errors. Typically, these motion-induced antenna pointing errors are small and random, and produce random variations of the off-axis e.i.r.p. density. In order to assess the impact of interference to other satellites it is necessary to model and quantify the e.i.r.p. density from these terminals.

This annex presents a statistical model to estimate the e.i.r.p. density levels due to antenna pointing errors and presents an approach to developing an illustrative statistical mask for the e.i.r.p. density in the off-axis directions. This illustrative statistical mask considers typical operational characteristics of terminals mounted on vehicles and can be used to limit the off-axis emissions from these terminals. For a satellite earth terminal the e.i.r.p. density in its off-axis directions is directly proportional to the e.i.r.p. density in the boresight direction. This annex provides a methodology to determine the appropriate levels for the boresight e.i.r.p. density so as to satisfy the above illustrative statistical mask.

# 2	Motion-induced antenna pointing errors

Under certain motion conditions of the antenna platform the boresight of the antenna will be displaced. The antenna pointing error can be represented by a random variable, , which is the angular distance between the actual and the intended directions of the antenna boresight. In many practical realizations, the antenna pointing error is measured in terms of its components: elevation error, , and azimuth error, a. These error components may be represented by mutually independent random variables whose statistical distributions are estimated by measurements carried out over representative drive paths. The probability density function (PDF) of x is denoted by fφx, where x = ,a. For illustrative purposes it is useful to represent these PDFs by well-known statistical distributions. Laboratory measurements of motion-induced antenna pointing errors have indicated that these pointing errors have long-tailed characteristics, that is, the PDF will not decay fast for large values of the antenna pointing error. The symmetric -stable (SS) distribution [Shao and Nikias, 1993; Samorodnitsky and Taqqu, 1994], is an example for a distribution with long-tailed characteristics and it is employed to represent, illustratively, the PDFs of the elevation and azimuth antenna pointing errors. The SS distribution has many parameters that can be used to generate different PDFs and the Gaussian distribution is a special case. The characteristic function of the SS distribution with zero location parameter is given as:

ψ(x) = e–cxα	(1)

where c > 0 is the scale parameter or the dispersion and , 0 <   2, is the characteristic exponent. The tail of the distribution is determined by , with smaller values giving rise to longer tails, and c is proportional to the width of the PDF. Note that when  = 2 the above gives the special case of the Gaussian distribution with mean zero and variance 2c2. Figure 1 shows the cumulative distribution function (CDF) of the SS distribution for c = 0.14 and 0.35º and for different values of . It is seen that by varying the values of these two parameters it is possible to represent many types of antenna pointing errors encountered in practice.

An intuitive justification for choosing the SS distribution to model the motion-induced antenna pointing errors can be given as follows. The observed antenna pointing errors depend on many parameters, for example, antenna characteristics; terrain conditions; antenna platform speed; elevation and azimuth angles; antenna tracking loop; location and position estimators and signal strength measurements. The errors contributed by all these different parameters may be assumed independent. In this case, the generalized central limit theorem [Samorodnitsky and Taqqu, 1994] can be applied to model the cumulative effect of these error sources by an SS distribution. Note that a similar central limit theorem argument has been used in long-range optical links to model the pointing and tracking errors with a Gaussian distribution [Chen and Gardner, 1989; Correl, 1996]. However, unlike the optical application, the operational environment considered in this annex includes rugged terrains; therefore, error components with longer-tailed distributions have to be considered.

Figure 1

The CDF of the absolute value of the antenna pointing error for different values of  and c

# 3	Statistical characterization of the off-axis e.i.r.p. density

In the preceding section the antenna pointing errors were considered to be random variables. These random pointing errors will cause the resulting e.i.r.p. density level to vary in a random manner. In this section the off-axis e.i.r.p. density level is considered to be a random variable and its CDF is determined.

In order to determine the off-axis e.i.r.p. density level, for illustrative purposes, consider the following normalized gain pattern for a parabolic dish antenna with circular aperture [Maral and Bousquet, 2000]:

G(ϕ) = 2n + 1(n + 1)!Jn + 1(πdλ × sin ϕ)(πdλ × sin ϕ)n + 12	(2)

where:

:	off-axis angle

Jn + 1:	Bessel function of the first kind and order (n + 1)

d:	diameter of the circular aperture

:	wavelength.

In the above, n is the aperture illumination parameter that corresponds to the following aperture illuminations:

n = 0, ideal uniform

n = 1, parabolic

n = 2, parabolic squared.

The main-lobe of many practical aperture systems falls between the normalized gain patterns corresponding to n = 1 and n = 2. Note that the side lobes of practical antennas may not be accurately represented by equation (2); however, this is not a shortcoming in the analysis since the focus of this work is on ultra small aperture terminals whose performance is limited by the main-lobe rather than the side-lobes.

Figure 2

Geometry representing an antenna pointing error, . The notation used is as follows:
earth terminal is at the origin, O, S is the intended satellite, OB is the 
antenna boresight direction,  is the off-axis angle at a point S , and 
θBSϕ is the angular distance between boresight direction and S direction

Consider the off-axis e.i.r.p. density level in the presence of an antenna pointing error, . As defined in § 2,  is the error in the boresight direction of the antenna. Figure 2 shows the geometry of the antenna boresight, OB, in the presence of an antenna pointing error. Here the earth terminal is at O, and S represents the location of the intended satellite so the axis of the antenna in the absence of pointing errors is OS. The off-axis angle is  and this direction is shown as OS. In the presence of antenna pointing errors, the angular distance between the antenna boresight and OS is denoted by BS. Observe that in the absence of antenna pointing errors BS = . Now the off-axis e.i.r.p. density level in the direction of OS can be expressed as:

Eφ(ϕ) = EB 2n + 1(n + 1)!Jn + 1(πdλ × sin θBSϕ)(πdλ × sin θBSϕ)n + 12	(3)

where EB is the e.i.r.p. density in the boresight direction. In order to determine the CDF of E() it is necessary to express this in terms of the underlying random variables,  and a. Observe that BS is a function of  and a, and using the geometry in Fig. 2 this can be expressed in terms of the elevation and azimuth angles in the directions OB and OS, as:

cos θBSϕ = cos εBSϕ– – ( cos εBSϕ+ + cos εBSϕ–) × sin2aBSϕ–2	(4)

where εBSϕ– = (εB –  εSϕ)and εBSϕ+ = (εB +εSϕ) are the difference and sum between the elevation angles in directions OB and OS, and aBSϕ– = (aB – aSϕ) is the difference between the azimuth angles in directions OB and OS, respectively. For given values of the locations of the earth terminal and points S and S , the following functions of the elevation and azimuth angles can be computed: εSSϕ– = (εS –  εSϕ), εSSϕ+=(εS+εSϕ), and aSSϕ– = (aS – aSϕ). Next, since in the absence of pointing errors the antenna boresight direction is along OS, εB = (εS –  φε) and aB = (aS – φa). Combining these expressions:

εBSϕ– = (εSSϕ– – φε)εBSϕ+ = (εSSϕ+ – φε)aBSϕ– = (aSSϕ– – φa)	(5)

For a given geometry of the earth terminal and the satellite points S and S, the quantities εSSϕ–, εSSϕ+, and aSSϕ– can be determined. Then, substituting equation (5) in equation (4) BS is expressed as a function of the errors in the elevation and azimuth angles and the predetermined elevation and azimuth angles to points S and S.

Using the above procedure and equation (3) the e.i.r.p. density level in the off-axis direction , E (), can be expressed in terms of the antenna pointing error random variables,  and a. Denote by fEφ the PDF of E (). Since, for illustrative purposes, the PDFs of  and a are represented by SS distributions fEφ can be determined using equations (3), (4) and (5). The desired PDF may be determined using either analytical techniques or Monte-Carlo simulations. The CDF of the e.i.r.p. density is PrEϕ (φ) < y = FEφ (y) = fEφ (x) dx. Note that this CDF is a function of the boresight e.i.r.p. density EB.

# 4	Computing the probability of exceeding a reference e.i.r.p. density level

In the previous section a procedure to determine the CDF of the off-axis e.i.r.p. density level was presented. Using this procedure the probability that the off-axis e.i.r.p. density level exceeds a certain reference threshold level can be determined. Denote this reference level by ERef (), which in general can be a function of . The probability that the e.i.r.p. density level is greater than this reference level in the off-axis direction , Pr{E() > ERef()}, is expressed as:

PrEφ (ϕ) > ERef (ϕ) = fEϕ (x) dx = 1 – FEφ (ERef (ϕ))	(6)

This is the complementary CDF (CCDF) of the off-axis e.i.r.p. density level computed at ERef () and is a function of the off-axis angle,  ; boresight e.i.r.p. density, EB; and the locations of the earth terminal and the satellite represented by the sum and difference of elevation and azimuth angles εSSϕ+, εSSϕ– and aSSϕ–. Intuitively, it is clear that by reducing EB the above probability can be reduced, and it is instructive to express this probability so that EB is an explicit parameter. To accomplish this equation (3) may be written as E () = EBG (BS), where G(BS) is the normalized antenna gain pattern such that G(0) = 1. The probability in equation (6) can be written as:

PrG (θBSϕ) > ERef (ϕ) / EB = 1 – FG(θBSϕ) (ERef (ϕ) / EB)	(7)

where FG(θBSϕ) is the CDF of G(BS) and is not a function of EB. The probability that the e.i.r.p. density level exceeds the reference level ERef () is as given above; however, this does not address the level of excess e.i.r.p. density above ERef (). This aspect can be addressed by examining the probability that the off-axis e.i.r.p. density level exceeds (EIRPexcess  ERef ()), where EIRPexcess  1 is a scale factor. Using this in equation (7) the required probability is:

Pr Eφ (ϕ)>(ERef(ϕ)  × EIRPexcess) = Pr G (θBSϕ)>ERef(ϕ)  × EIRPexcess / EB) = 1 – FG(θBSϕ) (ERef (ϕ)× EIRPexcess / EB)	(8)

The above probability is the CCDF of G(BS) computed at (ERef ()  EIRPexcess / EB).

The procedure for computing the probability in equation (8) is as follows:

Step 1: The underlying random variables here are the antenna pointing error components,  and a, whose PDFs, for illustrative purposes, are assumed to be known as in § 2.

Step 2: For known locations of the earth terminal, satellite and the off-axis direction, the sum and difference of elevation and azimuth angles, εSSϕ+ , εSSϕ− and aSSϕ−, are computed as described in § 3. These angles are then be used in equation (5) and the result substituted in equation (4) to express BS in terms of the random variables,  and a. The PDF of BS can then be determined using the PDFs of  and a. Making use of the relationship in equation (2), the PDF of BS so determined is then used to compute the CCDF of the random variable G(BS).

Step 3: Finally, the desired probability in equation (8) is determined from the CCDF of G(BS) with EB and e.i.r.p.excess as parameters.

# 5	An illustrative statistical e.i.r.p. density mask to limit off-axis emissions

In order to limit off-axis emissions, in the presence of motion-induced antenna pointing errors, an upper bound on the probability that the e.i.r.p. density level exceeds a reference level may be used. However, it is clear that the probability computed in equation (8) depends on the locations of the earth terminal and the satellite, and the off-axis angle. Since the earth terminal may be located anywhere on the Earth’s surface it is highly desirable to limit the off-axis emissions using a function that is independent of the earth terminal and satellite locations. Ideally, it is instructive to derive an upper bound for the probability PrE () > (ERef ()  EIRPexcess) by a single function, Pmax (EIRPexcess), which is applicable anywhere on the Earth’s surface and for all off-axis angles. This desired probability function, Pmax (EIRPexcess), limits the off-axis e.i.r.p. density emissions and constitutes a statistical mask on the e.i.r.p. density level.

To obtain a statistical e.i.r.p. density level mask as discussed above consider the special case when the points S and S are on the GSO and the earth terminal is placed on the equator and directly below S. For this configuration, S = 90º, εSϕ= (90∘ – ϕ)∘, aS= 90∘ and asφ=90∘ or 270∘, and it follows that εSS+ϕ= (180 – ϕ)∘, εSS–ϕ= ϕ and aSS–ϕ = 0∘ or 180∘. Using these expressions in equation (5) and substituting the result in equation (4), BS can be written as:

cos θBSϕ = cos(ϕ – φε) – (cos(ϕ – φε) – cos(ϕ – φε)) × sin2φa2	(9)

The CDF of G(BS) obtained using the above BS will not be a function of the specific elevation and azimuth angles from the earth terminal to the satellite; however, this CDF will be a function of the off-axis angle, . To derive a function that is applicable to all off-axis angles consider the maximum of the probability in equation (8) over all off-axis angles. From equation (8) this desired maximum probability is expressed as:

PEB = (EIRPexcess) = max ϕPr G (θBSϕ) > (ERef (ϕ) × EIRPexcess / EB)	(10)

The above function for the excess probability is not dependent on the specific locations of the earth terminal or the satellite, or the particular off-axis angle; therefore, this function is a suitable candidate for the illustrative off-axis e.i.r.p. density mask, Pmax (EIRPexcess).

To apply the above approach to limit the off-axis e.i.r.p. density emissions of a practical antenna system, the following should be specified as operational constraints: a reference off-axis e.i.r.p. density level, ERef () and the maximum probability that the e.i.r.p. density level may exceed the level (ERef ()  EIRPexcess) over all off-axis angles, Pmax (EIRPexcess), which is an illustrative statistical mask for the off-axis e.i.r.p. density level. For a particular antenna system the following values should be known: normalized antenna gain pattern, pointing error statistics and the locations of the earth terminal and the desired satellite. The objective is to set the boresight e.i.r.p. density, EB, of the antenna system so that the operational constraints on the off-axis e.i.r.p. density level are satisfied. This is accomplished by computing the CCDF of G(BS) as a function of EB as in equation (8), and then determining the appropriate value of EB so that this CCDF is upper bounded by the constraint Pmax (EIRPexcess) for all values of  and e.i.r.p.excess. An illustrative step-by-step method to utilize the probability function in equation (10) to limit the off-axis e.i.r.p. density level in a practical antenna system is given in § 7.

To derive a specific illustrative statistical mask for the e.i.r.p. density level consider the following reference level for the e.i.r.p. density:

ERef (ϕ) (dBW/40 kHz)= 25 – 25 log (ϕ) 2 ≤ϕ <7 4 7 ≤ ϕ <9.228 – 25 log (ϕ) 9.2 ≤ ϕ <48	(11)

This is the off-axis e.i.r.p. density mask as specified in Recommendation ITU-R S.728 combined with Note 1 in this Recommendation. Figure 3 shows the right-hand side of equation (10) (without maximization), PrG(BS ) > (ERef ()  EIRPexcess /EB) as a function of  for fixed values of e.i.r.p.excess and EB. Observe that when e.i.r.p.excess is varied the maximum of this probability will occur at different values of . Figure 4 shows this maximum value, PEB (EIRPexcess), for parameters  and c of the PDFs of the motion-induced antenna pointing error components,  and a, and the boresight e.i.r.p. density, EB. Here it is assumed that the above two random variables are identically distributed and mutually independent. For the antenna pattern given in equation (2) the following representative values were chosen for 14 GHz frequency band applications: d = 0.51 m, n = 1 and frequency = 14.2 GHz. Observe that, for larger values of  the PDF of the antenna pointing error will have a shorter tail and, therefore, PEB (EIRPexcess) will decay faster. Also, for smaller values of c the PDF of the antenna pointing errors will be narrower and this will result in a smaller probability for PEB (EIRPexcess). Clearly, the curves shown in Fig. 4 depend on EB: smaller values of EB will result in correspondingly smaller values forPEB (EIRPexcess). For the curves shown in this figure EB is set at its maximum value so that the resulting PEB (EIRPexcess) is just below its value corresponding to the parameters:  = 1.5, c = 0.35º and EB = 21.53 (dBW/40 kHz). In the next section, the details of determining the specific values for EB shown in this figure are discussed.

Figure 3

Right-hand side of equation (10) (excluding the maximization) as a function of the off-axis angle, ,
for different values of e.i.r.p.excess (dB). Other parameters:  = 1.5, c = 0.35º, 
EB = 21.53 dBW/40 kHz, n = 1 and frequency = 14.2 GHz

Figure 4

PEB (e.i.r.p.excess) given in equation (10) for different parameters of the PDFs of antenna 
pointing error components. Legend denotes , c (degrees) and EB (dBW/40 kHz)

Any of the curves shown in Fig. 4 are suitable candidates for the illustrative statistical mask for the off-axis e.i.r.p. density. In this annex the upper curve in this figure, which corresponds to the parameters  = 1.5, c = 0.35º and EB = 21.53 (dBW/40 kHz), is selected as an illustrative statistical mask for the e.i.r.p. density, Pmax (EIRPexcess). This curve is selected because some measurement results indicate that these parameter values ( and c) are representative of typical operating terrain conditions and vehicle speeds. The process of determining the value of EB is discussed in § 6. The above illustrative statistical off-axis e.i.r.p. density mask can be approximated by the following expression:

Pmax (x) = exp (0.016x2 – 0.561x – 1.297) 0 ≤ x ≤ 10	(12)

where x = e.i.r.p.excess (dB). In order for an antenna system to comply with this example mask, the probability for this antenna system as computed according to equation (10), PEB (e.i.r.p.excess), should be less than Pmax (EIRPexcess), that is:

maxEB PEB (EIRPexcess) ≤ Pmax (10 × log10 (EIRPexcess)) 1≤ EIRPexcess ≤10	(13)

where the maximum is over EB. As seen from the curves in Fig. 4, the example mask, Pmax (EIRPexcess (dB)), can be satisfied by antenna systems with various values for the parameters  and c with reasonable levels of EB.

# 6	Computing the boresight e.i.r.p. density

As seen from the derivations in the preceding section, the boresight e.i.r.p. density, EB, plays a key role in determining the performance of terminals mounted on vehicles. Observe that the probability function given in equation (10) imposes a limit on the boresight e.i.r.p. density. Also, increasing the boresight e.i.r.p. density increases interference to adjacent satellites, and Annex 2 presents a detailed analysis of interference from these terminals. The desired boresight e.i.r.p. density level must satisfy two competing demands: the need to transmit sufficient power to support reasonable data rates, and ensure that the resulting interference is acceptable to operators of adjacent satellites. The value of EB necessary to transmit at reasonable data rates in vehicle-mounted earth stations may be determined by comparing it with the corresponding value for static earth terminals. Figure 5 shows the e.i.r.p. density mask for static earth terminals as established in Recommendation ITU-R S.728 and expressed in equation (11). This figure also shows the maximum values of the off-axis e.i.r.p. density corresponding to an antenna of aperture diameter, d = 0.51 m. This off-axis e.i.r.p. density level for the antenna is obtained by gradually increasing EB until the resulting off-axis e.i.r.p. density pattern is just below the ITU-R S.728 mask. Clearly, the value of EB that satisfies the mask increases with the antenna aperture diameter and for d = 0.51 m, EB = 23 dB(W/40 kHz).

Figure 5

Maximum value of the boresight e.i.r.p. density subject to Recommendation 
ITU-R S.728. Parameters: d = 0.51 m, n = 1, frequency = 14.2 GHz

The results shown in Fig. 5 represent the case where there are no antenna pointing errors. Under antenna pointing errors, because of fluctuations of the off-axis e.i.r.p. density pattern, the boresight e.i.r.p. density has to be reduced. In the preceding section an illustrative statistical e.i.r.p. density mask for a particular antenna system was determined in equation (12). The desired value of EB that satisfies this statistical mask is determined by increasing its value until PEB(e.i.r.p.excess) just meets its maximum value as given on the right-hand side of equation (13).

Figure 6 shows the reduction of boresight e.i.r.p. density required to achieve the example mask defined in equation (12) so that antenna pointing errors can be accommodated. For fixed values of , larger values of c correspond to larger pointing errors and this will result in a larger reduction of the boresight e.i.r.p. density. As seen from this figure, a small reduction in the boresight e.i.r.p. density will be necessary to account for the antenna pointing errors, for example, when  = 1.5 and c = 0.2º this reduction is 0.9 dB and this increases to about 1.45 dB for  = 1.5 and c = 0.35º.

Figure 6

Reduction in boresight e.i.r.p. density necessary to satisfy the illustrative 
statistical mask Pmax (EIRPexcess) in equation (12)

# 7	An illustrative method to determine the boresight e.i.r.p. density to comply with the example statistical e.i.r.p. density mask

In this section a method is given to determine the compliance with the illustrative statistical off-axis e.i.r.p. density mask specified in equation (12) in § 5. Specifically, for a given set of elevation and azimuth angle errors, the method shows how to compute the maximum value of the boresight e.i.r.p. density of the antenna system. The illustrative method is as follows:

## 7.1	Input to the computation

a)	Representative sample values of the elevation and azimuth angle errors, φε(m) and φa(m),m = 1, 2, ..., M, where M is the sample size. These should correspond to real-time measurements or data collected from sample paths that have characteristics similar to where the terminal is expected to operate. It is assumed that the sample size M is sufficiently large so that the statistical quantities computed using these samples are reasonably good estimates for the desired statistical values.

b)	Elevation and azimuth angles to the wanted satellite,S, given, respectively, as εS and aS. Elevation and azimuth angles in the direction of S, given, respectively, as εSϕ and aSϕ. Figure 2 shows the relative geometry of S and S. Here S may be located at any point on the GSO and ϕ is a variable.

c)	Normalized gain pattern of the antenna, G(), where ϕ is the off-axis angle and, for illustrative purposes, it is assumed that the antenna pattern is symmetric about its boresight direction.

d)	A statistical off-axis e.i.r.p. density mask as in equation (12).

## 7.2	Estimating the CDF of G(θBSϕ)

e)	Using a) and b) above, compute the sum of the elevation angles toward the boresight and S directions, εBSϕ+; and the difference in the elevation and azimuth angles in these directions, εBSϕ− and aBSϕ−. Making use of equation (5), these are computed as:

εBSϕ−(m) = (εS−εSϕ−φε(m))εBSϕ+(m) = (εS+εSϕ−φε(m))aBSϕ−(m) = (aS−aSϕ−φa(m))

where the dependence on the sample index m is explicitly shown. Observe that εBSϕ−(m),εBSϕ+(m) and aBSϕ−(m) are functions of the off-axis angle ϕ.

f)	Substitute the above values in equation (4) to compute the angle between the boresight and S direction,θBSϕ(m). From equation (4):

θBSϕ(m) = cos−1cos εBSϕ−(m) −  cos εBSϕ+(m) + cos εBSϕ−(m) × sin2aBSϕ−(m)2

g)	By using the antenna gain pattern in c) and θBSϕ(m) computed above determine the antenna gain in the S direction,G (θBSϕ(m)). Note that G (θBSϕ(m)) can be considered to be a random variable with M samples, and it is a function of the off-axis angle ϕ.

h)	Estimate the CDF of G (θBSϕ), FG (θBSϕ), using the M samples computed in g). Observe that since FG (θBSϕ) is a function ofϕ, the CDF has to be computed for each value of the variableϕ.

## 7.3	Computing the maximum value for EBthat complies with the example statistical mask

i)	Choose appropriate values for EIRPexcess, 1  EIRPexcess  10, and EB, (EB,max – EB)  EB  EB,max, and using the CDF estimated in h) determine the probability in equation (8) for each value of ϕ. Here EB,max is the maximum boresight e.i.r.p. density in the absence of antenna pointing errors and EB accounts for the reduction in the boresight e.i.r.p. density due to antenna pointing errors; for illustrative purposes set EB to ΔEB,max/2.

Note that equation (8) should be determined for all values of interest of EIRPexcess and EB; however, if it is known that the reduction in the boresight e.i.r.p. density is small, this computation may be simplified by limiting the range of values of EB. Figure 6 shows the reduction in the value of EB for the specific system parameters considered in § 6. For example, since this reduction is small a value EB = ΔEB,max/3 is appropriate for the relevant system parameters.

j)	For fixed values of EIRPexcess and EB determine PEB(e.i.r.p.excess) using the expression given in equation (10), which is the maximum value of the probability computed in i) with ϕ as the variable. The expression for PEB(e.i.r.p.excess) is rewritten below:

PEB = (EIRPexcess) = max ϕPr G (θBSϕ) > (ERef (ϕ) × EIRPexcess / EB)

Observe that PEB(e.i.r.p.excess) is a non-decreasing function of EB, when it is considered as a function of EB for fixed values of EIRPexcess.

k)	Finally, determine the maximum value of EB so that PEB(e.i.r.p.excess) is less than the example mask in equation (12) for all values in the range of interest of EIRPexcess. The desired value of EB satisfies equation (13) and is rewritten as:

maxEB PEB (EIRPexcess) ≤ P (10 × log10 (EIRPexcess)) 1 ≤ EIRPexcess ≤ 10

Note that the above method is only for illustrative purposes and should not be construed to be the only method of computing the value of EB.

References

CHEN, C.-C. and GARDNER C.S., [March 1989] Impact of Random Pointing and Tracking Errors on the Design of Coherent and Incoherent Optical Intersatellite Communication Links, IEEE Trans. Comm. Vol. 37, 3, p. 252-260.

CORRELL, C., [1996] A Mathematical Model Describing the Effects of Pointing and Tracking Errors Due to “Mechanical Noise” in Coherent Optical Intersatellite Links, Inter. J. Sat. Comm. Vol. 14, .

MARAL, G. and BOUSQUET M., [2000] Satellite Communications Systems: Systems, Techniques and Technology, John Wiley & Sons.

SAMORODNITSKY, G. and TAQQU M.S., [1994] Stable Non-Gaussian Random Processes: Stochastic Models with Infinite Variance. Chapman & Hall/CRC.

SHAO, M. and NIKIAS C.L., [July 1993] Signal Processing with Fractional Lower Order Moments: Stable Processes and Their Applications. Proc. IEEE, Vol. 81, 7, p. 986-1010.

Annex 2

Methodology to assess interference levels resulting from variations
in earth station off-axis e.i.r.p. due to pointing errors caused
by movement of the vehicle-mounted platform

# 1	Introduction

The off-axis e.i.r.p. density level from terminals that are mounted on vehicle platforms is time-varying and hence will result in a time-varying interference signal at neighbouring geostationary satellites. This annex provides a methodology to analyse and quantify the time-varying interference resulting from vehicle-mounted earth stations. The interference effects from time-varying sources have been addressed in Recommendation ITU-R S.1323; this Recommendation also establishes the maximum permissible time allowance for short-term interference levels. This annex follows the guidelines established in Recommendation ITU-R S.1323 and develops a methodology to analyse the interference due to time-varying antenna pointing errors of terminals mounted on vehicles. The methodology provided will be useful in determining the appropriate level of the boresight e.i.r.p. density of these terminals such that they comply with the interference allowances to other satellite systems and satisfy the various performance objectives of those systems.

# 2	Interference assessment criteria

The performance of an FSS system critically depends on the interference it receives from other systems. Recommendation ITU-R S.1432 summarizes various considerations on interference that have been addressed in other ITU-R Recommendations. The interference criteria used in those relevant ITU-R Recommendations are based on the amount of interference that can be tolerated over long periods of time as well as during short time intervals. The former is known as the long-term criterion because the interference is averaged over a sufficiently long time. The latter criterion specifies the amount of interference over short time intervals; the interference over these intervals is typically time-varying and hence it is generally represented by a probability distribution.

Recommendation ITU-R S.1323 addresses time-varying interference impacts and establishes the maximum allowable levels of interference from different sources. However, in that Recommendation, there is no specific apportionment of the link unavailability for FSS networks for time-varying interference produced by GSO VMES. The time allowance for link unavailability due to interference from GSO/VMES is simply an example value. The methodology has the flexibility to use any apportionment, as may be agreed between administrations or to be addressed in a future ITU-R Recommendation.  The underlying assumption is that the satellite link is designed with sufficient link margin to account for propagation impairments such as signal fading due to rain, receiver noise variations, and long term interference effects from other satellite networks. To account for the impact of these degradations, the satellite link performance objectives are given in terms of outage values for the bit-error-rate or the carrier-to-noise power (C/N) ratio. For example, for a given set of (C/N) ratios and corresponding outage time allowance pairs, {(C/N)i,, pi %}, i = 1,2,…I, the (C/N) ratio should be less than (C/N)i only for pi % of the time (in any month).

According to Recommendation ITU-R S.1323 the propagation effects should account for no more than 90% of the link unavailability. Hence, the above statement can be re-stated as follows: the (C/N) ratio, computed in the absence of the time-varying interference, should be less than (C/N)i for at most pi %  90% of the time. The remaining 10% of the time allowance for link unavailability is allocated to the additional degradation due to the time-varying interference caused by the earth and space station emissions of all other satellite networks operating in the same frequency band. Hence, the overall (C/N) ratio, computed in the presence of propagation effects and the time-varying interference, should be less than (C/N)i only for pi % of the time, as required.

This document addresses the increase in interference to neighbouring satellite systems due to time-varying antenna pointing errors. This increase in interference is with respect to a terminal of the exact same characteristics but operating in a stationary environment and in the absence of antenna pointing errors. It is assumed that in this static case the terminal complies with the off-axis e.i.r.p. emission mask established in Recommendation ITU-R S.728 and satisfies the numerous interference requirements set forth in relevant recommendations. The increase in the long-term interference can be determined by averaging the interference due to the time-varying antenna pointing errors over a period Tavg and comparing this result with its corresponding value in the static case. The long-term interference parameter, Tavg, should represent a sufficiently long period so that it contains characteristic time variations of the interference signal. For this case, the interference level can be controlled by the boresight e.i.r.p. density of the terminal. Details of this methodology will be given in § 4.

To satisfy the short-term objectives the methodology given in Recommendation ITU-R S.1323 may be employed. However, it should be noted that this Recommendation has been established specifically for the case when the time-varying interference is due to non-GSO systems. This is emphasized by Note 1 of this Recommendation, which states that the 10% allocation for link outages due to interference sources, as discussed above, is not applicable to interference between GSO FSS systems. Therefore, since this document addresses interference originating from GSO FSS systems, the time allowance allocated for link outages for time-varying interference will be represented by the parameter Tallow%, rather than 10% as per Recommendation ITU-R S.1323. The objective of this document is to evaluate the effects of time-varying interference with respect to the static case, which includes the propagation effects and interference in the absence of time-varying antenna pointing errors.

Hence, the reference case for evaluating the effects of time-varying interference is considered to be the static case. Observe that the corresponding reference case in Recommendation ITU-R S.1323 is when the degradation is due to propagation effects only. Then, the performance objective is such that the link outages in the static case are allocated at most (100 – Tallow)% of the time allowance. Using the earlier expression, the (C/N) ratio computed with propagation effects and interference in the static case, should be less than (C/N)i for at most pi%  (100 – Tallow)% of the time. Consequently, the overall (C/N) ratio, computed in the presence of time-varying antenna pointing errors and propagation effects, is less than (C/N)i for pi % of the time, as required. As in the long-term interference case, the boresight e.i.r.p. density of the antenna can control the link outage; this aspect is discussed in detail in § 4.

# 3	Reference framework for the interference analysis

This section provides a reference framework used to assess the interference and it lists the parameters and notations used in the equations that follow.

Figure 7 illustrates the wanted and the interfering satellite networks. The wanted satellite is denoted as S1 and its transmit and receive terminals are denoted by T1 and R1, respectively. The interfering terminal is T2 and its intended satellite is S2. The victim receiver, R1, receives the signal from both satellites, S1 and S2, as shown in Fig. 7.

Figure 7

Wanted and interfering satellite networks. Terminals T1, R1 and S1 are in the 
wanted network; terminal T2 and satellite S2 are in the interfering network

The following is a list of parameters and the notation adopted in this Recommendation.

 :	off-axis angle from T2 to S1 in the absence of antenna pointing errors

 :	off-axis angle from R1 to S2

A:	rain fading in the uplink from T1 to S1

A:	rain fading in the downlink from S1 to R1

A:	rain fading in the uplink from interference source T2 to S1 or S2

Bs:	e.i.r.p. density (W/Hz) in the boresight direction in the static case, which is the value in the absence of time-varying antenna pointing errors at T2. Here, boresight is the direction in which the antenna gain is at a maximum

Bt:	e.i.r.p. density (W/Hz) in the boresight direction at T2 in the presence of time-varying antenna pointing errors. Note that this parameter corresponds to EB in equation (3)

B:	reduction in boresight e.i.r.p. density, B = Bs /Bt

(C/N)cs:	carrier-to-noise power ratio at R1 under clear sky conditions. C is the wanted carrier power received at R1 from T1

(C/N)s:	(C/N) ratio at R1 in the static case, which is due to rain fading and interference from T2 in the absence of time-varying antenna pointing errors

(C/N)t:	(C/N) ratio at R1 due to rain fading and interference from T2 due to time-varying antenna pointing errors

G1():	normalized directive gain of R1 antenna in the off-axis direction  (G1(0) = 1)

G2():	normalized directive gain of T2 antenna in the off-axis direction  (G2(0) = 1). Note that this parameter corresponds to G(φ) in equation (2) of Annex 1

G2,t():	normalized directive gain of T2 antenna in the direction of  in the presence of time-varying antenna pointing errors

: 	small signal gain at S1. (e.i.r.p. at S1 toward R1) = (power flux-density from T2)  λ24π  GS1,  uplink wavelength

: 	small signal gain at S2. (e.i.r.p. at S2 toward R1) = (power flux-density from T2)  λ24π  GS2,  uplink wavelength

(G/T)1:	receive antenna gain-to-noise temperature at R1

(G/T)S1:	receive antenna gain-to-noise temperature at S1 when receive direction is toward T2

(G/T)S2:	receive antenna gain-to-noise temperature at S2 when receive direction is toward T2

Is,1:	interference power from T2 received at R1, via S1, in the absence of time-varying antenna pointing errors

Is,2:	interference power from T2 received at R1, via S2, in the absence of time-varying antenna pointing errors

It,1:	interference power from T2 received at R1, via S1, in the presence of time-varying antenna pointing errors

It,2:	interference power from T2 received at R1, via S2, in the presence of time-varying antenna pointing errors

k:	Boltzmann constant, 1.38  10–23 J/K

log(X):	log10(X)

Lu:	uplink loss (clear sky) from T2 to S1 or S2

Ld:	downlink loss (clear sky) from S1 or S2 to R1

N:	receiver noise power at R1, corresponding noise temperature T

N:	noise power from S1 received at R1

:	noise power from S2 received at R1

Nr:	rain (sky) noise at receiver R1 due to rain temperature Tr (downlink)

: 	probability density function (pdf) of X

= Pr{X  x):	cumulative distribution function (cdf) of X

: 	variable expressed in dB, 10 log10 X

:	average value of the random variable X

Zs: 	Zs = (CN)cs(CN)s, degradation of (C/N) ratio due to effects of rain fading and static interference from terminal T2

Zt:	Zt =(CN)cs(CN)t, degradation of (C/N) ratio due to effects of rain fading and time-varying interference from terminal T2.

# 4	Short-term interference effects: degradation of (C/N) ratio

This section computes the degradation of the (C/N) ratio at  due to rain fading and interference from. The rain fading considered here will introduce statistical variations to the received (C/N) ratio. First, rain fading and interference from  in the absence of time-varying antenna pointing errors are considered to compute the cdf of Zs, which is the degradation of the (C/N) ratio at R1. This is discussed in § 4.1. Next, the time-varying antenna pointing errors are introduced at T2 and the cdf of the resulting degradation of the (C/N) ratio at R1, which is denoted by Zt, is determined. These cdfs are then used to compute the relative increase in the link unavailability due to the time-varying antenna pointing errors. This is discussed in § 4.2.

The analysis given in this section assumes that the victim receiver is subject to interference from only a single adjacent satellite; if the interference signals from other neighbouring satellites are not negligible they should be accounted for in a similar manner.

## 4.1	Static case: Terminal T2 transmitting in the absence of time-varying antenna pointing errors

The clear sky carrier-to-noise power ratio at the receiver, R1, for the satellite network shown in Fig. 1 is:

(CN)CS = CN↓ + N↑ + IS,1 + IS,2+ N↑,2 	(14)

Here the interference terms  and  are due to T2 transmitting with boresight e.i.r.p. density Bs in the static case, which is in the absence of time-varying antenna pointing errors. When T2 is a small aperture terminal it may be assumed that Bs complies with the off-axis e.i.r.p. mask given in Recommendation ITU-R S.728 and the relevant interference recommendations. Note that in equation (14) the term N + Is,1 is from  and Is,2 + N,2 is from. In the presence of uplink and downlink rain fading of the wanted signal, the (C/N) ratio is:

(CN)S = CA↑ A↓N↓ + N↑ /A↓+ Nr (1 – 1/A↓) + (IS,1 + IS,2)/(A↑,I A↓)N↑,2/A↓ 	(15)

Here it is assumed that the satellites S1 and S2 are very closely spaced so that uplink rain fading components to these satellites from T2 are approximately the same. Also, it is assumed that the downlink rain fading components from S1 and S2 to R1 are similar. When S1 and S2 are not closely spaced these assumptions may not be valid; in such cases, the correlation between the respective fading components should be taken into account. Note that the term, Nr (1 – 1 / A), (A  1), in the enominator denotes the additional receiver noise due to the rain temperature, .

From this the degradation of the (C/N) ratio in the presence of uplink and downlink rain fading can be expressed as:

Zs = (A↑  A↑,I) × (A↓ A↑,I  d1 + A↑,I d2 + d3)	(16)

where the link variables c1, c2, c3, c4, c5, d1, d2 and d3 are defined as:

(17)

These variables can be determined for a given set of link variables. Specifically:

c1 = G2 (0)G2 (ϕ) GS2GS2  G1 (θ)G1 (0)c2 = Bs G2 (ϕ) (GT)S1k Lu c3 =  (GT)1 GS1 (GT)S1Ld c4  = TrT↓c5 = k LuBs G2 (0) (GT)S2	(18)

where, in order to simplify the expressions, it is assumed that in the absence of rain fading the uplink propagation losses from T2 to satellites  and  are the same. A similar assumption holds for the downlink propagation losses from satellites  and  to R1.

Taking the logarithm of Zs in equation (16):

Zs = A↑ – A↑,I  + 10 log 10(A↓ +A↑, I)10 d1 + 10(A↑, I)10 d2 + d3 	(19)

It is easier to determine analytically the cdf of in the special case when the uplink rain fade component is ignored, that is . Note that this particular case is considered only because of analytical simplicity; in the more general case the rain fading component should not be ignored. In this case the cdf of the degradation can be expressed as:

Ps(z) = Pr (Zs ≤ z ) = pA↓ (u) pA↑ (v) dv du

where the region of integral is such that forthe value of satisfies . In the above it is assumed that the rain fading components, , are independent of each other.

Since is a monotonically increasing function of A↓ (≥ 0) and A↑ (≥ 0)  the region can be expressed as the region bounded by 0 ≤ A↑ ≤ z – 10 log (10A↓10 d1 + d2 + d3) and 0 ≤ A↓ ≤ 10 log ((10z10 – d2 – d3 ) / d1).

The above integral is then evaluated as:

ps (z) = pA↓ (u)pA↑(v) dvdu = pA↓ (u) pA↑ (v) dv du = pA↓ (u) pA↑ (z – 10  log(10u/10 d1 + d2 + d3)) du	(20)

where u' = 10 log ((10z / 10 – d2 – d3) d1).

## 4.2	Terminal T2 transmitting in the presence of time-varying antenna pointing errors

In this case time-varying antenna pointing errors are introduced at the transmit terminal T2. In order to limit additional interference in the presence of antenna pointing errors, the boresight e.i.r.p. density has to be reduced to  from. The (C/N) ratio in the presence of rain fading and the time-varying antenna pointing errors follows from equation (2) as:

(CN)t = CA↑ A↓N↓ + N↑/A↓ + Nr (1 – 1/A↓) + (It,1 + It,2)/(A↑,IA↓) +  N↑,2 /A↓	(21)

where the time-varying interference terms It,1 and It,2 can be expressed as fractions of their corresponding values in the static case:

It,1Is,1 = Bt G2,t (ϕ)Bs G2 (ϕ)It,2Is,2 = Bt G2,t (0)Bs G2 (0)	(22)

The above antenna gain factors,  and , can be obtained from the knowledge of the antenna gain pattern, for example, the normalized antenna gain pattern may be expressed as:

G(ϕ) = 2n + 1 (n + 1 )! jn + 1 (πd  λ × sin ϕ)(πd  λ × sin ϕ)n + 1	(23)

where  is the off-axis angle, Jn + 1 is the Bessel function of the first kind and order (n + 1), d is the diameter of the circular aperture and λ is the wavelength. In the above n is the antenna aperture illumination parameter and this corresponds to the following aperture illuminations: n = 0, ideal uniform; n = 1, parabolic; and n = 2, parabolic squared. The main lobe of many practical aperture systems falls between the gain patterns corresponding to n = 0 and n = 2. Note that the side lobes of practical antennas may not be accurately represented by equation (23).

The antenna gain factor in the absence of pointing errors, according to the notation used in equation (22), G2 () = G (). In the presence of antenna pointing errors, the errors in the boresight direction of the antenna can be characterized by elevation and azimuth errors denoted by  and a, respectively. Then the required angular error, which is the angle between the boresight direction of the antenna and the direction from T2 toward S1, can be expressed as (, , a) where the function  can be determined for a specific geometry as shown in § 3 of Annex 1. The antenna gain factor in equation (22) is now expressed as:

It,1 ≤ 1ΔB 1G2 (ϕ) Is,1	(24)

In the presence of antenna pointing errors, because of mispointing of the main beam of T2 away from S2, it can be seen that It,2 < Is,2. That is, for interference received at R1 via S2, the degradation due to antenna pointing errors is always less than its corresponding interference in the static case. For interference received via S1, from equation (22), the maximum value of the interference component occurs when G2,t (), that is, when the boresight of the antenna is aligned along the direction from T2 to S1. This demonstrates that, for the two-satellite system considered here, the time-varying interference power can be upper bounded, irrespective of the magnitude of the pointing error.

From equations (14) and (21) the degradation of the (C/N) ratio in the presence of antenna pointing errors is:

Zt = (A↑ A↑,I) × (A↓ A↑,I e1 +A↑,I e2 +G2,t(ϕ) / ΔB e3 + G2,t (0) ΔB e4) 	(25)

where the link variables are defined as:

e1 = 1 + c41 + c1 c2 c3 (1+c5) +  c2 c3 + c3e2 = c1 c2 c3 c5 + c3 – c41 + c1 c2 c3 (1+c5) +  c2 c3 + c3e3 = 1G2 (ϕ) c2 c3 1 + c1 c2 c3 (1+c5) +  c2 c3 + c3e4 = 1G2 (0) c1 c2 c3 1 + c1 c2 c3 (1+c5) +  c2 c3 + c3	(26)

Expressing equation (25) in logarithmic form:

Zt = A↑– A↑,I + 10 log 10(A↓ + A↑,I)/10 e1 + 10A↑,I10 e2 + 10(G2,t (ϕ) – Δ B)/10e3 + 10(G2,t (0) – Δ B)/10e4 (27)

As in the static case, for analytical simplicity, consider the special case when . In this case the cdf of can be expressed as:

Pt(z) = Pr(Zt ≤ z) =pG2,t(0)(w) pG2,t(φ) (v) pA↓(u) pA↑ (t) dt du dv dw

where the region of integral is such that for G2,t(0), G2,t(ϕ),A↓, A↑ ∈ Rz; Zt ≤ z. Here it is assumed that the respective random variables are statistically independent; note that, under general conditions, the random variables G2,t(0) and G2,t(ϕ) may not be statistically independent. In such cases the joint distribution of these two random variables should be considered in the above expression. Noting that  is a monotonically increasing function of the variablesG2,t(0), G2,t(ϕ), A↓ and A↑, the above integral can be expressed as:

Pt (z) = pG2,t(0) (w) pG2,t(φ) (v) pA↓ (u)  pA↑ (z  – 10 log (10u10 e1 + e2 + 10(v – Δ B)10e3 + 10(w – Δ B)10e4)) du dv dw	(28)

where the upper limits of the integrals are as follows:

u' = 10 log 10z10 – e2 – 10(v – Δ B)10 e3 –10(w – Δ B)10 e4e1 v' = 10 log 10z10 – e1 – e2 – 10(w – Δ B)10 e4 e3 + Δ Bw' = 10 log 10z10 – e1 – e2 – 10– Δ B10 e3 e4 + Δ B 	(29)

The relative increase in the link unavailability due to time-varying antenna pointing errors with respect to the total link unavailability,  (%), can be expressed as:

R(z)% = 100% × (1 – Pt (z) )– (1 – Ps (z))(1 – Pt (z) )	(30)

Note that it may be deduced from the discussion leading to equation (24) that for larger values of ,  could be greater than . This implies that in some cases, that is, by reducing the boresight e.i.r.p. density it is possible to make the time-varying interference less than the corresponding interference in the absence of time-varying antenna pointing errors. To compute as given in equation (30), assume the total link unavailability is pi% and (100 – Tallow)% of this unavailability value is allocated to static interference. The fade margin required under static conditions for this link unavailability, z, is computed from (1 – Ps (z)) = pi % × 100 − Tallow%. The overall link unavailability for this fade margin in the presence of time-varying interference is (1 – Pt (z)). From these the relative increase in the link unavailability for this fade margin, R(z)%, can be computed using the expression given in equation (30).

TABLE 1

Parameters for the link from T2 to R1

| Frequency uplink (GHz) | 14.2 |
| --- | --- |
| Uplink, downlink losses (dB) | 207, 205.3 (Ankara) 207.2, 205.3 (London) |
| Boltzmann’s constant (dBW/Hz/K) | −228.6 |
| Small signal gain at S1, (dB) | 175.2 |
| (G/T)S1 (dB/K) | 2 (Ankara) 4 (London) |
| Frequency downlink (GHz) | 11.7 |
| Tr (K) | 285 |
| Off-axis angle, from T2 to S1 | 2.22° (Ankara) 2.18° (London) |

TABLE 2

Parameters to compute rain attenuation

| S1 location (Eutelsat W1) | 10° E |
| --- | --- |
| S2 location | 12° E |
| T1 location (Canary Islands) | Latitude: 27.76° N Longitude: −15.63° E |
| T1 altitude above mean sea level (m) | 205 |
| R1 location (Matera, Italy) | Latitude: 40.39° N Longitude: 16.42° E |
| R1 altitude above mean sea level (m) | 527 |
| T2 location (Ankara, Turkey) | Latitude: 39.8° N Longitude: 32.8° E |
| T2 location (London, England) | Latitude: 51.5° N Longitude: −0.12° E |
| T2 altitude above mean sea level (m) | 200 (Ankara) 200 (London) |

# 5	Example calculation using the above methodology

This section provides an example calculation to illustrate the methodology given in § 4. Table 1 gives the link parameters used in this example. The rain attenuation components, ,  and , are computed according to Recommendation ITU-R P.618 using the parameters given in Table 2. In this example a large aperture antenna is assumed for the receive terminal R1. Since the antenna gain is such that G1 () << G1 (0), the interference received from S2 at R1 is neglected in this example.

Figure 8 shows the cdfs of the random variables G2,t (0) and G2,t () in the presence of time-varying antenna pointing errors. The antenna pointing errors correspond to azimuth and elevation errors generated from an SS distribution as discussed in § 2 of Annex 1, where the parameters of the distribution are:  = 1.5 and c = 0.35º. The specific antenna gain antenna pattern used is given in equation (23) with n = 1 and d = 0.51 m. In this example the off-axis angle from T2 to S1, , is 2.22º and the corresponding normalized antenna gain, G2 () = – 6.7 dB. This figure shows the fluctuations of G2,t () in the presence of antenna pointing errors. Note that in comparison to the static case, the interference to S1 will be increased when G2,t () > G2 () and decreased when G2,t () < G2 ().

Figure 8

The cdfs of normalized antenna gains, G2,t (0) and G2,t () in the presence 
of time-varying antenna pointing errors (T2 in Ankara, Turkey)

Figure 9 shows  for different values of the antenna boresight e.i.r.p. density. In this example a Monte-Carlo simulation of the degradation variables,  and  as given in equations (19) and (27), is carried out to determine the probabilities. In this example, for illustrative purposes, a 98% link availability condition is considered. As discussed in § 2, this is an input requirement to protect neighbouring satellites; hence for this example pi% = 2%. Suppose 90% (Tallow% = 10%) of the link unavailability is assigned to rain fading and static interference, that is (1 – Ps (z)) = (1 – 0.98)  90%. The corresponding fade margin, zmargin, that satisfies the above is determined so that (1 – Ps (zmargin)) = (1 – 0.98)  90%. Next, the overall link unavailability in the presence of antenna pointing errors and for this fade margin  1 – Pt  (zmargin) is determined. The value of R(zmargin) can be obtained from equation (30).

As seen from Fig. 19, in the presence of time-varying antenna pointing errors, the link unavailability can be substantially lowered by reducing the boresight e.i.r.p. density. Observe that, as stated above, for larger values of  the increase in the link unavailability could be such that. Also, for large values of the receive terminal (G/T) the receiver is more sensitive to interference received from the satellite; this increases the interference and thus increases the unavailability of the link.

Figure 9

R(z) for the link parameters in Tables 1 and 2. The legend denotes 
the (G/T) (dB/K) at receive terminal R1

# 6	Long-term interference effects

As stated earlier, the long-term interference power is computed by averaging the interference power over a satisfactorily long time period so that all representative variations of the time-varying interference signal are contained within this time period. Relevant ITU-R Recommendations that impose limits on the average interference power consider the interference power received to the total receiver system noise power ratio. In the absence of time-varying antenna pointing errors, considering the interference from  that is received at  via satellites  and , this ratio for the interference power can be expressed as:

fs = Is,1 + Is,2 N↓ + N↑ + Is,1 + Is,2 + N↑,2 = c2 c3  + c1 c2 c3 1 + c1 c2 c3 (1 + c5) + c2 c3  + c3  	(31)

Next, time-varying antenna pointing errors are introduced to the above static case and the interference power is averaged over Tavg to obtain the time-averaged values of the interference terms It,1 and It,2, which are denoted by  and . The average interference power as a fraction of the total system noise power can then be expressed as:

ft = It,1 + It,2N↓ + N↑ + It,1 + It,2 + N↑,2  = c2 c3  G2,t (ϕ)  (ΔB G2 (ϕ)) + c1c2c3 G2,t (0)  (ΔB G2 (0))1 +c3 + c2c3 G (ϕ)  (ΔB G2 (ϕ)) + c1c2c3 G2,t (0)  (ΔB G2 (0)) + c1c2c3c5  	(32)

Thus, the increase in the long-term average interference with respect to the total interference is expressed as:

RL (%) = 100 % × ft – fsft	(33)

# 7	An illustrative procedure to implement this methodology

This section presents a step-by-step procedure to implement the computations given in § 4. Specifically, the procedure, based on Monte-Carlo simulations, computes the increase in the link unavailability because of the time-varying antenna pointing errors. Note that this is an illustrative approach, as other approaches may be used.

## 7.1	Input to the computation

Link parameters: Longitudes and latitudes of ; longitudes of satellites S1 and S2; θ, G1(θ), (G/T)1, ; , normalized antenna gain pattern of , G2(); GS1, GS2, (G/T)S1, (G/T)S2; Ld, Lu, Tr.

Rain parameters: Rain rate (0.01% mm/h), altitude above mean sea level and rain height (for T1, R1 and T2). These parameters can also be computed using Recommendations ITU-R P.837 and ITU-R P.839.

Link unavailability: The required time percentages for link unavailability, pi %. Time allowance allocated for link outages for time-varying interferences, Tallow %.

Monte-Carlo simulation parameter: Length of random vectors, N.

Antenna pointing error characteristics: Azimuth and elevation error vectors of length N,  and a, generated as discussed in § 2 of Annex 1.

## 7.2	Degradation of (C/N) ratio in the absence of time-varying antenna pointing errors

Step 1:	Determine Bs, the boresight e.i.r.p. density of T2, using G2(φ) and according to Recommendation ITU-R S.728. It is assumed that this Bs complies with various interference and coordination requirements as summarized in Recommendation ITU-R S.1432.

Step 2: Compute link variables.

a)	Using the link parameters compute the variables c1, c2, c3, c4, and c5, as given in equation (18).

b)	Compute d1 and d2 as shown in equation (17).

Step 3: Determine uplink and downlink rain fades.

a)	Using the rain parameters determine the cdfs of the uplink and downlink rain fades, , according to Recommendation ITU-R P.618.

b)	From these cdfs determine 3 vectors of random variables, each of length N, for .

Step 4: Using equation (19) generate the vector of random variables for.

Step 5: Determine the cdf of , , using this vector of random variables.

Step 6: Compute the fade margin, , required for link unavailability, pi%  (100 – Tallow)%. The required  satisfies the following: (1 – Ps (z)) = pi / 100  (100 – Tallow) / 100.

## 7.3	Degradation of (C/N) ratio in the presence of time-varying antenna pointing errors

Step 7: Determine the link parameters e1, e2, e3 and e4 in equation (13).

Step 8: Generate two random vectors, each of length N, for G2,t (0) and G2,t ().

a)	As discussed in § 2 of Annex 1 generate two random vectors, each of length N, for pointing error variables  and a.

b)	Using the relative longitudes and latitudes and the pointing errors, compute the angle vectors from the boresight direction of T2 to satellites S1 and S2,  (, , a),  (,,a) and  (0, , a), as shown in § 3 of Annex 1. Note that according to the notation in Annex 1,  (, , a) corresponds to θBSϕ expressed in equation (4).

c)	Determine the random vectors of length N for G2,t (0) and G2,t () as follows: G2,t () = G2 ( (, , a)).

Step 9: Set as a parameter.

Step 10:	 Generate  in equation (12), which is a random vector of length N.

Step 11: Determine the cdf of , , using this vector of random variables.

Step 12: Determine the link availability for fade margin  computed in Step 6, which is .

Step 13: Determine the relative increase in link unavailability, , as in equation (30).

# 8	Summary

This annex has presented a methodology to assess the interference effects to neighbouring satellites due to time-varying antenna pointing errors of VMES.

The methodology specifically addresses the increase in interference with respect to a stationary terminal, which has exact same characteristics as the vehicle-mounted terminal but without the antenna pointing errors due to motion. The approach followed is similar to that established in Recommendation ITU-R S.1323 where propagation effects such as rain fading and receiver noise variations are responsible for at most 90% of the time allowance for link outages. In this Recommendation, the propagation effects and the interference due to a static terminal are assumed  to be responsible for (100 – Tallow)% of the corresponding time allowance. A methodology to assess the increase in long-term interference with respect to the static case is also presented. In the long-term interference analysis, the signal is averaged over a period Tavg, which is assumed to be sufficiently large so that the statistical characteristics of the interference are reasonably well represented within this period.

The boresight direction is defined as the direction in which the antenna gain is largest, which is the axis of the antenna.

Satellite links are usually designed with fade margins in the link to account for such degradations.