RECOMMENDATION  ITU-R  S.739*

Additional methods for determining if detailed coordination is
necessary between geostationary-satellite networks in the
fixed-satellite service sharing the same frequency bands

(1992)

The ITU Radiocommunication Assembly,

considering

a)	that FSS networks may share the same frequency bands;

b)	that these networks may cause and experience mutual interference;

c)	that this mutual interference can be minimized through coordination;

d)	that the method described in Recommendation ITU-R S.738 is very conservative;

e)	that additional methods may be useful to determine if detailed coordination is necessary,

recommends

1	that, to the extent data is available, the following methods be used, by mutual agreement between the administrations concerned, to determine if detailed coordination is necessary:

–	the normalized T /T method described in Annex 1;

–	the power density-averaging bandwidth method described in Annex 2;

2	that the following Notes should form part of this Recommendation:

NOTE 1 – The normalized T /T method for determining the interference level using the normalized values of the apparent increase in equivalent noise temperature, is based on the technique shown in Appendix S8 to the Radio Regulations, appropriately modified to give more accurate results. This approach consists in using a set of predetermined values for T /T which depends upon the types of wanted and interfering carriers involved, instead of the single 6% value given in Appendix S8.

NOTE 2 – The power density-averaging bandwidth method is an extension of the Appendix S8 T /T method that allows the calculation of the interference power for any interfered-with carrier bandwidth. This method may be applied for determining the need for detailed coordination and may also be used in more detailed coordination.

ANNEX  1

Method of the normalized equivalent noise
temperature of the satellite link

# 1	Introduction

This method is based on the technique shown in Appendix S8 to the Radio Regulations and described in Recommendation ITU-R S.738 appropriately modified to give accurate results. To do so, the threshold of 6% used in Appendix S8 is replaced by thresholds which depend on the carriers involved and which meet the ITU-R criteria.

# 2	Normalized values for the relative increase in the admissible equivalent link noise temperature

## 2.1	Definition

The method involves determining the overall increase in equivalent link noise temperature due to the various types of transmission in the two networks.

The normalized values for the relative increase in equivalent link noise temperature are given by:

ΔTTN = IN0 B2

where:

N0 :	thermal noise density corresponding to the equivalent noise temperature of the satellite link

I/N0 :	ratio of interfering power to the thermal noise density of the wanted carrier

B2 :	bandwidth defined by the ratio of the interfering carrier power P to its maximum spectral power density pm':

pm' = P'B2

## 2.2	Values for various carrier types

The method for computing I/N0 and (T /T )N  is given in § 3 to 9.

The calculation method for (T /T )N  depends on the type of wanted and interfering carriers. Five types are considered:

–	FDM-FM,

–	SCPC-FM,

–	digital SCPC (SCPC-DIG),

–	wideband digital (DIG-BB),

–	FM-television (FM-TV).

For a given type of wanted carrier and a given type of interfering carrier, the (T /T )N value obtained (using the method which corresponds to this pair of carrier types) depends on carrier parameters such as bandwidth and coding.

In order to limit the necessary computations, each general carrier type is broken down into various sub-headings so that, for each pair of carrier types, the carrier parameters no longer affect (T /T )N (as long as these parameters lie in the range specified for each type) (see Table 1). Thus (T /T )N can be determined simply from the knowledge of the type of the two carriers.

Carriers are classified according to their type.

An initial analysis of the carriers leads to the identification of about 50 different types:

–	about 20 FDM-FM, characterized by the number of channels and allocated band,

–	several SCPC-FM, characterized by the allocated band,

–	about 15 DIG-BB, characterized by the bit rate, type of coding, and number of states,

–	several SCPC-DIG, characterized by the bit rate, type of coding, and number of states,

–	several FM-TV, characterized by the allocated band and energy dispersal characteristics.

In order to simplify the presentation of the table of thresholds, the 50 types of carriers have been gathered in 12 categories as given in Table 2.

The corresponding values of T /T for each pair of wanted and interfering categories of types of carriers is given in Table 3.

# 3	Parameters used in the computation of I/N0 and (T /T)N

The parameters used are as follows:

B0:	bandwidth of the wanted signal (Hz)

B1:	bandwidth of the interfering signal (Hz)

I/N0:	ratio of interfering carrier power-to-noise power density

C/N0:	ratio of wanted carrier power-to-noise power density

C/I:	ratio of wanted-to-interfering carrier power

B2:	bandwidth defined by the ratio of interfering power P to its maximum spectral power density pm':

pm' = P'B2

:	fraction of interfering signal power received after filtering by the wanted signal receiver filter

N0:	thermal noise power density corresponding to the equivalent noise temperature of the satellite link

N:	noise power N  N0 · B0.

Admissible normalized values for the relative increase in equivalent link noise temperature are given by:

ΔTTN = IN0 B2

TABLE  1

Standard carrier types

| SCPCA: SCPC (analogue)  frequency deviation SCPCN: SCPC (digital) pm: frequency deviation (modulated carrier) NUM-LB: wideband (digital) pnm: frequency deviation (unmodulated carrier) N/V: number of channels bal: sweep frequency N/E: number of states st: frequency deviation (signal test) Boc: occupied bandwidth m frequency deviation (multiplex signal) FDM-FM No. | Type |  |  | N/V | Boc (MHz) | min (kHz) | max (kHz) |  | st (kHz) |  | m (kHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 12 | 1.3 |  | 12 | 1.13 | 12.0 | 60.0 |  | 108.5 |  | 159.0 |
| 2 | 12 | 2.5 |  | 12 | 2.2 | 12.0 | 60.0 |  | 238.9 |  | 350.0 |
| 3 | 24 | 2.5 |  | 24 | 1.96 | 12.0 | 108.0 |  | 163.4 |  | 275.0 |
| 4 | 60 | 2.5 |  | 60 | 2.25 | 12.0 | 252.0 |  | 136.5 |  | 276.0 |
| 5 | 72 | 2.5 |  | 72 | 2.25 | 12.0 | 300.0 |  | 124.5 |  | 261.0 |
| 6 | 60 | 5.0 |  | 60 | 3.96 | 12.0 | 252.0 |  | 270.1 |  | 546.0 |
| 7 | 132 | 5.0 |  | 132 | 4.45 | 12.0 | 552.0 |  | 223.5 |  | 529.0 |
| 8 | 192 | 5.0 |  | 192 | 4.51 | 12.0 | 804.0 |  | 180.0 |  | 459.0 |
| 9 | 96 | 7.5 |  | 96 | 5.87 | 12.0 | 408.0 |  | 359.8 |  | 799.0 |
| 10 | 192 | 7.5 |  | 192 | 6.40 | 12.0 | 804.0 |  | 297.2 |  | 758.0 |
| 11 | 252 | 7.5 |  | 252 | 6.74 | 12.0 | 1 052.0 |  | 259.7 |  | 733.0 |
| 12 | 132 | 10.0 |  | 132 | 7.50 | 12.0 | 552.0 |  | 430.0 |  | 1 020.0 |
| 13 | 252 | 10.0 |  | 252 | 8.49 | 12.0 | 1 052.0 |  | 357.4 |  | 1 009.0 |
| 14 | 312 | 10.0 |  | 312 | 8.96 | 12.0 | 1 300.0 |  | 320.0 |  | 1 005.0 |
| 15 | 252 | 15.0 |  | 252 | 12.39 | 12.0 | 1 052.0 |  | 576.4 |  | 1 627.0 |
| 16 | 432 | 15.0 |  | 432 | 12.95 | 12.0 | 1 796.0 |  | 400.2 |  | 1 479.0 |
| 17 | 432 | 20.0 |  | 432 | 17.99 | 12.0 | 1 796.0 |  | 615.8 |  | 2 276.0 |
| 18 | 612 | 20.0 |  | 612 | 17.70 | 12.0 | 2 540.0 |  | 453.7 |  | 1 996.0 |
| 19 | 432 | 25.0 |  | 432 | 20.59 | 12.0 | 1 796.0 |  | 727.3 |  | 2 688.0 |
| 20 | 792 | 25.0 |  | 792 | 22.34 | 12.0 | 3 284.0 |  | 498.4 |  | 2 494.0 |
| 21 | 972 | 25.0 |  | 972 | 25.00 | 12.0 | 4 028.0 |  | 410.0 |  | 2 274.0 |
| 22 | 972 | 36.0 |  | 972 | 35.99 | 12.0 | 4 028.0 |  | 796.7 |  | 4 417.0 |
| SCPCA No. | Type |  |  |  | Boc (kHz) | min (kHz) | max (kHz) |  |  (kHz) |  |  |
| 23 | 0.020 |  |  | – | 20.0 | 0.3 | 3.4 |  | 5.8 |  |  |
| 24 | 0.025 |  |  | – | 25.0 | 0.3 | 3.4 |  | 12.0 |  |  |
| 25 | 0.030 |  |  | – | 30.0 | 0.3 | 3.4 |  | 8.5 |  |  |
| 26 | 0.090 |  |  | – | 90.0 | 0.3 | 3.4 |  | 3.4 |  |  |
| 27 | 0.180 |  |  | – | 180.0 | 0.3 | 3.4 |  | 3.3 |  |  |
| SCPCN No. | Type |  |  | N/E | Boc (kHz) | Bite rate (kbit/s) |  |  |  |  |  |
| 28 | 0.064 |  |  | 4 | 38.0 | 64.0 |  |  |  |  |  |
| 29 | 0.085 |  |  | 4 | 50.0 | 85.0 |  |  |  |  |  |
| 30 | 0.128 |  |  | 4 | 150.0 | 128.0 |  |  |  |  |  |
| 31 | 0.256 |  |  | 4 | 300.0 | 256.0 |  |  |  |  |  |
| 32 | 0.512 |  |  | 4 | 600.0 | 512.0 |  |  |  |  |  |
| NUM-LB No. | Type |  |  | N/E | Boc (MHz) | Bite rate (kbit/s) |  |  |  |  |  |
| 33 | 2Q |  |  | 4 | 1.44 | 2.048 |  |  |  |  |  |
| 34 | 3Q |  |  | 4 | 1.84 | 3.072 |  |  |  |  |  |
| 35 | 4Q |  |  | 4 | 2.25 | 4.096 |  |  |  |  |  |
| 36 | 8Q |  |  | 4 | 5.0 | 8.448 |  |  |  |  |  |
| 37 | 10Q |  |  | 4 | 5.0 | 10.0 |  |  |  |  |  |
| 38 | 17Q |  |  | 4 | 10.2 | 17.0 |  |  |  |  |  |
| 39 | 25Q |  |  | 4 | 18.0 | 24.6 |  |  |  |  |  |
| 40 | 34Q |  |  | 4 | 20.6 | 34.368 |  |  |  |  |  |
| 41 | 40Q |  |  | 4 | 20.0 | 40.0 |  |  |  |  |  |
| 42 | 50Q |  |  | 4 | 25.6 | 50.0 |  |  |  |  |  |
| 43 | 120Q |  |  | 4 | 75.0 | 120.0 |  |  |  |  |  |
| 44 | 139Q |  |  | 4 | 82.0 | 139.264 |  |  |  |  |  |
| 45 | 147Q |  |  | 4 | 110.0 | 147.0 |  |  |  |  |  |
| FM-TV No. | Type |  |  (MHz) |  | Boc (MHz) | pm (MHz) |  | pnm (MHz) |  | bal (Hz) |  |
| 46 | TV.17 |  | 4.75 |  | 17.5 | 1.0 |  | 2.0 |  | 60/30 |  |
| 47 | TV.20 |  | 4.8 |  | 20.0 | 1.0 |  | 2.0 |  | 50 |  |
| 48 | TV.30 |  | 6.2 |  | 30.0 | 2.0 |  | 4.0 |  | 50 |  |
| 49 | TV.35 |  | 5.0 |  | 30.0 | 2.0 |  | 4.0 |  | 50/25 |  |
| 50 | TV.36 |  | 11.0 |  | 32.0 | 1.0 |  | 2.0 |  | 50 |  |

TABLE  2

Categories of carrier

| Type of carrier |  | No. |
| --- | --- | --- |
|  | Boc  3 MHz | 1-5 |
|  | 3 MHz  Boc  7 MHz | 6-11 |
| FDM-FM | 7 MHz  Boc  15 MHz | 12-16 |
|  | Boc > 15 MHz | 17-22 |
|  | Boc  3 MHz | 33-35 |
|  | 3 MHz  Boc  7 MHz | 36-37 |
| Wideband digital | 7 MHz  Boc  15 MHz | 38 |
|  | Boc  15 MHz | 39-45 |
|  | PSK | 28-32 |
| SCPC | CFM | 23-27 |
|  |   7 MHz | 46-49 |
| FM-TV |   7 MHz | 50 |
| Boc: occupied bandwidth : frequency deviation |  |  |

TABLE  3

Single carrier to single carrier T/T threshold values

|  | Interfering carrier | FDM-FM |  |  |  | Wideband digital |  |  |  | SCPC |  | FM-TV |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wanted (1) carrier | Boc (MHz) |  3 | 3-7 | 7-15 |  15 |  3 | 3-7 | 7-15 |  15 | PSK | CFM (2) |   7 |   7 |
|  |  3 | 13 | 12 | 12 | 11 | 8 | 10 | 10 | 8 | 9 | 1 223 | 11 | 11 |
|  | 3-7 | 23 | 14 | 12 | 12 | 11 | 10 | 10 | 8 | 29 | 4 350 | 11 | 13 |
| FDM-FM (3) | 7-15 | 40 | 20 | 14 | 12 | 17 | 10 | 10 | 8 | 56 | 8 458 | 12 | 19 |
|  |  15 | 102 | 46 | 24 | 14 | 40 | 19 | 11 | 8 | 148 | 22 257 | 23 | 45 |
|  |  3 | 15 | 10 | 9 | 9 | 9 | 9 | 9 | 9 | 21 | 3 085 | 9 | 9 |
| Wideband | 3-7 | 49 | 21 | 12 | 9 | 19 | 9 | 9 | 9 | 71 | 10 712 | 11 | 21 |
| digital (4) | 7-15 | 100 | 44 | 21 | 11 | 39 | 17 | 9 | 9 | 146 | 21 853 | 22 | 44 |
|  |  15 | 176 | 77 | 38 | 15 | 69 | 31 | 15 | 9 | 257 | 38 565 | 39 | 77 |
| SCPC | PSK (4) CFM (3) | 9 11 | 9 11 | 9 11 | 9 11 | 9 11 | 9 11 | 9 11 | 9 11 | 9 11 | 9 11 | 2 21 | 2 36 |
| FM-TV |   7   7 | 73 23 | 32 10 | 16 5 | 6 2 | 29 9 | 13 4 | 6 2 | 2 1 | 107 34 | 16 046 5 098 | 16 5 | 32 10 |
| NOTE 1 – When several equal power interfering carriers of one of the types given in Table 1 are included in the wanted bandwidth, these values should be decreased in accordance with the number of these interfering carrier. (1) The Table reflects the value for the most sensitive carrier in any range. (2) This Table should not be used for carrier types not included in Table 1. (3) Criterion used: 800 pW0p single entry and 7 000 pW0p total. For FM-TV interference a 20% allocation to external satellite interference is assumed. (4) Criterion used: 6% single entry and 70% total. For FM-TV interference a 20% allocation to external satellite interferences is assumed and a value of 12.3 dB is assumed for energy per bit to noise power density ratio (BER  1  10–6). |  |  |  |  |  |  |  |  |  |  |  |  |  |

# 4	Interference criteria

In the computation of (T /T)N for analogue FDM-FM signals, the equivalent link noise temperature should correspond to a noise power in a telephone channel of 7 000 pW0p for systems with frequency re-use and 6 500 pW0p for systems without frequency re-use (Recommendations ITU-R S.466 and ITU-R SF.356).

For digital signals, the equivalent link noise temperature should correspond to 70% (for systems with frequency re-use) and 65% (for systems without frequency re-use) of the total noise power level which would give rise to a bit error ratio of 10–6 (Recommendations ITU-R S.523 and ITU-R SF.558).

For TV/FM type signals, the criterion given in Recommendation ITU-R S.483 should be applied. Accordingly, taking into account interference from terrestrial radio links, the single-entry interference criterion referred to permissible video noise is 5%.

For an SCPC-FM signal, the criterion for interference from other than TV-FM signals is assumed to be 600 pW0p in a channel for an equivalent link noise temperature of 7 000 pW0p for systems with frequency re-use and 6 500 pW0p for systems without frequency re-use.

For SCPC-PSK and SCPC-CFM carriers, the criterion for interference from TV-FM signals should correspond to Recommendation ITU-R S.671.

It should be noted that if the spectrum of the wanted signal is broader than the spectrum of the interfering signal, total interference due to all interfering signals from the same network within the bandwidth of the wanted signal should be considered.

# 5	FDM-FM wanted carriers

At a 1 mW reference point, the psophometrically-weighted level Np of noise-like baseband interference is given in pW0p by:

10 log Np = 87.5 – B – 10 logCI dB = 87.5 – P + 10 log b + 10 log D(f,f0) – 20 logδff = –3 – 10 logCI

where:

B :	interference reduction factor

b :	telephone channel bandwidth (Hz)

 :	r.m.s. test-tone deviation of the wanted signal (Hz)

m :	top baseband frequency of the wanted multiplex signal (Hz)

D( , 0) :	convolution product of wanted and interfering spectra

0 :	separation between carrier frequencies of wanted and interfering signals (Hz)

 :	central frequency of the selected channel, located in the baseband of the wanted signal (Hz)

P 	  10 log p( /m): pre-emphasis (dB).

Thermal noise after demodulation is given by:

10 log Nth = 87.5 − P − 10 log CN0 + 10 log b − 20 log δff dB

where:

N0  kT :	noise density power on the wanted link

with	k :	Boltzmann’s constant

and	T :	equivalent satellite link noise temperature as defined in RR No. S1.174

thus:

10 log NpNth = 10 log IN0 − 3 + 10 log D(f, f0) dB

NpNth = IN0 ⋅ D(f, f0)2

The single entry criterion established by Recommendation ITU-R S.466 corresponds to Np  800 pW0p, for an Nth value equal to 7 000 or 6 500 pW0p. As an example, for 7 000 pW0p which is applicable to systems with frequency re-use:

IN0 = 0.1143 2D(f, f0) = 0.2286D(f, f0)

hence:

ΔTTN = 0.2286D(f, f0) ⋅ 1B2

# 6	SCPC-FM wanted carrier

## 6.1	Interference from an FM-TV carrier

In this case the 10 log C/I  13.5  2 log  – 3 log (i/10) criteria must be respected (Recommendation ITU-R S.671),

thus:

IN0 = CN0 ⋅ IC = CN0 ⋅ i0.3101.65⋅ δ0.2

thus:

ΔTTN = CN0 ⋅ i0.3101.65⋅ δ0.2 ⋅ 1B2

with:

δ = B0Δf and B2 = Δf

where:

 :	peak-to-peak frequency deviation of TV signal due to energy dispersal (Hz)

i :	percentage of total predemodulation noise allocated to internetwork interference.

For the example given in Table 1, thermal noise is given by:

10 log Nth  188.7 – 10 log C/N0 – 20 log  f                    dB

 :	r.m.s. deviation of the SCPC-FM wanted signal (Hz).

After companding, the following is generally obtained:

C/N0  014.9/ f 2

## 6.2	Interference from a carrier other than FM-TV

All other interfering signals have spectra significantly broader than the wanted signal spectrum (SCPC), thus:

ΔTTN = NpNth

where:

Np 	800 pW0p: permissible single-entry interference criterion

Nth 	6 500 pW0p or 7 000 pW0p corresponding to systems without or with frequency re-use. Thus for 7 000 pW0p:

ΔTTN = 8007 000 = 11.4%

# 7	Digital SCPC wanted carrier

## 7.1	Interference from an FM-TV carrier

In this case the criterion 10 log C/I  10 log C/N  6.4  3 log  – 8 log (i/10) must be respected (Recommendation ITU-R S.671).

thus:

IN0 = CN0 ⋅ IC = NN0 ⋅ i0.8101.44⋅ δ0.3

thus:

ΔTTN = i0.8101.44⋅ δ0.3 ⋅ B0B2

with:

δ = B0Δf and B2 = Δf

 :	peak-to-peak frequency deviation of TV signal due to energy dispersal (Hz)

i :	percentage of total pre-demodulation noise allocated to internetwork interference.

For the example given in Table 1, C/N0 is given by:

CN0 = EN0 ⋅ Du

where:

E :	energy per bit

Du :	useful bit rate

N0 :	noise power density.

## 7.2	Interference from a carrier other than FM-TV

All other signals have spectra significantly broader than the wanted signal spectrum (SCPC). Thus for systems with frequency re-use:

ΔTTN = 0.060.7 = 8.57%

# 8	Broadband digital wanted carrier

Recommendation ITU-R S.523 gives the ITU-R criterion: αINth = 670 = 8.75% (for systems with frequency re-use).

## 8.1	Interference from a digital carrier

–	if B0  B1:                 1

then:

IN0 = INth ⋅ NthN0 = INth ⋅ B0 = 0.0857 B0 and B2 = B1

thus:

ΔTTN = 0.0857 ⋅ B0B2 = 0.0857 ⋅ B0B1

–	if B0  B1:                 B0 /B1

then:

IN0 = αINth ⋅ NthN0 1α = 0.0857 B0 ⋅ B1B0 = 0.0857 B1 and B2 = B1

hence:

ΔTTN = 0.0857 ⋅ B1B2 = 0.0857

## 8.2	Interference from an analogue carrier

–	if B0  B1:                 1

then:

IN0 = INth ⋅ NthN0 = 0.0857 B0 and ΔTTN = 0.0857 ⋅ B0B2

–	if B0  B1:

IN0 = αINth ⋅ NthN0 1α = 0.0857 ⋅ B0Bα and ΔTTN = 0.0857 ⋅ B0α ⋅ B2

# 9	FM-TV wanted carrier

In this case, the criterion is: 10 log C/ I  X dB, where X could be a variable. However, for the example given in Table 1, the value of X has been taken as 35 dB.

Thus:

IN0 = IC ⋅ CN0 = 1103.5 ⋅ Cα ⋅ N0

–	if B0  B1:                 1

ΔTTN = CN0 ⋅ 1103.5 ⋅ 1B2

–	if B0  B1:

ΔTTN = CN0 ⋅ 1103.5 ⋅ 1α ⋅ B2

For the example given in Table 1, according to Recommendation ITU-R S.567, the required S/N is 53 dB. Allowing 20% of the total noise to external interference, the following must be satisfied:

SNth ≥ 54 dB for 99% of the time.

The TV video signal-to-noise ratio after demodulation is given by:

10 log SNth = 10 log CN0 + 20 log r1 ⋅ ΔFFm − 10 log Fm3 + P + Q

F :	frequency deviation at low FM-TV signal frequencies (Hz)

Fm :	maximum baseband frequency (Hz) of the FM-TV signal

P :	pre-emphasis

Q :	weighting

r1 :	video-to-luminance signal ratio.

–	In the case of a 625/50 TV system:

Q  13.2 dB               P  11 dB               r1  0.714

P  Q  24.2 dB

thus:

C/N0  10(54 – KTV)/10

with:

KTV = P + Q + 10 log 3 r21 ⋅ ΔF2F3m

= 24.2 + 10 log 1.53 ⋅ ΔF2F3m

# 10	Conclusion

This method can be used for determining the need for coordination and as it reflects better than Appendix S8 to the Radio Regulations the actual interference situation, a certain number of coordination procedures might be eliminated.

Also, as a contribution to future planning efforts based on multilateral coordination, this method can provide a more precise means of determining mutual interference.

ANNEX  2

A power density-averaging bandwidth method of determining
interference between satellite networks

This method for determining the mutual interference between satellite networks is based on using available information to develop a worst-case power density versus averaging bandwidth (the bandwidth over which the power density is averaged) function from which the interference power in any interfered-with carrier bandwidth can be estimated. When the interfering power density corresponding to an interfered-with carrier is used in the calculations given in Recommendation ITU-R S.738, the result is the I/N for the interfered-with carrier which is also numerically equal to the T /T for that carrier bandwidth. Since the I represents an interfering carrier power of the same bandwidth as the interfered-with carrier, a C/I may also be computed by the method described in Annex 2 to Recommendation ITU-R S.740. Reasonably accurate interference estimates may be made with a minimal amount of data which can be used to determine if detail coordination is necessary. This method is described in detail in Annex 3 to Recommendation ITU-R S.740.

*	 	Radiocommunication Study Group 4 made editorial amendments to this Recommendation in 2001 in accordance with Resolution ITU-R 44 (RA-2000).