EARTH  EXPLORATION-SATELLITE  SYSTEMS  AND  METEOROLOGICAL  SYSTEMS

RECOMMENDATION  ITU-R  SA.1025-1*

PERFORMANCE  CRITERIA  FOR  SPACE-TO-EARTH  DATA  TRANSMISSION  SYSTEMS  OPERATING
IN  THE  EARTH  EXPLORATION-SATELLITE  AND  METEOROLOGICAL-SATELLITE  SERVICES
USING  SATELLITES  IN  LOW-EARTH  ORBIT

(Questions ITU-R 139/7 and ITU-R 141/7)

(1994-1995)

Rec. ITU-R SA.1025-1

The ITU Radiocommunication Assembly,

considering

a)	that the hypothetical reference system specified in Recommendation ITU-R SA.1020 defines space-to-Earth links for direct readout of data and acquisition of recorded data;

b)	that the performance objectives for these transmissions must be consistent with the attendant functional requirements and with the performance limitations associated with the systems and frequency bands in which the requirements will be fulfilled;

c) 	that performance objectives for representative systems operating in the Earth exploration- and meteorological-satellite services are intended to provide guidelines for the development of actual systems (see Note 1);

d)	that performance objectives may be determined using the methodology of Recommendation ITU-R SA.1021;

e)	that performance objectives for Earth exploration- and meteorological-satellite services are a prerequisite for the establishment of the associated interference criteria,

recommends

1	that space-to-Earth data transmissions in the Earth exploration- and meteorological-satellite services using satellites in low-Earth orbit have the performance objectives specified for the frequency bands in Table 1.

TABLE  1

Performance objectives for space-Earth links in the Earth exploration- and
meteorological-satellite services using satellites in low-Earth orbit

| Frequency band (MHz) | Satellite service | Modulation | Applicable elevation angles | Minimum C/N or maximum BER | Percentage of time | Function and type of earth station |
| --- | --- | --- | --- | --- | --- | --- |
| 137-138 | Meteorological | Analogue | ³ 25° | 12 dB C/N (1) | 99.9 | Direct data readout, low gain antenna |
|  | Meteorological | Digital | ³ 5° | 10–6 BER | 99.9 | Direct data readout, tracking antenna |
| 400.15-401 | Meteorological | Digital | ³ 5° | 10–6 BER | 99.9 | Direct data readout, low gain antenna |
| 1 670(2)-1 710 | Meteorological and | Digital | ³ 5° | 10–3 BER | 99.99 | Direct data readout and recorded data acquisition, |
|  | Earth exploration | Digital | ³ 5° | 10–6 BER | 99.9 | tracking antenna |
| 7 450-7 550 | Meteorological | Digital | ³ 5° | 10–3 BER | 99.99 | Recorded data acquisition, tracking antenna |
|  | Meteorological | Digital | ³ 5° | 10–6 BER | 99.9 |  |
|  |  | Digital | ³ 5° | 10–3 BER | 99.99 | Direct data readout and recorded data acquisition, tracking antenna |
| 8 025-8 400 | Earth exploration | Digital | ³ 5° | 10–6 BER | 99.9 | Recorded data acquisition, tracking antenna |
|  |  | Digital | ³ 5° | 10–5 BER | 99.0 | Direct data readout, tracking antenna |
| - (1) A 10 dB C/N is more appropriate for receivers constructed after the mid-1970s. - (2) The Earth exploration-satellite service is only allocated in the 1 690-1 710 MHz portion of the band. NOTE 1 – Performance objectives for specific systems may differ from the objectives presented in this Recommendation; however, the objectives defined herein are used as a basis for deriving permissible levels of interference that are the minimum of interference thresholds to be accepted by specific systems. NOTE 2 – The performance objectives associated with an availability of 99.99% of the time relate to the need to synchronize the receiver to the data transmission frames and to avoid bit slips within a frame. For the purposes of deriving interference criteria, these objectives can be assumed to be met if the objectives associated with lower availability levels are met. NOTE 3 – For the band 137-138 MHz, the elevation angle of 25° and other parameters for analogue receivers correspond with a level of performance that is guaranteed by designers of some systems. The parameters for the digital receivers correspond with user requirements. NOTE 4 – In all cases in Table 1, it is assumed that earth station sites are selected to yield average levels of environmental radio-frequency noise within the band. For direct data readout stations, which may be deployed in large numbers by various operating entities, there is a risk that randomly selected sites will exhibit higher than average levels of environmental noise (especially man-made noise) that may hamper the ability to achieve the stated performance objectives. However, the variance of this noise over all locations is not large in relation to receiver thermal noise, even at frequencies as low as 137-138 MHz, such that the performance objectives can generally be met at over 95% of the possible locations given link power margins of a few decibels. In the case of recorded data acquisition stations, sites are carefully selected to avoid ambient noise levels that exceed the average level. |  |  |  |  |  |  |

_________________

*	This Recommendation should be brought to the attention of the World Meteorological Organization (WMO).