RECOMMENDATION  ITU-R  SA.1029

INTERFERENCE  CRITERIA  FOR  SATELLITE  PASSIVE  REMOTE  SENSING

(Question ITU-R 140/7)

(1994)

Rec. ITU-R SA.1029

The ITU Radiocommunication Assembly,

considering

a)	that certain frequency bands have been allocated for spaceborne microwave remote sensing;

b)	that some of these bands are also allocated to other radio services;

c)	that performance criteria for satellite passive remote sensing are contained in Recommendation ITU-R SA.1028;

d)	that interference criteria should be compatible with performance objectives;

e)	that interference criteria are a necessary prerequisite to the establishment of sharing criteria;

f)	that interference criteria can be stated in terms of interference power within a reference bandwidth;

g)	that the sensitivities of radiometric passive sensors are generally expressed as a temperature differential, DTe, given by:

where:

a :	receiver system constant

Ts :	operating noise temperature (the sum of the receiver noise referred to the antenna terminal and that noise entering via the antenna) (K)

B :	receiver bandwidth (Hz)

t :	total time of observation (s);

h)	that the radiometer threshold, or minimum discernible power change, is given by:

W

where k is Boltzmann’s constant, 1.38 ´ 10-23 J/K,

recommends

1.	that harmful interference to passive sensors may occur when unwanted signal levels exceed 20% of DP;

2.	that permissible interference levels and reference bandwidths for the frequency bands preferred for passive sensing of the Earth's land, oceans and atmosphere are as follows:

| Frequency (GHz) | Permissible interference level (dBW) | Interference reference bandwidth (MHz) |
| --- | --- | --- |
| Near 1.4 | -171 | - 27 |
| - Near 2.7 | -174 | - 10 |
| - Near 5 | -161 | - 100 |
| - Near 6 | -164 | - 100 |
| - Near 11 | -163 | - 20 |
| - Near 15 | -166 | - 50 |
| - Near 18 | -155 | - 100 |
| - Near 21 | -163 | - 100 |
| - 22.235 | -160 | - 100 |
| - Near 24 | -163 | - 100 |
| - Near 30 | -163 | - 100 |
| - Near 37 | -156 | - 100 |
| - 50 to 66 | -161/-166(1) | - 100 |
| - Near 90 | -153 | - 200 |
| - Above 100 | -160 | - 200 |

- (1) Second number for pushbroom sensors.

3.	that, in shared frequency bands, the interference levels given above can be exceeded for less than 5% of measurement cells within a sensor’s service area in the case where the interference occurs randomly and for less than 1% of measurement cells in the case where the interference occurs systematically at the same locations;

4.	that, in shared frequency bands, the interference thresholds and percentages of loss for specific systems must be greater than or equal to the values recommended above.

_________________