RECOMMENDATION ITU-R  SA.1160

INTERFERENCE  CRITERIA  FOR  DATA  DISSEMINATION  AND  DIRECT  
DATA  READOUT  SYSTEMS  IN  THE  METEOROLOGICAL-SATELLITE  
SERVICE  USING  SATELLITES  IN  GEOSTATIONARY  ORBIT

(Question ITU-R 141/7)

(1995)

Rec. ITU-R SA.1160

The ITU Radiocommunication Assembly,

considering

a)	that interference criteria are needed to ensure that systems can be designed to achieve adequate performance in the presence of interference;

b)	that interference criteria may be determined using the methodology described in Recommendation ITU-R SA.1022 and the performance objectives listed in Recommendation ITU-R SA.1159;

c)	that interference criteria assist in the development of criteria for sharing bands among systems, including those operating in other services;

d)	that systems in the meteorological-satellite (METSAT) service must specify interference thresholds at levels greater than or equal to the permissible levels;

e)	that Annex 1 presents the parameters of representative systems that provide the basis for permissible levels of interference for pertinent transmissions in the METSAT service,

recommends

1	that the interference levels specified in Table 1 be used as the permissible total levels of interfering signal power at the antenna output of stations operating in the METSAT service.

TABLE  1

Interference criteria for stations in the METSAT service
using spacecraft in geostationary orbit

| Frequency band (MHz) | Function and type of earth station | Interfering signal power (dBW) in the reference bandwidth to be exceeded for no more than 20% of the time | Interfering signal power (dBW) in the reference bandwidth to be exceeded for no more than p% of the time |
| --- | --- | --- | --- |
|  | Direct data readout, high gain antenna | –150.7 dBW per 2.6 MHz | –150.1 dBW per 2.6 MHz p = 0.025 |
| 1 670-1 710 | Data dissemination, low gain antenna | –162.4 dBW per 4 kHz | –159.8 dBW per 4 kHz p = 0.025 |
|  | Data dissemination, high gain antenna | –145.3 dBW per 2.11 MHz | –144.7 dBW per 2.11 MHz p = 0.025 |
| NOTE 1 – The interfering signal powers (dBW) in the reference bandwidths are specified for reception at elevation angles ³ 3°. NOTE 2 – The total interfering signal power level that may be exceeded for no more than x% of the time, where x is less than 20% but greater than the specified short-term time percentage (p% of the time), may be determined by interpolation between the specified values using a logarithmic scale (base 10) for percentage of time and a linear scale for interfering signal power density (dB). NOTE 3 – Using the guidelines in Recommendation ITU-R SA.1022, the permissible levels of interference may be scaled for application to stations with antenna gain or bandwidth values that differ from the specified values. NOTE 4 – Although the interference criteria are based on the systems described in Annex 1, the interference criteria apply to all systems that operate in the subject frequency bands and which provide the specified service functions. |  |  |  |

ANNEX  1

Basis for interference criteria

This Annex presents the parameters used as inputs to the methodology described in Recommendation ITU-R SA.1022 to determine the interference criteria. Tables 2 and 3 summarize these parameters for different types of transmission.

TABLE  2

Downlink performance analysis used as a basis for interference criteria 
of stations operating with satellites in geostationary orbit

| Performance factor | Direct data readout |  | Data dissemination |  |  |
| --- | --- | --- | --- | --- | --- |
| Link function | Sensor data |  | High resolution data |  |  |
| Modulation type | QPSK | QPSK |  | BPSK | BPSK |
| Frequency range (MHz) | 1 670-1 710 | 1 670-1 710 |  | 1 670-1 710 | 1 670-1 710 |
| Time (%) | 0.1 | 20 |  | 0.1 | 20 |
| 1. Transmitter output power (dBW) | 3 | 3 |  | 10.7 | 10.7 |
| 2. Filter/cable line losses (dB) | 2.9 | 2.9 |  | 2.9 | 2.9 |
| 3. Modulation losses (dB) | 0.5 | 0.5 |  | 0.5 | 0.5 |
| 4. Antenna pointing error (dB) | 0 | 0 |  | 0 | 0 |
| 5. Transmitter antenna gain (dBi) | 16.5 | 16.5 |  | 16.5 | 16.5 |
| 6. Transmitter e.i.r.p. (dBW) | 16.1 | 16.1 |  | 23.8 | 23.8 |
| 7. Antenna elevation angle (degrees) | 3 | 3 |  | 3 | 3 |
| 8. Satellite altitude (km) | 35 880 | 35 880 |  | 35 880 | 35 880 |
| 9. Free space loss (dB) | 189.4 | 189.4 |  | 189.4 | 189.4 |
| 10. Excess path loss including rain attenuation (dB) | 3 | 0 |  | 1 | 0 |
| 11. Receiver antenna gain (dBi) | 45.1 | 45.1 |  | 39.5 | 39.5 |
| 12. Antenna pointing error (dB) | 0.5 | 0.5 |  | 0.5 | 0.5 |
| 13. Receiver line loss (dB) | 0 | 0 |  | 0 | 0 |
| 14. Polarization mismatch loss (dB) | 0.2 | 0.2 |  | 0.2 | 0.2 |
| 15. Demodulator implementation loss (dB) | 2.2 | 2.2 |  | 1.9 | 1.9 |
| 16. Data modulation loss (dB) | 1 | 1 |  | 0 | 0 |
| 17. Received signal power (dBW) | –135.1 | –132.1 |  | –129.7 | –128.7 |
| 18. Date rate (kbit/s) | 2 600 | 2 600 |  | 2 110 | 2 110 |
| 19. Reference bandwidth (kHz) | 2 600 | 2 600 |  | 2 110 | 2 110 |
| 20. Data rate (dB bit/s) | 64.1 | 64.1 |  | 63.2 | 63.2 |
| 21. Received energy/bit, Eb (dB(W/Hz)) | –199.2 | –196.2 |  | –193.0 | –192.0 |
| 22. Receiver system noise temperature (K) | 117.5 | 117.5 |  | 269 | 269 |
| 23. Receiver noise spectral density (dB(W/Hz)) | –207.9 | –207.9 |  | –204.3 | –204.3 |
| 24. Total system I + N power density (dB(W/Hz)) | –207.9 | –207.9 |  | –204.3 | –204.3 |
| 25. Eb /N0 (dB) | 8.7 | 11.7 |  | 11.4 | 12.4 |
| 26. Link bit-error ratio | 1 ´ 10–6 | 1 ´ 10–6 |  | 1 ´ 10–6 | 1 ´ 10–6 |
| 27. Satellite data storage/handling error ratio | --- | --- |  | --- | --- |
| 28. Total bit-error ratio | 1 ´ 10–6 | 1 ´ 10–6 |  | 1 ´ 10–6 | 1 ´ 10–6 |
| 29. Required Eb /N0 | 10.8 | 10.8 |  | 10.8 | 10.8 |
| 30. Margin | –2.1 | 0.9 |  | 0.6 | 1.6 |
| Long-term or short-term margin (dB) | 0.9 | –2.1 |  | 1.6 | 0.6 |
| Interference power (dBW) | –150.1 | –150.7 |  | –144.7 | –145.3 |

TABLE  3

Downlink performance analysis used as a basis for interference criteria of small 
earth stations operating with satellites in geostationary orbit

| Performance factor | Data dissemination |  |
| --- | --- | --- |
| Link function | WEFAX |  |
| Modulation type | FM | FM |
| Frequency range (MHz) | 1 670-1 710 | 1 670-1 710 |
| Time (%) | 0.1 | 20 |
| 1. Transmitter output power (dBW) | 6.7 | 6.7 |
| 2. Filter/cable line losses (dB) | 2.9 | 2.9 |
| 3. Modulation losses (dB) | 0 | 0 |
| 4. Antenna pointing error (dB) | 0 | 0 |
| 5. Transmitter antenna gain (dBi) | 15.0 | 15.0 |
| 6. Transmitter e.i.r.p. (dBW) | 18.8 | 18.8 |
| 7. Antenna elevation angle (degrees) | 3 | 3 |
| 8. Satellite altitude (km) | 35 880 | 35 880 |
| 9. Free space loss (dB) | 189.4 | 189.4 |
| 10. Excess path loss including rain attenuation (dB) | 0.4 | 0 |
| 11. Receiver antenna gain (dBi) | 25.6 | 25.6 |
| 12. Antenna pointing error (dB) | 0 | 0 |
| 13. Receiver line loss (dB) | 2 | 2 |
| 14. Polarization mismatch loss (dB) | 0.2 | 0.2 |
| 15. Demodulator implementation loss (dB) | 0 | 0 |
| 16. Data modulation loss (dB) | 0 | 0 |
| 17. Received signal power (dBW) | –147.6 | –147.2 |
| 18. Necessary bandwidth (kHz) | 18 | 18 |
| 19. Reference bandwidth (kHz) | 4 | 4 |
| 20. Signal bandwidth (dB/Hz) | 36.0 | 36.0 |
| 21. Received power density C0 (dB(W/Hz)) | –183.6 | –183.2 |
| 22. Receiver system noise temperature (K) | 1 585 | 1 584 |
| 23. Receiver noise spectral density (dB(W/Hz)) | –196.6 | –196.6 |
| 24. Total systeme I + N power density (dB(W/Hz)) | –196.6 | –196.6 |
| 25. C0 /N0 (dB) | 13.0 | 13.4 |
| 26. Link bit error ratio | --- | --- |
| 27. Satellite data storage/handling error ratio | --- | --- |
| 28. Total bit-error ratio | --- | --- |
| 29. Required C0 /N0 | 10 | 10 |
| 30. Margin | 3.0 | 3.4 |
| Long-term or short-term margin (dB) | 3.4 | 3.0 |
| Interference power (dBW) | –159.8 | –162.4 |

_________________