RECOMMENDATION  ITU-R  SA.1165

TECHNICAL  CHARACTERISTICS  AND  PERFORMANCE  CRITERIA  FOR  
RADIOSONDE  SYSTEMS  IN  THE  METEOROLOGICAL  AIDS  SERVICE

(Question ITU-R 144/7)

(1995)

Rec. ITU-R SA.1165

The ITU Radiocommunication Assembly,

considering

a)	that upper-air meteorological measurements carried out by radiosondes are an essential element of the World Weather Watch Programme of the World Meteorological Organization (WMO);

b)	that radiosonde systems operating in the meteorological aids service have unique radiocommunication requirements;

c)	that these requirements affect assignments and other regulatory matters;

d)	that radiosondes in the meteorological aids service are flown on balloons and rockets and may operate with stations located on land or ships;

e)	that other types of radiosondes in the meteorological aids service are dropped from aircraft and operate with stations located on aircraft;

f)	that performance objectives for transmissions to and from radiosondes must be consistent with the attendant functional requirements and with the performance limitations associated with the systems and frequency bands in which the requirements will be fulfilled;

g)	that performance objectives for representative systems operating in the meteorological aids service are intended to provide guidelines for the development of actual systems that must operate in a frequency sharing environment;

h)	that performance objectives for particular systems may be determined using the methodology similar to that described in Recommendation ITU-R SA.1021;

j)	that performance objectives are a prerequisite for the determination of interference criteria,

recommends

1	that the characteristics described in Annex 1 be taken into account in connection with frequency assignments and other technical and operational regulatory matters concerning meteorological aids systems and their interaction with other services;

2	that meteorological aids systems have the performance objectives specified in Table 1.

TABLE  1

Performance objectives for links in the meteorological-aids service*

| Frequency band (MHz) | Function and transmitter platform and antenna | Modulation and bandwidth | Applicable elevation angles (degrees) | Minimum C / N (dB) | Required time availability (%) | Receiver platform and antenna |
| --- | --- | --- | --- | --- | --- | --- |
| 400.15-406 | Command and ranging, balloon-borne radiosonde | FM 280-480 kHz | ³ 3.0 | 12 | 99.9 | Earth-based receiver |
|  | Radiosonde dropped from aircraft | FM 480 kHz | Not applicable | 12 | 99.9 | Aircraft receiver |
| 1 668.4-1 700 | Direct data readout from balloon-borne radiosonde | AM To be determined | ³ 3.0 | 12 | 99.9(1) | Earth-based receiver, high gain tracking antenna |
|  | Direct data readout from balloon-borne radiosonde | FM 180 kHz | ³ 3.0 | 12 | 99.9 | Earth-based receiver, high gain tracking antenna |
| * Performance objectives for specific systems may differ from the objectives presented in this Recommendation; however, the objectives defined herein are used as a basis for deriving permissible levels of interference that are the minimum interference thresholds to be accepted by specific systems. (1) The system availability objective of 99.9% of the time (per flight) is specified in the context of overall reliability levels of about 97% per flight and 99.9% if a second release is available, respectively. The required reliability is fulfilled by launching a second radiosonde in cases where inadequate data are obtained from the initial flight. The specified radio path availability pertains to an individual flight. |  |  |  |  |  |  |

ANNEX  1

# 1	Introduction

Radiosondes are used for insitu upper-air measurements of meteorological parameters of the atmosphere up to an altitude of 35 km. They are also used for measuring other chemical and physical parameters of the atmosphere.

Radiosonde observations are performed by radiosondes borne by ascending balloons launched from land stations or ships, or by dropsondes with a parachute dropped from aircrafts. Radiosonde observations are carried out routinely by almost all countries, two to four times a day, in the framework of the World Weather Watch Programme of WMO for operational meteorology (weather analysis, warnings and forecasts). Other radiosonde observations are carried out to meet special national and international meteorological or environmental requirements.

Presently the use of the 400.15-406 MHz frequency band for radiosonde observations is very extensive, particularly in Europe where this frequency band is congested and can hardly accommodate any further radiosonde operation.

Information derived from the records of the WMO show that the 1 668.4-1 700 MHz frequency band is at present, used in many Caribbean and Pacific Islands, Antarctica, Bangladesh, Brazil, India, Indonesia, Japan, South Africa, Turkey and the United States of America as well as by some defence related operators. All known radiosonde systems in this band use directional antennas in reception.

# 2	Characteristics of radiosonde operation

Radiosondes are used for measuring pressure, temperature and humidity up to an altitude of 30 km or more, and also for upper-wind determination, with the complementary use of navigational aids systems. They are also used to measure solar and terrestrial radiation, electrical potential gradient, conductivity of the air, air-Earth current density and radioactivity in the free atmosphere, as well as concentration of pollutants. Radiosoundings are used to perform upper-air Ozone (O3) observations to monitor the status of the planetary Ozone layer, and are an essential tool for monitoring the global climate.

Radiosoundings are used to collect data for special meteorological forecasts in the case of hazardous environmental conditions; smog, pollution transportation, and leaks of radioactive or chemical gases and particles, and also for scientific research of special weather phenomena (e.g. winter storms, hurricanes, thunderstorms, etc.).

Meteorological radiosonde observations are carried out worldwide to provide synoptic meteorological observational data which is exchanged in real time, needed for the global weather analysis and forecasting supported by numerical models. The standard synoptic observations shall be done at 1200 h and 0000 h UTC, and intermediate observations at 0600 h and 1800 h. Radiosonde networks are implemented and operated by national meteorological services in compliance with recommended practices and procedures internationally agreed upon by WMO. The current implementation includes 940 land stations and tens of ships with radiosonde launching facilities. About 1 200 000 radiosoundings are yearly made, 800 000 observations being made for synoptic purposes, the rest for special use. About 20% of these radiosoundings are performed with radiosondes operating in the 1 680 MHz band. The current level of implementation is far from adequately meeting the meteorological requirements. The performance objective achievable by the year 2005, as set in the WMO Manual on the Global Observing System, includes radiosonde observations with a horizontal resolution of 250 km, four times a day.

# 3	Operational requirements

Apart from accuracy, the chief features required in radiosonde design are reliability, robustness, small weight, small bulk and small power consumption. Since a radiosonde is generally used only once, it should be designed for production at low cost. Ease and stability of calibration are also important factors. A radiosonde should be capable of providing data over a range of at least 200 km and operating in a temperature range from –90° to + 60 °C. Since the voltage of a battery

varies with both time and temperature, the radiosonde must be designed to accept the variations without exceeding the accuracy and radio frequency drift requirements. The associated ground equipment should not be unduly complicated or require frequent highly skilled maintenance. It is preferable, however, to keep the radiosonde itself as simple as possible, even at the expense of complication in the ground equipment, since failure of the latter is more readily corrected and since the costs of flight equipment should be kept to a minimum.

The major limitation of the radiosonde observation is the cost. In OECD countries one radiosonde observation costs about 400 ECU, from the total costs about one fourth is the cost of the radiosonde. The cost structure is noteworthy different in developing countries, where the cost of the radiosonde is the most important element. As meteorological data requirements are global, some developed countries are donating systems and part of the radiosondes for some developing countries with a view to sustained upper-air observations. There is therefore a strong requirement for keeping the radiosonde price as low as possible in order to ensure the continuation of the observations vital to operational meteorology, including its aspects related to the protection of life. In the total cost of the radiosonde the sensors and wind finding represent the major part, whereas the transmitters are intentionally kept as simple as possible, hence maintaining a low total price. Transmitter costs represent about 15-35% of the current radiosonde electronic equipment costs.

The ascent time of a full radiosounding is about 90 min, and the descent time is about half of the ascent time if a parachute is used. The radiosonde is usually still transmitting while descending. The maximum range of the proper reception of the radiosonde is about 200 km. The ascent speed is about 5 m/s and the trajectory depends on the prevailing wind conditions. In general, within an area of radius about 400 km around the radiosounding station the same downlink frequency cannot be used. In high density areas, more than ten radiosonde operators are located within the effective area of one radiosonde.

In Western and Northern European areas the radiosounding station network is dense. In addition meteorological and environmental monitoring and research activities and defense force related users are sharing the frequency band with the synoptic observations. Coordination between radiosonde operators is required to prevent interference between the radiosondes of different stations.

# 4	Future trends

The radiosonde services are provided on two frequency bands: 400.15-406 MHz and 1 668.4-1 700 MHz. There are important reasons to use both frequency bands for the radiosonde services.

## 4.1	400.15-406 MHz frequency band

The 400.15-406 MHz band is used for observations where the wind is determined using navigational aids radio transmitter networks (NAVAID). The networks and transmitters are: Omega, Alpha (Russian), Communications VLF and Loran-C. Because most of the Omega and Alpha services will be terminated within 10 years, the use of other systems such as GPS should be considered in the future. Loran-C is not worldwide, and the use of the Communications VLF only is questionable at the moment. Technically this band is more important since GPS C/A signals are transmitted on 1 575 MHz, and it may not be compatible with the 1 680 MHz radiosonde frequency. A 400 MHz radiotheodolite is technically and economically not feasible because of large dimensions of the antenna.

## 4.2	1 668.4-1 700 MHz frequency band

The 1 668.4-1 700 MHz band is important for the RDF (radiotheodolite, radio direction finder) based wind observations. The use of RDF wind observations facilitates a simple and less expensive design of radiosonde. Independence of the GPS operator is also usually required by defense related radiosonde users.

Future radiosonde systems need to be designed to improve their compatibility with other systems using a common frequency band.

# 5	Radiocommunication characteristics of current radiosonde systems

Radiosonde systems perform insitu measurement of the atmospheric pressure, temperature and relative humidity (PTU). The wind speed and direction is determined using either a NAVAID (navigational aids) method or (RDF) radio direction finding measuring the azimuth and elevation angle of the radiosonde in respect of the receiving antenna. The data downlink from the radiosonde to the ground receiving station is at the 400 MHz band in NAVAID, and at the 1 680 MHz band in RDF radiosondes. The Omega VLF and Loran-C signals are relayed to the ground receiver using a simple translator.

For 1 680 MHz radiosondes, signal losses of greater than 10 s will generally cause loss of ground receiver tracking. A radiosonde whose track is lost will only rarely be reacquired so track is lost even if the interfering signal goes away. The radio receiver will track the strongest amplitude signal in its instantaneous bandwidth (1.3 MHz). Wind velocity is sometimes measured using a transponding radiosonde. The radiosonde receives a signal near 400 MHz from the ground station and retransmits the pulse train at 1 680 MHz.

The radiosonde transmitting antennas are quarter wave length monopoles.

## 5.1	Transmitter characteristics

| 400 MHz radiosonde transmitters |  |
| --- | --- |
| Tuning range (MHz) | 400.15-406 |
| Maximum drift in flight (kHz) | ± 800 |
| Nominal output power (mW) | 200 |
| ITU-R emission type | F9D |
| Modulation | FM |
| Modulating PTU signal (kHz) | 7-10 |
| Deviation (kHz) | 45 ± 15 |
| Deviation caused by the VLF/Loran-C signal relay link (kHz) | 100/300 |
| Occupied bandwidth with Omega VLF (kHz) | 280 |
| Occupied bandwidth with Loran-C (kHz) | 480 |
| Occupied bandwidth with GPS | To be determined |
| Equivalent information rate | To be determined (1) |
| Out-of-band emission | To be determined |
| - (1) The information transmission rate is intended to indicate the actual data rate transferred from the radiosonde to the ground receiver. Because of the current modulation techniques used by radiosonde systems, further study is needed to estimate these values.. |  |

## 5.2	Remark

For reasons of these technical characteristics and civil and military requirements (e.g. simultaneous radiosounding hours, nearness of radiosounding stations, relaunch radiosonde, simultaneous twice or triple radiosoundings ...) the band 400.15-406 MHz dedicated to radiosoundings must be fully maintained for the meteorological aids service.

| 1 680 MHz radiosonde transmitters |  |  |
| --- | --- | --- |
| Tuning range (MHz) | 1 665-1 700 | 1 665-1 700 |
| Maximum drift in flight (Mz) | ± 4 | ± 4 |
| Nominal output power (mW) | 330 | 330 |
| ITU-R emission type | A9D | F9D |
| Modulation | MA, 100% | FM |
| Modulating PTU signal (kHz) | 7-10 | 7-10 |
| Deviation | Not applicable | 45 ± 15 |
| Wind finding | Detection of the reception angles | Detection of the reception angles |
| Occupied bandwidth (kHz) | To be determined | 180 |
| Information rate | To be determined | To be determined |
| Out-of-band emission | To be determined | To be determined |

## 5.3	Receiving antennas

Typical characteristics of currently used antennas are:

| Type | Omnidirectional (dipole, ground plane) | Directional corner reflector, six corners |
| --- | --- | --- |
| Frequency range (MHz) | 397-409 | 400-406 |
| Horizontal gain (dB) | Omnidirectional | 8 |
| Vertical gain (dB) | Omnidirectional | –3 |
| Amplifier NF (dB) | < 3.5 | < 2.5 |
| Amplifier gain (dB) | 13 | 20 |

| Type | Phased array | Conical scanned |
| --- | --- | --- |
| Frequency range (MHz) | 1 660-1 700 | 1 660-1 700 |
| 3 dB beamwidth (degrees) | 20 horizontal | 8.8 |
|  | 15 vertical |  |
| Gain (dBi) | 16 minimum | 28 |
| Side lobe attenuation (dB) | > 20 to the direction of specular ground reflection on flat terrain when elevation > 14° | 15 at ± 60° from boresight |
| Sensitivity (dBm) | –110 RF input 12 dB S + N/N | – 97 (tracking) |
| Automatic gain control (dBm) | Dynamic range –110 ... 0 |  |
| IF bandwidth, PTU measure (kHz) - Tracking (MHz) | 300 Not applicable | 180 1.3 |

_________________