RECOMMENDATION  ITU-R  SA.1258

SHARING  OF  THE  FREQUENCY  BAND  401-403  MHz  BETWEEN  THE  
METEOROLOGICAL  SATELLITE  SERVICE,  EARTH  EXPLORATION  SATELLITE  
SERVICE  AND  METEOROLOGICAL  AIDS  SERVICE

(Question ITU-R 217/7)

(1997)

Rec. ITU-R SA.1258

The ITU Radiocommunication Assembly,

considering

a)	that the World Administrative Radio Conference for Dealing with Frequency Allocations in Certain Parts of the Spectrum (Malaga-Torremolinos, 1992) (WARC-92) adopted Resolution No. 710 to upgrade the meteorological-satellite (METSAT) and the Earth exploration-satellite (EES) services to primary status in the band 401-403 MHz;

b)	that the band 400.15-406 MHz is currently allocated to the meteorological aids (MetAids) service on a primary basis;

c)	that METSAT and EES are currently allocated on a secondary basis worldwide in the sub-band 401-403 MHz;

d)	that the sub-band is used for the operation of data collection systems (DCS) on METSATs;

e)	that data collected by MetAids operations are vital to the prediction of weather world-wide which threatens life and property;

f)	sharing between METSAT, EES and MetAids has been successful due to careful coordination between the two services,

recognizing

1	that any reduction in spectrum available to MetAids in order to accommodate other services will result in higher levels of interference from MetAids to EES and METSATs;

2	that changes to the EES and METSAT allocation in the band 401-403 MHz may impact MetAids operations if careful coordination between the services is not ensured;

3	that achievable distance and/or frequency separation can be used to ensure that geostationary-satellite orbit (GSO) EES and METSAT operations do not interfere with MetAids operations;

4	that although simultaneous co-channel sharing is not possible between non-GSO EES, METSAT and MetAids, coordination in frequency, time, and distance can be accomplished,

recommends

1	that sharing is feasible between GSO EES, METSATs, and MetAids in the band 401-403 MHz, provided that the data collection platforms meet the following conditions: uplink e.i.r.p. density limitations of 22 dB(W/100 Hz) and transmission durations of up to 1 min per message;

2	that sharing is feasible between non-GSO EES, METSATs, and MetAids in the band 401-403 MHz, provided that the data collection platforms meet the following conditions: uplink e.i.r.p. density limitations of 7 dB(W/1 600 Hz) and transmission durations of up to 1 min per message with a duty cycle of less than 5%;

3	that systems which do not meet the restrictions of recommends 1 and 2 require coordination between MetAids operators, METSAT and EES operators prior to implementation.