RECOMMENDATION  ITU-R  SA.1414

CHARACTERISTICS  OF  DATA  RELAY  SATELLITE  SYSTEMS

(Questions ITU-R 117/7, ITU-R 118/7 and ITU-R 135/7)

(1999)

Rec. ITU-R SA.1414

The ITU Radiocommunication Assembly,

considering

a)	that data relay satellite (DRS) systems operate as described in Recommendation ITU-R SA.1018 – Hypothetical reference system for systems comprising data relay satellites in the geostationary orbit and user spacecraft in low-Earth orbits;

b)	that there is an increase in mission requirements and in space research activity conducted particularly in low-Earth orbit;

c)	that DRS provide support to many programmes/missions in the space research service and are vital to supporting both manned and unmanned space research telecommunications;

d)	that it is necessary to establish relevant criteria for sharing between DRS systems and other services operating in co-frequency bands;

e)	that the technical characteristics of representative DRS systems need to be considered in order to derive relevant sharing criteria,

recommends

1	that the characteristics of DRS systems, as described in Annex 1, may be used in interference sharing studies;

2	that the information provided in Annex 1 also be used as a guidance for deriving sharing criteria and coordination thresholds as appropriate for DRS systems.

ANNEX  1

TABLE  1

Forward Earth-to-DRS feeder link characteristics

| Transmitting earth station |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Network | Russia |  | United States of America | Europe |  | Japan |  |
| Location | Russia |  | United States of America(1) | Europe |  | Japan |  |
| Frequency range (GHz) | 14.5-15.34 selectable |  | 14.5-15.34 selectable | 28.6-29.8 selectable |  | 29.5-31 selectable |  |
| Link description | Composite(2) |  | Composite(2) | Decentralized(3) |  | Decentralized(3), (4) |  |
| Transmission rate |  256 kbit/s |  |  51Mbit/s |  10 Mbit/s |  |  50 Mbit/s |  |
| Modulation | FM(TV), PSK |  | PSK | PSK |  |  |  |
| Polarization | Circular |  | Linear | Linear |  | Circular |  |
| Antenna size (m) | 16 |  | 18.3 | 3 and 9 |  | 5, 9.2 and 13 |  |
| Tx antenna gain (dBi) | 62 |  | 66.4 | 57.6 and 67.6 |  | 63, 68.2 and 71.4 |  |
| Tx antenna radiation pattern | RR Appendix S7 |  |  |  |  |  |  |
| Necessary bandwidth (MHz) |  50.5 |  | 650 (composite) |  100 |  |  978 (composite) |  |
| Maximum power density (dB(W/Hz)) | –56 |  | –58 | –38 |  | –32.5 |  |
| Maximum e.i.r.p. density (dB(W/Hz)) | 4 |  | 8.4 | 19.6 |  | 38.9 |  |
| Receiving DRS |  |  |  |  |  |  |  |
| Orbital locations | Rec. ITU-R SA.1275 or Rec. ITU-R SA.1276 |  |  |  |  |  |  |
| Antenna size (m) | 1.6 | 2.2 |  |  | 0.4 |  | 2.0 |
| Rx antenna gain (dBi) | 43 | 47 |  |  | 40.2 |  | 53 |
| Rx antenna radiation pattern | Rec. ITU-R S.672 |  |  |  |  |  |  |
| System noise temperature (K) | 1 500 | 2 537 |  |  | 1 305 |  | 890 and 579 |
| Link availability (%) | 99.99 |  |  |  | 99.9 |  | 99.9 |
| Interference criterion | Rec. ITU-R SA.1155 |  |  |  |  |  |  |
| (1) The earth station for the United States of America network is located in White Sands (New Mexico). The coordinates of the station are: 32.5N, 106.60 W. (2) The composite link for the Russian network comprises one TV analogue channel and four sub-carriers to transmit digitally, a voice channel, a ranging channel and two data channels. The composite link for the United States of America network is composed of seven channels: One DRS command and ranging channel, one DRS pilot tone signal, one S-band (2 GHz) multiple access (S-MA) link, two S-band single access (S-SA) links and two Ku-band (14/11 GHz and 30/20 GHz) single access (K-SA) links. (3) The European DRS ground system consists of 12 earth stations, including the TT&C earth station, located in different countries within Europe. The earth station communicates with the DRS through its European coverage antenna. (4) The Japanese network employs a decentralized link concept that permits independent forward feeder links from different earth stations |  |  |  |  |  |  |  |

TABLE  2

Forward DRS-to-spacecraft link characteristics

| Transmitting DRS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Network | United States of America | Europe | Japan | United States of America | Russia | United States of America | Europe | Japan | United States of America |
| Orbital locations | Rec. ITU-R SA.1275 or Rec. ITU-R SA.1276 |  |  |  |  |  |  |  |  |
| Frequency range (GHz) | 2 ~ 2.1 | 2.025-2.11(1) |  |  | 13.4-13.8 |  | 22.55-23.55 |  |  |
| Link description | Multiple access (S-MA) link | Single access (S-SA) link |  |  | Single Access (Ku-SA) link |  | Single Access (Ka-SA) link |  |  |
| Transmission rate (bit/s) |  300 kbit/s |  1 Mbit/s |  6 Mbit/s |  300 kbit/s |  256 kbit/s |  25 Mbit/s (US) |  10 Mbit/s |  50 Mbit/s |  25 Mbit/s |
| Modulation | SQPN/PSK(2) |  |  |  | FM(TV), PSK | PSK |  |  |  |
| Polarization | Circular |  |  |  |  |  |  |  |  |
| Antenna size (m) | Phased array | 2.8 | 3.6 | 4.9 | 3 | 4.9 | 2.8 | 3.6 | 4.9 |
| Tx antenna gain (dBi) | 23 | 34 | 36.4 | 36.6 | 49 | 53.8 | 53.4 | 57.4 | 58.9 |
| Tx antenna radiation pattern |  | Rec. ITU-R S.672 |  |  |  |  |  |  |  |
| Necessary bandwidth (MHz) |  6 |  6 | 30 | 20 |  50.5 | 50 | 60 |  150 | 50 |
| Maximum power density (dB(W/Hz)) | –54 | –54.7 | – 44.5 | – 64 | – 67 | – 67.7 | – 65.1 | – 49.5 | – 64.5 |
| Maximum e.i.r.p. density (dB(W/Hz)) | –31 (US) | –20.7 | – 8.1 | –24.7 | –18 | –13.9 | –11.7 | –7.9 | –5.6 |

TABLE  2 (continued)

| Receiving spacecraft |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Network | United States of America | Europe | Japan | United States of America | Russia | United States of America |  | Europe | Japan |  | United States of America |  |
| Orbital locations | Mainly low-Earth orbit |  |  |  |  |  |  |  |  |  |  |  |
| Frequency range (GHz) | 2 ~ 2.1 | 2.025-2.11(1) |  |  | 13.4-13.8 |  |  | 22.55-23.55 |  |  |  |  |
| Antenna size (m) | Omnidirectional, arrays | Omnidirectional, arrays, parabolic   1.5 |  |  |  1.5 |  |  1.3 |  |  |  |  |  1.3 |
| Rx antenna gain (dBi) |  1.5 |  27.3 |  27.1 |  27.3 |  44 |  |  47 |  |  48.9 |  |  |  47 |
| Rx antenna radiation pattern |  | Rec. ITU-R S.672 for high gain antenna |  |  | Rec. ITU-R S.672 |  |  |  |  |  |  |  |
| System noise temperature (K) |  |  |  |  |   |  |   |  |  |  |  |   |
| Required Eb/N0 (dB) | ~10 | 9.5 | 10.5 | 9.5 | 9.5 |  | 9.5 |  | 10.8 |  |  | 9.5 |
| Required BER | 1  10–5 | 1  10–6 |  |  |  |  |  |  |  |  |  |  |
| Link reliability (%) | 99.99 | 99.9 |  | 99.99 |  |  | 99.9 |  |  |  |  | 99.99 |
| Interference criterion | Rec. ITU-R SA.1155 |  |  |  |  |  |  |  |  |  |  |  |
| SQPN: Staggered quadriphase pseudo-random noise (1) Transmit frequency is selectable in 5 MHz steps. (2) Signals with low data rate transmissions will be spread by a pseudo-random noise code so as to meet pfd limits. |  |  |  |  |  |  |  |  |  |  |  |  |

TABLE  3

Return spacecraft-to-DRS link characteristics

| Transmitting spacecraft |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Network | United States of America | Europe | Japan | United States of America | Russia | United States of America | Europe | Japan | United States of America |
| Orbital locations | Mainly low-Earth orbit |  |  |  |  |  |  |  |  |
| Frequency range (GHz) | 2.285-2.290 | 2.200-2.290(1) |  |  | 14.76-15.34 |  | 25.25-27.50 |  |  |
| Link description | Multiple access (S-MA) link | Single access (S-SA) link |  |  | Single access (Ku-SA) link |  | Single access (Ka-SA) link |  |  |
| Transmission rate |  3 Mbit/s |  1 Mbit/s |  12 Mbit/s |  6 Mbit/s |  512 kbit/s |  300 Mbit/s | 150 Mbit/s |  300 Mbit/s |  800 Mbit/s |
| Modulation | SQPN/PSK(2) |  |  |  | FM, PSK | PSK |  |  |  |
| Polarization | Circular |  |  |  |  |  |  |  |  |
| Antenna size (m) | Omnidirectional, arrays | Omnidirectional, arrays, parabolic   1.5 |  |  |  1.5 |  |  |  1.9 |  1.5 |
| Tx antenna gain (dBi) |  15 |  27.3 |  27.6 |  27.3 |  43 |  |  47 |  49.7 |  47 |
| Tx antenna radiation pattern |  | Rec. ITU-R S.672 for high gain antenna |  |  | Rec. ITU-R S.672 |  |  |  |  |
| Necessary bandwidth (MHz) | 6 |  6 | 20 | 12 |  50.5 |  225 |  300 |  300 |  300 |
| Maximum power density (dB(W/Hz)) | –34 | –51 | –55.7 | –38.6 |  | –19 | – 65.1 | –58.8 |  |
| Maximum e.i.r.p. density (dB(W/Hz)) | Compliant with pfd limits |  |  |  |  |  | –23 | –9.1 |  |

TABLE  3 (continued)

| Receiving DRS |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Network | United States of America | Europe | Japan |  | United States of America | Russia | United States of America | Europe |  | Japan | United States of America |
| Orbital locations | Recs. ITU-R SA.1275 or ITU-R SA.1276 |  |  |  |  |  |  |  |  |  |  |
| Frequency range (GHz) | 2.285-2.290 | 2.200-2.290(1) |  |  |  | 14.76-15.34 |  | 25.25-27.50 |  |  |  |
| Antenna size (m) | Phased array | 2.8 | 3.6 |  | 4.9 | 3 | 4.9 | 2.8 |  | 3.6 | 4.9 |
| Rx antenna gain (dBi) | 29 | 34.7 |  | 37.2 | 36 | 49 | 53.3 | 53.4 | 58.8 |  | 58 |
| Rx antenna radiation pattern |  | Rec. ITU-R S.672 |  |  |  |  |  |  |  |  |  |
| System noise temperature (K) |  |  |  |  |  | 680 | 907 |   |  |  |  |
| Link reliability (%) | 99.99 |  |  | 99.9 | 99.99 |  |  | 99.9 |  |  | 99.99 |
| Interference criterion | Rec. ITU-R SA.1155 |  |  |  |  |  |  |  |  |  |  |
| (1) Transmit frequency is selectable in 5 MHz steps, 100 kHz steps for Japanese DRS. (2) Signals with low data rate transmissions will be spread by a pseudo-random noise code so as to meet pfd limits. |  |  |  |  |  |  |  |  |  |  |  |

TABLE  4

Return DRS-to-Earth feeder link characteristics

| Transmitting DRS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Network | Russia |  | United States of America | Europe |  | Japan |  |
| Orbital locations | Recs. ITU-R SA.1275 or ITU-R SA.1276 |  |  |  |  |  |  |
| Frequency range (GHz) | 10.7-10.95 |  | 13.4-14.05 | 18.1-21.2 |  | 19.7-21.2 |  |
| Link description | Ku-band (14/11 GHz) return feeder |  |  | Ka-band (30/20 GHz) return feeder |  |  |  |
| Transmission rate (kbit/s) |  512 |  |  |  |  |  |  |
| Modulation | FM, PSK |  | PSK |  |  | SQPN/PSK |  |
| Polarization | Circular |  | Linear |  |  | Circular |  |
| Antenna size (m) | 1.6 |  | 2 | 0.4 |  | 2.0 |  |
| Tx antenna gain (dBi) | 43 |  | 46.1 | 40.2 |  | 49.5 |  |
| Tx antenna radiation pattern | Rec. ITU-R S.672 |  |  |  |  |  |  |
| Necessary bandwidth (MHz) |  50.5 |  | 650 (composite), 225 (dedicated) |  300 |  | 839 |  |
| Maximum power density (dB(W/Hz)) | – 66 |  | –57 | – 61 |  | – 40.9 |  |
| Maximum e.i.r.p. density (dB(W/Hz)) | –23 |  | –10.9 | –20.8 |  | 8.6 |  |
| Receiving earth station |  |  |  |  |  |  |  |
| Location | Russia | United States of America |  | Europe |  | Japan |  |
| Antenna size (m) | 16 | 18.3 |  |  | 3, 9 |  | 5, 9.2 and 13 |
| Rx antenna gain (dBi) | 62 | 65.5 |  |  | 54, 63.9 |  | 59.5, 67.7 |
| Rx antenna radiation pattern | RR Appendix S7 |  |  |  |  |  |  |
| System noise temperature (K) | 160 | 300 |  |  | 795 |  | 200 |
| Link availability (%) | 99.99 |  |  |  | 99.9 |  |  |
| Interference criterion | Rec. ITU-R SA.1155 |  |  |  |  |  |  |
| (1) The United States of America DRS transmits a dedicated and a composite link. Transmission rate for the dedicated link is 300 Mbit/s, for the composite link the transmission rate is on the order of 630 Mbit/s. (2) The European and Japanese networks employ a decentralized link concept that permits independent return feeder links to different earth station. |  |  |  |  |  |  |  |