RECOMMENDATION  ITU-R  SF.1193

CARRIER-TO-INTERFERENCE  CALCULATIONS  BETWEEN  EARTH  STATIONS
IN  THE  FIXED-SATELLITE  SERVICE  AND  RADIO-RELAY  SYSTEMS

(1995)

Rec. ITU-R SF.1193

The ITU Radiocommunication Assembly,

considering

a)	that during the planning stage of a satellite network or a radio-relay system it may be useful to calculate carrier-to-interference (C/I) ratios between carriers of the interfering and interfered-with networks for the purpose of determining levels of interference;

b)	that the C/I ratios can be used in the determination of interference levels contributing to performance degradation;

c)	that the C/I ratios can be used for all modulation methods and signal types;

d)	that the C/I calculation may be an element to complete coordination between satellite networks and radio-relay systems in accordance with the Radio Regulations;

e)	that Recommendation ITU-R S.741 describes C/I calculations between networks in the fixed-satellite service (FSS),

recommends

1	that when carrier-to-interference ratios (C/Is) are used to calculate the levels of interference between FSS earth stations and radio-relay systems the methods given in Annex 1 should be used to calculate the interference power in the interfered-with carrier.

ANNEX  1

Determination of carrier-to-interference ratios and baseband noise power
for coordination of carriers between FSS earth stations
and radio-relay systems

# 1	Types of interfering carriers

1.1	The types of interference experienced by FSS carriers from radio-relay systems can be classified as follows:

–	noise-like interference;

–	interference arising from a TV-FM carrier not employing energy dispersal;

–	interference from one TV-FM carrier to another (this category is not addressed in this Annex).

1.2	The types of interference experienced by radio-relay carriers from FSS networks can be classified as follows:

–	noise-like interference;

–	slowly swept interference arising from a TV-FM carrier modulated only with an energy dispersal (ED) signal;

–	interference from one TV-FM carrier to another (this category is not addressed in this Annex).

# 2	Types of FSS and FS carriers

2.1	For the types of carriers normally used in FSS networks, see § 2.1 of Annex 1 to Recommendation ITU-R S.741.

2.2	The types of carriers normally used in radio-relay systems are:

–	Analogue:

–	FDM-FM,

–	TV-FM,

–	Digital:

–	large bandwidth carriers,

–	intermediate bandwidth carriers,

–	TDMA carriers.

# 3	Methodologies used to assess interference into FSS and FS carriers

## 3.1	Interference into FDM-FM and CFDM-FM carriers

See § 3.1 of Annex 1 to Recommendation ITU-R S.741. However, further study is required for interference into and from radio-relay systems employing FDM-FM of a small modulation index.

## 3.2	Interference into a non-FDM-FM carrier

See § 3.2 of Annex 1 to Recommendation ITU-R S.741.

Table 1 of Recommendation ITU-R S.741 is also applicable.

## 3.3	Interference from noise-like digital carriers

See § 3.3 of Annex 1 to Recommendation ITU-R S.741.

## 3.4	Interference from noise-like analogue carriers

See § 3.4 of Annex 1 to Recommendation ITU-R S.741.

## 3.5	Interference from slowly-swept TV carrier

See § 3.5 of Annex 1 to Recommendation ITU-R S.741.

## 3.6	Interference from TV-FM carrier without energy dispersal

Under study.

# 4	Protection criteria for FSS and FS carriers

Recommendations ITU-R SF.356 and ITU-R SF.558 deal with the interference noise allowance to be made in the noise budget for FSS carriers. Recommendations ITU-R SF.357 and ITU-R SF.615 deal with the interference noise allowance to be made in the noise budget for FS carriers.

Table 2 of Recommendation ITU-R S.741 is not applicable to the case of FSS-FS interference. Further study is required to develop a similar table which is applicable to FSS-FS interference.