RECOMMENDATION  443-1

BANDWIDTH  MEASUREMENTS  AT  MONITORING  STATIONS

(Question 26/1, Study Programme 26A/1)

(1966-1978)

Rec. 443-1

The CCIR,

CONSIDERING

(a)	the need for the measurement of bandwidths of emissions at monitoring stations to promote efficient use of the radio-frequency spectrum;

(b)	that the equipment for measuring occupied bandwidths at transmitting stations does not produce accurate measurements when employed at monitoring stations;

(c)	the slow progress of studies concerning the best equipment and methods for occupied bandwidth measurements;

(d)	the need for a uniform estimate of bandwidths at monitoring stations, to enable the IFRB to compare the results obtained by different monitoring stations;

(e)	Recommendation 328 and Reports 275, 324 and 420 (Dubrovnik, 1986),

UNANIMOUSLY  RECOMMENDS

that, until bandwidth measurement methods have been developed making full allowance for the specific character of the activities of monitoring stations, these stations should apply the method consisting of the measurement of the x dB bandwidth at 6 dB and 26 dB.

The x dB bandwidth is defined in § 1.14 of Recommendation 328. The x dB measurement procedures are described in § 4.1 of Recommendation 328. Reports 275, 324 and 420 (Dubrovnik, 1986) give x dB level values for a number of classes of emission at which the x dB bandwidths are either equal to the necessary bandwidths, or stand in a given relationship thereto, provided that the requirements relating to the limitation of bandwidths and out-of-band emissions indicated in § 2 and 3 of Recommendation 328 are fulfilled.

Report 275 also describes various methods of establishing the “0 dB” reference levels. By selecting appropriate x dB levels and assuming that an adequate signal to noise ratio is available at the monitoring station, it is also possible to determine if out-of-band emission spectra conform to the curves for the out-of-band spectra defined in § 3 of Recommendation 328 and Part E of Report 977. The indications contained in § 2 of Recommendation 328 should be adopted as a basis for this procedure.

–––––––––––