BS 03 : Request to register an assignment in the Master Register with all technical characteristics as recorded in the plan for a station in operation.

Prepare an electronic notice file to request a frequency assignment to record in the Master Register with the same technical characteristics as it is recorded in the GE84 plan.having the unique identification code NCA/FM0200 for the Administration of Ghana (GHA)

Solution

To prepare this conform notice we will use the “Generate TB notices” functionality of TerRaNotices and we will select Ghana (GHA) as the notifying administration.

- The type of notice to select for notification of assignment under article 11 with all technical characteristics as in the plan is “TB2”

- The completed notice in text format.

The next pages provide screen shots from TerRaNotices

- Open TerRaNotices and select “Generate TB notices “

- Then select the Notice type TB2.

- Then select the Domain, Fragment, notifying Administration. And enter the unique identification code as requested by the exercise. Then proceed to retrieve the assignment.

- Select the retrieved assignment to proceed.

- Then Generate the required conform notice by completing the required information.

Please refer to the Preface to BRIFIC for the list of Address codes and operating Agencies concerning your Administration which are recorded by BR.  Send your update to the BR if necessary.

- Validate and save file which is ready to be notified via Wisfat.