BS 04: Modification of the GE06D Plan (Digital Sound T-DAB)

Prepare an electronic notice for Addition of frequency 208.064 MHz assigned to a digital sound broadcasting station, for the modification of the GE06D Plan.

Solution

To prepare this notice we will use the “Wizard” functionality of TerRaNotices and we will select Mali (MLI) as the notifying administration.

- The ”Wizard” will identify the notice type to use, once the minimum required fields (Administration, Fragment, Station class, Action and Frequency) are entered.

- The purpose of the exercise is to request the notice to be published in GE84 plan; therefore, the fragment is GE84

- The completed notice in text format.

- The next pages provide screen shots from TerRaNotices

- Open TerRaNotices and select “Wizard “

- Then select  MLI as notifying administration and choose option  Update to one of the world regional plans.

Note: the Wizard proposes the appropriate Notice types at the bottom of the screen corresponding to the given criteria at each step.

- Then specify the action as “ADD” and “Assignment” and continue.

- By specifying the class of station “BC”,  the Wizard identifies the appropriate Notice type at the bottom of the screen. And Finish and confirm the summary to open the identified notice type GS1.

- Finish and confirm the summary to open the identified notice type GS1.

- Fill in the required fields as requested.

To calculate the effective antenna height (m) at 36 different azimuths, open Tools and select the option “Calculate Effective antenna heights (eff_hgt) using the SRMT3 Terrain database.

Then enter the Geographical coordinates and the height of antenna above ground level (m) and submit. The values of effective antenna height (m) at 36 different azimuths in 10 degrees interval will be filled automatically.

- Once the required fields are completed, validate the notice and save file which is ready to be submitted via Wisfat. (Web Interface for Submission of Frequency Assignments/allotments for Terrestrial services)